| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-1044 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 7th FG IPTV meeting: Qawra, St Paul’s Bay, Malta, 11 – 18 December 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Pixelmetrix Corporation |  |  |  |
| Title: | Suggested modifications to document FG-IPTV-DOC-0154 “Performance monitoring for IPTV” |  |  |  |

Introduction: This contribution presents a suggested change to the document FG-IPTV-DOC-0154 “Performance monitoring for IPTV”

Proposal 1: Replace Section 7.5.1 Channel Attributes with the following text:

The following parameters have been added in the section “Channel Attribute parameters” because they define the elementary stream properties of the content. The elementary stream properties are important as they define the output of an audio/video encoder. Monitoring these parameters can directly validate the configuration correctness of the encoders in the system.

The new parameters added are:

- Channel Name

- Number of components in each channel

- Resolution of Video content

- Chroma format of Video

- Frame rate of Video

- GOP (Group of Pictures) size

- Number of audio channels in the service

- Encoding format of audio channels: Mode, Sampling frequency and Layer

- Language of Audio channels

Also, the monitoring parameters have been represented under Video and Audio categories for more clarity.

Proposed New Section:

7.5.1 Channel Attribute Parameters

- Program ID (PID)

- Channel Name

- Number of components in the channel

- Type: Video/ Audio / Data

- Conditional access: Enabled/ Disabled

- Age rating

- Bit rate of the channel

Video Parameters:

- Aspect ratio

- Bit rate of the Video

- Encoding format of video content

- Resolution of Video content

- Chroma format

- Frame rate

- GOP (Group Of Pictures) size

Audio Parameters

- Bit rate of each audio channel

- Number of Audio channels in the service

- Encoding format of the audio channels

  - Mode

  - Sampling frequency

  - Layer

- Language

Proposal 2: Change the name of section 8.4 “IPTV Service Attribute Monitoring” to “ Service Attribute Monitoring” and include monitoring methodologies related to “Service line-up Parameters”(section 7.4) and “Channel Attribute Parameters” (section 7.5) in this re-named section.

__________