| Summary INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-DOC-0186 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 7th FG IPTV meeting: Qawra, St Paul’s Bay, Malta, 11-18 December 2007 |  |  |
| OUTPUT DOCUMENT |  |  |  |  |
| Source: | Editor |  |  |  |
| Title: | Application layer error recovery mechanisms for IPTV Services |  |  |  |

The ability to deliver high levels of service quality to users is an essential aspect of IPTV services, and thus captured in many IPTV-related requirements. IPTV services may be degraded if the media decoders are exposed to impairments such as packet loss. Mechanisms are needed to reliably ensure the quality of experience of the IPTV service in the presence of such defects.

This document describes specific mechanisms and the applicability of these mechanisms to IPTV services and network conditions, and provides recommendations and guidance on their use. It has been determined that with the use of these mechanisms, consumer television quality can be achieved using the standard Y.1541 QoS Classes 0 and 1.

Keywords

Forward error correction, retransmission, IPTV, error recovery, QoS

Introduction

Application layer reliability is an important aspect for IPTV services. Data being delivered over IP networks may suffer from packet losses. In case of the delivery of video and audio data errors being exposed to the media decoder generally degrade the IPTV service quality. Therefore, reliability support for them is essential to IPTV service.

The material in this document is intended to address the requirement that the IPTV architecture provide a mechanism for resiliency in the service provider infrastructure to maintain a high QoE for video services. The document addresses specific realizations of the content delivery error recovery function and the content delivery error recovery client function in the IPTV architecture.

The document provides an overview on application layer error recovery mechanisms and related standards. Usage guidelines for the mechanisms as well as specific recommendations are provided.

Current status

All sections of the document are in good shape.

The document could be extended with additional error recovery mechanism for streaming and download applications (e.g. based on ongoing work in DVB).

Dependency or relationship to other FG IPTV documents

- IPTV network control aspects document; defining the relevant protocols and configuration information

- IPTV architecture document; defining the architectural support for application layer error recovery mechanisms, however the use of application layer error recovery mechanisms is independent of a specific architecture

- IPTV services requirements document; providing the basic requirements

- IPTV traffic management mechanisms for the support of IPTV services  document; considers error recovery mechanisms for the assignment of IPTV service components to network QoS classes

Contents

1	Scope	4

2	References	4

3	Definitions	5

4	Abbreviations and acronyms	5

5	Conventions	6

6	Introduction	6

7	Overview on error recovery mechanisms and related standards	7

7.1	Overview	7

7.2	Retransmission	7

7.3	Forward Error Correction	9

7.3.1	FEC for Streaming Applications	9

7.3.2	FEC for Download Services	10

7.4	Hybrid Combinations of FEC with Retransmissions and Feedback	11

8	Usage Guidelines	12

8.1	General Discussion on Retransmission and FEC	12

8.2	Relation of mechanisms to the QoS classes in Y.1541	13

8.2.1	Overview	13

8.2.2	DVB-IP AL-FEC	14

8.2.2.1	Parameters	14

8.2.2.2	Relation to Network Performance Parameters	14

8.2.2.3	Recommended Parameter Settings	15

9 	Recommendations	16

Bibliography	17

Application layer error recovery mechanisms for IPTV Services

(December, 2007)

# 1	Scope

The ability to deliver high levels of service quality to users is an essential aspect of IPTV services, and thus captured in many IPTV-related requirements. As IPTV services can easily be degraded if the media decoders are exposed to impairments such as packet losses, mechanisms are needed to reliably deliver good IPTV service quality in the presence of such defects.

This document describes specific mechanisms and discusses the applicability of the mechanism to IPTV services and network conditions, and provides recommendations and guidance on their use.

# 2	References

The following ITU-T working text and other references contain provisions, which, through reference in this text, constitute provisions of this working text. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this working text are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this working text does not give it, as a stand-alone document, the status of a Recommendation

- [ITU-T Y.1541]		ITU-T Recommendation Y.1541 (2002) - Internet protocol aspects – Quality of service and network performance. Network performance objectives for IP-based services

- [ITU-T IPTV-0147]		ITU-T FG IPTV-DOC-0147 (2007), Service scenarios for IPTV

- [ITU-T IPTV-0181]		ITU-T FG IPTV-DOC-0181 (2007), IPTV architecture

- [ITU-T IPTV-0185]		ITU-T FG IPTV-DOC-0185 (2007), Traffic management mechanism for the support of IPTV services

- [ITU-T IPTV-0191]	ITU-T FG IPTV-DOC-0191 (2007), IPTV Related Protocols

- [ATIS-0800005] 	ATIS-0800005 (2006) - ATIS IPTV Packet Loss Report

- [IETF RFC2733] 	IETF RFC2733 (1999) - An RTP Payload Format for Generic Forward Error Correction

- [ETSI EN301192] 	ETSI EN 301 192(2004) - Digital Video Broadcasting: DVB Specification for Data Broadcasting

- [ETSI TS102034]	ETSI TS 102 034v1.3.1 (2007) – Digital Video Broadcasting (DVB); Transport of MPEG-2 TS Based DVB Services over IP Based Networks

- [ETSI TS102472] 	ETSI TS 102 472 (2006) - Digital Video Broadcasting (DVB): IP Datacast over DVB-H: Content Delivery Protocols

- [ETSI TS 126346] 	ETSI TS 126 346v6.2.0 (2005) - Universal Mobile Telecommunications System (UMTS); Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs (3GPP TS 26.346 version 6.2.0 Release 6)

- [SMPTE 2022-1]	SMPTE specification 2022-1 (2007) - Forward Error Correction for Real-time Video/Audio Transport Over IP Networks

- [IETF RFC3550]	IETF RFC3550 (2003) - Real Time Protocol: A Transport Protocol for Real-Time Applications

- [IETF RFC3556]	IETF RFC3556 (2003) - Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol

- [IETF RFC3605]	IETF RFC3605 (2003) - Real Time Control Protocol (RTCP) Attribute in Session Description Protocol (SDP)

- [IETF RFC3611]	IETF RFC3611 (2003) Real Time Protocol Control Protocol Extended Reports (RTCP XR)

- [IETF RFC4585]	IETF RFC4585 (2004) - Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF)

- [IETF RFC4588]	IETF RFC4588 (2005) - RTP Retransmission Payload Format

- [IETF RFC5053]	IETF RFC5053 (2007) - Raptor Forward Error Correction Scheme for Object Delivery

# 3	Definitions

This section left intentionally blank

# 4	Abbreviations and acronyms

This working document uses the following abbreviations and acronyms:

3GPP	Third Generation Partnership Project

ALC	Asynchronous Layered Coding

AL-FEC	Application Layer FEC

ARQ	Automatic Repeat request

ATIS IIF	Alliance for Telecommunication Industry Solutions - IPTV Interoperability Forum

AVP	Audio-Visual Profile

CBR	Constant Bit Rate

CDP	Content Delivery Protocol

DSL	Digital Subscriber Line

DVB	Digital Video Broadcasting

DVB-H	Digital Video Broadcasting - Handheld

ECG	Electronic Content Guide

EPG	Electronic Program Guide

FEC	Forward Error Correction

FLUTE	File Delivery over Unidirectional Transport

IETF	Internet Engineering Task Force

IP 	Internet Protocol

IPDV 	IP packet Delay Variation

IPER 	IP packet Error Ratio

IPLR 	IP packet Loss Ratio

IPTD 	IP Packet Transfer Delay

IPTV	Internet Protocol TeleVision

ITU-T 	International Telecommunication Union – Telecommunication Standardization Sector

LCT	Layered Coding Transport

MBMS	Multimedia Broadcast/Multicast Service

MPEG	Moving Pictures Experts Group

MTBA	Mean Time Between Artefacts

NACK	Negative ACKnowledgement

P2P	Point-to-Point

QoE	Quality of Experience

QoS	Quality of Service

RFC	Request for Comments

RMT	Reliable Multicast Transmission

RTCP	Real-Time Control Protocol

RTP	Real-Time Protocol

RTT	Round Trip Time

TCP	Transmission Control Protocol

TS	Transport Stream

UDP	User Datagram Protocol

VoD	Video on Demand

XOR	eXclusive OR

# 5	Conventions

This section left intentionally blank.

# 6	Introduction

Application layer reliability is an important aspect for IPTV services. Data being delivered over IP networks may suffer from packet losses. In case of the delivery of video and audio data errors such as packet losses or bit errors being exposed to the media decoder generally degrade the IPTV service quality. Moreover, losses in the metadata such as electronic program guide (EPG), electronic content guide (ECG), and interactive user data may cause more severe problem in IPTV service. Therefore, reliability support for them is essential to IPTV service.

Retransmission, forward error correction (FEC), and hybrid combinations of both are known mechanisms for error recovery. When an error recovery scheme and the associated protocol is selected, at least the following aspects should be taken into account:

- type of IPTV service, e.g., real time streaming video, EPG, application data

- type of data delivery mechanisms, e.g., broadcast, multicast, unicast, overlay multicast, and P2P,

- protocol or processing overhead at senders and receivers,

- network bandwidth overhead  aspects

The material in this document is intended to address the requirement that the IPTV architecture provide a mechanism for resiliency in the service provider infrastructure to maintain a high QoE for video services [ITU-T IPTV-0147]. The document addresses specific realizations of the content delivery error recovery function and the content delivery error recovery client function in the IPTV architecture [ITU-T IPTV-0181]. Reference point E4 is used to exchange messages for delivering and requesting error recovery information [ITU-T IPTV-0181]. Protocol related aspects to application layer error recovery mechanisms are discussed in [ITU-T IPTV-0191].

The document is structured as follows: Clause 7 provides an overview on application layer error recovery mechanisms and related standards. Clause 8 provides usage guidelines for the mechanisms. Finally, Clause 9 provides conclusions and recommendations.

# 7	Overview on error recovery mechanisms and related standards

## 7.1	Overview

This clause introduces retransmission-based error recovery functions, FEC-based error recovery functions, and combinations of the two. Retransmission and FEC are two different techniques to recover packet losses during the delivery of IPTV services. The retransmission approach recovers from packet losses by requesting retransmission from the sender or intermediate retransmission server. The FEC approach operates by adding redundant information to the data at the sender.

An FEC-based error recovery protocol uses redundant information to allow the receiver to correct packet losses. With this redundant information, the receivers can recover from packet losses locally at the receiver. FEC may be used in unicast and multicast delivery.

Retransmission-based error recovery protocols use feedback messages to recover from packet losses, thereby requiring a return feedback path and one or more retransmission servers. On detecting a packet loss, e.g. by noting the gap of packet sequence number, a receiver requests a sender or designated repair servers to retransmit the lost packets. Retransmission may be either unicast or multicast depending on the distribution of clients reporting the errors.

## 7.2	Retransmission

RTP retransmission is one viable packet loss recovery technique for real-time applications.  Retransmitted RTP packets can be sent in a separate stream from the original RTP stream.  Like TCP retransmissions, it is assumed that feedback from receivers to senders is available, but, unlike TCP, RTP/UDP does not mandate congestion control by reducing the packet transmission rate, thereby making RTP more appropriate for broadcast–grade video.

The companion protocol to the Real Time Protocol (RTP), the Real Time Control Protocol (RTCP) as specified in [IETF RFC 3550] does not acknowledge single RTP packets but does report statistics on packet loss and jitter.  The source node can evaluate the statistics to decide if adaptation is appropriate.  Recently, an extension to the RTCP for the Audio-Visual Profile (AVP) enables receivers to provide, statistically, more immediate feedback to senders, allowing for efficient feedback-based repair mechanisms (e.g., retransmission) to be implemented.

More specifically, the RTP Retransmission related specifications from the IETF uses a simple system in which clients request retransmission of specific, lost packets by sending negative acknowledgements (RTCP NACK) to a feedback target/retransmission source over the RTCP flow of the RTP session.  A receiver can use a single NACK packet to request transmission for one or multiple lost packets.  Then the retransmission source responds with a retransmission of the missing packets over IP unicast or alternatively via IP multicast (for multicast sessions).

References for RTP, RTCP and RTP retransmissions are detailed in Table 7-1.

Table 7-1:  IETF RTP and RTCP RFCs status

| RFC | Date | Status | Title | Abstract | Reference |
| --- | --- | --- | --- | --- | --- |
| 3550 | 07-03 | S | Real Time Protocol: A Transport Protocol for Real-Time Applications | The Real Time Protocol provides end-to-end network transport functions suitable for applications transmitting real-time data, such as audio, video or simulation data, over multicast or unicast network services. | [IETF RFC3550] |
| 3556 | 07-03 | S | Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol | This document defines an extension to the Session Description Protocol (SDP) to specify two additional modifiers for the bandwidth attribute. These modifiers describe the bandwidth allowed for Real Time Control Protocol (RTCP) packets in a Real-time Transport Protocol (RTP) session. | [IETF RFC3556] |
| 3605 | 10-03 | S | Real Time Control Protocol (RTCP) Attribute in Session Description Protocol (SDP) | When the session crosses a network address translation device that also uses port mapping, the ordering of ports can be destroyed by the translation. To handle this, the document proposes an extension attribute to SDP. | [IETF RFC3605] |
| 3611 | 11-03 | S | Real Time Protocol Control Protocol Extended Reports (RTCP XR) | This document defines the Extended Report (XR) packet type for the Real Time Control Protocol (RTCP) and defines how the use of XR packets can be signaled by an application if it employs the Session Description Protocol (SDP). | [IETF RFC3611] |
| 4585 | 8-04 | S | Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF) | This document defines an extension to the Audio-visual Profile (AVP) that enables receivers to provide, statistically, more immediate feedback to the senders and thus allow for short-term adaptation and efficient feedback-based repair mechanisms to be implemented. | [IETF RFC4585] |
| 4588 | 9-05 | S | RTP Retransmission Payload Format | This document describes an RTP payload format for performing retransmissions. | [IETF RFC4588] |
| RFC Editor Queue | 3-06 |  | draft-ietf-avt-rtcpssm-13.txt | Extension that specifies how unicast NACK RTCP packets are correlated to the multicast sender for a particular RTP stream. | [b_IETF AVT-RTCPSSM] |

Legend: S = Proposed Standard

## 7.3	Forward Error Correction

Forward Error Correction (FEC) at the Application/Transport layers generally refers to packet erasure correction techniques. In these techniques, an amount of data is sent which is in total greater than the stream or the object to be communicated, with the property that the stream or the object can be reconstructed from any sufficiently large subset of the transmitted data. The stream or object is thus resilient to a certain amount of loss (at most the difference between the transmitted and the original data size).

### 7.3.1	FEC for Streaming Applications

In general, for streaming applications, there are considerable advantages in using systematic FEC codes, in which the original packets of the stream source packets are sent accompanied by a certain overhead of “repair” packets. The repair packets can be used to recover source packets which have been lost between sender and receiver.

Many possible application/transport FEC schemes for streaming media exist which could be applied to IPTV. The following are some of the erasure correction schemes for streaming media that have been standardized elsewhere:

- IETF RFC2733 [IETF RFC2733]

  - This defines a simple mechanism for applying short block parity codes to RTP streams. The scheme is limited by the small number of packets that can be protected as a block (24 packets). This RFC has not been widely implemented and will likely soon be obsolete by an update which provides slightly longer blocks (48 packets) and the possibility to apply unequal protection to different parts of each packet.

- ETSI TS 126 346 - Universal Mobile Telecommunications System (UMTS); Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs  [ETSI TS 126 346]

  - This standard defines a generic framework for application of FEC to media streams. The framework is not specific to RTP and operates just above the UDP layer. This framework could be used with many FEC codes, however 3GPP specify and require support of a single specific code (the Digital Fountain Raptor code).

- ETSI EN 301 192 Digital Video Broadcasting: DVB Specification for Data Broadcasting [ETSI EN 301 192]

  - This defines how a link layer erasure code is intended to be used with the DVB-H system for transmission to mobile terminals. This FEC scheme operates below the IP layer and is completely independent of applications and is based on a large Reed-Solomon erasure code.

- SMPTE Specification 2022-1: Forward Error Correction for Real-time Video/Audio Transport Over IP Networks [SMPTE-2022-1]

  - The Pro-MPEG COP3/SMPTE 2022 [SMPTE-2022-1] standard has been designed to ensure that high quality video that is used by broadcasters for their most valuable live video feeds are able to be transported over these networks. COP#3 FEC is designed to protect a video stream from burst packet losses. The generation of FEC packets in the COP #3 standard is based upon a matrix defined by the parameters L and D. FEC packets are generated by exclusive OR (XOR) of the media packets. While Pro-MPEG COP #3 FEC may be adequate for private IP links, it is not robust enough to handle the challenges associated with moving video over highly loss IP networks such as the Public Internet.

- ETSI TS 102 034 version 1.3.1 Transport of MPEG 2 Transport Stream (TS) Based DVB Services over IP Based Networks [ETSI TS 102 034]

  - DVB has recently completed an evaluation of FEC codes for IPTV applications within the IPI working group [b_DVB A115]. As a result of this activity, DVB has produced an FEC specification for IPTV based on a layered approach with a base layer consisting of a simply parity (XOR) code taken from [SMPTE-2022-1] and an enhancement layer based on the Raptor code as used in the 3GPP specification [ETSI TS 126 346]. The DVB-IP AL-FEC mechanism has been carefully reviewed and assessed in the DVB project to meet the requirements of an FEC mechanism in different IPTV environments with respect to performance, flexibility, and implementation aspects.

The IETF has also initiated a new working group fecframe to standardize a framework for application of FEC to media streams; this is along similar lines defined by 3GPP (Multimedia Broadcast/Multicast Service for 3G cellular networks). The framework will not specify a particular FEC code but will use an approach similar to that adopted by the IETF RMT working group, which standardized protocols for reliable file delivery over IP multicast. This approach allows the most powerful FEC codes to be used for streaming mechanisms as opposed to [IETF RFC 2733], which provides a flexible framework that applies to any type of streaming format. The DVB-IP AL-FEC makes use of this concept as well.

The FEC framework is currently in draft status within the IETF.

### 7.3.2	FEC for Download Services

References for the relevant IETF reliable multicast transport (RMT) documents are provided in Table 7-2. The IETF has defined FEC Building Block, which describes how the specification of protocols, which use FEC, could be separated from specification of the FEC codes themselves. This has resulted in creation of a set of plug & play specifications which can be combined according to the needs of a given application.

Table 7-2:  IETF RMT RFC status

| RFC | Date | Status | Title | Abstract | Reference |
| --- | --- | --- | --- | --- | --- |
| 2887 | 08-00 | I | The Reliable Multicast Design Space for Bulk Data Transfer | This document provides an overview of the design space and application constraints. | [b_IETF RFC2887] |
| 3048 | 01-01 | I | Reliable Multicast Transport Building Blocks for One-to-Many Bulk-Data Transfer | This document describes a framework for the standardization of bulk-data reliable multicast transport. | [b_IETF RFC3048] |
| 3269 | 04-02 | I | Author Guidelines for RMT Building Blocks and Protocol Instantiation Documents | This document provides general guidelines to assist the authors of Reliable Multicast Transport (RMT) building block and protocol instantiation definitions. | [b-IETF RFC3269] |
| 3450 | 12-02 | E | Asynchronous Layered Coding Protocol Instantiation | This document describes the Asynchronous Layered Coding (ALC) protocol, a massively scalable reliable content delivery protocol. | [b_IETF RFC3450] |
| 3451 | 12-02 | E | Layered Coding Transport (LCT) Building Block | Layered Coding Transport (LCT) provides transport level support for reliable content delivery and stream delivery protocols. The transport is specifically designed to support protocols using IP multicast, but also provides support to protocols that use unicast. | [b_IETF RFC3451] |
| 3453 | 12-02 | I | The Use of Forward Error Correction in Reliable Multicast | This memo describes the use of Forward Error Correction (FEC) codes to efficiently provide one-to-many reliable data transport. Different classes of FEC codes and some of their basic properties are described. | [b_IETF RFC3453] |
| 3695 | 02-04 | E | Compact Forward Error Correction (FEC) Schemes | This document introduces some Forward Error Correction (FEC) schemes with a more compact FEC payload. The schemes can deliver blocks of an object of indeterminate length. | [b_IETF RFC3695] |
| 3926 | 10-04 | E | FLUTE - File Delivery over Unidirectional Transport | This document defines a protocol for the unidirectional delivery of files that is suited to multicast networks. The specification builds on Asynchronous Layered Coding, the base protocol for massively scalable multicast. | [b_IETF RFC3926] |
| 5052 | 09-07 | E | Forward Error Correction Building Block | This document describes how to use Forward Error Correction (FEC) codes to efficiently provide and/or augment reliability for data transport. | [b_IETF RFC5052] |
| 5053 | 09-07 | S | Raptor Forward Error Correction Scheme for Object Delivery | This document describes a Fully-Specified Forward Error Correction (FEC) scheme, corresponding to FEC Encoding ID 1, for the Raptor forward error correction code and its application to reliable delivery of data objects. | [IETF RFC5053] |

Legend: I = Informative, E = Experimental, S = Proposed Standard

FEC in download delivery services, which could be applied to IPTV services, has been standardized elsewhere. All of them rely on the IETF RMT RFCs in Table 7-2, especially on FLUTE, LCT and ALC.

- ETSI TS 126 346 - Universal Mobile Telecommunications System (UMTS); Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs [ETSI TS 126 346]

  - For the download delivery service of this standard File Delivery over Unidirectional Transport (FLUTE) as specified in [b_IETF RFC3926] is applied. FLUTE provides mechanisms to signal and map the properties of a file to the Asynchronous Layered Coding (ALC) protocol [b_IETF RFC3451] such that receivers can assign these parameters to the received files. The object to be transmitted is partitioned in one or several source blocks. For each source block, additional repair symbols can be generated by applying an FEC code. FLUTE could be used with different FEC codes, however 3GPP specifies and requires support of a single specific code (the Raptor code [IETF RFC5053]). FLUTE with Raptor allows the support of very efficient scheduled multicasting as well as carousel services. The latter is efficiently supported by the fountain property of the Raptor code.

- ETSI TS 102 472 Digital Video Broadcasting: IP Datacast over DVB-H: Content Delivery Protocols [ETSI TS 102 472]

  - The Content Delivery Protocol (CDP) for file delivery in DVB-H re-uses the technology of 3GPP MBMS [ETSI TS 126 346] download delivery. The use of Raptor codes is strongly recommended, especially for files with size larger than 32 kByte.

## 7.4	Hybrid Combinations of FEC with Retransmissions and Feedback

FEC and retransmission technologies for error recovery are not necessary competing technologies, but they may be used combined and complementary. By applying such combinations, some interesting benefits may be obtained. The significance of the benefits depends, among others, on the considered service, the considered distribution mean, i.e. multicast or unicast, and/or the number of available retransmission servers. This clause provides a high-level overview of possible combinations.

For retransmission-based mechanisms when combined with an FEC repair mechanism, negative acknowledgements (NACKs) of packets may result in the transmission of repair packets instead of original data packets. This may be beneficial especially for the case of multicast transmission as the repair packets may serve the retransmission request of several receivers, which may have observed the loss of different data packets. Such a scheme may allow to reduce the average transmit bandwidth.

For FEC-based mechanisms, the introduction of feedback messages may be used to influence the sender strategy. For example, if receivers are aware that the sender will transmit some small amount of initial repair data for the current source block, retransmission requests need only to be sent in case this initial repair information is not sufficient. If the loss exceeds what can be repaired by the initial repair data, retransmission requests can be made and in response the sender can send additional repair data for the source block that is independent of the initial repair data. Such a scheme may reduce the amount of necessary feedback messages and therefore may allow reducing the amount of necessary retransmission servers when compared to conventional retransmission mechanisms. In an alternative setup, a default level of AL-FEC protection is provided which is capable of correcting all anticipated errors (i.e. the same level of AL-FEC protection as for an AL-FEC-only mechanism). For each FEC source block, the receiver may send an acknowledgement packet requesting that sending of FEC data for that source block should be terminated early, because the receiver has received enough data to recover the block. Such scheme may allow reducing the average bandwidth when compared to an AL-FEC-only mechanism.

# 8	Usage Guidelines

## 8.1	General Discussion on Retransmission and FEC

This clause discusses general usage guidelines for Retransmission and FEC.

The main advantage of FEC is that there is no need for a back channel to request retransmission from the sender. It therefore very suitable for uni-directional communications, but can obviously also be used on any (bidirectional) network. FEC introduces a fixed delay due to the generation and processing of the redundant information and as data must be buffered before the error recovery can take place. The delay depends, among others, on service bitrates, FEC block size, and sending arrangements.

In case of retransmission-based error recovery, since the error recovery is handled by requesting and receiving a retransmitted packet from a server with a copy of the original stream, the time needed to receive a repair packet is composed of the Round Trip Time (RTT) between the receiver and the recovery retransmission server plus any jitter imposed in the packet repair process.

Jitter could be caused by variations in time for detecting lost packet and for generating the repair packet due to server loads. Buffering at the receiver is needed to provide time to receive repair packets before the data stream is sent to the decoder. There are trade-offs among buffer size, latency through the buffers, server load based on the number of users on the server, and quality of the video delivery.

In terms of bandwidth consumption in case of FEC, since the redundant information should be always be sent along with the original packets, it generally consumes more average bandwidth than retransmission. Since a retransmission is done on demand, the additionally required average bandwidth for error recovery can be lowered. However, note that to serve the retransmission request fast enough, a certain peak bandwidth higher than the average bandwidth needs to be reserved for retransmission. Therefore, for the case of retransmission, the bandwidth being saved compared relative to FEC may generally only be used for lower priority traffic on the shared access.

The complexity in encoding and decoding FEC data may provide some computational overhead at both the sender and receiver. Retransmission approaches do not require extra decoder computing resources because the entire lost packet is retransmitted to the receiver.

In the retransmission-based approach, an intermediate retransmission server may receive many feedback messages from multiple receivers who experience packet losses. This may lead to the phenomenon of feedback implosion, the concept of receiving many feedback messages for common errors.

To reduce the problem of feedback implosion for retransmission-based mechanisms, a careful architectural design is necessary. When retransmission is used in multicast delivery, proper attention must be paid to the ratio of clients to retransmission servers to avoid any scalability issues. A distributed retransmission approach can be introduced to resolve the issues of timeliness, scalability, and feedback implosion. A local retransmission server placed in the vicinity of the receiver can be used to recover the losses across the access and consumer environment and a core retransmission server located near the head end can be used to recover losses in the core and common losses requested via the local retransmission server. However, such architecture may require additional network equipment to serve retransmission requests or the function may reside in servers already present for other purposes. Due to the different architectural design, feedback implosion is not existent for FEC-based approaches.

Any application layer recovery scheme has a non-zero probability that a complete recovery from packet loss cannot be done. Lowering the probability for missing packets is a balance between complexity of design and quality of the received video.  In case of FEC, if the packet loss exceeds the repair capabilities of the FEC scheme, recovery will not be possible. In case of retransmission, recovery may be deficient due to non-persistent Automatic Repeat reQuest (ARQ) protocols or late arrival of packets.

In these unlikely cases of failures, media post-decoder remediation schemes may need to be used.

## 8.2	Relation of mechanisms to the QoS classes in Y.1541

## 8.2.1	Overview

[ITU-T Y.1541] describes a number of QoS classes for IP networks. Specifically, Table 1/Y.1541 defines six IP network QoS classes and the respective network performance objectives. In addition, Table 3/Y.1541 proposes two additional provisional QoS classes mainly for the purpose to support sufficient QoS for digital television transmission. The difference to those in Table 1/Y.1541 is that these values need not to be met by networks until they are revised (up or down) based on operational experience.

Appendix VIII of [ITU-T Y.1541] considers digital television transmission on IP networks and concludes that, by the use of specific application layer error recovery mechanism digital television quality requirements can be met using the new provisional QoS classes.

However, as stated in the document additional experience on application layer error recovery mechanisms may allow revising the considerations on QoS classes 6 and 7. Error recovery mechanisms being considered in the ITU-T IPTV architecture are related to the QoS classes in Y.1541 and appropriate parameter settings should be applied.

For this relation, suitable performance metrics shall be applied. Table VIII.1/Y.1541 proposes some loss/error considerations. DVB and the ATIS IIF [ATIS-0800005] propose to measure the performance of mechanisms in terms of the Mean Time Between Artefacts (MTBA) in the video playout. Both have used a quality target of 4 hours MTBA, which also fits into the loss ratio recommendations of Table VIII.1/Y.1541.

Y.1541 QoS classes specify an upper bound on the packet loss ratio (Class 0-4: 10-3, provisional classes 6 and 7: 10-5), but do not specify packet loss patterns. However, suitable parameter settings to meet the requirements may be affected not only by the loss rate, but also by loss patterns. In order to present application layer reliability performance with different loss patterns, different packet loss models may be considered, e.g. independent random loss model or burst loss models.

Y.1541 also specifies limits on the IP Packet Transfer Delay (IPTD), IP Packet Delay Variation (IPDV), and IP Packet Error Rate (IPER). The effect of these parameters in relation to the error recovery mechanism is also of interest.

The performance and parameter settings for a certain mechanism may also depend on the service bit rate. This effect should also be considered when relating mechanisms to QoS classes.

## 8.2.2	DVB-IP AL-FEC

## 8.2.2.1	Parameters

The DVB AL-FEC code according to [ETSI TS 102 034], Annex E, is a block erasure code, meaning that it applies erasure protection to blocks of packets of the original stream. The code is basically fully determined by two parameters:

- The size of each of these blocks is usually expressed in terms of the protection period, which is the interval of time taken to send the packets of a block.

- For each block, the AL-FEC code provides a number of additional “repair” packets that can then be sent immediately after the original packets of the block (the “source” packets). The number of repair packets sent for each block is another parameter of the code, usually expressed as the FEC overhead – the ratio of repair packets to source packets.

There is a trade-off between these two parameters of the code, protection period and FEC overhead: in general if the protection period is increased then the FEC overhead required will decrease and vice versa, all other factors remaining equal. The ‘other factors’ are the packet loss rate and pattern and the quality target. This feature allows service operators to trade efficiency vs. channel switching times.

Note that the channel switching times correlate to the latency added by the AL-FEC. However, there are also many other things which contribute to channel change time, for example IGMP latency, the need to wait for an IDR frame, RTP buffering, video decoding buffer etc. These factors have already led to the development of a number of channel change acceleration techniques. A good survey is available in [b_ISMA TD00096] and some of the techniques described there are already deployed. These techniques can be used to mitigate the additional delay caused by the use of FEC, making it practical to consider relatively long protection periods without significantly impacting channel change time.

## 8.2.2.2	Relation to Network Performance Parameters

By some representative investigations, the AL-FEC mechanism is related to the different network parameters.

For the IP packet loss ratio (IPLR) two models are considered:

- An independent random loss model assumes that each packet is lost with independent probability. Although in practice IP packet losses are not independent, this channel provides some kind of baseline from which other cases can be assessed.

- A short burst loss model considers burst outages of fixed duration, occurring at independent random intervals (Poisson distribution). This is intended to simulate a DSL access line subject to electrical impulse noise. In this case each impulse causes an outage equal in length to the DSL inter-leaving depth, which we take to be 8ms.

For a streaming service such as IPTV the absolute IPTD is only important insofar as it affects channel change time and so need not be considered further here for the simulations. The IPTD is additive to the latency introduced by the AL-FEC mechanism.

The IP packet delay variation (IPDV) may affect performance if it results in packets arriving too late to be rendered to the user. The use of FEC mitigates this problem, since as long as each packet arrives before the appointed time to decode its FEC block there will be no problem. Packets at the beginning of a block could arrive extremely late and still arrive in time. On the other hand if packets at the end of the block arrive too late then cannot be used, but these packets may be considered lost and recovered by the FEC.

In general, as long as the IPDV is in the range of the protection period, or the protection period is greater than the IPDV as defined in Y.1541, then for QoS classes 0-4 the IPDV does not have any influence on the performance. Therefore, for QoS classes 0 and 1 and protection periods of at least 50ms, the IPDV does not influence the performance. Furthermore, it should be noted that with the addition of modest IPDV requirements, then Y.1541 Classes 2-4 would also be suitable for IPTV applications, especially if the protection period is relaxed.

It is expected that erroneous packets are detected by the UDP checksum and therefore are converted to packet losses. As the IPER for QoS classes 0-4 is a magnitude less than the IPLR, this effect is negligible.

## 8.2.2.3	Recommended Parameter Settings

The recommended parameter settings for the DVB-AL-FEC mechanism for QoS classes 0-4 are provided in the following. The influence of IPTD, IPDV, and IPER has been discussed in clause 8.3.2.2. Therefore, the benchmarking and recommended parameter setting are provided for IPLR of 10-3 for two channel models, namely independent random packet losses (random) and the short burst model (burst) with 8ms independent burst losses.

Results are obtained for Standard Definition (2.1Mbit/s) and High Definition (9.4Mbit/s) video streams. The streams are assumed to be CBR MPEG-2 Transport Streams (TS) encapsulated within RTP packets with 7 MPEG-2 TS packets per RTP packet to achieve a MTBA of at least 4 hours. Table 8-1 shows the required overhead for different bit rates, different channel models at IPLR 10-3, and different protection periods.

Table 8-1:  Required Overhead for DVB-IP AL-FEC for different bit rates, different channel models at IPLR of 10e-3, and different protection periods

| Protection Period | Random, 2.1 Mbit/s | Random, 9.4 MBit/s | Burst, 2.1 Mbit/s | Burst, 9.4 Mbit/s |
| --- | --- | --- | --- | --- |
| 100 ms | 16% | 5% | 20% | 12% |
| 200 ms | 8% | 3.5% | 10% | 6% |
| 400 ms | 5% | 3% | 7% | 4% |
| 600 ms | 4% | 2% | 4% | 2.5% |
| 800 ms | 3.5% | 2% | 4% | 2.5% |
| 1000 ms | 3% | 2% | 4% | 2% |

The results show that with modest additional delay and low to modest FEC overhead, a consumer television quality target of a MTBA of 4 hours can be achieved using the standard Y.1541 QoS Classes 0 and 1. The provisional QoS classes 6 and 7 are not required by the use of the DVB-IP AL-FEC mechanism. Note that in all cases the enhanced decoder according to, Annex E, sub-clause E.5.1.2 of [ETSI TS102 034], was applied as the minimum decoder could not fulfil the service requirements.

In addition, note that if the FEC source block structure is chosen with proper alignment, for example with the random access points of the video stream, then the FEC latency can be absorbed into the video decoding buffer latency. This would mean that the protection period is in general not additive to the end-2-end latency, but generally lower. With encoding parameters and alignment strategies, even no difference in the latency may be observed when FEC is used and when it is not.

# 9 	Recommendations

The support of an application layer error recovery mechanism is not required for all networks, in particular for networks that can fulfil the desired IPTV service requirements.

In the case that a network cannot fulfil the packet loss requirements necessary to achieve the IPTV service requirements, the use of an application layer error recovery mechanism is recommended.

For an application layer error recovery mechanism based on FEC, the DVB-IP AL-FEC in Annex E of [ETSI TS 102 034] is recommended.

- The relation of this AL-FEC mechanism to the QoS classes in Y.1541 [ITU-T Y.1541], especially recommended parameters settings for different QoS classes and different services is discussed in clause 8.2.2, specifically in Table 8-1. Consumer television quality can be achieved using the standard Y.1541 QoS Classes 0 and 1 together with the DVB-IP AL-FEC mechanism, low to modest overhead and the enhanced decoder according to [ETSI TS 102 034], Annex E, sub-clause E.5.1.2.

Note: An application layer error recovery mechanism based on retransmission may also be recommended.

# Bibliography

- [b_DVB A115]	DVB A115, Blue Book on AL-FEC Evaluations

- [b_IETF RFC2887]	IETF RFC2887 (2000) - The Reliable Multicast Design Space for Bulk Data Transfer

- [b_IETF RFC3048]	IETF RFC3048 (2001) - Reliable Multicast Transport Building Blocks for One-to-Many Bulk-Data Transfer

- [b_IETF RFC3269]	IETF RFC3269 (2002) - Author Guidelines for RMT Building Blocks and Protocol Instantiation Documents

- [b_IETF RFC3450]	IETF RFC3450 (2002) - Asynchronous Layered Coding Protocol Instantiation.

- [b_IETF RFC3451]		IETF RFC3451 (2002) - Layered Coding Transport (LCT) Building Block

- [b_IETF RFC3453]	IETF RFC3453 (2002) - The Use of Forward Error Correction in Reliable Multicast

- [b_IETF RFC3695]		IETF RFC3695 (2004) - Compact Forward Error Correction (FEC) Schemes

- [b_IETF RFC3962]		IETF RFC3962 (2004) - FLUTE - File Delivery over Unidirectional Transport

[b_IETF RFC5052]		IETF RFC5052 (2007) - Forward Error Correction (FEC) Building Block

- [b_IETF AVT-RTCPSSM] 	IETF AVT draft-ietf-avt-rtcpssm-13.txt (2007), RTCP Extensions for Single-Source Multicast Sessions with Unicast Feedback

- [b_ISMA TD00096]	Internet Streaming Media Alliance (ISMA) TD00096 (2007), “Fast channel changing in RTP” Internet Streaming Media Alliance

___________________

This material has been taken in large part from ATIS-0800005 [ATIS-0800005], clause 3.1.2.