| Genève, le 5 décembre 2006 UNION INTERNATIONALE DES TÉLÉCOMMUNICATIONS Bureau de la normalisation des télécommunications |  |
| --- | --- |

| Réf: Tél.: Fax: E-mail: | Circulaire TSB 126 COM 15/GJ +41 22 730 5515 +41 22 730 5853 tsbsg15@itu.int | - Aux administrations des Etats Membres de l'Union |
| --- | --- | --- |
|  |  | Copie: - Aux Membres du Secteur UIT-T; - Aux Associés de l'UIT-T; - Aux Président et Vice-Présidents de la Commission d'études 15; - Au Directeur du Bureau de développement des télécommunications; - Au Directeur du Bureau des radiocommunications |

| Objet: | Questionnaire sur les navires câbliers et les équipements submersibles |
| --- | --- |

Madame, Monsieur,

1	A sa dernière réunion tenue à Genève du 30 octobre au 10 novembre 2006, la Commission d'études 15 a décidé, dans le cadre des études effectuées au titre de la Question 8/15 (Caractéristiques des systèmes de transmission par câble sous-marin à fibres optiques), de réviser la Recommandation G.971 (Caractéristiques générales des systèmes de câbles optiques sous-marins) et de mettre à jour l'Appendice I existant qui contient des renseignements sur les navires câbliers et les équipements submersibles.

2	Il est prévu de soumettre, pour approbation, la Recommandation G.971 révisée à la prochaine réunion de la Commission d'études 15 (juin 2007).

3	A cette fin, nous comptons sur votre concours pour nous aider à examiner et à mettre à jour les renseignements figurant actuellement dans l'Appendice I de la Recommandation G.971.

Nous vous invitons donc à modifier, si nécessaire, les données existantes sur les navires câbliers et les équipements submersibles figurant dans l'Annexe I de la présente Lettre circulaire, dans laquelle est reproduit le texte de l'Appendice G.971.

Si les équipements figurant dans la liste ont déjà été éliminés, et/ou si de nouveaux navires câbliers et de nouveaux équipements submersibles ont été construits depuis 2004, veuillez les décrire (en anglais) comme indiqué dans l'Annexe II.

4	Nous vous prions de bien vouloir renvoyer tous ces renseignements à l'Editeur de la Recommandation G.971 avant la fin du mois de mars 2007:

M. K. Nakajima
NTT Access Network Service Systems Labs
1-7-1, Hanabatake, Tsukuba, Ibaraki
305-0805, Japon
E-mail: nakaji@ansl.ntt.co.jp
Télécopie: +81 29 868 6440

5	Je compte sur votre coopération pour formuler des réponses aussi précises que possible et les communiquer à l'Editeur, dont le nom et les coordonnées figurent ci-dessus, avant la date limite.

Veuillez agréer, Madame, Monsieur, l'assurance de ma haute considération.

H. Zhao
Directeur du Bureau de la
normalisation des télécommunications

Annexes: 2

ANNEX I (to TSB Circular 126)

Data on cable ships and submersible equipments of various countries

Section 1 – Cable ships

| Name of ship | Year of con-struc-tion | Dis-place-ment(tons) | Overall length (m) | Draft (m) | Normal speed (knots) | Range (auto-nomy) (nautical miles) | Number of tanks | Cable capacity |  |  | Cable gear |  |  |  | Maximum operating depth (m) | Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Cable |  | Re-peaters | Cable engine |  | Unwinding pulley |  |  |  |
|  |  |  |  |  |  |  |  | Cubic metres (m3) | Weight (tons) |  | Drum (diameter) (m) | Linear (pairs of wheels) | Bow sheave (diameter) (m) | Stern sheave (diameter) (m) |  |  |
|  |  |  |  |  |  |  | DENMARK Ships belonging to Tele Denmark |  |  |  |  |  |  |  |  |  |
| Peter Faber | 1982 | 3680 | 78.35 | Ice 3.8 Summer 5.0 | 13.0 | 7000 | 1 tank 1 hold | 310 230 | 600 400 | App. 10 | 3.0 |  | 2 x 3.0 | – | 4000 | Reinforced for operation in icefilled waters. A-frame for ROV. Two hydraulic double-drum warping winches. |
| Maersk Fighter | 1992/94 | 2961 | 82.5 | 6.24 | 15.7 Max | 7700 | 2 | 1263 | 2400 | 24 | 4.0 (25t) | 65 (4t) | 4000 | – | – | Laying/burying and repair of all types of cables (coaxial, optical fibre and power cables). Ploughs and ROV capability. |
| Heimdal | 1982 / 2000 | 11493 | 136.7 | 6.60 | 15 Max | 16000 | 4 | 4670 | 6224 | >120 | 4.0 (25t) | 20 (20t) | – | 2 x 3.0 | All | Laying/burying and repair of all types of cables (coaxial, optical fibre and power cables). Ploughs and ROV capability. |
| Maersk Defender | 1996 | 11980 | 96.0 | 8.70 | 12 | 17000 | 4 | 3158 | 6000 | 12 | 4.0 (25t) | 20 (20t) |  | 2 x 3.0 | All | Laying/burying and repair of all types of cables (coaxial, optical fibre and power cables). Ploughs and ROV capability. |
| Lodbrog | 1985 / 2002 | 12503 | 143.4 | 8.50 | 16.0 | 10000 | 6 | 2940 | 5040 | 84 | 2 x 4.0 (25t) | 2 x 6 (6t) | - | 2 x 3.0 | All | Laying/burying and repair of all types of cables (coaxial, optical fibre and power cables). ROV capability, SWL 8 ton. |
|  |  |  |  |  |  |  | FINLAND 1) Ship belonging to Sonera LTD |  |  |  |  |  |  |  |  |  |
| M/S Telepaatti | 1978 (modification) | 450 | 42.6 | 3.0 | 12 | – | 1 | – | 350 | – | 2 linear engines with 3 cater-pillar tracks on each | 3.0 |  | 300 |  | Laying of all types of telecom cables. Specially equipped for cable route survey and cable repair. Fully automatic autopilot and DP-system. |
|  |  |  |  |  |  |  | 2) Ship belonging to YIT Primatel |  |  |  |  |  |  |  |  |  |
| c/s Telepaatti | 1978 Modifi-cation 1999 | 450 | 42.6 | 3.0 | 10.5 | - | 1 | 250 | 260 | - | - | 2 linear engines with 3 cater-pillar tracks on each | 3.0 | - | 300 | Laying of all types of telecom cables and <150 mm power cables. Specially equipped for cable route survey and cable repair. Fully automatic autopilot and DP-system. |
|  |  |  |  |  |  |  | FRANCE 1) Ships belonging to France Telecom |  |  |  |  |  |  |  |  |  |
| Vercors | 1974 | 11 000 | 136 | 7.2 | 16.6 | 12 000 | 3 | 2425 | 4900 | 144 | 3.0 | 24 | 3.0 | Chute | All | Laying and repair of all types of telecom cables. Burying of cables with plough. |
| Leon Thevenin | 1983 | 6800 | 107 | 6.24 | 15.0 | 10 000 | 2 + 1 | 1420 | 2000 | 11 | 3.4 | 12 | 3.0 | Chute | All | Laying and repair of all types of telecom cables. Burying of cables using Scarab. |
| Raymond Croze | 1983 | 6800 | 107 | 6.24 | 15.0 | 10 000 | 2 + 1 | 1420 | 2000 | 11 | 3.4 | 12 | 3.0 | Chute | All | Laying and repair of all types of telecom cables. Burying of cables using Scorpio 2000. |
|  |  |  |  |  |  |  | 2) Ships belonging to Alda Marine |  |  |  |  |  |  |  |  |  |
| Ile de Sein Ile de Batz Ile de Brehat | 2002 | 18006 | 140.4 | 8.016 | 15.0 | 15000 | 2 + 2 | 3000 | 5500 | 202 | 4.0 | 21 | NA | 3.0 | All | Laying and repair of all types of telecom cables. Burying of cables with. 2/3m Rock plough. Sea state 7 A frame |
| Ile de Ré | 1983 rebuilt 2002 | 12687 | 143.4 | 7.23 | 16.0 | 11000 | 3 + 3 | 2900 | 4500 | 84 | 2 x 4.0 | NA | NA | 3.0 | All | Laying and repair of types of cable. No plough ROV to 2500m |
|  |  |  |  |  |  |  | ITALY 1) Ship belonging to Elettra TLC S.p.A |  |  |  |  |  |  |  |  |  |
| Teliri | 1996 | 6500 | 111.5 | 6.5 | 14.0 | 10 000 | 3 | 2000 | 2600 | 70 | N.2  3.5 | N.1  18wp | 3 | 4 | All | Laying and repair optical fibre systems |
| Gertament (ex John Cabot) | 1995 rebuilt 1998 | 5000 | 96.6 | 7.3 | 12.0 | 8000 | 3 | 600 | 1900 | 24 | N 1  3 | 1  18+ 1  6 | 3 | 3 | All | Laying, survey and repair optical fibre systems |
| Pertinacia | 2002 | 12650 | 129.9 | 8.4 | 14 | 10 000 | 2 | 4000 | 6400 | 2 x 100 | N 1 x 4 | 1 x 20 1 x 6 | – | 3 | All | Lay and repair optical fibre systems |
|  |  |  |  |  |  |  | 2) Ships belonging to Pirelli Cavi |  |  |  |  |  |  |  |  |  |
| Arabella | 1975 | 2620 | 76.66 | 5.18 | 11 | 2000 | 2 | 1100 | 2000 | – |  | – | – | 3 | All | Lay/repair. |
| G.Verne | 1984 | 16 900 | 128.5 | 8.5 | 10 | 8000 | 2 | 2600 | 8000 | 20 | 6.0 (50t) | 1 (Pads type 10t) | – | 6.0 | All | Stern only. |
|  |  |  |  |  |  |  | SPAIN Ships belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |  |  |  |  |
| Teneo | 1992 | 4000 | 81 | 5.7 | 14.5 | 4200 | 2 | 500 | 1000 | 20 | 2  3.5 | 1  9 | 2  3 | 1  3 | All | Lays and repairs of all types of telephone cables. |
| Atlantida | 1987 | 7853 | 114 | 6.5 | 15.7 | 6800 | 3 | 1500 | 2500 | 33 | 2  3.5 | 1  12 | 2  3 | 1  3 | All | Lays and repairs of all types of telephone cables. |
| Iberus | 1978 | 10 000 | 136.03 | 6.6 | 12.5 | 13 500 | 3 | 2580 | 4000 | 108 | 1  3 | 1  20 | – | 2  3 | All | Lays and repairs of all types of telephone cables. |
|  |  |  |  |  |  |  | JAPAN 1) Ships belonging to Kokusai Cable Ship (KCS) |  |  |  |  |  |  |  |  |  |
| KDD Ocean Link | 1992 | 11 700 | 133.2 | 7.0 | 15 | 10 000 | Main 3 Spare 4 | 2320 | 4500 | 100 | 3.6 | 21 | 3.2 | 4.0 | All | Laying by linear engine. Lays and repairs all types of submarine cables. |
| KDD Pacific Link | 1997 | – | 109.0 | 7.5 | 12 | – | Main 2 Spare 2 | 2720 | 4500 | – | 3.6 | 20 | – | 3.0 | All | Laying by linear engine. Lays and repairs all types of submarine cables. |
|  |  |  |  |  |  |  | 2) Ships belonging to NTT World Engineering Marine Corporation (NTT-WE Marine) |  |  |  |  |  |  |  |  |  |
| NTT Kuroshio Maru | 1974 | 5656 | 119.3 | 5.60 | 16.5 | 6883 | 3 | 1429 | 1900 | 95 | 3.8 | 8 (24 inch) | 3.0 | 3.0 | All | Lays and repairs all types of telephone cables. |
| Subaru | 1999 | 9557 | 123.3 | 7.0 | 13.2 | 8800 | Main 2 Spare 2 | 2770 | 4000 | 50 | 4.0 | 21 | – | 3.2 | All | Lays and repairs all types of telephone cables. |
|  |  |  |  |  |  |  | UNITED KINGDOM 1) Ship belonging to British Telecommunications plc |  |  |  |  |  |  |  |  |  |
| Sovereign | 1991 | 13 018 | 131 | 7.0 | 13.5 | 14 000 | 4 | 2800 | 6200 | 90 | 3.50 |  | 3.00 | 3.50 | All | Lays, repairs all types of coaxial and optical fibre cable. (Operated by C&W marine.) |
|  |  |  |  |  |  |  | 2) Ships belonging to Global Marine Systems Ltd |  |  |  |  |  |  |  |  |  |
| Alert | 1961 | 9477 | 130 | 7.1 | 14 | 10 000 | 3 | 1509 | 3100 | 48 | 2.98 |  | 2.98 | 2.98 | All | Laying by linear engine and sea-bed burial by plow. Lays/repairs all types of coaxial and optical fibre cables. |
| Cable Venture | 1962 | 16 983 | 153 | 8.97 | 12.5 | 10 000 | 4 + 1 (spare) | 5086 | 9000 | 400 | 2.80 |  | 3.00 | 3.39 | All | Laying by linear cable engine. Ploughs, lays and repairs armoured and lightweight cables. |
| Mercury | 1962 | 11 683 | 144 | 7.5 | 14.5 | 8000 | 3 | 2970 | 3500 | 144 | 3.05 |  | 3.50 | Chute 3.05 | All | Ditto (no plough). |
| Cable Enterprise | 1964 | 5759 | 113 | 5.84 | 13 | 8000 | 3 | 887 | 2150 | 30 | 2.8 |  | 3.00 | Chute 3.05 | All | Lays/repairs armoured cables. Repairs lightweight cables. (Note) |
| Monarch | 1975 | 4639 | 97 | 5.5 | 14 | 7000 | 4 | 417 | 850 | 12 | 3.00 |  | 3.00 | None | All | Lays/repairs armoured coaxial and optical fibre cables. Repairs lightweight coaxial and optical fibre cables. Detrenching/reburial by submersible jetting. |
|  |  |  |  |  |  |  | UNITED KINGDOM 2) Ships belonging to Global Marine Systems Ltd |  |  |  |  |  |  |  |  |  |
| Iris | 1976 | 4639 | 97 | 5.5 | 14 | 7000 | 4 | 417 | 850 | 12 | 3.00 |  | 3.00 | None | All | Lays/repairs armoured coaxial and optical fibre cables. Repairs lightweight coaxial and optical fibre cables. |
| MV Cable Installer | 1980 | 6065 | 89.42 | 5 | 12 | 42 days | 4 | 840 | 1600 | None | 3.0 | 4-track pair | – | 3.0 | – | Repeaterless installation vessel fully DP Cegelec 901 system. |
| Seaspread | 1980 | 10 887 | 116 | 6.8 | 13 | 65 days | 2 | 1010 | 1701 | – | 2  3 | – | – | 3 | All | Lays/repairs by aft drums. Burial by plough. Lays/repairs armoured and lightweight cables. |
| Pacific Guardian | 1984 | 7526 | 116 | 6.32 | 14.0 | 8000 | 3 | 1416 | 3470 | 96 | 3.5 |  | 3.00 | 3.00 | All | Laying by linear cable engine. Lays and repairs armoured and lightweight cables. |
| Sir Elic Sharp | 1988 | 7526 | 115 | 6.3 | 13.5 | 9600 | 3 | 1416 | 1700 | 96 | 2  3.5 | – | 3 | 3 | All | Laying by linear cable engine. Repairs and lays armoured and lightweight cables. Post lay/repair burial by integral ROV. |
| MV Cable Innovator | 1995 | – | 142 | 8.3 | 14.5 | 42 days | 4 | 4900 | 7500 | 180 | 4.0 | 21 pair (min) | – | 4.0 | – | Simplex D/P system. Lays/repairs cables. |
|  |  |  |  |  |  |  | MARSHALL ISLANDS Ship belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |  |  |  |  |
| CS Coastal Connector | 1997 Converted in 1996 | 6761 | 92.47 | 7.1 | 12.5 | 25 000 | 3 main 1 spare | 675 (main, total) 70 (spare) | 1600 | 30 | 2  3 | N/A | N/A | 2  3 | – | The CS Coastal Connector is stern laying design. She is capable of deploying the SCARAB II, SCARAB IV, and Pacific SCARAB I ROV's, as well as the Seabed Tractor |
| CS Tyco Provider | 1978, Converted in 1999 | 14 500 | 139.4 | 7.6 | 14.5 | 20 000 | 5 | 3349 | 6000 | 100+ | 2  4 | – | – | 2  3 | – | The CS Tyco Provider is a stern laying design. She is capable of deploying Sea Plow VIII. |
|  |  |  |  |  |  |  | NETHERLANDS ANTILLES Ship belonging to Tyco Submarine Systems Ltd. (charterer) |  |  |  |  |  |  |  |  |  |
| Dock Express 20 | 1983 | 21 731 | 169.52 | 8.79 | 12.5 | 20 500 | 3 main 2 spare | 4050 (main, total) 640 (spare, total) | 10 000 | 100+ | 1  3.0 | 1  3 module belt type | N/A | 2  3 |  | The Dock Express 20 is a stern laying design. She is capable of deploying the SCARAB II ROV, as well as the Seabed Tractor and Sea Plow VI. |
|  |  |  |  |  |  |  | UNITED STATES OF AMERICA Ships belonging to AT&T |  |  |  |  |  |  |  |  |  |
| CS Charles L. Brown | 1954, Reflag-ged in 1985 | 4298 | 99.94 | 5.6 | 13 | 7550 | 3 | 660 | 1186 | 30+ | 2  3 | N/A | 2  3 | N/A | – | The Charles L. Brown is primarily a repair ship. She is not fitted with any stern laying equipment. She is capable of deploying the SCARAB II ROV. |
| CS Global Link | 1990 | 16 375 | 145.7 | 8.08 | 15 | 10 000 | 3 main, 4 spare | 3258 (main, total) 164 (spare, total) | 6098 | 100+ | 2  3.7 | 1 Western Gear Tractor Type | 2  3 | 1 trough/ Chute type | – | The Global Link is capable of deploying the SCARAB II ROV's. |
| CS Global Mariner | 1993 | 15 638 | 151.5 | 7.8 | 13.8 | 10 000 | 2 main, 3 spare | 2172 (main, total) 447 (spare, total) | 4999 | 80+ | 2  3.7 | 1 Dowty 21 pair | 2  3 | 1 trough/ Chute type | – | The Global Mariner is capable of deploying the SCARAB II and SCARAB IV ROV's, as well as Sea Plow VII, Sea Plow VIII, and the Seabed Tractor. |
| CS Global Sentinel | 1991 | 16 375 | 145.7 | 8.08 | 15 | 10 000 | 3 main, 4 spare | 3258 (main, total) 164 (spare, total) | 6098 | 100+ | 2  3.7 | 1 Dowty 21 pair | 2  3 | 1 trough/ Chute type | – | The Global Sentinel is capable of deploying the SCARAB II and SCARAB IV, and Pacific SCARAB I ROV's, as well as Sea Plow VII and Sea Plow VIII. |
| NOTE – Only relatively short cables are laid and only shore-end. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Section 2 – Submersible equipments

| Type of submersible |  | Weight (tons) |  | Overall length (m) | Width (m) |  |  | Height (m) |  | Trenching system |  |  |  | Trenching |  |  | Propulsion |  | Max.operating depth (m) |  |  | Capability |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | FRANCE Submersibles belonging to France Telecom (FTRSI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ELISE2 Submersible Plough system |  | 17 |  | 7.60 | 2.90 |  |  | 2.95 |  | Ploughshare |  |  |  | Immediate burial up to 1.1 m |  |  | Towed by support ship |  | 1500 |  |  | Lay and bury all types of cables. |  |  |
| ELISE3 Submersible Plough system |  | 17 |  | 7.60 | 2.90 |  |  | 2.95 |  | Ploughshare |  |  |  | Immediate burial up to 1.1 m |  |  | Towed by support ship |  | 1500 |  |  | Lay and bury all types of cables. |  |  |
| Self-advancing buried system CASTOR2 |  | 12 |  | 7.0 | 2.40 |  |  | 3.00 |  | Trenching wheel or chain |  |  |  | Burial of existing cables down to 2 m |  |  | Tracked vehicle |  | 1000 |  |  | Burial of cables and pipes. Visual inspection. |  |  |
| Scarab 3 |  | 9 |  | 4.0 | 3.50 |  |  | 2.10 |  | High pressure water jets |  |  |  | Up to 60 cm depth |  |  | Thrusters (inspection) Back drive (burial) |  | 1000 (burial) 2000 (inspect) |  |  | Visual inspection, post lay burial, cable location, cable manipulation, cable cutting. |  |  |
| Remote control submersible Scorpio 2000 |  | 3.4 |  | 2.9 | 1.5 |  |  | 2.11 |  | High pressure water jets |  |  |  | Up to 60 cm depth |  |  | Thrusters |  | 1000 |  |  | Visual inspection, post lay burial, cable location/ manipulation/cutting. |  |  |
|  |  |  |  |  |  |  |  | ITALY 1) Submersibles belonging to Elettra TLC SpA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plough Taurus 1 |  | 14 |  | 9 | 4.6 |  |  | 4.5 |  | Plough share |  |  |  | Up to 1 mt |  |  | Plough Taurus 1 |  | 14 |  |  | 9 |  |  |
| Plough Taurus 2 |  | 16 |  | 9.5 | 4.5 |  |  | 5.1 |  | Plough share |  |  |  | Up to 1.5 mt |  |  | Plough Taurus 2 |  | 16 |  |  | 9.5 |  |  |
| ROV – Phoenix 2 |  | 6.8 |  | 4.8 | 2 |  |  | 2.6 |  | High/low pressure jetting |  |  |  | Up to 1.2 mt |  |  | ROV – Phoenix 2 |  | 6.8 |  |  | 4.8 |  |  |
|  |  |  |  |  |  |  |  | ITALY 2) Submersibles belonging to Pirelli Cavi |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plough 1 |  | 10 |  | 7 | 2.7 |  |  | 3 |  | Plough share |  |  |  | Up to 1 m |  |  | Plough 1 |  | 10 |  |  | 7 |  |  |
| Plough 2 |  | 9 |  | 8.5 | 3.8 |  |  | 3.5 |  | Plough share |  |  |  | Up to 1.2 m |  |  | Plough 2 |  | 9 |  |  | 8.5 |  |  |
|  |  |  |  |  |  |  |  | UNITED KINGDOM Submersibles belonging to Global Marine Systems Ltd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Submersible trencher |  | 17.0 |  | 6.6 | 4 |  |  | 3.4 |  | Fluidization and cutting jets and dredge pump |  |  |  | Up to 1 m depth with cutting and fluidization jets |  |  | Three vertical and four horizontal thrusters, track drive differential steering |  | 274 |  |  | Trench in existing cable and pipe. |  |  |
| Submersible Plough system |  | 9.75 |  | 6.1 | 2.6 |  |  | 2.6 |  | Ploughshare proceeded by disc |  |  |  | Immediate burial of cable on ploughing |  |  | Towed by support ship |  | 900 |  |  | Lay and bury cable, umbilical and pipe in one action giving full cable protection. |  |  |
| Remote control submersible 2 off Cirus A&B |  | 3.2 |  | 3.5 | 2.1 |  |  | 2.3 |  | Water jets |  |  |  | Trenching capability 0.3 m |  |  | Thrusters (7) |  | 1000 |  |  | Visual inspection, cable location/inspection/deburial, manipulation. Tools include cable cutter, cable gripper and 2 manipulators with line cutters. |  |  |
|  |  |  |  |  |  |  |  |  | UNITED KINGDOM Submersibles belonging to Global Marine Systems Ltd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plough 2 off A&B | 14.5 |  |  | 9 |  | 4.1 |  |  | 4 |  |  | Passive blade | Trenching capability 1.0 m |  |  | Towed |  |  |  | 1000 |  |  | Steerable, repeater burial. |  |
| Remote control submersible ROV 128 | 7.5 |  |  | 2.9 |  | 1.8 |  |  | 2.0 |  |  | Jetting tool | Trenching capability 0.6 m |  |  | Tracked burial Thrusters survey |  |  |  | 1000 (burial) 2000 (survey) |  |  | Tools include cable cutter, cable gripper and 2 manipulators with line cutters. |  |
| Underwater vehicle- MARLIN | 7.8 |  |  | 4.191 |  | 2.438 |  |  | 3.175 |  |  | Burial skid | To 1.0 m (Optimized for 0-30 kPa soil) |  |  | Hydraulic driven thrusters |  |  |  | 2500 |  |  | Burial, deburial, inspection. Maintenance and repair. Tools include cable cutter, cable gripper. |  |
| Scarab I – Umbilically tethered ROV | 3.2 |  |  | 2.74 |  | 1.82 |  |  | 1.52 |  |  | Jetting tool | Up to 0.6 m |  |  | Thrusters: 2 vertical 4 vectored |  |  |  | 2000 |  |  | Cable detection and inspection. Visual survey. Cable manipulation and cutting Debris elimination. Cable and repeater burial/ deburial. |  |
| Subtrack - ROV | 10.0 |  |  | 8.0 (Max) |  | 3.7 |  |  | 3.8 |  |  | Jetting tool | Burial to 1.0 m |  |  | Electro hydraulic track drives |  |  |  | 1000 |  |  | Cable burial and deburial. Inspection. Maintenance and repair. |  |
| EUREKA: Deepwater burial + trenching system | 17 (Max) |  |  | 5.5 |  | 4.2 |  |  | 3.85 |  |  | Jetting tool Rock wheel cutter Mechanical chain excavator | 1 m 1.2 m 2.2 m |  |  | Electro hydraulic track drives |  |  |  | 1500 |  |  | Capable of burying cable, small flexible flowlines and also rigid pipes. Can also debury cable and restore. Visual and electronic inspections. |  |
| Plough 5 | 14.0 |  |  | 9.0 |  | 4.6 |  |  | 3.7 |  |  | Passive blade | Variable from 0-1100 mm (600-900 mm in all conditions) |  |  | Towed |  |  |  | 1000 |  |  | Simultaneously lay and bury cables and umbilicals at varying depths. |  |
| Plough 6 and 7 | 14.0 |  |  | 9.0 |  | 4.6 |  |  | 3.7 |  |  | Passive blade | Max burial depth: 1100 mm |  |  | Towed |  |  |  | 1000 |  |  | Simultaneously lay and bury cables and umbilicals at varying depths. |  |
| Cable Plough 1000 mm | 14.4 |  |  | 9.75 |  | 4.1 |  |  | 3.9 |  |  | Passive blade | 1000 mm (Good conditions: 1100 mm; Repeaters/Joints: 500 mm) |  |  | Towed |  |  |  | 1000 |  |  | Simultaneously lay and bury cables and umbilicals at varying depths. |  |
|  |  |  |  |  |  |  |  |  | DENMARK Submersibles belonging to Telecom Denmark |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plough D | 13.5 |  |  | 9.0 |  | 4.6 |  |  | 3.7 |  |  | Plough share | Variable from 0-1100 mm (600-900 mm in all conditions) |  |  | Towed by host vessel |  |  |  | 1500 |  |  | Lay and bury telecom cables, power cables and umbilicals. Cables: Up to 120 mm (bary). Joints and repeaters: Up to 400 mm (pass). |  |
| Plough 7 | 13.5 |  |  | 9.0 |  | 4.6 |  |  | 3.7 |  |  | Plough share | Variable from 0-1100 mm (600-900 mm in all conditions) |  |  | Towed by surface vessel |  |  |  | 1000 |  |  | Lay and bury fibre optic cables, power cables and umbilicals. |  |
| Subtrack- Subsea tractor | 10.0 |  |  | 8.0 (Max) |  | 3.7 |  |  | 3.8 |  |  | Jetting tool | Burial to 1.0 m |  |  | Electro hydraulic track drives |  |  |  | 1000 |  |  | Cable burial and deburial. Inspection. Maintenance and repair. |  |
| Super Phantom S4-ROV | 0.09 |  |  | 1.5 |  | 0.75 |  |  | 0.6 |  |  | – | – |  |  | Thrusters 4 prop fwd/aft 2 prop vertical 2 prop transvers |  |  |  | 300 |  |  | Inspect cables and other underwater objects. Can also be used to inspect seabed conditions. |  |
|  |  |  |  |  |  |  |  |  | JAPAN 1) Submersibles belonging to KCS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MARCAS-II-ROV | Jet tool mode: 8.0 Track base mode: 7.5 |  |  | Jet tool mode: 2.9 Track base mode: 5.3 |  | Jet tool mode: 2.3 Track base mode: 4.0 |  |  | Jet tool mode: 3.2 Track base mode: 3.8 |  |  | Water jet tool | – |  |  | 4 horizontal, 2 vertical and 2 balance thrusters |  |  |  | Jet tool mode: 2500 Track base mode: 2000 |  |  | Post-lay burial, maintenance of cable. Can survey seabed. |  |
| MARCAS-SBT-ROV | 15 (minimum) 23 (maximum) |  |  | Jet tool mode: 9.5 Chain cutter mode: 13.0 Wheel cutter mode: 12.0 |  | Jet tool mode: 5.5 Chain cutter mode: 5.5 Wheel cutter mode: 5.5 |  |  | Jet tool mode: 4.4 Chain cutter mode: 4.4 Wheel cutter mode: 4.4 |  |  | 2.1 m Rear jet tool and 1 m Forward jet tool 1.2 m Wheel cutter 3 m Chain cutter |  |  |  | One single hydraulic thruster |  |  |  | 1500 |  |  | Lay and burial, post-lay burial, maintenance of cable, and survey of seabed. |  |
|  |  |  |  |  |  |  |  |  | JAPAN 2) Submersibles belonging to NTT-WE Marine |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plough-type MARK-5 submarine cable burying system | 19.0 |  |  | 9.1 |  | 4.0 |  |  | 4.0 |  |  | – | Up to 1.5 m depth immediate burial of cable on ploughing |  |  | Towed by support ship |  |  |  | 600 |  |  | Simultaneous or post-lay burial of cable. |  |
| Submarine cable repair burial and inspection system | 6.2 |  |  | 3.8 |  | 2.1 |  |  | 2.3 |  |  | Fluidization jets | Fluidization jets |  |  | Vertical and horizontal thrusters |  |  |  | 1000 |  |  | Post-lay burial maintenance of cable and survey of seabed. |  |
| Plough-type MARK-6 submarine cable burying system | 18 |  |  | 9.3 |  | 5.1 |  |  | 4.4 |  |  | – | Up to 2.0 m depth immediate burial of cable on ploughing |  |  | Towed by support ship |  |  |  | 1500 |  |  | Simultaneous or post-lay burial of cable. |  |
| Submarine cable repair burial and inspection system | 8.0 |  |  | 3.2 |  | 2.1 |  |  | 2.8 |  |  | Fluidization jets | Trenching capability 1.0 m |  |  | Vertical and horizontal thrusters |  |  |  | 2500 |  |  | Cable detection & inspection visual survey. Cable manipulation & cutting. Debris stockage. Cable & repeater burial/deburial. |  |
| SEA MOLE Tractor type Submarine cable Burial system | 23 |  |  | 8 |  | 6 |  |  | 3.5 |  |  | 3 types attachment (jetting tool, wheel cutter & chain cutter) | Trenching capability 2.0 m (max.) |  |  | – |  |  |  | 1000 |  |  |  |  |
|  |  |  |  |  |  |  |  |  | SPAIN 1) Submersible belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ARADO I | 12 |  |  | 9 |  | 4.6 |  |  | 4 |  |  | Plow-share | 1100 mm |  |  | Towed |  |  |  | 1500 |  |  | Bury cable from 19 to 40 mm. Bury repeaters until 380 mm. Velocity 1 m/s. |  |
| ARADO II | 12 |  |  | 9 |  | 4.6 |  |  | 4 |  |  | Plow-share | 1500 mm |  |  | Towed |  |  |  | 1500 |  |  | Bury cable from 17 to 150 mm. Bury repeaters until 380 mm. Velocity 1 m/s. |  |
| ARDI | 3.6 |  |  | 6.1 |  | 3 |  |  | 2.6 |  |  | Plow-share | 900 mm |  |  | Towed |  |  |  | 1500 |  |  | System to evaluate if land is arable. |  |
| NEREUS | 8.5 |  |  | 3.2 |  | 3.4 |  |  | 2.9 |  |  |  | 1 m |  |  | 150 KW |  |  |  | 2000 |  |  | Repair, inspect and bury all types of telephone cable 2  manipulating 7 functions. Velocity 3 knots. |  |
|  |  |  |  |  |  |  |  |  | SPAIN 1) Submersible belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SCARAB III | 8.5 |  |  | 4 |  | 3.9 |  |  | 2.1 |  |  |  | 0.6 |  |  | 180 KW |  |  |  | 2000 |  |  | Repair, inspect and bury all types of telephone cable 2 x manipulating 7 functions. Velocity 3.1 knots. |  |
|  |  |  |  |  |  |  |  |  | 2) Submersible belonging to Consorcio ESCARAB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ROV | 8.5 |  |  | 4.0 |  | 3.9 |  |  | 2.1 |  |  |  | Up to 1 m |  |  |  |  |  |  | 2000 1000 |  |  |  |  |
|  |  |  |  |  |  |  |  |  | UNITED STATES OF AMERICA Submersibles belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PACIFIC SCARAB I | 5.48 |  |  | 4.27 |  | 1.83 |  |  | 3.05 |  |  | Jetting Modules | 560 meters/hour. Soil hardness to 100 kPa. |  |  | 150HP Electro Hydraulically Powered using 8 thrusters |  |  |  | 2500 |  |  | PACIFIC SCARAB I Submersible Craft Assisting Repair and Burial is a tethered, swimming ROV capable of operating at depths of 2500 meters. It can locate, inspect, retrieve, and bury submarine cables. |  |
| SCARAB II | 3.45 |  |  | 3.7 |  | 2.1 |  |  | 2.3 |  |  | 35 HP Cable Jetter | 255m/hr depending on soil conditions. Soil hardness to 60 kPa. |  |  | Horizontal: 4×5 HP Electric Thrusters Vertical: 2×5 HP Electric Thrusters Aft Lateral: 1×10 HP Hydraulic Thruster Bow: 2×2.5 HP Hydraulic Thrusters |  |  |  | 1850 |  |  | SCARAB II Submersible Craft Assisting Repair and Burial is a tethered, swimming ROV capable of operating at depths of 1850 meters. It can locate, inspect, retrieve, and bury submarine cables. |  |
|  |  |  |  |  |  |  |  |  | UNITED STATES OF AMERICA Submersibles belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SCARAB IV |  | 4.6 | 3.4 |  |  |  | 2.02 |  | 1.96 |  | Jetting Modules |  |  | 530 meters/hour Soil hardness to 100 kPa | 150 HP Electro-Hydraulically Powered using 8 thrusters |  |  | 1850 |  |  | SCARAB IV Submersible Craft Assisting Repair and Burial is a tethered, swimming ROV capable of operating at depths of 1850 meters. It can locate, inspect, retrieve, and bury submarine cables. SCARAB IV is part of the ACMA SCARAB Agreement. |  |  |  |
| Sea Plow VI |  | 25.5 | 10.5 |  |  |  | 6.0 |  | 4.3 |  | Towed Plow System |  |  | 1.2 meter burial | Towed by ship |  |  | 1000 |  |  | Sea Plow VI is a towed burial tool employing state-of-the-art burial features. It can achieve 1.2 meter burial depth in up to 1000 meter water depth. |  |  |  |
| Sea Plow VII |  | 14.0 | 10.5 |  |  |  | 6.0 |  | 4.3 |  | Towed Plow System |  |  | 1.0 meter burial | Towed by ship. 1 Thruster for Launches and Recoveries |  |  | 1400 |  |  | Sea Plow VII is a towed burial tool employing state-of-the-art burial features. It can achieve 1.0 meter burial depth in up to 1400 meter water depth. |  |  |  |
| Sea Plow VIII |  | 19.3 | 9.2 |  |  |  | 5.5 |  | 3.6 |  | Towed Plow System with water jet assist |  |  | 1.5 meter burial | Towed by ship |  |  | 1500 |  |  | Sea Plow VIII is a towed burial tool employing state of the art burial features. It can achieve 1.5 meter burial depth in up to 1500 meter water depth. |  |  |  |

ANNEX II (to TSB Circular 126)

## Questionnaire on new cable ships and submersible equipment

<Cable ships>

| Country |  |  |  |  |
| --- | --- | --- | --- | --- |
| Organization |  |  |  |  |
| Name of ship |  |  |  |  |
| Year of construction |  |  |  |  |
| Displacement |  |  |  | (tons) |
| Overall length |  |  |  | (m) |
| Draft |  |  |  | (m) |
| Normal speed |  |  |  | (knots) |
| Range (autonomy) |  |  |  | (nautical miles) |
| Number of tanks |  |  |  |  |
| Cable capability | Cable | Cubic metres |  | (m3) |
|  |  | Weight |  | (tons) |
|  | Repeaters |  |  |  |
| Cable gear | Cable engine | (Drum) |  | (number) x (diameter) |
|  |  | (Linear) |  | (pairs of wheels) |
|  | Unwinding pulley | Bow sheave |  | (diameter, m) |
|  |  | Stern sheave |  | (diameter, m) |
| Maximum operating depth |  |  |  | (m) |
| Capability (general features and remarks) |  |  |  |  |

| Contact Affiliation Tel Fax E-mail |  |
| --- | --- |

<Submersible equipment for laying, burial, inspection and so on>

| Country |  |  |
| --- | --- | --- |
| Organization |  |  |
| Type of submersible |  |  |
| Weight |  | (tons) |
| Overall length |  | (m) |
| Width |  | (m) |
| Height |  | (m) |
| Trenching system |  |  |
| Trenching capability |  |  |
| Propulsion |  |  |
| Maximum operating depth |  | (m) |
| Capability (general features and remarks) |  |  |

| Contact Affiliation Tel Fax E-mail |  |
| --- | --- |

____________