INTERNATIONAL TELECOMMUNICATION UNION

Radiocommunication Bureau (Direct Fax N°. +41 22 730 57 85)

| Circular Letter CR/202 | 16 October 2003 |
| --- | --- |

To Administrations of Member States of ITU

Subject:	Resolution 609 (WRC-03)

To the Director-General

Dear Sir/Madam,

1	The World Radiocommunication Conference, Geneva 2003 (WRC-03), adopted Resolution 609 (Annex 1), whereby Consultation Meetings between administrations operating or planning to operate systems in the aeronautical radionavigation service (ARNS) and systems in the radionavigation-satellite service (RNSS) in the 1 164-1 215 MHz frequency band, should be held on a regular basis to achieve the level of protection for ARNS systems foreseen in resolves 1 of that Resolution.

2	In order to ensure the successful implementation of Resolution 609 (WRC-03), the Bureau is providing support in organising the first Consultation Meeting as foreseen in the Resolution.

- Administrations are invited to note that in the framework of Resolution 609, the Radiocommunication Bureau is only instructed to participate in Consultation Meetings.  The overall responsibility of such a Consultation Meeting however lies with the participating administrations in so much as such Meeting is not to be construed as, for example, a Study Group Meeting or a RRB Meeting organized by the Bureau.

4	Taking account of the proposed schedules for meetings of WP8D, 8B and SG8 and that many of those interested in participating will already be in Geneva, the first Consultation Meeting as foreseen in Resolution 609 (WRC-03) is scheduled for 8 and 9 December 2003 in the ITU Headquarters in Geneva.  The proceedings of the meeting and associated documentation will be in English only. A draft Agenda is contained in Annex 2.  Member States and Sector Members are invited to be represented at this meeting.

5	The Consultation Meeting should designate its own convener. To assist in this matter, administrations should hold informal exchange of ideas before the meeting in order to come up with a proposal for a convener.

./.

6	A web page, Res.609 Forum, has been established for participants to the Consultation Meeting on which will be posted any relevant information and documents for the meetings. This page can be found at http://www.itu.int/ITU-R/space/res609/. It will include a free-read only guest access for all participants to the Consultation Meetings. In order to facilitate message distribution and electronic working, registered users (TIES account required) would also have the possibility to post and reply to messages posted on this Forum. Registration for the Res.609 Forum may be arranged on-line at http://www.itu.int/ITU-R/space/res609/.

7	We would remind you that citizens of some countries are required to obtain a visa in order to enter and spend any time in Switzerland.  The visa must be requested and obtained from the office (embassy or consulate) representing Switzerland in your country or, if there is no such office in your country, from the one that is closest to the country of departure. If problems are encountered, the Union can, at the official request of the administration or company you represent, approach the competent Swiss authorities in order to facilitate delivery of the visa.  Any such request must specify the name and functions of the individual(s) for whom the visa(s) is/are requested and be accompanied by a copy of the approved registration form for the ITU conference or meeting in question.

8	The intended participation of your representative(s) should be advised no later than 14 November 2003, by means of the annexed form (Annex 3) (to be photocopied as required).

9	Kindly be informed that on behalf of the Bureau, Mr. Yvon HENRI, Head of Space Services Department (telephone: +41 22 730 5536; e-mail: yvon.henri@itu.int) will be the contact person in this matter.

Yours faithfully,

Valery Timofeev
	Director, Radiocommunication Bureau

Distribution:

- Administrations of Member States of ITU

- Members of the Radio Regulations Board

- Chairmen and Vice-Chairmen of Radiocommunication Study Groups and the

Special Committee on Regulatory/Procedural Matters

ANNEX 1

RESOLUTION  609  (WRC-03)

Protection of aeronautical radionavigation service systems from the equivalent power flux-density produced by radionavigation-satellite service networks 
and systems in the 1 164-1 215 MHz frequency band

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the band 960-1 215 MHz is allocated on a primary basis to the aeronautical radionavigation service (ARNS) in all Regions;

b)	that the band 1 164-1 215 MHz is also allocated on a primary basis to the radionavigation-satellite service (RNSS), subject to the condition in No. 5.328A that operation of RNSS systems shall be in accordance with this Resolution;

c)	that WRC-2000 provided for implementation of a provisional aggregate power flux-density (pfd) limit during the period between WRC-2000 and WRC-03, and requested ITU-R studies on the need for an aggregate pfd limit, and revision, if necessary, of the provisional pfd limit given in No. 5.328A;

d)	that this Conference has determined that protection of the ARNS from harmful interference can be achieved if the value of the equivalent power flux-density (epfd) produced by all the space stations of all RNSS (space-to-Earth) systems in the 1 164-1 215 MHz band does not exceed the level of –121.5 dB(W/m2) in any 1 MHz band;

e)	that only a limited number of RNSS systems are expected to be deployed in the 1 164-1 215 MHz band, and only a few of these systems at most would have overlapping frequencies;

f)	that ARNS systems can be protected without placing undue constraints on the development and operation of RNSS systems in this band;

g)	that to achieve the objectives in considering f), administrations operating or planning to operate RNSS systems will need to agree cooperatively through consultation meetings to equitably share the aggregate epfd in a manner to achieve the level of protection for ARNS systems that is stated in considering d);

h)	that it may be appropriate for representatives of administrations operating or planning to operate ARNS systems to be involved in determinations made pursuant to considering g);

i)	that this Conference has decided to apply the coordination provisions of Nos. 9.12, 9.12A and 9.13 to RNSS systems and networks for which complete coordination or notification information, as appropriate, is received by the Bureau after 1 January 2005,

noting

a)	that WRC-2000 invited ITU-R to conduct the appropriate technical, operational and regulatory studies on the overall compatibility between the radionavigation-satellite service and the aeronautical radionavigation service in the band 960-1 215 MHz;

b)	that WRC-2000 resolved to recommend that WRC-03 review the results of the studies,

recognizing

that under No. 7.5 of the Radio Regulations, interested administrations have the ability, at any time, to request the assistance of the Bureau with respect to Articles 9 and 11 and associated procedures,

resolves

1	that in order to protect ARNS systems, administrations shall ensure, pursuant to this Resolution, that the equivalent pfd level produced by all space stations of all RNSS systems does not exceed the level –121.5 dB(W/m2) in any 1 MHz band;

2	that administrations operating or planning to operate in the 1 164-1 215 MHz frequency band RNSS systems or networks shall, in collaboration, take all necessary steps, including, if necessary, by means of appropriate modifications to their systems or networks, to ensure that the aggregate interference into ARNS systems caused by such RNSS systems or networks operating co-frequency in these frequency bands is shared equitably among the systems identified in resolves 3 and does not exceed the level of the aggregate protection criterion given in resolves 1 above;

3	that administrations, in carrying out their obligations under resolves 1 and 2 above, shall take into account only those RNSS systems with frequency assignments in the band 1 164-1 215 MHz that have met the criteria listed in the Annex to this Resolution through appropriate information provided to the consultation meetings referred to in considering g);

4	that administrations, in developing agreements to carry out their obligations under resolves 1 and 2 above, shall establish mechanisms to ensure that all potential RNSS system operators and administrations are given full visibility of the process;

5	that in order to allow multiple RNSS systems to operate in the frequency band 1 164-1 215 MHz, no single RNSS system shall be permitted to use up the entire interference allowance specified in resolves 1 above in any 1 MHz of the 1 164-1 215 MHz band (see Recommendation 608);

6	that to achieve the objectives in resolves 1 and 2 above, administrations operating or planning to operate RNSS systems will need to agree cooperatively through consultation meetings to achieve the level of protection for ARNS systems that is stated in resolves 1;

7	that administrations participating in this process of epfd calculation should hold consultation meetings on a regular basis (e.g. yearly);

8	the administrations participating in the consultation meeting shall designate one administration that shall communicate to the Bureau the results of any aggregate sharing determinations made in application of resolves 2 above, without regard to whether such determinations result in any modifications to the published characteristics of their respective systems or networks (see Recommendation 608);

9	that administrations operating or planning to operate ARNS systems in the 1 164-1 215 MHz band should participate, as appropriate, in discussions and determinations relating to the resolves above;

10	that the methodology and the reference worst-case ARNS system antenna contained in Recommendation ITU-R M.1642 shall be used by administrations for calculating the aggregate epfd produced by all the space stations within all RNSS systems in the band 1 164-1 215 MHz,

instructs the Radiocommunication Bureau

1	to participate in consultation meetings mentioned under resolves 6 and to observe carefully results of the epfd calculation mentioned in resolves 1;

2	to determine whether the pfd level in recommends 1 of  Recommendation 608 is exceeded by any subject space station, and to report the findings of this determination to the participants in the consultation meeting;

3	to publish in the International Frequency Information Circular, the information referred to in resolves 8 and instructs the Radiocommunication Bureau 2,

invites the Radiocommunication Bureau

to examine the possibility, if needed, of developing software that can be used to calculate the equivalent pfd level mentioned under resolves 1,

invites administrations

1	to deal with RNSS intersystem matters, as required, as early as possible;

2	to provide the Bureau and all participants of the consultation meeting with access to appropriate software used to calculate the equivalent pfd level mentioned under resolves 1.

ANNEX  TO  RESOLUTION  609  (WRC-03)

Criteria for application of Resolution 609

1	Submission of appropriate Advance Publication information.

2	Entry into satellite manufacturing or procurement agreement, and entry into satellite launch agreement.

The RNSS system or network operator should possess:

i)	clear evidence of a binding agreement for the manufacture or procurement of its satellites; and

ii)	clear evidence of a binding agreement to launch its satellites.

The manufacturing or procurement agreement should identify the contract milestones leading to the completion of manufacture or procurement of satellites required for the service provision, and the launch agreement should identify the launch date, launch site and launch service provider. The notifying administration is responsible for authenticating the evidence of agreement.

The information required under this criterion may be submitted in the form of a written commitment by the responsible administration.

3	As an alternative to satellite manufacturing or procurement and launch agreements, clear evidence of guaranteed funding arrangements for the implementation of the project would be accepted. The notifying administration is responsible for authenticating the evidence of these arrangements and for providing such evidence to other interested administrations in furtherance of its obligations under this Resolution.

ANNEX 2

Draft Agenda

RNSS/ARNS CONSULTATION MEETING

Geneva 8-9 December 2003

- Opening of the meeting

- Confirmation of Convener

- Adoption of Agenda

- Terms of Reference

- RNSS systems to be considered

6. 	Aggregate interference calculation method

7.	Approval of results

8.	Communication to the BR

9.	Confirmation of next meeting

10.	Any other business

11.	Closure of the meeting

ANNEX 3

|  |  |  | Réunion de Consultation (Résolution 609 (CMR-03)) Genève, 8-9 décembre 2003 Consultation Meeting (Resolution 609 (WRC-03)) Geneva, 8-9 December 2003 Reunión de Consulta (Resolución 609 (CMR-03)) Ginebra, 8-9 de diciembre de 2003 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Formulaire d'inscription - Registration form - Formulario de inscripción |  |  |  |  |  |
|  |  | (À retourner au Bureau des Radiocommunications (Fax +41 22 730 66 00 / e-mail : linda.kocher@itu.int) avant le) (To be returned to the Radiocommunication Bureau (Fax +41 22 730 66 00 / e-mail : linda.kocher@itu.int) by) (Devuélvase a la Oficina de Radiocomunicaciones (Fax +41 22 730 66 00 / e-mail : linda.kocher@itu.int) antes del) 14.11.2003 |  |  |  |  |

| Répresentation / Representation / Representación: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ou/or/u |  |  |  |  |  |  |  |
|  | Etat Membre / Member State / Estado Miembro |  |  |  |  | Organisation/Organization/Organización |  |  |  |  |
| M/Mr./Sr. Mme/Mrs./Sra. Mlle/Miss/Srta. Ms |  |  |  |  |  |  |  |  |  |  |
| Nom/Family name/Apellido: Prénom/First name/Nombre: Accompagné de (Membre(s) de famille) - à spécifier, s.v.p. Accompanied by (Family member(s)) - please specify Acompañado por (Miembro(s) de la Familia) - por favor, especificar Titre officiel dans l'Administration ou l'Organisation: Official title in Administration or Organization: Título oficial en su Administración u Organización: Nom et adresse complète de l'Administration ou Organisation: Name and complete address of Administration or Organization: _ Nombre y dirección completa de la Administración u Organización: _______________________________________________________________________________________________________________ ___________________________________________________________________________________________________ |  |  |  |  |  |  |  |  |  |  |
|  | Tel: |  |  |  | Fax: |  |  |  |  |  |
|  | E-mail: |  |  |  |  |  |  |  |  |  |
| Adresse personnelle (facultatif) / Home address (optional) / Dirección personal (facultativo) ________________________________________________________________Tel: ______________________________ |  |  |  |  |  |  |  |  |  |  |
| Hôtel/Adresse à Genève (voir Note) / Hotel/Address in Geneva (see Note) / Hotel/Dirección en Ginebra (véase Nota) __________________________________ Tel: ______________________________ Note: La réservation doit se faire DIRECTEMENT avec l’Hôtel et non par le biais de l’UIT. Note: Reservations must be made DIRECTLY with the hotel and not through the ITU. Nota: Las reservas deben hacerse DIRECTAMENTE con el hotel y no a través de la UIT. |  |  |  |  |  |  |  |  |  |  |
| Date/Fecha: ___________________________ |  | Signature/Firma: ________________________________________________ |  |  |  |  |  |  |  |  |