| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-DOC-0197 |  |
|  |  |  | English only |  |
| WG(s): 6 |  | 7th FG IPTV meeting: Qawra, St Paul’s Bay, Malta, 11-18 December 2007 |  |  |
| OUTPUT DOCUMENT |  |  |  |  |
| Source: | Editor |  |  |  |
| Title: | IPTV Metadata |  |  |  |

Summary

The present document gives the overview of the metadata for IPTV services and describes its elements and delivery protocols, identifying relevant standards.

The IPTV metadata, the information on services and contents processed by the service and content delivery infrastructure, provides descriptive and structural framework for managing IPTV services.

Aspects of transport, representation, content provisioning, and security of metadata are covered.

Keywords

IPTV metadata

Current status

This document is complete.

Dependency or relationship to other FG IPTV documents:

- IPTV service requirements document

- Service Security and Content Protection Aspects for IPTV

- IPTV protocols

CONTENTS

1 	Scope	3

2 	References	3

3 	Definitions	4

3.1 	Terms defined elsewhere	4

3.2 	Terms defined in this Recommendation	5

4 	Abbreviations and acronyms	5

5	Conventions	5

6 	IPTV Metadata service overview	5

7 	Metadata service discovery	6

7.1	Service provider metadata	7

8 	Metadata delivery method	7

8.1 	Updating method	7

8.1.1 	Metadata fragments	8

8.2 	Metadata transport container	8

8.3 	Metadata change notification	9

9 	Metadata representation	9

10 	Metadata elements	9

10.1 	Service and content metadata	9

10.2 	User metadata	16

11	Metadata for content provisioning	18

12	Metadata aggregation management	18

13	User metadata security	18

14	Classification scheme	19

15	Identity management	19

16	Metadata API	19

17	Metadata framework	19

Bibliography	20

IPTV METADATA

1 	Scope

The present document gives the overview of the metadata for IPTV services, and describes its elements and delivery protocols, identifying relevant standards.

The IPTV metadata provides a descriptive and structural framework for managing IPTV services. The IPTV metadata is the information on services and contents processed by service and content delivery infrastructure.

2 	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this document. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this document are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this document does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T F.750]	ITU-T Recommendation F.750 (2004), Metadata Framework Recommendation

[ISO/IEC 15706]	ISO 15706 (2002), Information and documentation — International Standard Audiovisual Number (ISAN)

[ISO/IEC 15938-5]	ISO/IEC 15938-5(2003), Information technology -- Multimedia content description interface -- Part 5: Multimedia description schemes

[ISO/IEC 21000-7]	ISO/IEC 21000-7(2003), Information technology -- Multimedia framework (MPEG-21) — Part 7: Digital Item Adaptation

[ETSI TS 102 034]	ETSI TS 102 034 V1.2.1 (2006), Digital Video Broadcasting: Transport of MPEG-2 Based DVB Services over IP Based Networks

[ETSI TS 102 323]	ETSI TS 102 323 V1.2.1 (2006), Digital Video Broadcasting (DVB); Carriage and signalling of TV-Anytime information in DVB transport streams

[ETSI TS 102 471]	ETSI TS 102 471 V1.2.1 (2006), Digital Video Broadcasting (DVB); IP Datacast over DVB-H: Electronic Service Guide (ESG)

[ETSI TS 102 472]	ETSI TS 102 472 V1.2.1 (2006), Digital Video Broadcasting (DVB); IP Datacast over DVB-H: Content Delivery Protocols

[ETSI TS 102 539]	ETSI TS 102 539 V1.1.1 (2006), Digital Video Broadcasting: Carriage of Broadband Content Guide (BCG) information over Internet Protocol

[ETSI TS 102 822-3-1]	ETSI TS 102 822-3-1 V1.3.1 (2006), Broadcast and On-line Services: Search, select and rightful use of content on personal storage systems (“TV-Anytime”); Part 3: Metadata; Sub-part 1: Phase 1 - Metadata schemas

[ETSI TS 102 822-3-2]	ETSI TS 102 822-3-2 V1.3.1 (2006), Broadcast and On-line Services: Search, select and rightful use of content on personal storage systems (“TV-Anytime”); Part 3: Metadata; Sub-part 2: System aspects in a uni-directional environment

[ETSI TS 102 822-3-3]	ETSI TS 102 822-3-3 V1.1.1 (2006), Broadcast and On-line Services: Search, select and rightful use of content on personal storage systems (“TV-Anytime”); Part 3: Metadata; Sub-part 3: Phase 2 - Extended Metadata Schema

[ETSI TS 102 822-3-4]	ETSI TS 102 822-3-4 V1.1.1 (2006), Broadcast and On-line Services: Search, select and rightful use of content on personal storage systems (“TV-Anytime”); Part 3: Metadata; Sub-part 3: Phase 2 - Interstitial metadata

[ETSI TS 102 822-5-1]	ETSI TS 102 822-5-1 V1.2.1 (2006), Broadcast and On-line Services: Search, select and rightful use of content on personal storage systems (“TV-Anytime”); Part 5: Rights Management and Protection (RMP) Sub-part 1: Information for Broadcast Applications

[ETSI TS 102 822-6-1]	ETSI TS 102 822-6-1 V1.3.1 (2006), Broadcast and On-line Services: Search, select and rightful use of content on personal storage systems (“TV-Anytime”); Part 6: Delivery of metadata over a bi-directional network; Sub-part 1: Service and transport

[ETSI TS 102 822-6-3]	ETSI TS 102 822-6-3 V1.1.1 (2006), Broadcast and On-line Services: Search, select and rightful use of content on personal storage systems (“TV-Anytime”); Part 6: Delivery of metadata over a bi-directional network; Sub-part 3: Phase 2 - Exchange of Personal Profile

[ETSI TS 102 822-8]	ETSI TS 102 822-8 V1.1.1 (2006), Broadcast and On-line Services: Search, select and rightful use of content on personal storage systems (“TV-Anytime”); Part 8: Phase 2 - Interchange Data Format

[IETF RFC 3265]	IETFRFC 3265 (2002), Session Initiation Protocol (SIP)-Specific Event Notification

[IETF RFC 4287]	IETFRFC 4287 (2005), The Atom Syndication Format

[OASIS SOAP-MS]	OASIS 200401 (2004), Web Services Security: SOAP Message Security 1.0

[OASIS WSBN]	OASIS (2006), Web Services Base Notification 1.3 (WS-BaseNotification)

[OMA-REL]	Open Mobile Alliance (2005), DRM Rights Expression Language, Candidate Version 2.0

[SMPTE 380M]	SMPTE 380M-2004, Material Exchange Format (MXF) Descriptive Metadata. Scheme - 1

3 	Definitions

3.1 	Terms defined elsewhere

This document uses the following terms defined elsewhere:

3.1.1 	Metadata: Structured, encoded data that describe characteristics of information-bearing entities to aid in the identification, discovery, assessment, and management of the described entities. [FG-IPTV-DOC-0194]

3.2 	Terms defined in this Recommendation

This document defines the following terms:

3.2.1 	Metadata fragments: A metadata fragment is a self-consistent atomic portion of a metadata instance. In this context, self-consistency means that fragments can be obtained in a random order and each fragment can be transmitted and updated independently.

3.2.2	Metadata instance: A metadata instance is the data instance describing the instance of content or user, etc. A metadata instance has its data-model defined by corresponding metadata schema.

3.2.3	Metadata schema: A metadata schema is the representation format for specifying data-model describing target instance.

3.2.4	Usage environment: The usage environment is described by user characteristics, terminal capabilities, network characteristics and natural environment characteristics, where the contents being consumed.

4 	Abbreviations and acronyms

This document uses the following abbreviations and acronyms:

CS	Classification Scheme

EPG	Electronic Program Guide

ESG	Electronic Service Guide

FLUTE	File deLivery over Unidirectional Transport (IETF RFC 3926)

HTTP	Hypertext Transfer Protocol

ISAN	International Standard Audiovisual Number

OASIS	Organization for the Advancement of Structured Information Standards

PPV	Pay-Per View

QoS	Quality of Service

SDO	Standards Development Organization

SIP	Session Initiation Protocol

SOA	Service-Oriented Architecture

SOAP	Simple Object Access Protocol

VoD	Video-on-Demand

XML	eXtensible Markup Language

5	Conventions

No specific conventions have been used in this document.

6 	IPTV Metadata service overview

Figure 6-1 illustrates metadata flows between metadata sources and client side metadata applications, including user navigation methods and interactive program guides.

Metadata Server

Content

Provider

Service

Provider

Network

Provider

End-user

Client-Side

Metadata

Applications

Service / Content Metadata

User / Context Metadata

Figure 6-1: IPTV Metadata service overview

The Metadata Server is the entity responsible for aggregating metadata sets produced by Content Providers or Service Providers to describe services and content, as well as metadata sets generated or registered by metadata clients to describe end-user preference or context. These metadata sets are maintained in the database managed by the Metadata Server. The metadata maintained in the Metadata Server’s database is accessed by, delivered to or contributed from metadata clients through metadata delivery and exchange protocols. These clients are typically categorized as Web-based navigation servers maintained by the Service Provider or client-side applications running on the IPTV client. The Web servers provide web-pages to the IPTV client through logical interfaces between the Service Provider and end-user domain. These pages are consumed by Web browsers to aid end-users in obtaining their preferred content. Metadata directly consumed by a client-side metadata application is used for providing the network-transparent user interface for navigation, for example, a content overview listing integrated with local content storage management. The Metadata Server also stores and manages end-user profiles or context metadata required to support content or service adaptation.

The following clauses provide summarized requirements for IPTV Metadata with some solutions, elements or attributes defined in those standard specifications as relevant for gap analysis.

Standards references are listed in clause 2 of this document.

7 	Metadata service discovery

Locating a particular service in question is provided by the service discovery function. This functionality is implemented by service description metadata and the service discovery protocol. There are already a lot of solutions in this area; however, they usually have a limited scope constrained by the context of the standardization body and are not generally applicable, and not inter-operable. A unified service discovery mechanism is needed for all services defined in IPTV context. Metadata service would be categorized in one of those services. The Metadata Service should be discovered in the same manner as other service attachments.

[ETSI TS 102 539] specifies a mechanism for signalling and carrying metadata over an always-on bi-directional IP network. The specification allows for metadata describing both Content on Demand and live services already defined in [ETSI TS 102 034]. It specifies the mechanisms used for service discovery, service selection and the delivery of service discovery information.

7.1	Service provider metadata

The basic process of how a user starts consuming an IPTV service can be described in the following steps. User finds service provider and registers for the service usage. The entry point for finding service provider of user’s preference provides listings of service metadata entry point and their providers. User browses the available detailed offerings by service descriptions provided by those entry points. Then user selects offerings and starts consumption.

The description of the available IPTV service providers may contain the following information for each service provider;

- Unique Service Provider Identifier (e.g. a domain name)

- Service Provider Name in different languages

- Service Provider Description in different languages

- Entry point for subscription

- Entry point to get description of IPTV services offered by this service provider

- Etc.

8 	Metadata delivery method

The metadata delivery protocol is implemented on the basis of a common transport stratum. There are several styles of metadata delivery, such as unicast or multicast, subscribe-notify, query-response, or a mixture of those modes. These metadata delivery methods can be implemented on a SIP-based service platform, a WebService based service-oriented architecture (SOA) framework. The delivery protocol should be modular so that different kinds of metadata delivery can be supported by a single protocol suite.

- Push or pull mode of delivery

- Unicast or multicast, ensuring reliability or not.

- Query-response; metadata bi-directional transport, XQuery, etc.

[ETSI TS 102 539] specifies push and pull modes of content description delivery for container-based and query/response-based protocols. DVB adopts the DVB-STP multicast transport protocol for push mode and the HTTP unicast protocol for pull mode as defined in [ETSI TS 102 034]. A profile for unicast query/response delivery using Simple Object Access Protocol (SOAP) over HTTP specified in [ETSI TS 102 822-6-1] is also defined as optional. The query protocol can retrieve any portion of XML instance identified by XPath predefined or dynamically assigned by the content guide service. [ETSI TS 102 822-8] defines the interchange data format for the delivery of metadata and content referencing information from different data sources.

As per clause 8 of [ETSI TS 102 471] Metadata transport containers carrying ESG metadata instances are transported in FLUTE dynamic file delivery carousel sessions as described for file delivery in [ETSI TS 102 472].

8.1 	Updating method

Out-dated metadata should be updated automatically if delivered metadata is already cached or stored in the metadata client. The versioning and updating for delivered metadata is managed through a fragment updating method. The metadata consistency and reliability must be guaranteed by this method itself. When changes in the original metadata instance occur, it should be possible to notify clients with replicated instances of this metadata automatically. A mechanism should also be provided for maintaining the atomicity, consistency, isolation and durability of the related Metadata Fragments.

8.1.1 	Metadata fragments

A Metadata fragment is a self-consistent atomic portion of a metadata instance sent to the metadata client. In this context, self-consistency means that fragments can be obtained in a random order and each fragment can be transmitted and updated independently. The instance is identified through individual fragment IDs. The fragment can be defined or managed at the transport container level or the metadata representation, schema level.

- Metadata instance identification, versioning and updating

- Fragment definition; scope or unit, layer of schema or transport envelope, etc.

[ETSI TS 102 539] and [ETSI TS 102 471] rely on fragment updating method defined in [ETSI TS 102 822-6-1]. The detail of the metadata fragment structure is in [ETSI TS 102 822-3-2]. This document defines how to split the metadata description into a number of self-contained fragments and how to identify them and mange their versions and expiration periods.

Sub-clause 7.3 “The Fragment Management Information “of [ETSI TS 102 471] 7.3 provides the encapsulation mechanism for a set of ESG metadata fragments, by providing the ability to assign a unique identifier (fragment ID) for the lifetime of an ESG fragment and indicating the current version of an ESG fragment.

[IETF RFC 4287] 4.2.9 published, 4.2.15 updated

8.2 	Metadata transport container

Metadata must be transport protocol agnostic, thus a generic transport envelope or container format may be required. The transport container contains metadata instances. The header of transport container should contain information about the metadata representation scheme, such as the coding format, whether it is compressed or uncompressed, and the specific compression method used. If the metadata fragment is defined at the transport container level, the fragment should be encapsulated in the container with fragment management information, which allows the metadata client to identify new versions of a fragment without actually reading and comparing the content of the fragments. Fragment management information includes unique identification of the fragment, version of the fragment instance and timing information, such as the start and end time of availability of the instance. The metadata in this container may be secured with cryptographic methods that prevent unauthorized modification by intermediate entities on its distribution path. In this case, security and integrity management parameters may be included in the header of container. In some environment, the transport containers may be multiplexed with content delivery protocol data units, as in-band delivery.

- Generalised metadata envelope

- Fragment encapsulation, filtering information

- Security consideration

- In-band with content delivery

[ETSI TS 102 822-3-2] specifies the structure of the container carrying the metadata fragment. It describes the format of the container and additional requirements on the delivery layer to provide container identification, information about the type of data a container carries and the current version of a container.

Per clauses 6 and 7 of [ETSI TS 102 471] ESG fragments are encapsulated in the ESG Container. The ESG Container provides fragment aggregation, versioning and filtering. ESG fragments may be represented in uncompressed, GZIP-compressed and BiM-compressed forms.

8.3 	Metadata change notification

Notification of modifications to metadata within Metadata Server should be propagated to metadata clients. If metadata is pre-cached or stored in a local metadata client database, the Metadata Server must keep track of the most recent versions of the metadata instances already delivered. If the Metadata Server detects a modification, the most current version of those instances must be propagated to the clients and outdated metadata instances should be automatically discarded and updated to the latest version. Notification can be announced at a scheduled time or on demand by a client request. Clients should be able to declare their preferences for filtering the notifications.

- Change notification subscription, delivery

- Reliable transport for notification

- Subscribe-notify; session initiation protocol (SIP) specific event notification, Web service event notification, etc.

Web Services Notification [OASIS WSBN] provides a standardized way for a Web service, or other entity, to disseminate event notification to another set of clients. The SIP-specific framework for event notification [IETF RFC3265] also specifies a flexible way to provide generalized event notification services. Both types of event notification can be applied to the publish/subscribe service model for metadata delivery management systems.

9 	Metadata representation

Most of metadata instances which are generated according to industry standards and are compliant to them which are represented in extensible markup language (XML) are not interoperable. There is a strong requirement that the semantics and representation scheme of metadata should be based on a common standardized data model and format. Ideally, names and terms should be selected from a controlled vocabulary or thesaurus so that indexing, search and retrieval are more accurate. Standard representation, registration and maintenance procedures should be specified to ensure interoperability of controlled vocabularies. Existing and mature metadata standards, such as [ISO/IEC 15938-5] or [ETSI TS 102 822-3-1], shall be adopted or integrated to avoid reinvention of new metadata standards.

- Metadata in XML data

- Adopt existing standardized metadata schema as a base for further extension

- Integrated with controlled vocabulary or thesaurus management standard

10 	Metadata elements

This clause enumerates typical elements or attributes required for the standardized set of metadata.

10.1 	Service and content metadata

Service and content metadata are descriptive information and must be able to be parsed by machines for consumption by user interface. The end-user shall be able to navigate through a variety of service and content offerings in order to select a specific service to acquire the desired contents.

Service metadata includes detail description used for service discovery and selection to activate a specific IPTV service. Content metadata is produced by content providers and service providers. Content providers typically supply title, synopsis, genre, and other descriptive metadata about the content, while service providers assign the time of delivery, cost, and other information about the service.

Elements required in service and content metadata are;

Elements required for content;

- Identifier

  - [ETSI TS 102 822-3-1] 6.3.6 programId, 6.3.7 groupId (Group), 6.4.2 InstanceMetadataId, 6.4.3 serviceId, 6.6.5 segmentId, 6.6.6 groupId (SegmentGroup)

  - [ETSI TS 102 822-3-3] 8.4 item_id, 8.5 component_id, 8.6 crid (Package)

  - [ETSI TS 102 471] 5.4.1 serviceID, 5.5.1 serviceBundleID, 5.6.1 contentID, 5.7.1 scheduleId

  - [b-UPnP CDS2] B.8.3 upnp:programID, B.8.4 upnp:seriesID, B.8.5 upnp:channelID, B.9.1.1 upnp:channelGroupName@id, B.10.2 upnp:radioStationID

  - [IETF RFC 4287] 4.2.6 id

- Title/name (secondary title)

  - [ETSI TS 102 822-3-1] 6.3.4 Title (Program, Group), 6.4.2 Title (ProgramLocation), 6.4.3 Name (ServiceInformaion), 6.6.2 Title (Segment, SegmentGroup)

  - [ETSI TS 102 471] 5.4.1 ServiceName, 5.5.1 ServiceBundleName, 5.6.1 Title (Content)

  - [b-UPnP CDS2] B.8.1 upnp:programTitle, B.8.2 upnp:seriesTitle, B.9.1 upnp:channelGroupName, B.11.2 upnp:channelName

  - [IETF RFC 4287] 4.2.14 title

- Rights or copy rights information (including information about the content owner)

  - [IETF RFC 4287] 4.2.10 rights

- Name of content provider, service provider (e.g. broadcasting organization)

  - [ETSI TS 102 822-3-1] 6.4.3 Owner (brand owner of the service)

  - [ETSI TS 102 471] 5.4.1 ServiceProvider, 5.5.1 ServiceBundleProvider

  - [b-UPnP CDS2] B.9.4 upnp:serviceProvider

  - Primary (, secondary) spoken language

  - [ETSI TS 102 822-3-1] 6.3.4 Language (Program, Group), 6.3.5 AudioLanguage (Program, ProgramLocation) 6.4.3 ServiceLanguage

  - [ETSI TS 102 822-3-3] 6.1.1.1 AudioLanguage (Component)

  - [ETSI TS 102 471] 5.4.1 ServiceLanguage, 5.6.1 Language (Content), 5.10.7.1 Language (Component)

  - [b-UPnP CDS2] B.7.7 dc:language

- Primary (secondary) caption language (with speed parameter)

  - [ETSI TS 102 822-3-1] 6.3.4 CaptionLanguage (Program, Group)

  - [ETSI TS 102 471] 5.6.1 CaptionLanguage (Content), 5.10.7.1 OpenCaptionLanguage (Component)

- Genre

  - [ETSI TS 102 822-3-1] 6.3.4 Genre (Program, Group), 6.4.2 Genre (ProgramLocation), 6.4.3 ServiceGenre, 6.6.2 Genre (Segment, SegmentGroup)

  - [ETSI TS 102 471] 5.4.1 ServiceGenre, 5.5.1 ServiceBundleGenre, 5.6.1 Genre (Content)

  - [UPnP CDS2] B.4.1 upnp:genre

  - [IETF RFC 4287] 4.2.2 Category

- Keywords

  - [ETSI TS102 822-3-1] 6.3.4 Keyword (Program, Group), 6.6.2 Keyword (Segment, SegmentGroup)

  - [ETSI TS102 471] 5.6.1 Keyword (Content)

- Description (, synopsis, abstract)

  - [ETSI TS 102 822-3-1] 6.3.4 Synopsis (Program, Group), 6.4.2 Synopsis (ProgramLocation), 6.4.3 ServiceDescription, 6.6.2 Synopsis (Segment, SegmentGroup)

  - [ETSI TS 102 471] 5.4.1 ServiceDescription, 5.5.1 ServiceBundleDescription, 5.6.1 Synopsis (Content)

  - [b-UPnP CDS2] B.7.1 dc:description

  - [IETF RFC 4287] 4.2.13 Summary

- Credits (e.g. actor/actress, director, producer, scenario writer)

  - [ETSI TS 102 822-3-1] 6.3.4 CreditsList (Program, Group), 6.6.2 CreditsList (Segment, SegmentGroup)

  - [ETSI TS 102 471] 5.6.1 CreditsList (Content)

  - [b-UPnP CDS2] B.3.1 upnp:artist, B.3.2 upnp:actor, B.3.3 upnp:author, B.3.4 upnp:producer, B.3.5 upnp:director, B.3.6 dc:publisher, B.3.7 dc:contributor, B.1.7 dc:Title, B.1.8 dc:Creator

  - [IETF RFC 4287] 4.2.1 author, 4.2.3 contributor

- Awards

  - [ETSI TS 102 822-3-1] 6.3.4 AwardsList (Program, Group)

- Location and time of production

  - [ETSI TS 102 822-3-1] 6.3.4 ProductionLocation, ProductionDate (Program, Group)

  - [b-UPnP CDS2] B.7.4 upnp:region, B.7.6 dc:date

- Parental guidance or rating (with rating standards identifier)

  - [ETSI TS 102 822-3-1] 6.3.4 ParentalGuidance (Program, Group)

  - [ETSI TS 102 471] 5.4.1 ParentalGuidance (Service), 5.5.1 ParentalGuidance (ServiceBundle), 5.6.1 ParentalGuidance (Content)

- Review (with name or reviewing entity)

  - [ETSI TS 102 822-3-1] 6.3.8 Review

  - [b-UPnP CDS2] B.8.9 upnp:rating

- Type of contents, like promotional, advertisement

- Encrypted or not

  - [ETSI TS 102 471] 5.4.1 clearToAir (Service), 5.7.1 clearToAir (ScheduleEvent)

- Preview, supplementary video and descriptive audio

  - [ETSI TS 102 822-3-1] 6.3.4 RelatedMaterial (Program, Group), 6.6.2 RelatedMaterial (Segment, SegmentGroup)

  - [b-UPnP CDS2] B.5.4 dc:relation

- Related web site

  - [ETSI TS 102 822-3-1] 6.3.4 RelatedMaterial (Program, Group), 6.6.2 RelatedMaterial (Segment, SegmentGroup)

  - [ETSI TS 102 471] 5.4.1 RelatedMaterial (Service), 5.5.1 RelatedMaterial (ServiceBundle), 5.6.1 RelatedMaterial (Content)

  - [b-UPnP CDS2] B.5.4 dc:relation, B.2.1.19 res@contentInfoURI

  - [IETF RFC 4287] 4.2.7 link

- Information for invoking value-added applications, associated or un-associated, service provider-managed applications, e.g. news highlights, stock exchange, traffic conditions, sport results, in the form of print text, images and other associated assets as enhancements.

  - [ETSI TS 102 822-3-1] 6.3.4 RelatedMaterial (Program, Group), 6.6.2 RelatedMaterial (Segment, SegmentGroup)

- Codec or format

  - [ETSI TS 102 822-3-1] 6.3.5 Coding

  - [ETSI TS 102 822-3-3] 6.1.1.1 Coding (Audio), 6.1.1.2 Coding (Video)

  - [ETSI TS 102 471] 5.10.7.1 Codec, ProfileLevelIndication

- Aspect ratios, resolution, bit rate, frame rate for video

- [ETSI TS 102 822-3-1] 6.3.5 AspectRatio, HorizontalSize, VerticalSize, Bitrate, FrameRate

- [ETSI TS 102 822-3-3] 6.1.1.1 BitRate, AspectRatio, HorizontalSize, VerticalSize, FrameRate

- [ETSI TS 102 471] 5.10.7.1 Bandwidth, FrameRate

- [ETSI TS 102 822-3-1] B.2.1.10 res@resolution, B.2.1.6 res@bitrate

- Mono, stereo, multi-channel indication for audio

  - [ETSI TS 102 822-3-1] 6.3.5 MixType, NumOfChannels

  - [ETSI TS 102 822-3-3] 6.1.1.1 MixType, NumOfChannels

  - [ETSI TS 102 471] 5.10.7.1 Mode

  - [b-UPnP CDS2] B.2.19 res@nrAudioChannels

- Acquisition or delivery schedule, start/end availability, protocol and address

  - [ETSI TS 102 822-3-1] 6.4.2 PublishedStartTime, PublishedEndTime, StartOfAvailability, EndOfAvailability, ProgramURL

  - [ETSI TS 102 471] 5.7.1 PublishedStartTime, PublishedEndTime

  - [b-UPnP CDS2] B.11.3 upnp:scheduledStartTime, B.11.4 upnp:scheduledEndTime

- First/repeat delivery or live broadcast

  - [ETSI TS 102 822-3-1] 6.4.2 FirstShowing, LastShowing, Repeat, Live

  - [ETSI TS 102 471] 5.7.1 live, repeat

  - [b-UPnP CDS2] B.8.10 upnp:episodeType

- File format, File size

  - [ETSI TS 102 822-3-1] 6.3.5 FileFormat, FileSize

  - [ETSI TS 102 822-3-3] 6.1.3 FileFormat, FileSize

  - [ETSI TS 102 471] 5.10.7.1 FileFormat, Storage

  - [UPnP CDS2] B.2.1.3 res@size

- Duration

  - [ETSI TS 102 822-3-1] 6.3.4 Duration (Program, Group), 6.6.5 SegmentLocator (Segment), 6.6.6 Duration (SegmentGroup)

  - [ETSI TS 102 471] 5.6.1 Duration (Content)

  - [b-UPnP CDS2] B.2.1.4 res@duration

- Content expiration date

- Replacement timing

Elements required for collective content;

In some cases, content needs to be a collection of various kinds of sub-contents. For example, series is a collection of each episode. Previously, group is well known as a collection of same types of contents, and a package is a collection of various types of contents such as main content and related web pages.

Considering the relation between EPG metadata and EPG application, metadata for these collective contents need to be defined separately from the applications for presenting them.

- Total number of sub-contents

  - [ETSI TS 102 822-3-1] 6.3.7 numOfItems (Group), 6.6.6 numberOfSegments (SegmentGroup)

  - [b-UPnP CDS2] B.8.6 upnp:episodeCount

- Ordered flag to indicate whether the sub-contents are ordered or not

  - [ETSI TS 102 822-3-1] 6.3.7 ordered (Group), 6.6.6 ordered (SegmentGroup)

- IDs for sub-contents

  - [ETSI TS 102 822-3-1] 6.3.6 index

  - [ETSI TS 102 822-3-3] 8.4 item_id

  - [b-UPnP CDS2] B.8.7 upnp:episodeNumber

- Price and conditions of availability of a collective content

  - [ETSI TS 102 822-3-1] 6.3.4 PurchaseItem

  - [b-UPnP CDS2] B.9.5 upnp:price

- IDs for other collections of which this collection is a member

  - [ETSI TS 102 822-3-1] 6.3.6 MemberOf (Program), 6.3.7 MemberOf (Group)

  - Descriptions for each sub-content

    - Content Type

[ETSI TS 102 822-3-3] 6.1.3 ContentType

    - Elements for content

[ETSI TS 102 822-3-3] 8.4 Item

    - Description of Usage environment where each sub-content is properly consumed

[102 822-3-3] 6.2.8 TerminalInformation, NetworkInformation, NaturalEnvironmentInformation

    - Description of User Preferences and profile with which a user hopefully wants to consume the sub-content.

[ETSI TS 102 822-3-3] 6.2.8 TargetUserPreferences, UserInformation

    - Relation information between sub-contents such as one is preceded from other one.

[ETSI TS 102 822-3-3] 8.4 RelationType

Elements required for optional quality monitoring;

- Video quality scores of compressed video data [J.bitvqm draft]

Elements required for group of segment;

- Play-list for digesting (e.g. highlights)

  - [ETSI TS 102 822-3-1] 6.6.6 SegmentGroupInformationType

  - [ETSI TS 102 822-3-3] 8.4 SelectionType, ChoiceType

Elements required for service (or channel);

- Channel Number, Minor Channel Number (the minor channel number is used for two-part channel numbers)

- Service (or channel) logo

  - [ETSI TS 102 822-3-1] 6.4.3 Logo

  - [ETSI TS 102 471] 5.4.1 ServiceLogo

  - [b-UPnP CDS2] B.7.3 upnp:icon

- Type of service, such as video-on-demand (VOD), live streaming.

  - [ETSI TS 102 822-3-1] 6.4.2 live

  - [ETSI TS 102 471] 5.4.1 ServiceType, 5.6.1 ContentType, 5.7.1 live

- Time-table (schedule)

  - [102 822-3-1] 6.7.1 Schedule

  - [102 471] Compilation of 5.7.1 ScheduleEvent

- Near VoD support or not

- Free or subscribed

  - [ETSI TS 102 822-3-1] 6.4.2 Free (indicates free access or not)

  - [ETSI TS 102 471] 5.4.1 freeToAir

- Local Network Affiliate (describes the local network affiliate, such as KNSD is an NBC affiliate in San Diego, California; and KOMO is an ABC affiliate in Seattle, Washington.)

- Channel Content Advisory (used to label channels with adult content, enabling parents to block this content, if desired)

Elements required for describing delivery modes;

- Push or pull

- Caching policy

- Pre-recording (for subtitles, captions, additional textual information, associated application)

- Required quality of service (QoS) level

  - [ETSI TS 102 822-3-3] 6.2.8 NetworkInformation

Elements required for content adaptation;

  - Targeting (class of user profile, user preference)

  - [ETSI TS 102 822-3-3] 6.2.8 TargetUserPreferences, TargetUsageHistory, UserInformation

  - Interstitials (commercials and promos) replacement, replacement rules

  - [ETSI TS 102 822-3-4] 7.6.1 InterstitialBreakType

  - Consumption control parameter, based on usage environment (device, network, etc.)

  - [ETSI TS 102 822-3-3] 6.2.8 TerminalInformation, NetworkInformation, NaturalEnvironmentInformation

Elements required for Usage restrictions, usage rules;

[ETSI TS 102 471] 5.8.1 UsageConstraints

  - ValidityPeriod

  - [ETSI TS 102 822-5-1] 5.3 Time Window Start Date & Time Window End Date

  - [b-UPnP CDS2] B.2.1.14 res@validityStart, B2.1.15 res@validityEnd

  - [OMA-REL] 5.5.4 datetime, 5.5.5 interval

  - Output control (for display only or recording in storage device)

  - [ETSI TS 102 822-5-1] 5.3 High Definition Digital Export Control, Analogue Export Signalling, Analogue Standard Definition (SD) control, Simultaneous Rendering Counta

  - [UPnP CDS2] B.2.1.13 res@allowedUse (number of plays, copies) B.2.1.17 res@usageInfo

  - Trick mode enabled or not

  - [ETSI TS 102 822-5-1] 5.3 Buffer Duration

  - Maximum buffering size or duration

  - [102 822-5-1] 5.3 Buffer Duration

  - Schedule (, timeslots) for pay-per view (PPV)

  - Licensing and subscription information, e.g. retail price, billing models, discount, conditions and license information

  - [ETSI TS 102 822-3-1] 6.3.4 PurchaseItem

  - [ETSI TS 102 822-3-3] 6.4 ExtendedPurchaseItemType

  - [ETSI TS 102 471] 5.8.1 PurchaseType

  - [b-UPnP CDS2] B.9.5 upnp:price, B.9.6 upnp:payPerView

  - DRM type, clearing house or pricing server URL, license identification

  - [ETSI TS 102 822-3-1] 6.3.4 PricingServerURL

  - [ETSI TS 102 471] 5.8.1 PurchaseRequest, 5.9.1 PortalURL

  - [b-UPnP CDS2] B.2.1.5 res@protection

  - Restricted area

  - [ETSI TS 102 822-5-1] 5.3 Geographical Control

  - Expiration date or deletion management information

  - Recordable or not, in service provider storage or consumer storage

  - [b-UPnP CDS2] B.7.16 upnp:recordable

  - Payment models depends on amounts of interstitial content

  - Content adaptation allowed or not, i.e. by transcoding

10.2 	User metadata

User metadata is shared and a common structure describing profiles or context related to an end-user. It could be represented by end-user preference, usage history, context for contents consuming environment like device and transport capability, such as types of codec, QoS statistic, etc. They can be accessed by several applications over multiple service domains. These data elements are intended to be reusable between different applications. User metadata specifies customer specific information required to support functions such as notification delivery for preferred contents subscription, customization of contents, consumption policy control, interstitial ads insertion, etc.

Elements required in user metadata are;

Elements required for User profile or preference;

  - [ETSI TS 102 822-3-3] 9 ExtendedUserDescription

  - User identifier

  - [ETSI TS 102 822-3-1] 6.5.2.3 UserIdentifier

  - Demographic data

  - [ETSI TS 102 822-3-3] 6.2.1 BioGraphicInformation, 6.2.6 Location

  - Favorite channels

  - [ETSI TS 102 822-3-1] 6.5.2.2 UsagePreference ([ISO/IEC 15938-5] 15.2.3.3 PublicationSource)

  - Viewing behaviour

  - [ETSI TS 102 822-3-1] 6.5.1.1 UsageHistory

  - Multiple personal profiles identification

  - Personal profiles, personal preferences exchangeable or not, with other devices, e.g. PDRs

  - [ETSI TS 102 822-6-3] 5 User Profile Message Interface

  - Support "personally linked" segments

Elements required for audience measurement metadata;

Audience measurement metadata consists of elements in measurements and context information.

(Measurements)

  - Minimum information for channel identification, content identification where the concept of a channel may not exist (e.g. downloads and content-on-request), and platform identification

  - Additional information about viewed services and the interactive services

  - Other specific measures: e.g. information about receiver configuration, favorite channels, channel availability, quality and availability of services

  - Timing

(Context information)

- Identification of users

- State and use of the receiver

- Localization of the user for mobile environments

- User authorization

- Capabilities of the device to receive the AV services, information about the environment

Elements required for device and network description;

  - Interface capability, e.g. bit rate, bandwidth restrictions of network interface

    - [ISO/IEC 21000-7] 5.5.3 NetworkCapabilityType

  - Presentation capability (e.g. resolution, supported codec or formats, media profile)

    - [ISO/IEC 21000-7] 5.4 TerminalCapabilitiesType

Elements required for EPG UI representation;

- EPG graphical presentation control information like fonts, the icons, the pointer, the background pictures, and the sounds effect, etc.

- Certified skins of EPG appearances

11	Metadata for content provisioning

There are some metadata in the content provisioning phase exchanged across contributing networks that include both content providers and service providers. These descriptive metadata are required to provide details about the production, such as the title and responsible organization, location of the shot or shots, participants, and other annotating information. It is essential for contributors to be consistent in how the metadata is added, terms are used, and rules are followed, so that everyone involved provides information that is useful and understood by all parties within the metadata workflow, with minimal duplication of effort.

The followings are specific set of descriptive metadata expected to be exchanged in content provisioning phase.

- Titles, Group Relation ships, and Branding

- Identification

- Event and Publication

- Award

- Captions Description

- Annotation and Classification

- Setting Period

- Scripting

- Shot and Key Point

- Participant

- Contacts List, Person, Organization, and Location

- Contract and Right

- Image Format

- Device Parameters

A framework of descriptive metadata schemes is defined in DMS-1[SMPTE 380M]. DMS-1 is intended for B2B interchange.

12	Metadata aggregation management

There are some attributes required for managing the instance of metadata. It is likely that metadata from a number of independent content creators and publishers will need to be aggregated in so-called metadata aggregator. The owner information of original metadata should be managed properly during the process of metadata aggregation or delivery.

- Metadata Publisher, Metadata Owner

- Metadata Instance Copyright Line (contains the legal text associated with the metadata copyright)

[IETF RFC 4287] 4.2.1 author, 4.2.3 contributor, 4.2.10 rights

13	User metadata security

The authentication and integrity validation of sub-blocks of the user metadata must be possible where good privacy protection is required. It should be possible to specify access rights of valid user to metadata instances. Metadata servers may be enabled to impose different levels of authorization requirements on metadata.

- Access, privacy control for user profile, user metadata

[ETSI TS 102 822-6-3] 6 Notes on Privacy and Security, Messaging security by [OASIS SOAP-MS]

14	Classification scheme

The classification scheme (CS) is an MPEG-7 tool for the provision of controlled terminology for use in classification and is referred by some standardized metadata. The namespaces for CS extended or originally created for ITU-T IPTV services are specified and maintained with their detail CS definitions.

- Controlled terminology defined by standardized classification scheme (CS)

- CS Namespace maintenance

[ETSI TS 102 822-3-1] Annex.A Classification Scheme

15	Identity management

Identity management ensures uniqueness and integrity of content, service, content or service provider, user and device. It is required to allow service providers to offer service and contents efficiently. Those identifiers may be used for service customization, content adaptation or advertisements targeting. ITU-T should specify the syntax of those identifiers and their encoding restriction within ITU-T IPTV services.

[ISO/IEC 15706] ISAN identifies all types of audiovisual works and their relevant versions. Registrations and allocation are done via Registration Agencies. ISAN permanently identifies an audiovisual work and its versions over its lifecycle.

16	Metadata API

IPTV Middleware may provide the application programming interface (API) to access metadata information.metadata access API has already been standardized in DVB and other SDOs for specific ECG schema set. Careful evaluation is required for deciding which level of API classes should be provided to fulfil generalized application requirements.

17	Metadata framework

A general metadata framework is defined in [ITU-T F.750].

# Bibliography

[b_UPnP CDS2]	UPnP Forum (2006), ContentDirectory service:2 Service Template Version 1.01 For UPnP™ Version 1.0, Document version 1.00.

________________