|  | مكتب تقييس الاتصالات |  |
| --- | --- | --- |

|  |  | جنيف، 29 يوليو 2008 |
| --- | --- | --- |
| المرجع: | TSB Circular 231 COM 12/JKK | - إلى إدارات الدول الأعضاء في الاتحاد |
| الهاتف: الفاكس: البريد الإلكتروني: | +41 22 730 5780 +41 22 730 5853 tsbsg12@itu.int | نسخة إلى: - أعضاء قطاع تقييس الاتصالات؛ - المنتسبين إلى قطاع تقييس الاتصالات؛ - رئيس لجنة الدراسات 12 ونوابه؛ - مدير مكتب تنمية الاتصالات؛ - مدير مكتب الاتصالات الراديوية |

الموضوع:	لجنة الدراسات 12 تلتمس المشاركة في وضع نموذج معلمات غير تدخلي
لتقييم أداء التدفق متعدد الوسائط (P.NAMS)

الإجراء:	يرجى الرد في موعد أقصاه 30 سبتمبر 2008

حضرات السادة والسيدات،

تحية طيبة وبعد ...

1	تنوي لجنة الدراسات 12 في إطار المسألة 14/12 وضع معايير لنموذج معلمات غير تدخلي جديد لتقييم أداء التدفق متعدد الوسائط، يعرف مؤقتاً باسم P.NAMS.

2	ويرد في ملحق هذه الرسالة الحدود المرسومة لمواصفات هذا النموذج.

3	يرجى من المشاركين مراعاة ضرورة أن تتضمن ردودهم على هذه الدعوة للمشاركة العناصر التالية:

-	وصف مقتضب للنموذج المقترح أو لجزء منه،

-	قائمة بمجالات التطبيق وأساليب التشغيل التي يشملها النموذج،

-	ما إذا كان التعاون في وضع النموذج مقبولاً أم لا.

ويتعين على المشاركين أيضاً إجراء اختبارات ذاتية لتقييم النموذج أو القيام بأعمال تحضيرية أو تقييمات إحصائية.

4	وسأكون ممتناً لو تفضلتم بإرسال الرد مع بيان استعدادكم للمشاركة في وضع معايير النموذج P.NAMS إلى أمانة لجنة الدراسات 12بقطاع تقييس الاتصالات (tsbsg12@itu.int) في موعد أقصاه 30 سبتمبر 2008.

5	ويرجى إرسال أي طلب لالتماس المزيد من التفصيل أو الاستيضاح بشأن هذه الدعوة للمشاركة إلى أمانة لجنة الدراسات 12، وحبذا لو كان ذلك بالبريد الإلكتروني على العنوان: tsbsg12@itu.int.

6	وختاماً أود أن أشدد على أهمية مشاركتكم في وضع معايير هذا النموذج الجديد لأنها سوف تساعد في توجيه لجنة الدراسات 12 في جهودها في وضع توصية جديدة تصف نموذجاً موضوعياً لمعلمات تقييم النوعية من شأنه أن يتنبأ بتأثير أحوال تعطل ملحوظة في شبكة بروتوكول الإنترنت على النوعية كما يراها المستعمل النهائي في التدفق متعدد الوسائط وفي تطبيقات تلفزيون بروتوكول الإنترنت عبر مختلف أنساق النقل.

وتفضلوا بقبول فائق التقدير والاحترام.

مالكولم جونسون
مدير مكتب تقييس الاتصالات

الملحقات: 1

ANNEX
(to TSB Circular 231)

Terms of reference
of a non-intrusive parametric model for the assessment of performance of 
multimedia streaming (P.NAMS)

Scope

The current scope of P.NAMS (Non-intrusive parametric model for the assessment of performance of multimedia streaming) is given below:

This Recommendation describes an objective parametric quality assessment model that predicts the impact of observed IP network impairments on quality experienced by the end-user in multi-media streaming and IPTV applications over transport formats such as: RTP (over UDP), MPEG2 (over UDP or RTP/UDP), 3GPP-PSS (over RTP).

This model is restricted to information contained in packet headers. Information from decoding the bit-stream or the packet payload is not used.

This model predicts Mean Opinion Scores (MOS) on the ACR scale for audio and video parts of the stream, as well as global multi-media MOS score (as defined in ITU-T Rec P.911, for instance).

The primary applications for this model are monitoring of transmission quality for operations and maintenance purposes. The P.NAMS model may be deployed both in end-point locations and at mid-network monitoring points.

The primary quality prediction made by such a model is not based on the payload of the stream being analysed, but assumes a typical, or generic, audiovisual payload.

This Recommendation cannot provide a comprehensive evaluation of transmission quality as perceived by a particular end-user because its scores reflect the impairments on the IP network being measured, that may only be part of the end-to-end connection. Furthermore, the scores predicted by a parametric model (i.e. without access to content) necessarily reflect an average perceptual impairment.

The effects of audio level, noise, delay, (and corresponding similar video factors) and other impairments related to the payload are not reflected in the scores computed by such a model. Therefore, it is possible to have high scores with this Recommendation, yet have a poor quality of the stream overall.

Model Structure

The model consists of a core module only. Since the parameter extraction is trivial for transport level parameters (such as packet loss) there is no need to define a parameter extraction module. The application scenarios allow for many implementation locations (in a network node, in a mobile telephone, in a drive test tool, in a set-top box etc.). The parameter extraction procedure depends on the implementation location, which makes it even more unnecessary to define a parameter extraction module.

Application areas

Q.14/12 foresees two distinct application areas for P.NAMS:

- QCIF-QVGA (also known as " low bitrate mode "), mostly for mobile TV and Streaming with the sub application areas:

  - Linear Mobile TV over RTP (includes Mobile TV over a 3G mobile network with MBMS and with unicast, transport over RTP/UDP/IP)

  - Multimedia streaming (includes 3GPP PSS with transport over RTP/UDP/IP)

- SD and HD television (also known as " high bitrate mode "), mostly for IPTV with the sub application areas.

  - Linear broadcast TV (includes transmission over MPEG2-TS/RTP/UDP/IP, MPEG2-TS/UDP/IP and RTP/UDP/IP transport)

  - Video on-demand (includes transmission over MPEG2-TS/RTP/UDP/IP, MPEG2-TS/UDP/IP and RTP/UDP/IP transport)

Modes of operations

The P.NAMS model can be implemented in different locations in a network, with access to different input parameters. That leads to four modes of operation:

| Class | Name | Description |
| --- | --- | --- |
| Mid-point or End-point | Static Operation | The model uses information from the local transport layer, prior knowledge about coding and prior knowledge about the end-point. |
| Mid-Point | Non-embedded Dynamic Operation | The model uses information from the local transport layer, prior knowledge about coding and information about the end-point collected through signalling protocols |
| End-point/Mid-Point | Distributed Operation | The model, located inside the network, uses information from the transport layer measured at an end-point and collected through signalling protocols, prior knowledge about coding and information about the end-point collected through signalling protocols |
| End-point | Embedded Operation | The model uses information from the local transport layer, information from the end-point, and prior knowledge about coding |

An example of signalling protocols potentially used to carry end-point information is given for Universal Mobile Telecommunications System (UMTS), transparent end-to-end Packet-switched Streaming Service (PSS) in ETSI TS 126 234, V7.4.0. Another example would be a future extension of RTCP-XR (IETF RFC 3611) to video and multimedia metrics.

- Static operation mode inside the network

- Static operation mode inside a terminal

- Non-embedded dynamic operation mode

- Non-embedded distributed operation mode

- Embedded operation mode

In addition to the modes described above, the P.NAMS model can be divided up into two bitrate modes for low and high bitrates (corresponding to the main application areas, as mentioned above). That will lead to potentially eight different modes of P.NAMS.

P.NAMS Properties

| Video codecs | MPEG2, MPEG4 Visual Simple Profile, H.264 baseline profile, H.264 main profile, H.264 high profile |
| --- | --- |
| Audio codecs | AMR-NB, AMR-WB+, AAC-LC, G.729.1, to be completed |
| Total coded bitrates | 32 kbps – 20 Mbps |
| Video sizes | QCIF, QVGA, SD, HD |
| Quality degradations covered by the model | Lossy encoding, packet loss, initial buffering, freezing with and without skipping (rebuffering) Error concealment, or any kind of post-filter performed by the client, is included. Audio-video synchronisation is excluded (not possible to detect with a parametric model) |

P.NAMS Input Parameters

P.NAMS uses information from packet headers (RTP headers and MPEG2 Transport Stream headers), buffering information, information about codecs and encoded bitrates. P.NAMS does not use any payload information.

The possible common input parameters for all modes of operation are given in the table below:

| Input parameter | Value range, unit |
| --- | --- |
| Video codec | This information is not necessarily accessible in the same way depending on the implementation of the model For low bitrate mode, one of the following: - MPEG4 Visual Simple Profile - H.264 baseline profile For high bitrate mode, one of the following: - MPEG2 - H.264 baseline profile - H.264 main profile - H.264 high profile |
| Video size | QCIF, QVGA, SD, HD |
| Audio codec | AMR-NB, AMR-WB+, AAC-LC, G.729.1, to be completed |
| Coded total bitrate | 32 – 20480 (32 kbps to 20 Mbps)s |
| Type of video content | To be defined |
| Product name and version | e.g., Windows media ver.x |
| Group of pictures (GoP) | e.g., M = 2, N = 15, ... |
| Frame rate | e.g., 30 fps, 25 fps, ... |
| Number of slices | 1, 2, etc. |
| Type of audio content | To be defined |
| Audio sampling rate | e.g., 44.1 kHz, 22.05 kHz, .. |
| Number of audio channels | 5.1ch, Stereo, monaural |
| Audio frame length | e.g., 20ms, 64ms, … |
| Information on whether payload-unit-start indicator in TS header means the start of frame or of slice | Boolean |
| Information on whether marker bit in RTP header means the start of frame or of slice | Boolean |
| Information on whether random-access indicator in TS header means the start of I-frame or of IDR-frame | Boolean |
| Information on whether elementary-stream-propriety indicator in TS header means the start of I-frame or of IDR-frame | Boolean |

The possible specific input parameters for each of the 4 modes of operation are given in the table below:

| Input parameter | Static operation | Dynamic operation | Distributed operation | Embedded operation |
| --- | --- | --- | --- | --- |
| Packet arrival time | X | X | Reported back from the client to a mid-point | X |
| Packet header information | When RTP is used (without MPEG2-TS): IP, UDP and RTP headers When MPEG2-TS is used: MPEG2-TS header. IP, UDP and RTP headers if they are used | When RTP is used (without MPEG2-TS): IP, UDP and RTP headers When MPEG2-TS is used: MPEG2-TS header. IP, UDP and RTP headers if they are used | Reported back from the client to a mid-point : - Number of RTP packets lost in succession expressed as an integer equal or larger than 1. If successive loss occurs more than once each loss is represented with an integer. Each loss has an associated timestamp expressed in playing time. - Packet loss rate in percent (as a fractional value) - Corruption duration metric. A QoE value reported from 3GPP PSS (Packet Switched Streaming) capable terminals. | When RTP is used (without MPEG2-TS): IP, UDP and RTP headers When MPEG2-TS is used: MPEG2-TS header. IP, UDP and RTP headers if they are used |
| Jitter buffer size |  |  |  | X |
| Bit rate (audio and video) measured at end-point |  | X | X |  |
| Packet-loss ratio (audio and video) measured at end-point |  | X | X |  |
| Packet-discard ratio (audio and video) measured at end-point |  | X | X |  |
| Averaged burst packet loss length (audio and video) measured at end-point |  | X | X |  |
| Jitter buffer size measured at end-point |  | X | X |  |
| End-point Multimedia MOS measured at end-point |  | X | X |  |
| End-point Video MOS measured at end-point |  | X | X |  |
| End-point Audio MOS measured at end-point |  | X | X |  |
| Freezing with and without skipping | Estimation of freezing based on for example throughput and previous knowledge about buffer size (To be further defined). | For each freezing event: - Length in seconds (fractional value) - Location in media stream (seconds fractional value from start of measuring point, not including previous freezings) - Type (skipping, no skipping) |  | For each freezing event: - Length in seconds (fractional value) - Location in media stream (seconds fractional value from start of measuring point, not including previous freezings) - Type (skipping, no skipping) |

P.NAMS Output Parameters

The P.NAMS model output is estimated multimedia ACR MOS, with separate video and audio ACR MOS. P.NAMS should have different ACR MOS scales for different video sizes.*

Optional parameters can be found in the (non-exhaustive) list below:

- Jitter buffer size

- Bit rate (audio and video)

- Packet-loss ratio (audio and video)

- Packet-discard ratio (audio and video)

- Averaged burst packet loss length (audio and video)

- Burst packet loss variation range

- Mean packet size

- Delay

- Delay variation range

Requirements

Answers to this call for participation will include:

- A brief description of the proposed model or piece of model,

- The list of application areas and operation modes covered by the model (see the corresponding sections above),

- Whether a collaboration on the development of the model is acceptable or not.

It is also requested from proponents to commit on the production of new subjective databases to train and validate models.

The statistical evaluation procedure and selection criteria will be published at a further stage. They are still under discussion under Q.14.

Invitation for participation

Proponents who are willing to participate in the standardisation of P.NAMS are invited to send an indication of their participation, including the information required in the section "Requirements" above, to the ITU-T Study Group 12 secretariat (tsbsg12@itu.int) by September, 30th, 2008 (arrival date at the TSB). An additional indication to the Q14 email reflector (tsg12q14@itu.int) is appreciated.

Schedule

| Item no. | Description | Remark | Schedule |
| --- | --- | --- | --- |
| 1 | Official call for participation | With at least the following questions: - Preference for collaboration or competition? - What operation mode(s) and application area(s) from ToR are covered? Will also require commitment from proponents to produce new subjective databases to train and validate models | SG12 meeting in May 2008 |
| 2 | First draft of test plan | Includes the corresponding test methodology | SG12 meeting in May 2008 Postponed to October 2008 |
| 3 | Deadline for declaration of participation to model development |  | End September 2008 |
| 4 | Decisions on: - competition or collaboration - number of different models to standardise - mandatory outputs | Will depend on the received declaration of participation. Possible interim meeting? | October 2008 |
| 5 | Agreed test plan for model validation | Includes performance and accuracy requirements and criteria. | Competition: January 2009 Collaboration: November 2008 |
| 6 | Collection of training databases |  | T5 + 2 months |
| 7 | Submission of candidate model |  | T5 + 5 months |
| 8 | Collection of validation databases |  | T5 + 8 months |
| 9 | Evaluation of candidate model(s) |  | T5 + 9 months |
| 10 | Draft of P.NAMS |  | T5 + 10 months |
| 11 | Consent of P.NAMS |  | First SG12 meeting of 2010 |

Further remarks

Interested parties are invited to subscribe to the Question 14/12 e-mail reflector (tsg12q14@itu.int).

____________