| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |

Radiocommunication Bureau (Direct Fax N°. +41 22 730 57 85)

| Circular Letter CM/19 | 27 March 2009 |
| --- | --- |

To Administrations of Member States of the ITU

Subject:	Rules of Procedure approved by the Radio Regulations Board

- References:	Appendix 18 to the Radio Regulations

- To the Director-General

Dear Madam/Sir,

1	Pursuant to the provisions of Nos. 13.12 and 13.14 of the Radio Regulations, the Radio Regulations Board (RRB) at its 49th meeting (1-5 December 2008) approved new or modified Rules of Procedure.

2	The new Rules of Procedure concerning Appendix 18 (Rev.WRC-07) to the Radio Regulations dealing with the Table of transmitting frequencies in the VHF maritime mobile band are included in the Annex.

3	The Bureau remains at the disposal of administrations for any further information and clarifications that may be needed in respect to this matter.

Yours faithfully,

- Valery Timofeev

Director, Radiocommunication Bureau

- Annex:	Rules concerning Appendix 18 to the Radio Regulations (included in the Rules of Procedure – Edition 2009)

Distribution:
-	Administrations of Member States of the ITU

- International Maritime Organization (IMO)

- International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA)

ANNEX

# Rules concerning

## APPENDIX  18 to the RR

WRC-07 revised Appendix 18 and modified the scope of application of several channels listed in Appendix 18 (Rev.WRC-07). Such modification was carried out, inter alia, on channels 01, 07, 19, 20, 21, 60, 66, 78, 79, 80 and 81, by adding the specific note “m” against these channels, which indicates: “These channels may be operated as single frequency channels, subject to coordination with affected administrations”. The mention of note “m” against a given channel in the Table of transmitting frequencies in Appendix 18 is normally associated with the symbol “x” in the column “Single frequency”. However, WRC-07 omitted the indication “x” in the column “Single frequency” against channels 01, 07, 19, 20, 21, 60, 66, 78, 79, 80 and 81, although it added the note “m” against these channels.

Thus, the RRB considers that all channels to which the note “m” applies may be operated as single frequency channels and has placed the symbol “x*” in the appropriate locations of the table below, which appears to have been unintentionally omitted in the Radio Regulations by WRC-07.

Table of transmitting frequencies in the
VHF maritime mobile band

(See Appendix 18 (Rev.WRC-07))

| Channel designator | Notes | Transmitting frequencies (MHz) |  | Inter-ship | Port operations and ship movement |  | Public corres-pondence |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | From ship stations | From coast stations |  | Single frequency | Two frequency |  |
| 60 | m), o) | 156.025 | 160.625 |  | x* | x | x |
| 01 | m), o) | 156.050 | 160.650 |  | x* | x | x |
| 61 | m), o) | 156.075 | 160.675 |  | x | x | x |
| 02 | m), o) | 156.100 | 160.700 |  | x | x | x |
| 62 | m), o) | 156.125 | 160.725 |  | x | x | x |
| 03 | m), o) | 156.150 | 160.750 |  | x | x | x |
| 63 | m), o) | 156.175 | 160.775 |  | x | x | x |
| 04 | m), o) | 156.200 | 160.800 |  | x | x | x |
| 64 | m), o) | 156.225 | 160.825 |  | x | x | x |
| 05 | m), o) | 156.250 | 160.850 |  | x | x | x |
| 65 | m), o) | 156.275 | 160.875 |  | x | x | x |
| 06 | f ) | 156.300 |  | x |  |  |  |
| 66 | m), o) | 156.325 | 160.925 |  | x* | x | x |
| 07 | m), o) | 156.350 | 160.950 |  | x* | x | x |
| 67 | h) | 156.375 | 156.375 | x | x |  |  |
| 08 |  | 156.400 |  | x |  |  |  |
| 68 |  | 156.425 | 156.425 |  | x |  |  |
| 09 | i) | 156.450 | 156.450 | x | x |  |  |
| 69 |  | 156.475 | 156.475 | x | x |  |  |

| * Note: Added by the RRB until the necessary action is taken by a future WRC. Channel designator | Notes | Transmitting frequencies (MHz) |  | Inter-ship | Port operations and ship movement |  | Public corres-pondence |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | From ship stations | From coast stations |  | Single frequency | Two frequency |  |
| 10 | h), q) | 156.500 | 156.500 | x | x |  |  |
| 70 | f), j) | 156.525 | 156.525 | Digital selective calling for distress, safety and calling |  |  |  |
| 11 | q) | 156.550 | 156.550 |  | x |  |  |
| 71 |  | 156.575 | 156.575 |  | x |  |  |
| 12 |  | 156.600 | 156.600 |  | x |  |  |
| 72 | i) | 156.625 |  | x |  |  |  |
| 13 | k) | 156.650 | 156.650 | x | x |  |  |
| 73 | h), i) | 156.675 | 156.675 | x | x |  |  |
| 14 |  | 156.700 | 156.700 |  | x |  |  |
| 74 |  | 156.725 | 156.725 |  | x |  |  |
| 15 | g) | 156.750 | 156.750 | x | x |  |  |
| 75 | n) | 156.775 | 156.775 |  | x |  |  |
| 16 | f ) | 156.800 | 156.800 | DISTRESS, SAFETY AND CALLING |  |  |  |
| 76 | n) | 156.825 | 156.825 |  | x |  |  |
| 17 | g) | 156.850 | 156.850 | x | x |  |  |
| 77 |  | 156.875 |  | x |  |  |  |
| 18 | m) | 156.900 | 161.500 |  | x | x | x |
| 78 | m) | 156.925 | 161.525 |  | x* | x | x |
| 19 | m) | 156.950 | 161.550 |  | x* | x | x |
| 79 | m) | 156.975 | 161.575 |  | x* | x | x |
| 20 | m) | 157.000 | 161.600 |  | x* | x | x |
| 80 | m) | 157.025 | 161.625 |  | x* | x | x |
| 21 | m) | 157.050 | 161.650 |  | x* | x | x |
| 81 | m) | 157.075 | 161.675 |  | x* | x | x |
| 22 | m) | 157.100 | 161.700 |  | x | x | x |
| 82 | m), o) | 157.125 | 161.725 |  | x | x | x |
| 23 | m), o) | 157.150 | 161.750 |  | x | x | x |
| 83 | m), o) | 157.175 | 161.775 |  | x | x | x |
| 24 | m), o) | 157.200 | 161.800 |  | x | x | x |
| 84 | m), o) | 157.225 | 161.825 |  | x | x | x |
| 25 | m), o) | 157.250 | 161.850 |  | x | x | x |
| 85 | m), o) | 157.275 | 161.875 |  | x | x | x |
| 26 | m), o) | 157.300 | 161.900 |  | x | x | x |
| 86 | m), o) | 157.325 | 161.925 |  | x | x | x |
| 27 |  | 157.350 | 161.950 |  |  | x | x |
| 87 |  | 157.375 | 157.375 |  | x |  |  |
| 28 |  | 157.400 | 162.000 |  |  | x | x |
| 88 |  | 157.425 | 157.425 |  | x |  |  |
| AIS 1 | f ), l), p) | 161.975 | 161.975 |  |  |  |  |
| AIS 2 | f ), l), p) | 162.025 | 162.025 |  |  |  |  |

____________________