| Prov. | IFRB comments | Rel. Prov. | Rel. Rule |
| --- | --- | --- | --- |
| 839 cont. | 3.2 cont. The Board examined this situation and is of the view that: 3.2.1 the word "previous" is to be understood as being covered by RR1610 which states "Before ..."; 3.2.2 the reference to Article 14 is equivalent to the usual wording appearing in several footnotes, and therefore the use of the band 11.7 - 12.2 GHz by the fixed-satellite service is subject to the application of Article 14; 3.2.3 the sub-band 12.1 - 12.2 GHz resulted from the splitting in two parts by WARC-ORB-85 of the band 12.1 - 12.3 GHz, which was allocated to several services. It is probable that when the splitting was done, this part of RR839 was overlooked; the Board understood the condition relating to the application of Article 14 as being intended to protect the Appendix 30 Plan and therefore, considering that this sub-band is not allocated to other services, decided to identify as administrations whose services may be affected only those administrations having an assignment in the Regions 1 and 3 Plan whose assigned bandwidth overlaps the assigned band of the proposed fixed-satellite use and with respect to which the limits set forth in Annex 4 to Appendix 30 are exceeded. The Article 14 procedure has to be applied also with respect to the fixed service in Brazil and Peru (see RR842). 3.3 In the application of RR1616, the following administrations will be considered as affected: a) vis-à-vis their terrestrial services of Regions 1, 2 and 3, if they have assignments recorded in the MIFR with overlapping frequency bands with respect to which the power flux-density values generated by the space station exceed the following threshold limits: - 150 dB(W/m²) in 4 kHz for 0o < Q < 5o - 150 + 0.5 (Q - 5) dB(W/m²) in 4 kHz for 5o < Q < 25o - 140 dB(W/m²) in 4 kHz for Q ³ 25o |  |  |

| Prov. | IFRB comments | Rel. Prov. | Rel. Rule |
| --- | --- | --- | --- |
| 839 cont. | 3.3 cont. a) where Q is the angle of arrival, in degrees, on the surface of the Earth of the radio frequency cont. wave. PFD values will be calculated under free-space propagation conditions. b) vis-à-vis their broadcasting-satellite service (Regions 1 and 3), if they have assignments in the Appendix 30 Plan with overlapping assigned bandwidth and with respect to which the power flux-density values set forth in Annex 4 to Appendix 30 are exceeded. N.B. For the threshold values included in paragraph a) above the Board used the limits developed in CCIR Recommendation 674 for Region 2 and provisionally extended their utilization for Regions 1 and 3 subject to further review by the Study Group. |  |  |
| 844 | This provision is similar to RR838. The same comments apply. |  |  |
| 845 | 1) Paragraph 3.1 of the comments under RR839 applies, with Region 2 replaced by Region 3. 2) The last sentence of this provision was valid with the original title of Article 7 of Appendix 30. This title was revised by WARC-ORB-85 and this last sentence is no longer necessary. |  |  |
| 846 | 1) This provision is similar to RR836, permitting broadcasting-satellite assignments to be used for fixed-satellite service transmissions. The comments made under RR836 apply, with fixed-satellite service replaced by broadcasting-satellite service and vice versa. 2) Earth stations receiving fixed-satellite service transmissions will be treated as the earth stations of the broadcasting-satellite service and are not to be notified. |  |  |

| Prov. | IFRB comments | Rel. Prov. | Rel. Rule |
| --- | --- | --- | --- |
| 847 . | 1) Community reception is defined in RR124; however, the Board has no means to check any difference between community reception and individual reception (RR123) other than the power flux-density limit. Frequency assignment notices of the broadcasting-satellite service are treated under Resolution No. 33, and their examination under paragraph 5.2 of this Resolution takes account of Annex 5 to Appendix 30 and of Resolution No. 34. 2) The Board understands the reference to Annex 5 to Appendix 30 as being limited to the last indent of paragraph 3.16 of that Annex. |  |  |