1.	Introduction

This Annex is intended to describe the structure of the data file known as MRF-EXTENDED and provided as an off-load facility from the IFL-on-CD-ROM and the PLANS-on-CD-ROM.  This format is also used to provide administrations with copies or extracts of the Master International Frequency Register and described in a Letter referenced 50(PMT)/0.2412/88, dated 4 November 1988.  The record types of the MRF-EXTENDED are listed below.  The User Manuals included in the CD-ROM publications may provide additional information.

| Record name and record | type | Description |
| --- | --- | --- |
| IS-M-ASSGN-COM | A01 | Characteristics of the Assignment |
| IS-M-ASSGN-PROVN | A10 | Administration(s) coordinated with (re, Assignment) |
| IS-M-ENTRY-COM | E01 | Characteristics of the Entry (common) |
| IS-M-ENTRY-1A4 | E03 | Characteristics of the Entry Extension for AP1/A4, ST61 |
| IS-M-ENTRY-1A1 | E04 | Characteristics of the Entry Extension for AP1/A1 |
| IS-M-ENTRY-1A2 | E05 | Characteristics of the Entry Extension for AP1/A2, GE75 |
| IS-M-ENTRY-1A5 | E08 | Characteristics of the Entry Extension for AP1/A5, GE84 |
| IS-M-ENTRY-1A7 | E09 | Characteristics of the Entry Extension for AP1/A7, RJ81 |
| IS-M-ENTRY-1A6 | E10 | Characteristics of the Entry Extension for AP1/A6, GE89 |
| IS-M-ENTRY-PROVN | E90 | Administration(s) coordinated with (re, Entry) |
| IS-M-1A4-ANT | M31 | Radiations characteristics for AP1/A4, ST61 |
| IS-M-1A2-ANT | M51 | Radiations characteristics for AP1/A2, GE75 |
| IS-M-1A2-HORIZ | M52 | Antenna gain in the horizontal plane for AP1/A2, GE75 |
| IS-M-1A2-VERT | M53 | Antenna gain in the vertical plane for AP1/A2, GE75 |
| IS-M-1A5-ATTN | M81 | Sectors of attenuation with respect to maximum e.r.p. for AP1/A5 |
| IS-M-1A7-SECT | M91 | Sectors of limited radiations for AP1/A7, RJ81 |
| IS-M-1A7-AUG | M92 | Augmented antenna pattern for AP1/A7, RJ81 |
| IS-M-1A7-TWR | M93 | Antenna characteristics: tower description for AP1/A7, RJ81 |
| IS-M-SUBENT-SITE | S01 | Sub-entry characteristics: transmitter/receiver as a Station for AP1/A1 |
| IS-M-SUBENT-CIRC | S02 | Sub-entry characteristics: transmitter/receiver as a Circular Area for AP1/A1, AP1/B |
| IS-M-SUBENT-ZONE | S03 | Sub-entry characteristics: transmitter/receiver as a Zone for AP1/A1, AP1/B |

2.	Assignment Structure

A single file contains multiple record types necessary to describe selected assignment types, each of these record types are of different structure and different length.  The link between these different records is achieved by a group of data elements identical and identically positioned in all records within an assignment.  The hierarchical data structure of each assignment consists of zero, one or several occurrences of those record types which describe the kind of assignment.  The data structure diagram in Table No. 5 provides a picture of the hierarchy.  As a general principle, each notification must have one and only one assignment level record (A01), at least one entry level record (E01) and entry extension record associated (E03 to E10), and depending on the type of antenna or radiation description, the appropriate number of specialised records (M31 to M93).  Each entry of AP1/A1 and AP1/B notices will have at least one sub-entry record defining the circuit.  For a given type of assignment only part of the hierarchical structure is needed to register the pertinent characteristics.  The permitted relations for each notice type are shown in Table No. 6 above.

3.	Data Structures

Table No. 7 provides the data structures for all the record types.  These data structures are included on the CD-ROM, in the directory called \FILSTRUC and can easily be copied into programs.  Within one assignment the group of data elements which links the different record types are at the beginning of each record, the fields are PLAN-NAME, FREQ-SYMBOL-ASSGN, ASSGN-FREQ, COUNTRY-ADM-SYMBOL, STN-NAME, CLASS-OF-STN, NOTIFYING-ADM, SAT-NETWORK-SYMBOL, ASSGN-ID, they must be identical.  Within one entry of one assignment, all the relevant entry-extensions and sub-entries must in addition have the ENTRY-SEQ-NBR identical.

Within one assignment, all entries must be sequentially numbered starting at number 1 in the field ENTRY-SEQ-NBR and within one entry of one assignment, all sub-entries must be sequentially numbered starting at number 1 in the field SUBENT-SEQ-NBR.  The entry-extensions pertaining to a particular entry will keep the same ENTRY-SEQ-NBR as of the entry.  The record type, the MRF-RCD-TYPE field, will indicate the structure of the data within the remaining part of each record.

4.	Purpose of the notification

A notification is intended either under Article 12 of the Radio Regulations or under Article 4 of a particular Plan Final Acts. For the Plans, the field PLAN-NAME must contain the Plan name, such as ST61, GE75, RJ81, GE84, GE85, GE89 and for Article 12 notifications this field is left empty.  A notification is intended either as an addition of a new assignment, a suppression of a recorded assignment or a modification of a recorded assignment.

In case of an addition of a new assignment, all the necessary characteristics as described in the relevant Circular Letter for a particular notice type and summarised in Annex 1, must be filled in; it might include multiple entries, entry extensions for antenna or radiation details as well as Sub-entries, each corresponding to a different record.  The unique BR's identification number in the field ASSGN-ID, is allocated only when the notice is received by BR. Therefore a sequential and unique number must be given provisionally by the administration to identify the records pertaining to this new assignment.

The field ACTION-CODE in the unique A01 record type of the assignment must contain the character "A" for Addition.

In case of a suppression of a recorded assignment, all the necessary characteristics to identify unambiguously the assignment must be contained in the unique A01 record type, including the unique assignment identifier field called ASSGN-ID.

The field ACTION-CODE in this A01 record type must contain the character "S" for Suppression.

For the  case of a modification of a recorded assignment, instead of using indicators for every single data element or group of data elements which have to be added, modified or deleted as on paper notices; a concept of replacement notice is introduced.  The complete recorded assignment will be entirely replaced by the new electronic notice which must contain all the necessary characteristics as for an addition.

The unique assignment identifier field called ASSGN-ID must contain the unique identifier of the assignment intended to be replaced.  In case of multiple entries or sub-entries within the assignment, the sequence as described in paragraph 3 above, can be either kept as in the recorded assignment or re-numbered without any implication; the replacement is done at the assignment level.

The field ACTION-CODE in the unique A01 record type must contain the character "R" for Replacement.

The Administration identification number can be recorded in the nine positions, leftmost justified, of the field called SPARES-BYTES of the A01 record type.  For compatibility reasons with the existing definition in the LFL/WIC TABLES, it must be limited to 9 characters.

5.	Magnetic Media

The MRF-EXTENDED format can be used for submission either on magnetic tape or on diskette.  End-of-record characters are mandatory.

·	The tape must be restricted for very large amount of data, it must be unlabelled in 1600 or 6250 BPI, in EBCDIC or ASCII code with a clear indication of the code used, The file contained on the tape must have unblocked records.

·	On diskette, this format can also be used providing that the diskette is in ASCII using the IBM/PC compatible code and format.  Only 3.5" diskette of HIGH DENSITY (1.44 MBytes, HD) will be accepted.

6.	Samples

The MRF-EXTENDED data structure describes an assignment with different record types.

As a simple case, the records needed to describe an AP1/A5 notice assignment are one A01 record, one E01 record and one E08 record.  If a coordination with neighbouring countries took place then an E90 record is required indicating these countries and the pertinent provision.

A more complex case is an AP1/A7 notice assignment: one A01 record, one E01 record with the associated E09 record for the daytime, another E01 record with the associated E09 record for the night-time, each of these two entries could have a M93 record to describe the antenna.

Another case is an AP1/A4 notice for a television station assignment: one A01 record, the vision and the sound carriers will require the E01 records, and two E03 records.  Moreover, associated with each of these two entries, two sets of M31 records - one for the vision carrier and one for the sound carrier - will be needed to indicate each of the directions or sectors of radiations and the effective antenna heights in particular directions or sectors.

Finally, an AP1/A1 notice will generate an assignment with one A01 record, a first Entry with one E01 record and optionally one E04 record, and maybe two sub-entries S01 records for two receiving points; and a second entry with one E01 record for another direction and one S01 record for the receiving point.

TABLE No. 5
MRF-EXTENDED hierarchical structure

µ §

Note: Shadows reflect multiple occurrences of the same record type; doted lines indicate exclusive options

TABLE No.6 Table of relations between record types and notice types for Terrestrial Services

| Record name and type |  | AP1/A1 | AP1/B | AP1/C | AP1/A2 | GE75 | AP1/A4 | AP1/A5 | AP1/A6 | AP1/A7 | RJ81 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IS-M-ASSGN-COM | A01 | M | M | M | M | M | M | M | M | M | M | A01 |
| IS-M-ASSGN-PROVN | A10 | O | O | O | O | O | O | O | O | O | O | A10 |
| IS-M-ENTRY-COM | E01 | M | M | M | M | M | M | M | M | M | M | E01 |
| IS-M-ENTRY-1A4 | E03 |  |  |  |  |  | M |  |  |  |  | E03 |
| IS-M-ENTRY-1A1 | E04 | M |  |  |  |  |  |  |  |  |  | E04 |
| IS-M-ENTRY-1A2 | E05 |  |  |  | M | M |  |  |  |  |  | E05 |
| IS-M-ENTRY-1A5 | E08 |  |  |  |  |  |  | M |  |  |  | E08 |
| IS-M-ENTRY-1A7 | E09 |  |  |  |  |  |  |  |  | M | M | E09 |
| IS-M-ENTRY-1A6 | E10 |  |  |  |  |  |  |  | M |  |  | E10 |
| IS-M-ENTRY-PROVN | E90 | O | O | O | O | O | O | O | O | O | O | E90 |
| IS-M-1A4-ANT | M31 |  |  |  |  |  | R |  |  |  |  | M31 |
| IS-M-1A2-ANT | M51 |  |  |  | R | R |  |  |  |  |  | M51 |
| IS-M-1A2-HORIZ | M52 |  |  |  | R | R |  |  |  |  |  | M52 |
| IS-M-1A2-VERT | M53 |  |  |  | R | R |  |  |  |  |  | M53 |
| IS-M-1A5-ATTN | M81 |  |  |  |  |  |  | O |  |  |  | M81 |
| IS-M-1A7-SECT | M91 |  |  |  |  |  |  |  |  |  | R | M91 |
| IS-M-1A7-AUG | M92 |  |  |  |  |  |  |  |  | R | R | M92 |
| IS-M-1A7-TWR | M93 |  |  |  |  |  |  |  |  | R | R | M93 |
| IS-M-SUBENT-SITE | S01 | R |  |  |  |  |  |  |  |  |  | S01 |
| IS-M-SUBENT-CIRC | S02 | R | R |  |  |  |  |  |  |  |  | S02 |
| IS-M-SUBENT-ZONE | S03 | R | R |  |  |  |  |  |  |  |  | S03 |
|  |  | AP1/A1 | AP1/B | AP1/C | AP1/A2 | GE75 | AP1/A4 | AP1/A5 | AP1/A6 | AP1/A7 | RJ81 |  |

| M = Mandatory | O = Optional | R = Required in specific cases |
| --- | --- | --- |

| TABLE No. 7 MRF-EXTENDED record structure |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A01 | IS-M-ASSGN-COM | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| A01 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| A01 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| A01 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| A01 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B/5B | ASS_STN.DBF1) |  | CTRY_SYM |
| A01 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A/5A | ASS_STN.DBF |  | STN_NAME |
| A01 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| A01 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| A01 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| A01 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| A01 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| A01 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| A01 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: A01 |  | * | n/a |  |  |
| A01 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF1) |  | ER_IND |
| A01 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| A01 | IFRB-REGION | Character | 1 |  | 66 | X |  |  | REG | ASS_NTF.DBF |  | REGION |
| A01 | NOTICE-TYPE | Character | 3 |  | 67 | X(3) |  |  | C | ASS_NTF.DBF |  | NOTICE |
| A01 | PROVN-REF-CODE | Character | 8 |  | 70 | X(8) |  |  | * | ASS_NTF.DBF |  | PROVN |
| A01 | PREF-BAND | Numeric | 2 |  | 78 | 9(2) |  |  | 1C | ASSGN.DBF |  | PREFBAND |
| A01 | ASSGN-FREQ-BAND | Numeric | 8 | 1 | 80 | 9(7)V9 |  |  | 1D | n/a |  |  |
| A01 | DATE-BRING-USE | Numeric | 8 |  | 88 | 9(8) |  |  | 2C | ASSGN.DBF |  | D_INUSE |
| A01 | CALL-SIGN | Character | 20 |  | 96 | X(20) |  |  | 3A | ASSGN.DBF |  | CALLSIGN |
| A01 | GEO-SITE-COORDS |  |  |  |  |  |  |  | 4C/5C | n/a |  |  |
| A01 | GEO-SITE-LONG |  |  |  |  |  |  |  | 4C/5C | ASS_STN.DBF1) |  | LONG |
| A01 | ANG-DEGREES-LONG | Numeric | 3 |  | 116 | 9(3) |  |  | 4C/5C | ASS_STN.DBF1) |  | LONG |
| A01 | LONG-DIR-FLAG | Character | 1 |  | 119 | X |  |  | 4C/5C | ASS_STN.DBF1) |  | LONG |
| A01 | ANG-MM | Numeric | 2 |  | 120 | 9(2) |  |  | 4C/5C | ASS_STN.DBF1) |  | LONG |
| A01 | ANG-SS | Numeric | 2 |  | 122 | 9(2) |  |  | 4C/5C | ASS_STN.DBF1) |  | LONG |
| A01 | GEO-SITE-LAT |  |  |  |  |  |  |  | 4C/5C | ASS_STN.DBF1) |  | LAT |
| A01 | ANG-DEGREES-LAT | Numeric | 2 |  | 124 | 9(2) |  |  | 4C/5C | ASS_STN.DBF1) |  | LAT |
| A01 | LAT-DIR-FLAG | Character | 1 |  | 126 | X |  |  | 4C/5C | ASS_STN.DBF1) |  | LAT |
| A01 | ANG-MM | Numeric | 2 |  | 127 | 9(2) |  |  | 4C/5C | ASS_STN.DBF1) |  | LAT |
| A01 | ANG-SS | Numeric | 2 |  | 129 | 9(2) |  |  | 4C/5C | ASS_STN.DBF1) |  | LAT |
| A01 | RADIUS-CIRC-AREA | Numeric | 5 |  | 131 | 9(5) |  |  | 4D | ASS_CIRC.DBF |  | RADIUS |
| A01 | ZONE-ID | Character | 20 |  | 136 | X(20) |  |  | 4E | ASS_ZONE.DBF |  | ZONE_ID |
| A01 | GROUND-CONDUCTIVITY | Character | 1 |  | 156 | X |  |  | 4G | ASS_STN.DBF |  | GND_COND |
| A01 | NATURE-OF-SERVICE | Character | 2 |  | 157 | X(2) |  |  | 6B | ASSGN.DBF |  | SERVICE |
| A01 | EXPERIMENTAL-STN | Character | 2 |  | 159 | X(2) |  |  | 6C | ASSGN.DBF |  | EXP_IND |
| A01 | CLASS-OF-OP | Character | 1 |  | 160 | X |  |  | 7B | ASSGN.DBF |  | OP_CLS |
| A01 | CIRCUIT-OP-FLAG | Character | 1 |  | 161 | X |  |  | 10A | ASSGN.DBF |  | PROB_USE |
| A01 | OP-HH-FROM |  |  |  |  |  |  |  | 10A | ASSGN.DBF |  | OP_HH_FR |
| A01 | IFRB-CLOCK-HH | Numeric | 2 |  | 162 | 9(2) |  |  | 10A | ASSGN.DBF |  | OP_HH_FR |
| A01 | IFRB-CLOCK-MM | Numeric | 2 |  | 164 | 9(2) |  |  | 10A | ASSGN.DBF |  | OP_HH_FR |
| A01 | OP-HH-TO |  |  |  |  |  |  |  | 10A | ASSGN.DBF |  | OP_HH_TO |
| A01 | IFRB-CLOCK-HH | Numeric | 2 |  | 166 | 9(2) |  |  | 10A | ASSGN.DBF |  | OP_HH_TO |
| A01 | IFRB-CLOCK-MM | Numeric | 2 |  | 168 | 9(2) |  |  | 10A | ASSGN.DBF |  | OP_HH_TO |
| A01 | OP-PERIOD-CODE | Character | 3 |  | 170 | X(3) |  |  | 10A | ASSGN.DBF |  | PERIOD |
| A01 | OP-AGENCY-CODE | Character | 3 |  | 173 | X(3) |  |  | 12A | ASS_NTF.DBF |  | AGENCY |
| A01 | OP-ADM-CODE | Character | 2 |  | 176 | X(2) |  |  | 12B | ASS_NTF.DBF |  | ADDRESS |
| A01 | ASSGN-RMKS-13C | Character | 70 |  | 178 | X(70) |  |  | 13C | ASS_NTF.DBF |  | RMKS |
| A01 | SYNC-IND | Character | 1 |  | 248 | X |  |  | SYNC | ASSGN.DBF |  | SYNC_IND |
| A01 | SYNC-NETWORK | Numeric | 4 |  | 249 | 9(4) |  |  | SYNC | ASSGN.DBF |  | SYNC_NET |
| A01 | DATE-MRF-UPD | Character | 8 |  | 253 | X(8) | BR reserved |  | * | ASS_HIST.DBF |  | D_MRFUPD |
| A01 | WIC-NBR | Numeric | 4 |  | 261 | 9(4) | BR reserved |  | * | ASS_HIST.DBF |  | WICNO_P2 |
| A01 | WIC-PART | Character | 1 |  | 265 | X | BR reserved |  | * | ASS_HIST.DBF |  | PART_P2 |
| A01 | NOTIFICATION-REASON | Character | 1 |  | 266 | X |  |  | * | ASS_NTF.DBF |  | REASON |
| A01 | NOTIFICATION-OCCURS | Character | 1 |  | 267 | X |  |  | * | ASS_NTF.DBF |  | OCCURS |
| A01 | NOISE-ZONE | Character | 1 |  | 268 | X |  |  | * | ASS_STN.DBF |  | NOISE_ZN |
| A01 | PROVNL-DEF-FLAG | Character | 1 |  | 269 | X |  |  | * | ASS_NTF.DBF |  | PRVL_IND |
| A01 | SPARE-BYTES | Character | 10 |  | 270 | X(10) | Administration id number |  | * | ASS_NTF.DBF |  | ADM_NO |
| A01 | NOTICE-TYPE-INT-EXT | Character | 1 |  | 280 | X | BR reserved |  | * | ASS_NTF.DBF |  | INT_EXT |
| A01 | SITE-ALT | Numeric | 5 |  | 281 | S9(4) |  |  | 9E | ASS_STN.DBF |  | SITE_ALT |
| A01 | REF-AMRF-PMRF | Character | 9 |  | 286 | X(9) | BR reserved |  | * | ASS_NTF.DBF |  | ASS_REL |
| A01 | DATE-NOTICE-RECEIVED | Character | 8 |  | 295 | X(8) | BR reserved |  | RDATE | ASS_HIST.DBF |  | D_RCVD |
| A01 | PUB-PART-1 | Character | 5 |  | 303 | X(5) | BR reserved |  | * | ASS_HIST.DBF |  | WICNO_P1 |
| A01 | ACTION-CODE | Character | 1 |  | 308 | X | Update Code: A, R or S |  | * | ASS_NTF.DBF |  | ACTION |
| A01 |  | Sub-total | 244 |  |  |  |  |  |  |  |  |  |
| A01 |  | Total | 309 |  |  |  |  |  |  |  |  |  |
| A10 | IS-M-ASSGN-PROVN | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| A10 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| A10 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| A10 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| A10 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B/5B | ASS_STN.DBF1) |  | CTRY_SYM |
| A10 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A/5A | ASS_STN.DBF |  | STN_NAME |
| A10 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| A10 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| A10 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| A10 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| A10 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| A10 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| A10 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: A10 |  | * | n/a |  |  |
| A10 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF1) |  | ER_IND |
| A10 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| A10 |  |  |  |  | 66 |  | 99 TIMES |  |  |  |  |  |
| A10 | NOTIFYING-ADM | Character | 3 |  | Group | X(3) | Group |  | B | APR_ADM.DBF |  | ADM_SYM |
| A10 | SAT-NETWORK-SYMBOL | Character | 3 |  | Group | X(3) | Group |  |  | APR_ADM.DBF |  | CTRY_SYM |
| A10 | COORDN-AGREE-PROVN | Character | 12 |  | 659 | X(12) |  |  | 11 | APR_ADM.DBF |  | PROVN |
| A10 |  | Sub-total | 606 |  |  |  |  |  |  |  |  |  |
| A10 |  | Total | 671 |  |  |  |  |  |  |  |  |  |
| E01 | IS-M-ENTRY-COM | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| E01 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| E01 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| E01 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| E01 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B/5B | ASS_STN.DBF1) |  | CTRY_SYM |
| E01 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A/5A | ASS_STN.DBF |  | STN_NAME |
| E01 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| E01 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| E01 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| E01 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| E01 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| E01 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| E01 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: E01 |  | * | n/a |  |  |
| E01 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF1) |  | ER_IND |
| E01 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| E01 | ENTRY-SEQ-NOTIFY | Character | 3 |  | 66 | X(3) |  |  | * | ENT_NTF.DBF |  | ENT_REL |
| E01 | FREQ-SYMBOL-REF-CARR | Character | 1 |  | 69 | X |  |  | 1B | ENT_1A1.DBF2) |  | FREQ_U |
| E01 | REF-FREQ | Numeric | 10 | 5 | 70 | 9(5)V9(5) |  |  | 1B | ENT_1A1.DBF2) |  | FREQ |
| E01 | DATE-TYPE2-FLAG | Character | 1 |  | 80 | X | BR reserved |  | 2A/B/D | FDGS.DBF |  | ABD_IND |
| E01 | DATE-2A / -2B / -2D | Character | 8 |  | 81 | X(8) | BR reserved |  | 2A/B/D | FDGS.DBF |  | D_2ABD |
| E01 | DESIGN-OF-EMISS |  |  |  |  |  |  |  | 7A | n/a |  |  |
| E01 | NECESSARY-BANDWIDTH | Character | 4 |  | 89 | X(4) |  |  | 7A | ENTRY.DBF |  | BANDWDTH |
| E01 | CLASS-OF-EMISS | Character | 5 |  | 93 | X(5) |  |  | 7A | ENTRY.DBF |  | EMIS_CLS |
| E01 | POWER-TYPE-CODE | Character | 1 |  | 98 | X |  |  | 8 | ENTRY.DBF |  | PWR_Q |
| E01 | ANT-POWER-INPUT | Numeric | 4 | 1 | 99 | S9(2)V9 |  |  | 8A | ENTRY.DBF |  | PWR |
| E01 | POWER-RAD | Numeric | 5 | 1 | 103 | S9(3)V9 |  |  | 8B | ENT_1A1.DBF2) |  | RAD |
| E01 | POWER-CODE-EIV | Character | 1 |  | 108 | X |  |  | 8B | ENT_1A1.DBF2) |  | RAD_Q |
| E01 | AZIM-OF-MAX-RAD | Numeric | 4 | 1 | 109 | 9(3)V9 |  |  | 9A | ENT_1A1.DBF2) |  | AZIM |
| E01 | ND-RAD-CODE | Character | 2 |  | 113 | X(2) |  |  | 9A | ENT_1A1.DBF2) |  | ND_IND |
| E01 | POLARIZATION | Character | 5 |  | 115 | X(5) |  |  | 9D | ENT_1A1.DBF2) |  | POLARIZ |
| E01 | ANT-MAX-HEIGHT | Numeric | 6 | 1 | 120 | S9(4)V9 |  |  | 9E | ENT_1A1.DBF2) |  | HGT_SEA |
| E01 | ANT-HEIGHT-FLAG | Character | 1 |  | 126 | X |  |  | 9E | n/a |  |  |
| E01 | ANT-GAIN | Numeric | 4 | 1 | 127 | S9(2)V9 |  |  | 9G | ENT_1A1.DBF |  | GAIN |
| E01 | ANT-GAIN-TYPE | Character | 1 |  | 131 | X |  |  | 9G | ENT_1A1.DBF |  | GAIN_U |
| E01 | ANT-TYPE | Character | 12 |  | 132 | X(12) |  |  | 9J | ENT_1A1.DBF |  | ANT_CCIR |
| E01 | REGULAR-HH-OF-OP |  |  |  |  |  |  |  | 10B | n/a |  |  |
| E01 | REGULAR-HH-FLAG | Character | 1 |  | 144 | X |  |  | 10B | ENTRY.DBF |  | PROB_USE |
| E01 | OP-HH-FROM |  |  |  |  |  |  |  | 10B | n/a |  |  |
| E01 | IFRB-CLOCK-HH | Numeric | 2 |  | 145 | 9(2) |  |  | 10B | ENTRY.DBF |  | OP_HH_FR |
| E01 | IFRB-CLOCK-MM | Numeric | 2 |  | 147 | 9(2) |  |  | 10B | ENTRY.DBF |  | OP_HH_FR |
| E01 | OP-HH-TO |  |  |  |  |  |  |  | 10B | n/a |  |  |
| E01 | IFRB-CLOCK-HH | Numeric | 2 |  | 149 | 9(2) |  |  | 10B | ENTRY.DBF |  | OP_HH_TO |
| E01 | IFRB-CLOCK-MM | Numeric | 2 |  | 151 | 9(2) |  |  | 10B | ENTRY.DBF |  | OP_HH_TO |
| E01 | OP-PERIOD-CODE | Character | 3 |  | 153 | X(3) |  |  | 10B | ENTRY.DBF |  | PERIOD |
| E01 | SEASON-OF-OP | Character | 2 |  | 156 | X(2) |  |  | 10C | ENTRY.DBF |  | SEASON |
| E01 | SOLAR-INDEX-OF-OP | Character | 1 |  | 158 | X |  |  | 10C | ENTRY.DBF |  | SOLAR |
| E01 | FDG-REG | Character | 2 |  | 159 | X(2) | BR reserved |  | 13A1 | FDGS.DBF |  | FDG_RR |
| E01 | FDG-PLANS-COORDN | Character | 2 |  | 161 | X(2) | BR reserved |  | 13A2 | FDGS.DBF |  | FDG_CRDN |
| E01 | FDG-TEX | Character | 2 |  | 163 | X(2) | BR reserved |  | 13A3 | FDGS.DBF |  | FDG_TEX |
| E01 | FDG-EMISS-USE | Character | 2 |  | 165 | X(2) | BR reserved |  | 13A4 | FDGS.DBF |  | FDG_USE |
| E01 | FDG-JUSTIFICATION | Character | 4 |  | 167 | X(4) | BR reserved |  | 13B1 | FDGS.DBF |  | FDG_JUST |
| E01 | FDG-REQ-ACTION | Character | 4 |  | 171 | X(4) | BR reserved |  | 13B2 | FDGS.DBF |  | FDG_REQ |
| E01 | FDG-REF | Character | 11 |  | 175 | X(11) | 3 TIMES; BR reserved |  | 13B3 | FDGS.DBF |  | FDG_REF /1/2/3 |
| E01 | FDG-DTE-GRP | Character | 9 |  | 208 | X(9) | 3 TIMES; BR reserved |  | 13B4 | FDGS.DBF |  | FDG_RV(D) /1/2/3 |
| E01 | RMKS-13C | Character | 30 |  | 235 | X(30) |  |  | 13C | FDGS.DBF |  | FDG_RMKS |
| E01 | POWER-MAX-DENSITY | Numeric | 4 |  | 265 | S9(2)V9 | BR reserved |  | 8C | n/a |  |  |
| E01 |  | Sub-total | 205 |  |  |  |  |  |  |  |  |  |
| E01 |  | Total | 268 |  |  |  |  |  |  |  |  |  |
| E03 | IS-M-ENTRY-1A4 | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| E03 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| E03 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| E03 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| E03 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B | ASS_STN.DBF |  | CTRY_SYM |
| E03 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A | ASS_STN.DBF |  | STN_NAME |
| E03 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| E03 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| E03 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| E03 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| E03 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| E03 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| E03 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: E03 |  | * | n/a |  |  |
| E03 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF |  | ER_IND |
| E03 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| E03 | OFFSET | Character | 3 |  | 66 | X(3) |  |  | 1E | ENT_1A4.DBF |  | OFFSET |
| E03 | TV-SYSTEM | Character | 2 |  | 69 | X(2) |  |  | 7C1 | ENT_1A4.DBF |  | TVSYS |
| E03 | COLOUR-SYSTEM | Character | 2 |  | 71 | X(2) |  |  | 7C2 | ENT_1A4.DBF |  | COLORSYS |
| E03 | ANT-COUNT | Numeric | 2 |  | 73 | 9(2) |  |  | * | n/a |  |  |
| E03 |  | Sub-total | 9 |  |  |  |  |  |  |  |  |  |
| E03 |  | Total | 74 |  |  |  |  |  |  |  |  |  |
| E04 | IS-M-ENTRY-1A1 | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| E04 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| E04 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| E04 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| E04 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B | ASS_STN.DBF1) |  | CTRY_SYM |
| E04 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A | ASS_STN.DBF |  | STN_NAME |
| E04 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| E04 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| E04 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| E04 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| E04 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| E04 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| E04 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: E04 |  | * | n/a |  |  |
| E04 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF1) |  | ER_IND |
| E04 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| E04 | ELEV-ANG-MAX-DIRECTIV | Numeric | 4 | 1 | 66 | S9(2)V9 |  |  | 9B | ENT_1A1.DBF |  | ELEV_ANG |
| E04 | BEAMWIDTH-FLAG | Character | 1 |  | 70 | X |  |  | 9C | ENT_1A1.DBF |  | BEAMWD_Q |
| E04 | ANG-WIDTH-RAD-MAIN-LOBE | Numeric | 5 | 2 | 71 | 9(3)V9(2) |  |  | 9C | ENT_1A1.DBF |  | BEAMWDTH |
| E04 |  | Sub-total | 10 |  |  |  |  |  |  |  |  |  |
| E04 |  | Total | 75 |  |  |  |  |  |  |  |  |  |
| E05 | IS-M-ENTRY-1A2 | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| E05 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| E05 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| E05 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| E05 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B | ASS_STN.DBF |  | CTRY_SYM |
| E05 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A | ASS_STN.DBF |  | STN_NAME |
| E05 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| E05 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| E05 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| E05 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| E05 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| E05 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| E05 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: E05 |  | * | n/a |  |  |
| E05 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF |  | ER_IND |
| E05 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| E05 | ANT-POWER-KW | Numeric | 6 | 2 | 66 | 9(4)V9(2) |  |  | 8A | ENTRY.DBF |  | PWR |
| E05 | POWER-EFF-MON-RAD-KW | Numeric | 3 | 2 | 72 | 9V9(2) |  |  | 8B | ENT_A2.DBF |  | RAD |
| E05 | ANT-IND | Character | 1 |  | 75 | X |  |  | 9Q | ENT_1A2.DBF |  | ANT_TYPE |
| E05 | CHGE-NO-INCREASE | Character | 1 |  | 76 | X |  |  | * | FDGS.DBF |  | CHNG_IND |
| E05 | ANT-COUNT | Numeric | 2 |  | 77 | 9(2) |  |  | * | n/a |  |  |
| E05 |  | Sub-total | 13 |  |  |  |  |  |  |  |  |  |
| E05 |  | Total | 78 |  |  |  |  |  |  |  |  |  |
| E08 | IS-M-ENTRY-1A5 | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| E08 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| E08 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| E08 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| E08 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B | ASS_STN.DBF |  | CTRY_SYM |
| E08 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A | ASS_STN.DBF |  | STN_NAME |
| E08 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| E08 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| E08 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| E08 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| E08 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| E08 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| E08 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: E08 |  | * | n/a |  |  |
| E08 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF |  | ER_IND |
| E08 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| E08 | TRANSM-SYSTEM | Numeric | 1 |  | 66 | 9 |  |  | 7D | ENT_1A5.DBF |  | TRANSM |
| E08 | FIELD-STRENGTH-REF | Numeric | 5 | 2 | 67 | 9(3)V9(2) |  |  | * | ENT_1A5.DBF |  | FS_REF |
| E08 | FIELD-STRENGTH-USABLE | Numeric | 5 | 2 | 72 | 9(3)V9(2) |  |  | * | ENT_1A5.DBF |  | FS_USE |
| E08 | ANT-MAX-EFF-HEIGHT | Numeric | 5 |  | 77 | S9(4) |  |  | 9E | ENT_1A5.DBF |  | HGT_EFF |
| E08 | POWER-RAD-MAX-EFF-HORIZ | Numeric | 4 | 1 | 82 | S9(2)V9 |  |  | 8BH | ENT_1A5.DBF |  | RAD_HOR |
| E08 | POWER-RAD-MAX-EFF-VERT | Numeric | 4 | 1 | 86 | S9(2)V9 |  |  | 8BV | ENT_1A5.DBF |  | RAD_VERT |
| E08 | ANT-EFF-HEIGHT | Numeric | 6 | 1 | 90 | S9(4)V9 | 36 TIMES |  | 9E | HGT_AZIM.DBF |  | HGT_EFF |
| E08 | POWER-RAD-EFF-HORIZ | Numeric | 4 | 1 | 306 | S9(2)V9 | 36 TIMES |  | 8BH | RAD_H_AZ.DBF |  | RAD_HOR |
| E08 | POWER-RAD-EFF-VERT | Numeric | 4 | 1 | 450 | S9(2)V9 | 36 TIMES |  | 8BV | RAD_V_AZ.DBF |  | RAD_VERT |
| E08 | ATTN-COUNT | Numeric | 2 |  | 594 | 9(2) |  |  | * | n/a |  |  |
| E08 |  | Sub-total | 530 |  |  |  |  |  |  |  |  |  |
| E08 |  | Total | 595 |  |  |  |  |  |  |  |  |  |
| E09 | IS-M-ENTRY-1A7 | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| E09 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| E09 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| E09 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| E09 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B | ASS_STN.DBF |  | CTRY_SYM |
| E09 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A | ASS_STN.DBF |  | STN_NAME |
| E09 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| E09 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| E09 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| E09 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| E09 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| E09 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| E09 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: E09 |  | * | n/a |  |  |
| E09 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF |  | ER_IND |
| E09 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| E09 | ANT-POWER-KW | Numeric | 6 | 2 | 66 | 9(4)V9(2) |  |  | 8A | ENTRY.DBF |  | PWR |
| E09 | ANT-IND | Character | 1 |  | 72 | X |  |  | 9Q | ENT_1A7.DBF |  | ANT_TYPE |
| E09 | ELECT-HEIGHT-OMNI | Numeric | 5 | 2 | 73 | 9(3)V9(2) |  |  | 9F | ENT_1A7.DBF |  | HGT_ELEC |
| E09 | RAD-RMS | Numeric | 7 | 2 | 78 | 9(5)V9(2) |  |  | 9I | ENT_1A7.DBF |  | RAD |
| E09 | FIELD-STRENGTH-USABLE | Numeric | 5 | 2 | 85 | 9(3)V9(2) |  |  | * | ENT_1A7.DBF |  | FS_USE |
| E09 | MULT-CONSTANT | Numeric | 6 | 2 | 90 | 9(4)V9(2) |  |  | * | ENT_1A7.DBF |  | K_CONST |
| E09 | PATTERN-TYPE | Character | 1 |  | 96 | X |  |  | 9O | ENT_1A7.DBF |  | PATTERN |
| E09 | QUAD-FACTOR | Numeric | 6 | 2 | 97 | 9(4)V9(2) |  |  | 9P | ENT_1A7.DBF |  | Q_FACTOR |
| E09 | STN-STATUS | Character | 1 |  | 103 | X |  |  | * | ENTRY.DBF |  | OP_IND |
| E09 | CHGE-NO-INCREASE | Character | 1 |  | 104 | X |  |  | * | FDGS.DBF |  | CHNG_IND |
| E09 | TWR-COUNT | Numeric | 2 |  | 105 | 9(2) |  |  | * | n/a |  |  |
| E09 | AUG-COUNT | Numeric | 2 |  | 107 | 9(2) |  |  | * | n/a |  |  |
| E09 | SECT-COUNT | Numeric | 2 |  | 109 | 9(2) |  |  | * | n/a |  |  |
| E09 |  | Sub-total | 45 |  |  |  |  |  |  |  |  |  |
| E09 |  | Total | 110 |  |  |  |  |  |  |  |  |  |
| E10 | IS-M-ENTRY-1A6 | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| E10 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| E10 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| E10 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| E10 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B | ASS_STN.DBF |  | CTRY_SYM |
| E10 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A | ASS_STN.DBF |  | STN_NAME |
| E10 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| E10 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| E10 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| E10 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| E10 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| E10 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| E10 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: E10 |  | * | n/a |  |  |
| E10 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF |  | ER_IND |
| E10 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| E10 | TV-SYSTEM | Character | 2 |  | 66 | XX |  |  | 7C1 | ENT_1A6.DBF |  | TVSYS |
| E10 | COLOR-SYSTEM | Character | 1 |  | 68 | X |  |  | 7C2 | ENT_1A6.DBF |  | COLORSYS |
| E10 | FREQ-VISION | Numeric | 7 | 3 | 69 | 9(3)V9(3) |  |  | 1B | ENT_1A6.DBF |  | FREQ_V |
| E10 | OFFSET-VISION | Numeric | 3 |  | 76 | S9(2) |  |  | 1E | ENT_1A6.DBF |  | OFFSET_V |
| E10 | FREQ-SOUND | Numeric | 7 | 3 | 79 | 9(3)V9(3) |  |  | 1B | ENT_1A6.DBF |  | FREQ_S |
| E10 | OFFSET-SOUND | Numeric | 3 |  | 86 | S9(2) |  |  | 1E | ENT_1A6.DBF |  | OFFSET_S |
| E10 | RATIO-VISIONSOUND | Numeric | 2 |  | 89 | 9(2) |  |  | 8D | ENT_1A6.DBF |  | FS_REF |
| E10 | FIELD-STRENGTH-REF | Numeric | 5 | 2 | 91 | 9(3)V9(2) |  |  | * | ENT_1A6.DBF |  | FS_REF |
| E10 | FIELD-STRENGTH-USABLE | Numeric | 5 | 2 | 96 | 9(3)V9(2) |  |  | * | ENT_1A6.DBF |  | FS_USE |
| E10 | ANT-MAX-EFF-HEIGHT | Numeric | 5 |  | 101 | S9(4) |  |  | 9E | ENT_1A6.DBF |  | HGT_EFF |
| E10 | POWER-RAD-MAX-EFF-HORIZ | Numeric | 4 | 1 | 106 | S9(2)V9 |  |  | 8BH | ENT_1A6.DBF |  | RAD_HOR |
| E10 | POWER-RAD-MAX-EFF-VERT | Numeric | 4 | 1 | 110 | S9(2)V9 |  |  | 8BV | ENT_1A6.DBF |  | RAD_VERT |
| E10 | ANT-EFF-HEIGHT | Numeric | 6 | 1 | 114 | S9(4)V9 | 36 TIMES |  | 9E | HGT_AZIM.DBF |  | HGT_EFF |
| E10 | ATT-HORIZ | Numeric | 2 |  | 330 | 9(2) | 36 TIMES |  | 9NH | ATTN_H_V.DBF |  | ATTN |
| E10 | ATT-VERT | Numeric | 2 |  | 402 | 9(2) | 36 TIMES |  | 9NV | ATTN_H_V.DBF |  | ATTN |
| E10 |  | Sub-total | 474 |  |  |  |  |  |  |  |  |  |
| E10 |  | Total | 539 |  |  |  |  |  |  |  |  |  |
| E90 | IS-M-ENTRY-PROVN | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| E90 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| E90 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| E90 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| E90 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B/5B | ASS_STN.DBF1) |  | CTRY_SYM |
| E90 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A/5A | ASS_STN.DBF |  | STN_NAME |
| E90 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| E90 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| E90 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| E90 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| E90 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| E90 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| E90 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: E90 |  | * | n/a |  |  |
| E90 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF1) |  | ER_IND |
| E90 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| E90 |  |  |  |  |  |  | 99 TIMES |  |  |  |  |  |
| E90 | NOTIFYING-ADM | Character | 3 |  | 66 | X(3) | Group |  | B | APR_ADM.DBF |  | ADM_SYM |
| E90 | SAT-NETWORK-SYMBOL | Character | 3 |  |  | X(3) | Group |  |  | APR_ADM.DBF |  | CTRY_SYM |
| E90 | COORDN-AGREE-PROVN | Character | 12 |  | 659 | X(12) |  |  | 11 | APR_ADM.DBF |  | PROVN |
| E90 |  | Sub-total | 606 |  |  |  |  |  |  |  |  |  |
| E90 |  | Total | 671 |  |  |  |  |  |  |  |  |  |
| M31 | IS-M-1A4-ANT | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| M31 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| M31 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| M31 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| M31 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B/5B | ASS_STN.DBF |  | CTRY_SYM |
| M31 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A/5A | ASS_STN.DBF |  | STN_NAME |
| M31 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| M31 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| M31 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| M31 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| M31 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| M31 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| M31 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: M31 |  | * | n/a |  |  |
| M31 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF |  | ER_IND |
| M31 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| M31 | POWER-RAD | Numeric | 5 | 1 | 66 | S9(3)V9 |  |  | 8B | RAD_AZIM.DBF3) |  | RAD |
| M31 | POWER-CODE-EIV | Character | 1 |  | 71 | X |  |  | 8B | ENT_1A4.DBF |  | RAD_Q |
| M31 | ANT-EFF-HEIGHT | Numeric | 6 | 1 | 72 | S9(4)V9 |  |  | 9E | HGT_AZIM.DBF4) |  | HGT_EFF |
| M31 | AZIM-OF-MAX-RAD | Numeric | 4 | 1 | 78 | 9(3)V9 |  |  | 9E | RAD_AZIM.DBF5) |  | AZIM |
| M31 | ND-RAD-CODE | Character | 2 |  | 82 | X(2) |  |  | 9A | ENT_1A4.DBF |  | ND_IND |
| M31 | AZIM-OF-RAD-SECT-FROM | Numeric | 4 | 1 | 84 | 9(3)V9 |  |  | 9H | RAD_SECT.DBF |  | SECT_FR |
| M31 | AZIM-OF-RAD-SECT-TO | Numeric | 4 | 1 | 88 | 9(3)V9 |  |  | 9H | RAD_SECT.DBF |  | SECT_TO |
| M31 |  | Sub-total | 26 |  |  |  |  |  |  |  |  |  |
| M31 |  | Total | 91 |  |  |  |  |  |  |  |  |  |
| M51 | IS-M-1A2-ANT | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| M51 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| M51 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| M51 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| M51 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B | ASS_STN.DBF |  | CTRY_SYM |
| M51 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A | ASS_STN.DBF |  | STN_NAME |
| M51 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| M51 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| M51 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| M51 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| M51 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| M51 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| M51 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: M51 |  | * | n/a |  |  |
| M51 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF |  | ER_IND |
| M51 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| M51 | POWER-RAD-NOISE | Numeric | 9 | 1 | 66 | S9(7)V9 |  |  | 9I | RAD_SECT.DBF |  | RAD |
| M51 | RAD-UNITS | Character | 1 |  | 75 | X |  |  | 9I | RAD_1A2.DBF |  | RAD_U |
| M51 | AZIM-OF-RAD-SECT-FROM | Numeric | 4 | 1 | 76 | 9(3)V9 |  |  | 9H | RAD_SECT.DBF |  | SECT_FR |
| M51 | AZIM-OF-RAD-SECT-TO | Numeric | 4 | 1 | 80 | 9(3)V9 |  |  | 9H | RAD_SECT.DBF |  | SECT_TO |
| M51 |  | Sub-total | 18 |  |  |  |  |  |  |  |  |  |
| M51 |  | Total | 83 |  |  |  |  |  |  |  |  |  |
| M52 | IS-M-1A2-HORIZ | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| M52 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| M52 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| M52 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| M52 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B | ASS_STN.DBF |  | CTRY_SYM |
| M52 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A | ASS_STN.DBF |  | STN_NAME |
| M52 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| M52 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| M52 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| M52 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| M52 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| M52 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| M52 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: M52 |  | * | n/a |  |  |
| M52 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF |  | ER_IND |
| M52 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| M52 | ANT-GAIN | Numeric | 4 | 1 | 66 | S9(2)V9 | 36 TIMES |  | 9GH | GAIN_H.DBF |  | GAIN |
| M52 |  | Sub-total | 144 |  |  |  |  |  |  |  |  |  |
| M52 |  | Total | 209 |  |  |  |  |  |  |  |  |  |
| M53 | IS-M-1A2-VERT | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| M53 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| M53 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| M53 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| M53 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B | ASS_STN.DBF |  | CTRY_SYM |
| M53 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A | ASS_STN.DBF |  | STN_NAME |
| M53 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| M53 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| M53 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| M53 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| M53 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| M53 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| M53 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: M53 |  | * | n/a |  |  |
| M53 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF |  | ER_IND |
| M53 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| M53 | AZIM | Numeric | 4 | 1 | 66 | 9(2)V9 |  |  | 9GV | GAIN_V.DBF |  | AZIM |
| M53 | ANT-GAIN | Numeric | 4 | 1 | 67 | S9(2)V9 | 9 TIMES |  | 9GV | GAIN_V.DBF |  | GAIN |
| M53 |  | Sub-total | 40 |  |  |  |  |  |  |  |  |  |
| M53 |  | Total | 105 |  |  |  |  |  |  |  |  |  |
| M81 | IS-M-1A5-ATTN | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| M81 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| M81 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| M81 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| M81 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B | ASS_STN.DBF |  | CTRY_SYM |
| M81 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A | ASS_STN.DBF |  | STN_NAME |
| M81 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| M81 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| M81 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| M81 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| M81 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| M81 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| M81 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: M81 |  | * | n/a |  |  |
| M81 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF |  | ER_IND |
| M81 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| M81 | ATTN-DB | Numeric | 2 |  | 66 | 9(2) |  |  | 9N | RAD_SECT.DBF |  | RAD |
| M81 | ATTN-SECT-FROM | Numeric | 3 |  | 68 | 9(3) |  |  | 9H | RAD_SECT.DBF |  | SECT_FR |
| M81 | ATTN-SECT-TO | Numeric | 3 |  | 71 | 9(3) |  |  | 9H | RAD_SECT.DBF |  | SECT_TO |
| M81 |  | Sub-total | 8 |  |  |  |  |  |  |  |  |  |
| M81 |  | Total | 73 |  |  |  |  |  |  |  |  |  |
| M91 | IS-M-1A7-SECT | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| M91 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| M91 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| M91 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| M91 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B | ASS_STN.DBF |  | CTRY_SYM |
| M91 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A | ASS_STN.DBF |  | STN_NAME |
| M91 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| M91 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| M91 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| M91 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| M91 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| M91 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| M91 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: M91 |  | * | n/a |  |  |
| M91 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF |  | ER_IND |
| M91 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| M91 | AZIM-OF-RAD-SECT-FROM | Numeric | 4 | 1 | 66 | 9(3)V9 |  |  | 9H | RAD_SECT.DBF |  | SECT_FR |
| M91 | AZIM-OF-RAD-SECT-TO | Numeric | 4 | 1 | 70 | 9(3)V9 |  |  | 9H | RAD_SECT.DBF |  | SECT_TO |
| M91 | RAD-MAX-STRENGTH | Numeric | 6 | 2 | 74 | 9(4)V9(2) |  |  | * | RAD_SECT.DBF |  | RAD |
| M91 |  | Sub-total | 14 |  |  |  |  |  |  |  |  |  |
| M91 |  | Total | 79 |  |  |  |  |  |  |  |  |  |
| M92 | IS-M-1A7-AUG | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| M92 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| M92 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| M92 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| M92 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B | ASS_STN.DBF |  | CTRY_SYM |
| M92 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A | ASS_STN.DBF |  | STN_NAME |
| M92 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| M92 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| M92 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  |  |  |  |
| M92 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| M92 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| M92 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| M92 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: M92 |  | * | n/a |  |  |
| M92 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF |  | ER_IND |
| M92 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| M92 |  |  |  |  | 66 |  | 40 TIMES |  |  |  |  |  |
| M92 | AUG-NBR | Numeric | 2 |  | Group | 9(2) | Group |  | 9NA | AUG.DBF |  | AUG_ID |
| M92 | RAD-FIELD | Numeric | 7 | 2 | Group | 9(5)V9(2) | Group |  | 9IA | AUG.DBF |  | RAD |
| M92 | CTR-AZIM | Numeric | 5 | 2 | Group | 9(3)V9(2) | Group |  | 9AA | AUG.DBF |  | AZIM |
| M92 | AUG-SPAN | Numeric | 5 | 2 | Group | 9(3)V9(2) | Group |  | 9CA | AUG.DBF |  | SPAN |
| M92 |  | Sub-total | 760 |  |  |  |  |  |  |  |  |  |
| M92 |  | Total | 825 |  |  |  |  |  |  |  |  |  |
| M93 | IS-M-1A7-TWR | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| M93 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| M93 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| M93 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| M93 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B | ASS_STN.DBF |  | CTRY_SYM |
| M93 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A | ASS_STN.DBF |  | STN_NAME |
| M93 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| M93 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| M93 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| M93 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| M93 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| M93 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| M93 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: M93 |  | * | n/a |  |  |
| M93 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF |  | ER_IND |
| M93 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| M93 | TWR-NBR | Numeric | 2 |  | 66 | 9(2) |  |  | 9T1 | TOWER.DBF |  | TWR_ID |
| M93 | TWR-FIELD-RATIO | Numeric | 4 | 2 | 68 | 9(2)V9(2) |  |  | 9T2 | TOWER.DBF |  | FLDRATIO |
| M93 | PHASE-DIFF | Numeric | 6 | 2 | 72 | S9(3)V9(2) |  |  | 9T3 | TOWER.DBF |  | PHASEDIF |
| M93 | TWR-SPACING | Numeric | 6 | 2 | 78 | 9(4)V9(2) |  |  | 9T4 | TOWER.DBF |  | SPACING |
| M93 | TWR-ORIENT | Numeric | 5 | 2 | 84 | 9(3)V9(2) |  |  | 9T5 | TOWER.DBF |  | ORIENT |
| M93 | REF-POINT | Numeric | 1 |  | 85 | 9 |  |  | 9T6 | TOWER.DBF |  | REF_PT |
| M93 | TWR-ELECT-HEIGHT | Numeric | 5 | 2 | 90 | 9(3)V9(2) |  |  | 9T7 | TOWER.DBF |  | HGT_ELEC |
| M93 | TWR-STRUCTURE | Numeric | 1 |  | 91 | 9 |  |  | 9T8 | TOWER.DBF |  | STRUC |
| M93 | TLS-A | Numeric | 5 | 2 | 96 | 9(3)V9(2) |  |  | 9T9A | TOWER.DBF |  | TLS_A |
| M93 | TLS-B | Numeric | 5 | 2 | 101 | 9(3)V9(2) |  |  | 9T9B | TOWER.DBF |  | TLS_B |
| M93 | TLS-C | Numeric | 5 | 2 | 106 | 9(3)V9(2) |  |  | 9T9C | TOWER.DBF |  | TLS_C |
| M93 | TLS-D | Numeric | 5 | 2 | 111 | 9(3)V9(2) |  |  | 9T9D | TOWER.DBF |  | TLS_D |
| M93 |  | Sub-total | 50 |  |  |  |  |  |  |  |  |  |
| M93 |  | Total | 115 |  |  |  |  |  |  |  |  |  |
| S01 | IS-M-SUBENT-SITE | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| S01 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| S01 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| S01 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| S01 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B | ASS_STN.DBF1) |  | CTRY_SYM |
| S01 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A | ASS_STN.DBF |  | STN_NAME |
| S01 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| S01 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| S01 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| S01 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| S01 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| S01 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| S01 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: S01 |  | * | n/a |  |  |
| S01 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF1) |  | ER_IND |
| S01 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| S01 | ASSOC-EMISS-REC-FLAG | Character | 1 |  | 66 | X |  |  | * | ENT_STN.DBF |  | ER_IND |
| S01 | ASSOC-COUNTRY-SYMBOL | Character | 3 |  | 67 | X(3) |  |  | 5B | ENT_STN.DBF |  | CTRY_SYM |
| S01 | ASSOC-STN-NAME | Character | 20 |  | 70 | X(20) |  |  | 5A | ENT_STN.DBF |  | STN_NAME |
| S01 | GEO-SITE-COORDS |  |  |  |  |  |  |  | 5C | n/a |  |  |
| S01 | GEO-SITE-LONG |  |  |  |  |  |  |  | 5C | ENT_STN.DBF |  | LONG |
| S01 | ANG-DEGREES-LONG | Numeric | 3 |  | 90 | 9(3) |  |  | 5C | ENT_STN.DBF |  | LONG |
| S01 | LONG-DIR-FLAG | Character | 1 |  | 93 | X |  |  | 5C | ENT_STN.DBF |  | LONG |
| S01 | ANG-MM | Numeric | 2 |  | 94 | 9(2) |  |  | 5C | ENT_STN.DBF |  | LONG |
| S01 | ANG-SS | Numeric | 2 |  | 96 | 9(2) |  |  | 5C | ENT_STN.DBF |  | LONG |
| S01 | GEO-SITE-LAT |  |  |  |  |  |  |  | 5C | ENT_STN.DBF |  | LAT |
| S01 | ANG-DEGREES-LAT | Numeric | 2 |  | 98 | 9(2) |  |  | 5C | ENT_STN.DBF |  | LAT |
| S01 | LAT-DIR-FLAG | Character | 1 |  | 100 | X |  |  | 5C | ENT_STN.DBF |  | LAT |
| S01 | ANG-MM | Numeric | 2 |  | 101 | 9(2) |  |  | 5C | ENT_STN.DBF |  | LAT |
| S01 | ANG-SS | Numeric | 2 |  | 103 | 9(2) |  |  | 5C | ENT_STN.DBF |  | LAT |
| S01 |  | Sub-total | 39 |  |  |  |  |  |  |  |  |  |
| S01 |  | Total | 104 |  |  |  |  |  |  |  |  |  |
| S02 | IS-M-SUBENT-CIRC | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| S02 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| S02 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| S02 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| S02 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B/5B | ASS_STN.DBF |  | CTRY_SYM |
| S02 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A/5A | ASS_STN.DBF |  | STN_NAME |
| S02 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| S02 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| S02 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| S02 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| S02 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| S02 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| S02 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: S02 |  | * | n/a |  |  |
| S02 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF |  | ER_IND |
| S02 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| S02 | ASSOC-EMISS-REC-FLAG | Character | 1 |  | 66 | X |  |  | * | ENT_CIRC.DBF |  | ER_IND |
| S02 | STN-NAME-FLAG | Character | 1 |  | 67 | X |  |  | * | n/a |  |  |
| S02 | RADIUS-CIRC-AREA | Numeric | 5 |  | 68 | 9(5) |  |  | 4D/5F | ENT_CIRC.DBF |  | RADIUS |
| S02 | COORDS-CENTRE-CIRC-AREA |  |  |  |  |  |  |  | 4C/5E | n/a |  |  |
| S02 | LONG-CENTRE-CIRC-AREA |  |  |  |  |  |  |  | 4C/5E | ENT_CIRC.DBF |  | LONG |
| S02 | ANG-DEGREES-LONG | Numeric | 3 |  | 73 | 9(3) |  |  | 4C/5E | ENT_CIRC.DBF |  | LONG |
| S02 | LONG-DIR-FLAG | Character | 1 |  | 76 | X |  |  | 4C/5E | ENT_CIRC.DBF |  | LONG |
| S02 | ANG-MM | Numeric | 2 |  | 77 | 9(2) |  |  | 4C/5E | ENT_CIRC.DBF |  | LONG |
| S02 | LAT-CENTRE-CIRC-AREA |  |  |  |  |  |  |  | 4C/5E | ENT_CIRC.DBF |  | LAT |
| S02 | ANG-DEGREES-LAT | Numeric | 2 |  | 79 | 9(2) |  |  | 4C/5E | ENT_CIRC.DBF |  | LAT |
| S02 | LAT-DIR-FLAG | Character | 1 |  | 81 | X |  |  | 4C/5E | ENT_CIRC.DBF |  | LAT |
| S02 | ANG-MM | Numeric | 2 |  | 82 | 9(2) |  |  | 4C/5E | ENT_CIRC.DBF |  | LAT |
| S02 |  | Sub-total | 18 |  |  |  |  |  |  |  |  |  |
| S02 |  | Total | 83 |  |  |  |  |  |  |  |  |  |
| S03 | IS-M-SUBENT-ZONE | Type | Width | Dec | Position | Format | Remarks |  | IFL | Cross-Reference LFL/WIC TABLES |  |  |
| S03 | PLAN-NAME | Character | 4 |  | 1 | X(4) |  |  | * | ASS_NTF.DBF |  | PLAN |
| S03 | FREQ-SYMBOL-ASSGN | Character | 1 |  | 5 | X |  |  | 1A | ASSGN.DBF |  | FREQ_U |
| S03 | ASSGN-FREQ | Numeric | 10 | 5 | 6 | 9(5)V9(5) |  |  | 1A | ASSGN.DBF |  | FREQ |
| S03 | COUNTRY-ADM-SYMBOL | Character | 3 |  | 16 | X(3) |  |  | 4B/5B | ASS_STN.DBF |  | CTRY_SYM |
| S03 | STN-NAME | Character | 20 |  | 19 | X(20) |  |  | 4A/5A | ASS_STN.DBF |  | STN_NAME |
| S03 | CLASS-OF-STN | Character | 2 |  | 39 | X(2) |  |  | 6A | ASSGN.DBF |  | STN_CLS |
| S03 | NOTIFYING-ADM | Character | 3 |  | 41 | X(3) |  |  | B | ASS_NTF.DBF |  | ADM_SYM |
| S03 | SAT-NETWORK-SYMBOL | Character | 3 |  | 44 | X(3) |  |  |  | n/a |  |  |
| S03 | ASSGN-ID | Numeric | 9 |  | 47 | 9(9) |  |  | A | ASSGN.DBF |  | ASSGN_ID |
| S03 | ENTRY-SEQ-NBR | Character | 3 |  | 56 | X(3) |  |  | * | ENTRY.DBF |  | ENTRY_ID |
| S03 | SUBENT-SEQ-NBR | Character | 3 |  | 59 | X(3) |  |  | * | n/a |  |  |
| S03 | MRF-RCD-TYPE | Character | 3 |  | 62 | X(3) | Record Type: S03 |  | * | n/a |  |  |
| S03 | EMISS-REC-FLAG | Character | 1 |  | 65 | X |  |  | * | ASS_STN.DBF |  | ER_IND |
| S03 |  | Sub-total | 65 |  |  |  |  |  |  |  |  |  |
| S03 | ASSOC-EMISS-REC-FLAG | Character | 1 |  | 66 | X |  |  | * | ENT_ZONE.DBF |  | ER_IND |
| S03 | STN-NAME-FLAG | Character | 1 |  | 67 | X |  |  | * | n/a |  |  |
| S03 | ZONE-ID | Character | 20 |  | 68 | X(20) |  |  | 4E/5D | ENT_ZONE.DBF6) |  | ZONE_ID |
| S03 | GEO-SITE-COORDS |  |  |  | 88 |  | 6 TIMES |  | 5C | n/a |  |  |
| S03 | GEO-SITE-LONG |  |  |  | Group |  | Group |  | 5C | POLY_PT.DBF |  | LONG |
| S03 | ANG-DEGREES-LONG | Numeric | 3 |  | Group | 9(3) | Group |  | 5C | POLY_PT.DBF |  | LONG |
| S03 | LONG-DIR-FLAG | Character | 1 |  | Group | X | Group |  | 5C | POLY_PT.DBF |  | LONG |
| S03 | ANG-MM | Numeric | 2 |  | Group | 9(2) | Group |  | 5C | POLY_PT.DBF |  | LONG |
| S03 | ANG-SS | Numeric | 2 |  | Group | 9(2) | Group |  | 5C | POLY_PT.DBF |  | LONG |
| S03 | GEO-SITE-LAT |  |  |  | Group |  | Group |  | 5C | POLY_PT.DBF |  | LAT |
| S03 | ANG-DEGREES-LAT | Numeric | 2 |  | Group | 9(2) | Group |  | 5C | POLY_PT.DBF |  | LAT |
| S03 | LAT-DIR-FLAG | Character | 1 |  | Group | X | Group |  | 5C | POLY_PT.DBF |  | LAT |
| S03 | ANG-MM | Numeric | 2 |  | Group | 9(2) | Group |  | 5C | POLY_PT.DBF |  | LAT |
| S03 | ANG-SS | Numeric | 2 |  | Group | 9(2) | Group |  | 5C | POLY_PT.DBF |  | LAT |
| S03 |  | Sub-total | 112 |  |  |  |  |  |  |  |  |  |
| S03 |  | Total | 177 |  |  |  |  |  |  |  |  |  |

1)	In tables ASS_STN.DBF or ASS_CIRC.DBF or ASS_ZONE.DBF depending on the type of notices.
2)	In tables ENT_1A1.DBF or ENT_1B1.DBF or ENT_1C1.DBF or ENT_1A2.DBF or ENT_1A4.DBF or ENT_1A5.DBF or ENT_1A6.DBF or ENT_1A7.DBF depending on the 	type of notices.  It gives the reference frequency (1C)
3)	In tables RAD_AZIM.DBF or RAD_SECT.DBF depending if the radiation is for one azimuth or for one sector.
4)	In tables HGT_AZIM.DBF or HGT_SECT.DBF depending if the effective antenna height is for one azimuth or for one sector.
5)	In tables RAD_AZIM.DBF or/and HGT_AZIM.DBF depending if the azimuth is valid for both the radiation and the effective antenna height.
6)	In the column Format above, an S in a numeric field stands for Sign Leading Separate.

________________