| Received: 16 May 2007 TECHNOLOGY | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Document 8F/1308-E |
|  |  | 17 May 2007 |
|  |  | English only |

Subject:	Question ITU-R 229/8

Alliance for Telecommunications Industry Solutions (ATIS)*

ATIS WTSC Evaluation report for IP-ofDma 
imt-2000 RTT submission

The IP-OFDMA IMT-2000 RTT submission contained in 8F/1079(Rev.1) (along with 8F/1065 and 8F/1075) has been evaluated.  This evaluation assessed the information the proponents had supplied using the templates contained in M.1225 according to 8/LCCE/47.  This evaluation led to the identification of a number of areas where information had not been provided according to M.1225 or 8/LCCE/47 or exhibited other problems.  Furthermore an assessment was made as to whether the RTT submission meets the Requirements and Objectives for IMT-2000 (Attachment 4 to 8/LCCE/47).  The evaluation report provides the specifics of those identifications and assessments including five Appendices.

Radio transmission technologies description template

Attachment 3 to Circular Letter 8/LCCE/47 (Annex 1 to Recommendation ITU-R M.1225).

Description of the radio transmission technology

The RTT has to be described in a detailed form to get an overview and an understanding of the functionalities of the technical approach. This Annex provides a template to aid in the technical description of the characteristics of a candidate RTT.

The following technical parameters, the relevant templates given in Annex 2 and any additionally useful information, should be provided for each test environment for which the candidate RTT is proposed to operate. This can be done by preparing:

–	a separate template submission for each test environment; or

–	a single submission that includes multiple answers for those technical parameters impacted by a test environment.

In addition to the detailed technical description described below, proponents should assure that their submission meets the overall IMT-2000 objectives as defined in existing Recommendations (see § 4). Submittors should also state if the current ITU policy for IPR is met for their RTT proposals.

The following table describes the technical parameters needed to characterise a proposal. Proponents should feel free to add any new information if required for a better assessment of their proposal.

IMT-2000 may serve both mobile users as well as fixed wireless users sharing common geographical locations and frequency bands. As a result, certain parameters may be designed for one or the other type of user in combination. To account for fixed wireless use of a candidate RTT, the description given in the template should indicate when a parameter has been designed for dual use.

# Table of contents

A1.1	Test environment support

A1.2	Technical parameters

A1.3	Expected performances

A1.4	Technology design constraints

A1.5	Information required for terrestrial link budget template

A1.6	Satellite system configuration

| Index | Criteria and attribute | Proponents’ comments | Evaluator’s Comments |
| --- | --- | --- | --- |
| A1.1 | Test environment support |  |  |
| A1.1.1 | In what test environments will the RTT operate? | - indoor - outdoor to indoor and pedestrian, - vehicular - mixed | Template information is not supplied for each text environment specified. Only a non-compliant mixed environment data is supplied |
| A1.1.2 | If the RTT supports more than one test environment, what test environment does this technology description template address? | One template for all |  |
| A1.1.3 | Does the RTT include any features in support of FWA application? Provide detail about the impact of those features on the technical parameters provided in this template, stating whether the technical parameters provided apply for mobile as well as for FWA applications. | Yes (cf. Rec. ITU-R F.1763). Flexible mixed fixed and mobile design. - QoS - Dynamic bandwidth allocation - Continuous and variable bit rate support - Support of nomadic operation - Support of fixed wireless voice, image, video and data services. |  |
| A1.2 | Technical parameters NOTE 1 – Parameters for both forward link and reverse link should be described separately, if necessary. |  |  |
| A1.2.1 | What is the minimum frequency band required to deploy the system (MHz)? | 5 MHz or 10 MHz (10 MHz provides better performance). |  |
| A1.2.2 | What is the duplex method: TDD or FDD? | TDD |  |
| A1.2.2.1 | What is the minimum up/down frequency separation for FDD? | N/A |  |
| A1.2.2.2 | What is requirement of transmit/receive isolation? Does the proposal require a duplexer in either the mobile station (MS) or BS? | Does not require a duplexer. | Transmitter/Receiver isolation requirement is not specified For example requirements for transmit/receive guard time (RTG/TTG) and switch isolation (or equivalent) need to be specified |
| A1.2.3 | Does the RTT allow asymmetric transmission to use the available spectrum? Characterize. | Yes. The ratio of uplink to downlink transmission can be reconfigured on a system-wide basis. |  |
| A1.2.4 | What is the RF channel spacing (kHz)? In addition, does the RTT use an interleaved frequency plan? NOTE 1 – The use of the second adjacent channel instead of the adjacent channel at a neighbouring cluster cell is called “interleaved frequency planning”. If a proponent is going to employ an interleaved frequency plan, the proponent should state so in § A1.2.4 and complete § A1.2.15 with the protection ratio for both the adjacent and second adjacent channel. | 5 000 kHz or 10 000 kHz The RTT does not use an interleaved frequency plan |  |
| A1.2.5 | What is the bandwidth per duplex RF channel (MHz) measured at the 3 dB down points? It is given by (bandwidth per RF channel)  (1 for TDD and 2 for FDD). Provide detail. | For 5 MHz (TDD): about 4.7 MHz, depending on the permutation used. For 10 MHz (TDD): about 9.4 MHz, depending on the permutation used. | Precise Bandwidth for each permutation is not specified |
| A1.2.5.1 | Does the proposal offer multiple or variable RF channel bandwidth capability? If so, are multiple bandwidths or variable bandwidths provided for the purposes of compensating the transmission medium for impairments but intended to be feature transparent to the end user? | The RTT offers variable RF channel bandwidth capability through the use of OFDMA sub-channelization. | This is not clear. All terminals in the same deployment must use the same RF channel BW all the time (i.e. you cannot, at least in the current status of the IEEE specification, have few terminals with 5 MHZ RF channel and few other terminals with 10 MHz RF channel). In addition, an UL subchannel will have its tiles distributed over the band and hence, even though an MS will be using a small portion of the BW, its signal will span the whole RF channel. A similar argument can be done for the DL (preamble and DL MAP). If multiple Bandwidth capability is utilized, then the information for purpose of compensating for impairments needs to be supplied |
| A1.2.6 | What is the RF channel bit rate (kbit/s)? NOTE 1 – The maximum modulation rate of RF (after channel encoding, adding of in-band control signalling and any overhead signalling) possible to transmit carrier over an RF channel, i.e. independent of access technology and of modulation schemes. | DOWNLINK Distributed permutation of sub-carriers Assumptions: 10 MHz channel bandwidth, 32 data symbols per frame (35 symbols in sub-frame, 1 symbol for preamble, 2 symbols for control information), 5 ms frame duration, 64 QAM 5/6 code rate, 30 slots for 2 symbols, 48 data tones per slot. Maximum data rate: 23 040 kbit/s Note 1: The above numbers are calculated based on the maximum DL/UL ratio supported by IP-OFDMA. Note 2: The equivalent maximum data rate number for 5 MHz channel Bandwidth is 11520 kbit/s UPLINK Distributed permutation of sub-carriers Assumptions: 10 MHz channel bandwidth, 18 data symbols per frame (21 symbols in UL sub frame, 3 symbols for control channels), 5 ms frame duration, 16 QAM 3/4 code rate, 35 slots for 3 symbols, 48 data tones per slot. Maximum data rate: 6 048 kbit/s Note 1: The above numbers are calculated based on the maximum UL/DL ratio supported by IP-OFDMA. Note 2: The equivalent maximum data rate number for 5 MHz channel Bandwidth is 3024 kbit/s. | Numbers provided are user data rates and not channel bit rates. |
| A1.2.7 | Frame structure: describe the frame structure to give sufficient information such as: – frame length, – the number of time slots per frame, – guard time or the number of guard bits, – user information bit rate for each time slot, – channel bit rate (after channel coding), – channel symbol rate (after modulation), – associated control channel (ACCH) bit rate, – power control bit rate. NOTE 1 – Channel coding may include forward error correction (FEC), cyclic redundancy checking (CRC), ACCH, power control bits and guard bits. Provide detail. NOTE 2 – Describe the frame structure for forward link and reverse link, respectively. NOTE 3 – Describe the frame structure for each user information rate. | Frame length: 5 ms The number of time slots per frame: N/A The number of time symbols per frame: 48 symbols (including TTG and RTG gaps) The number of sub-carriers per each symbol: 512 and 1024 FFT for 5 and 10 MHz respectively. Resource allocation: 2 dimensional structure for frequency and time (see section 2.4 of the RTT System Description for more details) Sub-channel structure: see Section 2.2 of the RTT System Description for details Ratio of DL and UL sub-frame: Ranging from 35 symbols: 12 symbols to 26 symbols: 21 symbols (DL:UL) (35: 12), (34: 13), (33: 14), (32:15), (31: 16), (30: 17), (29: 18), (28: 19), (27: 20), (26: 21) TTG / RTG : 105.7 μs / 60 μs Common control overhead : 1 symbol per frame for preamble (see section 2.4 of the RTT System Description for more details) DOWNLINK (See A1.2.5.1) Distributed permutation of sub-carriers The number of sub-carriers per slot : 48 (data) + 8 (pilots) Guard sub-carrier: 184 (including DC sub-carrier) The channel bit or symbol rate is variable, depending on the number of allocated slots, and the modulation and coding rate. Power control rate: no power control Adjacent permutation of sub carriers The number of sub-carriers per slot : 48 (data) + 6 (pilots) Guard sub-carrier : 160 (including DC sub-carrier) | Control channel symbol rate and bit rate not specified. ACH bit rate not specified. Power control overhead is missing. Entry specifying no power control on downlink is in conflict with A3.2.3 |
| A1.2.7 |  | UPLINK Distributed permutation of sub-carriers The number of subcarriers per slot : 48 (data) + 24 (pilots) Guard subcarrier : 184 (including DC subcarrier) The channel bit or symbol rate is variable, depending on the number of allocated slots, and the modulation and coding rate. Power control rate : 200 Hz Adjacent permutation of subcarriers The number of subcarriers per slot : 48 (data) + 6 (pilots) Guard subcarrier : 160 (including DC subcarrier) |  |
| A1.2.8 | Does the RTT use frequency hopping? If so, characterize and explain particularly the impact (e.g. improvements) on system performance. | No | Is not rotation of the UL tiles (which distributed across the band) in some sense like frequency hopping. Also, the tone assignment in DL PUSC (partial usage of sub-carrier) and FUSC changes every OFDM symbol, so is this not also in some sense like frequency hopping? |
| A1.2.8.1 | What is the hopping rate? | N/A |  |
| A1.2.8.2 | What is the number of the hopping frequency sets? | N/A |  |
| A1.2.8.3 | Are BSs synchronized or non-synchronized? | Synchronized in frequency and in time for TDD operation, even though frequency hopping is not used |  |
| A1.2.9 | Does the RTT use a spreading scheme? | No | The RTT uses a CDMA ranging channel (MC-CDMA is proposed) for Handover, therefore spreading parameters need to be supplied. |
| A1.2.9.1 | What is the chip rate (Mchip/s)? Rate at input to modulator. | N/A |  |
| A1.2.9.2 | What is the processing gain? 10 log (chip rate/information rate). | N/A |  |
| A1.2.9.3 | Explain the uplink and downlink code structures and provide the details about the types (e.g. personal numbering (PN) code, Walsh code) and purposes (e.g. spreading, identification, etc.) of the codes. | N/A |  |
| A1.2.10 | Which access technology does the proposal use: TDMA, FDMA, CDMA, hybrid, or a new technology? In the case of CDMA, which type of CDMA is used: frequency hopping (FH) or direct sequence (DS) or hybrid? Characterize. | OFDMA | MC-CDMA is used for ranging and UL sounding, therefore spreading parameters need to be supplied. |
| A1.2.11 | What is the baseband modulation technique? If both the data modulation and spreading modulation are required, describe in detail. What is the peak to average power ratio after baseband filtering (dB)? | DOWNLINK QPSK, 16 QAM, 64 QAM for data modulation. Spreading modulation does not apply. UPLINK QPSK, 16 QAM for data modulation. Spreading modulation does not apply. PAPR is about 12 dB without any PAPR reduction scheme. | PAPR for Downlink not specified. Spreading information not supplied (Ranging channel uses spreading). |
| A1.2.12 | What are the channel coding (error handling) rate and form for both the forward and reverse links? E.g., does the RTT adopt: – FEC or other schemes? – Unequal error protection? Provide details. – Soft decision decoding or hard decision decoding? Provide details. – Iterative decoding (e.g. turbo codes)? Provide details. – Other schemes? | Convolutional Coding and Convolutional Turbo Coding are supported Modulation schemes: QPSK, 16 QAM and 64 QAM for downlink, QPSK and 16 QAM for uplink. Coding rates: QPSK 1/2, QPSK 3/4, 16 QAM 1/2, 16 QAM 3/4, 64 QAM 1/2, 64 QAM 2/3, 64 QAM 3/4, 64 QAM 5/6. Coding repetition rates: 1x, 2x, 4x and 6x. Unequal error protection: None Soft decision decoding and iterative decoding: It is an implementation issue not covered by the specification. |  |
| A1.2.13 | What is the bit interleaving scheme? Provide detailed description for both uplink and downlink. | The bit interleaving scheme is the same for both uplink and downlink. All encoded data bits shall be interleaved by a block interleaver with a block size corresponding to the number of coded bits per the encoded block size. | Interleaver details not specified. Need details for example on delay, number of frames, depth, etc. |
| A1.2.14 | Describe the approach taken for the receivers (MS and BS) to cope with multipath propagation effects (e.g. via equalizer, Rake receiver, etc.). | To cope with the multipath propagation effect, the cyclic prefix and 1-tap equalizer are employed. The length of cyclic prefix is 1/8 of symbol duration thus 11.4 μs. |  |
| A1.2.14.1 | Describe the robustness to intersymbol interference and the specific delay spread profiles that are best or worst for the proposal. | The intersymbol interference can be removed by the use of sufficiently longer cyclic prefix than delay spread. | If a longer delay spread (longer than the CP can handle) is encountered, performance will suffer depending on the power delay profile of the multipath. Here, the cyclic prefix is specified as 1/8 of an OFDM symbol which corresponds to 11.4 sec. Any multipath beyond that will results in inter-carrier interference (ICI) and hence will gracefully degrade the performance, depending on the power delay profile of the channel. If the power delay profile has large energy beyond the 11.4 sec delay spread, there will be an increase in ICI, and orthogonality between carriers will be lost. In addition, there will be increased channel estimation loss. Take for example Veh B channel, the performance will be bad. In addition, in one of the answers in Annex 2, the RTT claims support of cell sizes that are up to 35 Km in size. The key question here is that for large cells, what is the probability the delay spread will be within (or reasonable close to) 11.4 sec? |
| A1.2.14.2 | Can rapidly changing delay spread profile be accommodated? Describe. | Yes, delay spread variation within the length of cyclic prefix does not cause the intersymbol interference. |  |
| A1.2.15 | What is the adjacent channel protection ratio? NOTE 1 – In order to maintain robustness to adjacent channel interference, the RTT should have some receiver characteristics that can withstand higher power adjacent channel interference. Specify the maximum allowed relative level of adjacent RF channel power (dBc). Provide detail how this figure is assumed. | Min adjacent channel rejection at BER=10-6 for 3 dB degradation C/I 11 dB – 16 QAM, 3/4 coding rate 4 dB – 64 QAM, 2/3 coding rate Min non-adjacent channel rejection at BER=10-6 for 3 dB degradation C/I 30 dB – 16 QAM, 3/4 coding rate 23 dB - 64 QAM, 2/3 coding rate |  |
| A1.2.16 | Power classes | Mobile Station Peak Transmit power (dBm) for 16QAM 1. 18 <= Ptx,max < 21 2. 21 <= Ptx,max < 25 3. 25 <= Ptx,max < 30 4. 30 <= Ptx,max Peak Transmit power (dBm) for QPSK 1. 20 <= Ptx,max < 23 2. 23 <= Ptx,max < 27 3. 27 <= Ptx,max < 30 4. 30 <= Ptx,max |  |
| A1.2.16.1 | Mobile terminal emitted power : what is the radiated antenna power measured at the antenna? For terrestrial component, give (dBm). For satellite component, the mobile terminal emitted power should be given in e.i.r.p. (effective isotropic radiated power) (dBm). | See A.1.2.16 | Terminal radiated power at the antenna is not specified |
| A1.2.16.1.1 | What is the maximum peak power transmitted while in active or busy state? | See A.1.2.16 | Maximum peak power is not specified |
| A1.2.16.1.2 | What is the time average power transmitted while in active or busy state? Provide detailed explanation used to calculate this time average power. | See A.1.2.16 | Average transmitter power is not supplied |
| A1.2.16.2 | Base station transmit power per RF carrier for terrestrial component | See A.1.2.16 | Base station transmit power is not specified |
| A1.2.16.2.1 | What is the maximum peak transmitted power per RF carrier radiated from antenna? | Not limited by RTT | Base station maximum peak transmitted power is not specified |
| A1.2.16.2.2 | What is the average transmitted power per RF carrier radiated from antenna? | Not limited by RTT | Base station average transmitted power is not specified |
| A1.2.17 | What is the maximum number of voice channels available per RF channel that can be supported at one BS with 1 RF channel (TDD systems) or 1 duplex RF channel pair (FDD systems), while still meeting ITU-T Recommendation G.726 performance requirements? | The maximum number of voice channels per 1 RF channel depends on the bit rate and sampling rate supported by the codecs defined in the G.726. For instance, in case of the bit rate of 16 kbit/s with 20 msec sampling rate, up to 256 users can be supported simultaneously by a 10 MHz RF channel, while meeting the delay requirements of VoIP. In the case of a 5 MHz channel up to 120 users can be supported. The capacity calculated assumes a blocking-limited scenario with Voice Activity Factor = 1, DL 64 QAM 5/6, and UL 16QAM 3/4. | Evaluator cannot complete analysis for this item without specific codec scenario as used for related A3 entries |
| A1.2.18 | Variable bit rate capabilities : describe the ways the proposal is able to handle variable baseband transmission rates. For example, does the RTT use: – adaptive source and channel coding as a function of RF signal quality? – Variable data rate as a function of user application? – Variable voice/data channel utilization as a function of traffic mix requirements? Characterize how the bit rate modification is performed. In addition, what are the advantages of your system proposal associated with variable bit rate capabilities? | Variable bit rate is supported by the flexible resource allocation. By assigning the variable number of sub-channels and using various modulations and coding rates frame by frame, the bit rate for each user can be variable frame by frame. Modulation and coding rate is usually defined by user's RF signal quality (CQI). For higher data rates, the bit rate information is provided to the receiver via scheduling mechanisms and associated control signaling every frame. | Modulation and Coding schemes need to be specified |
| A1.2.18.1 | What are the user information bit rates in each variable bit rate mode? | The user information bit rates are variable according to the number of sub-channels assigned and modulation and coding rate used. DOWNLINK BW: 10 MHz Modulation : QPSK, 16 QAM, 64 QAM Coding rate : 1/2, 2/3, 3/4, 5/6 3312 kbit/s (1/2, QPSK, (DL:UL)=(26:21) symbols) ~ 23040 kbit/s (5/6, 64 QAM, (DL:UL)=(35:12) symbols). See equation below. Note 1: The above numbers are calculated based on the maximum DL/UL ratio supported by IP-OFDMA. Note 2: The equivalent maximum data rate number for 5 MHz channel Bandwidth is 11520 kbit/s UPLINK BW: 10 MHz Modulation : QPSK, 16 QAM Coding rate : 1/2, 3/4 1008 kbit/s (1/2, QPSK, (DL:UL)=(35:12) symbols) ~ 6048 kbit/s (3/4, 16 QAM, (DL:UL)=(26:21) symbols). See equation below. Note 1: The above numbers are calculated based on the maximum UL/DL ratio supported by IP-OFDMA. Note 2: The equivalent maximum data rate number for 5 MHz channel Bandwidth is 3024 kbit/s. Equation used: PHY Data Rate=(Data sub-carriers/Symbol period) × (information bits per symbol) | Note 1 needs to be clarified. One can not achieve both peaks rates at the same time since the maxim DL/UL ratio implies minimum UL/DL ratio User bit rate in each variable bit rate mode (MCS and repetition rate, UL/DL ratio) needs to be supplied |
| A1.2.19 | What kind of voice coding scheme or codec is assumed to be used in proposed RTT? If the existing specific voice coding scheme or codec is to be used, give the name of it. If a special voice coding scheme or codec (e.g. those not standardized in standardization bodies such as ITU) is indispensable for the proposed RTT, provide detail, e.g. scheme, algorithm, coding rates, coding delays and the number of stochastic code books. | Due to the IP-based characteristics of the radio interface it can utilize any speech codec. | Specific codec assumed for the performance information supplied in A3 is not specified |
| A1.2.19.1 | Does the proposal offer multiple voice coding rate capability? Provide detail. | Yes. The RTT supports flexible data rate for each user and also provide variety scheduling services. A constant bit rate is provided by UGS service, while a variable bit rate is provided by ErtPS service. See A.1.2.18, A1.2.20.1 and A1.2.20.2 | Specific voice coding rates are not specified |
| A1.2.20 | Data services : are there particular aspects of the proposed technologies which are applicable for the provision of circuit-switched, packet-switched or other data services like asymmetric data services? For each service class (A, B, C and D) a description of RTT services should be provided, at least in terms of bit rate, delay and BER/frame error rate (FER). | Yes, a wide range of data services and applications with varied QoS requirements are supported. These are summarized in Table 7 of Section 0 in this submission (8F/1079R1). | Bit rate, delay, and BER/FER are not supplied for each service class |
|  | NOTE 1 – See Recommendation ITU-R M.1224 for the definition of: – “circuit transfer mode”, – “packet transfer mode”, – “connectionless service”, and for the aid of understanding “circuit switched” and “packet switched” data services. |  |  |
|  | NOTE 2 – See ITU-T Recommendation I.362 for details about the service classes A, B, C and D. |  |  |
| A1.2.20.1 | For delay constrained, connection oriented (Class A). | The RTT provides UGS (unsolicited grant service), corresponding to the Class A. UGS is characterized as constant and low data rates and low delay data service. |  |
| A1.2.20.2 | For delay constrained, connection oriented, variable bit rate (Class B). | The RTT provides rtPS (real-time polling service), corresponding to the Class B. rtPS is utilized for low to high data rate services. The RTT provides ErtPS (extended real-time polling service) as well. ErtPS is utilized for low data rate and low delay data services. |  |
| A1.2.20.3 | For delay unconstrained, connection oriented (Class C). | The RTT provides nrtPS (non-real-time polling service), corresponding to the Class C. nrtPS is utilized for high data rate services. |  |
| A1.2.20.4 | For delay unconstrained, connectionless (Class D). | The RTT provides BE (best effort service) corresponding to the Class D. BE is utilized for moderate data rate services. |  |
| A1.2.21 | Simultaneous voice/data services: is the proposal capable of providing multiple user services simultaneously with appropriate channel capacity assignment? | Yes, multiple parallel services are supported with different QoS requirements. Each service is associated with a set of QoS parameters that quantify aspects of its behavior. These parameters are managed using the dynamic service provisions, represented by the DSA and DSC message dialog. |  |
|  | NOTE 1 – The following describes the different techniques that are inherent or improve to a great extent the technology described above to be presented. Description for both BS and MS are required in attributes from § A1.2.22 through § A1.2.23.2. |  |  |
| A1.2.22 | Power control characteristics : is a power control scheme included in the proposal? Characterize the impact (e.g. improvements) of supported power control schemes on system performance. | Yes. A closed loop power control scheme and an open loop power control scheme are included. By means of these power control schemes, the interference level is reduced and the uplink system level throughput is increased. | No power control overhead information is provided. The power control rate is specified in A.1.2.7 above, however, the overhead information required to do this power control is not provided. Sending a power control command that is made off 44 bits at 200 Hz is a lot more expensive than, say, a power control sent at 800 Hz but only with 1 bit. So specifying the rate is not enough It is understood that the open loop power control rate is 200 Hz (which is implicit by the fact that DL sub frame is sent at 200 Hz). However, can closed loop power control be done at 200 Hz for all users given the amount of overhead required? If not, what will be the impact on performance. . |
| A1.2.22.1 | What is the power control step size (dB)? | Power control step size is variable ranging from 0.25 dB to 32 dB. An 8-bit signed integer in power control information element indicates the power control step size in 0.25 dB units. Normally implemented in 1 dB increments. |  |
| A1.2.22.2 | What are the number of power control cycles per second? | The power control cycle of closed-loop power control is dependent on the rate of power control information element transmission, but less than 200 Hz. Due to TDD nature, the open loop power control cycle is inherently identical to the number of frames per seconds, thus 200 Hz. |  |
| A1.2.22.3 | What is the power control dynamic range (dB)? | The minimum power control dynamic range is 45 dB. |  |
| A1.2.22.4 | What is the minimum transmit power level with power control? | The RTT supports 45 dB under the full power assumption | Minimum transmit power is not specified since the full power assumption is not specified |
| A1.2.22.5 | What is the residual power variation after power control when RTT is operating? Provide details about the circumstances (e.g. in terms of system characteristics, environment, deployment, MS-speed, etc.) under which this residual power variation appears and which impact it has on the system performance. | The accuracy for power level control can vary from ± 0.5 dB to ± 2 dB depending on the power control step size. Single step size m ¦ Required relative accuracy ¦m¦ = 1dB ¦ +/- 0.5 dB ¦m¦ = 2dB ¦ +/- 1 dB ¦m¦ = 3dB ¦ +/- 1.5 dB 4dB< ¦m¦< = 10dB ¦ +/- 2 dB Two exception points of at least 10 dB apart are allowed over the 45 dB range, where in these two points an accuracy of up to +/- 2 dB is allowed for any size step. |  |
| A1.2.23 | Diversity combining in MS and BS : are diversity combining schemes incorporated in the design of the RTT? | Yes |  |
| A1.2.23.1 | Describe the diversity techniques applied in the MS and at the BS, including micro diversity and macro diversity, characterizing the type of diversity used, for example: – time diversity: repetition, Rake-receiver, etc., – space diversity: multiple sectors, multiple satellite, etc., – frequency diversity: FH, wideband transmission, etc., – code diversity: multiple PN codes, multiple FH code, etc., – other scheme. Characterize the diversity combining algorithm, for example, switch diversity, maximal ratio combining, equal gain combining. Additionally, provide supporting values for the number of receivers (or demodulators) per cell per mobile user. State the dB of performance improvement introduced by the use of diversity. For the MS: what is the minimum number of RF receivers (or demodulators) per mobile unit and what is the minimum number of antennas per mobile unit required for the purpose of diversity reception? These numbers should be consistent to that assumed in the link budget template of Annex 2 and that assumed in the calculation of the “capacity” defined at § A1.3.1.5. | The standard supports beamforming, transmit/receive diversity and MIMO. The receiver also supports maximal ratio combining. There is no need for a Rake receiver because it is an OFDM system. | Degree of diversity improvement is not specified. Minimum number of receivers is not specified. The minimum number of antennas is not specified |
| A1.2.23.2 | What is the degree of improvement expected (dB)? Also indicate the assumed conditions such as BER and FER. | Please refer to Section 2.3 (of 8F/1079R1) |  |
| A1.2.24 | Handover/automatic radio link transfer (ALT) : do the radio transmission technologies support handover? Characterize the type of handover strategy (or strategies) which may be supported, e.g. MS assisted handover. Give explanations on potential advantages, e.g. possible choice of handover algorithms. Provide evidence whenever possible. | Yes. The RTT supports handover and also provides means for expediting handover. Each base station broadcasts the information on the list of neighboring base stations and their channel information such as the operating center frequency, preamble index and synchronization periodically. The channel information in this broadcasting is used for a mobile station to synchronize with the neighboring base station. After a mobile station monitors the signal strength of a neighboring base station and seeks suitable base station(s) for handover, the mobile station or its serving base station can initiate handover by handover request message. But only the mobile station can transmit handover indication message to the its serving base station. After transmitting handover indication message, the mobile station stops monitoring the downlink frame of its serving base station and performs network re-entry to target base station. To reduce the handover latency further, the serving base station provides the target base station with network entry information on a mobile station to be handed over the target base station. Further information is available in the IEEE 802.16 standard; Section 6.3.22 MAC layer handover procedures. | Very limited information provided on Handover (e.g. hard or soft handover, etc). |
| A1.2.24.1 | What is the break duration (s) when a handover is executed? In this evaluation, a detailed description of the impact of the handover on the service performance should also be given. Explain how the estimate was derived. |  | No answer is provided |
| A1.2.24.2 | For the proposed RTT, can handover cope with rapid decrease in signal strength (e.g. street corner effect)? Give a detailed description of: – the way the handover detected, initiated and executed, – how long each of this action lasts (minimum/maximum time (ms)), – the time-out periods for these actions. | Yes. A base station broadcasts the criterion which is being used for mobile station to request handover. The mobile station issues handover request message whenever the criterion is met. The handover criterion depends on the implementation but usually the received signal strength by a mobile station is used. Further information is available in the IEEE 802.16 standard; Section 11.1.7 MOB-NBR-ADV message encodings. | Very limited information provided on Handover. Details requested are not supplied |
| A1.2.25 | Characterize how the proposed RTT reacts to the system deployment (e.g. necessity to add new cells and/or new carriers) particularly in terms of frequency planning. | All base stations can use the same frequency or different frequency depending on the frequency reuse deployment scenario. OFDMA sub-channelization allows various permutations of sub-carriers. A distributed permutation of sub-carriers, e.g., PUSC (partial usage of sub-carrier) in this RTT, minimizes interferences from neighboring cells and/or sectors in case of the frequency reuse of 1. Different operators usually use different frequencies. |  |
| A1.2.26 | Sharing frequency band capabilities : to what degree is the proposal able to deal with spectrum sharing among IMT-2000 systems as well as with all other systems: – spectrum sharing between operators, – spectrum sharing between terrestrial and satellite IMT-2000 systems, – spectrum sharing between IMT-2000 and non-IMT-2000 systems, – other sharing schemes. | The proposed RTT utilizes OFDMA which has inherent interference protection capabilities due to allocation of a varying subset of available sub-carriers to different users. This capability, complemented by interference mitigation techniques described in Report ITU-R M.2045 such as use of appropriate filters and linear power amplifiers would ensure excellent potential for optimum spectrum sharing between the proposed RTT and other IMT-2000 systems. ITU-R WP 8F is in the process of performing sharing studies between fixed/nomadic and mobile IEEE 802.16 and IMT-2000. Preliminary results show similarities with the case of coexistence between IMT-2000 TDD and FDD technologies as captured in Reports ITU-R M.2030 and ITU-R M.2045. |  |
| A1.2.27 | Dynamic channel allocation : characterize the dynamic channel allocation (DCA) schemes which may be supported and characterize their impact on system performance (e.g. in terms of adaptability to varying interference conditions, adaptability to varying traffic conditions, capability to avoid frequency planning, impact on the reuse distance, etc.). | Various permutations of OFDMA sub-carriers enable dynamic usage of the spectrum among cells to balance the load and/or average interferences. |  |
| A1.2.28 | Mixed cell architecture : how well does the RTT accommodate mixed cell architectures (pico, micro and macrocells)? Does the proposal provide pico, micro and macro cell user service in a single licensed spectrum assignment, with handoff as required between them? (terrestrial component only). NOTE 1 – Cell definitions are as follows: – pico – cell hex radius: r  100 m – micro: 100 m  r  1 000 m – macro: r  1 000 m. | The proposed RTT can support flexible frequency reuse operation thus mixed cell architecture is supported well on the same or different frequencies depending on the implementation. |  |
| A1.2.29 | Describe any battery saver/intermittent reception capability. |  |  |
| A1.2.29.1 | Ability of the MS to conserve standby battery power : provide details about how the proposal conserves standby battery power. | The battery power saving of mobile station is supported by the sleep mode and the idle mode operations. Since the RTT basically provides packet-based transmission, both two modes operate in a slotted mode. In those modes, a mobile station communicates to its serving base station only in a listening interval and saves its power consumption otherwise. The information on listening, sleep and idle intervals are determined by the negotiation between the base station and the mobile station before the mobile station transits to either of two modes. A mobile station maintains the connection to its serving base station even in the sleep mode, while a mobile station in the idle mode returns system resources relevant to the existing connection to a base station. In latter case, the mobile station is managed by the multiple base stations grouped in a paging zone. Further information can be found in the IEEE 802.16 standard Sections 6.3.21, Sleep Mode, and 6.3.24, Idle Mode. |  |
| A1.2.30 | Signalling transmission scheme : if the proposed system will use RTTs for signalling transmission different from those for user data transmission, describe the details of the signalling transmission scheme over the radio interface between terminals and base (satellite) stations. | The same RTT is used for both user data and signaling transmission. |  |
| A1.2.30.1 | Describe the different signalling transfer schemes which may be supported, e.g. in connection with a call, outside a call. Does the RTT support: – new techniques? Characterize. – Signalling enhancements for the delivery of multimedia services? Characterize. | Flexible message-based signaling scheme is used. |  |
| A1.2.31 | Does the RTT support a bandwidth on demand (BOD) capability? BOD refers specifically to the ability of an end-user to request multi-bearer services. Typically, this is given as the capacity in the form of bits per second of throughput. Multi-bearer services can be implemented by using such technologies as multi-carrier, multi-time slot or multi-codes. If so, characterize these capabilities. NOTE 1 – BOD does not refer to the self-adaptive feature of the radio channel to cope with changes in the transmission quality (see § A1.2.5.1). | Yes. The scheduling service is provided for both downlink and uplink traffic. In order for the scheduler to make an efficient resource allocation and provide the desired QoS and data rate in the uplink, mobile stations must feedback accurate and timely information as to the traffic conditions and QoS requirements. To this end, multiple uplink bandwidth request mechanisms, such as bandwidth request through ranging channel, piggyback request and polling are provided to support uplink bandwidth requests. Frequency and time resource allocation in both downlink and uplink is on a per frame basis to duly react to the traffic and channel conditions. Additionally, the amount of resource in each allocation can range from one slot to the entire frame. Further information can be found in the IEEE 802.16 standard, Sections 6.3.6 Bandwidth Allocation and Request mechanism, 6.3.7.3 DL-MAP, 6.3.7.4 UL-MAP, and 8.4.4 Frame Structure. |  |
| A1.2.32 | Does the RTT support channel aggregation capability to achieve higher user bit rates? | No |  |
| A1.3 | Expected performances. |  |  |
| A1.3.1 | For terrestrial test environment only. |  |  |
| A1.3.1.1 | What is the achievable BER floor level (for voice)? NOTE 1 – The BER floor level is evaluated under the BER measuring conditions defined in Annex 2 using the data rates indicated in § 1 of Annex 2. | Coded BER floor is implementation-dependent but achievable floor is significantly below GoS requirements (10-3) within the specified ranges of tolerable delay spread (20 µs) and Doppler shifts (250Hz). | BER floor is not specified |
| A1.3.1.2 | What is the achievable BER floor level (for data)? NOTE 1 – The BER floor level is evaluated under the measuring conditions defined in Annex 2 using the data rates indicated in § 1 of Annex 2. | Coded BER floor is implementation-dependent but achievable floor is significantly below GoS requirements (10-6) within the specified ranges of tolerable delay spread (20 µs) and Doppler shifts (250 Hz). | BER floor is not specified This Doppler corresponds to approx. 120 km/hr. However, the simulation results included in the proposal only assumes 10% of the users at that speeds and the majority of users are at speeds of 30 km/hr or less. So the claim that the error floor will be below the required GoS requirements at those speeds and delay spreads is not substantiated. Additionally, no link curves or data from link level simulations are supplied. |
| A1.3.1.3 | What is the maximum tolerable delay spread (ns) to maintain the voice and data service quality requirements? NOTE 1 – The BER is an error floor level measured with the Doppler shift given in the BER measuring conditions of Annex 2. | The maximum specified range of delay spread (20 µsec in Vehicular B) can be tolerated without an equalizer. | This entry specifies that 20 sec delay spread can be tolerated without an equalizer while A1.2.14 specifies that an 11.4 µsec cyclic prefix and a 1-tap equalizer are used. How can 20 µsec delay spread be accommodated with a 11.2 µsec cyclic prefix? No data for Vehicular B is supplied. No link curves are supplied for Vehicular B channel model. |
| A1.3.1.4 | What is the maximum tolerable Doppler shift (Hz) to maintain the voice and data service quality requirements? NOTE 1 – The BER is an error floor level measured with the delay spread given in the BER measuring conditions of Annex 2. | At least 500 Hz, based on the observation that Doppler frequency shows about 570 Hz for 250 km/h at 2.5 GHz |  |
| A1.3.1.5 | Capacity : the capacity of the radio transmission technology has to be evaluated assuming the deployment models described in Annex 2 and technical parameters from § A1.2.22 through § A1.2.23.2. |  |  |
| A1.3.1.5.1 | What is the voice traffic capacity per cell (not per sector): provide the total traffic that can be supported by a single cell (E/MHz/cell) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) for FDD mode or contiguous bandwidth of 30 MHz for TDD mode. Provide capacities for all penetration values defined in the deployment model for the test environment in Annex 2. The procedure to obtain this value is described in Annex 2. The capacity supported by not a standalone cell but a single cell within contiguous service area should be obtained here. | See Section 2.3 (of 8F/1079R1) for details Voice capacity (ITU Vehicular path loss model, Pedestrian B 3 channel model): - 90 Erlangs/MHz/cell for reuse 3, SIMO,10 MHz PUSC - 80 Erlangs/MHz/cell for reuse 3, SIMO, 5 MHz PUSC | The Erlang capacity mentioned number does not make sense in the light of the number of VoIP users that can be supported provided in A1.2.17. What is the blocking probability? What is the average call duration? It appears that Erlangs capacity was calculated for uplink and downlink separately and then summed. The results appear to be overstated by at least a factor of 2. In addition, the overhead requirements for encryption do not appear to be considered. See Appendix 3. Results are given with a model using Vehicular path loss and Pedestrian B3 channel model which is mixing environment information. Results are to be given for each test environment indicated on the cover sheet (Indoor, Outdoor to Indoor Pedestrian, Vehicular). The deployment models specified in Annex 2 of M.1225 were not used in this analysis. |
| A1.3.1.5.2 | What is the information capacity per cell (not per sector): provide the total number of user-channel information bits which can be supported by a single cell (Mbit/s/MHz/cell) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) for FDD mode or contiguous bandwidth of 30 MHz for TDD mode. Provide capacities for all penetration values defined in the deployment model for the test environment in Annex 2. The procedure to obtain this value is described in Annex 2. The capacity supported by not a standalone cell but a single cell within contiguous service area should be obtained here. | See reference 0 (of 8F/1079R1) for details : Data capacity (PUSC, ITU Vehicular, 60% Pedestrian B 3, 30% Vehicular A 30, 10% Vehicular A 120, DL:UL=28:9) SIMO: 10 MHz DL = 3.57 Mbps/MHz/cell UL = 1.59 Mbps/MHz/cell 5 MHz DL = 3.45 Mbps/MHz/cell UL = 1.6 Mbps/MHz/cell MIMO: 10 MHz DL = 5.52 Mbps/MHz/cell UL = 2.10 Mbps/MHz/cell - - Beamforming technology increases the spectral efficiency of the system. | Results are given with a model using a mixed test environment which does not match the deployment model for the mixed test environment in M.1225. The non-mandated mix uses a different path loss model than that specified in M.1225. Furthermore individual test environment results are not supplied for each test environment indicated on the cover sheet (Indoor, Outdoor to Indoor Pedestrian, Vehicular). Channel B is to be used for this analysis Outage for this data needs to be supplied as it is questionable using the assumptions stated in the 8F/1079Rev 1. See Appendices 1, 2, and 5.. |
| A1.3.1.6 | Does the RTT support sectorization? If yes, provide for each sectorization scheme and the total number of user-channel information bits which can be supported by a single site (Mbit/s/MHz) (and the number of sectors) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) in FDD mode or contiguous bandwidth of 30 MHz in TDD mode. | Yes, the RTT supports sectorization. The sectorization and frequency reuse schemes are implementation-dependent and consequently, so are the capacities achieved. The tri-sector scheme is the typical scenario with frequency reuse 1 or reuse 3. |  |
| A1.3.1.7 | Coverage efficiency : the coverage efficiency of the radio transmission technology has to be evaluated assuming the deployment models described in Annex 2. |  |  |
| A1.3.1.7.1 | What is the base site coverage efficiency (km2/site) for the lowest traffic loading in the voice only deployment model? Lowest traffic loading means the lowest penetration case described in Annex 2. | See Link Budget in Section 2.3.4 (of 8F/1079R1) | Link budget constructed by the evaluation group indicates significant discrepancies from the link budgets given in 8F/1079 Rev 1. See Appendix 4. In addition, there do not appear to be any implementation losses included in the link budget calculations. |
| A1.3.1.7.2 | What is the base site coverage efficiency (km2/site) for the lowest traffic loading in the data only deployment model? Lowest traffic loading means the lowest penetration case described in Annex 2. | See Link Budget in Section 2.3.4 (of 8F/1079R1) | Link budget constructed by the evaluation group indicates significant discrepancies from the link budgets given in 8F/1079 Rev 1. See Appendix 4. In addition, there do not appear to be any implementation losses included in the link budget calculations. |
| A1.3.2 | For satellite test environment only |  |  |
| A1.3.2.1 | What is the required C/N0 to achieve objective performance defined in Annex 2? |  |  |
| A1.3.2.2 | What are the Doppler compensation method and residual Doppler shift after compensation? |  |  |
| A1.3.2.3 | Capacity : the spectrum efficiency of the radio transmission technology has to be evaluated assuming the deployment models described in Annex 2. |  |  |
| A1.3.2.3.1 | What is the voice information capacity per required RF bandwidth (bit/s/Hz)? |  |  |
| A1.3.2.3.2 | What is the voice plus data information capacity per required RF bandwidth (bit/s/Hz)? |  |  |
| A1.3.2.4 | Normalized power efficiency : the power efficiency of the radio transmission technology has to be evaluated assuming the deployment models described in Annex 2. |  |  |
| A1.3.2.4.1 | What is the supported information bit rate per required carrier power-to-noise density ratio for the given channel performance under the given interference conditions for voice? |  |  |
| A1.3.2.4.2 | What is the supported information bit rate per required carrier power-to-noise density ratio for the given channel performance under the given interference conditions for voice plus data? |  |  |
| A1.3.3 | Maximum user bit rate (for data) : specify the maximum user bit rate (kbit/s) available in the deployment models described in Annex 2. | The maximum bit rates are well above 20160 kbit/s. (DL/UL ratio = 2:1, PUSC, 64QAM, 5/6 coding rate) | Maximum user data rates for the different deployment models are not supplied. Supply maximum bit rates for maximum DL/UL ratio and maximum UL/DL ratio (assuming meeting deployment model) |
| A1.3.4 | What is the maximum range (m) between a user terminal and a BS (prior to hand-off, relay, etc.) under nominal traffic loading and link impairments as defined in Annex 2? | See Link Budget in Section 2.3 (of 8F/1079R1). The maximum range depends on the deployment and the QoS of a connection | Maximum range not supplied for the mandated deployment mode |
| A1.3.5 | Describe the capability for the use of repeaters. | Repeaters can be used. There is nothing in the technology that precludes the use of repeaters. |  |
| A1.3.6 | Antenna systems : fully describe the antenna systems that can be used and/or have to be used; characterize their impacts on systems performance, (terrestrial only); e.g., does the RTT have the capability for the use of: – remote antennas: describe whether and how remote antenna systems can be used to extend coverage to low traffic density areas; – distributed antennas: describe whether and how distributed antenna designs are used, and in which IMT-2000 test environments; – Smart antennas (e.g., switched beam, adaptive, etc.): describe how smart antennas can be used and what is their impact on system performance; – other antenna systems. | The air-interface does not place any restrictions on the types of antenna systems such as smart antenna technologies, including Beamforming, Transmit/Receive diversity and MIMO, as well as a combination of these like Beamforming plus MIMO. The uses of remote and distributed antennas are not precluded. |  |
| A1.3.7 | Delay (for voice) | Voice services are provided in the PS-domain with appropriate QoS setting (UGS, rtPS or ErtPS) |  |
| A1.3.7.1 | What is the radio transmission processing delay due to the overall process of channel coding, bit interleaving, framing, etc., not including source coding? This is given as transmitter delay from the input of the channel coder to the antenna plus the receiver delay from the antenna to the output of the channel decoder. Provide this information for each service being provided. In addition, a detailed description of how this parameter was calculated is required for both the uplink and the downlink. | The minimum delay is roughly 10ms assuming a 5ms TDD frame and the maximum is implementation and traffic load-dependent (scheduling metric, traffic load, buffer sizes, retransmission scheme etc) | What were the assumptions used to arrive at 10ms? Does this include any retransmission attempts? |
| A1.3.7.2 | What is the total estimated round trip delay (ms) to include both the processing delay, propagation delay (terrestrial only) and vocoder delay? Give the estimated delay associated with each of the key attributes described in Fig. 6 that make up the total delay provided. | Assuming a 20 ms vocoder, 5ms frame and ignoring queuing delay (typically <30ms), the RTD delay is approximately 60 ms | Information in conflict with information in A3.3.2. D1 and D2 delays need to be specified. |
| A1.3.7.3 | Does the proposed RTT need echo control? | Yes |  |
| A1.3.8 | What is the MOS level for the proposed codec for the relevant test environments given in Annex 2? Specify its absolute MOS value and its relative value with respect to the MOS value of ITU-T Recommendation G.711 (64 k PCM) and ITU-T Recommendation G.726 (32 k ADPCM). NOTE 1 – If a special voice coding algorithm is indispensable for the proposed RTT, the proponent should declare detail with its performance of the codec such as MOS level. (See § A1.2.19) | The RTT supports VoIP and is not limited to any particular codecs. Applications/implementations determine the choice of codec. | No specific vocoder used for the analysis is specified. No MOS comparison with G.711 or G.726 is supplied as required. |
| A1.3.9 | Description of the ability to sustain quality under certain extreme conditions. |  |  |
| A1.3.9.1 | System overload (terrestrial only) : characterize system behaviour and performance in such conditions for each test services in Annex 2, including potential impact on adjacent cells. Describe the effect on system performance in terms of blocking grade of service for the cases that the load on a particular cell is 125%, 150%, 175%, and 200% of full load. Also describe the effect of blocking on the immediate adjacent cells. Voice service is to be considered here. Full load means a traffic loading which results in 1% call blocking with the BER of 1 x 10–3 maintained. | The RTT provides many features that can be used to ensure optimal loading in the event of system overload. Among these are admission control, handover, rate adaptation, fractional frequency reuse and power control. | Blocking data is not supplied for 125%, 150%, 175%, and 200% of full load |
| A1.3.9.2 | Hardware failures : characterize system behaviour and performance in such conditions. Provide detailed explanation on any calculation. | This is implementation-dependent. The RTT does not preclude any means to build in redundancy or other reliability features. |  |
| A1.3.9.3 | Interference immunity : characterize system immunity or protection mechanisms against interference. What is the interference detection method? What is the interference avoidance method? | In addition to frequency reuse, and intelligent scheduling/RRM, the RTT’s TDD OFDM interface is inherently robust against delay spread, suitable for multi-user detection and supports various smart antenna schemes. Also, the RTT does not preclude any means to cancel interference or to protect against interference |  |
| A1.3.10 | Characterize the adaptability of the proposed RTT to different and/or time-varying conditions (e.g. propagation, traffic, etc.) that are not considered in the above attributes of § A1.3. | The RTT supports modulation and coding adaptation, HARQ, power control and opportunistic scheduling |  |
| A1.4 | Technology design constraints |  |  |
| A1.4.1 | Frequency stability : provide transmission frequency stability (not oscillator stability) requirements of the carrier (include long term – 1 year – frequency stability requirements (ppm)). |  |  |
| A1.4.1.1 | For BS transmission (terrestrial component only). | BS frequency tolerance ≤  2ppm of carrier frequency BS to BS frequency accuracy ≤  1% of subcarrier spacing |  |
| A1.4.1.2 | For MS transmission. | MS to BS frequency synchronization tolerance ≤ 2% of the subcarrier spacing |  |
| A1.4.2 | Out-of-band and spurious emissions : specify the expected levels of base or satellite and mobile transmitter emissions outside the operating channel, as a function of frequency offset. | Base stations and terminals supporting this RTT will comply with local, regional, and international regulations for out of band and spurious emissions, wherever applicable. Similar to other IMT-2000 RTTs, terminals adhering to a single global mask will be used to provide global roaming. | Out of band and spurious emissions are not specified |
| A1.4.3 | Synchronisation requirements : describe RTT’s timing requirements, e.g. – Is BS-to-BS or satellite land earth station (LES)-to-LES synchronisation required? Provide precise information, the type of synchronisation, i.e., synchronisation of carrier frequency, bit clock, spreading code or frame, and their accuracy. – Is BS-to-network synchronisation required? (terrestrial only). – State short-term frequency and timing accuracy of BS (or LES) transmit signal. – State source of external system reference and the accuracy required, if used at BS (or LES) (for example: derived from wireline network, or GPS receiver). – State free run accuracy of MS frequency and timing reference clock. – State base-to-base bit time alignment requirement over a 24 h period (µs). | BS-to-BS synchronisation : Yes. All BSs should be time and frequency synchronized to a common source signal. The common source signal is typically provided by GPS. BS-to-network synchronisation: No. BS-to-network synchronisation is not required. Frequency accuracy : BS frequency tolerance ≤  2ppm of carrier frequency Timing accuracy ≤ 1usec compared to reference timing. Source of external system reference and the accuracy : GPS (the synchronizing reference shall be a 1 ps timing pulse and a 10 MHz frequency reference) Free run accuracy : MS frequency tolerance ≤ maximum 2% of the subcarrier spacing Timing tolerance: 25% of minimum guard interval( (Tb/32)/4) The BS's timing accuracy is required to be 1 μs compared to reference timing when GPS locked. | Achieving a MS free run timing accuracy of 2% of subcarrier spacing is difficult and adds complexity. |
| A1.4.4 | Timing jitter : for BS (or LES) and MS give: – the maximum jitter on the transmit signal, – the maximum jitter tolerated on the received signal. Timing jitter is defined as r.m.s. value of the time variance normalized by symbol duration. | BS The BS's timing accuracy is required to be 1μsec compared to reference timing. MS MS Transmit symbol timing accuracy within ± (Tb/32)/4 |  |
| A1.4.5 | Frequency synthesizer : what is the required step size, switched speed and frequency range of the frequency synthesizer of MSs? | Frequency step size : 200 and 250 KHz Switched speed : 200 μs Frequency range : 5, 10 MHz Start frequencies are various, depending on channel bandwidth and profile | Frequency range of the synthesizer is specified to be 5 or 10 MHz, however it is unreasonable to have a synthesizer range be the same as the Bandwidth of the technology. Clarify step size. |
| A1.4.6 | Does the proposed system require capabilities of fixed networks not generally available today? | No |  |
| A1.4.6.1 | Describe the special requirements on the fixed networks for the handover procedure. Provide handover procedure to be employed in proposed RTT in detail. | The RTT supports handover and also provides means for expediting handover. Each base station broadcasts the information on the list of neighboring base stations and their channel information such as the operating center frequency, preamble index and synchronization periodically. The channel information in this broadcasting is used for a mobile station to synchronize with the neighboring base station. After a mobile station monitors the signal strength of a neighboring base station and seeks suitable base station(s) for handover, the mobile station or its serving base station can initiate handover by handover request message. But only the mobile station can transmit handover indication message to the its serving base station. After transmitting handover indication message, the mobile station stops monitoring the downlink frame of its serving base station and performs network re-entry to target base station. To reduce the handover latency further, the serving base station provides the target base station with network entry information on a mobile station to be handed over the target base station. |  |
| A1.4.7 | Fixed network feature transparency |  |  |
| A1.4.7.1 | Which service(s) of the standard set of ISDN bearer services can the proposed RTT pass to users without fixed network modification. | Convergence Sublayer in the proposed RTT supports interface to various fixed networks such as ATM, Ethernet, IP, and VLAN. | Question not answered as to ISDN services provided |
| A1.4.8 | Characterize any radio resource control capabilities that exist for the provision of roaming between a private (e.g., closed user group) and a public IMT-2000 operating environment. | Handover between the different access networks is basically supported. Furthermore, Operator ID in the signalling during the handover enable mobile stations to recognize the operator of access network they are handed over to. | Information concerning radio resource control capabilities for provision of roaming between private and public IMT-2000 systems is not provided. |
| A1.4.9 | Describe the estimated fixed signalling overhead (e.g., broadcast control channel, power control messaging). Express this information as a percentage of the spectrum which is used for fixed signalling. Provide detailed explanation on your calculations. | The fixed MAP overhead is typically about 10% in a 10 MHz channel with a 5ms frame size. |  |
| A1.4.10 | Characterize the linear and broadband transmitter requirements for BS and MS (terrestrial only). | BS - Tx dynamic Range = 10 dB - Spectral flatness according to the following: ≤ ±2 dB for spectral lines from – Nused/4 to 1 and +1 to Nused/4 Within +2/-4 dB for spectral lines from – Nused/2 to Nused/4 and +Nused/4 to Nused/2 - Per sub-carrier flatness ≤ 0.1 dB - Power difference between adjacent subcarriers according to the following: Tx downlink radio frame shall be time-aligned with the 1pps timing pulse within 1 μsec - Tx relative constellation error according to the following: QPSK-1/2 ≤ -15.0 dB QPSK-3/4 ≤ -18.0 dB 16QAM-1/2 ≤ -20.5 dB 16QAM-3/4 ≤ -24.0 dB 64QAM-1/2 (if 64-QAM supported) ≤ -26.0 dB 64QAM-2/3 (if 64-QAM supported) ≤ -28.0 dB 64QAM-3/4 (if 64-QAM supported)≤ -30.0 dB |  |
|  |  | MS - Tx dynamic Range = 45 dB - Tx power level min adjustment step = 1 dB - Tx power level min relative step accuracy = ± 0.5 dB - Spectral flatness according to the following: ≤ ±2 dB for spectral lines from –Nused/4 to –1 and +1 to Nused/4 Within +2/-4 dB for spectral lines from –Nused/2 to –Nused/4 and +Nused/4 to Nused/2 - Power difference between adjacent subcarriers ≤ 0.1 dB - Tx relative constellation error according to the following: QPSK-1/2 ≤ -15.0 dB QPSK-3/4 ≤ -18.0 dB 16QAM-1/2 ≤ -20.5 dB 16QAM-3/4 ≤ -24.0 dB |  |
| A1.4.11 | Are linear receivers required? Characterize the linearity requirements for the receivers for BS and MS (terrestrial only). | BS No. The PAPR of the proposed RTT is around 12dB, and which is not required a stringent linear receiver. | Receiver linearity requirements are not specified In conflict with A3.6.7 and A3.2.2.4 No MS information is provided. |
| A1.4.12 | Specify the required dynamic range of receiver (terrestrial only). | BS Max input level on-channel reception tolerance = -45 dBm Max input level on-channel damage tolerance = -10 dBm MS Max input level on-channel reception tolerance = -30 dBm Max input level on-channel damage tolerance = 0 dBm BS and MS Max input level sensitivity (Distributed permutation of subcarriers) for 10 MHz case: -88.5 dBm - QPSK-1/2 -85.1 dBm - QPSK-3/4 -82.8 dBm - 16QAM-1/2 -78.7 dBm - 16QAM-3/4 -77.6 dBm - 64QAM-1/2 -74.5 dBm - 64QAM-2/3 -73.4 dBm - 64QAM-3/4 -71.5 dBm - 64QAM-5/6 |  |
|  |  | Max input level sensitivity (Distributed permutation of subcarriers) for 5 MHz case: -91.5 dBm - QPSK-1/2 -88.1 dBm - QPSK-3/4 -85.8 dBm - 16QAM-1/2 -81.7 dBm - 16QAM-3/4 -80.6 dBm - 64QAM-1/2 -77.5 dBm - 64QAM-2/3 -76.4 dBm - 64QAM-3/4 -74.5 dBm - 64QAM-5/6 Sensitivity numbers are calculated based on assumption of repetition factor 1 and Distributed permutation of subcarriers. |  |
| A1.4.13 | What are the signal processing estimates for both the handportable and the BS? – MOPS (millions of operations per second) value of parts processed by DSP (digital signal processing), – gate counts excluding DSP, – ROM size requirements for DSP and gate counts (kbytes), – RAM size requirements for DSP and gate counts (kbytes). NOTE 1 – At a minimum the evaluation should review the signal processing estimates (MOPS, memory requirements, gate counts) required for demodulation, equalization, channel coding, error correction, diversity processing (including Rake receivers), adaptive antenna array processing, modulation, A-D and D-A converters and multiplexing as well as some IF and baseband filtering. For new technologies, there may be additional or alternative requirements (such as FFTs etc.). NOTE 2 – The signal processing estimates should be declared with the estimated condition such as assumed services, user bit rate and etc. | It is an implementation issue not covered by the description. | Signal processing estimates are not provided |
| A1.4.14 | Dropped calls : describe how the RTT handles dropped calls. Does the proposed RTT utilize a transparent reconnect procedure – that is, the same as that employed for handoff? | No specific process to handle call dropping recovery is defined. However, mobile station can recover the connection after call dropping by means of the Idle mode re-entry procedure. |  |
| A1.4.15 | Characterize the frequency planning requirements: – frequency reuse pattern: given the required C/I and the proposed technologies, specify the frequency cell reuse pattern (e.g. 3-cell, 7-cell, etc.) and, for terrestrial systems, the sectorization schemes assumed; – characterize the frequency management between different cell layers; – does the RTT use an interleaved frequency plan? – are there any frequency channels with particular planning requirements? – all other relevant requirements. NOTE 1 – The use of the second adjacent channel instead of the adjacent channel at a neighbouring cluster cell is called “interleaved frequency planning”. If a proponent is going to employ an interleaved frequency plan, the proponent should state so in § A1.2.4 and complete § A1.2.15 with the protection ratio for both the adjacent and second adjacent channel. | The RTT supports frequency reuse configuration of 1 and 3. In order for MS to provide BS with a correct DL channel quality information, MS is required to properly measure CINR of preamble with considering the frequency reuse configuration: i.e. For frequency reuse of 3, consider the modulated subcarriers of the preamble only. For frequency reuse of 1, consider both the un-modulated and the modulated subcarriers of the preamble. There are 114 different preamble code sets in the proposed RTT to differentiate the cell ID and sector ID's per each sector. The RTT can use both the interleaved frequency plan and the non-interleaved frequency plan. |  |
| A1.4.16 | Describe the capability of the proposed RTT to facilitate the evolution of existing radio transmission technologies used in mobile telecommunication systems migrate toward this RTT. Provide detail any impact and constraint on evolution. |  | Question on evolution of existing technologies is not answered |
| A1.4.17 | Are there any special requirements for base site implementation? Are there any features which simplify implementation of base sites? (terrestrial only) | No | RTT specifies GPS synchronization of Base stations. This is not specified in this entry. |
| A1.5 | Information required for terrestrial link budget template Proponents should fulfil the link budget template given in Table 6 and answer the following questions. | see Section 2.3 (of 8F/1079R1)Link Budget | Data duty cycle need to be taken into account for link budget computation in 8F/1079R1. Please justify -171 dBm/Hz noise + interference level. Please provide link curves with channel estimation error. Please justify log-normal fade margin of 5.56 dB. See Appendix 4 |
| A1.5.1 | What is the BS noise figure (dB)? | 4 dB used for Section 2.3 (of 8F/1079R1) Link Budget Max 8 dB Noise Figure is considered in RTT. |  |
| A1.5.2 | What is the MS noise figure (dB)? | 7 dB see Section 2.3 (of 8F/1079R1)Link Budget Max 8 dB Noise Figure is considered in RTT for MS clients supporting multi band operation. |  |
| A1.5.3 | What is the BS antenna gain (dBi)? | 15 dBi (see Section 2.3 (of 8F/1079R1)Link Budget) |  |
| A1.5.4 | What is the MS antenna gain (dBi)? | -1 dBi (see Section 2.3 (of 8F/1079R1)Link Budget) |  |
| A1.5.5 | What is the cable, connector and combiner losses (dB)? | 0 dB (see Section 2.3 (of 8F/1079R1)Link Budget) | Cable and connector loss for BS is not specified |
| A1.5.6 | What are the number of traffic channels per RF carrier? | Function of required QoS |  |
| A1.5.7 | What is the RTT operating point (BER/FER) for the required Eb/N0 in the link budget template? | 1% FER |  |
| A1.5.8 | What is the ratio of intra-sector interference to sum of intra-sector interference and inter-sector interference within a cell (dB)? | Depends on environment and receiver implementation | Ratio of intra-sector interference is not provided |
| A1.5.9 | What is the ratio of in-cell interference to total interference (dB)? | Negligible at low Doppler (<300 Hz) and depends on receiver implementation at high Doppler | Ratio of in-cell interference to total interference is not specified. Subcarrier frequency errors could be a source of in-cell interference |
| A1.5.10 | What is the occupied bandwidth (99%) (Hz)? | Depends on nominal bandwidth, permutation scheme, and on the subchannelization. For the case considered in Section 2.3 (of 8F/1079R1) Link Budget, it is approximately 9.2 MHz on the downlink and 2.4 MHz on the uplink |  |
| A1.5.11 | What is the information rate (dBHz)? | Depends on service rate with the maximum subject to the channel bandwidth employed. (see Section 2.3 (of 8F/1079R1) Link Budget) | Maximum information rate is not supplied |
| A1.6 | Satellite system configuration (applicable to satellite component only): Configuration details in this subsection are not to be considered as variables. They are for information only. |  | Not Applicable |
| A1.6.1 | Configuration of satellite constellation |  | Not Applicable |
| A1.6.1.1 | GSO, HEO, MEO, LEO or combination? |  | Not Applicable |
| A1.6.1.2 | What is the range of height where satellites are in active communication? |  | Not Applicable |
| A1.6.1.3 | What is the orbit inclination angle? |  | Not Applicable |
| A1.6.1.4 | What are the number of orbit planes? |  | Not Applicable |
| A1.6.1.5 | What are the number of satellites per orbit plane? |  | Not Applicable |
| A1.6.2 | What is the configuration of spot beams/cell layout pattern? |  | Not Applicable |
| A1.6.3 | What is the frequency reuse plan among spot beams? |  | Not Applicable |
| A1.6.4 | What is the service link G/T of satellite beam (average, minimum)? |  | Not Applicable |
| A1.6.5 | What is the service link saturation e.i.r.p. of each beam (average, minimum), when configured to support ‘hot spot’? |  | Not Applicable |
| A1.6.6 | What is the service link total saturation e.i.r.p. per satellite? |  | Not Applicable |
| A1.6.7 | Satellite e.i.r.p. per RF carrier for satellite component. |  | Not Applicable |
| A1.6.7.1 | What is the maximum peak e.i.r.p. transmitted per RF carrier? |  | Not Applicable |
| A1.6.7.2 | What is the average e.i.r.p. transmitted per RF carrier? |  | Not Applicable |
| A1.6.8 | What is the feeder link information? |  | Not Applicable |
| A1.6.9 | What is the slot timing adjustment method (mainly applicable to TDMA system)? |  | Not Applicable |
| A1.6.10 | What is the satellite diversity method, if applicable? |  | Not Applicable |

# Summary of IMT-2000 Requirements and Objectives and Compliance Template

Attachment 4 to Circular-letter 8/LCCE/47

This attachment highlights the requirements and objectives for IMT-2000 that will be used in the evaluation of radio transmission technologies for IMT-2000. For the sake of brevity, the requirements and objectives are summarized in topical form and in some cases, several individual sections from the source Recommendation have been consolidated into one topic. (A complete listing of all existing Recommendations and Task Group 8/1 documents for IMT-2000 reviewed in preparing this summary is provided in Attachment 5 to this letter). The reader is encouraged to use the source documents to assess whether a candidate set of radio transmission technologies (SRTT) meets the stated requirements and objectives.

## Introduction to IMT-2000

The concept of a small, light weight and convenient pocket communicator is a fundamental part of IMT-2000, and provides terminal mobility which is complementary to the personal mobility and service profile management provided by Universal Personal Telecommunication (UPT), under study in the Telecommunication Standardization Sector (ITU-T).

A number of different radio environments are involved in covering the range from very small indoor cells with high capacity all the way through large outdoor terrestrial cells and on to satellite coverage. A major focus of the ITU-R standards work on IMT-2000 is to maximize the commonality between the various radio interfaces involved in order to simplify the task of building multi-mode mobile terminals which may be needed to cover more than one operating environment.

An important part of the ITU-R studies on IMT-2000 is the potential for these new mobile radio technologies to provide cost effective and flexible access to the global telecommunications networks in developing countries and under-developed parts of developed countries.

The integrated terrestrial and satellite components of IMT-2000 are complementary in terms of service provision. Together they cover the wide range of user densities, service types, and available service sets which comprise IMT-2000. Each component has particular advantages and constraints.

The terrestrial component provides, economically, high quality telecommunication services typically to areas of high to very high user densities. The satellite component provides users with quality telecommunication services primarily on a virtually global coverage basis, and is most economic outside those areas covered by the terrestrial component. Additionally to providing this global coverage, the satellite component may, in more densely populated areas, precede and encourage later coverage by the terrestrial component.

IMT-2000 are third generation systems which aim to advance and unify the diverse systems we see in the mid 1990s into a radio infrastructure capable of offering a wide range of services around the year 2000 in many different environments. The key features and objectives of IMT-2000 as compared to pre-IMT-2000 (which refer to mobile services that are currently in service or will be introduced prior to IMT-2000), are as follows:

### Global System

•	a global standard promoting a high degree of commonality of design worldwide while incorporating a variety of systems;

•	use of a small pocket terminal worldwide, but also the accommodation of a variety of other terminal types;

•	bigger marketplace leading to lower costs;

•	worldwide common frequency band;

•	worldwide roaming based on terminal mobility;

•	worldwide, off-the-shelf compatible equipment.

### New Services and Capabilities

•	provision of capability which enables new voice and data services which are significantly more advanced than pre-IMT-2000 technologies;

•	availability to mobile users of a range of voice and non-voice services, including packet data and multimedia services;

•	higher service quality, in particular voice;

•	high quality and integrity, comparable to the fixed network;

•	significantly higher user bit rate capability;

•	flexible radio bearer;

•	the capability to provide bandwidth on demand supporting a wide range of data rates, from simple low rate paging messages through voice to high rates associated with video or file transfer;

•	support for asymmetrical data capabilities which require high rates in one direction but much lower rates in the other;

•	improved security;

•	improved ease of operation;

•	intelligent network (IN) based service creation and service profile management based on ITU-T Q.1200-series of Recommendations;

•	coherent systems management based on ITU-T M.3000-series of Recommendations.

### Evolution and Migration

•	flexibility for evolution of systems, and migration of users, both from pre-IMT-2000 and evolution within IMT-2000;

•	compatibility of services within IMT-2000 and with the fixed telecommunications network (e.g., PSTN/ISDN);

•	provision of a framework for the continuing expansion of mobile network services and access to services and facilities of the fixed network;

•	an open architecture which will permit easy introduction of advances in technology and of different applications;

•	ability to coexist and interwork with pre-IMT-2000.

### Flexibility: Multi-Environment Capabilities

•	accommodation of a maximum level of interworking between networks of different types to provide customers with greater coverage, seamless roaming and consistency of services;

•	integrated satellite/terrestrial networks;

•	provision of services by more than one network in any coverage area;

•	provision of these services over a wide range of user densities and coverage areas;

•	provision of services to both mobile and fixed users in urban, rural and remote regions;

•	wider range of operating environments, including aeronautical and maritime;

•	a modular structure which will allow the system to start from as small and simple a configuration as possible and grow as needed, in size and complexity;

•	caters to needs of developing countries;

•	flexibility to utilize adaptive software downloadable terminals to support multiband and multi-environment capabilities;

•	key parameters of bandwidth, transmission quality and delay can be selected, negotiated, mixed and matched by the requirements of the service according to the instantaneous capability of the radio channel;

•	better (e.g., more efficient) use of the radio spectrum than pre-IMT-2000 consistent with providing services at acceptable costs, taking into account their differing demands for data rates, symmetry, channel quality, and delay.

IMT-2000 supports a wide range of services based on those of the fixed telecommunication network and those specific to mobile users. IMT-2000 users will in most circumstances not notice the fact that a radio link is used to connect their terminal to the world's telecommunication network.

Services range from basic wide area paging, through voice telephony (probably the prime requirement of the personal terminal), digital data services, to audio and visual communications. One of the overall service objectives of IMT-2000 is to enable the provision of simultaneous voice, data and video (i.e. multimedia). Requirements for the radio transmission technologies must therefore take into account the support of multimedia services. The actual services obtained by the user will depend on the terminal capabilities, the set of services subscribed to, and the services set provided by the relevant network operator. Services requiring high transmission rates are most likely to be found in high density areas, such as business centres.

The different service aspects and service environments for IMT-2000, and the importance of making the best use possible of the limited radio spectrum, call for a very flexible solution to the radio interface and its interworking with the transmission technology used in the fixed networks (e.g. "resource on demand" in a dynamic way, within fractions of a second). Broadband ISDN (B-ISDN) technologies will also be at an advanced stage when IMT-2000 is introduced.  Therefore, B-ISDN access and interworking and the flexibility of Asynchronous Transfer Mode (ATM) transmission technology are taken into account and efficient interworking with ATM is defined. IMT-2000 also need to be interconnected to many earlier analogue and digital fixed networks.

Work on IMT-2000 has already aided the development of second generation systems and it will provide significant advances beyond them. These include high quality advanced personalized telecommunications, support of worldwide roaming, a wide range of services and radio environments, highly efficient use of radio and network resources and cost-efficient overall network operation.

## IMT-2000 Requirements and Objectives Compliance Template

The following tables divide the summary of the requirements and objectives for IMT-2000, which will be used in the evaluation of radio transmission technologies, into three categories. Table 1 provides technical requirements and objectives for which a performance metric has been quantified. Table 2 provides generic requirements and objectives which specify functionality or a qualitative performance attribute. Requirements and objectives which require a subjective assessment or for which additional specificity is required to determine conformance are provided in Table 3.

Any of these requirements and objectives may apply to either the terrestrial or satellite component of IMT-2000, or both.  The tables indicate whether each item is an objective or a requirement, as well as the source ITU Recommendations, which should be used for assessing whether the stated requirements and objectives are met.

Those who respond with a proposal should indicate whether the candidate set of radio transmission technologies (SRTT) meets the IMT-2000/FPLMTS requirement or objective by checking the appropriate box in Tables 1 and 2.  Comments may be attached as an aid in providing additional information regarding the SRTT performance.  With respect to the subjective requirements or objectives in Table 3, descriptive information should be provided explaining how the candidate SRTT supports the concept specified in the Recommendation.

### List of acronyms used in the Tables

ATM		Asynchronous Transfer Mode

DTMF		Dual Tone Multi Frequency

EMC		Electromagnetic Compatibility

FPLMTS	Future Public Land Mobile Telecommunications Systems

IMT-2000	International Mobile Telecommunications-2000

ISDN		Integrated Services Digital Network

B-ISDN	Broadband Integrated Services Digital Network

PSTN		Public Switched Telephone Network

UPT		Universal Personal Telecommunications

WARC		World Administrative Radio Conference

- Table 1

Technical Requirements and Objectives Relevant to the Evaluation of Candidate Radio Transmission Technologies

| ITEM Ref | IMT-2000 Item Description | Obj /Req | Source | Proponents Meets?* | Evaluators Meets? | Annex 1/3 Reference | Evaluators’ Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Voice and data performance requirements |  |  |  |  |  |  |  |
| T1.1 | One-way end to end delay less than 40 ms** | Req | G.174, § 7.5 |  Yes  No |  Yes  No | A1.3.7.2 A3.3.2 | No information supplied to show that you can achieve 40 ms delay with VoIP A3.3.2 states that one-way delay is 70 ms. |
| T1.2 | For mobile videotelephone services, the IMT-2000 terrestrial component should operate so that the maximum overall delay (as defined in ITU-T Rec. F.720) should not exceed 400 ms, with the one way delay of the transmission path not exceeding 150 ms | Req | Suppl. F.720, F.723, G.114 |  Yes  No |  Yes  No |  | Again, no information supplied to show that you can achieve these delays. |
| T1.3 | Speech quality should be maintained during £ 3% frame erasures over any 10 second period. The speech quality criterion is a reduction of £ 0.5 mean opinion score unit (5 point scale) relative to the error-free condition (G.726 at 32 kb/s) | Req | G.174, § 7.11 & M.1079 § 7.3.1 |  Yes  No |  Yes  No | A1.2.19 A1.3.8 A3.3.7 | Voice quality is not demonstrated. Specifically, data to verify the G.726 error free performance and no more than 0.5 MOS degradation with 3% frame error rate is not supplied. |
| T1.4 | DTMF signal reliable transport (for PSTN is typically less than one DTMF errored signal in 104) | Req | G.174, § 7.11 & M.1079 § 7.3.1 |  Yes  No |  Yes  No |  | No Information to support yes is provided |
| T1.5 | Voiceband data support including G3 facsimile | Req | M.1079 § 7.2.2 |  Yes  No |  Yes  No |  | No information to support yes is provided |
| T1.6 | Support packet switched data services as well as circuit switched data; requirements for data performance given in ITU-T G.174 | Req | M.1034-1 § 10.1.5, 10.2.4 |  Yes  No See note |  Yes  No | A1.2.20 | Note 1 specifically states that circuit switched data is not supported |
| Radio interfaces and subsystems, network related performance requirements |  |  |  |  |  |  |  |
| T1.7 | Network interworking with PSTN and ISDN in accordance with Q.1031 and Q.1032 | Req | M.687-2 § 5.4 |  Yes  No |  Yes  No | A1,4,7.1 A3.5.3.1 | Information in A3.5.3.1 states that ISDN is not natively supported |
| T1.8 | Meet spectral efficiency and radio channel performance requirements of M.1079 | Req | M.1034 –1§ 11.3.3/4 |  Yes  No |  Yes  No |  |  |
| T1.9 | Provide phased approach with data rates up to 2 Mbit/s in phase 1 | Obj | M.687-2, § 1.1.6 |  Yes  No |  Yes  No |  |  |
| T1.10 | Maintain bearer channel bit-count integrity (e.g. synchronous data services and many encryption techniques) | Obj | M.1034-1, § 10.2.5 |  Yes  No |  Yes  No |  |  |
| T1.11 | Support for different cell sizes, for example - Mega cell Radius ~100-500 km Macro cell Radius £ 35 km, Speed £ 500 km/h Micro cell Radius £ 1 km, Speed £ 100 km/h Pico cell Radius £ 50m, Speed £10 km/h | Obj | M.1035 § 10.1 |  Yes  No |  Yes  No | A1.3.1.3 A3.4.1.4 A3.4.2.2 | No information provided to show that you can support a large cell considering the amount of delay spread that must be dealt with Again 500 km/hr, no data provided, ability to do channel estimation at those speeds is questionable. See comments in A3.4.1.4 |
| Application of IMT-2000 for fixed services and developing countries |  |  |  |  |  |  |  |
| T1.12 | Circuit noise - idle noise levels in 99% of the time about 100 pWp | Obj | M.819-2, § 10.3 |  Yes  No |  Yes  No |  | No information provided |
| T1.13 | Error performance - as specified in ITU-R F.697 | Obj | M.819-2, § 10.4 |  Yes  No |  Yes  No |  | No information provided |
| T1.14 | Grade of service better than 1% | Obj | M.819-2, § 10.5 |  Yes  No |  Yes  No | A1.3.9.1 A1.4.14 A3.3.1 | No information provided to support this. Blocking metric used in submission was 2% |

- Table 2

Generic Requirements and Objectives Relevant to the Evaluation of Candidate Radio Transmission Technologies

|  | IMT-2000 Item Description | Obj /Req | Source | Proponents Meets?* | Evaluators Meets? | Annex 1/3 Reference | Evaluators’ Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Radio interfaces and subsystems, network related performance requirements |  |  |  |  |  |  |  |  |
| T2.1 | Security comparable to that of PSTN/ISDN | Obj | M.687-2 § 4.4 |  Yes  No |  Yes  No | A1.4.7.1 | 8F/1079 Rev 1 Section 1.4.7 states security capabilities including AES encryption |  |
| T2.2 | Support mobility, interactive and distribution services | Req | M.816-1 § 6 |  Yes  No |  Yes  No | A3.4.1.7 |  |  |
| T2.3 | Support UPT and maintain common presentation to users | Obj | M.816 § 4 |  Yes  No No assessment supplied |  Yes  No |  | No answer was supplied |  |
| T2.4 | Voice quality comparable to the fixed network (applies to both mobile and fixed service) | Req | M.819-2 Table 1, M.1079 § 7.1 |  Yes  No |  Yes  No | A3.3.7 | Voice quality is not demonstrated. Specifically, data to verify the G.726 error free performance and no more than 0.5 MOS degradation with 3% frame error rate is not supplied. |  |
| T2.5 | Support encryption and maintain encryption when roaming and during handover | Req | M.1034-1 § 10.3.1/2 |  Yes  No |  Yes  No |  | No information provided to support the answer that encryption can be maintained during handover or roaming. |  |
| T2.6 | Network access indication similar to PSTN (e.g. dialtone) | Req | M.1034-1 § 10.1.9 |  Yes  No See note |  Yes  No |  | No information provided to support answer |  |
| T2.7 | Meet safety requirements and legislation | Req | M.1034-1 § 10.6.1 |  Yes  No |  Yes  No |  | Not clear from the proposal whether they can meet the E911 requirements. |  |
| T2.8 | Meet appropriate EMC regulations | Req | M.1034-1 § 10.6.2 |  Yes  No |  Yes  No |  |  |  |
| T2.9 | Support multiple public/private/ residential IMT-2000 operators in the same locality | Req | M.1034-1 § 11.1.2 |  Yes  No |  Yes  No | A1.4.8 |  |  |
| T2.10 | Support multiple mobile station types | Req | M.1034-1 § 11.1.4 |  Yes  No |  Yes  No |  |  |  |
| T2.11 | Support roaming between IMT-2000 operators and between different IMT-2000 radio interfaces/ environments | Req | M.1034-1 § 11.2.2 |  Yes  No |  Yes  No |  | No information supplied to show that the RTT can do that |  |
| T2.12 | Support seamless handover between different IMT-2000 environments such that service quality is maintained and signalling is minimized | Req | M.1034-1 § 11.2.3 |  Yes  No |  Yes  No | A1.2.24, A1.4.6.1 A3.2.10, A3.3.3, A3.3.4 |  |  |
| T2.13 | Simultaneously support multiple cell sizes with flexible base location, support use of repeaters and umbrella cells as well as deployment in low capacity areas | Req | M.1034-1 § 11.2.5.1/2/3/6 |  Yes  No |  Yes  No | A1.2.28 A1.3.5 |  |  |
| T2.14 | Support multiple operator coexistence in a geographic area | Req | M.1034-1 § 11.2.5.4 |  Yes  No |  Yes  No |  | No information on guard bands and out of band emissions has been provided to justify ability to coexist |  |
| T2.15 | Support different spectrum and flexible band sharing in different countries including flexible spectrum sharing between different IMT-2000 operators (see M.1036) | Req | M.1034-1 § 11.2.8.1/2 |  Yes  No |  Yes  No | A1.2.26, A3.4.2.2.1, A3.4.2.2.2 | No information supplied to show that the RTT can do that |  |
| T2.16 | Support mechanisms for minimizing power and interference between mobile and base stations | Req | M.1034-1 § 11.2.8.3 |  Yes  No |  Yes  No | A1.3.9.3 | RTT does not describe any interference management scheme |  |
| T2.17 | Support various cell types dependent on environment (M.1035 § 10.1) | Req | M.1034-1 § 11.2.9 |  Yes  No |  Yes  No |  |  |  |
| T2.18 | High resistance to multipath effects | Req | M.1034-1 § 11.3.1 |  Yes  No |  Yes  No | A1.2.14, A3.4.2.2.2 | Delay spreads larger than the Cyclic Prefix will cause a problem. It is not clear that Vehicular B channel model can be accommodated. Severity of the problem will depend on the power delay profile of the multipath (e.g. Veh B). In any case the resistance is not high in this instance. |  |
| T2.19 | Support appropriate vehicle speeds (as per § 7) NOTE: applicable to both terrestrial and satellite proposals | Req | M.1034-1 § 11.3.2 |  Yes  No |  Yes  No |  |  |  |
| T2.20 | Support possibility of equipment from different vendors | Req | M.1034-1 § 11.1.3 |  Yes  No |  Yes  No |  |  |  |
| T2.21 | Offer operational reliability as least as good as 2nd generation mobile systems | Req | M.1034-1 § 11.3.5 |  Yes  No |  Yes  No |  |  |  |
| T2.22 | Ability to use terminal to access services in more than one environment, desirable to access services from one terminal in all environments | Obj | M.1035 § 7.1 |  Yes  No |  Yes  No |  |  |  |
| T2.23 | End-to-end quality during handover comparable to fixed services | Obj | M.1034-1 § 11.2.3.4 |  Yes  No |  Yes  No | A1.2.24, A3.3.3, A3.3.4 | No Information to assess. |  |
| T2.24 | Support multiple operator networks in a geographic area without requiring time synchronization | Obj |  |  Yes  No |  Yes  No |  | The RTT describes a TDD system which requires synch. |  |
| T2.25 | Layer 3 contains functions such as call control, mobility management and radio resource management some of which are radio dependent. It is desirable to maintain layer 3 radio transmission independent as far as possible | Obj | M.1035 § 8 |  Yes  No |  Yes  No |  | No Information to assess. |  |
| T2.26 | Desirable that transmission quality requirements from the upper layer to physical layers be common for all services | Obj | M.1035 § 8.1 |  Yes  No |  Yes  No |  |  |  |
| T2.27 | The link access control layer should as far as possible not contain radio transmission dependent functions | Obj | M.1035 § 8.3 |  Yes  No |  Yes  No |  |  |  |
| T2.28 | Traffic channels should offer a functionally equivalent capability to the ISDN B-channels | Obj | M.1035 § 9.3.2 |  Yes  No |  Yes  No | A1.4.7.1, A3.5.3.1 | Information in A3.5.3.1 states that ISDN is not natively supported |  |
| T2.29 | Continually measure the radio link quality on forward and reverse channels | Obj | M.1035 § 11.1 |  Yes  No |  Yes  No |  |  |  |
| T2.30 | Facilitate the implementation and use of terminal battery saving techniques | Obj | M.1035 § 12.5 |  Yes  No |  Yes  No | A1.2.29 | power control & sleep mode provided |  |
| T2.31 | Accommodate various types of traffic and traffic mixes | Obj | M.1036 § 1.10 |  Yes  No |  Yes  No | A1.2.18 A1.2.27, A1.2.28, A1.3.10 |  |  |
| Application of IMT-2000 for fixed services and developing countries |  |  |  |  |  |  |  |  |
| T2.32 | Repeaters for covering long distances between terminals and base stations, small rural exchanges with wireless trunks etc. | Req | M.819-2 Table 1 |  Yes  No |  Yes  No | A1.3.5 |  |  |
| T2.33 | Withstand rugged outdoor environment with wide temperature and humidity variations | Req | M.819-2 Table 1 |  Yes  No |  Yes  No |  |  |  |
| T2.34 | Provision of service to fixed users in either rural or urban areas | Obj | M.819-2 § 4.1 |  Yes  No |  Yes  No | A1.4.7, A3.4.2.4 |  |  |
| T2.35 | Coverage for large cells (terrestrial) | Obj | M.819-2 § 7.2 |  Yes  No |  Yes  No | A1.2.28 | No information provided to show that you can support a large cell considering the amount of delay spread that must be dealt with |  |
| T2.36 | Support for higher encoding bit rates for remote areas | Obj | M.819-2 § 10.1 |  Yes  No |  Yes  No |  |  |  |
| Additional satellite- component specific requirements and objectives |  |  |  |  |  |  |  |  |
|  | Links between the terrestrial and satellite control elements for handover and exchange of other information | Req | M.818-1 § 3.0 |  Yes  No |  Yes  No | Not Applicable | Not Applicable |  |
|  | Take account for constraints for sharing frequency bands with other services (WARC-92) | Obj | M.818-1 § 4.0 |  Yes  No |  Yes  No | Not Applicable | Not Applicable |  |
|  | Compatible multiple access schemes for terrestrial and satellite components | Obj | M.818-1 § 6.0 |  Yes  No |  Yes  No | Not Applicable | Not Applicable |  |
|  | Service should be comparable quality to terrestrial component as far as possible | Obj | M.818-1 § 10.0 |  Yes  No |  Yes  No | Not Applicable | Not Applicable |  |
|  | Use of satellites to serve large cells for fixed users | Obj | M.819-2 § 7.1 |  Yes  No |  Yes  No | Not Applicable | Not Applicable |  |
|  | Key features (e.g. coverage, optimization, number of systems) | Obj | M.1167 § 6.1 |  Yes  No |  Yes  No | Not Applicable | Not Applicable |  |
|  | Radio interface general considerations | Req | M.1167 § 8.1.1 |  Yes  No |  Yes  No | Not Applicable | Not Applicable |  |
|  | Doppler effects | Req | M.1167 § 8.1.2 |  Yes  No |  Yes  No | Not Applicable | Not Applicable |  |

- Table 3

Subjective Requirements and Objectives Relevant to the Evaluation of Candidate Radio Transmission Technologies*

|  | IMT-2000 Item Description | Obj /Req | Source | Proponents’ Comments | Annex 1/3 Reference | Evaluators’ Comments |
| --- | --- | --- | --- | --- | --- | --- |
| T3.1 | Fixed Service - Power consumption as low as possible for solar and other sources | Req | M.819-2 Table 1 | These are implementation dependent and are not restricted by the RTT definition | A1.2.29.1, A3.2.5.1, A3.6.15 | Power control and sleep mode are supported |
| T3.2 | Minimize number of radio interfaces and radio sub-system complexity, maximize commonality (M.1035 § 7.1) | Req | M.1034-1 § 11.2.1 | Yes |  |  |
| T3.3 | Minimize need for special interworking functions | Req | M.1034-1 § 11.2.4 | Yes. Interworking functions are only needed when interfacing to non-IP networks. | A3.5.3.1 | Interworking functions are necessary for supporting ISDN services |
| T3.4 | Minimum of frequency planning and inter-network coordination and simple resource management under time-varying traffic | Req | M.1034-1 § 11.2.6 | Yes | A3.4.2.2.1, A3.4.2.3.1 | System is not stable under reuse 1 with time varying traffic. |
| T3.5 | Support for traffic growth, phased functionality, new services or technology evolution | Req | M.1034-1 § 11.2.7 | Yes |  | Agree with proponent |
| T3.6 | Facilitate the use of appropriate diversity techniques avoiding significant complexity if possible | Req | M.1034-1 § 11.2.10 | Yes |  | Agree with proponent |
| T3.7 | Maximize operational flexibility | Req | M.1034-1 § 11.2.11 | Yes |  | Agree with proponent |
| T3.8 | Designed for acceptable technological risk and minimal impact from faults | Req | M.1034-1 § 11.2.12 | Yes |  | No Information to assess. |
| T3.9 | When several cell types are available, select the cell that is the most cost and capacity efficient | Obj | M.1034-1 §[9.2] M.1035 § 10.3.3 | Yes |  | No Information to assess. |
| T3.10 | Minimize terminal costs, size and power consumption, where appropriate and consistent with other requirements | Obj | M.1036 § 2.1.12 | Yes | A1.2.29.1, A3.2.5.1, A3.6.15 |  |

| Index | Criteria and attributes |  | Q or q |  | Gn |  | Related attributes in Annex 1 | Proponents Comments | Evaluators Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A3.1 | Spectrum efficiency : The following entries are considered in the evaluation of spectrum efficiency |  |  |  |  |  |  |  |  |
| A3.1.1 | For terrestrial environment |  |  |  |  |  |  |  |  |
| A3.1.1.1 | Voice traffic capacity (E/MHz/cell) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) for FDD mode or contiguous bandwidth of 30 MHz for TDD mode. This metric must be used for a common generic continuous voice bearer with characteristics 8 kbit/s data rate and an average BER 1  10-3 as well as any other voice bearer included in the proposal which meets the quality requirements (assuming 50% voice activity detection (VAD) if it is used). For comparison purposes, all measures should assume the use of the deployment models in Annex 2, including a 1% call blocking. The descriptions should be consistent with the descriptions under criterion § 6.1.7 – Coverage/power efficiency. Any other assumptions and the background for the calculation should be provided, including details of any optional speech codecs being considered. |  | Q and q |  | G1 |  | A1.3.1.5.1 | TDD mode Voice capacity using VoIP: -90 Erlangs/MHz/cell for reuse 3, SIMO, 10 MHz PUSC Subchannelization -80 Erlangs/MHz/cell for reuse 3, SIMO, 5 MHz PUSC Subchannelization Assumptions: -ITU vehicular path loss model -Pedestrian B3 channel model | Reuse definition needs to be clarified Is VoIP services only supported in reuse 3 system? The voice traffic capacity has to taken into account the frequency reuse. Does 90 Erlangs/MHz/cell taken into account of reuse 3? I.e., does this number imply 900 Erlangs or 2700 Erlangs over the 30 MHz bandwidth. Results are given with a model using Vehicular path loss and Pedestrian B3 channel model which is mixing environment information. What should be used along with the Ped B3 channel is the corresponding ITU pedestrian path loss model and alternatively a vehicular channel model should be used with the ITU vehicular path loss model. Results are to be given for each test environment indicated on the cover sheet (Indoor, Outdoor to Indoor Pedestrian, Vehicular). Furthermore, the deployment models in Annex 2 of M.1225 are to be used. The results are to be supplied using 1% blocking and 50% Voice Activity Detection, however, the submission used 2% blocking and 40% VAD (page 54, section 2.3.1.3). How is the effect of handoff latency modeled in this analysis? How are VoIP services supported by the hard handoff algorithm. For the voice capacity using VoIP, What needs to be reported along with the number of Erlangs/MHz/cell (for a given reuse pattern) is also some metric to describe the latency or outage. Provide mean and 97% tail latency distribution. How many base station and mobile station transmitter and receiver antennas are used for this capacity? For a fair comparison with other technologies, only mandatory features of the standard should be used to meet the requirements. Is the bandwidth request contention based? How is the request queuing delay simulated? If not, provide overhead computation for pre-allocated UL-MAP for all VoIP users in the system. Is the transmitter side packet drop captured in FER accounting? How is the FCH, DL-MAP, UL-MAP reliability and overhead modeled? How is CQI feedback and ACK feedback reliability and overhead modeled? Clarify which codec is used since 4 options are listed. Explain why is 40% voice activity used instead of 50% as required by M.1225? Provide details on power control and rate adaptation algorithms on DL and UL. Provide average DL single antenna C/I for reuse 3. Provide average UL pathloss distribution. Provide interference over thermal distribution on UL. Provide link level simulations. Provide detailed description on system and link simulation interface. With mixed channel model simulations, VoIP capacity is shown to be < 9 Erlangs/MHz/cell for reuse 1, SIMO, 10 MHz PUSC Subchannelizatoin. When 90 users are loaded in 3 sectors, the 10% of VoIP users exceeds 97% tail latency of 100 ms latency on DL. DL dimension in a TDD 2:1 10 MHz system is shown to be exhausted when 135 VoIP calls are admitted into each cell. Hence the upper bound on the VoIP capacity is 13.5 Erlang/Mhz/cell in reuse 1 system. See Appendix 3 |
| A3.1.1.2 | Information capacity (Mbit/s/MHz/cell) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) for FDD mode or contiguous bandwidth of 30 MHz for TDD mode. The information capacity is to be calculated for each test service or traffic mix for the appropriate test environments. This is the only measure that would be used in the case of multimedia, or for classes of services using multiple speech coding bit rates. Information capacity is the instantaneous aggregate user bit rate of all active users over all channels within the system on a per cell basis. If the user traffic (voice and/or data) is asymmetric and the system can take advantage of this characteristic to increase capacity, it should be described qualitatively for the purposes of evaluation. |  | Q and q |  | G1 |  | A1.3.1.5.2 | For the packet data bearer (UDD) service: Data capacity: -DL SIMO 5MHz= 3.45 Mbit/s/MHz/cell -DL SIMO 10MHz = 3.57 Mbit/s/MHz/cell -UL SIMO 5MHz = 1.6 Mbit/s/MHz/cell -DL MIMO 10MHz= 5.52 Mbit/s/MHz/cell -UL SIMO 10MHz= 1.59 Mbit/s/MHz/cell -UL MIMO 10MHz= 2.1 Mbit/s/MHz/cell Assumptions: - PUSC, ITU vehicular, 60% Pedestrian B 3, 30% Vehicular A 30, 10% Vehicular A 120, -DL:UL=28:9 (payload only) | Results are given with a model using a mixed test environment which does not match the deployment model for the mixed test environment in M.1225. The non-mandated mix uses a different path loss model than that specified in M.1225. Furthermore individual test environment results are not supplied for each test environment indicated on the cover sheet (Indoor, Outdoor to Indoor Pedestrian, Vehicular). Channel B is to be used for this analysis Outage for this data needs to be supplied as it is questionable using the assumptions stated in the 8F/1079Rev 1. See Appendices 1, 2, and 5. Results used log normal shadowing standard deviation of 8 dB rather than the mandated 10 dB. It is apparent that these numbers assume ideal channel estimates. In addition, for the UL MIMO case, it seems also that ideal case of MMSE combining is also assumed to separate the two users (ideal MMSE combing corresponds to the case where the two user’s channel are completely orthogonal, hence the MMSE in this case will be interference free and will be almost equivalent to MRC combining with 2 Rx antennas ). These two critical assumptions explain the 32% gain in throughput in the UL MIMO mode compared to the UL SIMO mode. It needs to be pointed out, however, that this is unrealistic. Our simulation for UL spatial multiplexing suggest a maximum of 10-12% improvement in throughput with two antennas. The DL MIMO specified by the proposal is Vertical spatial Multiplexing (8.4.8.1.4). The submission claims 55% improvements in the DL MIMO throughput relative to a baseline 1 transmit, 2 receive antenna SIMO case. This percentage of improvement is possible only if the receiver has perfect knowledge of the channel (which is exactly what is stated in the assumption table) as well as there is no gap-to-capacity due to particular channel code being used. Hence, it seems that this 55% gain is unrealistic. In fact, according to our MIMO simulations (which assumes a practical system implementation and proportional fair scheduling as it is assumed in the submission), gains of the 2x2 MIMO case with respect to the base line 1x2 SIMO case are only noticeable for users with a geometry of 8 dB or more. However, the percentage of users that will have such geometry is no more than 10% (under these circumstances the gains are ~ 20%). Therefore, it is doubtful that any practical system implementation will achieve this 55% gain in throughput. What frequency reuse is assumed for the simulation? How many base station and mobile station transmitter and receiver antennas are used for this capacity? For a fair comparison with other technologies, only mandatory features of the standard should be used to meet the requirements. Does SIMO imply 1Tx 2 Rx? Simulation assumptions include 4 Rx options. What’s the FER distribution among users? How is the FCH, DL-MAP, UL-MAP reliability and overhead modeled? How is CQI feedback and ACK feedback reliability and overhead modeled? Overhead accounting is clearly insufficient. Provide details on power control and rate adaptation algorithms on DL and UL. Provide average DL single antenna C/I for reuse 3. Provide average UL pathloss distribution. Provide interference over thermal distribution on UL. Provide link level simulations. Provide detailed description on system and link simulation interface. A TDD 2:1 10 MHz system is simulated with realistic channel estimation performance and control channel modeling. The DL capacity of the proposed technology is shown to be 0.9 Mbps/MHz/cell in a 3 sector/cell system with frequency reuse 1. Results in A3.1.1.1 and A3.1.1.2 do not use the required M.1225 deployment models for the given test environment, especially power, area, coverage, and user density. All the results are to use a consistent set of assumptions between the deployment models and the link budgets for simulations. Furthermore, Table 11 from M.1225 (deployment results matrix) is missing in submission. |
| A3.1.2 | For satellite environment These values (§ A3.1.2.1 and A3.1.2.2) assume the use of the simulation conditions in Annex 2. The first definition is valuable for comparing systems with identical user channel rates. The second definition is valuable for comparing systems with different voice and data channel rates. |  |  |  |  |  |  |  |  |
| A3.1.2.1 | Voice information capacity per required RF bandwidth (bit/s/Hz) |  | Q |  | G1 |  | A1.3.2.3.1 | NA |  |
| A3.1.2.2 | Voice plus data information capacity per required RF bandwidth (bit/s/Hz) |  | Q |  | G1 |  | A1.3.2.3.2 | NA |  |
| A3.2 | Technology complexity – Effect on cost of installation and operation The considerations under criterion § 6.1.2 – Technology complexity apply only to the infrastructure, including BSs (the handportable performance is considered elsewhere). |  |  |  |  |  |  |  |  |
| A3.2.1 | Need for echo control The need for echo control is affected by the round trip delay, which is calculated as shown in Fig. 6. Referring to Fig. 6, consider the round trip delay with the vocoder (D1, ms) and also without that contributed by the vocoder (D2, ms). NOTE 1 – The delay of the codec should be that specified by ITU-T for the common generic voice bearer and if there are any proposals for optional codecs include the information about those also. |  | Q |  | G4 |  | A1.3.7.2 A1.3.7.3 | Echo control is needed for voice applications. The voice delay is also dependent on the codec used. Selection of the codec is implementation dependent and no specific codec is mandated. Echo control is used on the MS and also optionally on a need basis at the BS or Gateways. The performance characteristics meet the delay requirements outlined in ITU-R M.1079. | Detailed delay analysis is not provided. D1 and D2 delays are to be specified. |
| A3.2.2 | Transmitter power and system linearity requirements NOTE 1 – Satellite e.i.r.p. is not suitable for evaluation and comparison of RTTs because it depends very much on satellite orbit. The RTT attributes in this section impact system cost and complexity, with the resultant desirable effects of improving overall performance in other evaluation criteria. They are as follows. |  |  |  |  |  |  |  |  |
| A3.2.2.1 | Peak transmitter/carrier (Pb) power (not applicable to satellite) |  | Q |  | G1 |  | A1.2.16.2.1 | This is not limited by RTT but rather by regulations for the specific RF bands. Mobile Station @ 2.5GHz 23 dBm EIRP (Power class I, QPSK, Refer to Section A3.2.2.2) | Peak Transmitter powers are not supplied (is to be the same as used in link budgets) |
|  | Peak transmitter power for the BS should be considered because lower peak power contributes to lower cost. Note that Pb may vary with test environment application. This is the same peak transmitter power assumed in Appendix 2, link budget template (Error: Reference source not found). |  |  |  |  |  |  | This is not limited by RTT but rather by regulations for the specific RF bands. |  |
| A3.2.2.2 | Broadband power amplifier (PA) (not applicable to satellite) Is a broadband power amplifier used or required? If so, what are the peak and average transmitted power requirements into the antenna as measured in watts. |  | Q |  | G1 |  | A1.4.10 A1.2.16.2.1 A1.2.16.2.2 A1.5.5 A1.2.5 | A broadband power amplifier is required. Tx Power is not limited by RTT but by regulations. BS - Tx dynamic range = 10 dB - Spectral flatness as per conditions in A.1.4.10 - Peak Tx power on BS is limited only by regulations and not by the RTT. MS - Tx dynamic range = 45 dB - Spectral flatness as per conditions in A.1.4.10 - 4 power classes are supported as shown below: Peak Transmit power (dBm) for 16QAM 1. 18 <= Ptx,max < 21 2. 21 <= Ptx,max < 25 3. 25 <= Ptx,max < 30 4. 30 <= Ptx,max Peak Transmit power (dBm) for QPSK 1. 20 <= Ptx,max < 23 2. 23 <= Ptx,max < 27 3. 27 <= Ptx,max < 30 4. 30 <= Ptx,max |  |
| A3.2.2.3 | Linear base transmitter and broadband amplifier requirements (not applicable to satellite) |  |  |  |  |  |  |  |  |
| A3.2.2.3.1 | Adjacent channel splatter/emission and intermodulation affect system capacity and performance. Describe these requirements and the linearity and filtering of the base transmitter and broadband PA required to achieve them. | q |  | G3 |  | A1.4.2 A1.4.10 |  | Base stations and terminals supporting this RTT will comply with local, regional, and international regulations for out of band and spurious emissions, wherever applicable. | Adjacent channel emission is not supplied. Additionally, Transmitter linearity and filtering and PA requirements are not specified. |
| A3.2.2.3.2 | Also state the base transmitter and broadband PA (if one is used) peak to average transmitter output power, as a higher ratio requires greater linearity, heat dissipation and cost. | Q and q |  | G2 |  | A1.4.10 A1.2.16.2.1 A1.2.16.2.2 |  | These are implementation dependent. The PAPR of the proposed RTT is around 12dB |  |
| A3.2.2.4 | Receiver linearity requirements (not applicable to satellite) Is BS receiver linearity required? If so, state the receiver dynamic range required and the impact of signal input variation exceeding this range, e.g., loss of sensitivity and blocking. | q |  | G4 |  | A1.4.11 A1.4.12 |  | BS Max input level on-channel reception tolerance = -45 dBm Max input level on-channel damage tolerance = -10 dBm MS Max input level on-channel reception tolerance = -30 dBm Max input level on-channel damage tolerance = 0 dBmBS/MS BS and MS Max input level sensitivity (Distributed permutation of subcarriers) for 10 MHz case: -88.5 dBm - QPSK-1/2 -85.1 dBm - QPSK-3/4 -82.8 dBm - 16QAM-1/2 -78.7 dBm - 16QAM-3/4 -77.6 dBm - 64QAM-1/2 -74.5 dBm - 64QAM-2/3 -73.4 dBm - 64QAM-3/4 -71.5 dBm - 64QAM-5/6 Max input level sensitivity (Distributed permutation of subcarriers) for 5 MHz case: -91.5 dBm - QPSK-1/2 -88.1 dBm - QPSK-3/4 -85.8 dBm - 16QAM-1/2 -81.7 dBm - 16QAM-3/4 -80.6 dBm - 64QAM-1/2 -77.5 dBm - 64QAM-2/3 -76.4 dBm - 64QAM-3/4 -74.5 dBm - 64QAM-5/6 Sensitivity numbers are calculated based on assumption of repetition factor 1 and Distributed permutation of subcarriers. | Receiver linearity requirements are not stated |
| A3.2.3 | Power control characteristics (not applicable to satellite) Does the proposed RTT utilize transmitter power control? If so, is it used in both forward and reverse links? State the power control range, step size (dB) and required accuracy, number of possible step sizes and number of power controls per second, which are concerned with BS technology complexity. | Q and q |  | G4 |  | A1.2.22 A1.2.22.1 A1.2.22.2 A1.2.22.3 A1.2.22.4 A1.2.22.5 |  | Open loop and closed loop transmitter power control methods are used. Power control is done on the DL as well as the UL. Power control step size is variable ranging from 0.25 dB to 32 dB. An 8-bit signed integer in power control information element indicates the power control step size in 0.25 dB units. Normally implemented in 1 dB increments. The power control cycle of closed-loop or open-loop power control is dependent on the rate of power control information element transmission, but less than 200 Hz. The accuracy for power level control can vary from ± 0.5 dB to ± 2 dB depending on the power control step size. Single step size m ¦ Required relative accuracy ¦m¦ = 1dB¦ ± 0.5 dB ¦m¦ = 2dB¦± 1 dB ¦m¦ = 3dB¦± 1.5 dB 4dB <¦m¦< = 10 dB¦± 2 dB Two exception points of at least 10 dB apart are allowed over the 45 dB range, where in these two points an accuracy of up to ± 2 dB is allowed for any size step. The minimum power control dynamic range is 45 dB. The RTT supports 45 dB under the full power assumption | What needs to be stated here also is that the UL power control is message based and each message is 44 bits. What needs to be evaluated here is that how many users in a fully loaded cell can be power controlled at the full PC rate or equivalently, what is the maximum PC rate that can be used for a fully loaded cell/sector, and whether that will be sufficient to provide the expected performance. Please provide system level study with power control rate / system performance and overhead tradeoff. |
| A3.2.4 | Transmitter/receiver isolation requirement (not applicable to satellite) If FDD is used, specify the noted requirement and how it is achieved. | q |  | G3 |  | A1.2.2 A1.2.2.2 A1.2.2.1 |  | Not Applicable as it is TDD. | Transmitter/Receiver isolation requirement is not specified For example requirements for transmit/receive guard time (RTG/TTG) and switch isolation (or equivalent) need to be specified |
| A3.2.5 | Digital signal processing requirements |  |  |  |  |  |  |  |  |
| A3.2.5.1 | Digital signal processing can be a significant proportion of the hardware for some radio interface proposals. It can contribute to the cost, size, weight and power consumption of the BS and influence secondary factors such as heat management and reliability. Any digital circuitry associated with the network interfaces should not be included. However any special requirements for interfacing with these functions should be included. This section of the evaluation should analyse the detailed description of the digital signal processing requirements, including performance characteristics, architecture and algorithms, in order to estimate the impact on complexity of the BSs. At a minimum the evaluation should review the signal processing estimates (MOPS, memory requirements, gate counts) required for demodulation, equalization, channel coding, error correction, diversity processing (including Rake receivers), adaptive antenna array processing, modulation, A-D and D-A converters and multiplexing as well as some IF and baseband filtering. For new technologies, there may be additional or alternative requirements (such as FFTs). Although specific implementations are likely to vary, good sample descriptions should allow the relative cost, complexity and power consumption to be compared for the candidate RTTs, as well as the size and the weight of the circuitry. The descriptions should allow the evaluators to verify the signal processing requirement metrics, such as MOPS, memory and gate count, provided by the RTT proponent. | Q and q |  | G2 |  | A1.4.13 |  | The Hardware requirements are implementation dependent. For 5 MHz a 512 FFT and for 10 MHz and 1024 FFT is required. Memory and Processing needs are very much specific to the type of hardware. | Digital signal processing requirements are not supplied. At a minimum the evaluation should review the signal processing estimates (MOPS, memory requirements, gate counts). When using 512 and 1024 point FFT for 5 MHz and 10 MHz (chip x1 sampling), respectively, the required analog filters’ implementation complexity needs to be specified |
| A3.2.5.2 | What is the channel coding/error handling for both the forward and reverse links? Provide details and ensure that implementation specifics are described and their impact considered in DSP requirements described in § A3.2.5.1. | q |  | G4 |  | A1.2.12 A1.4.13 |  | An 8bit CRC is used for MAC PDU errors. Forward Error Correction schemes Convolutional Coding and Convolutional Turbo Coding are supported Modulation schemes: QPSK, 16 QAM and 64 QAM for downlink, QPSK and 16 QAM for uplink. Coding rates: QPSK 1/2, QPSK 3/4, 16 QAM 1/2, 16 QAM 3/4, 64 QAM 1/2, 64 QAM 2/3, 64 QAM 3/4, 64 QAM 5/6. Coding repetition rates: 1x, 2x, 4x and 6x. |  |
| A3.2.6 | Antenna systems |  |  |  |  |  |  |  |  |
|  | The implementation of specialized antenna systems while potentially increasing the complexity and cost of the overall system can improve spectrum efficiency (e.g. smart antennas), quality (e.g. diversity), and reduce system deployment costs (e.g. remote antennas, leaky feeder antennas). |  |  |  |  |  |  | MS: 1 Tx Antenna 2 Rx Antennas BS: 2 or more Tx Antennas 2 or more Rx Antennas Both MIMO and Beamforming support are mandatory at the Mobile Stations. Base Stations may support either MIMO or Beamforming. In general, it is expected for Beamforming to be deployed in scenarios where increased coverage is required (urban and suburban scenarios), while MIMO is expected to be employed in scenarios requiring high system capacity (urban scenarios). For MIMO operation: Adaptive switching between STC and SM is supported, see Section 1.3. 5 for a detailed description. Two transmit and two or more receive antennas are employed at the BS; one transmit and two receive antennas are supported at the MS. The typical antenna spacing at the BS and MS is 10 λ and 0.5 λ, respectively, where λ stands for the carrier wavelength. Regarding the type of equalizers for the SM MIMO mode, either minimum mean squared error (MMSE) or maximum-likelihhod (ML) based receivers will be implemented by MS vendors. Regarding the CSI, this is based either on physical or effective carrier-to-interference-and-noise ratio (CINR), while the communication of the MIMO mode is also enabled by the Mobile WiMAX system profiles. Please see also Section 1.3.5 for a detailed description. For Beamforming operation: Typically, a BS transceiver is equipped with 4 transmit and receive antennas but larger number of antennas can be used. The antenna spacing depends on the used Beamforming algorithm and can range from 0.5 λ to 3 λ. Regarding the weight update operation, see also Section 1.3.5, this is based on channel sounding, which is the process of channel estimation during the uplink operation for updating the antenna weights to be used for the subsequent transmission to a particular user in the downlink. Note that due to the channel reciprocity enabled by the TDD operation, the weights are accurate for low MS speeds, e.g., up to 30 km/h, while a graceful degradation of the performance is expected for higher speeds. Certainly, the accuracy of the antenna weights is also highly dependent on the specific Beamforming algorithm used at the BS, which may lead to smaller performance degradation at higher MS speeds. | The proposal indicates 2 or more transmit antennas at the base station. For MIMO operation, the indication in the proposal is that two MIMO streams are supported. The RTT needs to clarify how will that be done in the case where the base station have more than 2 transmit antennas (4 transit antennas for example), i.e. how those 2 spatial streams will be mapped to the 4 transmit antennas. |
|  | NOTE 1 – For the satellite component, diversity indicates the number of satellites involved; the other antenna attributes do not apply. |  |  |  |  |  |  |  |  |
| A3.2.6.1 | Diversity : describe the diversity schemes applied (including micro and macro diversity schemes). Include in this description the degree of improvement expected, and the number of additional antennas and receivers required to implement the proposed diversity design beyond and omni-directional antenna. | Q |  | G2 |  | A1.2.23 A1.2.23.1 A1.2.23.2 |  | When the MIMO option is deployed: In the downlink, both transmit diversity and receive diversity is supported through the use of STC (use of the Alamouti code which is a space-time block coding code for two transmit antennas, while two receive antennas are used at the MS for receive diversity). Note that when SM is used, although there is also inherent transmit and receive diversity due to the use of two antennas at both the BS and MS, the target is the increase of the peak rate by transmitting two data streams over one OFDMA symbol per subcarrier, see also Section 1.3.5 for a detailed description. In the uplink where CSM (collaborative spatial multiplexing) is supported, receive diversity is applied by the use of two or more receive antennas at the BS. Depending on the propagation environment (mainly characterized by the frequency and time diversity of the link-level channel model), the signal-to-noise ratio (SNR) gain of STC ranges from 4 dB to 7dB compared to a single antenna system; the SNR gain of SM ranges from 2 dB to 4 dB compared to a single antenna system, where there is double data throughput supported by SM compared to the single antenna system. Regarding the CSM mode, higher gains on the order of 1 dB to 2 dB are expected compared to the SM gains reported above. When the Beamforming option is applied: In the downlink, transmit diversity is supported, while receive diversity is also applied when two receive antennas are used at the MS. In the uplink, receive diversity is supported by using multiple antenna reception at the BS. For a typical implementation of 4 receive and transmit antennas for Bemaforming, the SNR gains at both the uplink and the downlink are expected to range from 6 dB to 12 dB. | In order to get transmit diversity in the MIMO mode, the BS station must use a number of transmit antennas that is larger than the number of DL streams being transmitted and this would require provisions in the RTT so that the base station can map those independent streams to the antennas. Those provisions do not exist in the RTT submision. In addition, receive diversity at the MS will be obtained only when the number of receive antennas used at the MS is larger than the DL streams being sent by the basestation. When uplink CSM is used, received diversity will be achieved at the base station only if the number of receive antennas at the base station is larger than the number of users that are involved in the CSM or one of those user has a very weak signal at the base station. The gains listed here needs to be verified. Also, need to state at which BER, PER these gains are achieved. In addition, it is questionable that you can get 2-4 dB SNR gain from SM compared to the single antenna system while at the same time the throughput is doubled |
| A3.2.6.2 | Remote antennas : describe whether and how remote antenna systems can be used to extend coverage to low traffic density areas. | q |  | G2 |  | A1.3.6 |  | These can be used for extending coverage. Performance is implementation and deployment scenario specific. | No details given on how this is done in the current RTT |
| A3.2.6.3 | Distributed antennas : describe whether and how distributed antenna designs are used. | q |  | G3 |  | A1.3.6 |  | They can be used in microcellular environments. | No details given on how this is done in the current RTT |
| A3.2.6.4 | Unique antenna : describe additional antenna systems which are either required or optional for the proposed system, e.g., beam shaping, leaky feeder. Include in the description the advantage or application of the antenna system. | q |  | G4 |  | A1.3.6 |  | MIMO and Beamforming types of Smart Antenna capability are supported. MIMO is used for capacity enhancements. Beamforming is used for coverage enhancement. |  |
| A3.2.7 | BS frequency synchronization/time alignment requirements Does the proposed RTT require base transmitter and/or receiver station synchronization or base-to-base bit time alignment? If so, specify the long term (1 year) frequency stability requirements, and also the required bit-to-bit time alignment. Describe the means of achieving this. | Q and q |  | G3 |  | A1.4.1 A1.4.3 |  | As it is a TDD system, BS synchronization is required. Methods used are implementation dependent. GPS based methods are typically used. BS frequency tolerance ≤ ± 2ppm of carrier frequency BS to BS frequency accuracy ≤ ± 1% of subcarrier spacing MS to BS frequency synchronization tolerance ≤ 2% of the subcarrier spacing. Time alignment between BS and MS is achieved using the Downlink Preambles and the Uplink ranging operation which corrects time offset errors. The OFDMA Cyclic Prefix marks the Symbol level time alignment. |  |
| A3.2.8 | The number of users per RF carrier/frequency channel that the proposed RTT can support affects overall cost – especially as bearer traffic requirements increase or geographic traffic density varies widely with time. Specify the maximum number of user channels that can be supported while still meeting ITU-T Recommendation G.726 performance requirements for voice traffic. | Q |  | G1 |  | A1.2.17 |  | The maximum number of voice channels per 1 RF channel depends on the bit rate and sampling rate supported by the codecs defined in the G.726. For instance, in case of the bit rate of 16 kbit/s with 20 msec sampling rate, up to 256 users can be supported simultaneously by a 10 MHz RF channel, while meeting the delay requirements of VoIP. In the case of a 5 MHz channel up to 120 users can be supported. The performance characteristics meet the delay and traffic requirements outlined in ITU-R M.1079. | What is the delay requirement VoIP that is referred to here? What are the exact simulation setup? What is the reuse frequency used? This number of users is also inconsistent in the light of the answer to A3.1.1.1 above regarding the Erlang Capacity/MHz/Cell. |
| A3.2.9 | Base site implementation/installation requirements (not applicable to satellite) BS size, mounting, antenna type and height can vary greatly as a function of cell size, RTT design and application environment. Discuss its positive or negative impact on system complexity and cost. | q |  | G1 |  | A1.4.17 |  | No RTT specific requirements exist. | RTT specifies GPS synchronization of Base stations. This is not specified in this entry. Proposal requires two transmitters at the Base Station. |
| A3.2.10 | Handover complexity Consistent with handover quality objectives defined in criterion § 6.1.3, describe how user handover is implemented for both voice and data services and its overall impact on infrastructure cost and complexity. | Q and q |  | G1 |  | A1.2.24 A1.4.6.1 |  | Simple Hard Handover and Optimized Hard Handover is supported. As the MS is only attached to one BS at a time significantly less complexity is expected. As voice is supported as an application over the IP data bearer the handover is always treated as a data connection. Base stations and Mobile stations implement the ability to buffer data during handover as well the protocols necessary for handover. See section 2.2.2.2 for handover performance analysis. | It is our understanding that only hard handover is supported in the RTT system. Provide evidence that HHO is sufficient for reuse-1. |
| A3.3 | Quality |  |  |  |  |  |  |  |  |
| A3.3.1 | Transparent reconnect procedure for dropped calls Dropped calls can result from shadowing and rapid signal loss. Air interfaces utilizing a transparent reconnect procedure – that is, the same as that employed for hand-off – mitigate against dropped calls whereas RTTs requiring a reconnect procedure significantly different from that used for hand-off do not. | q |  | G2 |  | A1.4.14 |  | Voice is supported as an application over the RTT. The RTT is primarily designed to support Voice using Voice Over IP Protocols. MAC connections that provide reliable Quality of Service for Voice Over IP data flows are supported. These data connections are managed using timers and well as MAC layer signaling to ensure a reliable connection is maintained. Transparent reconnects are provided by the application layer for the voice traffic. As the RTT supports Adaptive Modulation and Coding, and Link Adaptation methods, the MAC level transport connections are managed to make them reliable. |  |
| A3.3.2 | Round trip delay, D1 (with vocoder (ms)) and D2 (without vocoder (ms)) (See Fig. 6). NOTE 1 – The delay of the codec should be that specified by ITU-T for the common generic voice bearer and if there are any proposals for optional codecs include the information about those also. (For the satellite component, the satellite propagation delay is not included.) | Q |  | G2 |  | A1.3.7.1 A1.3.7.2 |  | Assuming G.729 with a vocoder delay of 20ms for a 20 Byte voice sample. D1 = 20ms (vocoder) + 50ms (max one-way air interface delay) x 2 = 120ms D2 = 50ms x 2 = 100ms |  |
| A3.3.3 | Handover/ALT quality Intra switch/controller handover directly affects voice service quality. Handover performance, minimum break duration, and average number of handovers are key issues. | Q |  | G2 |  | A1.2.24 A1.2.24.1 A1.2.24.2 A1.4.6.1 |  | Handover signaling is designed to minimize loss of data. Handover latency is <= 50ms if no network re-entry is required. This ensures minimum disruption to data transfer. If NW re-entry is required the latency is <= 85ms. Handover frequency is scenario specific. | Very limited information provided on Handover. Is only Hard Handover supported? Average number of handovers is not supplied |
| A3.3.4 | Handover quality for data There should be a quantitative evaluation of the effect on data performance of handover. | Q |  | G3 |  | A1.2.24 A1.2.24.1 A1.2.24.2 A1.4.6.1 |  | Handover for voice and data are treated the same way in this RTT. | No quantitative information is provided |
| A3.3.5 | Maximum user bit rate for data (bit/s) A higher user bit rate potentially provides higher data service quality (such as high quality video service) from the user’s point of view. | Q |  | G1 |  | A1.3.3 |  | The maximum bit rates are well above 20160 kbit/s. (DL/UL ratio = 2:1, PUSC, 64QAM, 5/6 coding rate) | Maximum user bit rate is not supplied |
| A3.3.6 | Channel aggregation to achieve higher user bit There should also be a qualitative evaluation of the method used to aggregate channels to provide higher bit rate services. | q |  | G4 |  | A1.2.32 |  | No channel aggregation is necessary as IP-OFDMA can operate over the entire 10 MHz channel. However, flexible allocation of subchannels (in frequency domain) within an RF channel can be used to dynamically allocate bandwidth to individual users for various bit rate services (see also Section s 1.3.1 to 1.3.3) . |  |
| A3.3.7 | Voice quality Recommendation ITU-R M.1079 specifies that FPLMTS speech quality without errors should be equivalent to ITU-T Recommendation G.726 (32 kbit/s ADPCM) with desired performance at ITU-T Recommendation G.711 (64 kbit/s PCM). NOTE 1 – Voice quality equivalent to ITU-T Recommendation G.726 error free with no more than a 0.5 degradation in MOS in the presence of 3% frame erasures might be a requirement. | Q and q |  | G1 |  | A1.2.19 A1.3.8 |  | The vocoder is independent of the RTT. Any suitable vocoder can be used as voice is supported over using Voice over IP protocol. Therefore the MOS values for the G.726 or any other vocoder used will apply. | Voice quality is not demonstrated. Specifically, data to verify the G.726 error free performance and no more than 0.5 MOS degradation with 3% frame error rate is not supplied. |
| A3.3.8 | System overload performance (not applicable to satellite) Evaluate the effect on system blocking and quality performance on both the primary and adjacent cells during an overload condition, at e.g. 125%, 150%, 175%, 200%. Also evaluate any other effects of an overload condition. | Q and q |  | G3 |  | A1.3.9.1 |  | System overload causes graceful degradation as data transmission bandwidth can be traded off for lower quality connections. As adaptive modulation and coding are supported the system adapts to the load conditions as per the policies implemented. | Blocking and quality performance are not specified for 125%, 150%, 175%, and 200% load. |
| A3.4 | Flexibility of radio technologies |  |  |  |  |  |  |  |  |
| A3.4.1 | Services aspects |  |  |  |  |  |  |  |  |
| A3.4.1.1 | Variable user bit rate capabilities Variable user bit rate applications can consist of the following: – adaptive signal coding as a function of RF signal quality; – adaptive voice coder rate as a function of traffic loading as long as ITU-T Recommendation G.726 performance is met; – variable data rate as a function of user application; – variable voice/data channel utilization as a function of traffic mix requirements. Some important aspects which should be investigated are as follows: – how is variable bit rate supported? – what are the limitations? Supporting technical information should be provided such as – the range of possible data rates, – the rate of changes (ms). | q and Q |  | G2 |  | A1.2.18 A1.2.18.1 |  | The user bit rates are variable according to the number of subchannels assigned and modulation and coding rate used. The rates can be changed every 5ms which is every frame. The DL-MAP and UL-MAP signal the changes every frame. DOWNLINK BW: 10 MHz Modulation : QPSK, 16 QAM, 64 QAM Coding rate : 1/2, 2/3, 3/4, 5/6 Data rates: 9.6 kbit/s to 23040 kbit/s UPLINK BW: 10 MHz Modulation : QPSK, 16 QAM Coding rate : 1/2, 3/4 Data rates: 9.6kbit/s to 6048 kbit/s |  |
| A3.4.1.2 | Maximum tolerable Doppler shift, Fd (Hz) for which voice and data quality requirements are met (terrestrial only) Supporting technical information: Fd | q and Q |  | G3 |  | A1.3.1.4 |  | Fd ~500 Hz Voice and Data are treated the same way from the Physical layer perspective. | This Doppler corresponds to a speed of approx. 250 km/hr @ 2.5 GHz carrier frequency. However, no data in the submitted proposal support this claim. The mobility profile included in the proposal only go up 120 km/hr for only 10% of the users with the majority of users below 30 km/hr. This claim is very important since it will affect the performance of channel estimation. |
| A3.4.1.3 | Doppler compensation method (satellite component only) What is the Doppler compensation method and residual Doppler shift after compensation? | Q and q |  | G3 |  | A1.3.2.2 |  | NA |  |
| A3.4.1.4 | How the maximum tolerable delay spread of the proposed technology impact the flexibility (e.g., ability to cope with very high mobile speed)? | q |  | G3 |  | A1.3.1.3 A1.2.14 A1.2.14.1 A1.2.14.2 A1.3.10 |  | ~20µs of delay spread can be tolerated without an equalizer. | If a longer delay spread (longer than the CP can handle) is encountered, performance will suffer depending on the power delay profile of the multipath. Here, the cyclic prefix is specified as 1/8 of an OFDM symbol which corresponds to 11.4 sec. Any multipath beyond that will results in inter-carrier interference (ICI) and hence will gracefully degrade the performance, depending on the power delay profile of the channel. If the power delay profile has large energy beyond the 11.4 sec delay spread, there will be an increase in ICI, and orthogonality between carriers will be lost. In addition, there will be increased channel estimation loss. Take for example Veh B channel, the performance will be bad. In addition, in one of the answers in Annex 2, the RTT claims support of cell sizes that are up to 35 Km in size. The key question here is that for large cells, what is the probability the delay spread will be within (or reasonable close to) 11.4 sec? |
| A3.4.1.5 | Maximum user information bit rate, Ru (kbit/s) How flexibly services can be offered to customers ? What is the limitation in number of users for each particular service? (e.g. no more than two simultaneous 2 Mbit/s users) | Q and q |  | G2 |  | A1.3.3 A1.3.1.5.2 A1.2.31 A1.2.32 |  | Assuming 10 MHz PUSC: - 23040 kbit/s for the Downlink (DL:UL=35:12) - 6048 kbit/s for the Uplink for (DL:UL=26:21) Services are very flexible as the Subchannels can be grouped to increase data rates. |  |
| A3.4.1.6 | Multiple vocoder rate capability – bit rate variability, – delay variability, – error protection variability. | Q and q |  | G3 |  | A1.2.19 A1.2.19.1 A1.2.7 |  | Yes. Vocoders are however independent of the RTT and are implementation specific. The data transports for voice can operate at varying levels of Packet error rate and using H-ARQ can significantly boost performance. | Specific voice coding rates are not specified Information on bit rate variability , delay variability, and error protection variability is not provided |
| A3.4.1.7 | Multimedia capabilities The proponents should describe how multimedia services are handled. The following items should be evaluated: – possible limitations (in data rates, number of bearers), – ability to allocate extra bearers during of the communication, – constraints for handover. | Q and q |  | G1 |  | A1.2.21 A1.2.20 A1.3.1.5.2 A1.2.18 A1.2.24 A1.2.30 A1.2.30.1 |  | The Data bearers have no constraints on the type of media they can carry. However typically they are mapped to the QoS of the media type being transmitted. There are no limits on the number of bearers as long as bandwidth is available. Extra bearers can be allocated during communication. There are no handover constraints as long as coverage is available. | Does the number of CIDs limit the number of bearers? |
| A3.4.2 | Planning |  |  |  |  |  |  |  |  |
| A3.4.2.1 | Spectrum related matters |  |  |  |  |  |  |  |  |
| A3.4.2.1.1 | Flexibility in the use of the frequency band The proponents should provide the necessary information related to this topic (e.g., allocation of sub-carriers with no constraints, handling of asymmetric services, usage of non-paired band). | q |  | G1 |  | A1.2.1 A1.2.2 A1.2.2.1 A1.2.3 A1.2.5.1 |  | A 5 MHz or 10 MHz TDD carrier may be deployed with 1:3:3 frequency re-use or 1:3:1 reuse. |  |
| A3.4.2.1.2 | Spectrum sharing capabilities The proponent should indicate how global spectrum allocation can be shared between operators in the same region. The following aspects may be detailed: – means for spectrum sharing between operators in the same region, – guardband between operators in case of fixed sharing. | q and Q |  | G4 |  | A1.2.26 |  | The proposed RTT utilizes OFDMA which has inherent interference protection capabilities due to allocation of a varying subset of available sub-carriers to different users. So spectrum sharing is carried out using multiple channel carriers. The guard bands are RF band specific. | Guard bands are not specified Explanation is needed as to why guardbands are RF band specific. What are the requirements for guardbands between IP-OFDMA and IMT-2000 technologies in the same band? |
| A3.4.2.1.3 | Minimum frequency band necessary to operate the system in good conditions Supporting technical information: – impact of the frequency reuse pattern, – bandwidth necessary to carry high peak data rate. | Q and q |  | G1 |  | A1.2.1 A1.4.15 A1.2.5 |  | 5 MHz or 10 MHz 1x3x3 PUSC or 1x3x1 PUSC may be used. 10 MHz gives the optimal data rate. |  |
| A3.4.2.2 | Radio resource planning |  |  |  |  |  |  |  |  |
| A3.4.2.2.1 | Allocation of radio resources The proponents and evaluators should focus on the requirements and constraints imposed by the proposed technology. More particularly, the following aspects should be considered: – what are the methods used to make the allocation and planning of radio resources flexible? – what are the impacts on the network side (e.g. synchronization of BSs, signalling,)? – other aspects. Examples of functions or type of planning required which may be supported by the proposed technology: – DCA, – frequency hopping, – code planning, – time planning, – interleaved frequency planning. NOTE 1 – The use of the second adjacent channel instead of the adjacent channel at a neighbouring cluster cell is called “interleaved frequency planning”. In some cases, no particular functions are necessary (e.g. frequency reuse  1). | q |  | G2 |  | A1.2.25 A1.2.27 A1.4.15 |  | Subchannelization schemes and zones namely PUSC and AMC are supported to provide flexibility in utilizing the frequency and time resources. Sectorized deployments are possible with flexible frequency re-use (1x3x3 or 1x3x1) using PUSC subchannelization schemes. Slots of multiple subchannels and OFDM symbols are used to manage the resource allocation granularity BSs need to be synchronized. This is typically done using GPS on the BS. No frequency planning is required across cells. |  |
| A3.4.2.2.2 | Adaptability to adapt to different and/or time varying conditions (e.g., propagation, traffic) How the proposed technology cope with varying propagation and/or traffic conditions? Examples of adaptive functions which may be supported by the proposed technology: – DCA, – link adaptation, – fast power control, – adaptation to large delay spreads. Some adaptivity aspects may be inherent to the RTT. | q |  | G2 |  | A1.3.10 A1.2.27 A1.2.22 A1.2.14 |  | Subchannelization and slot structure capability provides the ability to schedule frequency/time resources to mitigate the effects of propagation losses and also for traffic load balancing. Link adaptation schemes with CQI feedback capability allow operating the link more efficiently. H-ARQ also allows operations at high packet error rates resulting higher spectral efficiency as higher order coding and modulation rates can be used. The OFDMA symbol structure is designed to reduce the effects of delay spreads up to 20µs. | Symbol structure designed to reduce the effects of delay spreads up to 20 sec needs explanation since the cyclic prefix is 11.2 sec |
| A3.4.2.3 | Mixed cell architecture (not applicable to satellite component) |  |  |  |  |  |  |  |  |
| A3.4.2.3.1 | Frequency management between different layers What kind of planning is required to manage frequencies between the different layers? e.g. – fixed separation, – dynamic separation, – possibility to use the same frequencies between different layers. Possible supporting technical information: – guard band. | q and Q |  | G1 |  | A1.2.28 A1.4.15 |  | Hierarchical layered cells are possible. The type of frequency planning is implementation/deployment scenario specific. The same frequencies can be used across layers by proper segmentation of the PUSC Subchannels. | Explanation is needed as to how the same frequencies can be used across layers by proper segmentation of the PUSC Subchannels. |
| A3.4.2.3.2 | User adaptation to the environment What are the constraints to the management of users between the different cell layers? e.g. – constraints for handover between different layers, – adaptation to the cell layers depending on services, mobile speed, mobile power. | q |  | G2 |  | A1.2.28 A1.3.10 |  | The RTT does not impose constraints on the management of users between different cell layers in such a hierarchical deployment. |  |
| A3.4.2.4 | Fixed-wireless access |  |  |  |  |  |  |  |  |
| A3.4.2.4.1 | The proponents should indicate how well its technology is suited for operation in the fixed wireless access environment. Areas which would need evaluation include (not applicable to satellite component): – ability to deploy small BSs easily, – use of repeaters, – use of large cells, – ability to support fixed and mobile users within a cell, – network and signaling simplification. | q |  | G4 |  | A1.1.3 A1.3.5 A1.4.17 A1.4.7 A1.4.7.1 |  | The RTT is very much suited for fixed wireless access as well. Pico or Micro cells or Macro cells and repeaters are possible. Both fixed and mobile users can work in the same cell. Network signaling for fixed devices are simpler compared to mobile devices. |  |
| A3.4.2.4.2 | Possible use of adaptive antennas (how well suited is the technology) (not applicable to satellite component) Is RTT suited to introduce adaptive antennas? Explain the reason if it is. | q |  | G4 |  | A1.3.6 |  | Yes the RTT supports adaptive antenna/Beamforming solutions. | Explanation of the reason the RTT is suited is needed. |
| A3.4.2.4.3 | Existing system migration capability | q |  | G1 |  | A1.4.16 |  | NA |  |
| A3.5 | Implication on network interface |  |  |  |  |  |  |  |  |
| A3.5.1 | Examine the synchronization requirements with respect to the network interfaces. Best case : no special accommodation necessary to provide synchronization. Worst case : special accommodation for synchronization is required, e.g. additional equipment at BS or special consideration for facilities. | q |  | G4 |  | A1.4.3 |  | Synchronization of the BSs across the network is required and this is typically accomplished using GPS. |  |
| A3.5.2 | Examine the RTTs ability to minimize the network infrastructure involvement in cell handover. Best case : neither PSTN/ISDN nor mobile switch involvement in handover. Worst case : landline network involvement essential for handover. | q |  | G3 |  | A1.2.24 A1.4.6.1 |  | Handover within the same ASN (Access Service Network) does not involve the CSN (Core Service Network). In most handover scenarios with neighboring cells there is minimal involvement of the CSN. Only the BS and ASN GW may need to be involved in these scenarios. |  |
| A3.5.3 | Landline feature transparency |  |  |  |  |  |  |  |  |
| A3.5.3.1 | Examine the network modifications required for the RTT to pass the standard set of ISDN bearer services. Best case : no modifications required. Worst case: substantial modification required, such as interworking functions. | q |  | G1 |  | A1.4.7.1 |  | ISDN is supported as an application running over the IP protocol and is not natively supported. As voice is supported using Voice over IP protocols, the use of ISDN is only involved interworking functions between the IP networks and PSTN. |  |
| A3.5.3.2 | Examine the extent of the PSTN/ISDN involvement in switching functionality. Best case : all switching of calls is handled by the PSTN/ISDN. Worst case : a separate mobile switch is required. | q |  | G2 |  | A1.4.6 A1.4.8 |  | PSTN/ISDN is not used for switching within the IP network. |  |
| A3.5.3.3 | Examine the depth and duration of fading that would result in a dropped call to the PSTN/ISDN network. The robustness of an RTTs ability to minimize dropped calls could be provided by techniques such as transparent reconnect. | Q and q |  | G3 |  | A1.2.24 A1.4.14 |  | Voice is supported as an application over the RTT. The robustness of the link maintained is implementation dependent. The RTT supports HARQ and hence can operate in higher Packer Error Rates up to 10%. | Depth of fading and duration tolerated are not supplied |
| A3.5.3.4 | Examine the quantity and type of network interfaces necessary for the RTT based on the deployment model used for spectrum and coverage efficiencies. The assessment should include those connections necessary for traffic, signalling and control as well as any special requirements, such as soft handover or simulcast. | Q |  | G2 |  | A1.2.30 A1.2.30.1 A1.4.9 |  | The RTT design is to minimize impacts on the network. All the connections necessary for traffic, signaling and control terminate on the BS for PHY/MAC layer. The Radio Resource Management functions implemented over the IP protocol reside in the ASN. So most RTT configuration parameters are controlled on the BS which is interfaced using an IP connection to the ASN-GW . |  |
| A3.6 | Handportable performance optimization capability |  |  |  |  |  |  |  |  |
| A3.6.1 | Isolation between transmitter and receiver Isolation between transmitter and receiver has an impact on the size and weight of the handportable. | Q |  | G2 |  | A1.2.2 A1.2.2.1 A1.2.2.2 |  | As the RTT is a TDD based technology, no specific isolation requirements exist. | Transmitter/Receiver isolation requirement is not specified For example requirements for transmit/receive guard time (RTG/TTG) and switch isolation (or equivalent) need to be specified |
| A3.6.2 | Average terminal power output P0 (mW) Lower power gives longer battery life and greater operating time. | Q |  | G2 |  | A1.2.16.1.2 |  | This is implementation dependent. The terminals have different power classes to which they belong as shown in A3.2.2.2.2. | Average transmitter power is not specified |
| A3.6.3 | System round trip delay impacts the amount of acoustical isolation required between hand portable microphone and speaker components and, as such, the physical size and mechanical design of the subscriber unit. NOTE 1 – The delay of the codec should be that specified by ITU-T for the common generic voice bearer and if there are any proposals for optional codecs include the information about those also. (For the satellite component, the satellite propagation delay is not included.) | Q and q |  | G2 |  | A1.3.7 A1.3.7.1 A1.3.7.2 A1.3.7.3 |  | The Round trip delay will be well within the ITU-T specified limits for a typical Voice application that may be implemented using the RTT. | Round trip delay is not supplied |
| A3.6.4 | Peak transmission power | Q |  | G1 |  | A1.2.16.1.1 |  | This is not limited by RTT but by regulations. The peak terminal power output P0 = 1000 mW (Power class 3). Also see A3.2.2.2.2 for more details. | Maximum peak MS transmit power is specified as 1 W (Power Class 3) however, A3.2.2.2 and A1.2.16 gives a Power Class 4 with an unspecified maximum power. |
| A3.6.5 | Power control characteristics Does the proposed RTT utilize transmitter power control? If so, is it used in both forward and reverse links? State the power control range, step size (dB) and required accuracy, number of possible step sizes and number of power controls per second, which are concerned with mobile station technology complexity. |  |  |  |  |  |  | Yes the RTT does utilize transmitter power control for both Downlink and Uplink. | Frequency of power control is not specified. |
| A3.6.5.1 | Power control dynamic range Larger power control dynamic range gives longer battery life and greater operating time. | Q |  | G3 |  | A1.2.22 A1.2.22.3 A1.2.22.4 |  | The minimum power control dynamic range is 45 dB. |  |
| A3.6.5.2 | Power control step size, accuracy and speed | Q |  | G3 |  | A1.2.22 A1.2.22.1 A1.2.22.2 A1.2.22.5 |  | The accuracy for power level control can vary from ± 0.5 dB to ± 2 dB depending on the power control step size. Single step size m ¦ Required relative accuracy ¦m¦ = 1dB¦ ± 0.5 dB ¦m¦ = 2dB¦ ± 1 dB ¦m¦ = 3dB¦ ± 1.5 dB 4dB< ¦m¦< = 10dB¦ ± 2 dB Two exception points of at least 10 dB apart are allowed over the 45 dB range, where in these two points an accuracy of up to +/- 2 dB is allowed for any size step. |  |
| A3.6.6 | Linear transmitter requirements | q |  | G3 |  | A1.4.10 |  | Linear transmitters are used on the BS and MS. | Entry specifies that linear transmitters are used but no requirements are specified. |
| A3.6.7 | Linear receiver requirements (not applicable to satellite) | q |  | G3 |  | A1.4.11 |  | Linear receivers are used on the BS and MS. | Entry specifies that linear receivers are used but no requirements are specified, while A1.4.11 specifies that linear receivers are not required. |
| A3.6.8 | Dynamic range of receiver The lower the dynamic range requirement, the lower the complexity and ease of design implementation. | Q |  | G3 |  | A1.4.12 |  | 80dB for the MS receiver and 65dB for the BS receiver |  |
| A3.6.9 | Diversity schemes Diversity has an impact on hand portable complexity and size. If utilized describe the type of diversity and address the following two attributes. | Q and q |  | G1 |  | A1.2.23 A1.2.23.1 A1.2.23.2 |  | MIMO and Beamforming are supported. Within the MIMO scheme both Transmit Diversity and Spatial Multiplexing are supported. |  |
| A3.6.10 | The number of antennas | Q |  | G1 |  | A1.2.23.1 |  | BS: 2 Tx, 2 Rx MS: 1 Tx, 2 Rx |  |
| A3.6.11 | The number of receivers | Q |  | G1 |  | A1.2.23.1 |  | BS: 2 Receivers MS : 2 Receivers |  |
| A3.6.12 | Frequency stability Tight frequency stability requirements contribute to handportable complexity. | Q |  | G3 |  | A1.4.1.2 |  | BS frequency tolerance ≤ ± 2ppm of carrier frequency BS to BS frequency accuracy ≤ ± 1% of subcarrier spacing MS to BS frequency synchronization tolerance ≤ 2% of the subcarrier spacing |  |
| A3.6.13 | The ratio of “off (sleep)” time to “on” time | Q |  | G1 |  | A1.2.29 A1.2.29.1 |  | This implementation dependent and is programmable by the BS or MS implementations. | Sleep ratio is not specified |
| A3.6.14 | Frequency generator step size, switched speed and frequency range Tight step size, switch speed and wide frequency range contribute to handportable complexity. Conversely, they increase RTT flexibility. | Q |  | G2 |  | A1.4.5 |  | Frequency step size : 200 and 250 KHz Switched speed : 200 μsec Frequency range : 5, 10 MHz | Frequency range of the synthesizer is specified to be 5 or 10 MHz, however it is unreasonable to have a synthesizer range be the same as the Bandwidth of the technology. Clarify step size. |
| A3.6.15 | Digital signal processing requirements Digital signal processing can be a significant proportion of the hardware for some radio interface proposals. It can contribute to the cost, size, weight and power consumption of the BS and influence secondary factors such as heat management and reliability. Any digital circuitry associated with the network interfaces should not be included. However any special requirements for interfacing with these functions should be included. This section of the evaluation should analyse the detailed description of the digital signal processing requirements, including performance characteristics, architecture and algorithms, in order to estimate the impact on complexity of the BSs. At a minimum the evaluation should review the signal processing estimates (MOPS, memory requirements, gate counts) required for demodulation, equalization, channel coding, error correction, diversity processing (including Rake receivers), adaptive antenna array processing, modulation, A-D and D-A converters and multiplexing as well as some IF and baseband filtering. For new technologies, there may be additional or alternative requirements (such as FFTs). Although specific implementations are likely to vary, good sample descriptions should allow the relative cost, complexity and power consumption to be compared for the candidate RTTs, as well as the size and the weight of the circuitry. The descriptions should allow the evaluators to verify the signal processing requirement metrics, such as MOPS, memory and gate count, provided by the RTT proponent. | Q and q |  | G1 |  | A1.4.13 |  | These are again implementation dependent. | Digital signal processing requirements are not supplied. At a minimum the evaluation should review the signal processing estimates (MOPS, memory requirements, gate counts). |
| A3.7.1.1 | Base site coverage efficiency The number of base sites required to provide coverage at system start-up and ongoing traffic growth significantly impacts cost. From § 1.3.2 of Annex 2, determine the coverage efficiency, C (km2/base sites), for the lowest traffic loadings. Proponent has to indicate the background of the calculation and also to indicate the maximum coverage range. | Q |  | G1 |  | A1.3.1.7 A1.3.1.7.1 A1.3.1.7.2 A1.3.4 |  | 80-95% at system startup 95-100% in a mature system See section 2.2.4.2 for more details. | Base site coverage efficiency is not supplied. The evaluation group’s outage simulation results indicate a significant outage problem with the simulation assumptions stated in the submission. See Appendices 1, 2, and 5. Link budget constructed by the evaluation group indicates significant discrepancies from the link budgets given in 8F/1079 Rev 1. See Appendix 4. |
| A3.7.1.2 | Method to increase the coverage efficiency Proponent describes the technique adopted to increase the coverage efficiency and drawbacks. Remote antenna systems can be used to economically extend vehicular coverage to low traffic density areas. RTT link budget, propagation delay system noise and diversity strategies can be impacted by their use. Distributed antenna designs – similar to remote antenna systems – interconnect multiple antennas to a single radio port via broadband lines. However, their application is not necessary limited to providing coverage, but can also be used to economically provide continuous building coverage for pedestrian applications. System synchronization, delay spread, and noise performance can be impacted by their use. | q |  | G1 |  | A1.3.5 A1.3.6 |  | MIMO and Beamforming can be used to increase coverage efficiency. Remote or Distributed antenna systems can also be used. However the use of these methods is deployment scenario specific based on the implementations. |  |
| A3.7.2 | Satellite Normalized power efficiency Supported information bit rate per required carrier power-to-noise density ratio for the given channel performance under the given interference conditions for voice Supported information bit rate per required carrier power-to-noise density ratio for the given channel performance under the given interference conditions for voice plus data mixed traffic. | Q |  | G1 |  | A1.3.2.4 A1.3.2.4.1 A1.3.2.4.2 |  | NA |  |

# 4	References

[1]	IEEE Std 802.16e-2005, Amendment for Combined Fixed and Mobile Broadband Wireless Access Systems, December 2005 http://standards.ieee.org/getieee802/download/802.16e-2005.pdf

[2]	IEEE Std 802.16™-2004, IEEE Standard for Local and metropolitan area networks: Part 16: Air Interface for Fixed Broadband Wireless Access Systems, June 2004 http://standards.ieee.org/getieee802/download/802.16-2004.pdf

[3]	WiMAX Forum™ Mobile System Profile, Release 1.0 Approved Specification, Revision 1.2.2: 2006-11-17

http://www.wimaxforum.org/technology/documents/WiMAX_Forum_Mobile_System_Profile_v1_2_2.pdf

[4]	Mobile WiMAX – Part I: A Technical Overview and Performance Evaluation, August 2006, WiMAX Forum

http://www.wimaxforum.org/news/downloads/Mobile_WiMAX_Part1_Overview_and_Performance.pdf

[5]	Mobile WiMAX Network Architecture and Specifications

http://www.wimaxforum.org/technology/documents/WiMAX_NWG_Stage_2_VandV_Readiness_Draft.zip

[6]	3GPP TSG-RAN-1, "Effective SIR Computation for OFDM System-Level Simulations", R1-03-1370, Meeting #35, Lisbon, Portugal, November 2003

[7]	Hujun Yin and Siavash Alamouti, “OFDMA – A Broadband Wireless Access Technology”, IEEE Proc. of Sarnoff Symposium, March 2006

Table 23

The purpose of this Generic link budget is not clear. Also this link budget uses 38 kbit/s and 115 kbit/s for the Outdoor to Indoor Pedestrian test environment which is less than the mandated data rate of 384 kbit/s.

Appendices

Appendix 1

Downlink outage analysis of IP-OFDMA IMT-2000 RTT submission

# Introduction

This appendix provides the results of a system simulation study of IP-OFDMA carried out in a 19 cell deployment of 3-sectored cells with 10 users per sector. The study was carried out as part of the evaluation of the submission, 8F/1079 . The study has determined that the compliance of that submission to the coverage requirement of 95% for data users is highly questionable.

The performance evaluation in this document uses the assumptions stated in the submission, 8F/1079(Rev.1) . This document is related to another document pertaining to an evaluation of the performance claims within 8F/1079(Rev.1) Error: Reference source not foundError: Reference source not found

# References

ITU-R Recommendation M.1225 “Guideline for evaluation of radio transmission technologies for IMT-2000”

Document 8F/1079(Rev.1) “Additional technical details supporting IP-OFDMA as an IMT-2000 terrestrial radio interface” (WiMAX Forum)

# Abbreviations

| Abbrev. | Description |
| --- | --- |
| ACK (Ack) | Acknowledgment |
| ARQ | Automatic repeat Request |
| BS | Base Station |
| CDF | Cumulative Distribution Function |
| CQI | Channel Quality Information |
| DL-MAP | Resource Allocation Message |
| HARQ | Hybrid ARQ |
| IEEE | Institute of Electrical and Electronics Engineers |
| IMT | International Mobile Telecommunication |
| IP | Internet Protocol |
| IP-OFDMA | See IP and OFDMA |
| ITU | International Telecommunications Union |
| MAC | Media Access Control Layer |
| MAN | Metropolitan Area Network |
| MCS | Modulation and Coding Scheme |
| MIMO | Multi-Input Multi-Output |
| OFDMA | Orthogonal Frequency Division Multiple Access |
| PHY | Physical Layer |
| TCP | Transport Control Protocol |
| WiMAX | Industry monicker for systems conformant to the IEEE 802.16 standard |
| WiMAX Forum | Industry Group for specifying network, test and conformance for IEEE 802.16-based radio networks |

# Simulation conditions

The simulation parameters are detailed in Table 1 and are the same as those stated in section 2.3.3 of the proponent’s submission to the ITU .

Table 1

Simulation Parameters

| Parameters | Simulated Value | IP-OFDMA Parameters (From Table 13 of R01-WP8F-C-1079) |
| --- | --- | --- |
| Number of cells | 19 with wrap-around | 19 with users in center cell |
| Number of sectors | 3 | 3 |
| Users per sector | 10 | 10 |
| Frequency | 2.5 GHz | 2.5 GHz |
| Channel bandwidth | 10 MHz | 10 MHz |
| BS-to-BS distance | 2.8 km | 2.8 km |
| Minimum mobile-to-BS distance | 36 m | 36 m |
| Antenna Pattern | 70 degrees BW, 20 dB Front-Back Ratio | 70 degrees BW, 20 dB Front-Back Ratio |
| BS Antenna Height | 32 m | 32 m |
| Mobile Terminal Height | 1.5 m | 1.5 m |
| BS Antenna Gain | 15 dBi | 15 dBi |
| Mobile Antenna Gain | -1 dBi | -1 dBi |
| Penetration Loss | 10 dB (all users subject to constant) | 10 dB (all users subject to constant) |
| Deployment | Mixed deployment as in IP-OFDMA: 60 % pedestrian users, 40% vehicular A, distributed between medium speed (30% at 30 km/h) and high speed (10% at 120 km/h) | Mixed deployment unlike M.1225: : 60 % pedestrian users, 40% vehicular A, distributed between medium speed (30% at 30 km/h) and high speed (10% at 120 km/h) |
| BS Transmit Power | 43 dBm | 43 dBm |
| MS Noise Figure | 7 | 7 |
| Path Loss | Cost 231 | Cost 231 |
| Shadowing | 8 dB | 8 dB |
| Inter-BS shadowing correlation | 0.5 | 0.5 |
| Traffic | Full Buffer TCP Traffic | Full Buffer FTP Traffic |
| Reuse | 1x3x1 | 1x3x1 |
| BS Antennas | 2 | 2 |
| Mobile terminal Antennas | 2 | 2 |
| MIMO scheme | Dynamic Switching STC/SM | Dynamic Switching STC/SM |
| Antenna Correlation | No correlation or 0.7 | Not specified |
| DL-MAP MIMO Scheme | Cyclic Delay Diversity | Not specified |
| Max. HARQ Transmissions | 4 | 4 |

The simulator populates 19 cells with three sectors each with users subject to data traffic on the downlink. Each sector has an average of 10 users per sector that are distributed uniformly through the entire system. The 19 cells form a consistent deployment in a topological sense so that every cell in the simulator has the same interference statistics as any other. Multiple runs, each of 150 seconds duration of the simulation are used to sufficient averaging of statistics. The TCP layer is given enough time to reach steady state before statistics are gathered.

In our study, an attempt has been made to model performance of the mechanism for resource allocation using the DL-MAP message. In addition, it has been assumed that the DL-MAP message is coded at the lowest rate of Rate 1/2 CTC with QPSK modulation, using the maximum number of 6 repetitions of the basic code word. Since two transmit antennas are available, cyclic delay diversity is used on the DL-MAP in order to provide better diversity, improving performance for worst-case users. While Hybrid ARQ may be used to improve the performance of the user data, DL-MAP messages have to be received correctly each time so that a terminal might know its allocation and receive data appropriately.

Users are chosen using a proportional fairness criterion and allocated a data rate as per their channel conditions. Channel estimation is ideal, as is CQI measurement and feedback. The allocation is signalled to the user and the data transfer is attempted if the signalling channel is error-free. Failure to receive the DL-MAP message results in an implicit declaration of a block error for the data transmission, which could lead to a retransmission. For Chase HARQ, only incorrectly received data blocks from frames where the DL-MAP was correctly received are combined. Errors in the DL-MAP message lead to inefficiency in the HARQ process, and thus to throughput loss.

A mutual information based model is used for the link-to-system interface to model block error performance due to the fading channel realization. The same model is also used at the terminal to determine the preferred MCS for link adaptation. Error-free signalling of such information to the BS is assumed.

We define outage as being a condition where there is no data communication.

Outage results

User data rate is defined as the average data rate of correctly received PHY layer bits seen by a user terminal, after taking the behaviour of TCP with respect to those bits into consideration.

When TCP is present in the stack, the transmitter sending to a severely disadvantaged user is constantly timing out for each transmitted packet, and eventually ceases operation due to an inability to come out of backoff. Consequently, that user gets zero data throughput on the downlink. No explicit dropping mechanism is used. However, implicit termination of TCP connections occurs as expected.

Figure 1 looks at performance with TCP using the distribution of user data rates. It is seen that up to 11% of the users have no data transmissions reaching them when the antennas are decorrelated, and 17% of users are unable to communicate when antenna correlation is assumed to be 0.7. These figures are unacceptable in respect to the requirement of 95% mandated within the document M.1225.

The poor outage performance is partly due to the inability of the MAC resource allocation mechanism to reach users that are deployed in cell sites that are spaced as indicated in the 8F/1079 document [2]. It is unclear from [2] if this aspect has been taken into account.

Figure 1

(a)

(b)

CDF of user data rates for full buffer traffic with TCP: (a) complete CDF, (b) CDF illustrating the outage performance. The figure has curves representing performance with antenna correlation of 0 and 0.7.  It is seen that 11% of users are not able to communicate when antenna correlation is 0, while 17% of users are in outage when the antennas are correlated.

Conclusions

There is one significant result from this study. TCP connections are not likely to be sustainable for disadvantaged users within an IP-OFDMA network with the parameter settings used in this evaluation. Since the conditions of the simulation study within this appendix are the same as those stated in the submission 8F/1079 Rev 1, we must deduce that the submission exposes serious omissions on the part of the proponent.

Since only overall throughput results have been used in spectral efficiency results submitted in the document, 8F/1079 [2], it is unclear that that outage effects have been modelled.  Obviously, throughput results are not very meaningful if a high percentage of users are in outage. It appears that the performance of the resource allocation mechanism is a significant contributor to outage, and should be taken into account in the evaluation.  Given the questions we raise about performance of the resource allocation, analysis of performance of the IP-OFDMA system cannot proceed without further information from the proponent.

Thus the main conclusion of this study is that the IP-OFDMA system is incapable of coverage efficiency of 95% as required by M.1225 . In fact, the coverage efficiency of the IP-OFDMA system is about 83-89% under the simulation conditions stated in the submission 8F/1079 .

Appendix 2

Uplink Outage Analysis of IP-OFDMA IMT-2000 RTT submission

# Introduction

This document provides the results of a system simulation study of IP-OFDMA carried out in a 19 cell deployment of 3-sectored cells with multiple users being simultaneously scheduled in each sector on the uplink, the number of such users being dependent on the service level being provided. The study, comprising this document and a companion downlink evaluation Error: Reference source not found, was carried out as part of the evaluation of the submission, 8F/1079 Rev 1 .. Our study has determined that the compliance of that submission to the coverage requirement of 95% for data users is highly questionable.

# References

ITU-R Recommendation M.1225 “Guideline for evaluation of radio transmission technologies for IMT-2000”

Document 8F/1079(Rev.1) “Additional technical details supporting IP-OFDMA as an IMT-2000 terrestrial radio interface” (WiMAX Forum)

Abbreviations

| Abbrev. | Description |
| --- | --- |
| MCS | Modulation and Coding Scheme |
| ARQ | Automatic repeat Request |
| BLER | Block Error Rate |
| BS | Base Station |
| CDF | Cumulative Distribution Function |
| CRC | Cyclic redundancy Check |
| CTC | Convolutional Turbo Coding |
| GMH | Generic MAC Header |
| HARQ | Hybrid ARQ |
| IEEE | Institute of Electrical and Electronics Engineers |
| IMT | International Mobile Telecommunication |
| IP | Internet Protocol |
| IP-OFDMA | See IP and OFDMA |
| ITU | International Telecommunications Union |
| MAC | Media Access Control Layer |
| MAN | Metropolitan Area Network |
| MIMO | Multi-Input Multi-Output |
| MRC | Maximal Ratio Combining |
| OFDMA | Orthogonal Frequency Division Multiple Access |
| PHY | Physical Layer |
| QPSK | Quadrature Phase-Shift Keying |
| RR | Round Robin |
| SINR | Signal to Interference and Noise Ratio |
| WiMAX | Industry monicker for systems conformant to the IEEE 802.16 standard |
| WiMAX Forum | Industry Group for specifying network, test and conformance for IEEE 802.16-based radio networks |

# Simulation conditions

The simulation parameters are detailed in Table 1 and are the same as those stated in section 2.3.3 of the proponent’s submission . This appendix will focus purely on performance of the IP-OFDMA system under the assumptions made by the proponent’s submission.

Table 2

Simulation Parameters

| Parameters | Simulated Value | IP-OFDMA Parameters (From Table 13 of R01-WP8F-C-1079) |
| --- | --- | --- |
| Number of cells | 19 with wrap-around | 19 with users in center cell |
| Frequency | 2.5 GHz | 2.5 GHz |
| Channel bandwidth | 10 MHz | 10 MHz |
| BS-to-BS distance | 2.8 km | 2.8 km |
| Minimum mobile-to-BS distance | 36 m | 36 m |
| Antenna Pattern | 70 degrees BW, 20 dB Front-Back Ratio | 70 degrees BW, 20 dB Front-Back Ratio |
| BS Antenna Height | 32 m | 32 m |
| Mobile Terminal Height | 1.5 m | 1.5 m |
| BS Antenna Gain | 15 dBi | 15 dBi |
| Mobile Antenna Gain | -1 dBi | -1 dBi |
| Penetration Loss | 10 dB (all users subject to constant) | 10 dB (all users subject to constant) |
| Deployment | Pedestrian B | Mixed deployment: 60 % pedestrian users, 40% vehicular A, distributed between medium speed (30% at 30 km/h) and 10% at 120 km/h |
| MS Transmit Power | 200 mW | 200 mW |
| BS Noise Figure | 4 | 4 |
| Path Loss | Cost 231 | Cost 231 |
| Shadowing | 8 dB | 8 dB |
| Inter-BS shadowing correlation | 0.5 | 0.5 |
| Traffic | Full Buffer or VoIP | Full Buffer |
| Reuse | 1x3x1 | 1x3x1 |
| BS Antennas | 2 | 2 |
| Mobile terminal Antennas | 1 | 1 |
| MIMO scheme | MRC | MRC |
| Antenna Correlation | No correlation | Not specified |
| Number of subchannels | 35 | 35 (assumed, as per specifications) |

The simulator populates 19 cells with three sectors each with users subject to data traffic on the uplink. Users are distributed uniformly through the entire system, and the number of users per sector is fixed for the duration of the simulation. The 19 cells form a consistent deployment in a topological sense so that every cell in the simulator has the same interference statistics as any other.  Fifty simulation runs were used with randomized user positions per run.

In our study, an attempt has been made to model performance on the uplink, concentrating on outage. Thus, no attempt has been made to perform a complete uplink system simulation. The simulation does not attempt to calculate the cell throughput; rather it attempts to characterize the performance of the most disadvantaged users.

The sub-channelization mechanism is used in IP-OFDMA to dynamically change the bandwidth allocated to users. Thus, it can be used to concentrate the mobile terminal power in a limited bandwidth so as to allow the signal to reach the BS with a significantly higher signal to noise ratio than can be supported if the entire bandwidth is used. In our simulation, we exploit the sub-channelization mechanism to allocate at most as many subchannels as users can support based on their signal to noise ratio characteristics and the level of service required. One uplink subchannel with 5 uplink slots (i.e, 15 symbols are allocated to data on the uplink), and rate ½ coding with QPSK and 6 repeats of the basic code word can support 40 bits over a 5 ms frame, which is insufficient to carry even a bandwidth request header or Generic MAC Header (GMH). Nevertheless, we shall evaluate the performance with this case, and see that a significant number of users still are unable to achieve acceptable performance.  Later, we shall vary the minimum subchannel allocation to the disadvantaged users as a function of the type of traffic.

In each run of the simulation, subchannels are allocated to users in each cell depending on an SNR target that is set to achieve a 10% BLER on the first transmission. Multiple subchannels are allocated to users that have sufficient SNR in proportion to the number they can support given the target SNR, but each user is allocated a minimum number of subchannels even if his long term average SNR is insufficient to meet the target. The allocation is kept constant for the duration of the simulation run and does not change with instantaneous changes in the channel characteristics. HARQ is used as a means to improve the SINR and lower the BLER using multiple transmissions.

Power control is used on the uplink to set a maximum target SNR of 10 dB on the allocated bandwidth. A mobile terminal that is not capable of achieving this target transmits at maximum power over a single subchannel.

The BLER performance of the users after four HARQ transmissions is used to characterize outage. Only the most robust modulation and coding scheme is used (QPSK, rate ½ CTC with six repeats). Maximal ratio combining is used at the two receiver antennas, and Chase combining is used at the receiver to combine multiple transmissions. The instantaneous SINR from the different transmissions is combined to model the effective combined SINR after HARQ.  A mutual information based model is used for the link-to-system interface to model block error performance due to the frequency variations characterizing the fading channel realization.  The Pedestrian B channel model is used for all users with a speed of 3 mph. The delay for a retransmission is 20 ms.  Ideal ACK/NACK signalling and assignment mechanisms (UL-MAP) are assumed.

Two cases are considered for the scheduling. The entire ensemble of fading realizations is used to approximate the BLER performance of round-robin scheduling. The performance of proportional fairness scheduling, on the other hand, is evaluated using only those realizations where the instantaneous channel energy is above the mean. This sort of opportunistic scheduling tends to favour IP-OFDMA in comparison to what proportionally fair scheduling will actually yield. In reality, delay constraints will prevent the system from scheduling users only when their instantaneous channel values are above their fading mean values.

The traffic cases considered, the minimum subchannel allocation and the number of users per sector are defined in Table 2. The number of users per sector is chosen so that they can fit within the available 35 channels with margin.  The users in the non-disadvantaged conditions will receive more than the minimum allocation in accordance with the SNR seen by them.

Table 3

Traffic cases considered and their characteristics

| Traffic case | MAC Header bytes | HARQ CRC bytes | MAC CRC bytes | RTP/UDP/IP Compressed header bytes | Encryption Overhead bytes | User data bytes | Total Bytes | Minimum subchannel allocation at most robust MCS | Users per sector |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference, impractical | 0 | 2 | 0 |  |  | 3 | 5 | 1 | 10 |
| Only MAC header or Data without MAC-level ARQ | 6 | 2 | 0 |  |  | 2 | 10 | 2 | 10 |
| Data with MAC-level ARQ | 6 | 2 | 4 |  |  | 1 | 15 | 3 | 8 |
| Data with MAC-level ARQ and AES-CCM encryption | 6 | 2 | 4 |  | 12 | 1 | 25 | 5 | 5 |
| G. 729 Voice, no MAC-level ARQ, no AES-CCM | 6 | 2 | 0 | 2 | 0 | 20 | 30 | 6 | 4 |
| G.729 Voice, no MAC-level ARQ, AES-CCM encryption | 6 | 2 | 0 | 2 | 12 | 23 (3 filler) | 45 | 9 | 3 |

It is to be noted that section 2.3.3 of the proponent’s submission to the ITU [2] only discusses data capacity, and not voice capacity. Nevertheless, the aim of the IP-OFDMA system is to provide voice services using VoIP over the data service. Thus, we have chosen to also evaluate the performance of voice services using the same deployment model in section 2.3.3 for evaluation purposes.

# Outage results and discussion

In the following figures, we plot the CDF of the the BLER achieved after four Chase-combined transmissions for QPSK modulation with rate ½ CTC and six repetitions. The results are in a linear scale, with the lowest value indicating the percentage of users with BLER below 1%. The results with round-robin scheduling and opportunistic scheduling are plotted for the different traffic cases considered.  As expected, the performance degrades as the minimum subchannel allocation increases since the transmit power is spread out over a wider bandwidth.

For data transmission, the BLER after HARQ will have to be resolved by the outer MAC-layer ARQ scheme. In general, the ARQ mechanisms at the MAC layer require quite a low BLER to operate efficiently. Common targets for such BLER are below 1%; BLER in excess of 20% will cause significant protocol delays. Thus, the delay performance seen by these disadvantaged users will be unacceptable.

For voice and transparent data operation (i.e. data operation without MAC-level ARQ), the BLER after HARQ is an indicator of quality at the application layer. For voice, a BLER of 3% is typically the limit of acceptable quality.

Figure 2

CDF of BLER for the Reference Traffic Case

Figure 1 shows the results for the reference case, which, as discussed earlier, is impractical for any data transfer, Figure 1 shows that about 5% of users have a residual BLER of 20% after 4 HARQ transmissions. With more realistic traffic cases, we see significantly degraded performance compared to the reference cases.

Figure 3

CDF of BLER for MAC header transmission alone, or Data without MAC-level ARQ

For transparent data operation without MAC-level ARQ, from Figure 2, we find that almost 10% of users have a BLER in excess of 20%. This quality level is beyond what most applications can handle.  It is to be noted that this is the best case performance with minimal user data of only a couple of bytes per frame. With more user data, the number of subchannels needed will increase, and we expect to see worse performance.

Figure 4

CDF of BLER for data with MAC-level ARQ

Figure 5

CDF of BLER for data with MAC-level ARQ and AES-CCM Encryption

For non-transparent data with MAC-level ARQ, Figure 3 and Figure 4 indicate that about 13% and 20% of users, respectively for cases with and without encrypton, see a residual BLER of 20% after 4 HARQ transmissions. This has to be handled by the MAC-level ARQ, and is a higher level of errors than most ARQ mechanisms can handle without significant delays.

Figure 6

CDF of BLER for G.729 Voice with no MAC-level ARQ and no encryption

Figure 7

CDF of BLER for G.729 Voice with no MAC-level ARQ and AES-CCM Encryption

Figures 5 and 6 indicate that 25%-30% of users will have unacceptable voice BLER (above 3%) for the cases when AES-CCM encryption is disabled or enabled. This is an unacceptable level of outage for the voice service.

# Conclusions

A simulation study was performed that calculated the outage on the uplink with the deployment parameters stated for the data evaluation in the Document 8F/1079(Rev.1) [2]. This study indicates that a high number of users are in outage for data and voice services with the chosen deployment parameters. It can be expected that voice will be an important service that needs to be provided under any deployment, thus it is pertinent to evaluate voice performance under the assumptions for data deployment also, and it is found that there could be significant voice outage.

It is also to be noted that the performance evaluation in this study was done under the assumption of ideal channel estimation, ideal assignment mechanisms and ideal feedback mechanisms for ACK/NACK. The number of pilot symbols on the uplink of IP-OFDMA is limited, and the resultant inefficiency in channel estimation is expected to cause significant degradation in performance over and above those seen in this study.

While the Document 8F/1079(Rev.1) provides spectral efficiency estimates for data derived from overall throughput results, it is unclear that those results have modelled outage effects [2].  Indeed, throughput results are not very meaningful if a high percentage of users are in outage. Our results indicate that there is significant cause for concern on this front. Therefore, the evaluation of the performance of the IP-OFDMA system cannot proceed without further information from the proponent on the outage experienced by users in the system.

Appendix 3

Encryption and protocol overhead analysis

# Introduction

This document points out concerns regarding two protocol functions mentioned within the proponents’ submission, 8F/1079 Rev 1 . The IEEE 802.16 specification allows the use of Payload Header Suppression or Header Compression within the Convergence Sub-layer of the MAC protocol. Simulation information submitted with the proponents’ submission indicates that PHS can reduce the size of RTP/UDP/IP to 2 bytes of overhead.

In addition, the MAC layer defines the use of encryption to ensure privacy. However, it is not clear that this overhead is accounted for in the document 8F/1079.

As introduction, the reader is pointed to Section 1.2 of , where the “IP-OFDMA” Network reference Model is described, with subsections titled “Scalability, Extensibility, Coverage and Operator Selection,” and “Security,” that respectively describe the availability of packet header compression and privacy. In addition, the IEEE 802.16 specification, which “IP-OFDMA” is based on and draws from, has slightly more detailed descriptions of the same topics.

# References

ITU-R Recommendation M.1225 “Guideline for evaluation of radio transmission technologies for IMT-2000”.

Document 8F/1079 Revision 1 “Additional technical details supporting IP-OFDMA as an IMT-2000 terrestrial radio interface” (WiMAX Forum).

Song et al, “Header Compression specific Convergence layer,” Contribution IEEE C802.16e-04/523 (KAIST/Samsung), 2004.

Larzon et al, “The Lightweight User Datagram Protocol (UDP-Lite),” RFC 3828, IETF, 2004.

Bormann et al, “ROHC Framework and four profiles: RTP, UDP, ESP, and uncompressed,” IETF, RFC 3095, 2001.

IEEE, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems; Amendment 2: Physical and Medium Access Control Layers for Combined Fixed and Mobile Operation in Licensed Bands and Corrigendum 1,” 2006.

ITU, “Requirements and Objectives Template,” in Attachment 4 to Circular Letter 8/LCCE/47.

Casner, S. and V. Jacobson, "Compressing IP/UDP/RTP Headers for Low-Speed Serial Links", RFC 2508, IETF, February 1999.

# Abbreviations

| Abbrev. | Description |
| --- | --- |
| AES | Advanced Encryption Standard |
| CRTP | Compressed RTP |
| ESP | Encapsulating Security Payload |
| IEEE | Institute of Electrical and Electronics Engineers |
| IETF | Internet Engineering Task Force |
| IMT | International Mobile Telecommunication |
| IP | Internet Protocol |
| ITU | International telecommunications Union |
| MAC | Media Access Control |
| MAN | Metropolitan Area Network |
| OFDMA | Orthogonal Frequency Division Multiple Access |
| PHS | Payload Header Suppression |
| PHY | Physical Layer |
| RFC | Request for Comments |
| ROHC | Robust Header Compression |
| RTP | Real Time Protocol |
| RTT | Radio Transmission Technology |
| UDP | User Datagram Protocol |
| UGS | Unsolicited Grant Service |
| VoIP | Voice over IP |
| WiMAX | Industry monicker for systems conformant to the IEEE 802.16 standard |
| WiMAX Forum | Industry Group for specifying network, test and conformance for IEEE 802.16-based radio networks |

Header compression and payload header suppression

In Section 2.3.1.2 of the Document 8F/1079, it is assumed that for voice telephony, all protocol headers can be suppressed except for the RTP sequence number which amounts to 2 bytes. We interpret the term “suppressed” to mean the application of PHS as defined in the IEEE 802.16 specification . Nevertheless, we analyze this claim under two conditions: the use of PHS and the use of more robust header compression schemes such as ROHC.

Payload header suppression

Payload Header Suppression (PHS) relies on suppressing unchanging fields in the header. The known fields of the header are reconstructed in the receiver. In reality, conventional PHS is incapable of suppressing the 12-byte protocol header below 7 bytes .  The fields that are incompressible are denoted in Table 4.

Table 4

Fields in the RTP header that cannot be suppressed

| Field | Bits |
| --- | --- |
| Marker | 1 |
| Payload Type | 7 |
| Seq. Num | 16 |
| Time stamp | 32 |

In addition, the UDP checksum of 2 bytes is usually present in most stack implementations. Within the IP Header, the IPv4 header checksum, IPv4 identification (IP-ID), IPv4 Total Length or IPv6 Payload length fields cannot be suppressed. This is especially true for the IP-ID which changes every packet. While vendor-specific implementations of the stack can suppress many of these fields, it is not recommended for several reasons:

- Creation of specialized suppression rules at the base station leads to non-standard implementations and will increase cost and complexity.

- Suppression of the checksum at the terminal will have to be confined to specialized devices. Moreover general purpose devices such as computers will have to live with inclusion of the checksum.

- Standardization of such schemes is unnecessary since better header compression mechanisms such as ROHC are readily available.

Also, IPv6 mandates the use of UDP checksums .

In the conventional sense, PHS can therefore reduce the 40-byte or 60-byte RTP/UDP/IP packet to a total of 15 bytes with IPv4 and 11 bytes for IPv6. In the presence of channel errors, the average header length will proportionally increase due to the need to update the full header periodically.

# CRTP and ROHC

CRTP is a well understood method of compressing RTP/UDP/IP headers and is described in RFC 2508 . As with TCP/IP header compression, CRTP relies on transmission of delta-changes between successive packets to ensure that a minimum of header information is sent over the link layer. CRTP can compress the header to a minimum of 4 bytes including the UDP checksum.

A better method of compressing RTP/UDP/IP headers is the Robust Header Compression described in RFC 3095 . ROHC offers varying levels of compression depending on the nature of the protocol headers. For a voice telephony call, ROHC can probably compress the 40-byte RTP/UDP/IPv4 header to a steady state value of 3-4 bytes, including the 2-byte UDP checksum and depending on the profile used.    Thus, the assumed header size for the “suppression” used in 8F/1079 is lower than the minimum possible by at least 1 byte.

It is also possible that a flow will see changes in IPv6 Hop Limit, IPv4 Time to Live, IPv6 Traffic Class, IPv4 Type of Service. ROHC can handle all these changes by sending a few (3-6) octets extra while it is unclear how PHS would handle such changes.

One concern about all header compression schemes is the fact that they do not perform very well at high speech frame error rates. At high loads, a significant number of users (up to 5%) could have frame error rates exceeding 3%. In the presence of a long run of frame errors, header information is lost and a context update is needed to recover context. CRTP has been designed for relatively low error rate channels, and does not perform very well in the presence of error rates likely to be seen on wireless links. In the presence of channel errors, CRTP will have to rely on periodic context updates of the full header to recover context at the receiver for every frame error. The average header length in an error-prone channel will therefore be longer than the minimum of 4 bytes including the UDP checksum.

If ROHC is used in unidirectional mode, then long runs of frame errors will require that the receiver wait for a periodic context update. In bi-directional mode, ROHC can signal the transmitter to send a context update expediently; however, the delay in feedback will cause a temporary increase in frame errors. The error resilience of ROHC is configurable to handle between 3-13 consecutive errors. Yet, a long term FER of 3% can lead to occasional bursts of errors that require context updates. We do not expect this to be a dominant effect due to the error-resilience of the design.

# Encryption overhead

Section 1.4.7 of 8F/1079 specifically indicates that “AES-CCM is the cipher used for protecting all the user data over the “IP-OFDMA” MAC interface.” Also, the requirements and objectives template for IMT-2000 systems published as Circular letter 8/LCCE/47  specifies that candidate technologies must have PSTN/ISDN equivalent security and support for encryption as objectives. In addition, a candidate technology has a requirement to support encryption and maintain encryption when roaming and during handover.

It is not clear if the encryption overhead of AES (12 bytes per MAC frame) is accounted for in the data and voice simulations provided as part of 8F/1079. The proponents indicate that 32 bytes of “grant” are provided per user with UGS. This is insufficient to support a voice packet with protocol, MAC and encryption headers.

# Conclusion

The voice capacity results in A1.3.1.5.1 and A.3.1.1.1 of the proponents’ submission are brought into question in light of the arguments within this appendix. Likewise, data capacity results in A.1.3.1.5.2 and A.3.1.1.2 are also questionable.

Appendix 4

Link budget analysis of IP-OFDMA IMT-2000 RTT submission

# Introduction

The link budget calculation in section 2.3.4 of the 8F/1079 submission [1] from the Wimax Forum was examined.  A link budget taking into account different characteristics of the system is presented.  We find that the link budget presented in [1] is optimistic.

# References

Document 8F/1079(Rev.1) “Additional technical details supporting IP-OFDMA as an IMT-2000 terrestrial radio interface” (WiMAX Forum).

Table 5

Link budget calculation for “IP-OFDMA” DL and UL for Voice

| Parameter | DL Value | UL Value |
| --- | --- | --- |
| Traffic channel power | 30 dBm | 23 dBm |
| Antenna gains +losses | 11 dB | 11 dB |
| Time of Transmit | -26.5 dB (22 symbols) | -28.1 dB (15 symbols) |
| Total Energy available at source | 14.5 dB | 5.9 dB |
| Bits to be sent | 25.3 dB (336 bits) | 25.3 dB (336 bits) |
| No (including NF) | -169 | -169 |
| Eb/No at source | 158.2 | 149.6 |
| Required Eb/No | 7.3 | 7.8 |
| Path Loss w/o Lognormal Fading Margin | 150.9 dB | 141.8 dB |
| Lognormal Fading Margin | 11.4 dB | 11.4 dB |
| Path Loss | 139.5 dB | 130.4 dB |
| Values from Table 20 | 156.6 dB | 154.6 dB |

Number of bits: 160 bits for voice (8kbps vocoder with 20ms framing) + 6 bytes GMH + 2 bytes CRC + 2 bytes RTP/UDP/IP + 12 bytes AES overhead

Pilot power has not been considered in the above link budget.  Nominally, this is 1.1 dB on the Downlink and 1.8 dB on the Uplink.

# Conclusion

The link budget figures shown above are considerably worse than those presented in 8F/1079(Rev.1).  The proponents are urged to modify the submitted link budgets accordingly.  System simulations should also take the modified link budget into account.

Appendix 5

Additional information for Outage Analysis of IP-OFDMA 
IMT-2000 RTT submission

# Introduction

We explore two aspects of the performance presented in section 2.3.3 of the 8F/1079 submission [1] from the WiMaX forum. We first examine the DL-MAP performance in light of results available from the literature. Next, we examine the deployment in section 2.3.3 in terms of the link budget necessary for the operation of the DL-MAP. Both explorations find serious deficiencies in the system simulation presented.

# References

Document 8F/1079 Revision 1 “Additional technical details supporting IP-OFDMA as an IMT-2000 terrestrial radio interface” (WiMAX Forum).

R. Bachu et al (Motorola), “Linear Interference Cancellation for Downlink MAP Reception in 802.16e,” PIMRC’2006.

# DL-MAP performance issues

In [2], Motorola has presented link and system simulation results for the performance of DL-MAP decoding using MRC and interference cancellation receivers.  The results of [2] show that link performance improvements due to the interference cancelling receiver are highly sensitive to the presence of multiple interferers, and the ratios of their powers. This phenomenon carries over to the system simulation, wherein it is seen that the gain of the interference cancellation receiver over the MRC receiver, while appreciable at high coding rates, is only of the order of 2% at low coding rates (e.g, rate ½ coding with 6 repetitions).   According to the paper,  a 1% or less FER for the DL-MAP message for 95% of the users in an environment with penetration loss was achieved only by using 4-antenna cyclic switched transmit diversity in conjunction with an interference cancelling receiver. While 8F/1079 details results for 2 transmit antennas, it was not specified within the document whether the resource allocation mechanism using the DL-MAP message was simulated while generating the capacity results.

The results of [3] had shown that there could be significant outage on the IP-OFDMA downlink due to insufficient performance of the DL-MAP message. The study in [3] had used an MRC receiver for the DL-MAP message in conjunction with 2-antenna cyclic switched transmit diversity. A question that was raised about [3] was whether the use of an interference cancelling receiver for the DL-MAP could solve the outage problem. The minimal improvement demonstrated by [2] for the interference cancellation scheme, particularly for low coding rates, indicates that the outage problem identified in [3] still remains.

Section 2.3.3 of [1] and the paper [2] use different values for the path loss model, the cell size, the transmit power, and the penetration loss.  To compare the two environments, we have plotted the received power as a function of distance from the base station for both cases. Note that any shadowing and fading losses are omitted from the path loss calculation, but these are expected to be similar for both cases. In addition, these will only decrease the available signal power. Figure 1 shows the results of the comparison.

Figure 8

Received power in the two environments

Figure 1 shows that, at the claimed cell size in section 2.3.3 of 8F/1079 [1], the received power is almost 13 dB lower than that at the cell edge in [2]. Thus, the simulation of [1] penalizes the worst-case users significantly with respect to noise. It is well-known that interference cancellation mechanisms work well mainly in good signal conditions. Also, the simulation in [1] can utilize a maximum of 2-antenna cyclic switched transmit diversity, and will thus have worse performance than the system of [2] even under similar conditions. The additional path loss can only worsen the system performance. Thus, it is highly questionable that acceptable outage performance can be met on the downlink by the simulation of [1], a conclusion that is supported by the results of [3].

# Issues with Link Budget and Cell Size

In section 2.3.4.2 of [1], a generic link budget is presented that is based on the system parameters and channel propagation model of section 2.3.3. With this generic link budget, the maximum path loss for a data rate on the downlink of 2.88 Mbps is found to be 133.7 dB, accounting for 5.56 dB of shadow fading margin, and 10 dB margin for penetration loss. Adjusting for the data rate corresponding to the DL-MAP message (using QPSK, rate ½ coding with 6 repeats), we find that the maximum path loss is 137.6 dB.

Figure 2 plots the path loss function of section 2.3.3 as a function of distance. The cell size used in section 2.3.3, and the maximum allowable path loss are also shown in the figure.  A large gap between the maximum path loss and the edge of the cell is clearly seen, and indicates that a high number of users will be unable to close the link for the DL-MAP message, thereby leading to outage.

Figure 9

Link budget and cell size aspects of 8F/1079 data simulation

From Figure 2 and the uplink link budget in section 2.3.4.2, it can be seen that similar outage problems exist with respect to uplink performance also, as was pointed out by the results of [4].

_______________

*	 	Submitted on behalf of ATIS’ Wireless Technologies and Systems Committee (WTSC).

*	Explanation is requested when the candidate SRTT checks the No box.

*	*	The source Recommendation suggests numerical limits for the overall delay, but provides no guidance about the measurement techniques. Moreover there is an apparent inconsistency with ITU-T Recommendation G.114, where the value of 40 ms is indicated as the ‘objective’ value. These issues are addressed in a liaison statement sent to the relevant ITU groups. Until TG 8/1 receives a response and resolves this issue, proponents should submit candidates providing delay values using the methodology specified in Recommendation ITU-R M.1225.

The RTT is purely a Packet switch data technology. Circuit switched data is not supported. But will support seamless interworking with circuit switched systems using media gateways and support for QoS classes.

*	Explanation is requested when the candidate SRTT checks the No box.

These are application specific and not mandated by the RTT. But applications may support this.

*	 	Descriptive information should be provided explaining how the candidate SRTT supports the concept specified in the Recommendation.

It has been assumed in this study that Full Buffer FTP Traffic is indistinguishable from Full Buffer TCP Traffic, meaning that the TCP layer exists in the data pipe.

In reality, the performance of the resource allocation channel is bound to be worse than what has been assumed if Sub-DL-MAP messages with higher coding rates are employed.

This means that the PHY overhead of 2 octets and the MAC overhead of 22 octets (10 octets with no encryption) have not been removed from the count of bits seen after each MAC frame of 5 ms. TCP behaviour with respect to correctly delivered bits is translated down to the PHY layer.