|  | مكتب تقييس الاتصالات |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | جنيف، 26 يناير 2010 |  |
| المرجع: |  | TSB Circular 84 COM 15/GJ | - إلى إدارات الدول الأعضاء في الاتحاد؛ - إلى أعضاء قطاع تقييس الاتصالات؛ - إلى المنتسبين إلى قطاع تقييس الاتصالات؛ |  |
| الهاتف: الفاكس: البريد الإلكتروني: |  | 6356 +41 22 730 +41 22 730 5853 tsbsg15@itu.int | نسخة إلى: - رئيس لجنة الدراسات 15 ونوابه؛ - مدير مكتب تنمية الاتصالات؛ - مدير مكتب الاتصالات الراديوية |  |

الموضوع:	استبيان بشأن سفن مد الكبلات والأجهزة القابلة للغمر

حضرات السادة/السيدات،

تحية طيبة وبعد،

1	قررت لجنة الدراسات 15 في اجتماعها الأخير (جنيف، 28 سبتمبر – 9 أكتوبر 2009)، في إطار الدراسات التي تجريها بموجب المسألة 8/15 (خصائص الأنظمة الكبلية البحرية بالليف البصري)، مراجعة التوصية ITU-T G.971 (الخصائص العامة للأنظمة الكبلية البحرية بالليف البصري) وتحديث التذييل I الحالي الذي يحتوي على البيانات المتعلقة بسفن مد الكبلات والأجهزة القابلة للغمر.

2	ومن المزمع تقديم التوصية  G.971المراجعة للموافقة عليها في الاجتماع القادم للجنة الدراسات 15 (31 مايو –
11 يونيو (2010.

3	وتحقيقاً لهذا الغرض فإن مساعدتكم ضرورية لإعادة النظر في المعلومات الواردة في التذييل I الحالي للتوصية G.971 وتحديثها. ولذا ندعوكم إلى تعديل، عند الاقتضاء، البيانات الحالية بشأن سفن مد الكبلات والأجهزة القابلة للغمر المبينة في الملحق 1 بهذه الرسالة المعممة التي تحتوي على نص التذييل I للتوصية G.971.

وإذا تم استبعاد بعض الأجهزة الواردة في القائمة، و/أو إنشاء سفن مد الكبلات وأجهزة قابلة للغمر جديدة منذ 2007، يرجى وصفها (باللغة الإنكليزية) على النحو المبين في الملحق 2.

4	ويرجى منكم إعادة كل هذه المعلومات إلى محرر التوصية G.971 قبل 31 مارس 2010:

M. K. Nakajima
NTT Access Network Service Systems Laboratories
1-7-1, Hanabatake, Tsukuba, Ibaraki
305-0805, Japon

البريد الإلكتروني:  nakaji@ansl.ntt.co.jp

الفاكس: +81 29 868 6142

5	وإنن‍ي أعوّل كثيراً على تعاونكم في التحقق قدر الإمكان من دقة ردودكم والحرص على أن تصل الردود إلى المقرر المشارك المذكور أعلاه قبل الموعد النهائي.

وتفضلوا بقبول فائق التقدير والاحترام.

مالكولم جونسون
مدير مكتب تقييس الاتصالات

الملحقات: 2

ANNEX 1 
(to TSB Circular 84)

Data on cable ships and submersible equipments of various countries

- 1.1	Cable ships

| Name of ship | Year of cons-truction | Dis-place-ment (tons) | Overall length (m) | Draft (m) | Normal speed (knots) | Range (auto-nomy) (nautical miles) | Number of tanks | Cable capacity |  |  | Cable gear |  |  |  | Max operating depth (m) | Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Cable |  | Re-peaters | Cable engine |  | Unwinding pulley |  |  |  |
|  |  |  |  |  |  |  |  | Cubic metres (m3) | Weight (tons) |  | Drum (diameter) (m) | Linear (pairs of wheels) | Bow sheave (diameter) (m) | Stern sheave (diameter) (m) |  |  |
|  |  |  |  |  |  |  | DENMARK Ships belonging to Tele Denmark |  |  |  |  |  |  |  |  |  |
| Peter Faber | 1982 | 3680 | 78.35 | Ice 3.8 Summer 5.0 | 13.0 | 7000 | 1 tank 1 hold | 310 230 | 600 400 | App. 10 | 3.0 |  | 2  3.0 | – | 4000 | Reinforced for operation in ice-filled waters. A-frame for ROV. Two hydraulic double-drum warping winches. |
| Lodbrog | 1985/ 2002 | 12'503 | 143.4 | 8.50 | 16.0 | 10'000 | 6 | 2940 | 5040 | 84 | 2  4.0 (25 t) | 2  6 (6 t) | – | 2  3.0 | All | Laying/burying and repair of all types of cables (coaxial, optical fibre and power cables). ROV capability, SWL 8 ton. |
|  |  |  |  |  |  |  | FINLAND 1) Ship belonging to Sonera Ltd |  |  |  |  |  |  |  |  |  |
| M/S Telepaatti | 1978 (modifi-cation) | 450 | 42.6 | 3.0 | 12 | – | 1 | – | 350 | – | 2 linear engines with 3 caterpillar tracks on each | 3.0 |  | 300 |  | Laying of all types of telecom cables. Specially equipped for cable route survey and cable repair. Fully automatic autopilot and DP-system. |
|  |  |  |  |  |  |  | 2) Ship belonging to YIT Primatel |  |  |  |  |  |  |  |  |  |
| c/s Telepaatti | 1978 Modifi-cation 1999 | 450 | 42.6 | 3.0 | 10.5 | – | 1 | 250 | 260 | – | – | 2 linear engines with 3 cater-pillar tracks on each | 3.0 | – | 300 | Laying of all types of telecom cables and <150 mm power cables. Specially equipped for cable route survey and cable repair. Fully automatic autopilot and DP-system. |
|  |  |  |  |  |  |  | FRANCE 1) Ships belonging to France Telecom Marine |  |  |  |  |  |  |  |  |  |
| Chamarel (formerly Vercors) | 1974 | 11'000 | 136 | 7.2 | 16.0 | 12'000 | 3 | 2425 | 4900 | 144 | 3.0 | 24 | 3.0 | Chute | All | Laying and repair of all types of telecom cables. Burying of cables with plough and 200 kW Hector 4. |
| Léon Thevenin | 1983 | 6800 | 107 | 6.24 | 15.0 | 10'000 | 2 + 1 | 1420 | 2000 | 11 | 3.4 | 12 | 3.0 | Chute | All | Laying and repair of all types of telecom cables. Burying of cables using 300 kW Hector 5. |
| Raymond Croze | 1983 | 6800 | 107 | 6.24 | 15.0 | 10'000 | 2 + 1 | 1420 | 2000 | 11 | 3.4 | 12 | 3.0 | Chute | All | Laying and repair of all types of telecom cables. Burying of cables using 250 kW Hector 3. |
| René Descartes | 2002 | 15'450 | 114.50 | 7.42 | 16.0 | 12'000 | 4 | 3250 | 5500 | 210 | 4.0 | 20 | Aft sheave 3.0 m | Sheave | All | Stem concept cable ship. Laying and repair of all types of telecom cables. Burying of cables with plough and 250 kW ROV Hector 6. |
|  |  |  |  |  |  |  | 2) Ships belonging to Alda Marine |  |  |  |  |  |  |  |  |  |
| Ile de Sein Ile de Batz Ile de Brehat | 2002 | 18'006 | 140.4 | 8.016 | 15.0 | 15'000 | 2 + 2 | 3000 | 5500 | 202 | 4.0 | 21 | NA | 3.0 | All | Laying and repair of all types of telecom cables. Burying of cables with. 2/3m Rock plough. Sea state 7 A-frame |
| Ile de Ré | 1983 rebuilt 2002 | 12'687 | 143.4 | 7.23 | 16.0 | 11'000 | 3 + 3 | 2900 | 4500 | 84 | 2 × 4.0 | NA | NA | 3.0 | All | Laying and repair of types of cable. ROV to 2500m. A plough is available. |
|  |  |  |  |  |  |  | ITALY 1) Ship belonging to Elettra TLC S.p.A |  |  |  |  |  |  |  |  |  |
| Teliri | 1996 | 6500 | 111.5 | 6.5 | 14.01 | 10'000 | 3 | 2000 | 2600 | 70 | 2  3.5 | 18 | 3 | 4 | All | Laying and repair optical fibre systems |
| Certamen (ex John Cabot) | 1966 rebuilt 1998 | 5000 | 96.6 | 7.3 | 12.0 | 8000 | 3 | 600 | 1900 | 24 | 1  3.0 | 18 (on the stern) + 6 (on the bow) | 3 | 3 | All | Laying, survey and repair optical fibre systems |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No more a Cable ship. Converted as pipelay ship in 2007 |
|  |  |  |  |  |  |  | ITALY 2) Ships belonging to Prysmian Cavi e Sistemi Energia S.r.l. |  |  |  |  |  |  |  |  |  |
| Giulio Verne | 1984 | 16'900 | 133.18 | 8.5 | 10 | 7000 | 2 | 2600 | 7000 | 10 | 6.0 (55 t) | 1 (Pads type 10 t) | – | 6.0 | All | Lay and repair from the stern |
|  |  |  |  |  |  |  | SPAIN Ships belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |  |  |  |  |
| Teneo | 1992 | 4000 | 81 | 5.7 | 14.5 | 4200 | 2 | 500 | 1000 | 20 | 2  3.5 | 1  9 | 2  3 | 1  3 | All | Lays and repairs of all types of telephone cables. |
|  |  |  |  |  |  |  | JAPAN 1) Ships belonging to Kokusai Cable Ship (KCS) |  |  |  |  |  |  |  |  |  |
| KDD Ocean Link | 1992 | 11'700 | 133.2 | 7.0 | 15 | 10'000 | Main 3 Spare 4 | 2600 | 4500 | 57 | 3.6 | 21 | 3.2 | 4.0 | All | Laying by linear engine. Lays and repairs all types of submarine cables. |
| KDD Pacific Link | 1997 | 11'207 | 109.0 | 7.5 | 11 | 10'000 | Main 2 Spare 2 | 2720 | 4500 | 50 | 3.6 | 20 | – | 3.0 | All | Laying by linear engine. Lays and repairs all types of submarine cables. |
|  |  |  |  |  |  |  | 2) Ships belonging to NTT World Engineering Marine Corporation (NTT-WE Marine) |  |  |  |  |  |  |  |  |  |
| Subaru | 1999 | 9557 | 123.3 | 7.0 | 13.2 | 8800 | Main 2 Spare 2 | 2770 | 4000 | 50 | 4.0 | 21 | – | 3.2 | All | Lays and repairs all types of telephone cables. |
| C/S VEGA | 1984 | 1336 | 74.3 | 4.6 | 13.5 | 4500 | 2 | 169 | 250 | – | 3.0 | 6 | 2.5 | 2.0 | All | Lays and repairs for non-powered telephone cable system. DP, ROV system |
|  |  |  |  |  |  |  | UNITED KINGDOM 1) Ship belonging to British Telecommunications plc |  |  |  |  |  |  |  |  |  |
| Sovereign | 1991 | 13'018 | 131 | 7.0 | 13.5 | 14'000 | 4 | 2800 | 6200 | 90 | 3.50 |  | 3.00 | 3.50 | All | Lays, repairs all types of coaxial and optical fibre cable. (Operated by C&W marine.) |
|  |  |  |  |  |  |  | UNITED KINGDOM 2) Ships belonging to Global Marine Systems Ltd |  |  |  |  |  |  |  |  |  |
| Alert | 1961 | 9477 | 130 | 7.1 | 14 | 10'000 | 3 | 1509 | 3100 | 48 | 2.98 |  | 2.98 | 2.98 | All | Laying by linear engine and seabed burial by plow. Lays/repairs all types of coaxial and optical fibre cables. |
| Cable Venture | 1962 | 16'983 | 153 | 8.97 | 12.5 | 10'000 | 4 + 1 (spare) | 5086 | 9000 | 400 | 2.80 |  | 3.00 | 3.39 | All | Laying by linear cable engine. Ploughs, lays and repairs armoured and lightweight cables. |
| Mercury | 1962 | 11'683 | 144 | 7.5 | 14.5 | 8000 | 3 | 2970 | 3500 | 144 | 3.05 |  | 3.50 | Chute 3.05 | All | Ditto (no plough). |
| Cable Enterprise | 1964 | 5759 | 113 | 5.84 | 13 | 8000 | 3 | 887 | 2150 | 30 | 2.8 |  | 3.00 | Chute 3.05 | All | Lays/repairs armoured cables. Repairs lightweight cables. (Note) |
| Monarch | 1975 | 4639 | 97 | 5.5 | 14 | 7000 | 4 | 417 | 850 | 12 | 3.00 |  | 3.00 | None | All | Lays/repairs armoured coaxial and optical fibre cables. Repairs lightweight coaxial and optical fibre cables. Detrenching/reburial by submersible jetting. |
| Iris | 1976 | 4639 | 97 | 5.5 | 14 | 7000 | 4 | 417 | 850 | 12 | 3.00 |  | 3.00 | None | All | Lays/repairs armoured coaxial and optical fibre cables. Repairs lightweight coaxial and optical fibre cables. |
| MV Cable Installer | 1980 | 6065 | 89.42 | 5 | 12 | 42 days | 4 | 840 | 1600 | None | 3.0 | 4-track pair | – | 3.0 | – | Repeaterless installation vessel fully DP Cegelec 901 system. |
| Seaspread | 1980 | 10'887 | 116 | 6.8 | 13 | 65 days | 2 | 1010 | 1701 | – | 2  3 | – | – | 3 | All | Lays/repairs by aft drums. Burial by plough. Lays/repairs armoured and lightweight cables. |
| Pacific Guardian | 1984 | 7526 | 116 | 6.32 | 14.0 | 8000 | 3 | 1416 | 3470 | 96 | 3.5 |  | 3.00 | 3.00 | All | Laying by linear cable engine. Lays and repairs armoured and lightweight cables. |
| Sir Elic Sharp | 1988 | 7526 | 115 | 6.3 | 13.5 | 9600 | 3 | 1416 | 1700 | 96 | 2  3.5 | – | 3 | 3 | All | Laying by linear cable engine. Repairs and lays armoured and lightweight cables. Post lay/repair burial by integral ROV. |
|  |  |  |  |  |  |  | UNITED KINGDOM 2) Ships belonging to Global Marine Systems Ltd |  |  |  |  |  |  |  |  |  |
| MV Cable Innovator | 1995 | – | 142 | 8.3 | 14.5 | 42 days | 4 | 4900 | 7500 | 180 | 4.0 | 21 pairs (min) | – | 4.0 | – | Simplex D/P system. Lays/repairs cables. |
|  |  |  |  |  |  |  | MARSHALL ISLANDS Ship belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |  |  |  |  |
| CS Coastal Connector | 1997 Conver-ted in 1996 | 6761 | 92.47 | 7.1 | 12.5 | 25'000 | 3 main 1 spare | 675 (main, total) 70 (spare) | 1600 | 30 | 2  3 | N/A | N/A | 2  3 | – | The CS Coastal Connector is a stern-laying design. She is capable of deploying the SCARAB II, SCARAB IV, and Pacific SCARAB I ROVs, as well as the Seabed Tractor. |
| CS Tyco Provider | 1978, Converted in 1999 | 14'500 | 139.4 | 7.6 | 14.5 | 20'000 | 5 | 3349 | 6000 | 100+ | 2  4 | – | – | 2  3 | – | The CS Tyco Provider is a stern-laying design. She is capable of deploying Sea Plow VIII. |
|  |  |  |  |  |  |  | NETHERLANDS ANTILLES Ship belonging to Tyco Submarine Systems Ltd. (charterer) |  |  |  |  |  |  |  |  |  |
| Dock Express 20 | 1983 | 21'731 | 169.52 | 8.79 | 12.5 | 20'500 | 3 main 2 spare | 4050 (main, total) 640 (spare, total) | 10 000 | 100+ | 1  3.0 | 1  3 module belt type | N/A | 2  3 |  | The Dock Express 20 is a stern-laying design. She is capable of deploying the SCARAB II ROV, as well as the Seabed Tractor and Sea Plow VI. |
|  |  |  |  |  |  |  | UNITED STATES OF AMERICA Ships belonging to AT&T |  |  |  |  |  |  |  |  |  |
| CS Charles L. Brown | 1954, Reflag-ged in 1985 | 4298 | 99.94 | 5.6 | 13 | 7550 | 3 | 660 | 1186 | 30+ | 2  3 | N/A | 2  3 | N/A | – | The Charles L. Brown is primarily a repair ship. She is not fitted with any stern-laying equipment. She is capable of deploying the SCARAB II ROV. |
| CS Global Link | 1990 | 16'375 | 145.7 | 8.08 | 15 | 10'000 | 3 main, 4 spare | 3258 (main, total) 164 (spare, total) | 6098 | 100+ | 2  3.7 | 1 Western Gear Tractor Type | 2  3 | 1 trough/ Chute type | – | The Global Link is capable of deploying the SCARAB II ROVs. |
|  |  |  |  |  |  |  | UNITED STATES OF AMERICA Ships belonging to AT&T |  |  |  |  |  |  |  |  |  |
| CS Global Mariner | 1993 | 15' 638 | 151.5 | 7.8 | 13.8 | 10'000 | 2 main, 3 spare | 2172 (main, total) 447 (spare, total) | 4999 | 80+ | 2  3.7 | 1 Dowty 21 pairs | 2  3 | 1 trough/ Chute type | – | The Global Mariner is capable of deploying the SCARAB II and SCARAB IV ROVs, as well as Sea Plow VII, Sea Plow VIII, and the Seabed Tractor. |
| CS Global Sentinel | 1991 | 16' 375 | 145.7 | 8.08 | 15 | 10' 000 | 3 main, 4 spare | 3258 (main, total) 164 (spare, total) | 6098 | 100+ | 2  3.7 | 1 Dowty 21 pairs | 2  3 | 1 trough/ Chute type | – | The Global Sentinel is capable of deploying the SCARAB II and SCARAB IV, and Pacific SCARAB I ROVs, as well as Sea Plow VII and Sea Plow VIII. |
| NOTE – Only relatively short cables are laid and only shore-end. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- 1.2	Submersible equipments

| Type of submersible | Weight (tons) | Overall length (m) | Width (m) | Height (m) |  | Trenching system |  | Trenching |  | Propulsion |  | Max operating depth (m) | Max pulling tension (tons) | Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | FRANCE Submersibles belonging to France Telecom Marine |  |  |  |  |  |  |  |  |  |  |
| ELISE2 Submersible Plough system | 17 | 7.60 | 2.90 | 2.95 |  | Ploughshare |  | Immediate burial up to 1.1 m |  | Towed by support ship |  | 1500 |  | Lay and bury all types of cables. |
| ELISE3 Submersible Plough system | 17 | 7.60 | 2.90 | 2.95 |  | Ploughshare |  | Immediate burial up to 1.1 m |  | Towed by support ship |  | 1500 |  | Lay and bury all types of cables. |
| Self-advancing buried system CASTOR2 | 12 | 7.0 | 2.40 | 3.00 |  | Trenching wheel or chain |  | Burial of existing cables down to 2 m |  | Tracked vehicle |  | 1000 |  | Burial of cables and pipes. Visual inspection. |
| ROVs HECTOR 3, 4, 5 & 6 | 9 | 4.0 | 3.50 | 2.10 |  | High-pressure water jets |  | Up to 1.5 m depth |  | Thrusters (inspection) Back drive (burial) |  | 2000 |  | Visual inspection, post-lay burial, cable location, cable manipulation, cable cutting. |
|  |  |  |  | FRANCE Submersibles belonging to France Telecom Marine |  |  |  |  |  |  |  |  |  |  |
| Remote control submersible Scorpio 2000 | 3.4 | 2.9 | 1.5 | 2.11 |  | High-pressure water jets |  | Up to 60 cm depth |  | Thrusters |  | 1000 |  | Visual inspection, post-lay burial, cable location/ manipulation/cutting. |
|  |  |  |  | ITALY 1) Submersibles belonging to Elettra TLC SpA |  |  |  |  |  |  |  |  |  |  |
| Plough Taurus 1 | 14 | 9 | 4.6 | 4.5 |  | Plough share |  | Up to 1 m |  | Towed by cable ship |  | 1500 | 50 | Lay and bury all types of cables. |
| Plough Taurus 2 | 16 | 9.5 | 4.5 | 5.1 |  | Plough share |  | Up to 1.5 m |  | Towed by cable ship |  | 1500 | 50 | Lay and bury all types of cables. |
| ROV – Phoenix 2 | 6.8 | 4.8 | 2 | 2.6 |  | High/low-pressure jetting |  | Up to 1.2 m |  | 8 Hydraulic thrusters |  | 1000 |  | Visual inspection, post-lay burial, cable location/manipulation/cutting. |
| ROV-T200 | Free-fly mode 6, Track mode 7 | 3.1 | 2 | 2.2 |  | High/low-pressure jetting |  | Up to 1.2 m |  | 4 vertical and 4 horizontal thrusters |  | 2500 |  | Visual inspection, post-lay burial, cable location/manipulation/cutting. |
|  |  |  |  | UNITED KINGDOM Submersibles belonging to Global Marine Systems Ltd |  |  |  |  |  |  |  |  |  |  |
| Submersible trencher | 17.0 | 6.6 | 4 | 3.4 |  | Fluidization and cutting jets and dredge pump |  | Up to 1 m depth with cutting and fluidization jets |  | Three vertical and four horizontal thrusters, track drive differential steering |  | 274 |  | Trench in existing cable and pipe. |
| Submersible Plough system | 9.75 | 6.1 | 2.6 | 2.6 |  | Ploughshare proceeded by disc |  | Immediate burial of cable on ploughing |  | Towed by support ship |  | 900 |  | Lay and bury cable, umbilical and pipe in one action giving full cable protection. |
| Remote control submersible 2 off Cirus A&B | 3.2 | 3.5 | 2.1 | 2.3 |  | Water jets |  | Trenching capability 0.3 m |  | Thrusters (7) |  | 1000 |  | Visual inspection, cable location/inspection/deburial, manipulation. Tools include cable cutter, cable gripper and 2 manipulators with line cutters. |
| Plough 2 off A&B | 14.5 | 9 | 4.1 | 4 |  | Passive blade |  | Trenching capability 1.0 m |  | Towed |  | 1000 |  | Steerable, repeater burial. |
| Remote control submersible ROV 128 | 7.5 | 2.9 | 1.8 | 2.0 |  | Jetting tool |  | Trenching capability 0.6 m |  | Tracked burial Thrusters survey |  | 1000 (burial) 2000 (survey) |  | Tools include cable cutter, cable gripper and 2 manipulators with line cutters. |
| Underwater vehicle- MARLIN | 7.8 | 4.191 | 2.438 | 3.175 |  | Burial skid |  | To 1.0 m (Optimized for 0-30 kPa soil) |  | Hydraulic driven thrusters |  | 2500 |  | Burial, deburial, inspection. Maintenance and repair. Tools include cable cutter, cable gripper. |
|  |  |  |  | UNITED KINGDOM Submersibles belonging to Global Marine Systems Ltd |  |  |  |  |  |  |  |  |  |  |
| Scarab I – Umbilically tethered ROV | 3.2 | 2.74 | 1.82 | 1.52 |  | Jetting tool |  | Up to 0.6 m |  | Thrusters: 2 vertical 4 vectored |  | 2000 |  | Cable detection and inspection. Visual survey. Cable manipulation and cutting Debris elimination. Cable and repeater burial/deburial. |
| Subtrack - ROV | 10.0 | 8.0 (Max) | 3.7 | 3.8 |  | Jetting tool |  | Burial to 1.0 m |  | Electro-hydraulic track drives |  | 1000 |  | Cable burial and deburial. Inspection. Maintenance and repair. |
| EUREKA: Deepwater burial + trenching system | 17 (Max) | 5.5 | 4.2 | 3.85 |  | Jetting tool Rock wheel cutter Mechanical chain excavator |  | 1 m 1.2 m 2.2 m |  | Electro-hydraulic track drives |  | 1500 |  | Capable of burying cable, small flexible flowlines and also rigid pipes. Can also debury cable and restore. Visual and electronic inspections. |
| Plough 5 | 14.0 | 9.0 | 4.6 | 3.7 |  | Passive blade |  | Variable from 0-1100 mm (600-900 mm in all conditions) |  | Towed |  | 1000 |  | Simultaneously lay and bury cables and umbilicals at varying depths. |
| Plough 6 and 7 | 14.0 | 9.0 | 4.6 | 3.7 |  | Passive blade |  | Max burial depth: 1100 mm |  | Towed |  | 1000 |  | Simultaneously lay and bury cables and umbilicals at varying depths. |
| Cable Plough 1000 mm | 14.4 | 9.75 | 4.1 | 3.9 |  | Passive blade |  | 1000 mm (Good conditions: 1100 mm; Repeaters/Joints: 500 mm) |  | Towed |  | 1000 |  | Simultaneously lay and bury cables and umbilicals at varying depths. |
|  |  |  |  | DENMARK Submersibles belonging to Telecom Denmark |  |  |  |  |  |  |  |  |  |  |
| Plough D | 13.5 | 9.0 | 4.6 | 3.7 |  | Plough share |  | Variable from 0-1100 mm (600-900 mm in all conditions) |  | Towed by host vessel |  | 1500 |  | Lay and bury telecom cables, power cables and umbilicals. Cables: Up to 120 mm (bury). Joints and repeaters: Up to 400 mm (pass). |
| Plough 7 | 13.5 | 9.0 | 4.6 | 3.7 |  | Plough share |  | Variable from 0-1100 mm (600-900 mm in all conditions) |  | Towed by surface vessel |  | 1000 |  | Lay and bury fibre optic cables, power cables and umbilicals. |
| Subtrack- Subsea tractor | 10.0 | 8.0 (Max) | 3.7 | 3.8 |  | Jetting tool |  | Burial to 1.0 m |  | Electro-hydraulic track drives |  | 1000 |  | Cable burial and deburial. Inspection. Maintenance and repair. |
|  |  |  |  | DENMARK Submersibles belonging to Telecom Denmark |  |  |  |  |  |  |  |  |  |  |
| Super Phantom S4-ROV | 0.09 | 1.5 | 0.75 | 0.6 |  | – |  | – |  | Thrusters 4 prop fwd/aft 2 prop vertical 2 prop transverse |  | 300 |  | Inspect cables and other underwater objects. Can also be used to inspect seabed conditions. |
|  |  |  |  | JAPAN 1) Submersibles belonging to KCS |  |  |  |  |  |  |  |  |  |  |
| MARCAS-II-ROV | Jet tool mode: 8.0 Track base mode: 7.5 | Jet tool mode: 2.9 Track base mode: 5.3 | Jet tool mode: 2.3 Track base mode: 4.0 | Jet tool mode: 3.2 Track base mode: 3.8 |  | Water jet tool |  | Up to 1.0 m Track base mode: 1.5 m |  | 4 horizontal, 2 vertical and 2 balance thrusters |  | Jet tool mode: 2500 Track base mode: 2000 |  | Post-lay burial, maintenance of cable. Can survey seabed. |
| MARCAS-III-ROV | Jet tool mode: 17.0 | 6.3 | 3.7 | 3.4 |  | Water jet tool |  | Up to 3.0 m |  | Thrusters(8) |  | 2500 |  | Post-lay burial, maintenance of cable. Can survey seabed. |
| PLOW-II | 18.5 Jet tool mode: 20.0 | 9.5 | 5.6 | 5.0 |  | Plough share Water jet tool |  | Up to 3.0 m |  | Towed by cable ship |  | 1500 Jet tool mode: 200 | 80 | Simultaneously lay and bury cables and umbilicals at varying depth. |
|  |  |  |  | 2) Submersibles belonging to NTT-WE Marine |  |  |  |  |  |  |  |  |  |  |
| Plough-type 6 Ssubmarine cable burying system | 18 | 9.3 | 5.1 | 4.4 |  | – |  | Up to 2.0 m depth immediate burial of cable on ploughing |  | Towed by support ship |  | 1500 |  | Simultaneous or post-lay burial of cable. |
| CARBIS-II ROV system | 8.0 | 3.2 | 2.1 | 2.8 |  | Water jetting |  | Trenching capability 1.5 m |  | Vertical and horizontal thrusters |  | 2500 |  | Cable detection & inspection visual survey. Cable manipulation & cutting. Cable & repeater burial. |
| ROV system (C/S VEGA) | 8 | 3.8 | 2.5 | 2.9 | Water jetting |  |  | Trenching capability 1.5 m |  | Vertical and horizontal thrusters |  | 2000 |  | Cable detection & inspection visual survey. Cable manipulation & cutting. Cable & repeater burial. |
|  |  |  |  | SPAIN 1) Submersible belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |  |  |  |  |  |
| ARADO I | 12 | 9 | 4.6 | 4 | Plow-share |  |  | 1100 mm |  | Towed |  | 1500 |  | Bury cable from 19 to 40 mm. Bury repeaters until 380 mm. Velocity 1 m/s. |
| NEREUS | 8.5 | 3.2 | 3.4 | 2.9 |  |  |  | 1 m |  | 150 kW |  | 2000 |  | Repair, inspect and bury all types of telephone cable 2  manipulating 7 functions. Velocity 3 knots. |
|  |  |  |  | UNITED STATES OF AMERICA Submersibles belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |  |  |  |  |  |
| PACIFIC SCARAB I | 5.48 | 4.27 | 1.83 | 3.05 |  |  | Jetting modules |  | 560 metres/hour. Soil hardness to 100 kPa. |  | 150 HP Electro-hydraulically powered using 8 thrusters | 2500 |  | PACIFIC SCARAB I Submersible Craft Assisting Repair and Burial is a tethered, swimming ROV capable of operating at depths of 2500 metres. It can locate, inspect, retrieve, and bury submarine cables. |
| SCARAB II | 3.45 | 3.7 | 2.1 | 2.3 |  |  | 35 HP cable jetter |  | 255 m/hr depending on soil conditions. Soil hardness to 60 kPa. |  | Horizontal: 4 × 5 HP electric thrusters Vertical: 2 × 5 HP electric thrusters Aft lateral: 1 × 10 HP hydraulic thruster Bow: 2 × 2.5 HP hydraulic thrusters | 1850 |  | SCARAB II Submersible Craft Assisting Repair and Burial is a tethered, swimming ROV capable of operating at depths of 1850 metres. It can locate, inspect, retrieve, and bury submarine cables. |
| SCARAB IV | 4.6 | 3.4 | 2.02 | 1.96 |  |  | Jetting modules |  | 530 metres/hour Soil hardness to 100 kPa |  | 150 HP electro-hydraulically powered using 8 thrusters | 1850 |  | SCARAB IV Submersible Craft Assisting Repair and Burial is a tethered, swimming ROV capable of operating at depths of 1850 metres. It can locate, inspect, retrieve, and bury submarine cables. SCARAB IV is part of the ACMA SCARAB Agreement. |
| Sea Plow VI | 25.5 | 10.5 | 6.0 | 4.3 |  |  | Towed plow system |  | 1.2 metre burial |  | Towed by ship | 1000 |  | Sea Plow VI is a towed burial tool employing state-of-the-art burial features. It can achieve 1.2 metre burial depth in up to 1000 metre water depth. |
| Sea Plow VII | 14.0 | 10.5 | 6.0 | 4.3 |  |  | Towed plow system |  | 1.0 metre burial |  | Towed by ship. 1 thruster for launches and recoveries | 1400 |  | Sea Plow VII is a towed burial tool employing state-of-the-art burial features. It can achieve 1.0 metre burial depth in up to 1400 metre water depth. |
| Sea Plow VIII | 19.3 | 9.2 | 5.5 | 3.6 |  |  | Towed plow system with water jet assist |  | 1.5 metre burial |  | Towed by ship | 1500 |  | Sea Plow VIII is a towed burial tool employing state of the art burial features. It can achieve 1.5 metre burial depth in up to 1500 metre water depth. |

ANNEX 2 
(to TSB Circular 84)

- Questionnaire on new cable ships and submersible equipments

<Cable ships>

| Country |  |  |  |  |
| --- | --- | --- | --- | --- |
| Organization |  |  |  |  |
| Name of ship |  |  |  |  |
| Year of construction |  |  |  |  |
| Displacement |  |  |  | (tons) |
| Overall length |  |  |  | (m) |
| Draft |  |  |  | (m) |
| Normal speed |  |  |  | (knots) |
| Range (autonomy) |  |  |  | (nautical miles) |
| Number of tanks |  |  |  |  |
| Cable capability | Cable | Cubic metres |  | (m3) |
|  |  | Weight |  | (tons) |
|  | Repeaters |  |  |  |
| Cable gear | Cable engine | (Drum) |  | (number) x (diameter) |
|  |  | (Linear) |  | (pairs of wheels) |
|  | Unwinding pulley | Bow sheave |  | (diameter, m) |
|  |  | Stern sheave |  | (diameter, m) |
| Maximum operating depth |  |  |  | (m) |
| Capability (general features and remarks) |  |  |  |  |

| Contact Affiliation Tel Fax E-mail |  |
| --- | --- |

<Submersible equipment for laying, burial, inspection and so on>

| Country |  |  |
| --- | --- | --- |
| Organization |  |  |
| Type of submersible |  |  |
| Weight |  | (tons) |
| Overall length |  | (m) |
| Width |  | (m) |
| Height |  | (m) |
| Trenching system |  |  |
| Trenching capability |  |  |
| Propulsion |  |  |
| Maximum operating depth |  | (m) |
| Capability (general features and remarks) |  |  |

| Contact Affiliation Tel Fax E-mail |  |
| --- | --- |

________________