| Telecommunication Standardization Bureau |  |
| --- | --- |

Geneva, 26 January 2010

| Ref: Tel: Fax: | TSB Circular 84 COM 15/GJ +41 22 730 6356 +41 22 730 5853 | - To Administrations of Member States of the Union - To ITU-T Sector Members; - To ITU-T Associates; |
| --- | --- | --- |
| E-mail: | tsbsg15@itu.int | Copy: - To the Chairman and Vice-Chairmen of Study Group 15; - To the Director of the Telecommunication Development Bureau; - To the Director of the Radiocommunication Bureau |

| Subject: | Questionnaire on cable ships and submersible equipments |
| --- | --- |

Dear Sir/Madam,

1	Study Group 15 at its last meeting (Geneva, 28 September – 9 October 2009) decided, in the framework of the studies conducted under Question 8/15 (Characteristics of optical fibre submarine cable systems), to revise the Recommendation ITU-T G.971 (General features of optical fibre submarine cable systems) and to update the existing Appendix I where data on cable ships and submersible equipment are reported.

2	The revised Recommendation G.971 is planned to be submitted for consent at the next SG 15 meeting (31 May – 6 June 2010).

3	To that end, your assistance is needed to review and update the information at present contained in the Appendix I to the Recommendation G.971. You are thus requested to modify, if necessary, the existing data on cable ships and submersible equipments shown in Annex 1 of this Circular letter, where the text of Appendix I of G.971 is reproduced.

If the equipment in the list has already been discarded, and/or if new cable ships and submersible equipments have been constructed since 2007, please describe them (in English) as shown in Annex 2.

4	Please return all this information to the Editor of Recommendation G.971, before 31 March 2010:

Mr. K. Nakajima
NTT Access Network Service Systems Laboratories
1-7-1, Hanabatake, Tsukuba, Ibaraki
305-0805, Japan

E-mail: nakaji@ansl.ntt.co.jp 
Facsimile: +81 29 868 6142

5	I rely on your cooperation in making sure that your replies are as accurate as possible and reach the above-mentioned Associate Rapporteur before the deadline.

Yours faithfully,

Malcolm Johnson
Director of the Telecommunication
Standardization Bureau

Annexes: 2

ANNEX 1 
(to TSB Circular 84)

Data on cable ships and submersible equipments of various countries

- 1.1	Cable ships

| Name of ship | Year of cons-truction | Dis-place-ment (tons) | Overall length (m) | Draft (m) | Normal speed (knots) | Range (auto-nomy) (nautical miles) | Number of tanks | Cable capacity |  |  | Cable gear |  |  |  | Max operating depth (m) | Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Cable |  | Re-peaters | Cable engine |  | Unwinding pulley |  |  |  |
|  |  |  |  |  |  |  |  | Cubic metres (m3) | Weight (tons) |  | Drum (diameter) (m) | Linear (pairs of wheels) | Bow sheave (diameter) (m) | Stern sheave (diameter) (m) |  |  |
|  |  |  |  |  |  |  | DENMARK Ships belonging to Tele Denmark |  |  |  |  |  |  |  |  |  |
| Peter Faber | 1982 | 3680 | 78.35 | Ice 3.8 Summer 5.0 | 13.0 | 7000 | 1 tank 1 hold | 310 230 | 600 400 | App. 10 | 3.0 |  | 2  3.0 | – | 4000 | Reinforced for operation in ice-filled waters. A-frame for ROV. Two hydraulic double-drum warping winches. |
| Lodbrog | 1985/ 2002 | 12'503 | 143.4 | 8.50 | 16.0 | 10'000 | 6 | 2940 | 5040 | 84 | 2  4.0 (25 t) | 2  6 (6 t) | – | 2  3.0 | All | Laying/burying and repair of all types of cables (coaxial, optical fibre and power cables). ROV capability, SWL 8 ton. |
|  |  |  |  |  |  |  | FINLAND 1) Ship belonging to Sonera Ltd |  |  |  |  |  |  |  |  |  |
| M/S Telepaatti | 1978 (modifi-cation) | 450 | 42.6 | 3.0 | 12 | – | 1 | – | 350 | – | 2 linear engines with 3 caterpillar tracks on each | 3.0 |  | 300 |  | Laying of all types of telecom cables. Specially equipped for cable route survey and cable repair. Fully automatic autopilot and DP-system. |
|  |  |  |  |  |  |  | 2) Ship belonging to YIT Primatel |  |  |  |  |  |  |  |  |  |
| c/s Telepaatti | 1978 Modifi-cation 1999 | 450 | 42.6 | 3.0 | 10.5 | – | 1 | 250 | 260 | – | – | 2 linear engines with 3 cater-pillar tracks on each | 3.0 | – | 300 | Laying of all types of telecom cables and <150 mm power cables. Specially equipped for cable route survey and cable repair. Fully automatic autopilot and DP-system. |
|  |  |  |  |  |  |  | FRANCE 1) Ships belonging to France Telecom Marine |  |  |  |  |  |  |  |  |  |
| Chamarel (formerly Vercors) | 1974 | 11'000 | 136 | 7.2 | 16.0 | 12'000 | 3 | 2425 | 4900 | 144 | 3.0 | 24 | 3.0 | Chute | All | Laying and repair of all types of telecom cables. Burying of cables with plough and 200 kW Hector 4. |
| Léon Thevenin | 1983 | 6800 | 107 | 6.24 | 15.0 | 10'000 | 2 + 1 | 1420 | 2000 | 11 | 3.4 | 12 | 3.0 | Chute | All | Laying and repair of all types of telecom cables. Burying of cables using 300 kW Hector 5. |
| Raymond Croze | 1983 | 6800 | 107 | 6.24 | 15.0 | 10'000 | 2 + 1 | 1420 | 2000 | 11 | 3.4 | 12 | 3.0 | Chute | All | Laying and repair of all types of telecom cables. Burying of cables using 250 kW Hector 3. |
| René Descartes | 2002 | 15'450 | 114.50 | 7.42 | 16.0 | 12'000 | 4 | 3250 | 5500 | 210 | 4.0 | 20 | Aft sheave 3.0 m | Sheave | All | Stem concept cable ship. Laying and repair of all types of telecom cables. Burying of cables with plough and 250 kW ROV Hector 6. |
|  |  |  |  |  |  |  | 2) Ships belonging to Alda Marine |  |  |  |  |  |  |  |  |  |
| Ile de Sein Ile de Batz Ile de Brehat | 2002 | 18'006 | 140.4 | 8.016 | 15.0 | 15'000 | 2 + 2 | 3000 | 5500 | 202 | 4.0 | 21 | NA | 3.0 | All | Laying and repair of all types of telecom cables. Burying of cables with. 2/3m Rock plough. Sea state 7 A-frame |
| Ile de Ré | 1983 rebuilt 2002 | 12'687 | 143.4 | 7.23 | 16.0 | 11'000 | 3 + 3 | 2900 | 4500 | 84 | 2 × 4.0 | NA | NA | 3.0 | All | Laying and repair of types of cable. ROV to 2500m. A plough is available. |
|  |  |  |  |  |  |  | ITALY 1) Ship belonging to Elettra TLC S.p.A |  |  |  |  |  |  |  |  |  |
| Teliri | 1996 | 6500 | 111.5 | 6.5 | 14.01 | 10'000 | 3 | 2000 | 2600 | 70 | 2  3.5 | 18 | 3 | 4 | All | Laying and repair optical fibre systems |
| Certamen (ex John Cabot) | 1966 rebuilt 1998 | 5000 | 96.6 | 7.3 | 12.0 | 8000 | 3 | 600 | 1900 | 24 | 1  3.0 | 18 (on the stern) + 6 (on the bow) | 3 | 3 | All | Laying, survey and repair optical fibre systems |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No more a Cable ship. Converted as pipelay ship in 2007 |
|  |  |  |  |  |  |  | ITALY 2) Ships belonging to Prysmian Cavi e Sistemi Energia S.r.l. |  |  |  |  |  |  |  |  |  |
| Giulio Verne | 1984 | 16'900 | 133.18 | 8.5 | 10 | 7000 | 2 | 2600 | 7000 | 10 | 6.0 (55 t) | 1 (Pads type 10 t) | – | 6.0 | All | Lay and repair from the stern |
|  |  |  |  |  |  |  | SPAIN Ships belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |  |  |  |  |
| Teneo | 1992 | 4000 | 81 | 5.7 | 14.5 | 4200 | 2 | 500 | 1000 | 20 | 2  3.5 | 1  9 | 2  3 | 1  3 | All | Lays and repairs of all types of telephone cables. |
|  |  |  |  |  |  |  | JAPAN 1) Ships belonging to Kokusai Cable Ship (KCS) |  |  |  |  |  |  |  |  |  |
| KDD Ocean Link | 1992 | 11'700 | 133.2 | 7.0 | 15 | 10'000 | Main 3 Spare 4 | 2600 | 4500 | 57 | 3.6 | 21 | 3.2 | 4.0 | All | Laying by linear engine. Lays and repairs all types of submarine cables. |
| KDD Pacific Link | 1997 | 11'207 | 109.0 | 7.5 | 11 | 10'000 | Main 2 Spare 2 | 2720 | 4500 | 50 | 3.6 | 20 | – | 3.0 | All | Laying by linear engine. Lays and repairs all types of submarine cables. |
|  |  |  |  |  |  |  | 2) Ships belonging to NTT World Engineering Marine Corporation (NTT-WE Marine) |  |  |  |  |  |  |  |  |  |
| Subaru | 1999 | 9557 | 123.3 | 7.0 | 13.2 | 8800 | Main 2 Spare 2 | 2770 | 4000 | 50 | 4.0 | 21 | – | 3.2 | All | Lays and repairs all types of telephone cables. |
| C/S VEGA | 1984 | 1336 | 74.3 | 4.6 | 13.5 | 4500 | 2 | 169 | 250 | – | 3.0 | 6 | 2.5 | 2.0 | All | Lays and repairs for non-powered telephone cable system. DP, ROV system |
|  |  |  |  |  |  |  | UNITED KINGDOM 1) Ship belonging to British Telecommunications plc |  |  |  |  |  |  |  |  |  |
| Sovereign | 1991 | 13'018 | 131 | 7.0 | 13.5 | 14'000 | 4 | 2800 | 6200 | 90 | 3.50 |  | 3.00 | 3.50 | All | Lays, repairs all types of coaxial and optical fibre cable. (Operated by C&W marine.) |
|  |  |  |  |  |  |  | UNITED KINGDOM 2) Ships belonging to Global Marine Systems Ltd |  |  |  |  |  |  |  |  |  |
| Alert | 1961 | 9477 | 130 | 7.1 | 14 | 10'000 | 3 | 1509 | 3100 | 48 | 2.98 |  | 2.98 | 2.98 | All | Laying by linear engine and seabed burial by plow. Lays/repairs all types of coaxial and optical fibre cables. |
| Cable Venture | 1962 | 16'983 | 153 | 8.97 | 12.5 | 10'000 | 4 + 1 (spare) | 5086 | 9000 | 400 | 2.80 |  | 3.00 | 3.39 | All | Laying by linear cable engine. Ploughs, lays and repairs armoured and lightweight cables. |
| Mercury | 1962 | 11'683 | 144 | 7.5 | 14.5 | 8000 | 3 | 2970 | 3500 | 144 | 3.05 |  | 3.50 | Chute 3.05 | All | Ditto (no plough). |
| Cable Enterprise | 1964 | 5759 | 113 | 5.84 | 13 | 8000 | 3 | 887 | 2150 | 30 | 2.8 |  | 3.00 | Chute 3.05 | All | Lays/repairs armoured cables. Repairs lightweight cables. (Note) |
| Monarch | 1975 | 4639 | 97 | 5.5 | 14 | 7000 | 4 | 417 | 850 | 12 | 3.00 |  | 3.00 | None | All | Lays/repairs armoured coaxial and optical fibre cables. Repairs lightweight coaxial and optical fibre cables. Detrenching/reburial by submersible jetting. |
| Iris | 1976 | 4639 | 97 | 5.5 | 14 | 7000 | 4 | 417 | 850 | 12 | 3.00 |  | 3.00 | None | All | Lays/repairs armoured coaxial and optical fibre cables. Repairs lightweight coaxial and optical fibre cables. |
| MV Cable Installer | 1980 | 6065 | 89.42 | 5 | 12 | 42 days | 4 | 840 | 1600 | None | 3.0 | 4-track pair | – | 3.0 | – | Repeaterless installation vessel fully DP Cegelec 901 system. |
| Seaspread | 1980 | 10'887 | 116 | 6.8 | 13 | 65 days | 2 | 1010 | 1701 | – | 2  3 | – | – | 3 | All | Lays/repairs by aft drums. Burial by plough. Lays/repairs armoured and lightweight cables. |
| Pacific Guardian | 1984 | 7526 | 116 | 6.32 | 14.0 | 8000 | 3 | 1416 | 3470 | 96 | 3.5 |  | 3.00 | 3.00 | All | Laying by linear cable engine. Lays and repairs armoured and lightweight cables. |
| Sir Elic Sharp | 1988 | 7526 | 115 | 6.3 | 13.5 | 9600 | 3 | 1416 | 1700 | 96 | 2  3.5 | – | 3 | 3 | All | Laying by linear cable engine. Repairs and lays armoured and lightweight cables. Post lay/repair burial by integral ROV. |
|  |  |  |  |  |  |  | UNITED KINGDOM 2) Ships belonging to Global Marine Systems Ltd |  |  |  |  |  |  |  |  |  |
| MV Cable Innovator | 1995 | – | 142 | 8.3 | 14.5 | 42 days | 4 | 4900 | 7500 | 180 | 4.0 | 21 pairs (min) | – | 4.0 | – | Simplex D/P system. Lays/repairs cables. |
|  |  |  |  |  |  |  | MARSHALL ISLANDS Ship belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |  |  |  |  |
| CS Coastal Connector | 1997 Conver-ted in 1996 | 6761 | 92.47 | 7.1 | 12.5 | 25'000 | 3 main 1 spare | 675 (main, total) 70 (spare) | 1600 | 30 | 2  3 | N/A | N/A | 2  3 | – | The CS Coastal Connector is a stern-laying design. She is capable of deploying the SCARAB II, SCARAB IV, and Pacific SCARAB I ROVs, as well as the Seabed Tractor. |
| CS Tyco Provider | 1978, Converted in 1999 | 14'500 | 139.4 | 7.6 | 14.5 | 20'000 | 5 | 3349 | 6000 | 100+ | 2  4 | – | – | 2  3 | – | The CS Tyco Provider is a stern-laying design. She is capable of deploying Sea Plow VIII. |
|  |  |  |  |  |  |  | NETHERLANDS ANTILLES Ship belonging to Tyco Submarine Systems Ltd. (charterer) |  |  |  |  |  |  |  |  |  |
| Dock Express 20 | 1983 | 21'731 | 169.52 | 8.79 | 12.5 | 20'500 | 3 main 2 spare | 4050 (main, total) 640 (spare, total) | 10 000 | 100+ | 1  3.0 | 1  3 module belt type | N/A | 2  3 |  | The Dock Express 20 is a stern-laying design. She is capable of deploying the SCARAB II ROV, as well as the Seabed Tractor and Sea Plow VI. |
|  |  |  |  |  |  |  | UNITED STATES OF AMERICA Ships belonging to AT&T |  |  |  |  |  |  |  |  |  |
| CS Charles L. Brown | 1954, Reflag-ged in 1985 | 4298 | 99.94 | 5.6 | 13 | 7550 | 3 | 660 | 1186 | 30+ | 2  3 | N/A | 2  3 | N/A | – | The Charles L. Brown is primarily a repair ship. She is not fitted with any stern-laying equipment. She is capable of deploying the SCARAB II ROV. |
| CS Global Link | 1990 | 16'375 | 145.7 | 8.08 | 15 | 10'000 | 3 main, 4 spare | 3258 (main, total) 164 (spare, total) | 6098 | 100+ | 2  3.7 | 1 Western Gear Tractor Type | 2  3 | 1 trough/ Chute type | – | The Global Link is capable of deploying the SCARAB II ROVs. |
|  |  |  |  |  |  |  | UNITED STATES OF AMERICA Ships belonging to AT&T |  |  |  |  |  |  |  |  |  |
| CS Global Mariner | 1993 | 15' 638 | 151.5 | 7.8 | 13.8 | 10'000 | 2 main, 3 spare | 2172 (main, total) 447 (spare, total) | 4999 | 80+ | 2  3.7 | 1 Dowty 21 pairs | 2  3 | 1 trough/ Chute type | – | The Global Mariner is capable of deploying the SCARAB II and SCARAB IV ROVs, as well as Sea Plow VII, Sea Plow VIII, and the Seabed Tractor. |
| CS Global Sentinel | 1991 | 16' 375 | 145.7 | 8.08 | 15 | 10' 000 | 3 main, 4 spare | 3258 (main, total) 164 (spare, total) | 6098 | 100+ | 2  3.7 | 1 Dowty 21 pairs | 2  3 | 1 trough/ Chute type | – | The Global Sentinel is capable of deploying the SCARAB II and SCARAB IV, and Pacific SCARAB I ROVs, as well as Sea Plow VII and Sea Plow VIII. |
| NOTE – Only relatively short cables are laid and only shore-end. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- 1.2	Submersible equipments

| Type of submersible |  | Weight (tons) |  | Overall length (m) | Width (m) |  | Height (m) |  |  | Trenching system |  | Trenching | Propulsion | Max operating depth (m) |  | Max pulling tension (tons) |  |  | Capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | FRANCE Submersibles belonging to France Telecom Marine |  |  |  |  |  |  |  |  |  |  |  |  |
| ELISE2 Submersible Plough system |  | 17 |  | 7.60 | 2.90 |  | 2.95 |  |  | Ploughshare |  | Immediate burial up to 1.1 m | Towed by support ship | 1500 |  |  |  |  | Lay and bury all types of cables. |
| ELISE3 Submersible Plough system |  | 17 |  | 7.60 | 2.90 |  | 2.95 |  |  | Ploughshare |  | Immediate burial up to 1.1 m | Towed by support ship | 1500 |  |  |  |  | Lay and bury all types of cables. |
| Self-advancing buried system CASTOR2 |  | 12 |  | 7.0 | 2.40 |  | 3.00 |  |  | Trenching wheel or chain |  | Burial of existing cables down to 2 m | Tracked vehicle | 1000 |  |  |  |  | Burial of cables and pipes. Visual inspection. |
| ROVs HECTOR 3, 4, 5 & 6 |  | 9 |  | 4.0 | 3.50 |  | 2.10 |  |  | High-pressure water jets |  | Up to 1.5 m depth | Thrusters (inspection) Back drive (burial) | 2000 |  |  |  |  | Visual inspection, post-lay burial, cable location, cable manipulation, cable cutting. |
|  |  |  |  |  |  |  | FRANCE Submersibles belonging to France Telecom Marine |  |  |  |  |  |  |  |  |  |  |  |  |
| Remote control submersible Scorpio 2000 |  | 3.4 |  | 2.9 | 1.5 |  | 2.11 |  |  | High-pressure water jets |  | Up to 60 cm depth | Thrusters | 1000 |  |  |  |  | Visual inspection, post-lay burial, cable location/ manipulation/cutting. |
|  |  |  |  |  |  |  | ITALY 1) Submersibles belonging to Elettra TLC SpA |  |  |  |  |  |  |  |  |  |  |  |  |
| Plough Taurus 1 |  | 14 |  | 9 | 4.6 |  | 4.5 |  |  | Plough share |  | Up to 1 m | Towed by cable ship | 1500 |  | 50 |  |  | Lay and bury all types of cables. |
| Plough Taurus 2 |  | 16 |  | 9.5 | 4.5 |  | 5.1 |  |  | Plough share |  | Up to 1.5 m | Towed by cable ship | 1500 |  | 50 |  |  | Lay and bury all types of cables. |
| ROV – Phoenix 2 |  | 6.8 |  | 4.8 | 2 |  | 2.6 |  |  | High/low-pressure jetting |  | Up to 1.2 m | 8 Hydraulic thrusters | 1000 |  |  |  |  | Visual inspection, post-lay burial, cable location/manipulation/cutting. |
| ROV-T200 |  | Free-fly mode 6, Track mode 7 |  | 3.1 | 2 |  | 2.2 |  |  | High/low-pressure jetting |  | Up to 1.2 m | 4 vertical and 4 horizontal thrusters | 2500 |  |  |  |  | Visual inspection, post-lay burial, cable location/manipulation/cutting. |
|  |  |  |  |  |  |  | UNITED KINGDOM Submersibles belonging to Global Marine Systems Ltd |  |  |  |  |  |  |  |  |  |  |  |  |
| Submersible trencher |  | 17.0 |  | 6.6 | 4 |  | 3.4 |  |  | Fluidization and cutting jets and dredge pump |  | Up to 1 m depth with cutting and fluidization jets | Three vertical and four horizontal thrusters, track drive differential steering | 274 |  |  |  |  | Trench in existing cable and pipe. |
| Submersible Plough system |  | 9.75 |  | 6.1 | 2.6 |  | 2.6 |  |  | Ploughshare proceeded by disc |  | Immediate burial of cable on ploughing | Towed by support ship | 900 |  |  |  |  | Lay and bury cable, umbilical and pipe in one action giving full cable protection. |
| Remote control submersible 2 off Cirus A&B |  | 3.2 |  | 3.5 | 2.1 |  | 2.3 |  |  | Water jets |  | Trenching capability 0.3 m | Thrusters (7) | 1000 |  |  |  |  | Visual inspection, cable location/inspection/deburial, manipulation. Tools include cable cutter, cable gripper and 2 manipulators with line cutters. |
| Plough 2 off A&B |  | 14.5 |  | 9 | 4.1 |  | 4 |  |  | Passive blade |  | Trenching capability 1.0 m | Towed | 1000 |  |  |  |  | Steerable, repeater burial. |
| Remote control submersible ROV 128 |  | 7.5 |  | 2.9 | 1.8 |  | 2.0 |  |  | Jetting tool |  | Trenching capability 0.6 m | Tracked burial Thrusters survey | 1000 (burial) 2000 (survey) |  |  |  |  | Tools include cable cutter, cable gripper and 2 manipulators with line cutters. |
| Underwater vehicle- MARLIN |  | 7.8 |  | 4.191 | 2.438 |  | 3.175 |  |  | Burial skid |  | To 1.0 m (Optimized for 0-30 kPa soil) | Hydraulic driven thrusters | 2500 |  |  |  |  | Burial, deburial, inspection. Maintenance and repair. Tools include cable cutter, cable gripper. |
|  |  |  |  |  |  |  | UNITED KINGDOM Submersibles belonging to Global Marine Systems Ltd |  |  |  |  |  |  |  |  |  |  |  |  |
| Scarab I – Umbilically tethered ROV |  | 3.2 |  | 2.74 | 1.82 |  | 1.52 |  |  | Jetting tool |  | Up to 0.6 m | Thrusters: 2 vertical 4 vectored | 2000 |  |  |  |  | Cable detection and inspection. Visual survey. Cable manipulation and cutting Debris elimination. Cable and repeater burial/deburial. |
| Subtrack - ROV |  | 10.0 |  | 8.0 (Max) | 3.7 |  | 3.8 |  |  | Jetting tool |  | Burial to 1.0 m | Electro-hydraulic track drives | 1000 |  |  |  |  | Cable burial and deburial. Inspection. Maintenance and repair. |
| EUREKA: Deepwater burial + trenching system |  | 17 (Max) |  | 5.5 | 4.2 |  | 3.85 |  |  | Jetting tool Rock wheel cutter Mechanical chain excavator |  | 1 m 1.2 m 2.2 m | Electro-hydraulic track drives | 1500 |  |  |  |  | Capable of burying cable, small flexible flowlines and also rigid pipes. Can also debury cable and restore. Visual and electronic inspections. |
| Plough 5 |  | 14.0 |  | 9.0 | 4.6 |  | 3.7 |  |  | Passive blade |  | Variable from 0-1100 mm (600-900 mm in all conditions) | Towed | 1000 |  |  |  |  | Simultaneously lay and bury cables and umbilicals at varying depths. |
| Plough 6 and 7 |  | 14.0 |  | 9.0 | 4.6 |  | 3.7 |  |  | Passive blade |  | Max burial depth: 1100 mm | Towed | 1000 |  |  |  |  | Simultaneously lay and bury cables and umbilicals at varying depths. |
| Cable Plough 1000 mm |  | 14.4 |  | 9.75 | 4.1 |  | 3.9 |  |  | Passive blade |  | 1000 mm (Good conditions: 1100 mm; Repeaters/Joints: 500 mm) | Towed | 1000 |  |  |  |  | Simultaneously lay and bury cables and umbilicals at varying depths. |
|  |  |  |  |  |  |  | DENMARK Submersibles belonging to Telecom Denmark |  |  |  |  |  |  |  |  |  |  |  |  |
| Plough D |  | 13.5 |  | 9.0 | 4.6 |  | 3.7 |  |  | Plough share |  | Variable from 0-1100 mm (600-900 mm in all conditions) | Towed by host vessel | 1500 |  |  |  |  | Lay and bury telecom cables, power cables and umbilicals. Cables: Up to 120 mm (bury). Joints and repeaters: Up to 400 mm (pass). |
| Plough 7 |  | 13.5 |  | 9.0 | 4.6 |  | 3.7 |  |  | Plough share |  | Variable from 0-1100 mm (600-900 mm in all conditions) | Towed by surface vessel | 1000 |  |  |  |  | Lay and bury fibre optic cables, power cables and umbilicals. |
| Subtrack- Subsea tractor |  | 10.0 |  | 8.0 (Max) | 3.7 |  | 3.8 |  |  | Jetting tool |  | Burial to 1.0 m | Electro-hydraulic track drives | 1000 |  |  |  |  | Cable burial and deburial. Inspection. Maintenance and repair. |
|  |  |  |  |  |  |  | DENMARK Submersibles belonging to Telecom Denmark |  |  |  |  |  |  |  |  |  |  |  |  |
| Super Phantom S4-ROV |  | 0.09 |  | 1.5 | 0.75 |  | 0.6 |  |  | – |  | – | Thrusters 4 prop fwd/aft 2 prop vertical 2 prop transverse | 300 |  |  |  |  | Inspect cables and other underwater objects. Can also be used to inspect seabed conditions. |
|  |  |  |  |  |  |  | JAPAN 1) Submersibles belonging to KCS |  |  |  |  |  |  |  |  |  |  |  |  |
| MARCAS-II-ROV |  | Jet tool mode: 8.0 Track base mode: 7.5 |  | Jet tool mode: 2.9 Track base mode: 5.3 | Jet tool mode: 2.3 Track base mode: 4.0 |  | Jet tool mode: 3.2 Track base mode: 3.8 |  |  | Water jet tool |  | Up to 1.0 m Track base mode: 1.5 m | 4 horizontal, 2 vertical and 2 balance thrusters | Jet tool mode: 2500 Track base mode: 2000 |  |  |  |  | Post-lay burial, maintenance of cable. Can survey seabed. |
| MARCAS-III-ROV |  | Jet tool mode: 17.0 |  | 6.3 | 3.7 |  | 3.4 |  |  | Water jet tool |  | Up to 3.0 m | Thrusters(8) | 2500 |  |  |  |  | Post-lay burial, maintenance of cable. Can survey seabed. |
| PLOW-II |  | 18.5 Jet tool mode: 20.0 |  | 9.5 | 5.6 |  | 5.0 |  |  | Plough share Water jet tool |  | Up to 3.0 m | Towed by cable ship | 1500 Jet tool mode: 200 |  | 80 |  |  | Simultaneously lay and bury cables and umbilicals at varying depth. |
|  |  |  |  |  |  |  | 2) Submersibles belonging to NTT-WE Marine |  |  |  |  |  |  |  |  |  |  |  |  |
| Plough-type 6 Ssubmarine cable burying system |  | 18 |  | 9.3 | 5.1 |  | 4.4 |  |  | – |  | Up to 2.0 m depth immediate burial of cable on ploughing | Towed by support ship | 1500 |  |  |  |  | Simultaneous or post-lay burial of cable. |
| CARBIS-II ROV system |  | 8.0 |  | 3.2 | 2.1 |  | 2.8 |  |  | Water jetting |  | Trenching capability 1.5 m | Vertical and horizontal thrusters | 2500 |  |  |  |  | Cable detection & inspection visual survey. Cable manipulation & cutting. Cable & repeater burial. |
| ROV system (C/S VEGA) |  | 8 |  | 3.8 | 2.5 |  | 2.9 |  |  | Water jetting |  | Trenching capability 1.5 m | Vertical and horizontal thrusters | 2000 |  |  |  |  | Cable detection & inspection visual survey. Cable manipulation & cutting. Cable & repeater burial. |
|  |  |  |  |  |  |  | SPAIN 1) Submersible belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |  |  |  |  |  |  |  |
| ARADO I |  | 12 |  | 9 | 4.6 |  | 4 |  |  | Plow-share |  | 1100 mm | Towed | 1500 |  |  |  |  | Bury cable from 19 to 40 mm. Bury repeaters until 380 mm. Velocity 1 m/s. |
| NEREUS |  | 8.5 |  | 3.2 | 3.4 |  | 2.9 |  |  |  |  | 1 m | 150 kW | 2000 |  |  |  |  | Repair, inspect and bury all types of telephone cable 2  manipulating 7 functions. Velocity 3 knots. |
|  |  |  |  |  |  |  |  | UNITED STATES OF AMERICA Submersibles belonging to Tyco Submarine Systems Ltd. |  |  |  |  |  |  |  |  |  |  |  |
| PACIFIC SCARAB I | 5.48 |  | 4.27 |  |  | 1.83 |  | 3.05 | Jetting modules |  | 560 metres/hour. Soil hardness to 100 kPa. |  | 150 HP Electro-hydraulically powered using 8 thrusters |  | 2500 |  |  | PACIFIC SCARAB I Submersible Craft Assisting Repair and Burial is a tethered, swimming ROV capable of operating at depths of 2500 metres. It can locate, inspect, retrieve, and bury submarine cables. |  |
| SCARAB II | 3.45 |  | 3.7 |  |  | 2.1 |  | 2.3 | 35 HP cable jetter |  | 255 m/hr depending on soil conditions. Soil hardness to 60 kPa. |  | Horizontal: 4 × 5 HP electric thrusters Vertical: 2 × 5 HP electric thrusters Aft lateral: 1 × 10 HP hydraulic thruster Bow: 2 × 2.5 HP hydraulic thrusters |  | 1850 |  |  | SCARAB II Submersible Craft Assisting Repair and Burial is a tethered, swimming ROV capable of operating at depths of 1850 metres. It can locate, inspect, retrieve, and bury submarine cables. |  |
| SCARAB IV | 4.6 |  | 3.4 |  |  | 2.02 |  | 1.96 | Jetting modules |  | 530 metres/hour Soil hardness to 100 kPa |  | 150 HP electro-hydraulically powered using 8 thrusters |  | 1850 |  |  | SCARAB IV Submersible Craft Assisting Repair and Burial is a tethered, swimming ROV capable of operating at depths of 1850 metres. It can locate, inspect, retrieve, and bury submarine cables. SCARAB IV is part of the ACMA SCARAB Agreement. |  |
| Sea Plow VI | 25.5 |  | 10.5 |  |  | 6.0 |  | 4.3 | Towed plow system |  | 1.2 metre burial |  | Towed by ship |  | 1000 |  |  | Sea Plow VI is a towed burial tool employing state-of-the-art burial features. It can achieve 1.2 metre burial depth in up to 1000 metre water depth. |  |
| Sea Plow VII | 14.0 |  | 10.5 |  |  | 6.0 |  | 4.3 | Towed plow system |  | 1.0 metre burial |  | Towed by ship. 1 thruster for launches and recoveries |  | 1400 |  |  | Sea Plow VII is a towed burial tool employing state-of-the-art burial features. It can achieve 1.0 metre burial depth in up to 1400 metre water depth. |  |
| Sea Plow VIII | 19.3 |  | 9.2 |  |  | 5.5 |  | 3.6 | Towed plow system with water jet assist |  | 1.5 metre burial |  | Towed by ship |  | 1500 |  |  | Sea Plow VIII is a towed burial tool employing state of the art burial features. It can achieve 1.5 metre burial depth in up to 1500 metre water depth. |  |

ANNEX 2 
(to TSB Circular 84)

- Questionnaire on new cable ships and submersible equipments

<Cable ships>

| Country |  |  |  |  |
| --- | --- | --- | --- | --- |
| Organization |  |  |  |  |
| Name of ship |  |  |  |  |
| Year of construction |  |  |  |  |
| Displacement |  |  |  | (tons) |
| Overall length |  |  |  | (m) |
| Draft |  |  |  | (m) |
| Normal speed |  |  |  | (knots) |
| Range (autonomy) |  |  |  | (nautical miles) |
| Number of tanks |  |  |  |  |
| Cable capability | Cable | Cubic metres |  | (m3) |
|  |  | Weight |  | (tons) |
|  | Repeaters |  |  |  |
| Cable gear | Cable engine | (Drum) |  | (number) x (diameter) |
|  |  | (Linear) |  | (pairs of wheels) |
|  | Unwinding pulley | Bow sheave |  | (diameter, m) |
|  |  | Stern sheave |  | (diameter, m) |
| Maximum operating depth |  |  |  | (m) |
| Capability (general features and remarks) |  |  |  |  |

| Contact Affiliation Tel Fax E-mail |  |
| --- | --- |

<Submersible equipment for laying, burial, inspection and so on>

| Country |  |  |
| --- | --- | --- |
| Organization |  |  |
| Type of submersible |  |  |
| Weight |  | (tons) |
| Overall length |  | (m) |
| Width |  | (m) |
| Height |  | (m) |
| Trenching system |  |  |
| Trenching capability |  |  |
| Propulsion |  |  |
| Maximum operating depth |  | (m) |
| Capability (general features and remarks) |  |  |

| Contact Affiliation Tel Fax E-mail |  |
| --- | --- |

________________