|  |  | International Telecommunication Union |  |  |
| --- | --- | --- | --- | --- |
| ITU-T | G.711.1 |  |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 3 (10/2010) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Coding of voice and audio signals |  |  |  |
|  | Wideband embedded extension for G.711 pulse code modulation Amendment 3: Lossless compression of G.711 bitstreams in G.711.1 |  |  |  |
| – | Recommendation ITU-T G.711.1 (2008) – Amendment 3 |  |  |  |

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TRANSMISSION MEDIA AND OPTICAL SYSTEMS CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of voice and audio signals | G.710–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| MULTIMEDIA QUALITY OF SERVICE AND PERFORMANCE – GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DATA OVER TRANSPORT – GENERIC ASPECTS | G.7000–G.7999 |
| PACKET OVER TRANSPORT ASPECTS | G.8000–G.8999 |
| ACCESS NETWORKS | G.9000–G.9999 |

For further details, please refer to the list of ITU-T Recommendations.

Recommendation ITU-T G.711.1 Wideband embedded extension for G.711 pulse code modulation Amendment 3 Lossless compression of G.711 bitstreams in G.711.1

Summary Amendment 3 to Recommendation ITU-T G.711.1 introduces new Annex C and two new Appendices II and III. Recommendation ITU-T G.711.1 describes an ITU-T G.711 embedded wideband speech and audio coding algorithm operating at 64, 80 and 96 kbit/s. The encoder input and decoder outputs are sampled at 16 kHz by default, but 8-kHz sampling is also supported. When sampled at 16 kHz, the output of the ITU-T G.711.1 coder can encode signals with a bandwidth of 50-7000 Hz at 80 and 96 kbit/s, and for 8-kHz sampling the output may produce signals with a bandwidth ranging from 50 up to 4000 Hz, operating at 64 and 80 kbit/s (the bandwidth of the narrow-band signal output from the decoder is characterized by the built-in split-band filterbank which has a frequency cut-offs at 4000 Hz). At 64 kbit/s, Recommendation ITU-T G.711.1 is compatible with Recommendation ITU-T G.711, hence an efficient deployment in existing ITU-T G.711-based voice over IP (VoIP) infrastructures is foreseen. The coder operates on 5 ms frames, has a maximum algorithmic delay of 11.875 ms, and has a worst-case computational complexity of 8.70 weighted million operations per second (WMOPS). The encoder produces an embedded bitstream structured in three layers corresponding to three available bit rates: 64, 80 and 96 kbit/s. The bitstream can be truncated at the decoder side or by any component of the communication system to adjust the bit rate to the desired value, but since it does not contain any information on which layers are contained, an implementation would require outband signalling on which layers are available. The underlying algorithm has a three-layer coding structure: log companded pulse code modulation (PCM) of the lower band including noise feedback, embedded PCM extension with adaptive bit allocation for enhancing the quality of the base layer in the lower band, and weighted vector quantization coding of the higher band based on modified discrete cosine transformation (MDCT). Annex A defines an alternative implementation of the ITU-T G.711.1 algorithm using floating-point arithmetic to facilitate its use on hardware optimized for floating-point operations. The accompanying floating-point C-code is fully interoperable with the fixed-point C-code and provides equivalent quality. Annex B contains the RTP payload format, capability identifiers and parameters for signalling of ITU-T G.711.1 capabilities using Recommendation ITU-T H.245. The packet format is fully compatible with the corresponding ITU-T G.711.1 RTP definitions to allow seamless interoperability. Annex C describes an algorithm applying ITU-T G.711.0 lossless compression algorithm to ITU-T G.711.1. As Recommendation ITU-T G.711.0 is more efficient when applied to large frame sizes, to achieve efficient compression rate as many ITU-T G.711.1 frames as possibly supported by ITU-T G.711.0 are encoded together. The use of this extension introduces no quality degradation when compared to Recommendation ITU-T G.711.1, as it is a lossless encoding of the ITU-T G.711 portion of the ITU-T G.711.1 bitstream. Furthermore, there is no additional algorithmic delay, the delay will be the same as the one of ITU-T G.711.1, plus the selected size of the packet minus five milliseconds. It keeps the same robustness against packet losses as Recommendation ITU-T G.711.1 and no error propagation in case of frame errors. The proposed scheme can easily be transcoded to ITU-T G.711.1 or ITU-T G.711.0 at minimum complexity. Appendix I describes a supplementary postfilter for use in the decoder. This postfilter enhances the quality of the decoded signal when a legacy G.711 or only the basic log companded PCM part of the G.711.1 bitstream are available. It is intended for end-user terminals and usage in tandem scenarios should be avoided (such as in a signal mixer or bitstream translators). Appendices II and III provide information on frame size selection and on decoding of the ITU-T G.711.0 bitstream part of G.711.1 LLC bitstreams, respectively. This Recommendation includes an electronic attachment containing a non-exhaustive set of test signals for use with the ANSI C code. ANSI C source code is provided for both the main body's fixed-point arithmetic implementation as well as for Annex A's floating-point alternative specification. This edition incorporates changes needed to correct defects in the pre-published text of this Recommendation. All updates are related to typos and editorial corrections of the text, i.e., the algorithm description, and do not modify the other parts of the Recommendation including Annex A, such as bit-exactness of the supplied C-source code.

History Edition; Recommendation; Approval; Study Group. 1.0; ITU-T G.711.1; 2008-03-15; 16. 1.1; ITU-T G.711.1 (2008) Amend. 1; 2008-11-13; 16. 1.2; ITU-T G.711.1 (2008) Amend. 2; 2009-03-16; 16. 1.3; ITU-T G.711.1 (2008) Amend. 3; 2010-10-14; 16.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications, information and communication technologies (ICTs). The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure, e.g., interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met. The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementers are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database at http://www.itu.int/ITU-T/ipr/.

ITU  2011

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

Table of Contents

Page

Annex C – Lossless compression of ITU-T G.711 PCM compatible bitstream in ITU-T G.711.1		1

C.1	Scope		1

C.2	References		1

C.3	Abbreviations and acronyms		1

C.4	Conventions		2

C.5	General description of the ITU-T G.711.1 LLC coder		2

C.6	ITU-T G.711.1 LLC encoder		3

C.7	ITU-T G.711.1 LLC decoder		4

C.8	Description of the transmitted parameters		5

C.9	Bit-exact description of the ITU-T G.711.1 LLC codec		5

Appendix II – Frame size selection		7

Appendix III – Decoding of the ITU-T G.711.0 bitstream part of ITU-T G.711.1 LLC bitstreams		8

III.1	Case 1: When the first boundary octet of the ITU-T G.711.0 bitstream and the number of octet Ni for the ITU-T G.711.0 bitstream are given		8

III.2	Case 2: When the first boundary octet of the ITU-T G.711.0 bitstream and the number of ITU-T G.711.1 frames Fn are given		8

Bibliography		9

Electronic attachment 	– ANSI-C source code reference implementation

Associated test vectors

Recommendation ITU-T G.711.1

Wideband embedded extension for G.711 pulse code modulation

Amendment 3

Lossless compression of G.711 bitstreams in G.711.1

Modifications introduced by this amendment are shown in revision marks. Unchanged text is replaced by ellipsis (…). Some parts of unchanged text (clause numbers, etc.) may be kept to indicate the correct insertion points.

…

Annex C

Lossless compression of ITU-T G.711 PCM compatible bitstream in ITU-T G.711.1

(This annex forms an integral part of this Recommendation.)

## C.1	Scope

This annex describes an algorithm extending ITU-T G.711.1 for the lossless compression of ITU-T G.711 bitstream in an efficient way.

Annex C is organized as follows. The abbreviations, acronyms and conventions used throughout this annex are defined in clauses C.3 and C.4, respectively. The general description of the ITU-T G.711.1 LLC coder is discussed in clause C.5. Clauses C.6 and C.7 describe the ITU-T G.711.1 LLC encoder and decoder, respectively. Clause C.8 describes the transmitted parameter indices, and clause C.9 describes the reference software.

## C.2	References

In addition to the normative references in clause 2, the following reference applies to this annex:

[ITU-T G.711.0]	Recommendation ITU-T G.711.0 (2009), Lossless compression of G.711 pulse code modulation.

## C.3	Abbreviations and acronyms

In addition to the acronyms defined in clause 4, the following acronym applies to this annex:

LLC	Lossless Coding

## C.4	Conventions

The notational conventions are detailed below:

–	L0, L1 and L2 refer to the ITU-T G.711.1 layers and are respectively ITU-T G.711 compatible core, lower-band enhancement, and higher-band enhancement bitstreams.

## C.5	General description of the ITU-T G.711.1 LLC coder

The ITU-T G.711.1 LLC reduces the transmission bandwidth of ITU-T G.711.1 frames by replacing its core ITU-T G.711 bitstream with ITU-T G.711.0 in a bit-exact way. The straightforward way to apply ITU-T G.711.0 on ITU-T G.711.1 frames is to encode ITU-T G.711 data of each ITU-T G.711.1 frame by ITU-T G.711.0 encoder with 5 ms frame length. Nevertheless, during the characterization of ITU-T G.711.0, it has been shown that the compression ratio of ITU-T G.711.0 is higher when longer frames are used. To achieve higher compression, all the ITU-T G.711 data of multiple ITU-T G.711.1 frames are grouped and encoded with ITU-T G.711.0 encoder. The ITU-T G.711.0 can then favour frame lengths larger than 5 ms to achieve the best compression.

The new ITU-T G.711.1 LLC extension has no quality degradation compared to ITU-T G.711.1, as it losslessly encodes the ITU-T G.711.1 core bitstream. There is no additional algorithmic delay to ITU-T G.711.1, except the one that could be implied by the usage of frame sizes larger than 5 ms. It keeps the same robustness against packet losses as ITU-T G.711.1.

### C.5.1	Algorithmic delay

Algorithmic delay of the ITU-T G.711.1 LLC varies depending on the ITU-T G.711.1 LLC frame length L, where L is defined by the number of ITU-T G.711.1 frames Fn contained in an ITU-T G.711.1 LLC frame (L = 5 × Fn ms).

The algorithm delay is (6.875 + L) ms. Note that the algorithmic delay of ITU-T G.711.1 is 11.875 ms. For a 5 ms framing (Fn = 1, L = 5), the ITU-T G.711.1 LLC has no additional algorithmic delay to ITU-T G.711.1 codec.

### C.5.2	Computational complexity and storage requirements

The worst case complexity is the addition of the worst case complexity of ITU-T G.711.1 codec and the worst case complexity of ITU-T G.711.0 codec.

Table C.5-1 – Worst case computational complexity of the 
ITU-T G.711.1 LLC coder (WMOPS)

| Encoder | Decoder | Total |
| --- | --- | --- |
| 6.5 | 3.9 | 10.4 |

Table C.5-2 – Memory requirements of the ITU-T G.711.1 LLC coder

|  | Encoder | Decoder |
| --- | --- | --- |
| Static RAM (k octets) | 0.19 | 1.50 |
| Scratch RAM (k octets) | 4.25 | 2.07 |
| Data ROM (k octets) | 7.69 |  |
| Program ROM (number of basic ops) | 5497 |  |

### C.5.3	Coder description

The description of the coding algorithm of this Recommendation is made in terms of bit-exact fixed-point mathematical operations. The ANSI C code indicated in clause C.9, which constitutes an integral part of this Recommendation, reflects this bit-exact, fixed-point descriptive approach. The mathematical descriptions of the encoder and decoder can be implemented in other fashions, possibly leading to a codec implementation not complying with this Recommendation. Therefore, the algorithm description of the ANSI C code of clause C.9 shall take precedence over the mathematical descriptions whenever discrepancies are found. A non-exhaustive set of test signals, which can be used with the ANSI C code, is available as an electronic attachment to this Recommendation.

## C.6	ITU-T G.711.1 LLC encoder

The general encoder description focuses on the use of multiple ITU-T G.711.1 frames in an ITU-T G.711.1 LLC frame. Note that the mode of those multiple ITU-T G.711.1 frames within an ITU-T G.711.1 LLC frame must be the same. Figure C.6-1 illustrates the encoder diagram used to combine ITU-T G.711.1 bitstream for ITU-T G.711.1 LLC. In order to maximize the coding efficiency of ITU-T G.711.0, the framing of the input ITU-T G.711.1 bitstream is firstly modified to group all the L0 bits together. Those grouped L0 bits (ITU-T G.711 compatible bitstream) are then losslessly encoded using ITU-T G.711.0 with a larger frame-size. A proposed selection of the ITU-T G.711.0 frame length is described in Appendix II. Finally, the ITU-T G.711.0 bitstream (lossless coding of the core ITU-T G.711 bitstream) and ITU-T G.711.1 extension layers are combined in an ITU-T G.711.1 LLC bitstream.

When Fn ITU-T G.711.1 frames are carried in a single ITU-T G.711.1 LLC frame, all L0 layer bits (ITU-T G.711 compatible bitstream) are extracted from the ITU-T G.711.1 bitstream and grouped together. Fn can vary from one ITU-T G.711.1 LLC frame to another. The selected ITU-T G.711.0 frame lengths are arbitrarily defined as long as they meet the following condition:

(C-1)

where L is the ITU-T G.711.1 LLC frame length calculated by L = 5 × Fn, K is the number of ITU-T G.711.0 frames in the ITU-T G.711.1 LLC framing, and Ln, n = 1,..., K are selected ITU-T G.711.0 frame lengths.

Figure C.6-1 – Encoder block diagram of ITU-T G.711.1 LLC

## C.7	ITU-T G.711.1 LLC decoder

Figure C.7-1 illustrates the decoder diagram of the ITU-T G.711.1 LLC. Firstly, the ITU-T G.711.0 bitstream is decoded by the ITU-T G.711.0 decoder to obtain the ITU-T G.711.1 L0 bitstream. Then, the losslessly decoded ITU-T G.711.1 L0 bitstream is recombined with ITU-T G.711.1 extension layer bitstream to recover the ITU-T G.711.1 bitstream.

Some informative implementations for decoding ITU-T G.711.0 bitstream embedded in an ITU-T G.711.1 LLC frame are shown in Appendix III.

Figure C.7-1 – Decoder block diagram of ITU-T G.711.1 LLC

## C.8	Description of the transmitted parameters

Table C.8-1 shows transmitted parameters for an ITU-T G.711.1 LLC frame. Note that the existence of symbols L1(n), L2(n) and the number of those symbols vary depending on the number of ITU-T G.711.1 frames in an ITU-T G.711.1 LLC frame and the ITU-T G.711.1 operating mode.

Table C.8-1 – Transmitted parameters

| Layer | Symbol | Description | Bits |
| --- | --- | --- | --- |
|  | Fn | Number of ITU-T G.711.1 frames in an ITU-T G.711.1 LLC frame | 8 |
| L0 |  | ITU-T G.711.0 encoded L0 bistream | Variable (8 × Ni) |
| L1, L2 | L1(n), L2(n) | Layer 1 and 2 bitstream | (80 + 80) × Fn |

## C.9	Bit-exact description of the ITU-T G.711.1 LLC codec

The ANSI C code simulating the ITU-T G.711.1 LLC codec in 16-bit fixed-point is contained in the electronic attachment to this Recommendation.

### C.9.1	Use of the simulation software

The command lines for the encoder and decoder of the ITU-T G.711.1 LLC are as follows:

–	encoder [-options] <law> <inputfile> <bitstreamfile>

–	decoder [-options] <law> -mode <modenum> <codefile> <bitstreamfile> <outputfile>

Two options have been added to the ITU-T G.711.1 encoder command line:

–	-g711llc, to indicate that the ITU-T G.711.1 LLC is enabled;

–	-Fn #, to indicate the number of ITU-T G.711.1 frames in an ITU-T G.711.1 LLC frame.

One additional option is needed for the ITU-T G.711.1 decoder command line:

–	-g711llc, to indicate that the ITU-T G.711.1 LLC is enabled.

Note that the argument to -Fn should be able to vary from one ITU-T G.711.1 LLC frame to another, but is not implemented in this simulation software.

### C.9.2	Organization of the simulation software

The organization is based on the ITU-T G.711.1 simulation software. The modification of the simulation software updated main functions in encoder.c and decoder.c of ITU-T G.711.1 and integrating ITU-T G.711.0 codec functions as a static library into ITU-T G.711.1 project.

…

Appendix II

Frame size selection

(This appendix does not form an integral part of this Recommendation.)

This appendix to ITU-T G.711.1 is for use with ITU-T G.711.1 Annex C and provides a reference implementation as follows:

Step 1: Select the maximum frame length ln in the ITU-T G.711.0 frame length set (5/10/20/30/40 ms) which is less than or equal to L. The length ln is as the current ITU-T G.711.0 coding frame length.

Step 2: Update L parameter as L = L – ln and repeat step 1 until current L is zero.

Appendix III

Decoding of the ITU-T G.711.0 bitstream part of ITU-T G.711.1 LLC bitstreams

(This appendix does not form an integral part of this Recommendation.)

This appendix to ITU-T G.711.1 is for use with ITU-T G.711.1 Annex C and provides two possible informative approaches for decoding the ITU-T G.711.0 bitstream.

The decoding of the ITU-T G.711.0 bitstream embedded in the ITU-T G.711.1 LLC can work either of following ways (but other ways are possible):

## III.1	Case 1: When the first boundary octet of the ITU-T G.711.0 bitstream and the number of octet Ni for the ITU-T G.711.0 bitstream are given

1)	Initialize the number of processed octets Pi (Pi = 0).

Repeat 2) to 6) until Pi ≥ Ni (until all Ni octets are processed).

2)	Try to read min{320 + 1, Ni – Pi} octets to the internal buffer from the Pith octet of the ITU-T G.711.0 bitstream, where Ni – Pi is the remaining octets not yet processed.

Note that the maximum possible code size of an ITU-T G.711.0 frame is 320 + 1 bytes.

3)	Decode the first octet in the internal buffer and get the number of ITU-T G.711 samples Nt that should be decoded for the current ITU-T G.711.0 bitstream being processed.

4)	Decode the following octets until the Nt ITU-T G.711 samples are retrieved for the first ITU-T G.711.0 frame and count the processed octets in the internal buffer Pt.

5)	Now the decoder knows how many octets were in the current ITU-T G.711.0 frame being processed (Pt ≤ 321 octets).

6)	Forward the counter for the number of processed octet Pi + = Pt.

## III.2	Case 2: When the first boundary octet of the ITU-T G.711.0 bitstream and the number of ITU-T G.711.1 frames Fn are given

1)	Initialize the number of decoded samples Ns and number of processed octets Pi (Ns = 0, = 0). Set Ni as the largest possible size for the expected ITU-T G.711.0 encoded bitstream. In general, Ni ≥ 41 × Fn is enough.

Repeat 2) to 6) until Ns ≥ 40 × Fn (until number of samples for Fn ITU-T G.711.1 frames are processed).

2)	Try to read min{320 + 1, Ni – Pi} octets to the internal buffer from the Pith octet of the ITU-T G.711.0 bitstream, where Ni – Pi is the number of remaining octets not yet processed.

Note that the maximum possible code size of an ITU-T G.711.0 frame is 320 + 1 bytes.

3)	Decode the first octet in the internal buffer and get the number of ITU-T G.711 samples Nt that should be decoded for the first ITU-T G.711.0 bitstream in the internal buffer.

4)	Decode the following octets until Nt ITU-T G.711 samples are retrieved for the first ITU-T G.711.0 frame and count the processed octets in the internal buffer Pt.

5)	Now the decoder knows how many octets were in the current ITU-T G.711.0 frame being processed (Pt ≤ 321 octets).

6)	Update Ns + = Nt and forward the counter for the number of processed octet Pi + = Pt.

Bibliography

…

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | Telecommunication management, including TMN and network maintenance |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Terminals and subjective and objective assessment methods |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks, open system communications and security |
| Series Y | Global information infrastructure, Internet protocol aspects and next-generation networks |
| Series Z | Languages and general software aspects for telecommunication systems |

____________________

This Recommendation contains an electronic attachment containing a floating point reference implementation and a non-exhaustive set of test signals for use with the reference implementation.