|  |  | International Telecommunication Union |  |  |
| --- | --- | --- | --- | --- |
| ITU-T | G.722 |  |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Appendix IV (11/2009) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Coding of voice and audio signals |  |  |  |
|  | 7 kHz audio-coding within 64 kbit/s Appendix IV: A low-complexity algorithm for packet-loss concealment with ITU-T G.722 |  |  |  |
|  | Recommendation ITU-T G.722 – Appendix IV |  |  |  |

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TRANSMISSION MEDIA AND OPTICAL SYSTEMS CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of voice and audio signals | G.710–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| MULTIMEDIA QUALITY OF SERVICE AND PERFORMANCE – GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DATA OVER TRANSPORT – GENERIC ASPECTS | G.7000–G.7999 |
| PACKET OVER TRANSPORT ASPECTS | G.8000–G.8999 |
| ACCESS NETWORKS | G.9000–G.9999 |

For further details, please refer to the list of ITU-T Recommendations.

Recommendation ITU-T G.722 7 kHz audio-coding within 64 kbit/s Appendix IV A low-complexity algorithm for packet-loss concealment with ITU-T G.722

Summary Packet loss concealment (PLC) algorithms, also known as frame erasure concealment algorithms, hide transmission losses in audio systems where the input signal is encoded and packetized, sent over a network, received and decoded before play out. PLC algorithms can be found in most standard CELP-based speech coders. The algorithm described in Appendix IV of Recommendation ITU-T G.722 provides a method for ITU-T G.722. The decoder comprises three stages: lower sub-band decoding, higher sub-band decoding and quadrature mirror filter (QMF) synthesis. In the absence of frame erasures, the decoder structure is identical to ITU-T G.722, except for the storage of the two decoded signals, of the high and low bands. In case of frame erasures, the decoder is informed by the bad frame indication (BFI) signalling. It then performs an analysis of the past lower-band reconstructed signal and extrapolates the missing signal using linear-predictive coding (LPC), pitch-synchronous period repetition and adaptive muting. Once a good frame is received, the decoded signal is cross-faded with the extrapolated signal. In the higher band, the decoder repeats the previous frame pitch-synchronously, with adaptive muting and high-pass post-processing. The adaptive differential pulse code modulation (ADPCM) states are updated after each frame erasure. The PLC algorithm described in this appendix meets the same quality requirements as the PLC in Appendix III of Recommendation ITU-T G.722, but with a lower complexity. Almost no additional complexity is added compared with normal ITU-T G.722 decoding (worst-case additional complexity is 0.07 WMOPS). An alternative quality-complexity trade-off is provided by Appendix III. This revision of Appendix IV of Recommendation ITU-T G.722 introduces a minor correction in clause IV.6.1.4 and in line 277 of g722.c (no other ANSI-C source code files were changed). The revised ANSI-C source code is labelled Release 1.2 and is available as an electronic attachment to this appendix.

Source Appendix IV to Recommendation ITU-T G.722 was agreed on 6 November 2009 by ITU-T Study Group 16 (2009-2012).

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications, information and communication technologies (ICTs). The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementers are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database at http://www.itu.int/ITU-T/ipr/.

ITU  2010

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

Appendix IV – A low-complexity algorithm for packet loss concealment with G.722		1

IV.1	Scope		1

IV.2	References		1

IV.3	Abbreviations		1

IV.4	Notations and conventions		2

IV.5	General description of the ITU-T G.722 PLC algorithm		3

IV.6	Functional description of the ITU-T G.722 PLC algorithm		4

IV.7	Bit-exact description of the ITU-T G.722 PLC algorithm		15

Electronic attachment – ANSI-C code

Recommendation ITU-T G.722

7 kHz audio-coding within 64 kbit/s

Appendix IV

A low-complexity algorithm for packet-loss 
concealment with ITU-T G.722

## IV.1	Scope

This appendix describes a low-complexity packet loss concealment (PLC) algorithm for use by the ITU-T G.722 audio decoder to limit speech quality degradation in the presence of packet loss. The statistical analysis of the ITU-T G.722 PLC selection test results has demonstrated that this appendix meets the same quality requirements as Appendix III. This is achieved with almost no additional complexity compared with ITU-T G.722 normal decoding (0.07 WMOPS), but with a lower quality than Appendix III. Accordingly, this algorithm is suitable for applications that may encounter frame erasures or packet losses, with a special focus on applications having strong complexity constraints and on low-cost devices. As examples, these applications include DECT next generation and VoIP.

The references, abbreviations and notations used throughout this appendix are defined in clauses IV.2, IV.3 and IV.4, respectively. Clause IV.5 gives a general description of the algorithm and includes delay and complexity information. Clause IV.6 contains the detailed functional description of the algorithm. Finally, clause IV.7 provides the information related to the simulation software.

## IV.2	References

[ITU-T G.191 Ann.A]	Recommendation ITU-T G.191 (2005), Software tools for speech and audio coding standardization. Annex A: List of software tools available.

[ITU-T G.192]			Recommendation ITU-T G.192 (1996), A common digital parallel interface for speech standardization activities.

[ITU-T G.722]			Recommendation ITU-T G.722 (1988), 7 kHz audio-coding within 64 kbit/s.

[ITU-T G.729]			Recommendation ITU-T G.729 (2007), Coding of speech at 8 kbit/s using conjugate-structure algebraic-code-excited linear prediction (CS-ACELP).

## IV.3	Abbreviations

This appendix uses the following abbreviations and acronyms:

FIR	Finite Impulse Response

HB	Higher Band

HPF	High Pass Filter

IIR	Infinite-Impulse Response

LB	Lower Band

LP	Linear Prediction

LPC	Linear Predictive Coding

LSB	Least Significant Bit

LTP	Long-Term Prediction

MSB	Most Significant Bit

PLC	Packet Loss Concealment

QMF	Quadrature Mirror Filter

WB	Wideband

WMOPS	Weighted Million Operations Per Second

## IV.4	Notations and conventions

Throughout this appendix, the terms "frame" and "packet" are considered to be equivalent.

The notation used in [ITU-T G.722] is followed. The notational conventions are detailed below:

–	Each frame comprises 2L samples at 16 kHz, where L = 80 for 10-ms frames and 160 for 20-ms frames.

–	Time-domain signals are denoted by their symbol and a sample index between parentheses [e.g., y(n)]. The symbol n is used as sample index.

–	By convention, for signals sampled at 8 kHz, the current frame corresponds to n = 0, , L – 1. Samples with an index n  0 are past samples, and samples with index n  L are future samples.

Table IV.1 – List of symbols

| Type | Name | Description |
| --- | --- | --- |
| Filters | A(z) | 8th-order LPC filter |
|  | B(z) | 2nd-order LPC filter |
|  | Hpre(z) | Lower-band high-pass filter |
|  | Hpost(z) | Higher-band high-pass filter |
|  | Hdec(z) | 4:1 decimation filter |
| Signals | xl(n) | Lower-band ADPCM decoder output |
|  | yl(n) | Lower-band extrapolated signal |
|  | zl(n) | Lower-band reconstruction |
|  | xh(n) | Higher-band ADPCM decoder output |
|  | yh(n) | Higher-band extrapolated signal |
|  | zh(n) | Higher-band reconstruction |
|  | e(n) | Lower-band LP residual signal |
| Parameters | ai | Lower-band LP coefficients |
|  | T0 | Pitch delay in lower band |
|  | g_mute_lb | Lower-band muting factor |
|  | g_mute_hb | Higher-band muting factor |

## IV.5	General description of the ITU-T G.722 PLC algorithm

The proposed ITU-T G.722 PLC algorithm is integrated in the standard ITU-T G.722 decoder. Note that the ITU-T G.722 encoder is unchanged (identical to clause 3 of [ITU-T G.722]).

### IV.5.1	Modified ITU-T G.722 decoder

The modified ITU-T G.722 decoder is illustrated in Figure IV.1. Decoding is performed in two subbands, which are combined using the QMF synthesis filterbank of [ITU-T G.722]. Compared to [ITU-T G.722], this decoder includes a mechanism to conceal frame erasures, shown in the highlighted (grey-shaded) blocks.

Figure IV.1 – Block diagram of ITU-T G.722 decoder with PLC 
(the highlighted blocks correspond to the PLC algorithm)

The modified ITU-T G.722 decoder generates an output signal sampled at 16 kHz and divided into frames of 10 or 20 ms. Its behaviour depends on the type of the current and previous frame (either good or bad frame):

–	Without frame erasures (i.e., in the presence of good frames only):

The bit stream of the lower band (LB) is decoded according to the specified mode (1, 2 or 3 indicating 64, 56 or 48 kbit/s, respectively). The cross-fading block does not change the reconstructed signal, i.e., zl(n)  xl(n) similarly, the bit stream of the higher band (HB) is decoded. The A and B switches select uh(n)  xh(n) and zh(n)  uh(n)  xh(n), respectively. The wideband reconstruction y(n) is obtained, as in [ITU-T G.722], from the QMF synthesis filterbank. The decoded signals zl(n) and zh(n) are stored to be used in case of erasure in future frames.

–	In case of frame erasure:

•	In the lower band, for the first erased frame, short- and long-term predictors are updated using the past valid signal zl(n), n < 0. Class information is also extracted. The signal yl(n) is generated using these predictors and the class information. The signal for the erased frame is reconstructed as zl(n) = yl(n), n  0, , L – 1. In addition, ADPCM states are updated. The process of erased frame reconstruction and ADPCM states update is repeated until a good frame is received. Note that not only the missing frame is generated, but also an additional 10 ms of signal, yl(n), n  L, , L + 79, to be used for cross-fading. Once a good frame is received, the signal reconstructed by the ADPCM decoder, xl(n), n  0, , L – 1, and the extrapolated signal, yl(n), n  0, , L – 1 are cross-faded. This cross-fading is used only during the first 10 ms following the last erasure.

•	In the higher band, the missing frame is extrapolated using the past signal zh(n), n < 0, and ADPCM states are updated. The extrapolated signal yh(n) is obtained by repeating pitch-synchronously the previous frame of zh(n). The "A" switch selects uh(n) = yh(n), n  0, , L – 1. The signal uh(n) is high-pass filtered by a remove-DC filter Hpost to obtain vh(n). The "B" switch selects zh(n) = vh(n), n  0, , L – 1. This process is repeated until a good frame is received. Once a good frame is received, the "A" switch selects the ADPCM decoder output: uh(n) = xh(n), n  0, , L – 1. During the first 4 s following the last erasure, the "B" switch selects: zh(n) = vh(n), n  0, , L – 1; after 4 s, the B switch selects: zh(n) = uh(n), n  0, , L – 1.

### IV.5.2	Delay and complexity

The proposed ITU-T G.722 PLC operates with no extra delay.

The computational complexity and storage requirements are summarized in Table IV.2.

Table IV.2 – Complexity figures of the ITU-T G.722 PLC algorithm

a)		Observed worst-case complexity, in WMOPS, based on STL2005 (figures in brackets represent the additional WMOPS compared with ITU-T G.722 decoding)

| Frame length | 10 ms | 20 ms |
| --- | --- | --- |
| ITU-T G.722 decoding (no PLC) | 3.11 | 3.10 |
| ITU-T G.722 with PLC | 3.18 [0.07] | 3.15 [0.05] |

b)		Memory requirement (figures in brackets represent the maximum additional complexity compared with ITU-T G.722 decoding)

| Frame length | Static RAM (in 16-bit kwords) | Scratch RAM (in 16-bit kwords) | Total RAM | Program ROM (in number of basic ops and function calls) | Data ROM (in 16-bit kwords) |
| --- | --- | --- | --- | --- | --- |
| 10 ms | 967 [863] | 692 [452] | 1659 [1315] | 1061 [615] | 882 [109] |
| 20 ms |  | 963 [452] | 1899 [1315] |  |  |

Compared to the ITU-T G.722 reference decoder, the PLC algorithm brings an additional worst-case complexity of 0.07 WMOPS. Note that the complexity peak occurs at the first valid frame after an erasure.

## IV.6	Functional description of the ITU-T G.722 PLC algorithm

### IV.6.1	Lower-band decoding

#### IV.6.1.1	ADPCM decoder in case of a good frame

Same as clauses 4.1, 4.2 and 4.3 of [ITU-T G.722].

In addition, the counter cnt_mute_lb and muting factor g_mute_lb (used in clause IV.6.1.2.7) for adaptive muting are reset:

cnt_mute_lb  0

g_mute_lb  1

and the signal zl(n) is stored to be used in case of erasure in future frames.

#### IV.6.1.2	Extrapolation of missing frame: Case of bad frame following a good frame

The extrapolation of a missing frame in the lower band is illustrated in Figure IV.2. It comprises analysis of the past valid signal zl(n) n,  0, followed by synthesis of the signal yl(n), n  0, , 
L – 1. The missing frame in the lower band corresponds to xl(n), n  0, , L – 1 where L = 80 for 10-ms frames and L = 160 for 20-ms frames.

The past signal zl(n), n  –297, , –1 is buffered using a buffer length of 297 samples, which can be divided as follows:

–	the 288 samples corresponding to twice the maximal pitch delay (2 × 144) used in the PLC algorithm;

–	one sample for pitch jitter; and

–	eight samples used for LPC memory.

This buffer length makes it possible to store the last two pitch periods of the lower-band reconstruction.

Figure IV.2 – Block diagram of lower-band extrapolation of missing frame

##### IV.6.1.2.1		LP analysis

The short-term analysis and synthesis filters, A(z) and 1/A(z), are based on eighth-order linear prediction (LP) filters. The LP analysis filter is defined as:

(IV-1)

The LP analysis consists of two parts: windowing and autocorrelation computation, and the Levinson-Durbin algorithm. The autocorrelation computation – including 60-Hz bandwidth expansion and 40-dB white-noise correction – is identical to that in clause 3.2.1 of [ITU-T G.729], with only one difference. The LP window here is an asymmetrical Hamming window defined as:

(IV-2)

This window wlp(n), which is limited to 80 samples (10 ms at 8-kHz sampling frequency) to reduce complexity, is applied to the last 10 ms of zl(n), n  –80, , –1. The Levinson-Durbin algorithm is identical to clause 3.2.2 of [ITU-T G.729].

After the LP analysis, the past signal zl(n), n  –289, , –1, is filtered through A(z) to obtain the residual signal e(n), n  –289, , –1

(IV-3)

##### IV.6.1.2.2		Pre-processing

A high-pass filter protects against undesired low-frequency components. A first-order pole/zero filter with a cut-off frequency of 50 Hz is used. This filter is given by:

(IV-4)

The past signal zl(n), n  –288, , –1, is filtered through Hpre(z) to obtain the pre-processed signal zlpre(n), n  –288, , –1.

##### IV.6.1.2.3		LTP analysis

The PLC algorithm uses pitch period repetition. The pitch period or pitch delay, T0, is determined on the past valid pre-processed signal just before erasure, zlpre(n), n  –288, , –1. T0 is estimated in open loop by a long-term predictive (LTP) analysis.

Figure IV.3 – Block diagram of LTP analysis

As illustrated in Figure IV.3, pitch estimation is conducted in the following steps:

–	The signal zlpre(n), n  –288, , –1, is low-pass filtered by Hdec(z), where:

(IV-5)

is an eighth-order FIR filter, and decimated by a factor of 4 to obtain the signal t(n), , sampled at 2 kHz. The filter memories are initialized to 0 each time.

–	The signal t(n), n  –72, , –1, is weighted by a filter B(z/), where B(z)  1 – b1z–1 – b2z–2 and  = 0.94, to obtain the signal tw(n), n  –72, , –1. The coefficients of B(z) are obtained by 2nd-order LP analysis of t(n) using the windowing, autocorrelation computation and Levinson-Durbin algorithm described in the previous paragraph. Note that only the last 72 samples of the window wlp(n), n  –72, , –1, are used, which gives a 36-ms time support at 2-kHz sampling frequency.

–	A first estimation Tds of the pitch delay is computed in the weighted decimated signal domain by normalized cross-correlation as follows:

a)	Initialization: Tds = 18.

b)	Computation of the normalized cross-correlation:

, 	(IV-6)

c)	Determination of the first delay i0 in [1,35] for which r(i) < 0. Note that if , steps d) and e) are omitted.

d)	Determination of the lower bound for the maximum correlation search:

e)	Search for the maximum correlation in :

(IV-7)

A procedure favouring the smaller pitch values to avoid choosing pitch multiples is also applied.

–	The first pitch delay estimation Tds is then refined in the pre-processed signal domain by searching the cross-correlation maximum in the neighbourhood of T  4Tds to obtain T0 as follows:

(IV-8)

with:

(IV-9)

–	The value Rmax is computed as Rmax = R(T0).

##### IV.6.1.2.4		Signal classification

The PLC strategy uses signal characteristics to optimize quality. For instance, if the frame preceding an erasure is a non-stationary segment (e.g., plosives), the signal should be rapidly muted; if this frame is a stationary segment (e.g., strongly voiced speech), it can be pitch-synchronously repeated and slowly damped. Classification is used in the proposed PLC algorithm for LP residual extrapolation and muting control.

The signal zl(n), n  –288, , –1 preceding an erasure is classified into one out of five possible classes, which are defined below:

–	TRANSIENT for transients with large energy variation (e.g., plosives);

–	UNVOICED for unvoiced signals;

–	VUV_TRANSITION corresponding to a transition between voiced and unvoiced signals;

–	WEAKLY_VOICED for weakly voiced signals (e.g., onset or offset of vowels);

–	VOICED for voiced signals (e.g., steady vowels).

The features used for classification are listed below:

–	Normalized correlation Rmax, which is a side product of the LTP analysis.

–	Subband energy ratio, which is obtained here in the log domain by taking the difference between the lower- and higher-band ADPCM scale factors, NBH – NBL using the ITU-T G.722 notations. NBL and NBH are computed as in clause 3.5 of [ITU-T G.722].

–	Zero-crossing rate zcr of zl(n), n  –80, , –1, defined as:

(IV-10)

where the comparisons  and  give a binary result (1 for true, 0 for false) and "AND" is the AND bit operation.

–	Number cnt_peak of detected large peaks in the last pitch period:

(IV-11)

where the comparison > gives a binary result (1 for true, 0 for false) and T0 is the pitch delay estimated by the LTP analysis. The counter, cnt_peak, represents the number of detected large peaks in the last pitch period.

Based on these features, the signal category, class, is obtained by heuristics according to the flowchart shown in Figure IV.4. Note that if class is set to UNVOICED, the pitch delay T0 may be modified to avoid artefacts due to low-pitch delay values. Note also that if class is not VOICED, and T0 is even, T0 is increased by 1. The so-called pitch delay T0 determines the repetition period used in the residual signal generation procedure.

Figure IV.4 – Classification flowchart

##### IV.6.1.2.5		Modification/pitch repetition of LP residual

A pitch repetition procedure is used to estimate the LP residual e(n), n  0, , L – 1 in the missing frame from the residual signal of the repetition period. As mentioned above, the repetition period contains the last valid T0 residual samples e(n), n  –T0, , – 1.

LP residual modification

Before performing the pitch repetition procedure, the repetition period is modified if class is not VOICED. The modification consists in limiting the magnitude of each sample of the repetition period as follows:

(IV-12)

where:

(IV-13)

Pitch repetition of LP residual

The LP residual e(n), n  0, , L – 1, in the missing frame is extrapolated based on the classification result:

–	If class is VOICED, the missing signal, e(n), n  0, , L – 1, is obtained by repeating pitch-synchronously the repetition period:

(IV-14)

–	If class is not VOICED, the pitch-synchronous repetition procedure is modified to avoid over-voicing by introducing, sample by sample, a small jitter using the following procedure. The samples of the repetition period can be viewed as grouped two by two; then, every two samples forming a group are swapped and the swapped groups are concatenated to form the extrapolated residual signal. If T0 < L, the extrapolated residual signal extends the repetition period and the procedure is iterated. With this procedure, the missing signal, e(n), n  0, , L – 1 is obtained as:

(IV-15)

This procedure is illustrated in Figure IV.5.

Figure IV.5 – LP residual e(n) extrapolation with jitter (if class is not VOICED)

In addition, 80 extra samples (10 ms), e(n), n  L, , L  79 are generated using the above equations for the purpose of cross-fading.

##### IV.6.1.2.6		LP synthesis

After LP synthesis, the reconstructed missing frame is given by:

(IV-16)

where e(n), n  0, , L – 1, is the extrapolated residual signal and L is the frame length.

In addition, 80 samples (10 ms), ylpre(n), n  L, , L  79 are generated using the above equation; these samples are used for cross-fading.

##### IV.6.1.2.7		Adaptive muting

The energy of the reconstructed signal is controlled by applying to each sample a gain factor that is computed and adapted sample by sample. Thus, the synthesized signal ylpre(n) is muted sample by sample with an adaptive muting factor g_mute_lb for n  0, , L – 1 to obtain the reconstructed lower-band signal yl(n):

(IV-17)

The extra synthesis needed for cross-fading, ylpre(n), n  L, , L  79, is muted in the same way: yl(n)  g_mute_lb  ylpre (n).

The calculation of g_mute_lb is performed using several parameters, inc_mute, fac1, fac2p, and fac3p, for 10- and 20-ms frames, as well as cf10 for 10-ms frames. These parameters depend on the value of class as indicated in Table IV.3.

Table IV.3 – Adaptive muting parameters

| Parameter | class = TRANSIENT | class = UV_TRANSITION | Other cases |
| --- | --- | --- | --- |
| inc_mute | 4 | 2 | 1 |
| fac1 | 409 | 10 | 10 |
| fac2p | 409 | 10 | 20 |
| fac3p | 409 | 399 | 190 |
| cf10 | 0 | 399 | 20 |

Computation of muting factor for the first erased 10-ms frame

In this case, the muting factor is adapted sample by sample with fac1 as follows, for n = 0, ... , 79:

where g_mute_lb has been initialized to 1 (see clause IV.6.1.1).

Then cnt_mute_lb is updated at the end of the first erased 10-ms frame:

where cnt_mute_lb has been initialized to 0 (see clause IV.6.1.1).

The muting factor for the extra synthesis is also adapted sample by sample with cf10 in the same way for n = 80, ... , 159:

Then cnt_mute_lb is also updated at the end of the extra frame:

The muting factor adaptation is illustrated in Figure IV.6.

Figure IV.6 – Muting factor as a function of the sample index for 10-ms frames 
(80 samples for current frame + 80 samples for cross-fading part)

Computation of muting factor for other cases (20-ms frames or consecutive erased
10-ms frames)

In these cases, the muting factor g_mute_lb is also adapted sample by sample with the adaptation factor fac1, for n  0, , L – 1. However, g_mute_lb may be further decreased depending on the value of cnt_mute_lb. The amount of decrease also depends on class. The procedure is the following:

if

if

if

The muting factor for the extra synthesis needed for cross-fading, ylpre(n), n  L, , L  79, is muted in the same way for n  L, , L  79.

NOTE – For the first erased 20-ms frame, g_mute_lb and cnt_mute_lb have been initialized before the first erasure (see clause IV.6.1.1).

The muting factor adaptation is illustrated in Figure IV.7. The gain values for the first 160 samples are identical to those shown in Figure IV.6.

Figure IV.7 – Muting factor as a function of the sample index

#### IV.6.1.3	Extrapolation of missing frame: Case of bad frame following a bad frame

In the case of a bad frame following a bad frame, the analysis parameters computed for the first erased frame (ai, i = 1, , 8, T0, class) are kept. The signal generated in the previous frame for cross-fading is copied to yl(n) n  0, , 79. The last L samples, yl(n), n  80, , L  79 including the 10-ms part used for cross-fading with the next frame, are synthesized as described in clause IV.6.1.2.

#### IV.6.1.4	Update of ADPCM decoder states

The states of the lower-band ADPCM decoder are updated after extrapolating missing frames to help in recovery from frame erasures. This update is more elaborate than a simple ADPCM decoder reset. However, to minimize complexity, the ADPCM states are updated based on available or a priori information, without additional processing. The states are modified as follows, using ITU-T G.722 notation:

DLT = 0,

PLT = ,

RLT =

SL =

SZL =

if

DETL = 32

NBL = 0

#### IV.6.1.5	Cross-fading

The cross-fading is detailed in Table IV.4. The cross-fading window is based on the Bartlett window (triangular) with a time support of 10 ms.

Table IV.4 – Cross-fading operation

|  |  | Current frame |  |
| --- | --- | --- | --- |
|  |  | Bad | Good |
| Previous frame | Bad | zl(n) yl(n), n 0, , L – 1 | n 0, , 79 and zl(n) xl(n), n 80, , L – 1 |
|  | Good | zl(n) yl(n), n 0, , L – 1 | zl(n) xl(n), n 0, , L – 1 |

### IV.6.2	Higher-band decoding

#### IV.6.2.1	ADPCM decoder in case of a good frame

Same as clauses 4.1, 4.2 and 4.3 of [ITU-T G.722].

In addition, the counter cnt_mute_hb and muting factor g_mute_hb (used in clause IV.6.2.2.2) for adaptive muting are reset:

and the signal zh(n) is stored to be used in case of erasure in future frames.

#### IV.6.2.2	Extrapolation of missing frame

The extrapolation of a missing frame in the higher band uses the past signal zh(n), n = –160, ..., –1, buffered using a buffer length of 160 samples.

##### IV.6.2.2.1		Repetition

The extrapolation of a missing frame in the higher band consists of pitch synchronous repeating of the previous signal zh(n) if class = VOICED; otherwise, the repetition period is set to 80 samples (10 ms). In other words:

, 	(IV-18)

where Th  T0 if class = VOICED, Th  80 otherwise.

##### IV.6.2.2.2		Adaptive muting

As for the lower-band reconstructed signal, the energy of the higher-band reconstructed signal is also controlled by applying a gain factor computed and adapted sample by sample. The synthesized signal yhpre(n) is muted sample by sample with an adaptive muting factor g_mute_hb for n  0, , L – 1 to obtain the reconstructed higher-band signal yh(n). The muting of the higher band is identical to the muting of the lower band described in clause IV.6.1.2.7. However, since no cross-fading is used in the higher band, a separate counter cnt_mute_hb and muting factor g_mute_hb are used as cnt_mute_hb is always 80 samples in advance compared to cnt_mute_lb after one erasure.

#### IV.6.2.3	High-pass post-processing

In case of frame erasures, a DC offset of very small magnitude may appear in the higher-band reconstruction uh(n) n  0, , L – 1 and also affect the first consecutive good frames. After the QMF synthesis, this introduces an 8-kHz component. To avoid this annoying high-frequency noise, a first-order pole/zero filter with a cut-off frequency of 50 Hz is used. This filter is given by:

(IV-19)

The signal uh(n) is filtered through Hpost(z) to obtain vh(n) where n  0, , L – 1.

This filter is used during the erased frames and the first 4 s following the erasure.

#### IV.6.2.4	Update of ADPCM decoder states

Similar to lower-band decoding, the states of the higher-band ADPCM decoder are updated after extrapolating a missing frame. The update is described below using ITU-T G.722 notation:

NBH = NBH/2

DETH = scaleh(NBH)

if

NBH = 0

DETH = 8

The update is restricted to the higher-band scale factor.

### IV.6.3	QMF synthesis filterbank

Same as clause 4.4 of [ITU-T G.722], except that the shift of 22 samples of the delay line after each call of the qmf_rx function (for each two output samples) is replaced by the use of a linear buffer of length L+22 in case of bad frame decoding. The memory part of this buffer is updated at the beginning of each frame, and the filter memory is saved at the end of each frame.

## IV.7	Bit-exact description of the ITU-T G.722 PLC algorithm

The current version of the ANSI-C code simulating the candidate ITU-T G.722 PLC algorithm in 16-bit fixed-point is Release 1.2. The following subclauses summarize the use of this simulation code, and how the software is organized. The G722PLC algorithm is implemented using the ITU-T G.722 code from the ITU-T software tool library (STL2005). The mathematical descriptions of the PLC algorithm (clauses IV.5 and IV.6) can be implemented in several other fashions. Therefore, the algorithm description of the ANSI-C code of this clause shall take precedence over the mathematical descriptions of clauses IV.5 and IV.6 whenever discrepancies are found.

### IV.7.1	Use of the simulation software

The command line for the ITU-T G.722 decoder with PLC is as follows:

decg722 [-fsize N] g192_bst output

where N is the frame size at 16 kHz (default: 160).

The output file is a sampled data file containing 16-bit PCM signals.

The mapping table of the encoded bit stream is contained in the simulation software.

Organization of the simulation software

See Tables IV.5 and IV.6.

Table IV.5 – List of tables added by the PLC algorithm

| Table name | Size | Description |
| --- | --- | --- |
| G722PLC_lpc_win_80 | 80 | LPC window |
| G722PLC_lag_h | 8 | Lag window for bandwidth expansion (high part) |
| G722PLC_lag_l | 8 | Lag window for bandwidth expansion (low part) |
| G722PLC_fir_lp | 9 | Coefficients of low-pass quarter-band decimation filter |
| G722PLC_b_hp | 3 | High-pass filter coefficients (numerator) |
| G722PLC_a_hp | 3 | High-pass filter coefficients (denominator) |

Table IV.6 – List of files (ITU-T G.722 decoder with PLC)

a)	Identical files – from ITU-T G.191 STL software

| Filename | Description |
| --- | --- |
| softbit.c | ITU-T G.722 soft bit handling |
| g722_com.h | ITU-T G.722 additional header file |
| softbit.h | ITU-T G.722 soft bit handling |
| ugstdemo.h | Definitions for UGST demo programs |

b)	Modified files – from ITU-T G.191 STL software

| Filename | Description |
| --- | --- |
| decg722.c | ITU-T G.722 decoder interface |
| g722.c | ITU-T G.722 main decoder routines |
| g722.h | ITU-T G.722 main header file |
| funcg722.c | ITU-T G.722 library |
| funcg722.h | ITU-T G.722 library |

c)	New files

| Filename | Description |
| --- | --- |
| g722_plc.c, g722_plc.h | PLC library and headers |
| g722_plc_tables.c | PLC tables |
| oper32_b.c, oper32_b.h | Additional basic operators and headers |

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | Telecommunication management, including TMN and network maintenance |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Terminals and subjective and objective assessment methods |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks, open system communications and security |
| Series Y | Global information infrastructure, Internet protocol aspects and next-generation networks |
| Series Z | Languages and general software aspects for telecommunication systems |

____________________

This appendix includes an electronic attachment containing the relevant ANSI-C code.