|  |  | International Telecommunication Union |  |  |
| --- | --- | --- | --- | --- |
| ITU-T | G.107 |  |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (04/2009) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS International telephone connections and circuits – Transmission planning and the E-model |  |  |  |
|  | The E-model: a computational model for use in transmission planning |  |  |  |
|  | Recommendation ITU-T G.107 |  |  |  |

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| Transmission planning and the E-model | G.100–G.109 |
| General Recommendations on the transmission quality for an entire international telephone connection | G.110–G.119 |
| General characteristics of national systems forming part of international connections | G.120–G.129 |
| General characteristics of the 4-wire chain formed by the international circuits and national extension circuits | G.130–G.139 |
| General characteristics of the 4-wire chain of international circuits; international transit | G.140–G.149 |
| General characteristics of international telephone circuits and national extension circuits | G.150–G.159 |
| Apparatus associated with long-distance telephone circuits | G.160–G.169 |
| Transmission plan aspects of special circuits and connections using the international telephone connection network | G.170–G.179 |
| Protection and restoration of transmission systems | G.180–G.189 |
| Software tools for transmission systems | G.190–G.199 |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TRANSMISSION MEDIA AND OPTICAL SYSTEMS CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| MULTIMEDIA QUALITY OF SERVICE AND PERFORMANCE – GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DATA OVER TRANSPORT – GENERIC ASPECTS | G.7000–G.7999 |
| PACKET OVER TRANSPORT ASPECTS | G.8000–G.8999 |
| ACCESS NETWORKS | G.9000–G.9999 |

For further details, please refer to the list of ITU-T Recommendations.

Recommendation ITU-T G.107 The E-model: a computational model for use in transmission planning

Summary Recommendation ITU-T G.107 gives the algorithm for the so-called E-model as the common ITU-T transmission rating model. This computational model can be useful to transmission planners, to help ensure that users will be satisfied with end-to-end transmission performance. The primary output of the model is a scalar rating of transmission quality. A major feature of this model is the use of transmission impairment factors that reflect the effects of modern signal processing devices. In the 2000 version of this Recommendation, an enhanced version of the E-model was provided in order to better take into account the effects of room noise at the send side, and quantizing distortion. With the 2002 version, the impairment due to random packet loss was included in a parametric way for different codecs. Since the 2003 version, an enhanced modelling of quality in case of low talker sidetone levels is provided. The 2005 version enabled more accurate quality predictions for codecs under (short-term) dependent packet loss. This Recommendation now includes Appendix II describing a provisional impairment factor framework for wideband speech transmission. A new reference implementation is given in Appendix III replacing the previous Annex C which has been deleted. The online version of this Recommendation includes an electronic attachment containing a reference implementation of the E-Model in PHP4 scripting language. This electronic attachment is available free of charge on the ITU publications website at http://www.itu.int/rec/T-REC-G.107.

History Edition; Recommendation; Approval; Study Group. 1.0; ITU-T G.107; 1998-12-03; 12. 2.0; ITU-T G.107; 2000-05-18; 12. 3.0; ITU-T G.107; 2002-07-14; 12. 4.0; ITU-T G.107; 2003-03-16; 12. 5.0; ITU-T G.107; 2005-03-01; 12. 5.1; ITU-T G.107 (2005) Amend. 1; 2006-06-13; 12. 6.0; ITU-T G.107; 2008-08-29; 12. 7.0; ITU-T G.107; 2009-04-29; 12.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications, information and communication technologies (ICTs). The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g., interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met. The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU [had/had not] received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementers are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database at http://www.itu.int/ITU-T/ipr/.

ITU  2010

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1	General		1

1.1	Scope		1

1.2	References		1

2	The E-model, a computational model for use in transmission planning		2

2.1	Introduction		2

2.2	Source code		2

3	The structure and basic algorithms of the E-model		2

3.1	Calculation of the transmission rating factor, R		3

3.2	Basic signal-to-noise ratio, Ro		4

3.3	Simultaneous impairment factor, Is		4

3.4	Delay impairment factor, Id		5

3.5	Equipment impairment factor, Ie		6

3.6	Advantage factor, A		7

3.7	Default values		8

Annex A – Conditions of using the E-model		9

A.1	Examples of conditions where caution must be exercised when using the E-model		9

A.2	Conditions for which the performance of the E-model has been improved by updating from the earlier version		10

Annex B – Quality measures derived from the transmission rating factor R		12

Appendix I – Calculation of R from MOSCQE values		15

Appendix II – Provisional impairment factor framework for wideband speech transmission		16

Appendix III – Reference Implementation of the E-model in ITU-T G.107		19

Bibliography		20

Electronic attachment – E-model in PHP4 scripting language

Recommendation ITU-T G.107

The E-model: a computational model for use in transmission planning

# 1	General

## 1.1	Scope

This Recommendation describes a computational model, known as the E-model, that has proven useful as a transmission planning tool for assessing the combined effects of variations in several transmission parameters that affect conversational quality of 3.1 kHz handset telephony. This computational model can be used, for example, by transmission planners to help ensure that users will be satisfied with end-to-end transmission performance whilst avoiding over-engineering of networks. It must be emphasized that the primary output from the model is the "rating factor" R but this can be transformed to give estimates of customer opinion. Such estimates are only made for transmission planning purposes and not for actual customer opinion prediction (for which there is no agreed-upon model recommended by the ITU-T).

This version of this Recommendation now includes a packet loss parameter and enhanced talker sidetone modelling.

The E-model has not been fully verified by field surveys or laboratory tests for the very large number of possible combinations of input parameters. For many combinations of high importance to transmission planners, the E-model can be used with confidence, but for other parameter combinations, E-model predictions have been questioned and are currently under study. Accordingly, caution must be exercised when using the E-model for some conditions; for example, the E-model may give inaccurate results for combinations of certain types of impairments. Annex A provides further information in this regard.

## 1.2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T G.108]	Recommendation ITU-T G.108 (1999), Application of the E-model: A planning guide.
<http://web.itu.int/rec/T-REC-G.108>

[ITU-T G.109]	Recommendation ITU-T G.109 (1999), Definition of categories of speech transmission quality.
<http://web.itu.int/rec/T-REC-G.109>

[ITU-T G.113]	Recommendation ITU-T G.113 (2009), Transmission impairments due to speech processing.
<http://web.itu.int/rec/T-REC-G.113>

[ITU-T P.10]	Recommendation ITU-T P.10/G.100 (2006), Vocabulary for performance and quality of service.
<http://web.itu.int/rec/T-REC-P.10/G.100>

[ITU-T P.833]	Recommendation ITU-T P.833 (2001), Methodology for derivation of equipment impairment factors from subjective listening-only tests.
<http://web.itu.int/rec/T-REC-P.833>

[ITU-T P.834]	Recommendation ITU-T P.834 (2002), Methodology for the derivation of equipment impairment factors from instrumental models.
<http://web.itu.int/rec/T-REC-P.834>

[ITU-T P.862]	Recommendation ITU-T P.862 (2001), Perceptual evaluation of speech quality (PESQ): An objective method for end-to-end speech quality assessment of narrow-band telephone networks and speech codecs.
<http://web.itu.int/rec/T-REC-P.862>

# 2	The E-model, a computational model for use in transmission planning

## 2.1	Introduction

The complexity of modern networks requires that, for transmission planning, the many transmission parameters are not only considered individually but also that their combined effects are taken into account. This can be done by "expert, informed guessing", but a more systematic approach is desirable, such as by using a computational model. The output from the model described here is a scalar quality rating value, R, which varies directly with the overall conversational quality. [ITU-T G.113] gives guidance about specific impairments, including combined effects based upon a simplification of the model. However, the output can also give nominal estimates of user reactions, for instance in the form of percentages finding the modelled connection "good or better" or "poor or worse", as described in Annex B. Furthermore, detailed guidance on the proper application of the E-model, as described in this Recommendation, is provided in [ITU-T G.108]. In addition, the definition of categories of speech transmission quality can be found in [ITU-T G.109].

## 2.2	Source code

The electronic attachment to this Recommendation contains the source code of the core E-model implementation, which is currently available on the ITU-T SG 12 website. The purpose of this code is to ensure that own implementations of the E-model provide results that are consistent with the reference implementation of the website.

# 3	The structure and basic algorithms of the E-model

The E-model is based on the equipment impairment factor method, following previous transmission rating models. It was developed by an ETSI ad hoc group called "Voice Transmission Quality from Mouth to Ear".

The reference connection, as shown in Figure 1, is split into a send side and in a receive side. The model estimates the conversational quality from mouth to ear as perceived by the user at the receive side, both as listener and talker.

Figure 1 – Reference connection of the E-model

The transmission parameters used as an input to the computation model are shown in Figure 1. Values for room noise and for the D-factors are handled separately in the algorithm for the send side and receive side and may be of different amounts. The parameters SLR, RLR and circuit noise Nc are referred to a defined 0 dBr point. All other input parameters are either considered as values for the overall connection, such as OLR (in any case, the sum of SLR and RLR), number of qdu, equipment impairment factors Ie and advantage factor A, or referred to only for the receive side, such as STMR, LSTR, WEPL (for calculation of Listener Echo) and TELR.

There are three different parameters associated with transmission time. The absolute delay Ta represents the total one-way delay between the send side and receive side and is used to estimate the impairment due to excessive delay. The parameter mean one-way delay T represents the delay between the receive side (in talking state) and the point in a connection where a signal coupling occurs as a source of echo. The round-trip delay Tr only represents the delay in a 4-wire loop, where the "double reflected" signal will cause impairments due to listener echo.

## 3.1	Calculation of the transmission rating factor, R

According to the equipment impairment factor method, the fundamental principle of the E-model is based on a concept given in the description of the OPINE model (see [b-ITU-T P-Sup.3]).

Psychological factors on the psychological scale are additive.

The result of any calculation with the E-model in a first step is a transmission rating factor R, which combines all transmission parameters relevant for the considered connection. This rating factor R is composed of:

(3-1)

Ro represents in principle the basic signal-to-noise ratio, including noise sources such as circuit noise and room noise. The factor Is is a combination of all impairments which occur more or less simultaneously with the voice signal. Factor Id represents the impairments caused by delay and the effective equipment impairment factor Ie-eff represents impairments caused by low bit-rate codecs. It also includes impairment due to randomly distributed pack losses. The advantage factor A allows for compensation of impairment factors when there are other advantages of access to the user. The term Ro and the Is and Id values are subdivided into further specific impairment values. The following clauses give the equations used in the E-model.

## 3.2	Basic signal-to-noise ratio, Ro

The basic signal-to-noise ratio Ro is defined by:

(3-2)

The term No [in dBm0p] is the power addition of different noise sources:

(3-3)

Nc [in dBm0p] is the sum of all circuit noise powers, all referred to the 0 dBr point.

Nos [in dBm0p] is the equivalent circuit noise at the 0 dBr point, caused by the room noise Ps at the send side:

(3-4)

where OLR  SLR  RLR. In the same way, the room noise Pr at the receive side is transferred into an equivalent circuit noise Nor [in dBm0p] at the 0 dBr point.

(3-5)

The term Pre [in dBm0p] is the "effective room noise" caused by the enhancement of Pr by the listener's sidetone path:

(3-6)

Nfo [in dBm0p] represents the "noise floor" at the receive side,

(3-7)

with Nfor usually set to 64 dBmp.

## 3.3	Simultaneous impairment factor, Is

The factor Is is the sum of all impairments which may occur more or less simultaneously with the voice transmission. The factor Is is divided into three further specific impairment factors:

(3-8)

Iolr represents the decrease in quality caused by too-low values of OLR and is given by:

(3-9)

where:

(3-10)

The factor Ist represents the impairment caused by non-optimum sidetone:

(3-11)

where:

(3-12)

The impairment factor Iq represents impairment caused by quantizing distortion:

(3-13)

where:

(3-14)

(3-15)

and:

(3-16)

(3-17)

In this equation, qdu means the number of qdu for the whole connection between the send side and receive side.

NOTE – If an impairment factor Ie is used for a piece of equipment, then the qdu value for that same piece of equipment must not be used.

## 3.4	Delay impairment factor, Id

Also Id, the impairment factor representing all impairments due to delay of voice signals is further divided into the three factors Idte, Idle and Idd:

(3-18)

The factor Idte gives an estimate for the impairments due to talker echo:

(3-19)

where:

(3-20)

(3-21)

(3-22)

For values of T  1 ms, the talker echo should be considered as sidetone, i.e., Idte  0. The computation algorithm furthermore combines the influence of STMR to talker echo. Taking into account that low values of STMR may have some masking effects on the talker echo and, for very high values of STMR, the talker echo may become more noticeable, the terms TERV and Idte are adjusted as follows:

For STMR  9 dB:

In equation 3-21, TERV is replaced by TERVs, where:

(3-23)

For 9 dB  STMR  20 dB:

the above-given equations 3-19 to 3-22 apply.

For STMR  20 dB:

In equation 3-18, Idte is replaced by Idtes, where:

(3-24)

The factor Idle represents impairments due to listener echo. The equations are:

(3-25)

where:

(3-26)

The factor Idd represents the impairment caused by too-long absolute delay Ta, which occurs even with perfect echo cancelling.

For Ta  100 ms:

For Ta  100 ms:

(3-27)

with:

(3-28)

## 3.5	Equipment impairment factor, Ie

The values for the equipment impairment factor Ie of elements using low bit-rate codecs are not related to other input parameters. They depend on subjective mean opinion score test results as well as on network experience. Refer to Appendix I of [ITU-T G.113] for the currently recommended values of Ie.

Specific impairment factor values for codec operation under random packet-loss have formerly been treated using tabulated, packet-loss dependent Ie-values. Now, the packet-loss robustness Factor Bpl is defined as a codec-specific value. The packet-loss dependent effective equipment impairment factor Ie-eff is derived using the codec-specific value for the equipment impairment factor at zero packet-loss Ie and the packet-loss robustness factor Bpl, both listed in Appendix I of [ITU-T G.113] for several codecs. With the packet-loss probability Ppl, Ie-eff is calculated using the equation:

(3-29)

BurstR is the so-called burst ratio, which is defined as:

When packet loss is random (i.e., independent) BurstR = 1; and

when packet loss is bursty (i.e., dependent) BurstR > 1.

For example, for packet loss distributions corresponding to a 2-state Markov model with transition probabilities p between a "found" and a "loss" state, and q between the "loss" and the "found" state, the burst ratio can be calculated as:

(3-30)

As can be seen from equation 3-29, the effective equipment impairment factor in case of Ppl = 0 (no packet-loss) is equal to the Ie value defined in Appendix I of [ITU-T G.113].

Please refer to Annex A for the range of parameter values the algorithm has been validated for.

## 3.6	Advantage factor, A

Due to the specific meaning of the advantage factor A there is, consequently, no relation to all other transmission parameters. Some provisional values are given in Table 1.

Table 1 – Provisional examples for the advantage factor A

| Communication system example | Maximum value of A |
| --- | --- |
| Conventional (wirebound) | 0 |
| Mobility by cellular networks in a building | 5 |
| Mobility in a geographical area or moving in a vehicle | 10 |
| Access to hard-to-reach locations, e.g., via multi-hop satellite connections | 20 |

It should be noted that the values in Table 1 are only provisional. The use of the factor A and its selected value in a specific application is up to the planner's decision. However, the values in Table 1 should be considered as absolute upper limits for A. Additional background information on the advantage factor A can be found in Appendix II to [ITU-T G.113].

## 3.7	Default values

For all input parameters used in the algorithm of the E-model, the default values are listed in Table 2. It is strongly recommended to use these default values for all parameters which are not varied during planning calculation. If all parameters are set to the default values, the calculation results in a very high quality with a rating factor of R  93.2.

Table 2 – Default values and permitted ranges for the parameters

| Parameter | Abbr. | Unit | Default value | Permitted range | Remark |
| --- | --- | --- | --- | --- | --- |
| Send loudness rating | SLR | dB | +8 | 0 ... +18 | (Note 1) |
| Receive loudness rating | RLR | dB | +2 | 5 ... +14 | (Note 1) |
| Sidetone masking rating | STMR | dB | 15 | 10 ... 20 | (Notes 2, 4) |
| Listener sidetone rating | LSTR | dB | 18 | 13 ... 23 | (Note 2) |
| D-Value of telephone, send side | Ds | – | 3 | –3 ... +3 | (Note 2) |
| D-Value of telephone, receive side | Dr | – | 3 | 3 ... +3 | (Note 2) |
| Talker echo loudness rating | TELR | dB | 65 | 5 ... 65 |  |
| Weighted echo path loss | WEPL | dB | 110 | 5 ... 110 |  |
| Mean one-way delay of the echo path | T | ms | 0 | 0 ... 500 |  |
| Round-trip delay in a 4-wire loop | Tr | ms | 0 | 0 ... 1000 |  |
| Absolute delay in echo-free connections | Ta | ms | 0 | 0 ... 500 |  |
| Number of quantization distortion units | qdu | – | 1 | 1 ... 14 |  |
| Equipment impairment factor | Ie | – | 0 | 0 ... 40 | (Note 5) |
| Packet-loss robustness factor | Bpl | – | 4.3 | 4.3 ... 40 | (Notes 3, 5) |
| Random packet-loss probability | Ppl | % | 0 | 0 ... 20 | (Notes 3, 5) |
| Burst ratio | BurstR | – | 1 | 1 … 8 | (Notes 3, 6) |
| Circuit noise referred to 0 dBr-point | Nc | dBm0p | 70 | 80 ... 40 |  |
| Noise floor at the receive side | Nfor | dBmp | 64 | – | (Note 3) |
| Room noise at the send side | Ps | dB(A) | 35 | 35 ... 85 |  |
| Room noise at the receive side | Pr | dB(A) | 35 | 35 ... 85 |  |
| Advantage factor | A | – | 0 | 0 ... 20 |  |
| NOTE 1 – Total values between microphone or receiver and 0 dBr-point. NOTE 2 – Fixed relation: LSTR = STMR + D. NOTE 3 – Currently under study. NOTE 4 – Equation 3-24 provides also predictions for STMR > 20 dB. However, such values can hardly be measured in a reliable way because the measurement device will mainly cover the acoustic coupling, and not the electrical one. NOTE 5 – If Ppl > 0%, then the Bpl must match the codec, packet size, and PLC assumed. NOTE 6 – E-model predictions for values of BurstR > 2 are only valid if the packet loss percentage is Ppl < 2%. |  |  |  |  |  |

The 2000 version of this Recommendation provided an enhanced version of the E-model algorithm (see Annex A).

Due to the changes made in the 2000 version of this Recommendation, the resulting rating R with all parameter values set to default has slightly changed (from R = 94.2 to R = 93.2). For practical planning purposes, however, this slight deviation should be considered insignificant.

Annex A

Conditions of using the E-model

(This annex forms an integral part of this Recommendation)

NOTE  The assessment and enhancement of the E-model algorithm are for further study. New results will be included as soon as they become available.

## A.1	Examples of conditions where caution must be exercised when using the E-model

The overall level of the equipment impairment factors

Some experimental investigations suggest that the general tendency of the equipment impairment factors is too pessimistic, so that a hidden security margin may be incorporated.

The overall additivity property of the model

The E-model supposes that different kinds of impairments are additive on the scale of the transmission rating factor R. This feature has not been checked to a satisfactory extent. Especially, very few investigations are available regarding the interaction of low bit-rate codecs with other kinds of impairments, e.g., with room noise. Additionally, the order effects when tandeming several low bit-rate codecs remain uncertain.

The coverage of talker sidetone

Some experiments show that the E-model disregards some masking effects occurring for talker sidetone, namely in conjunction with circuit noise, room noise at receive side, and low delay talker echo (< 10 ms).

The advantage factor A

Up to now it has not been clarified under which conditions the given values for the advantage factor should be applied. It is expected that these values may depend, e.g., on the user group, and that the absolute values will change in the long term.

Derivation methodology for new equipment impairment factors

A new methodology for deriving equipment impairment factors from subjective listening quality tests has been adopted as [ITU-T P.833]. A new methodology for deriving equipment impairment factors from instrumental models such as [ITU-T P.862] has been adopted as [ITU-T P.834].

Predictions for different types of room noise, and different frequency shapes in the communication channel, in the sidetone path and in the echo path

The E-model regards the effect of room noise only by means of an A-weighted level. The actual opinion on the speech communication quality may depend even on the type and disturbance of the environmental noise. The frequency characteristics of the communication channel, of the sidetone and of the echo path are not explicitly regarded by the E-model, but only implicitly by means of loudness ratings. However, they may affect the perceived transmission quality.

## A.2	Conditions for which the performance of the E-model has been improved by updating from the earlier version

The effect of room noise at send side

With the present enhanced E-model algorithm (2000 version), the Lombard effect (the fact that the speaker adopts his/her pronunciation and speaking level according to the noise environment) is no longer disregarded. This has, in the 1998 version, led to too pessimistic E-model predictions for high room noise levels Pr.

Predictions for quantizing distortion

In case of the 1998 version of the E-model, subjective test results for MNRU reference conditions were very often more pessimistic than E-model predictions. The graphs in Figure A.1 have been derived from the 1998 version and the 2000 version of the E-model with all other parameters at their default values.

Figure A.1  Relation between the number of qdu and E-model rating R

With respect to the slightly enhanced algorithm of the E-model, as given in this Recommendation, the relation between the parameter qdu and the E-model rating R has been changed in order to align the algorithm better with the available subjective test results.

Predictions for codec performance under random packet loss

Impairments due to codecs under packet-loss conditions were formerly handled by using codec-dependent tabulated equipment impairment factors for different packet-loss rates (in former versions of Appendix I of [ITU-T G.113]). As the aim is to reduce the amount of tabulated data for usage with the E-model, possibilities of replacing tabulated Ies for packet loss by corresponding equations were investigated. The chosen approach leads to results very similar to those previously defined as Ie for all codecs covered in the 2001 version of Appendix I of [ITU-T G.113].

Predictions for codec performance under dependent packet loss

With this version of the algorithm, loss distributions characterized by medium (short-term) loss dependencies (as opposed to long-term loss dependencies) have been integrated in the E-model. Up to now, the included approach has been evaluated only for the ITU-T G.729(A) codec, but is assumed to be applicable also to the ITU-T G.723.1 codec, and supposedly other codecs. Pending further verification, the algorithm should not be used with burst ratios higher than BurstR = 2.0. The model can also be applied to higher burst ratios than 2.0, if packet loss percentages Ppl are lower than 2%.

–	The effect of talker sidetone

Estimates of voice quality as a function of STMR for values > 15 dB, as provided by the 2002 version of this Recommendation, were too pessimistic and did not accurately match the results obtained in auditory tests. This proved to be especially important for telephones in North America that are typically specified to have nominal values of STMR from 16 to 18 dB.

In the current revised version of the E-model algorithm, this observation is reflected by modifying the corresponding equation for Ist as a function of sidetone (STMR), see equation 3-11.

As mentioned in the main body of this Recommendation, talker echo may become more noticeable for quiet values of STMR. This is addressed by switching from Idte to Idtes, see equation 3-24. To remain consistent, the talker echo threshold from STMR > 15 dB (2002 version of this Recommendation) was extended to STMR > 20 dB (the current version of this Recommendation). The modifications have no impact for values of STMR < 15 dB. Consequently, the quality prediction for the transmission rating factor R for the default settings (STMR = 15 dB) does not differ from that predicted by the previous model version (2002). The default value of R is 93.2 for both the previous and the current versions. The situation is depicted in Figure A.2.

Figure A.2  Comparison of R versus STMR for the current and the previous
versions of the E-model algorithm

Annex B

Quality measures derived from the transmission rating factor R

(This annex forms an integral part of this Recommendation)

The transmission rating factor R can lie in the range from 0 to 100, where R  0 represents an extremely bad quality and R  100 represents a very high quality. The E-model provides a statistical estimation of quality measures. The percentages for a judgement good or better (GoB) or poor or worse (PoW), are obtained from the R-factor by means of the Gaussian error function:

(B-1)

The equations are:

(B-2)

(B-3)

An estimated mean opinion score (MOSCQE) for the conversational situation in the scale 1-5 can be obtained from the R-factor using the equations:

For R  0:

For 0  R  100:		(B-4)

For R  100:

This equation can be inverted in the range  to calculate R from MOSCQE, see Appendix I. GoB, PoW and MOSCQE as functions of R are depicted in Figures B.1 and B.2, respectively.

Figure B.1 – Good or better (GoB) and poor or worse (PoW) 
as functions of rating factor R

Figure B.2 – MOSCQE as function of rating factor R

In some cases, transmission planners may not be familiar with the use of quality measures such as the R rating factor obtained from planning calculations, and thus provisional guidance for interpreting calculated R factors for planning purposes is given in Table B.1. This table also contains equivalent transformed values of R into estimated conversational MOSCQE, GoB and PoW.

Table B.1 – Provisional guide for the relation between R-value and user satisfaction

| R-value (lower limit) | MOSCQE (lower limit) | GoB (%) (lower limit) | PoW (%) (upper limit) | User satisfaction |
| --- | --- | --- | --- | --- |
| 90 | 4.34 | 97 | 0 | Very satisfied |
| 80 | 4.03 | 89 | 0 | Satisfied |
| 70 | 3.60 | 73 | 6 | Some users dissatisfied |
| 60 | 3.10 | 50 | 17 | Many users dissatisfied |
| 50 | 2.58 | 27 | 38 | Nearly all users dissatisfied |

Appendix I

Calculation of R from MOSCQE values

(This appendix does not form an integral part of this Recommendation)

In the range , R can be calculated from MOSCQE using the formula:

(I-1)

with:

(I-2)

and:

(I-3)

The function arctan2 (x, y) is implemented in ANSI C as the function atan2 (y, x). Users should note that the order of the two parameters differs in this case.

Appendix II

Provisional impairment factor framework for wideband speech transmission

(This appendix does not form an integral part of this Recommendation)

The E-model is based on the "impairment factor principle". According to this principle, it is possible to transform all relevant degradations to a "transmission rating scale" (R-scale) which represents the overall quality in a bidirectional communication situation, taking into account the entire channel mouth-to-ear. For the narrow-band (NB) case, the transmission rating scale ranges from R = 0 (lowest possible quality) to R = 100 (optimum quality). On this scale, a default NB transmission channel including logarithmic PCM coding and a noise floor (default parameter values according to Table 2) obtains a rating of R = 93.2. For a wideband speech transmission channel, the quality is generally judged better than for a narrow-band channel. Thus, this scale range has to be extended in order to be also applicable for wideband transmission scenarios. In what follows, an extension is defined for a wideband transmission channel of 50-7000 Hz (WB), as it is defined in [b-ITU-T G.722].

Unfortunately, it is not possible to obtain direct human judgements on the R-scale, as this scale has additivity properties which are not reflected by ordinary rating scales. Instead, for NB conditions, ITU-T recommends collecting judgements on a 5-point absolute category rating scale, see [b-ITU-T P.800]. The mean rating, averaged over all test participants and stimuli reflecting the same circuit condition, is then called a mean opinion score (MOS).

It has been shown that MOS ratings differ between tests where only NB stimuli are presented, and tests where both NB/WB or purely WB stimuli are presented, as the use of the scale is largely influenced by the stimulus set. On the other hand, there is also experimental evidence that judgements for WB samples collected in a purely WB context do not differ significantly from those collected in a mixed NB/WB context [b-Barriac] and [b-Takahashi]. In addition to the stimulus bandwidth, test results are influenced by the test participant group, the language, the participants' native country, etc., [b-Möller-01]. For a NB context, an average S-shaped relationship is defined between the R-scale (range [0;100]) and MOS ratings (range [1;4.5]) collected from "average" test participants in an "average" experimental setting, see Annex B and Appendix I.

For a WB or a mixed NB/WB context, the R-scale has to be extended in a way which leaves the NB use of the scale unaffected, including the position of the reference connection (default parameter settings according to Table 2). Such an extension can be based on pairs of auditory tests in which the same (NB) test stimuli have been judged once in a purely NB and once in a mixed NB/WB context. The judgements on these common stimuli define a relationship between the use of the MOS-scale in a NB and in a mixed NB/WB context [b-Raake].

Two pairs of tests have been carried out and will be considered in the following. Details on parts of the tests can be found in [b-Möller-02]. The MOS results from these tests have been transformed to the R-scale, using the NB transformation rule given in Annex B. The resulting RNB values (NB test) and RNB/WB values (mixed NB/WB test) for the conditions which were common in each pair are displayed in Figure II.1.

Figure II.1 – Comparison between R-values derived in a NB 
and in a mixed NB/WB context

Due to the use of the NB relationship between MOS and R for deriving the RNB/WB values, the maximum RNB/WB value corresponding to MOS = 4.5 equals 100. The corresponding RNB value of the panels in Figure II.1 shows the amount by which the R-scale has to be extended in a NB/WB context. This maximum value is around Rmax = 129. In other words, the NB transmission rating scale of the E-model has to be extended by approximately 29% to reflect the quality improvement when migrating from NB to WB. This extended R-scale is a "universal" R-scale; it is applicable to both NB and WB transmission channels.

Assuming a maximum value of Rmax = 129 of the extended R-scale, WB equipment impairment factors Ie,wb can be calculated for different WB speech codecs. For this purpose, MOS results from auditory tests have to be transformed to the extended R-scale; the Ie,wb value is then defined as the difference between the R-value corresponding to the respective codec and the R-value corresponding to the "direct" channel:

(II-1)

In the NB case, a "direct" channel is usually associated with a standard ISDN connection, including a channel limitation defined in [b-ITU-T G.712], a logarithmic PCM codec according to [ITU-T G.711], a passband frequency shape corresponding to an intermediate reference system defined in [b-ITU-T P.48], and all other transmission parameters set to their default values given in Table 2; this "direct" NB channel results in R = 93.2. In the WB case, no similar standard channel is yet defined. Here, it is assumed that the "direct" WB channel has a channel frequency response as defined in [b-ITU-T G.722], and involves only linear PCM (16-bit quantization, sampling frequency fs = 16 kHz). This "direct" WB channel should result in R = 129.

On the basis of seven tests which are partially described in [b-Möller-02], provisional Ie,wb values have been derived for a number of narrow-band and wideband codecs, operating at different bit rates. These values can be found in Appendix IV of [ITU-T G.113].

In order to include the quality impact of packet loss on quality of wideband-coded speech, first preliminary results are available. Based on the Wideband Impairment Factor framework laid out in this Appendix II, it was found that Equation (3-29) can be used for wideband speech codecs under uniform packet loss, too, writing:

(II-2)

Corresponding values for Bpl,wb can be found in Appendix IV of [ITU-T G.113].

Appendix III

Reference Implementation of the E-model in ITU-T G.107

(This appendix does not form an integral part of this Recommendation)

The reference implementation of the E-model is no longer provided in the traditional qbasic listing (which was provided in Annex C of the 2005 version of this Recommendation and all previous versions), since it was no longer found to be appropriate. Therefore, this Recommendation provides a new implementation of the E-model in PHP4 scripting language.

The associated electronic attachment (which is available free of charge on the ITU publications website at http://www.itu.int/rec/T-REC-G.107) contains the following files in the Software folder:

–	General instructions: readme.txt

–	PHP4 script: calc.php

–	3 required Java scripts:

•	wz_tooltip.js

•	tip_centerwindow.js

•	tip_followscroll.js

–	Copyright notice

In order to run this software on any server outside ITU, one will need to obtain a license from the organization indicated in the software Copyright notice.

However, the very same PHP4 script has been implemented on the ITU website where everyone has the right to use that software from that web server under their own responsibility at their own risk without limitation, subject to no particular conditions, no royalties are due, etc. No software copyright license agreements are required for such usage. That tool is available here:

http://www.itu.int/ITU-T/studygroups/com12/emodelv1/

Bibliography

[b-ITU-T G.711]	Recommendation ITU-T G.711 (1988), Pulse code modulation (PCM) of voice frequencies.
<http://web.itu.int/rec/T-REC-G.711>

[b-ITU-T G.712]	Recommendation ITU-T G.712 (2001), Transmission performance characteristics of pulse code modulation channels.
<http://web.itu.int/rec/T-REC-G.712>

[b-ITU-T G.722]	Recommendation ITU-T G.722 (1988), 7 kHz audio-coding within 64 kbit/s.
<http://web.itu.int/rec/T-REC-G.722>

[b-ITU-T G.723.1]	Recommendation ITU-T G.723.1 (1996), Dual rate speech coder for multimedia communications transmitting at 5.3 and 6.3 kbit/s.
<http://web.itu.int/rec/T-REC-G.723.1>

[b-ITU-T G.729]	Recommendation ITU-T G.729 (2007), Coding of speech at 8 kbit/s using conjugate-structure algebraic-code-excited linear prediction (CS-ACELP).
<http://web.itu.int/rec/T-REC-G.729>

[b-ITU-T P.48]	Recommendation ITU-T P.48 (1988), Specification for an intermediate reference system.
<http://web.itu.int/rec/T-REC-P.48>

[b-ITU-T P.800]	Recommendation ITU-T P.800 (1996), Methods for subjective determination of transmission quality.
<http://web.itu.int/rec/T-REC-P.800>

[b-ITU-T P-Sup.3]	ITU-T P-series Recommendations – Supplement 3 (1993), Models for predicting transmission quality from objective measurements.
<http://www.itu.int/rec/T-REC-P.Sup3>

[b-Barriac]	Barriac, V., Le Saout, J.-Y., and Lockwood C. (2004), Discussion on Unified Methodologies for the Comparison of Voice Quality of Narrowband and Wideband Scenarios, ETSI Workshop on Wideband Speech Quality in Terminals and Networks: Assessment and Prediction, pp. 75-79.

[b-Möller-01]	Möller, S. (2000), Assessment and Prediction of Speech Quality in Telecommunications, Springer.

[b-Möller-02]		Möller, S., Raake, A., Kitawaki, N., Takahashi, A., Wältermann, M. (2006). Impairment Factor Framework for Wideband Speech Codecs, IEEE Trans. Audio, Speech and Language Processing 14(6), pp. 1969-1976.

[b-Raake]	RAAKE, A. (2006), Speech Quality of VoIP – Assessment and Prediction, Chichester, UK, Wiley.

[b-Takahashi]		Takahashi, A., Kurashima, A., and Yoshino, H. (2005), Subjective Quality Index for Compatibly Evaluating Narrowband and Wideband Speech, Prague, Czech Republic, MESAQIN.

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | Telecommunication management, including TMN and network maintenance |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Terminals and subjective and objective assessment methods |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks, open system communications and security |
| Series Y | Global information infrastructure, Internet protocol aspects and next-generation networks |
| Series Z | Languages and general software aspects for telecommunication systems |

____________________

The online version of this Recommendation includes an electronic attachment containing a reference implementation of the E-Model in PHP4 scripting language. This electronic attachment is available free of charge on the ITU publications website at http://www.itu.int/rec/T-REC-G.107.

Conversational quality in this context refers to transmission characteristics, e.g., long transmission times, effects of talker echoes, etc. However, the E-model as described in this Recommendation is not intended to model transmission impairments during double talk situations.

The probability of losing a packet is regarded as independent on the reception state (received/lost) of the previous packet.

The source of Table B.1 is Table 1 of [ITU-T G.109].