|  |  | International Telecommunication Union |  |  |
| --- | --- | --- | --- | --- |
| ITU-T | G.720.1 |  |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (01/2010) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Coding of voice and audio signals |  |  |  |
|  | Generic sound activity detector |  |  |  |
|  | Recommendation ITU-T G.720.1 |  |  |  |

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TRANSMISSION MEDIA AND OPTICAL SYSTEMS CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of voice and audio signals | G.710–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| MULTIMEDIA QUALITY OF SERVICE AND PERFORMANCE – GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DATA OVER TRANSPORT – GENERIC ASPECTS | G.7000–G.7999 |
| PACKET OVER TRANSPORT ASPECTS | G.8000–G.8999 |
| ACCESS NETWORKS | G.9000–G.9999 |

For further details, please refer to the list of ITU-T Recommendations.

Recommendation ITU-T G.720.1 Generic sound activity detector

Summary Recommendation ITU-T G.720.1 describes an independent front-end processing module implementing a generic sound activity detector (GSAD) that can be applied prior to signal processing applications and can operate on narrow-band or wideband audio input using a 10-ms frame length (without lookahead), such as used by speech or audio codecs. The primary function of the GSAD is to indicate the input frame activity for performing voice activity detection (VAD). For an active frame, it further indicates if the input frame is speech or music (speech/music discrimination), and for an inactive frame it indicates whether the frame is a silence frame or an audible noise frame (silence detection). The GSAD can also operate when only the primary function of indicating the input frame activity is used. In order to apply GSAD in specific cases, an adaptation layer may be required. An external control signal indicates to the GSAD algorithm which one of the three different operating points to use, namely: bandwidth-saving, balanced and quality-preferred operating points. For the activity detection functionality, these operating points provide selectable balancing between bandwidth saving and audio quality, which can be utilized for high-performance silence compression schemes that can balance between the end-user's speech and audio subjective quality needs and the system and network traffic requirements. The three different operating points also control the GSAD emphasis and balance between speech and music classification for the active frames, which can be utilized for fine-tuning of source-controlled audio compression systems. The VAD module uses a dual-parameters classification scheme, where one parameter is a differential zero crossing rate measure and the other parameter is a modified segmental: signal to noise ratio (SNR) measure. An initial VAD decision is made with a pair of inequalities, with factors that are adaptive to the long term SNR of the input signal. A final VAD decision is obtained by an adaptive hangover scheme. The speech/music discrimination module calculates the variance of a spectral deviation measure and applies an adaptive threshold to make an initial decision between speech and music. Two spectral peakiness measures further modify that initial decision and a one-frame hangover is used to obtain the final speech/music discrimination decision. The silence detection module uses an energy threshold to discriminate between a silence frame and an audible noise frame. The main body of this Recommendation provides a detailed description of the overall GSAD configuration, including the operating points; the VAD module; the speech/music discrimination module and the silence detection module. Annex A describes a standalone generic voice activity detector (GVAD) that can be applied prior to signal processing applications and can operate on narrow-band or wideband audio input using 10 ms frame length (without lookahead), such as used by speech or audio codecs. Its function is to indicate the input frame activity. In order to apply GVAD in specific cases, an adaptation layer may be required. The Recommendation also contains an electronic attachment with the ANSI C source code which forms an integral part of this Recommendation, and a set of test vectors. The set of test vectors is also available for download from the ITU-T Test Signal Database at: http://www.itu.int/net/ITU-T/sigdb/speaudio/Gseries.htm#G.720.1.

History Edition; Recommendation; Approval; Study Group. 1.0; ITU-T G.720.1; 2010-01-13; 16.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications, information and communication technologies (ICTs). The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g., interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementers are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database at http://www.itu.int/ITU-T/ipr/.

ITU  2011

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

Table of Contents

Page

1	Scope		1

2	References		1

3	Definitions		1

3.1	Terms defined elsewhere		1

3.2	Terms defined in this Recommendation		1

4	Abbreviations and acronyms		2

4.1	Glossary of acronyms		2

4.2	Glossary of symbols		2

5	Conventions		5

6	General description of the GSAD algorithm		5

6.1	Input sampling rate		5

6.2	Operating frame size		5

6.3	Delay		5

6.4	Configurations		5

6.5	Complexity and memory cost		6

7	Detailed description of the GSAD algorithm		6

7.1	Detailed description of the VAD module		7

7.2	Detailed description of the speech/music discrimination module		18

8	Organization of the reference C code		22

Annex A – Generic voice activity detector		23

A.1	Scope		23

A.2	References		23

A.3	Definitions		23

A.4	Abbreviations and acronyms		23

A.5	Conventions		23

A.6	General description of the GVAD algorithm		24

A.7	Detailed description of the GVAD algorithm		24

A.8	Use of the simulation software		25

Bibliography		26

Recommendation ITU-T G.720.1

Generic sound activity detector

# 1	Scope

The generic sound activity detector (GSAD) is an independent front-end processing module which can be applied prior to signal processing applications that operate on narrow-band or wideband audio input at 10-ms frame length (without lookahead), such as speech or audio codecs. Its primary function is to indicate the input frame activity. For an active frame it further indicates if the input frame is speech or music, and for an inactive frame it indicates whether the frame is a silence frame or an audible noise frame. In order to apply GSAD in specific cases, an adaptation layer may be required.

This Recommendation is organized as follows. References, definitions, abbreviations/acronyms and conventions are defined in clauses 2, 3, 4 and 5 respectively. Clause 6 gives a general description of the GSAD algorithm including the input sampling rate, the operating frame length, the algorithmic delay, the configurations and the complexity and memory cost. The detailed description of the GSAD algorithm is described in clause 7, where clause 7.1 describes the VAD module and the speech/music discrimination module is described in clause 7.2. Finally, in clause 8, the organization of the ANSI C code is described.

# 2	References

This Recommendation does not make normative reference to any other standards.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation does not use terms defined elsewhere.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	background signal: The interfering signals (e.g., background noise) at the background of the dominating speech or music signals.

3.2.2	balanced operating point: Generic sound activity detector (GSAD) operating point that provides a middle point performance between the other two operating points in either voice activity detection or speech/music discrimination. See clause 6.4.

3.2.3	bandwidth-saving operating point: Generic sound activity detector (GSAD) operating point privileging accurate detection for background signals in voice activity detection, or privileging accurate detection for speech signals in speech/music discrimination. See clause 6.4.

3.2.4	foreground signal: The dominating information signal (speech or music) in the speech channel.

3.2.5	quality-preferred operating point: Generic sound activity detector (GSAD) operating point privileging accurate detection for foreground signals in voice activity detection, or privileging accurate detection for music signals in speech/music discrimination. See clause 6.4.

3.2.6	silence detector: A function or device which identifies frames whose levels are below a silence threshold.

3.2.7	speech/music discriminator: A function or device which classifies audio input into either speech or music.

# 4	Abbreviations and acronyms

This Recommendation uses the abbreviations and acronyms defined in clause 4.1 and the symbols used throughout this Recommendation are listed in clause 4.2.

## 4.1	Glossary of acronyms

DZCR			Differential Zero Crossing Rate

FFT			Fast Fourier Transform

GSAD			Generic Sound Activity Detection/Detector

GSAD_NB		Generic Sound Activity Detecting mode for Narrow-band signals

GSAD_WB		Generic Sound Activity Detecting mode for Wideband signals

GVAD			Generic Voice Activity Detection/Detector

GVAD_NB		GVAD mode for Narrow-band signals

GVAD_WB	GVAD mode for Wideband signals

MSSNR		Modified Segmental SNR

NB			Narrow-band

RMS			Root Mean Square

SDF			Spectral Density Function

SiD			Silence Detector

SMD			Speech/Music Discriminator

SNR			Signal to Noise Ratio

VAD			Voice Activity Detection/Detector

VAD_NB		Voice Activity Detecting mode for Narrow-band signals

VAD_WB		Voice Activity Detecting mode for Wideband signals

WB			Wideband

WMOPS		Weighted Million Operations Per Second

ZCR			Zero Crossing Rate

## 4.2	Glossary of symbols

|  | An offset factor for |
| --- | --- |
|  | The moving average ofat the m-th frame |
|  | The moving average ofat the m-th frame |
| bgd_frm_cnt | The counter counting the number of background frames in the background music detection |
| con_frm_cnt | The counter counting the number of consecutive frames in the background estimate update procedure |
|  | A constant that depends on the operating point, used to calculate |
| Dp2s(i) | The power distance between the i-th local spectral peak and its adjacent four spectral bins on its two sides |
| Dp2v(j) | The normalized peak to valley distance of the j-th local spectral peak |
|  | An initial difference measure between and |
|  | An average measure of |
|  | A final difference measure between and |
| dtmf_flg | The flag indicating the presence of a DTMF signal |
| DZCR | The differential zero crossing rate of the input frame |
| Eband(i) | The energy of the i-th sub-band |
| Eband_buf_min(i) | The minimum energy of the i-th sub-band of the past frames |
| Eband_old(i) | The energy of the i-th sub-band of the previous frame |
|  | The moving average of the energy of the i-th sub-band of estimated background signal |
|  | The moving average of the power of the i-th sub-band over past frames |
|  | The power mean of the 16 whitened sub-bands in the calculation of frequency stability |
| Evh(j) | The higher frequency spectral valley of the j-th local spectral peak |
| Evl(j) | The lower frequency spectral valley of the j-th local spectral peak |
| fluxbgd | The fluctuation of the background signal |
|  | A spectral deviation measure for the m-th frame |
| FPdBov | The signal power in dBov |
| hang_b_mus | The hangover counter for background music |
| hang_f_mus | The hangover counter for foreground music |
| hang_sp | The hangover counter for speech |
|  | A counter on the MSSNR high values |
| high_fst_cnt | The counter counting the number of frames with high frequency-stability |
| idx_peakloc(i) | The position of the i-th local spectral peak in the spectrum |
| idx_peakmax(j) | The location of the local spectral peak which has the j-th largest Dp2s in the spectrum |
| idx_peakmax_old | The location of the local spectral peak which has the maximum Dp2s in the previous frame |
|  | The location of in its spectrum |
| ivad | The initial VAD decision |
|  | A counter on the MSSNR low values |
| low_SP_cnt | The counter counting the number of frames with low spectral peakiness |
| lsnr | The long term SNR of the input frame |
|  | A maximal value of MSSNR over past frames |
|  | The moving average of at the m-th frame |
| msnr(i) | The modified SNR of the i-th sub-band |
| MSSNR | The modified segmental SNR |
|  | A first peakiness measure |
|  | A second peakiness measure |
| peakloc(i) | The power of the i-th local spectral peak |
|  | The largest local spectral peak of the m-th input frame |
| peak_flux_cnt | The counter counting the spectral peak fluctuation |
| reset_flg | The flag indicating whether to reset counters used in the background estimate update procedure |
| rms | The RMS of the input frame |
|  | The long term RMS of the background signal of the m-th frame |
|  | The long term RMS of the foreground signal of the m-th frame |
| s(i) | The i-th sample in the input frame |
| snr(i) | The SNR of the i-th sub-band of the input frame |
| Semp(i) | The i-th pre-emphasized sample |
|  | The k-th spectral bin of the m-th frame |
| SP | The spectral peakiness of the input frame |
| SPsum | The spectral peakiness accumulator storing the sum of the frame peakinesses in the background music detection |
| stafq | The frequency stability of the input frame |
|  | Adaptive threshold on |
|  | An adaptive threshold of the MSSNR |
|  | Lowest desired value for |
|  | Highest desired value for |
| thrbgd | The threshold for background frame identification |
| thrfst | The threshold for high frequency-stability identification |
| thrSP_low | The threshold for low spectral peakiness identification |
| thrvad | The threshold for the initial VAD decision |
| tone_sta_cnt | The counter counting the number of frames with high tone stability |
| update_cnt | A counter for counting consecutive background frames |
| update_flg | The flag indicating whether to update the background estimate |
|  | The variance of at the m-th frame |
|  | A threshold on |
| ZCR | The zero crossing rate of the input frame |
|  | The moving average of the ZCR of the estimated background signal |

# 5	Conventions

The following conventions apply to this Recommendation:

–	|x| denotes the absolute value of x; e.g., |12| = 12, |–3| = 3

–	sgn[x] denotes the sign of x; e.g., sgn[3] = 1, sgn[–5] = –1

–	MAX[x0, x1, x2, ... xN–1] denotes the maximum of x0, x1, x2, ... xN–1; e.g., MAX[0, –1, 3, 5, 2] = 5

–	MIN[x0, x1, x2, ... xN–1] denotes the minimum of x0, x1, x2, ... xN–1; e.g., MIN[0, –1, 3, 5, 2] = –1

–	log(x) denotes the logarithm in base 10 of x; e.g., log(100) = 2

–	p[m] denotes the value of parameter p at the m-th input frame. Current frame is assumed when [m] is omitted

–	Σ denotes summation

–	o.w. denotes "otherwise"

# 6	General description of the GSAD algorithm

## 6.1	Input sampling rate

The GSAD can accept both narrow-band (NB) and wideband (WB) audio input signals sampled at 8 and 16 kHz, respectively. The sampling rate of the input signal is provided to the GSAD by an external signal.

## 6.2	Operating frame size

GSAD operates on a 10-ms basis, i.e., frames of 80 samples for NB audio input and of 160 samples for WB audio input. Consecutive GSAD indications can be combined to indicate the activity for frames with a multiple of 10 ms.

## 6.3	Delay

GSAD does not introduce lookahead, therefore the added delay of the GSAD is 0 ms (algorithmic delay of 10 ms).

## 6.4	Configurations

GSAD operates on narrow-band (NB) and wideband (WB) audio input signals, where a control signal indicates the bandwidth of the input signal. For both bandwidths, a second control signal sets GSAD to operate in a generic sound activity detecting mode (GSAD_NB and GSAD_WB) or in a voice activity detecting mode (VAD_NB and VAD_WB). When GSAD operates in a generic sound activity detecting mode, the input signal first passes through the voice activity detector. Frames that are detected as active signal frames are further differentiated as either speech frames or music frames. Frames that are detected as inactive signals are further classified as either audible noise frames or silence frames.

Table 1 describes the GSAD output bits.

Table 1 – Description of the GSAD output bits

| VAD decision | GSAD decision | Main output |  |
| --- | --- | --- | --- |
|  |  | First bit | Second bit |
| Active | Speech | 1 | 1 |
|  | Music | 1 | 0 |
| Inactive | Noise | 0 | 1 |
|  | Silence | 0 | 0 |

When GSAD operates in a generic sound activity detecting mode, both bits are output, which can be interpreted as four possible values indicating a speech ("3"), music ("2"), audible noise ("1") or silence frame ("0"). When the GSAD operates in a voice activity detection mode, only the first bit is outputted, which can be interpreted as "1" or "0" indicating an active or inactive frame, respectively.

In addition, a third control signal selects the desired operating point from three possible ones. Setting the third control signal to "0" selects the balanced operating point, setting it to "1" selects the quality-preferred operating point and setting the third control signal to "2" selects the bandwidth-saving operating point. When GSAD operates in a voice activity detection mode, the three operating points provide the ability to select a preferred balance between the bandwidth saving and the audio quality. The bandwidth-saving operating point provides maximal bandwidth saving while maintaining acceptable audio quality, while the quality-preferred operating point provides higher audio quality with less bandwidth saving. The balanced operating point provides an optimum performance which is between the bandwidth-saving operating point and the quality-preferred operating point. For example, for the speech database used in the GSAD selection phase with added car, office and babble background noises, the bandwidth-saving operating point saves approximately 30% of the bandwidth compared to the balanced operating point, and approximately, 50% of the bandwidth compared to the quality-preferred operating point. When the GSAD operates in a generic sound activity detecting mode, the operating points also control the operation of the speech/music discrimination module. The bandwidth-saving operating point is tuned to correctly classify most of the active speech frames, and the quality-preferred operating point is tuned to correctly classify most of the active music frames. The balanced operating point provides a middle point performance between the other two operating points.

## 6.5	Complexity and memory cost

Table 2 shows the GSAD complexity in WMOPS for its different modes and signal sampling frequencies. The RAM used for GSAD is 3284 bytes and the table ROM is 1674 bytes.

Table 2 – Complexity of the GSAD

| Modes | Complexity (WMOPS) |
| --- | --- |
| GSAD_WB | 2.935 |
| GSAD_NB | 1.897 |
| VAD_WB | 2.397 |
| VAD_NB | 1.475 |

# 7	Detailed description of the GSAD algorithm

Figure 1 is the block diagram of the GSAD system. The output of the VAD is a binary flag indicating the activity of the input frame. Active frames will be further classified into speech or music frames by the speech/music discriminator (SMD) and inactive frames will be further classified into silence or audible noise frames by the silence detector (SiD).

VAD

SMD

SiD

Input Signal

Speech/Music

Active?

Y

N

Silence/Audible noise

Figure 1 – Block diagram of the GSAD system

## 7.1	Detailed description of the VAD module

Figure 2 is the general diagram depicting the high-level operation of the VAD module, where each box indicates the number of the relevant clauses in the text.

Pre-processing and power spectrum calculation

(clause 7.1.1)

Parameters calculation

(clauses 7.1.2, 7.1.3, 7.1.4, 7.1.5)

Initial VAD decision

(clause 7.1.6)

VAD hangover

(clause 7.1.7)

Background signal estimation and identification

(clauses 7.1.8, 7.1.9, 7.1.10, 7.1.11, 7.1.14)

Input Signal

Figure 2 – General diagram of the VAD module

### 7.1.1	Pre-processing and power spectrum calculation

The input signal to the GSAD is a sampled audio signal at 10-ms frames, which means that each frame consists of 80 samples, for NB input, and of 160 samples, for WB input. The input samples are first pre-emphasized then windowed by an asymmetric window. The pre-emphasis is done by

(7.1-1)

where  is the i-th pre-emphasized sample, s(i) is the i-th original sample. The asymmetric window (128 points for NB and 256 points for WB) covers all samples of the current frame and also samples (48 samples for NB input and 96 samples for WB input) at the end of the past frame. A fast Fourier transform (FFT) of 128 points for NB input and of 256 points for WB input is performed on the windowed samples. For the m-th frame, the power spectrum elements  are obtained by the square root of the sum of squares of the real and the imaginary parts of the complex FFT coefficients, where k is the index for the power spectrum elements. In the following,  is denoted by  when describing the processing of the current frame, for brevity.

### 7.1.2	Differential zero crossing rate (DZCR) calculation

The zero crossing rate (ZCR) of the input frame is extracted from the original non-preprocessed time domain samples of the input frame:

(7.1-2)

where N is the number of samples per frame (N = 80 for NB input and N = 160 for WB input) and s(i) is the i-th original sample. The DZCR of the input frame is calculated by:

(7.1-3)

where  is the moving average of the ZCR of the estimated background signal. For the calculation of , refer to clause 7.1.14.

### 7.1.3	Modified segmental SNR (MSSNR) calculation

The spectrum of the input frame is divided into 16 non-equal sub-bands in the frequency domain. The energy of each sub-band is calculated by:

(7.1-4)

where i is the index of sub-band, l(i) is the lower bound of the i-th sub-band, h(i) is the higher bound of the i-th sub-band, S(k) is the power spectrum of the k-th spectral bin,  is the energy of the i-th sub-band of the previous frame and is a weighting factor. The exact lower and higher bounds of each sub-band are given in Table 3 for NB input and in Table 4 for WB input.

Table 3 – Boundaries of the sub-bands for NB input

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| l(i) | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 17 | 20 | 23 | 27 | 31 | 36 | 42 | 49 | 56 |
| h(i) | 3 | 5 | 7 | 9 | 11 | 13 | 16 | 19 | 22 | 26 | 30 | 35 | 41 | 48 | 55 | 63 |

Table 4 – Boundaries of the sub-bands for WB input

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| l(i) | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 17 | 20 | 23 | 27 | 37 | 49 | 63 | 79 | 97 |
| h(i) | 3 | 5 | 7 | 9 | 11 | 13 | 16 | 19 | 22 | 26 | 36 | 48 | 62 | 78 | 96 | 127 |

The weighting factor  is determined by the long term SNR, lsnr, as (for the calculation of lsnr, refer to clause 7.1.4):

(7.1-5)

For the first input frame,  is set to 1.

The sub-band SNR is calculated as:

(7.1-6)

where  is the moving average of the energy of the i-th sub-band of the estimated background signal (for the calculation of , refer to clause 7.1.14).

The modified sub-band SNR is calculated by:

(7.1-7)

Finally, the modified segmental SNR (MSSNR) is obtained by:

(7.1-8)

### 7.1.4	Long term SNR estimation

The root mean square (RMS) of the input frame is calculated based on the original non-processed samples of the input frame:

(7.1-9)

where rms is the RMS of the input frame, s(i) is the i-th sample of the non-preprocessed input frame, N is the number of samples per frame (N = 80 for NB input and N = 160 for WB input).

Following, a long term RMS of the background signal and a long term RMS of the foreground signal are estimated. For the m-th frame, when the background update flag update_flg is set (for the setting of update_flg, refer to clause 7.1.14), the long term RMS of the background signal is updated by:

(7.1-10)

where  and  are the long term RMSs of the background signal of the m-th and the (m–1)-th frames respectively, and  is an adaptive factor controlling the update speed of the :

(7.1-11)

Similarly, for the m-th frame, when its MSSNR is greater than a threshold, the long term RMS of the foreground signal is updated by:

(7.1-12)

where  and  are the long term RMS of the foreground signal of the m-th and the 
(m–1)-th frames respectively, and is an adaptive factor controlling the update speed of the :

(7.1-13)

Finally, the long term SNR, lsnr, is obtained by:

(7.1-14)

### 7.1.5	Background fluctuation estimation

The fluctuation of the background signal fluxbgd is estimated over background frames. When the MSSNR of the input frame is below a threshold of 15, the input frame is considered as background frame and the fluxbgd is updated by:

(7.1-15)

where  (the factor controlling the update speed of the fluxbgd) is determined by:

(7.1-16)

In equation 7.1-16, the initialization period consists of the first 100 frames whose MSSNR is below the threshold of 15.

### 7.1.6	Initial VAD decision

The initial VAD decision is made based on a set of inequalities involving the calculated DZCR and MSSNR, where the inequalities' factors are adapted according to the long term SNR. The initial VAD decision is made by comparing a pair of linear combinations of DZCR and MSSNR to a threshold that adapts to the long term SNR, the background fluctuation and the operating point. The decision rule is:

(7.1-17)

where  is a factor determined by the long term SNR and thrvad is a VAD decision threshold that is determined jointly by the long term SNR, background fluctuation and the operating point. The initial VAD decision, ivad, is either "1" (active) or "0" (inactive).

The determination of  and thrvad is described below. First, the long term SNR, lsnr, is classified into four categories: very high SNR, high SNR, medium SNR and low SNR, where each corresponds to the following values of :

(7.1-18)

Further, the background fluctuation is classified into three categories: high, medium and low fluctuation. Threshold thrvad is chosen from a NB or WB table indexed by the long term SNR category, the background fluctuation category and the operating point. Basically, thrvad is biased for different operating points for the same condition (i.e., the same long term SNR and the same background fluctuation), but for the very high SNR condition, the bias is ignored.

### 7.1.7	VAD hangover

The final VAD decision is made by passing the initial VAD decision through a hangover procedure. The hangover procedure consists of three independent hangover mechanisms operating in parallel (see Figure 3).

hang_sp>0? or

hang_f_mus>0? or

hang_b_mus>0?

Final VAD = 1 ; hang_sp--;  hang_f_mus--; hang_b_mus--;

hang_b_mus<0?

hang_f_mus<0?

hang_sp<0?

hang_sp = 0

hang_f_mus = 0

hang_b_mus = 0

initial VAD decision = 0?

Initial VAD decision

Final VAD = 1

Y

N

Y

Y

Y

Y

Final VAD = 0

N

Figure 3 – Flow chart of the hangover procedure

In Figure 3, hang_sp, hang_f_mus and hang_b_mus are three independent hangover counters respectively for speech, foreground music and background music. The hangover for speech addresses low energy frames where misdetection can occur at the offset of speech bursts. The hangover for foreground music helps to resolve occasional misdetections of foreground music signals and the hangover for background music helps to maintain the quality of the music background. In addition, the first 200 inactive final VAD indications are forced to "1" to protect the beginning of the input where misdetections can occur due to false initialization of the background estimate. The hang_sp is reset to a maximum value hangs when five successive foreground frames are detected. The value of hangs is determined jointly by the long term SNR, lsnr, and the operating point. For the reset of hang_b_mus and hang_f_mus, refer to clauses 7.1.12 and 7.1.14, respectively.

### 7.1.8	Calculation of tone stability and DTMF detection

A set of local spectral peaks of the power spectrum of the input frame is identified within the spectral band between 250 Hz and 3300 Hz. A local spectral peak is identified as being higher than its immediate left and right neighbouring spectral bins.

For each i-th local spectral peak, Dp2s(i), which is the power distance between it and its adjacent four spectral bins on its two sides, is calculated:

(7.1-19)

where Sj is the power of the j-th spectral bin, peakloc(i) is the power of the i-th local spectral peak, and idx_peakloc(i) is the position of the i-th local spectral peak in the spectrum. The frame index [m] is omitted for brevity.

The positions of the three local spectral peaks with the largest three Dp2s values are found and denoted as ,  and , where:

(7.1-20)

For each of the three stored spectral positions, their distance to the position of the spectral peak that has the maximum Dp2s in the previous frame, , is obtained by:

(7.1-21)

If any of Dp2old(0), Dp2old(1), or Dp2old(2) is less than 2, the counter tone_sta_cnt for tone stability is incremented by 1, and it is used for the background update decision (see clause 7.1.14). The value of idx_peakmax(0) is used to update idx_peakmax_old for next frame's analysis. Further, the three maxima of the local spectral peaks are identified and normalized by the average of the spectral power of the input frame from the third to the 64th spectral bins, except for the three spectral peak maxima. If the sum of the three normalized maxima is greater than one threshold, or the sum of the two largest of the three normalized maxima is greater than a second threshold, or the largest among the three normalized maxima is greater than a third threshold, a DTMF signal is detected and indicated by setting the DTMF flag dtmf_flg.

### 7.1.9	Calculation of spectral peak fluctuation

The global spectral peak of the m-th input frame is identified as the largest local spectral peak of that frame. The position of the m-th input frame's global spectral peak in its spectrum  is compared to that of the previous frame . If  is different from , the counter for spectral peak fluctuation peak_flux_cnt is incremented by 1. The value of peak_flux_cnt is used in the background update decision (see clause 7.1.14).

### 7.1.10	Calculation of spectral peakiness

The local spectral peaks identified in clause 7.1.8 are used to calculate the spectral peakiness of the input frame. For each of the j-th local spectral peaks, a lower frequency spectral valley Evl(j) and a higher frequency spectral valley Evh(j) are defined by the lowest value of the power spectrum in the four FFT bins with frequency below the bin of the j-th local spectral peak, and by the lowest value of the power spectrum in the four FFT bins with frequency above the bin of the j-th local spectral peak, respectively. This can be mathematically expressed as:

(7.1-22)

(7.1-23)

where S(k) is the k-th power spectral bin of the input frame, idx_peakloc(j) is the location of the j-th local spectral peak in the spectrum. The normalized peak-to-valley distance is calculated by:

(7.1-24)

The spectral peakiness, SP, is obtained by summing the three maxima of Dp2v(j) over all values of j.

### 7.1.11	Calculation of frequency stability

The frequency stability stafq is used in the background update decision. The spectrum of the input frame is whitened by a moving average of the spectra over past frames and stafq is obtained as the power deviation of the 16 whitened sub-bands:

(7.1-25)

where  is the energy of the i-th sub-band of the input frame (see clause 7.1.3),  is the moving average of the energy of the i-th sub-band over past frames, is the power mean of the 16 whitened sub-bands calculated by:

(7.1-26)

is updated every time after the stafq calculation using:

(7.1-27)

### 7.1.12	Background music detection

Detection for background music is done for every 100 past background frames. Background music is detected if the sum of the spectral peakinesses over the 100 past background frames, SPsum, is above a music detection threshold. A hangover of 1000 frames is set once music background is detected, but it will be fast exited if SPsum is sufficiently low. Moreover, the music detection threshold is further "biased" depending on whether the background music hangover exits. The procedure of the background music detection is depicted in Figure 4.

update_flg = 0?

bgd_frm_cnt++;

SPsum=SPsum+SP;

bgd_frm_cnt = 100?

hang_b_mus > 0?

SPsum > 1100?

SPsum > 1200?

bgd_frm_cnt =  0;

SPsum = 0;

hang_b_mus =  1000;

SPsum < 400?

Y

Y

Y

Y

Y

N

hang_b_mus = 0;

N

N

N

exit;

Y

exit;

N

Figure 4 – Flow chart of the background music detection procedure

In Figure 4, the background frame is indicated by the background update flag update_flg ("0" for background frame), bgd_frm_cnt is a counter counting the number of background frames, hang_b_mus is the hangover counter for background music, SPsum is the spectral peakiness accumulator storing the sum of the frame peakinesses.

### 7.1.13	Silence detection

The signal power in dBov of the input frame is calculated by:

(7.1-28)

where rms is the RMS of the input frame (see clause 7.1.4). If FPdBov is below a silence threshold of –56 dBov, by default the input frame is indicated as silence frame.

### 7.1.14	Background estimate update

A background decision threshold thrbgd is first determined jointly from the long term SNR, lsnr, and the background music hangover, hang_b_mus.

(7.1-29)

The decision whether to update the background signal estimate consists of the following steps (see also the flowchart in Figure 5).

Step 1) If the MSSNR of the input frame is not greater than thrbgd, a counter for counting consecutive background frames, update_cnt, is incremented by 1. If update_cnt is greater than M frames and no DTMF signal is detected and the foreground music hangover hang_f_mus is not greater than 0, the background update flag update_flg, and the reset flag, reset_flg, are set. The value of M is determined from lsnr:

(7.1-30)

If update_flg is set, the input frame will be used to update the background estimate. The purpose of reset_flg is described later in this clause. The counting for consecutive background frames tolerates occasional background frames whose MSSNRs are greater than thrbgd. If update_cnt ≥ M but the MSSNR of the input frame is higher than thrbgd, update_cnt is set to be M. The update_cnt is reset to 0 if more than nine consecutive frames whose MSSNRs are greater than 50 are detected.

Step 2) While the update_flg is not set at Step 1, background noise is sought within time windows of 30 frames in sequence and used to update the background estimate if there is one. The possibility of containing background frames is estimated for each time window by analysing the tonality and the stability of the frames within the time window. If all or most of the frames within the time window are recognized as background frames and the few non-background frames are not at the end of the time window, the latest frame within the time window (i.e. the input frame) is used to update the background estimate by setting the background update flag update_flg. Otherwise, if the time window is believed (but with less confidence) to contain background frames, the minimum energies for each sub-band over frames within the time window are used to update the sub-band energies of the background estimate. Otherwise, the time window is considered not to have background frames and analysis for another time window starts.  If the input frame (the latest frame within the current time window) is used for background estimate update, successive frames which have high frequency stability are also used for the background estimate update until a frame with low frequency stability is encountered. Details of step 2 are described below.

For each input frame, if the update_flg is not set at Step 1 and if no DTMF signal is detected and the foreground music hangover hang_f_mus is not greater than 0, the following operations a) to d) are performed, otherwise the reset_flg is set:

a)	a counter con_frm_cnt for counting the number of consecutive frames is incremented by 1;

b)	the spectral peakiness SP of the input frame is compared to a threshold thrSP_low which is 15 for NB input and 12.7 for WB input. If SP<thrSP_low, counter low_SP_cnt (that counts the number of frames with low spectral peakiness) is incremented by 1;

c)	the frequency stability stafq is compared to a threshold, thrfst, which is 12 for NB input and 10 for WB input. If stafq<thrfst, a counter, high_fst_cnt, for counting the number of frames with high frequency-stability is incremented by 1;

d)	the sub-band energies Eband(i), i=0,1,...15, of the input frame are used to update the minimum sub-band energy buffer Eband_buf_min(i), i=0,1,...15, for those bands where Eband(i)<Eband_buf_min(i).

N

Y

MSSNR≤thrbgd ?

MSSNR>50?

burst_cnt ++

update_cnt>M? and

no DTMF detected and hang_f_mus≤0?

update_cnt≥M?

Set update_ cnt=M

stafq<thrfst?

con_frm_cnt=30?

low_SP_cnt>15?

high_fst_cnt>28 and

tone_sta_cnt<12 and
stafq<12 and peak_flux_cnt>15?

high_fst_cnt ++

SP<thrSP_low?

low_SP_cnt ++

peak_flux_cnt>15 and tone_sta_cnt<12?

Use the minimum band energy buffer to update the background estimate

low_SP_cnt<5 and high_fst_cnt>28?

set hang_f_mus=10

No DTMF detected and hang_f_mus≤0?

con_frm_cnt++

Update minimum band energy buffer

N

exit

Y

con_frm_cnt>30?

stafq<12?

Set the update_flg=1

Set burst_cnt=0

Y

update_cnt++

N

e

Y

N

d

N

Y

a, b

N

b

Y

Y

Y

e

N

e

N

N

Y

e

N

Y

Y

b, c

N

Y

a

N

Y

Set the reset_flg=1

b

N

c

Y

b

N

b

a

e

Y

e

N

burst_cnt≥10?

Set update_ cnt=0

d

e

Figure 5 – Flowchart of the background estimate update procedure

Once con_frm_cnt reaches 30, the low_SP_cnt is compared to a threshold of 15. If low_SP_cnt is not greater than 15, the reset_flg is set. Otherwise, the following condition is examined:

(7.1-31)

where tone_sta_cnt and peak_flux_cnt are obtained in clauses 7.1.8 and 7.1.9, respectively. If the condition above is satisfied, the background update flag, update_flg, is set to 1. Otherwise, if the following condition is satisfied:

(7.1-32)

the sub-band energies Eband_buf_min(i), i=0,1,...15, stored in the minimum sub-band energy buffer are used to update the background estimate. Otherwise, the reset_flg is set. Moreover, if low_SP_cnt <5 and high_fst_cnt > 28, foreground music is considered to be detected and the foreground music hangover hang_f_mus is set to 10.

If con_frm_cnt exceeds 30, only the frequency stability, stafq, is examined. If stafq<12, the background update flag update_flg is set. Otherwise, the reset_flg is set to 1.

If reset_flg is set to 1, the parameters con_frm_cnt, low_SP_cnt, high_fst_cnt, tone_sta_cnt and peak_flux_cnt are all reset to 0.

If update_flg is set to 1, the background estimate for ZCR and sub-band energies are updated by:

(7.1-33)

(7.1-34)

where  and  are the moving average of the ZCR of the background signal at the current and the previous frames respectively, and  are the moving averages of the energy of the i-th sub-band of the background signal at the current and the previous frames, i = 0,1,2,…,15.  is initialized at the first four input frames as the average of the current and the past frames' ZCR.  for all i is initialized at the first four input frames as the energy of the i-th sub-band of the input frame, which is further limited by a minimum value. If a successive high level signal is detected during the first four input frames,  and  for all i are reset.

## 7.2	Detailed description of the speech/music discrimination module

This clause describes the speech/music discrimination module. The calculation of the main discrimination parameter, the variance of the flux, is described in clause 7.2.1; the calculation of two peakiness measures is described in clause 7.2.2. Clause 7.2.3 provides the details of the speech/music discrimination decision algorithm.

### 7.2.1	Calculation of the flux and the variance of the flux

The flux parameter measures the normalized distance between the power spectrum of the current frame and the power spectrum of the previous frames. The difference is measured for the spectral band between 62 Hz to 2914 Hz for both narrow-band (NB) and wideband (WB) signals. Let  denote the i-th power spectrum component of the m-th frame. The flux for the m-th frame is calculated by:

(7.2-1)

The variance of the flux is calculated for the last 20 frames that are detected as "active" frames by a simple threshold on the MSSNR value (see clause 7.1.3 for the description of the MSSNR calculations). The values of the flux for each active frame are stored in a FIFO buffer, which is updated only for active frames. The MSSNR of the m-th frame is denoted in the sequel by . It should be noted that the active frames for the calculation of the variance of the flux, which is based on MSSNR, are different from the active frames indicated by the VAD algorithm part of the GSAD.

For the first 20 active frames the variance is set to 0 and at the 20th active frame an average of the flux, mov_flux, is initialized to the arithmetic average of the flux value of the first 20 active frames. For each frame after the first 20 active frames, the average is updated for each active frame by:

(7.2-2)

The variance of the flux for the m-th frame, , is calculated as:

(7.2-3)

where the index m is incremented only for the active frames. The values of  for the first 20 active frames are multiplied by a linearly ramping window. A buffer of the last 120 frames of the active  is used by the SMD algorithm.

### 7.2.2	Calculation of two spectral-peaks peakiness measures

Two measures of the peakiness of the large peaks of the power spectrum are calculated. The peaks of the power spectrum are found by searching the power spectrum for samples that are higher than their immediate neighbouring samples. The K highest-value spectral peaks, denoted as  K ≤ 5, are used (if less peaks are found, K might be less than 5).

The first peakiness measure, P1, is calculated by:

(7.2-4)

The second peakiness measure, P2, is calculated by:

(7.2-5)

The moving averages of the two peakiness measures are calculated by:

(7.2-6)

and

.	(7.2-7)

### 7.2.3	Speech/Music discrimination decision

The SMD algorithm uses a buffer of L=120 past  parameters and a binary-value buffer that contains information on  for the past 512 frames.

An adaptive threshold  is calculated using parameter for the past 512 frames. At the first step, a parameter which follows the maximal value of  for each frame m is calculated. The maximal value parameter, , is set to  if  is larger than  and is otherwise decreased by a multiplicative factor of 0.9999. This reduction allows a slow adaptation of the maximum value when  is decreased over a long period. An adaptive threshold is set by multiplying  by a constant that depends on the operating point:

(7.2-8)

where  is 0.5 for the bandwidth-saving and the balanced operating points, and 0.45 for the quality-preferred operating point.

The information about  parameters is saved in a binary buffer which indicates, for each entry m, if  is higher than . Using this binary buffer, for each frame after the first 300 frames, a 2-bin histogram of the last K frames is calculated, where  counts the number of frames in the buffer where  is higher than  and  counts the number of frames in the buffer where  is not higher than . Obviously, only  needs to be calculated, since . The value of K is incremented by 1 for each frame after the first 300 frames until it reaches 512, when it is fixed. The calculation of  is done simply by considering the current frame m (increment  if  is higher than ) and the frame m–512 (decrement  if the last element in the buffer is 1, which means that ). From , the histogram difference measure , which is between –1 and 1, is calculated according to:

(7.2-9)

The final histogram difference measure is generated as a moving average of the instantaneous histogram difference and is further "biased" with an offset factor, , which depends on the operating point and is a small negative or positive number:

(7.2-10)

where, for the first 300 frames,  is set to . The average difference measure is then limited to be between  and , where , and is denoted by . The adaptive threshold for  is given by the linear equation:

(7.2-11)

where A and B are calculated by:

(7.2-12)

(7.2-13)

is the desired highest value for  and  is the desired lowest value for . For music signals (which typically have a lower var_flux), the threshold is set to a higher value, which helps in preferring the detection of music signals. Similarly, for speech signals (that typically have a higher var_flux), the threshold is set to a lower value, which helps in preferring the detection of speech signals.

Using the adaptive threshold, , the ratio of the times  was above that adaptive threshold for the past J frames is calculated for the frames which are declared active based on the threshold on MSSNR, or the first frame where the VAD is active and the threshold of MSSNR was not achieved yet. This definition holds also for the reset mechanism described below. If the ratio is above 0.5, the raw SMD decision is set to speech and if the ratio is below 0.5, the raw SMD decision is set to music. The value of J is incremented for each frame declared active by the criterion above until it reaches 120, when it is fixed. For the first 75 active frames, the ratio is "biased" toward speech. This is done to help avoiding some speech-to-music misclassification at the beginning, where the decision is based on the yet-unstable adaptive threshold.

A special mechanism is used to detect the end of long music segments and to reset the SMD at such conditions. This mechanism uses the fact that typical music segments are characterized by high MSSNR over a long period of time. As a first step, a segment of at least 500 frames where MSSNR is continuously above the fixed value of 8 is detected. At the end of such segment, which is called "falling edge", the algorithm checks in the next segment of 75 frames if, for at least 40 of these frames, MSSNR is below 15. If such low MSSNR segment is detected, the algorithm declares an end of music segment. The "falling edge" conditions last for 150 frames. Once a low MSSNR segment is detected, the SMD algorithm is being "reset". The following operation is to set  to  for a period of 400 frames, thus neutralizing the previously calculated preference for music. The reset also sets J to 0, causing the previously accumulated values of var_flux in the buffer not to be used, as well as the application of a "bias" toward speech for the next 75 frames.

The values of the two moving averages of the spectral-peak peakiness measures are used to correct the raw SMD decision toward music:

–	For NB signals, if  or , the raw SMD decision is set to music.

–	For WB signals, if  or , the raw SMD decision is set to music.

The final SMD decision is obtained by a one frame hangover of the raw SMD decision.

# 8	Organization of the reference C code

Table 5 lists the files that comprise the ANSI-C source code attached to this Recommendation and which is an integral part of this Recommendation. For this edition of the Recommendation, the software is labelled as "Release 1.0a". Proper compilation of the code can be checked with the test vectors also provided as an electronic attachment to this Recommendation.

Table 5 – Organization of the GSAD source files

| File/directory name | Description |
| --- | --- |
| commandpars.c/h | Reading the command line |
| ereal_fft_fx.c/h | Modified real FFT |
| gsad.c | Main function |
| gsad_math_adv_fx.c/h | Arithmetic functions |
| parameters_fx.c/h | Parameter calculation functions |
| preproc_fx.c/h | Pre-processing functions |
| vad_fx.c/h | Main VAD function |
| smd_fx.c/h | Main SMD function |
| rom_fx.c/h | ROM tables |
| constdef_fx.h | Macro defined for constants |
| wmops.c/h | Complexity and RAM measuring tool |
| stl2005_basop | Directory containing the source code for the ITU-T fixed-point basic operators adapted from [b-ITU-T G.191 (2005)] (basop32_fx.c, basop32_fx.h, control_fx.c, control_fx.h, count_fx.c, count_fx.h, enh1632_fx.c, enh1632_fx.h, enh40_fx.c, enh40_fx.h, move_fx.h, patch_fx.h, stl_fx.h, typedef_fx.h, and typedefs_fx.h) |

Annex A

Generic voice activity detector

(This annex forms an integral part of this Recommendation)

# A.1	Scope

The generic voice activity detector (GVAD) is an independent front-end processing module which can be applied prior to signal processing applications such as, but not limited to, speech or audio codecs. GVAD operates on narrow-band or wideband audio input at 10-ms frame length (without lookahead). Its function is to indicate the input frame activity. In order to apply GVAD in specific cases, an adaptation layer may be required.

The GVAD is bit-exact with the VAD module of the main body of this Recommendation, with the speech/music discriminator, the silence detector of the main Recommendation removed to save implementation cost for applications requiring VAD only.

This annex is organized as follows. References, definitions, abbreviations/acronyms and conventions are defined in clauses A.2, A.3, A.4 and A.5, respectively. Clause A.6 gives a general description of the GVAD algorithm including the input sampling rate, the operating frame length, the algorithmic delay, the configurations and the complexity and memory cost. The detailed description of the GVAD algorithm is described in clause A.7. Finally in clause A.8, use of the simulation software is described. References to the main Recommendation are made for those parts of the description that are the same as the main Recommendation.

# A.2	References

See clause 2.

# A.3	Definitions

## A.3.1	Terms defined in this annex

This annex defines the following terms:

A.3.1.1	balanced operating point: Generic voice activity detector (GVAD) operating point that provides a middle point performance between the other two operating points. See clause A.6.4.

A.3.1.2	bandwidth-saving operating point: Generic voice activity detector (GVAD) operating point privileging accurate detection for background signals. See clause A.6.4.

A.3.1.3	quality-preferred operating point: Generic voice activity detector (GVAD) operating point privileging accurate detection for foreground signals. See clause A.6.4.

# A.4	Abbreviations and acronyms

See clause 4.

# A.5	Conventions

This annex does not define any conventions.

# A.6	General description of the GVAD algorithm

## A.6.1	Input sampling rate

The GVAD can accept both narrow-band (NB) and wideband (WB) audio input signals sampled at 8 and 16 kHz, respectively. The sampling rate of the input signal is provided to the GVAD by an external signal.

## A.6.2	Operating frame size

GVAD operates on a 10-ms basis, i.e. frame of 80 samples for NB audio input and 160 samples for WB audio input. Consecutive GVAD indications can be combined to indicate the activity for frames with a multiple of 10 ms.

## A.6.3	Delay

GVAD does not introduce lookahead, therefore the added delay of the GVAD is 0 ms (algorithmic delay of 10 ms).

## A.6.4	Configurations

GVAD includes two control parameters. GVAD operates on narrow-band (NB) and wideband (WB) audio input signals, where a control signal indicates the bandwidth of the input signal.

In addition, a second control signal selects the desired operating point from three possibilities.  Setting this second control signal to "0" selects the balanced operating point, setting it to "1" selects the quality-preferred operating point and setting it to "2" selects the bandwidth-saving operating point. These three operating points provide the ability to select a preferred balance between the bandwidth saving and the audio quality. The bandwidth-saving operating point provides maximal bandwidth saving while maintaining acceptable audio quality, while the quality-preferred operating point provides higher audio quality with less bandwidth saving. The balanced operating point provides an optimum performance which is between the bandwidth-saving operating point and the quality-preferred operating point. For example, for the speech database used in the ITU-T G.720.1 selection phase with added car, office and babble background noises, the bandwidth-saving operating point saves approximately 30% of the bandwidth compared to the balanced operating point and approximately 50% of the bandwidth compared to the quality-preferred operating point.

GVAD outputs a binary value where "1" indicates an active frame and "0" indicates an inactive frame.

## A.6.5	Complexity and memory cost

Table A.1 shows the GVAD complexity in WMOPS for NB and WB signal sampling frequencies respectively. The RAM used for GVAD (static and dynamic) is 2436 bytes and the table ROM is 1674 bytes.

Table A.1 – Complexity of the GVAD

| Modes | Complexity (WMOPS) |
| --- | --- |
| GVAD_WB | 2.397 |
| GVAD_NB | 1.475 |

# A.7	Detailed description of the GVAD algorithm

The detailed description of the GVAD algorithm follows exactly that of the VAD module (see clause 7.1).

# A.8	Use of the simulation software

The bit-exact, fixed point simulation software of GVAD is defined by compilation flag "LC" in the ANSI-C source code of the main body of this Recommendation. By defining the compilation flag "LC" in the ITU-T G.720.1 source code, the compiler will generate the executable for GVAD. The command line instruction for GVAD thus becomes:

gsad  rate_option  op_option  speech_file  decision_file

Required arguments:

rate_option:    8k or 16k

===========================================================

8k:       8 kHz sampling rate

16k:    16 kHz sampling rate

===========================================================

op_option:    0, 1 or 2

===========================================================

0:  Balanced operating point

1:  Quality-preferred operating point

2:  Bandwidth-saving operating point

===========================================================

speech_file: 	Input speech file name

decision_file:	Output signal class indicator file name

Proper compilation of the code can be checked with the test vectors provided for ITU-T G.720.1 bit-exactness check.

Bibliography

[b-ITU-T G.191 (2005)]	Recommendation ITU-T G.191 (2005), Software tools for speech and audio coding standardization.

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | Telecommunication management, including TMN and network maintenance |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Terminals and subjective and objective assessment methods |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks, open system communications and security |
| Series Y | Global information infrastructure, Internet protocol aspects and next-generation networks |
| Series Z | Languages and general software aspects for telecommunication systems |

____________________

This Recommendation contains an electronic attachment with the ANSI C source code which forms an integral part of this Recommendation, and a set of test vectors. The set of test vectors is also available for download from the ITU-T Test Signal Database at: http://www.itu.int/net/ITU-T/sigdb/speaudio/Gseries.htm#G.720.1.

Note that, because the C source code of the main body of this Recommendation is used to generate the GVAD executable, the default name of the generated GVAD executable is kept the same to that of the main body of ITU-T G.720.1 ("gsad").