|  |  | International Telecommunication Union |  |  |
| --- | --- | --- | --- | --- |
| ITU-T | G.729.1 |  |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Corrigendum 1 (08/2009) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Coding of voice and audio signals |  |  |  |
|  | G.729-based embedded variable bit-rate coder: An 8-32 kbit/s scalable wideband coder bitstream interoperable with G.729 Corrigendum 1: Corrections to source code |  |  |  |
|  | Recommendation ITU-T G.729.1 (2006) – Corrigendum 1 |  |  |  |

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TRANSMISSION MEDIA AND OPTICAL SYSTEMS CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of voice and audio signals | G.710–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| MULTIMEDIA QUALITY OF SERVICE AND PERFORMANCE – GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DATA OVER TRANSPORT – GENERIC ASPECTS | G.7000–G.7999 |
| PACKET OVER TRANSPORT ASPECTS | G.8000–G.8999 |
| ACCESS NETWORKS | G.9000–G.9999 |

For further details, please refer to the list of ITU-T Recommendations.

Recommendation ITU-T G.729.1 G.729-based embedded variable bit-rate coder: An 8-32 kbit/s scalable wideband coder bitstream interoperable with G.729 Corrigendum 1 Corrections to source code

Summary Corrigendum 1 to Recommendation ITU-T G.729.1 contains the details of several problems that have been identified recently in the C-code of the main body of Recommendation ITU-T G.729.1, and of its Annexes B, C and D. Corrigendum 1 does not introduce any changes to the text description of ITU-T G.729.1 (including Annexes A, B, C and D). Corrigendum 1 revises the source code to Release 1.5, which is found in the electronic attachment. The attachment incorporates the ANSI C source code for the main body of ITU-T G.729.1, Annex B, Annex C and Annex D (including updated test vectors for Annex C) and all approved corrections till 29 September 2009.

Source Corrigendum 1 to Recommendation ITU-T G.729.1 (2006) was approved on 29 August 2009 by ITU-T Study Group 16 (2009-2012) under Recommendation ITU-T A.8 procedures.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications, information and communication technologies (ICTs). The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g., interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementers are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database at http://www.itu.int/ITU-T/ipr/.

ITU  2010

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1	Scope		1

2	Details of the C-code changes		1

2.1	Corrections to G.729.1 main body		1

2.2	Corrections to ITU-T G.729.1 Annex B		1

2.3	Corrections to ITU-T G.729.1 Annex C		2

2.4	Corrections to ITU-T G.729.1 Annex D		4

Recommendation ITU-T G.729.1

G.729-based embedded variable bit-rate coder: An 8-32 kbit/s scalable
wideband coder bitstream interoperable with G.729

Corrigendum 1

Corrections to source code

# 1	Scope

This corrigendum contains the details of the problems that have been identified and the source code corrections which have been included to address these. Corrigendum 1 does not introduce any changes to the text description of ITU-T G.729.1 (including Annexes A, B, C and D).

The text below is organized into clauses for each of the affected files and the details of the corrections and is provided as assistance to the reader in understanding the modifications introduced. It should be noted that, since the C code of ITU-T G.729.1 takes precedence over the text description, the definitive source of information is the ANSI C source code found in the electronic attachment to this corrigendum.

Corrigendum 1 revises the source code to Release 1.5.

# 2	Details of the C-code changes

## 2.1	Corrections to G.729.1 main body

A problem in case of low delay mode at 14 kbit/s in presence of frame erasures has been reported. It was identified that the problem is due to the usage of the local array BfAq prior to its initialization in some specific conditions (low delay mode at 14 kbit/s, one single erased frame).

Therefore, in the file G729EV_MAIN_decod.c, lines 394-403:

IF(sub(decstat->decStG729.bfi_cnt, 1) == 0)

{

G729EV_FEC_mod_adapt_codebook(decstat->decStG729.old_exc, ind_fer[2],

pitch_buf, L_enr_q, BfAq + 3 * G729EV_G729_MP1);

}

ELSE

{

G729EV_FEC_mod_adapt_codebook(decstat->decStG729.old_exc, ind_fer[2],

pitch_buf, L_enr_q, decstat->mem_Az + 3 * G729EV_G729_MP1);

}

are replaced by the code:

G729EV_FEC_mod_adapt_codebook(decstat->decStG729.old_exc, ind_fer[2],

pitch_buf, L_enr_q, decstat->mem_Az + 3 * G729EV_G729_MP1);

This correction does not change the bit exactness of the output, the test vectors do not need to be replaced.

## 2.2	Corrections to ITU-T G.729.1 Annex B

The same problems described in clause 2.1 for G.729.1 main body exist for Annex B, which specifies an alternative implementation based on floating point arithmetic.

In the file G729EV_MAIN_decod.c, lines 293-298:

if (decstat->decStG729.bfi_cnt == 1)

G729EV_FEC_mod_adapt_codebook(decstat->decStG729.old_exc, ind_fer[2],

pitch_buf, enr_q,  BfAq + 3 * G729EV_G729_MP1);

else

G729EV_FEC_mod_adapt_codebook(decstat->decStG729.old_exc, ind_fer[2],

pitch_buf, enr_q, decstat->mem_Az + 3 * G729EV_G729_MP1);

are replaced by the code:

G729EV_FEC_mod_adapt_codebook(decstat->decStG729.old_exc, ind_fer[2],

pitch_buf, enr_q, decstat->mem_Az + 3 * G729EV_G729_MP1);

## 2.3	Corrections to ITU-T G.729.1 Annex C

### 2.3.1	Correction 1 – Alignment with Amd.5

As corrected in Amd.5 for ITU-T G.729.1 main body, in file G729EV_CELP2S_decod.c, line 558:

nb_celp_param->power_fix[i0_2sub] = L_temp2;

is replaced by the code:

nb_celp_param->power_fix[i0_2sub] = L_shl(L_temp2, 2);

### 2.3.2	Correction 2 – Alignment with Amd.5

As corrected in Amd.5 for ITU-T G.729.1 main Body, ITU-T G.729.1 Annex B and ITU-T G.729.1 Annex D, in file G729EV_CELP2S_encod.c, when the bitrate is equal to 8 kbit/s, no 12 kbit/s synthesis needs to be computed, but the memory for 12 kbit/s CELP should be updated by the memory of 8 kbit/s CELP.

Therefore, in the file G729EV_CELP2S_encod.c, lines 934-935:

IF(sub(pCodStat->rate, 8000) == 0)

G729EV_G729_Copy(synth_ptr, DataSynth, G729EV_G729_L_FRAME);

are replaced by the code:

IF (sub(pCodStat->rate, 8000) == 0){

G729EV_G729_Copy(synth_ptr, DataSynth, G729EV_G729_L_FRAME);

G729EV_G729_Copy(pCodStat->mem_syn, pCodStat->mem_synth, G729EV_G729_M);

}

### 2.3.3	Correction 3 – Alignment with Amd.5

As corrected in Amd.5 for ITU-T G.729.1 main body, ITU-T G.729.1 Annex B and ITU-T G.729.1 Annex D, the C source code of the initialization procedure in the file G729EV_CELP2S_encod.c, line 55:

G729EV_G729_Set_zero(pCodStat->old_speech, G729EV_G729_L_TOTAL);

is replaced by the code:

G729EV_G729_Set_zero(pCodStat->old_speech, G729EV_G729_MEM_SPEECH);

### 2.3.4	Correction 4 – Addition of an initialization statement

The array Sid_Layer[] indicates if narrow-band enhancement layer happens for each 10 ms frame (i=1,2). For G.729B bitstream input mode, Sid_Layer[i] should be set as 0 for both 10 ms frames, since no enhancement layer is used for this bitstream.

Therefore, in the file G729EV_MAIN_decoder.c, after line 350, where the code is:

nb_bits_read = nb_bits_sent;

a new line of code is added:

Sid_Layer[i] = 0;

### 2.3.5	Correction 5 – Solving usage of uninitialized array

The problem reported in clause 2.1 is also present in Annex C.

Therefore, in the file G729EV_MAIN_decod.c, lines 446-455:

IF(sub(decstat->decStG729.bfi_cnt, 1) == 0)

{

G729EV_FEC_mod_adapt_codebook(decstat->decStG729.old_exc, ind_fer[2],

pitch_buf, L_enr_q, BfAq + 3 * G729EV_G729_MP1);

}

ELSE

{

G729EV_FEC_mod_adapt_codebook(decstat->decStG729.old_exc, ind_fer[2],

pitch_buf, L_enr_q, decstat->mem_Az + 3 * G729EV_G729_MP1);

}

are replaced by the code:

G729EV_FEC_mod_adapt_codebook(decstat->decStG729.old_exc, ind_fer[2],

pitch_buf, L_enr_q, decstat->mem_Az + 3 * G729EV_G729_MP1);

### 2.3.6	Correction 6 – Bitrate switching

A problem has been discovered in both Annex C and Annex D which contain the bitrate switching algorithm.  The problem causes the algorithm to wrongly reset the counter (decstat->count_rcv_dtx) to perform a smooth narrowband to wideband transition during the DTX/CNG mode for some cases. The correct counter reset is necessary to achieve the desired performance in all test vectors.

Therefore, in the file G729EV_MAIN_decod.c, lines 1056-1068:

#if(WMOPS)

test();

#endif

/* First SID after speech */

IF ((sub(decstat->prev_ftyp_null, 1) == 0) && (sub(decstat-

>prev_rate, 14000) < 0))

{

decstat->count_rcv_dtx = (Word16) 0;

#if(WMOPS)

move16();

#endif

}

ELSE IF(sub(decstat->old_SID_rate, RATE_SID_NB + RATE_SID) <= 0)

{

are replaced by the code:

/* First SID after speech */

IF (sub(decstat->prev_ftyp_null, 1) == 0)

{

if (sub(decstat->prev_rate, 14000) < 0)

{

decstat->count_rcv_dtx = (Word16) 0;

#if(WMOPS)

move16();

#endif

}

}

ELSE IF(sub(decstat->old_SID_rate, RATE_SID_NB + RATE_SID) <= 0)

{

## 2.4	Corrections to ITU-T G.729.1 Annex D

### 2.4.1	Correction 1 – Addition of an initialization statement

The problem reported in clause 2.3.4 is also present in Annex D.

Therefore, in the file G729EV_MAIN_decoder.c, after line 327, where the code is:

nb_bits_read = nb_bits_sent;

a new line of code is added:

Sid_Layer[i] = 0;

### 2.4.2	Correction 2 – Solving usage of uninitialized array

The problem reported in clause 2.1 is also present in Annex D.

Therefore, in the file G729EV_MAIN_decod.c, lines 345-350:

if (decstat->decStG729.bfi_cnt == 1)

G729EV_FEC_mod_adapt_codebook(decstat->decStG729.old_exc, ind_fer[2],

pitch_buf, enr_q, BfAq + 3 * G729EV_G729_MP1);

else

G729EV_FEC_mod_adapt_codebook(decstat->decStG729.old_exc, ind_fer[2],

pitch_buf, enr_q, decstat->mem_Az + 3 * G729EV_G729_MP1);

are replaced by the code:

G729EV_FEC_mod_adapt_codebook(decstat->decStG729.old_exc, ind_fer[2],

pitch_buf, enr_q, decstat->mem_Az + 3 * G729EV_G729_MP1);

### 2.4.3	Correction 3 – Bitrate switching

The problem reported in clause 2.3.6 is also present in Annex D.

Therefore, in the file G729EV_MAIN_decod.c, lines 852-856:

/* First SID after speech */

if ((decstat->prev_rate < 14000) && (decstat->prev_ftyp_null == 1))

decstat->count_rcv_dtx = 0;

else

{

are replaced by the code:

/* First SID after speech */

if (decstat->prev_ftyp_null == 1)

{

if (decstat->prev_rate < 14000)

{

decstat->count_rcv_dtx = 0;

}

}

else

{

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | Telecommunication management, including TMN and network maintenance |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Terminals and subjective and objective assessment methods |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks, open system communications and security |
| Series Y | Global information infrastructure, Internet protocol aspects and next-generation networks |
| Series Z | Languages and general software aspects for telecommunication systems |

____________________