| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G272 |
| --- | --- |

| Title: | Non-CE10: Core Transform Design for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong Yan Ye | Tel: Email: | +1 858 210 4800 jie.dong@interdigital.com yan.ye@interdigital.com |
| Source: | InterDigital Communication LLC |  |  |

_____________________________

# Abstract

This contribution presents the core transform design for HEVC, including 4×4, 8×8, and 16×16 forward/inverse transforms. The design has these features: (1) 16 bit data representation before and after each transform stage, (2) transforms can be implemented by full factorization, partial butterfly, or matrix multiplication, (3) 4×4 and 8×8 transforms are orthogonal, and 16×16 transform is nearly orthogonal, (4) N×N transform matrix is reused as the even part of the 2N×2N transform matrix, (5) no need for correction of different norms of basis vectors during quantization/dequantization. This design was integrated into HM4.0 and its R-D performance was tested under the common test conditions specified in JCTVC-F900. The average BD-rate increase is in the range of 0.1% to 0.2%, based on different test settings.

# Definitions of Transform Matrices

The transform matrices of 4×4, 8×8, and 16×16 transforms are given as in (1), (2), and (3), respectively.

T4=[

128   128   128   128

167    70   -70  -167

128  -128  -128   128

70  -167   167   -70

]

(1)

T8=[

256   256   256   256   256   256   256   256

360   297   198    72   -72  -198  -297  -360

334   140  -140  -334  -334  -140   140   334

297   -72  -360  -198   198   360    72  -297

256  -256  -256   256   256  -256  -256   256

198  -360    72   297  -297   -72   360  -198

140  -334   334  -140  -140   334  -334   140

72  -198   297  -360   360  -297   198   -72

]

(2)

T16=[

512   512   512   512   512   512   512   512   512   512   512   512   512   512   512   512

714   680   629   561   459   357   238    68   -68  -238  -357  -459  -561  -629  -680  -714

720   594   396   144  -144  -396  -594  -720  -720  -594  -396  -144   144   396   594   720

696   464   114  -366  -630  -718  -556  -188   188   556   718   630   366  -114  -464  -696

668   280  -280  -668  -668  -280   280   668   668   280  -280  -668  -668  -280   280   668

648    16  -558  -690  -234   466   724   316  -316  -724  -466   234   690   558   -16  -648

594  -144  -720  -396   396   720   144  -594  -594   144   720   396  -396  -720  -144   594

552  -312  -696    72   720   192  -648  -456   456   648  -192  -720   -72   696   312  -552

512  -512  -512   512   512  -512  -512   512   512  -512  -512   512   512  -512  -512   512

456  -648  -192   720   -72  -696   312   552  -552  -312   696    72  -720   192   648  -456

396  -720   144   594  -594  -144   720  -396  -396   720  -144  -594   594   144  -720   396

316  -724   466   234  -690   558    16  -648   648   -16  -558   690  -234  -466   724  -316

280  -668   668  -280  -280   668  -668   280   280  -668   668  -280  -280   668  -668   280

188  -556   718  -630   366   114  -464   696  -696   464  -114  -366   630  -718   556  -188

144  -396   594  -720   720  -594   396  -144  -144   396  -594   720  -720   594  -396   144

68  -238   357  -459   561  -629   680  -714   714  -680   629  -561   459  -357   238   -68    ]

(3)

# Forward Transform, Quantization, Dequantization, and Inverse Transform

Denote the bit-depth of the input to transform as BD, and denote (BD-9) as ΔBD. The block input to transform has the size M×N. When NSQT is enabled, M might be different from N.

## Forward Transform

The input to forward transform is a prediction residual block, denoted as XM×N. To perform a 2-D forward transform on XM×N, the M rows and N columns in XM×N are transformed in each dimension sequentially.

- M is equal to N or M is smaller than N.

The horizontal transform is performed first, as in (4), followed by right shifting with a proper number of bits to store intermediate result in 16-bit.

UM×N = ( XM×N × TNT + f ) >> ( log2(N) + log2(TN(0,0)) + ΔBD - 7 )                           (4)

After the horizontal transform, vertical forward transform is performed on the intermediate block UM×N, as shown in (5).

YM×N = (TM × UM×N + f ) >> (log2(M)+ log2( TM(0,0) ) )                                   (5)

- M is greater than N.

The vertical transform is performed first, as in (6).

UM×N = ( TM × XM×N + f ) >> ( log2(M)  + log2(TM(0,0)) + ΔBD – 7 )                         (6)

After the vertical transform, horizontal transform is performed on the intermediate block UM×N, as shown in (7).

YM×N = (UM×N × TNT + f  ) >> ( log2(N) + log2(TN(0,0) ) )                                 (7)

## Inverse Transform

The input to inverse transform is a dequantized block YM×N. To perform a 2-D inverse transform on YM×N, the M rows and N columns in YM×N are transformed in a sequential manner.

- M is equal to N or M is greater than N.

The horizontal transform is performed first, as in (8).

VM×N = ( YM×N × TN + f ) >>  ( log2( TN(0,0) ) + 1 )                                     (8)

After the horizontal transform, vertical inverse transform is performed on the intermediate block VM×N, as shown in (9).

XM×N = ( TMT × VM×N + f ) >> (log2( TM(0,0) ) + 6 - ΔBD)                            (9)

- M is smaller than N.

The vertical transform is performed first, as in (10).

VM×N = ( TMT × YM×N + f  ) >> ( log2( TM(0,0) ) + 1)                                         (10)

After the vertical transform, horizontal transform is performed on the intermediate block VM×N, as shown in (11).

XM×N = ( VM×N × TN + f ) >>  (log2( TN(0,0) ) + 6 – ΔBD )                                  (11)

## Quantization and Dequantization

Quantization and dequantization in this proposal are the same as in HM4.0 [1].

# Simulation Results

This design was integrated into HM4.0 [1] and its R-D performance was tested under the common test conditions specified in [2]. The average BD-rate increase is in the range of 0.1% to 0.2%, based on different test settings.

TABLE 1 R-D performance based on all intra settings (measured by BD-rate)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.1% | 0.3% | 0.2% | 0.3% | 0.5% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class E | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |

TABLE 2 R-D performance based on random access settings (measured by BD-rate)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.4% | 0.2% | 0.2% | 0.7% | 0.9% |
| Class B | 0.1% | 0.2% | 0.2% | 0.2% | 0.1% | 0.3% |
| Class C | 0.2% | 0.0% | 0.2% | 0.2% | 0.1% | 0.2% |
| Class D | 0.2% | 0.2% | -0.1% | 0.2% | 0.0% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% | 0.3% |
|  | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% | 0.3% |

TABLE 3 R-D performance based on low delay B settings (measured by BD-rate)

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | -0.1% | 0.2% | 0.2% | 0.3% |
| Class C | 0.2% | 0.2% | -0.1% | 0.2% | 0.4% | -0.1% |
| Class D | 0.2% | 0.3% | 0.1% | 0.2% | 0.0% | 0.1% |
| Class E | 0.3% | 0.0% | 0.0% | 0.0% | -0.2% | 0.4% |
| Overall | 0.2% | 0.2% | 0.0% | 0.2% | 0.1% | 0.2% |
|  | 0.2% | 0.2% | -0.1% | 0.2% | 0.1% | 0.2% |

TABLE 4 R-D performance based on low delay P settings (measured by BD-rate)

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.3% | 0.0% | 0.2% | 0.3% | 0.3% |
| Class C | 0.2% | 0.3% | -0.2% | 0.2% | 0.1% | 0.3% |
| Class D | 0.2% | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% |
| Class E | 0.2% | 0.0% | -0.5% | 0.1% | -0.6% | 0.7% |
| Overall | 0.2% | 0.2% | -0.1% | 0.2% | 0.0% | 0.3% |
|  | 0.2% | 0.2% | -0.2% | 0.2% | 0.0% | 0.2% |

# Conclusion

This contribution presents the core transform design for HEVC, including 4×4, 8×8, and 16×16 forward/inverse transforms. The design has these features: (1) 16 bit data representation before and after each transform stage, (2) transforms can be implemented by full factorization, partial butterfly, or matrix multiplication, (3) 4×4 and 8×8 transforms are orthogonal, and 16×16 transform is nearly orthogonal, (4) N×N transform matrix is reused as the even part of the 2N×2N transform matrix, (5) no need for correction of different norms of basis vectors during quantization/dequantization. This design was integrated into HM4.0 and its R-D performance was tested under the common test conditions specified in JCTVC-F900. The average BD-rate increase is in the range of 0.1% to 0.2%, based on different test settings. We suggest putting this design into the core experiment for further investigation.

# Reference

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0/

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900.

# Patent rights declaration(s)

InterDigital Communications LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).