| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0117 |
| --- | --- |

| Title: | AHG13: On inter-RPS prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ying Chen Ye-Kui Wang 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-658-5804 aramasub@qualcomm.com 1-858-845-6589 cheny@qualcomm.com 1-858-651-8345 yekuiw@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current working draft of HEVC, inter prediction between RPS candidates in SPS is enabled. In this document, a simplification of the current syntax design for inter-RPS prediction is proposed. The bit count increase for signalling short-term reference picture sets in the SPS for the proposed method, in comparison to the syntax structure in the latest working draft is claimed to be negligible, which is less than one byte for the random-access test condition and less than five bytes for the low-delay test condition for the entire bitstream.

# Introduction

In the current HEVC WD, prediction among the short-term RPS candidates inside an SPS is enabled by the highlighted syntax design as shown in the short-term reference picture set syntax table copied below.

We received complaints from implementers about the complexity of the syntax and associated decoding process.

In some cases, when the RPS of a picture A is used to predict the RPS of a picture B, most likely the picture A will be added into the RPS of the picture B.

In most cases, used_by_curr_pic_flag can be predicted from the flags associated with the reference RPS. It is less efficient and not clean to still signal this flag for inter-RPS prediction.

| short_term_ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| inter_ref_pic_set_prediction_flag | u(1) |
| if( inter_ref_pic_set_prediction_flag ) { |  |
| delta_idx_minus1 | ue(v) |
| delta_rps_sign | u(1) |
| abs_delta_rps_minus1 | ue(v) |
| for( j = 0; j <= NumDeltaPocs[ RIdx ]; j++ ) { |  |
| used_by_curr_pic_flag[ j ] | u(1) |
| if( !used_by_curr_pic_flag[ j ] ) |  |
| use_delta_flag[ j ] | u(1) |
| } |  |
| } |  |
| else { |  |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

# Proposal

## Inter-RPS simplification

In this document, the following modifications to the inter-RPS prediction are:

- delta_rps_sign and abs_delta_rps_minus1 are merged into one syntax element;

- used_by_curr_pic_flag for each reference picture in the inter-predicted RPS is not signalled and always derived to be the same value as that flag of the corresponding picture in the reference RPS;

- an inter-predicted RPS can include not only the pictures in a reference RPS, but also the current picture of a reference RPS.

- an inter-predicted RPS can choose to a certain number of pictures from the reference RPS that have the smallest delta POC value, and a certain number of pictures from the reference RPS that have the smallest delta POC value.

### Short-term reference picture set syntax

| short_term_ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| inter_ref_pic_set_prediction_flag | u(1) |
| if( inter_ref_pic_set_prediction_flag ) { |  |
| delta_idx_minus1 | ue(v) |
| delta_rps_sign | u(1) |
| abs_delta_rps_minus1 | ue(v) |
| delta_rps_plus1 | se(v) |
| for( j = 0; j <= NumDeltaPocs[ RIdx ]; j++ ) { |  |
| used_by_curr_pic_flag[ j ] | u(1) |
| if( !used_by_curr_pic_flag[ j ] ) |  |
| use_delta_flag[ j ] | u(1) |
| } |  |
| add_predictor_rps_pic_flag | u(1) |
| num_ref_pics_drops_from_begin | ue(v) |
| num_ref_pics_drops_from_end | ue(v) |
| } |  |
| else { |  |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

### Short-term reference picture set semantics

delta_rps_plus1 minus 1 specifies the POC difference between picA and picB, where picA is associated with DeltaPoc[ RIdx ] and picB is associated with DeltaPoc[ idx ]. Note picA is called the current picture of DeltaPoc[ RIdx ] and picB is called the current picture of DeltaPoc[ idx ].

The variable DeltaRPS is derived as follows.

DeltaRPS = delta_rps_plus1 - 1

add_predictor_rps_pic_flag equal to 1 indicates that the previous picture in decoding order using the reference RPS (with index equal to Ridx = idx - delta_idx_minus1 - 1) is included in the RPS with index equal to idx. add_predictor_rps_pic_flag equal to 0 indicates the previous picture in decoding order of the reference RPS (with index equal to RIdx ) is not included in the RPS with index equal to idx.

num_ref_pics_drops_from_begin specifies the number of reference pictures starting from the beginning of the reference RPS (with index equal to RIdx) that are not used to predict the current RPS (with index equal to idx). Let RNPP and RNNP be  num_positive_pics and num_negative_pics of the reference RPS, respectively. The value of num_ref_pics_drops_from_begin shall be in the range of 0 to RNPP + RNNP -- 1, inclusive.

num_ref_pics_drops_from_end specifies the number of reference pictures starting from the end of the reference RPS (with index equal to RIdx) that are not used to predict the current RPS (with index equal to idx). num_ref_pics_drops_from_end is in the range of 0 to RNPP + RNNP – 1 - num_ref_pics_drops_from_begin, inclusive.

Note that it is assumed that the reference RPS (short-term) has been sorted in a way that all the entries are ordered in the ascending order of POC values.

### Decoding processes

When inter_ref_pic_set_prediction_flag is equal to 1, the variables DeltaPocS0[ idx ][ i0 ], UsedByCurrPicS0[ idx ][ i0 ], DeltaPocS1[ idx ][ i1 ], UsedByCurrPicS1[ idx ][ i1 ], NumNegativePics[ idx ] and NumPositivePics[ idx ] are derived as follows.

for( j = num_ref_pics_drops_from_begin, i0 = 0, i1 = 0, insertFlag = add_predictor_rps_pic_flag; j <= NumDeltaPocs[ RIdx ]- num_ref_pics_drops_from_end; j++ )
{
	if  ((DeltaPocSorted[ RIdx ][ j ]>DeltaRPS | | 
		j= = NumDeltaPocs[ RIdx ]- num_ref_pics_drops_from_end)  && insert_flag) {
		DPoc = DeltaRPS
		currFlag = 1
		insertFlag = 0
		if (j< NumDeltaPocs[ RIdx ]) j--	
	}
	else {
		DPoc = DeltaPocSorted[ RIdx ][ j ] + DeltaRPS
		currFlag = UsedByCurrPic[ Ridx ][ j ]
	}
	if( DPoc < 0 ) {
		DeltaPocS0[ idx ][ i0 ] = DPoc
		UsedByCurrPicS0[ idx ][ i0 ] = currFlag
		i0++
	} else {
		DeltaPocS1[ idx ][ i1  ] = DPoc
		UsedByCurrPicS1[ idx ][ i1 ] = currFlag
		i1++
	} 
}
NumNegativePics[ idx ] = i0

NumPositivePics[ idx ] = i1

Regardless the value of inter_ref_pic_set_prediction_flag, the following process is used to derive a sorted list of delta POC values and the associated used_by_curr_pic_sx_flag values of the entries in the RPS. The arrays DeltaPocSorted[ idx ] and UsedByCurrPic[ idx ][ ] are derived as follows.

for(j = 0, i = NumNegativePics[ idx ] - 1; i >= 0; i--, j++) {
	DeltaPocSorted[ idx ][ j ] = DeltaPocS0[ idx ][ i ]
	UsedByCurrPic[ idx ][ j ] = UsedByCurrPicS0[ idx ][ i ]
}
for(j = NumNegativePics[ idx ], i = NumNegativePics[ idx ] ; i < NumPositivePics[ idx ] + NumNegativePics[ idx ]; i++, j++) {
	DeltaPocSorted[ idx ][ j ] = DeltaPocS1[ idx ][ i ]
	UsedByCurrPic[ idx ][ j ] = UsedByCurrPicS1[ idx ][ i ]
}

## Profile for inter-RPS

It is identified that the current design for inter-RPS prediction in HEVC WD7 is complicated. Therefore it is proposed to disable inter-RPS prediction in the Main profile unless it is simplified.

# Results

We present the results obtained using the proposed method for random access (RA) and low-delay (LD) common test conditions. Table 1 and Table 2 provide a comparison of the number of bits signalled in the SPS for short-term RPS. Compared to the inter-RPS prediction structure in HEVC WD7, the simplified method uses 4% more bits for the RA case, and 27% more bits for the LD case. These increases are minimal, and only translate to actual bit increase of 5 bits and 36 bits for the LD and RA cases, respectively for each sequence parameter set.

Table 1: Comparison of number of bits required to signal the short-term RPS candidates in the SPS in the RA common test condition

| RPS ID | POC | GOP size = 8 | HM6.1 |  | HM6.1 with simplified inter-RPS |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | pred | Bits | pred | Bits |
| 0 | 8n+8 | -8 -10 -12 -16 | 0 | 29 | 0 | 29 |
| 1 | 8n+4 | -4 -6 4 | 1 | 15 | 1 | 14 |
| 2 | 8n+2 | -2 -4 2 6 | 1 | 10 | 1 | 10 |
| 3 | 8n+1 | -1 1 3 7 | 1 | 10 | 1 | 12 |
| 4 | 8n+3 | -1 -3 1 5 | 1 | 12 | 1 | 8 |
| 5 | 8n+6 | -2 -4 -6 2 | 1 | 12 | 1 | 10 |
| 6 | 8n+5 | -1 -5 1 3 | 1 | 10 | 0 | 21 |
| 7 | 8n+7 | -1 -3 -7 1 | 1 | 12 | 1 | 8 |
| 8 | 8 | -8 | 0 | 13 | 0 | 13 |
| 9 | 4 | -4 4 | 1 | 10 | 1 | 12 |
| 10 | 2 | -2 2 6 | 1 | 9 | 1 | 10 |
|  |  | Total STRPS bits |  | 142 |  | 147 |
|  |  | % increase |  |  |  | 3.52% |

Table 2: Comparison of number of bits required to signal the short-term RPS candidates in the SPS in the LD common test condition

| RPS ID | POC | GOP size = 4 | HM6.1 |  | HM6.1 with simplified inter-RPS |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | pred | Bits | pred | Bits |
| 0 | 4n+1 | -1 -5 -9 -13 | 0 | 27 | 0 | 27 |
| 1 | 4n+2 | -1 -2 -6 -10 | 1 | 10 | 1 | 8 |
| 2 | 4n+3 | -1 -3 -7-11 | 1 | 10 | 1 | 8 |
| 3 | 4n+4 | -1 -4 -8 -12 | 1 | 10 | 1 | 8 |
| 4 | 1 | -1 | 0 | 7 | 0 | 7 |
| 5 | 2 | -1 -2 | 1 | 6 | 1 | 6 |
| 6 | 3 | -1 -2 -3 | 1 | 7 | 1 | 6 |
| 7 | 4 | -1 -2 -3 -4 | 1 | 8 | 1 | 6 |
| 8 | 5 | -1 -2 -3 -5 | 0 | 10 | 0 | 17 |
| 9 | 6 | -1 -2 -3 -6 | 0 | 10 | 0 | 17 |
| 10 | 7 | -1 -2 -3 -7 | 0 | 10 | 0 | 19 |
| 11 | 8 | -1 -2 -4 -8 | 0 | 10 | 0 | 21 |
| 12 | 9 | -1 -2 -5 -9 | 0 | 10 | 0 | 21 |
|  |  | Total STRPS bits |  | 135 |  | 171 |
|  |  | % increase |  |  |  | 26.67% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).