| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0077r2 WG11 Number: m27405 |
| --- | --- |

| Title: | Additional VUI and SEI for chroma sampling filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan Kimihiko Kazui 4-1-1 Kamikodanaka, Nakahara-ku, Kawasaki, 211-8588 Japan Pankaj Topiwala, Daisy Liu Madhu Krihnan, 750 N. Atlantic Ave., Cocoa Beach, FL 32931 | Tel: Email: Tel: Email: Tel: Email: | +81-44-549-2152 takeshi.chujoh@toshiba.co.jp +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com +1-410-730-6922 pankajtva@gmail.com |
| Source: | TOSHIBA Corporation, FUJITSU LABORATORIES LTD and FastVDO LLC. |  |  |

_____________________________

- Abstract

Since chroma formats that are called 4:2:0, 4:2:2 and 4:4:4 are used at the video coding technology, some kinds of chroma format conversions are needed when output video is translated or displayed. Although the sample location of chroma format can be specified at VUI in current draft specification, there is no information regarding chroma sampling filter. This contribution provides a framework to send recommended up-sampling and down-sampling filter coefficients for chroma format conversion by using VUI and SEI message. Especially, it is possible to solve a problem of error accumulation when chroma conversions are repeated.

Introduction

Since chroma formats that are called 4:2:0, 4:2:2 and 4:4:4 are used at the video coding technology, some kinds of chroma format conversions are needed when output video is translated or displayed. Although the sample location of chroma format can be specified at VUI in current draft specification [1], there is no information regarding chroma sampling filter. Therefore, some losses of chroma format conversion are caused. Especially, it is known that there is a problem of error accumulation when the chroma format is converted repeatedly.

For example, when a 4:2:0 video decoder and encoder are connected via a 4:2:2 serial interface such as SMPTE ST 292-1, the 4:2:0 data is decoded and converted to 4:2:2 by up-sampling the color difference component. In the 4:2:0 video encoder, the 4:2:2 video data is converted to 4:2:0 video data by down-sampling the color difference component. In this case, there typically exists a color difference mismatch between the 4:2:0 video data from the decoder and the 4:2:0 video data to be encoded.

Several stages of codec concatenation are common through the video processing chain. As a result, color difference signal mismatch between 4:2:0 video data input to 4:2:0 video encoder and 4:2:0 video output from 4:2:0 video decoder is accumulated and the degradation becomes visible.

SMPTE RP 2050-1:2012 [2] and EG 2050-2:2012 [3] define filter coefficient sets of 4:2:2/4:2:0 and 4:2:0/4:2:2 inter-format conversion to minimize degradation caused by concatenation. This is achieved by making the 4:2:2/4:2:0 inter-format filter sets satisfy complementing reconstruction filters. The use of these filter coefficients preserves the color difference signal resolution of the first 4:2:0 sub-sampling.

The property of chroma sampling filter is reported in [4]. Below figures showing are excerpted from [4]. In this figure, “Non-Degrade 4:2:0 filter Set” means the chroma sampling filter of SMPTE RP 2050-1:2012, and “Reference Filter Set” means the chroma sampling filter in MPEG-2 TM5 [5].

Regarding design problem for chroma sampling filter, there are another discussions. In JCTVC-K0211[6], several filters are designed to minimize loss in a single 4:2:2/4:2:0/4:2:2 (or 4:4:4/4:2:0/4:4:4) conversion.

In this contribution, considering comments on JCTVC-K0152 [7] at the Shanghai meeting, we would like to propose a specification of additional VUI and SEI message for chroma sampling filter generally due to reduce conversion losses of chroma format.

| Test sequence | PSNR degradation |
| --- | --- |
| Original sequence “Character” 1,920x1,080/60i |  |
| VQEG test sequence “Mobile & Calendar” 720x576/50i |  |
| VCEG test sequence “F1 Car” 720x576/50i |  |

Figure 1	Example of PSNR degradation suppression in chroma component

Chroma sampling filter hint SEI

Syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| else if( payloadType = = XXX) |  |
| chroma_sampling_filter_hint( payloadSize) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( !byte_aligned( ) ) { |  |
| bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

Semantics

Table D-1 – Persistence scope of prefix SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| buffering period | The remainder of the bitstream |
| … | … |
| chroma sampling filter hint | The remainder of the bitstream |

- D.1.X	Chroma sampling filter hint SEI syntax

| chroma_sampling_filter_hint( payloadSize ) { | Descriptor |
| --- | --- |
| target_format_idc | ue(v) |
| perfect_reconstruction_flag | u(1) |
| if( ver_chroma_filter_idc == 1 ) { |  |
| num_vertical_filters | ue(v) |
| NumVertFilters = num_vertical_filters |  |
| if( NumVertFilters > 0 ) |  |
| for( i=0; i < NumVertFilters; i++) { |  |
| vert_tap_length_minus1[ i ] | ue(v) |
| for( j=0; j <= vert_tap_length_minus1[ i ]; j++) { |  |
| vert_filter_coeff[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| if( hor_chroma_filter_idc == 1 ) { |  |
| num_horizontal_filters | ue(v) |
| NumHorFilters = num_horizontal_filters |  |
| if( NumHorFilters > 0 ) |  |
| for( i=0; i < NumHorFilters; i++) { |  |
| hor_tap_length_minus1[ i ] | ue(v) |
| for( j=0; j <= hor_tap_length_minus1[ i ]; j++) { |  |
| hor_filter_coeff[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

- D.2.X	Chroma sampling filter hint SEI semantics

The chroma sampling filter hint SEI message provides the recommended coefficients of a chroma sampling filter. If the coefficients of chroma sampling filter both at an encoder and a decoder are designed based on the coefficients signalled in the chroma sampling filter hint SEI message, the color difference degradation wiil be expected to be minimized.

target_format_idc specifies the output of chroma sampling relative to that of the luma sampling as specified in subclause 6.2. The value of target_format_idc shall be in the range of 1 to 3, inclusive.

perfect_reconstruction_flag equal to 1 specifies that the chroma sampling filter coefficients satisfy perfect reconstruction condition which mean that up-sampled results are identical when down-sampling and up-sampling are repeated. perfect_reconstruction_flag equal to 0 indicates that the chroma sampling filter coefficients may not satisfy perfect reconstruction condition.

num_vertical_filters specifies the number of filters used for chroma sampling in vertical direction.

vert_tap_length_minus1[] + 1 specifies the number of tap length of a filter in vertical direction. The value of vert_tap_length_minus1 shall be in the range of 0 to 31, inclusive.

vert_filter_coeff[][] specifies the value of a filter coefficient in vertical direction. The value of vert_filter_coeff[i][] shall be in the range of −231 + 1 to 231 − 1, inclusive.

FV[DistinctParityFlag][i] is set to vert_filter_coeff[i][].

num_horizontal_filters specifies the number of filters used for chroma sampling in horizontal direction.

hor_tap_length_minus1[] + 1 specifies the number of tap length of a filter in horizontal direction. The value of hor_tap_length_minus1 shall be in the range of 0 to 31, inclusive.

hor_filter_coeff[][] specifies the value of a filter coefficient in horizontal direction. The value of hor_filter_coeff[][] shall be in the range of −231 + 1 to 231 − 1, inclusive.

FH[0][i] is set to hor_filter_coeff[i][].

Additional VUI parameters

VUI parameters syntax

| vui_parameters( ) { | Descriptor |
| --- | --- |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| chroma_filter_info_present_flag | u(1) |
| if( chroma_filter_info_present_flag) { |  |
| ver_chroma_filter_idc | u(8) |
| hor_chroma_filter_idc | u(8) |
| ver_filtering_process_flag | u(1) |
| } |  |
| } |  |
| } |  |

VUI parameters semantics

chroma_loc_info_present_flag equal to 1 specifies that chroma_sample_loc_type_top_field, chroma_sample_loc_type_bottom_field, chroma_filter_infor_present_flag and chroma_filter_info are present. chroma_loc_info_present_flag equal to 0 specifies that chroma_sample_loc_type_top_field, chroma_sample_loc_type_bottom_field, chroma_filter_infor_present_flag and chroma_filter_info are not present.

When chroma_format_idc is not equal to 1, chroma_loc_info_present_flag should be equal to 0.

chroma_filter_info_present_flag equal to 1 specifies that chroma_filter_info is present. If chroma_filter_info_present_flag is equal to 0, chroma_filter_info is not present in the bitstream and is inferred as 0.

ver_chroma_filter_idc specified the preferred chroma sampling filter in vertical direction for the output decoded pictures and for input pictures at an encoder which takes the output decoded pictures as an input.

When ver_chroma_filter_idc is equal to 1, the coefficients of chroma sampling filter in vertical direction are signalled in the chroma sampling filter hint SEI. When vert_chroma_filter_idx is not equal to 0, the coefficients of chroma sampling filter are specified in Table E-X1.

hor_chroma_filter_idc specified the preferable chroma sampling filter in horizontal direction for the output decoded pictures and possibly for input pictures at an encoder which takes the output decoded pictures as an input.

When hor_chroma_filter_idc is equal to 1, the coefficients of chroma sampling filter in horizontal direction are signalled in the chroma sampling filter hint SEI. When hor_chroma_filter_idx is not equal to 0, the coefficients of chroma sampling filter are specified in Table E-X2.

ver_filtering_process_flag specified how chroma sampling filter in vertical direction is performed. If ver_filtering_process_flag is equal to 1, chroma sampling filter is applied in field-basis (i.e. only chroma samples of the same parity is used for filtering). Otherwise, chroma sampling filter is applied in frame-basis.

Variable DistinctParityFlag is derived as follows:

DistinctParityFlag = ( (vert_filtering_process_flag == 1 ) ? 1: 0

Table E-X1 speficies the coefficients of chroma sampling filter in vertical direction. When vert_chroma_filter_idc is greater than 1, the usage of filter coefficients FV[][]is specified in Table E-X1_1.

Table E-X1	Chroma filter index in vertical direction

| Value | Vertical chroma sampling filter | Informative Remark |
| --- | --- | --- |
| 0 | Unspecified | Chroma filter is unknown or is determined by the application. |
| 1 | User-defined | Filter coefficients are specified in the chroma sampling filter hint SEI mesasge |
| 2 | FV[0][0] = {-3, -19, 34, 500, 500, 34, -19. 3} FV[0][1] = {19, 103, 1037, -135} FV[1][0] = {-8, -26, 115, 586, 409, -48, -4, 0} FV[1][1] = {24, -41, 1169, -128} FV[1][2] = {-76, 783, 330, -13} | SMPTE RP 2050-1:2012 Both chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field shall be equal to 0. |
| 3 | FV[0][0] ={1,-3,10,16,10,-1,1} FV[0][1] ={-1,5,5,-1} | If filter index 3 is used, Chroma sample type should be 2,3,4,5 and DistinctParityFlag=0 |
| 4 … 255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

Table E-X1_1	Usage of filter coefficients in vertical direction

| Chroma sample type | DistinctParityFlag | NumVertFilters | Usage |
| --- | --- | --- | --- |
| 0 or 1 | 0 | 3 | FV[0][0] is used for down-sampling of every line. FV[0][1] is used for up-sampling of every even line FV[0][2] is used for up-sampling of every odd line |
|  | 1 | 3 | FV[1][0] is used for down-sampling of every line in top-field. Flipped FV[1][0] is used for down-sampling of every line in bottom-field. FV[1][1] is used for up-sampling of every even line in top-field. FV[1][2] is used for up-sampling of every odd line in top-field. Flipped FV[1][2] is used for up-sampling of every even line in bottom-field. Flipped FV[1][1] is used for up-sampling of every odd line in bottom-field. |
| 2 or 3 | 0 | 3 | FV[0][0] is used for down-sampling of every line. FV[0][1] is used for up-sampling of every even line. FV[0][2] is used for up-sampling of every odd line. |
|  | 1 | 5 | FV[1][0] is used for down-sampling of every line in top-field. FV[1][1] is used for down-sampling of every line in bottom-field. FV[1][2] is used for up-sampling of every even line in top-field. FV[1][3] is used for up-sampling of every odd line in top-field. FV[1][4] is used for up-sampling of every even line in bottom-field. Flipped FV[1][4] is used for up-sampling of every odd line in bottom-field. |
| 4 or 5 | 0 | 3 | FV[0][0] is used for down-sampling of every line. FV[0][1] is used for up-sampling of every even line. FV[0][2] is used for up-sampling of every odd line. |
|  | 1 | 5 | FV[1][0] is used for down-sampling of every line in top-field. FV[1][1] is used for down-sampling of every line in bottom-field. FV[1][2] is used for up-sampling of every even line in top-field. Flipped FV[1][2] is used for up-sampling of every odd line in top-field. FV[1][3] is used for up-sampling of every even line in bottom-field. FV[1][4] is used for up-sampling of every odd line in bottom-field. |

Figure E-X1_2	(Informative) Design principle of vertical up/down-sampling filter for interlaced scan.
The number of filters is according to the variations of positional relationship between sampled pixels and a generated pixel.

Table E-X2 speficies the coefficients of chroma sampling filter in horizontal direction. When hor_chroma_filter_idc is greater than 1, the usage of filter coefficients FH[][]is specified in Table E-X2_1.

Table E-X2	Chroma filter index in horizontal direction

| Value | Vertical chroma sampling filter | Informative Remark |
| --- | --- | --- |
| 0 | Unspecified | Chroma filter is unknown or is determined by the application. |
| 1 | User-defined | Filter coefficients are specified in the chroma sampling filter hint SEI mesasge |
| 2 | FH[0][0] = {-1, 2, 6, 2, -1} FH[0][1] = {1} FH[0][2] = {1, 1} | ITU-T Rec. T.800 ¦ ISO/IEC15444-1, 5/3 filter |
| 3 | FH[0][0] = {1, 0, -3, 0, 10, 16, 10,0, -3, 0, 1} FH[0][1] = {1} FH[0][2] = {-1, 5, 5, -1} | If filter index 3 is used, Chroma sample type should be 0, 2 or 4. Only. |
| 4 … 255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

Table E-X2_1	Usage of filter coefficients in horizontal direction

| Chroma sample type | NumHorFilters | Usage |
| --- | --- | --- |
| 0 or 2 or 4 | 3 | FH[0][0] is used for down-sampling of every column. FH[0][1] is used for up-sampling of every even column FH[0][2] is used for up-sampling of every odd column |
| 1 or 3 or 5 | 2 | FH[0][0] is used for down-sampling of every column. FH[0][1] is used for up-sampling of every even column Flipped FH[0][1] is used for up-sampling of every odd column |

1

1

1

1

1

1

Down

Up

E

O

1

3

1

3

Down

Up

E

O

1

1

1

1

1

1

Down

Up

E

O

1

1

Chroma sample type

=

2

or

3

Chroma sample type

=

4

or

5

Legend

Up

-

sampled chroma position

Down

-

sampled chroma position

a

Even

-

tap filter

with vertical shift a

/(

a

+

b

)

a

Odd

-

tap filter

b

b

Chroma sample type

=

0

or

1

‘

a

’

and

‘

b

’

are relative vertical

distances from target pixel

E

:

Even line

,

O

:

Odd line

Figure E-X2_2	(Informative) Design principle of vertical up/down-sampling filter for interlaced scan.

1

1

1

1

1

1

Down

Up

E

O

3

1

Down

Up

E

O

1

1

3

1

Chroma sample type

=

0

,

2

or

4

Chroma sample type

=

1

,

3

or

5

Legend

Up

-

sampled chroma position

Down

-

sampled chroma position

a

Even

-

tap filter

with horizotal shift a

/(

a

+

b

)

a

Odd

-

tap filter

b

b

‘

a

’

and

‘

b

’

are relative horizontal

distances from target pixel

E

:

Even column

,

O

:

Odd column

Figure E-X2_3	(Informative) Design principle of horizontal up/down-sampling filter.

NOTE1 – SMPTE EG2050-2:2012 provides details on a possible implementation of the filter defined in SMPTE RP2050-1:2012.

References

[1] 	B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 9,” Joint Collaborative Team on Video Coding, JCTVC-K1003, Shanghai, Oct. 2012.

[2] 	“4:2:2 / 4:2:0 Format Conversion Minimizing Color Difference Signal Degradation in Concatenated Operations – Filtering,” SMPTE RP2050-1:2012, Jan. 2012.

[3] 	“4:2:2 / 4:2:0 Format Conversion Minimizing Color Difference Signal Degradation in Concatenated Operations – Application,” SMPTE EG2050-2:2012, Jan. 2012.

[4] 	A. Nakagawa, “The 4:2:2/4:2:0 Perfect Reconstruction Filter Set and Its Application in HD-SNG, ” SMPTE Motion Imaging Journal, vol.118, no.5, pp. 29-39, July/August 2009.

[5] 	“Test Model 5,” ISO/IEC JCT1/SC29/WG11 MPEG93/N0400, ITU-T Q.2/15 AVC-491b, April 1993.

[6] 	W. Dai, M. Krhnan and P. Topiwala, “AHG7: Colour spaces and chroma sampling methods for higher chroma format coding,” Joint Collaborative Team on Video Coding, JCTVC-K0211, Shanghai, Oct. 2012.

[7] 	T. Chujoh, “AHG7: Chroma sampling filter hint SEI,” Joint Collaborative Team on Video Coding, JCTVC-K0152, Shanghai, Oct. 2012.

Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

FUJITSU LIMITED may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

FastVDO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The content written in the attached PFD document is equivalent to [4].