| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 37th Meeting: Geneva, CH, 4–10 October 2019 | Document: JCTVC-AK0021-v2 |
| --- | --- |

| Title: | Some HEVC errata items |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang San Diego, CA 92121, USA | Tel: Email: | +1-908-903-3888 ye-kui.wang@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

This contribution reports some text bugs in the latest HEVC specification text and proposes corresponding fixes.

# Reported errata items for HEVC

## On semantics of nal_hrd_parameters_present_flag and vcl_hrd_parameters_present_flag

Change the semantics of nal_hrd_parameters_present_flag and vcl_hrd_parameters_present_flag as follows (additions are yellow-highlighted, removals are strikethrough in red fonts):

nal_hrd_parameters_present_flag equal to 1 specifies that NAL HRD parameters (pertaining to Type II bitstream conformance) are present in the hrd_parameters( ) syntax structure. nal_hrd_parameters_present_flag equal to 0 specifies that NAL HRD parameters are not present in the hrd_parameters( ) syntax structure.

NOTE 1 – When nal_hrd_parameters_present_flag is equal to 0, the Type II conformance of the bitstream cannot be verified without provision of the NAL HRD parameters and all buffering period SEI messages, and, when vcl_hrd_parameters_present_flag is also equal to 0, all the picture timing and decoding unit informatoin SEI messages, by some means not specified in this Specification.

vcl_hrd_parameters_present_flag equal to 1 specifies that VCL HRD parameters (pertaining to Type I all bitstream conformance) are present in the hrd_parameters( ) syntax structure. vcl_hrd_parameters_present_flag equal to 0 specifies that VCL HRD parameters are not present in the hrd_parameters( ) syntax structure.

NOTE 2 – When vcl_hrd_parameters_present_flag is equal to 0, the Type I conformance of the bitstream cannot be verified without provision of the NAL HRD parameters and all buffering period SEI messages, and, when nal_hrd_parameters_present_flag is also equal to 0, all the picture timing and decoding unit informatoin SEI messages, by some means not specified in this Specification.

## On setting of HighestTid to sps_max_sub_layers_minus1

The wording (two instances) of "HighestTid is set equal to sps_max_sub_layers_minus1" in clauses 8.1.2 and F.8.1.2 is not clear, as there may be multiple layers referring to multiple SPSs and consequently there may be multiple instances of sps_max_sub_layers_minus1 involved. Which one should be chosen?

Suggestion: To either replace " sps_max_sub_layers_minus1" with "vps_max_sub_layers_minus1" or clarify that the sps_max_sub_layers_minus1 value used is the greatest value of sps_max_sub_layers_minus1 among the referenced SPSs.

## On HRD text

- In C.3.3, change the following:

Otherwise (PicOutputFlag is equal to 1 and DpbOutputTime[ n ] is greater than AuCpbRemovalTime[ n ] ), the current picture is output later and will be stored in the DPB (as specified in clause ‎C.3.4) and is output at time DpbOutputTime[ n ] unless indicated not to be output by the decoding or inference of no_output_of_prior_pics_flag equal to 1 at a time that precedes DpbOutputTime[ n ].

to

Otherwise (PicOutputFlag is equal to 1 and DpbOutputTime[ n ] is greater than AuCpbRemovalTime[ n ] ), the current picture is output later and will be stored in the DPB (as specified in clause ‎C.3.4) and is output at time DpbOutputTime[ n ] unless indicated not to be output by NoOutputOfPriorPicsFlag equal to 1.

- In F.13.3.3, change the following:

Otherwise (PicOutputFlag is equal to 1 and DpbOutputTime[ n ] is greater than AuCpbRemovalTime[ n ] ), the current picture is output later and will be stored in the DPB (as specified in clause ‎F.13.3.4) and is output at time DpbOutputTime[ n ] unless indicated not to be output by the decoding or inference of NoOutputOfPriorPicsFlag equal to 1 at a time that precedes DpbOutputTime[ n ].

to

Otherwise (PicOutputFlag is equal to 1 and DpbOutputTime[ n ] is greater than AuCpbRemovalTime[ n ] ), the current picture is output later and will be stored in the DPB (as specified in clause ‎F.13.3.4) and is output at time DpbOutputTime[ n ] unless indicated not to be output by NoOutputOfPriorPicsFlag equal to 1.

- In C.5.1, change the following:

A decoder claiming conformance to a specific profile, tier and level shall be able to successfully decode all bitstreams that conform to the bitstream conformance requirements specified in clause ‎C.4, in the manner specified in Annex ‎A, provided that all VPSs, SPSs and PPSs referred to in the VCL NAL units and appropriate buffering period and picture timing SEI messages are conveyed to the decoder, in a timely manner, either in the bitstream (by non-VCL NAL units), or by external means not specified in this Specification.

to

A decoder claiming conformance to a specific profile, tier and level shall be able to successfully decode all bitstreams that conform to the bitstream conformance requirements specified in clause ‎C.4, in the manner specified in Annex ‎A, provided that all VPSs, SPSs and PPSs referred to in the VCL NAL units and appropriate buffering period, picture timing, and decoding unit information SEI messages are conveyed to the decoder, in a timely manner, either in the bitstream (by non-VCL NAL units), or by external means not specified in this Specification.

## On SingleLayerSeiList

In D.3.1, there is a discrepancy (of the SEI payloadType 2, i.e., the pan-scan rectangle SEI message) between the definition of the variable SingleLayerSeiList and the subsequent NOTE providing an explanation of SingleLayerSeiList, copied and pasted below:

The list SingleLayerSeiList is set to consist of the payloadType values 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 134 to 151, inclusive, 154 to 159, and 200 to 201, inclusive.

NOTE 3 – SingleLayerSeiList consists of the payloadType values of the SEI messages specified in Annex D excluding 0 (buffering period), 1 (picture timing), 4 (user data registered by Recommendation ITU-T T.35), 5 (user data unregistered), 130 (decoding unit information) and 133 (scalable nesting).

Due to that this list is mainly used to constrain the value of bitstream_subset_flag of the scalable nesting SEI message, and that the value 2 is the pan-scan rectangle SEI message, I think the NOTE is correct, i.e., the value 2 should be added to the definition. This can be confirmed by checking the editing history, e.g., by taking a look at JCTVC-R1008-v1 of the Sapporo meeting in July 2014.

# Patent rights declaration(s)

Futurewei Technologies, Inc. does not have any current or pending patent rights relating to the technology described in this contribution (to the extent of the personal awareness of the contributor).