| Question(s): | 2 |  | Meeting, date: |  | 19 - 30 July 2010 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 2 | Intended type of document (R-C-TD): |  | C |
| Source: | Australia. |  |  |  |  |  |
| Title: | Proposed amendment to H.460.23 NAT/FW determination |  |  |  |  |  |
| Contact: | Simon Horne Spranto Australia Pty Ltd. Australia |  |  |  | Tel: +61 412 984 323 Fax: Email: s.horne@spranto.com.au |  |
| Contact: |  |  |  |  | Tel: Fax: Email: |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

Summary

This contribution proposes an extension of the existing ITU-T H.460.23/.24 recommendation to enable direct media flows between distinct restricted and port restricted cone type NAT/FW devices.

# 1.	Introduction

The existing H.460.23 document provides a method to advertise during registration that the endpoint supports probing for same NAT (H.460.24 Annex A). This contribution seeks to extend this is to also advertise Cone NAT probing (proposed new H.460.24 Annex B).

Contribution requests the following alteration to Clause 8.

# 8	NAT probe support

If the endpoint supports probing for a direct route when behind the same NAT/FW device (supports ITU-T H.460.24 Annex A), then this shall be indicated by the SameNATProbe parameter.

Table 3a – Same NATProbe parameter

| Parameter name: | SameNATProbe |
| --- | --- |
| Parameter description: | Indicates whether the endpoint supports probing for Same NAT (ITU-T H.460.24 Annex A). |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 2 |
| Parameter type: | bool |
| Parameter cardinality: | One and only one |

If the endpoint supports probing Cone NAT for a direct route (supports ITU-T H.460.24 Annex B), then this shall be indicated by the ConeNATProbe parameter.

Table 3b – ConeNATProbe parameter

| Parameter name: | ConeNATProbe |
| --- | --- |
| Parameter description: | Indicates whether the endpoint supports Cone NAT probing (ITU-T H.460.24 Annex B). |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 7 |
| Parameter type: | bool |
| Parameter cardinality: | One and only one |

__________________