| Question(s): | 2 |  | Meeting, date: |  | 19 - 30 July 2010 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 2 | Intended type of document (R-C-TD): |  | C |
| Source: | Australia. |  |  |  |  |  |
| Title: | Proposed amendment to H.460.24 Point to Point media through NAT/FW |  |  |  |  |  |
| Contact: | Simon Horne Spranto Australia Pty Ltd. Australia |  |  |  | Tel: +61 412 984 323 Fax: Email: s.horne@spranto.com.au |  |
| Contact: |  |  |  |  | Tel: Fax: Email: |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

Summary

This contribution proposes an extension of the existing ITU-T H.460.23/.24 recommendation to enable direct media flows between distinct restricted and port restricted cone type NAT/FW devices.

# 1.	Introduction

The current H.460.24 recommendation puts forward methodology to enable devices behind distinct NAT/FW devices to stream media between each other without the need to proxy media via an intermediary device. There are however conditions under the recommendation where media would need to be proxied. To further reduce these conditions and improve scalability it is proposed to include an Annex B to allow endpoints residing behind certain types of NAT/FW devices that otherwise would need to be proxied to be able to probe for a direct route.

This contribution proposes to

Amend Clauses 7.3, 8, 9.4 and 9.5,

Amend Tables 8 and 10 and,

Include new Annex B.

Proposed changes clause 7.3

Optionally, the endpoint may support Annex A of this Recommendation. This shall be notified by the SameNATProbe parameter (refer to Table 8a).

Table 8a – SameNATProbe parameter

| Parameter name: | SameNATProbe |
| --- | --- |
| Parameter description: | Indicates whether the endpoint supports probing for the same NAT (supports ITU-T H.460.24 Annex A). |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 7 |
| Parameter type: | bool |
| Parameter cardinality: | One and only one |

Optionally, the endpoint may support Annex B of this Recommendation. This shall be notified by the ConeNATProbe parameter (refer to Table 8b)

Table 8b – ConeNATProbe parameter

| Parameter name: | ConeNATProbe |
| --- | --- |
| Parameter description: | Indicates whether the endpoint supports Cone NAT probing (supports ITU-T H.460.24 Annex B). |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 9 |
| Parameter type: | bool |
| Parameter cardinality: | One and only one |

Proposed changes clause 8

Same NAT/FW: This indicates that the two endpoints reside behind the same NAT/FW device. Since it is impossible to detect the network topography behind the NAT/FW device, media shall be proxied. However the devices are to probe a direct media pathway on their own and, if successful, select to send media directly (this requires Annex A support).

ConeNAT Probe: This indicates that the two endpoints reside behind distinct NAT/FW devices and both devices and gatekeeper support Annex B.  From media strategy calculations, it was determined that media must be initially proxied and once media is established the two devices may probe for a direct route via the procedure detailed in Annex B.

NAT media failure: This indicates that there is no resolvable method to stream media between the endpoints and the call will fail to establish bidirectional media flows.

Table 9 – Media strategy indicator

| Value | Strategy |
| --- | --- |
| 0 | Unknown |
| 1 | No assistance |
| 2 | Local media master |
| 3 | Remote media master |
| 4 | Local proxy |
| 5 | Remote proxy |
| 6 | Full proxy |
| 7 | Same NAT/FW (Annex A) |
| 8 | Cone NAT Probe (Annex B) |
| 100 | NAT media failure |

Proposed changes clause 9.4

## 9.4	General NAT strategy determination

Table 10 contains an overview of the general strategy to be employed given the NAT types that are returned from the ITU-T H.460.23 test.

Table 10 – NAT strategy calculation matrix
(for values, refer to Table 9)

|  | Local type (Table 8 of [ITU-T H.460.23]) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Remote type |  | 0 | 1 | 2 | 3 | 4 | 5 |
|  | 0 | 0 | 1 | 2 | 4 | 4 | 4 |
|  | 1 | 1 | 1 | 3 | 3 | 3 | 3 |
|  | 2 | 3 | 2 | 2 | 3 | 3 | 3 |
|  | 3 |  | 5 |  |  | 2 | 2 |  | 8 |  |  | 8 | 8 |  |
|  | 4 |  | 5 |  |  | 2 | 2 |  | 8 |  |  | 8 | 4 |  |
|  | 5 |  | 5 |  |  | 2 | 2 |  | 8 |  |  | 4 | 4 |  |

NOTE – P2Pnat media is considered strategy 1, 2,3 or 8. Strategies 4 and 5 require the support of an ITU-T H.460.19 server.

For the purpose of Table 10, where an ITU-T H.460.19 server is not available for media strategy indicators 4 (local proxy) ,5 (remote proxy) or 8 (ConeNATProbe), these values shall be deemed to be media strategy indicator 100, indicating call failure (no route to destination). Where both parties and the local gatekeeper do not support Annex B of this recommendation the NAT strategy 8 (ConeNATProbe) shall be replaced with NAT strategy 4 (local proxy).

Proposed changes clause 9.5

If the remote endpoint is not behind a NAT (NAT type 1) and supports remote NAT or has remote NAT type 2 (full cone NAT), then disable all proxy functions and ITU-T H.245 address rewrites. STUN support (if enabled) on the local endpoint shall be disabled. The remote endpoint, if behind a NAT/FW, shall perform a STUN routine to open the remote RTP/RTCP ports and provide the public IP of the NAT/FW device and the STUN assigned ports in its ITU-T H.245 address. It shall also ignore the ITU-T H.245 address provided, if any, and wait until it receives the first RTP/RTCP packets on the remote RTP/RTCP assigned ports. It shall then assign the received apparent source address and port of the remote endpoint as the target to which to send media. Once media is received from the local endpoint, the pinhole in the local NAT/FW has been opened and media can then now flow bidirectionally to/from the local endpoint. This shall be indicated by media strategy indicator 3 (remote media master).

If matching is not obtained with any of the above and the local gatekeeper supports [ITU-T H.460.19], then disable all proxy functions and ITU-T H.245 address rewrites, and disable STUN support (if enabled) on the remote endpoint. Media shall proxy locally only. This shall be indicated by media strategy indicator 4 (local proxy). If, on the condition that both endpoints and the local gatekeeper support Annex B and the NAT type combination is suitable for Annex B (refer Table B.1), the gatekeeper may provide Annex B support. The endpoints shall follow the procedure prescribed in Annex B to probe for a direct route. This shall be indicated by a media strategy indicator 8 (ConeNATProbe).

If matching is not obtained with any of the above and remote gatekeeper supports [ITU-T H.460.19], then disable all proxy functions and ITU-T H.245 address rewrites, and disable STUN support (if enabled) on the local endpoint. Media shall proxy remotely only. This shall be indicated by media strategy indicator 5 (remote proxy).

Annex B

Cone NAT probing

(This annex forms an integral part of this Recommendation)

With certain configurations, it may be possible where a call is placed between two endpoints behind distinct NAT/FW devices and neither are Type 2 (Full Cone NAT) to coerce direct media to flow directly between the endpoints. Removing the need to proxy media. Understanding the NAT behaviour as per RFC 3489 and using an ordered progression of probing with RTP/RTCP probe packets, direct media pathways may be established. Table B.1 indicates the types of NAT combinations supported by this Annex.

Table B.1 – NAT Type combinations supported for Annex B

| Local | Remote |
| --- | --- |
| 3 | 3, 4, 5 |
| 4 | 3 , 4 |
| 5 | 3 |

Where both endpoints support this Annex and having been signalled a Media Strategy of 8, (Cone Nat Probe), the endpoints shall follow the steps in this Annex to establish direct media connectivity.

Figure B-1 – Sample call flow for Direct Media establishment via Annex B

## B.1	Background

NAT Behaviour plays a large component in establishing direct media pathways between endpoints. For instance, if either of the endpoints resides behind a Type 2 full cone NAT (ITU-T H.460.23 Table 8) then Media strategy 2 or 3 (Refer Table 9) may be utilized. In doing so, take advantage of the behaviour where in media may be sent to an opened pin hole in the NAT/FW without the device behind that NAT/FW having first sent a packet out to the originator of the media stream. While Type 2 NAT/FW is common there are conditions where direct media is not easily established and media would need to proxy. The purpose of this Annex is to probe for and establish direct media pathways and remove the requirement to proxy media via an H.460.19 server.

Table B.2 – OID for Cone NAT Probe generic message

| Object identifier value | OID name |
| --- | --- |
| { itu-t (0) recommendation (0) h (8) 460 24 AnnexB (2) } | H.460.24 OLC Cone NAT Probe |

Table B.3 – ITU-T H.245 generic parameters

| Identifier | Request | Raw type | Encoded type |
| --- | --- | --- | --- |
| 1 | Alternate Transport Addresses | H460.24 Annex B AlternativeAddresses | octetString |

## B.2	Procedure to probe for direct connectivity

Once media connectivity has been established via an H.460.19 server, the gatekeeper shall initiate the Annex B direct media probe. The gatekeeper shall make the determination of which endpoint to commence the Annex B procedure with. The selection shall be the local endpoint (registered locally) or calling endpoint, however if their NAT type is Type 5 Symmetric then it shall be the remote or called endpoint. All genericRequests and genericResponse messaging is for gatekeeper to endpoint and visa-versa and shall not be forwarded to the other endpoint. The genericIndication messaging shall be routed end to end.

The gatekeeper shall transmit a genericRequest message containing the OID of Table B.2 and a generic parameter as per Table B.3 containing an encoded AlternateAddresses element. The element shall contain a sequence of AlternateAddress fields containing the detected apparent source addresses for media and control and session id for each session requiring direct media probing. The sessionCUI field shall be omitted. Once the endpoint has received the genericRequest, it shall send at least 3 RTP/SRTP/RTCP/SRTCP probe packets (refer B.3 for format) to the corresponding media/control addresses for each session from the associated local media/control addresses for that media session.

It is not expected there to be a response to the probe packets as this action is designed merely to open media pinholes in the local NAT/FW to the remote NAT/FW. Once the probe packets have been sent the endpoint shall respond back to the gatekeeper via a genericResponse message containing the OID of Table B.2 and an encoded AlternateAddresses element containing a sequence of AlternateAddress fields. Each AlternateAddress shall contain the applicable session ID and an endpoint generated sessionCUI identifier for the purpose of identifying, authenticating and allocating received RTCP probe packets. The media and control addresses fields shall be omitted.

Upon receipt of the genericResponse, the gatekeeper initiates a genericRequest to the other endpoint with an encoded AlternateAddress element containing a sequence of AlternateAddress fields containing the detected apparent source addresses for media and control, session id and received sessionCUI for each session requiring direct media probing of the first NAT/FW. The second endpoint upon receiving the genericRequest, shall send at least 3 RTP/SRTP/RTCP/SRTCP probe packets (refer B.3 for format) to the corresponding media/control addresses for each session from the associated local media/control addresses with the received sessionCUI for that media session. Once the probe packets have been sent the endpoint shall respond back to the gatekeeper via a genericResponse message containing the OID of Table B.2. The AlternateAddress element shall be omitted.

It is expected that the first endpoint may receive the probe packets from the second endpoint and it shall authenticate the appropriate RTCP probe packets for each session and respond back with at least 3 response probe packets to the apparent detected source address of the received probe packet. The first endpoint shall send to the gatekeeper a genericIndication message containing the OID of Table B.2 with an encoded AlternateAddress element containing a sequence of AlternateAddress fields with the sessionID (optional fields omitted) for each detected direct media pathway. This will notify the gatekeeper that a direct media pathway has been achieved for that session.

Upon receiving the response probe packet the second endpoint shall signal to the gatekeeper a genericIndication containing the OID of Table B.2 with an encoded AlternateAddress element containing a sequence of AlternateAddress fields with the sessionID (optional fields omitted) for each verified direct media pathway.

The receipt of the genericIndication message from both endpoints shall also indicate to the ITU-T H.460.19 server that media and control for the associated session will be transmitted directly.

Note that during this brief transition period, RTP or RTCP packets may be received at the ITU-T H.460.19 server or on either the original or alternate/detected port pairs

## B.3	Probe packet format

For RTP/SRTP the probe packet shall have a payload type equal to the media payload type, empty payload size and sequence number starting at some arbitrary value and incrementing by one for each new probe packet.

The RTCP/SRTCP probe packet shall be constructed as an Application-Defined RTCP packet in accordance with clause 6.7 of [IETF RFC 3550] with the settings specified in Table B.4.

Table B.4 – Format of RTCP probe packet

| Parameter | Value |
| --- | --- |
| Packet type | 204 (in accordance with clause 12.1 of [IETF RFC 3550]) |
| Subtype | Probe type (values as per Table B.5) |
| Name | "24.2" |
| Application data | Call Identifier or SHA-1(Call Identifier ¦¦ sessionCUI)* |

*SHA-1 is required if sessionCUI supplied in genericRequest

Table B.5 – Probe types

| Parameter | Indication |
| --- | --- |
| 0 | Request |
| 1 | Reponse |

Annex B  ASN.1 Code

MEDIA-TRAVERSAL {itu-t(0) recommendation(0) h(8) 460 24 2} DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

TransportAddress

FROM MULTIMEDIA-SYSTEM-CONTROL;

AlternateAddresses ::= SEQUENCE

{

addresses		SEQUENCE OF AlternateAddress,

...

}

AlternateAddress ::= SEQUENCE

{

sessionID	  INTEGER(0..255),

sessionCUI   IA5String                 OPTIONAL,

rtpAddress	  TransportAddress	  OPTIONAL,

rtcpAddress	  TransportAddress	  OPTIONAL,

...

}

END

_______________________________