| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, 24, AND 25/16 |  |  |  | AVD-4047 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Andover, MA, USA, 18 - 22 July, 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Alcatel-Lucent |  |  |  |
| Title: | H.248.34 extension proposal – A signalling transport mechanism for Decadic Dialling in H.248.34 Stimulus Analogue Line Package |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This paper provides a solution by extending the existing Stimal package Analogue Line package H.248.34 (ITU-T Rec. H.248.34 (01/2005)) to enable both MG and MGC to support the Decadic (Loop Disconnect) Dialing service as the existing PSTN network.

Unlike in-band DTMF dialling, decadic dialling is detected by the MG and translated into an extended H.248.34 event on a digit-by-digit basis. This event is sent to the MGC which in turn translates the message into the equivalent V5 PSTN protocol.

# 1.	Introduction

It has come to our attention that implementation of decadic dialling service is not clear using current Recommendation H.248.34.

Implementing decadic dialling service can keep the characteristics of decadic dialling the same as it is with a V5 Local Exchange. An example of the characteristics of decadic dialling is that it is not possible to provide some POTS supplementary services to subscribers with decadic dialling phone sets. In this way pulsed digits are not changed into in-band DTMF digits, so decadic dialling phones will act the same as they do when connected to a V5 AN.

When keeping the characteristics of decadic dialling service is required, this H.248.34 STIMAL package extension can be used to map the dialled pulsed digits to V5 PSTN protocol. By doing this way MGC could request the MG to send detected loop disconnect digits.

The rest of this paper is organized as follows. In Section 2, it specifies the extended event and illustrates a couple of basic call scenarios using decadic and DTMF dialling respectively. Section 3 describes additional impacts to current Recommendation H.248.34 and proposes the revisions for discussion. Section 4 concludes the paper.

# 2.	Extended H.248.34 Event Implementation

The additional H.248.34 Digits Event (defined in clause 5.2) is specified as below. A basic originating call scenario using decadic dialling is illustrated in this section. An DTMF dialling scenario is also depicted for background information.

## 2.1	Recommendation Extension

Below describes the pulsed digits extension definition for the H.248.34 according to the H.248.1 packages format.

5.2 Events

-------------------------

5.2.5	Pulsed Digits

Event Name: 	Pulsed Digits

Event ID: 		digits (0x0005)

Description:

Activation of the "Digits" event results in all the pulse dialled digits being reported to the MGC.

5.2.5.1	EventsDescriptor parameters

None

5.2.5.2	ObservedEventsDescriptor parameters

5.2.5.2.1	Digit

Parameter Name:	Digit

ParameterID: 		digit (0x0001)

Description:

Pulse dialled digit that is recognized by the MG is reported via the ObservedEvents Descriptor.

One digit per parameter.

Type: 				String

Optional:			no

Possible values:		A sequence of the characters '0' through to '9'

Default:				no

5.5	Procedures

--------------

5.5.11	Event – Digits

The event "Digits" is used by the MG to report to the MGC, the occurrence of pulse dialling (loop disconnect digits) on the analogue POTS line. The "Digit" parameter identifies the detected digit.

## 2.2 	Basic Call Scenarios

In below Figure 1 a basic call scenario for an originating call using decadic dialling is depicted. Figure 2 depicts a basic call scenario for and originating call using DTMF dialling. Note that both scenarios have the extended Event Digits in H.248.34 SIMAL mode.

The MGC should request the detection of these individual digits on all calls (i.e., Events=xxx {stimal/digits, stimal/stedsig, stimal/pulsedsig, g/sc}).  Whenever a decadic dialling digit is detected on the decadic dialling phone, then the digit (A sequence of the characters '0' through to '9') should be sent to the MGC in the extended H.248.34 STIMAL event.

The MG sends the digits this way only if it receives decadic digits. DTMF digits should still be handled transparently with in the voice band, which means the DTMF tone must be backhauled in-band to the LE over the V5 bearer channels.

Figure 1

Figure 2

# 3.	Further Discussion

Apart from the extension there are a few addition points in H.248.34 that may need to be revised due to the extension.

## 3.1	Revision of Clause 5.5.7 Signal-Digits

ITU-T Rec. H.248.34 (01/2005) 5.5.7	Signal – Digits

Original statement: “The signal "Digits" is sent by the MGC to request the MG to send loop disconnect digits to an analogue line.”

Revised statement: “The signal "Digits" is sent by the MGC to the MG to indicate that a certain loop digits shall be sent to an analogue line.”

The revision is in accordance with Recommendation ITU-T G.964 (03/2001). See below clause from ITU-T G.964 for reference.

ITU-T G.964 (03/2001) B.3.9 Digit-signal information element

--------------

Another example of the use of this signal in the direction LE to AN is to provide digit information for a DDI PBX. In this case, the LE may request an acknowledgement from the AN when the digit has been pulsed out by the user port to the DDI PBX. This allows the LE in some networks to control the interdigit timing.

## 3.2	Revision of Table 6 in Clause 5.5.10

ITU-T Rec. H.248.34 (01/2005) 5.5.10	Guidelines for mapping of the V5 PSTN protocol

Table 6 provides a mapping of V5 information elements to H.248 concepts

Original statement:

| Table 6/H.248.34 – Detailed mapping of V5 PSTN Protocol Information Elements |  |
| --- | --- |
| Digits Signal | In the MGC to MG direction, the V5 information element is mapped to the signal "digit" in this package, with the V5 parameters mapped as follows: • Digit Information – Directly mapped to the "Digits" Parameter. • Digit Ack Request Indicator – See text regarding the V5 "pulse notification" information element. In the direction of MG to MGC, the Loop Disconnect digits detected by the MG are reported as a "digit completion" event as part of H.248 digit map handling. |

Revised statement:

| Table 6/H.248.34 – Detailed mapping of V5 PSTN Protocol Information Elements |  |
| --- | --- |
| Digits Signal | In the MGC to MG direction, the V5 information element is mapped to the signal "digit" in this package, with the V5 parameters mapped as follows: • Digit Information – Directly mapped to the "Digits" Parameter. • Digit Ack Request Indicator – See text regarding the V5 "pulse notification" information element. In the direction of MG to MGC, the Loop Disconnect (Decadic) digits detected by the MG are reported as Digits Events defined in extended clause 5.2.5. |

This revision is to reflect the extension which enables the decadic dialling service.

The original way of handling loop disconnect (decadic) digits is inappropriate for decadic digits requirement because “digit completion” event is for DTMF dialling according to ITU-T Rec. H.248.16 (2002)/Cor.1 (03/2004). See below statement from ITU-T Rec. H.248.16 for reference.

ITU-T Rec. H.248.16 (2002)/Cor.1 (03/2004) clause Summary

ITU-T Rec. H.248.16 defines two packages that provide enhanced digit collection capabilities for H.248.1:

– extended DTMF digit map completion event, incorporating detailed reporting of timeouts,digit buffering control, and reporting and control of processing of extra events;

– enhanced DTMF digit map completion event, incorporating additional digit collection procedures for reporting a completion event.

## 3.3	Revision of Table I.2 in Appendix I

Appendix I

V5 to H.248 mapping overview

Table I.2 indicates which information elements are implemented as events and/or signals in the stimulus analogue line package and is based on Table 17/G.964.

Original table information:

Table I.2/H.248.34 – Information element identifier coding

| Name | Reference G.964 (clause) | Implemented as an event in stimulus analogue line package | Implemented as a signal in stimulus analogue line package | Mapped | Not required |
| --- | --- | --- | --- | --- | --- |
| Steady-signal | 13.4.7.4 | X | X | X |  |
| Digit-signal | 13.4.7.5 |  | X | X |  |
| Recognition-time | 13.4.7.6 | X |  | X |  |

Revised table information:

Table I.2/H.248.34 – Information element identifier coding

| Name | Reference G.964 (clause) | Implemented as an event in stimulus analogue line package | Implemented as a signal in stimulus analogue line package | Mapped | Not required |
| --- | --- | --- | --- | --- | --- |
| Steady-signal | 13.4.7.4 | X | X | X |  |
| Digit-signal | 13.4.7.5 | X | X | X |  |
| Recognition-time | 13.4.7.6 | X |  | X |  |

The revision is in accordance with Recommendation ITU-T G.964 (03/2001). See below clause and Figure 3 from ITU-T G.964 for reference.

ITU-T G.964 (03/2001) section 13.4.7.5 Digit-signal

The purpose of the Digit-signal information element is either to indicate to the AN that a certain digit shall be sent to the subscriber's equipment or that a particular digit transmitted by the subscriber's equipment has been detected at the PSTN user port.

Figure 3

# 4.	Conclusion

This extended H.248.34 Digits Event enables the decadic dialling capability on both MG and MGC. To support decadic dialling MG must first defect, then relay via this H.248.34 extended event, the dialled digit to the MGC, which in turn relays the dialled digit towards the PSTN using existing V5 PSTN protocol.

This extension is in accordance with Recommendation ITU-T G.964 (03/2001). It does not impact the current ITU-T G.964.

Characteristics of decadic dialling are kept in this way. Therefore decadic dialling phones will act the same as they do when connected to a V5 AN.

__________________