| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4062 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.248.80: Updates |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document proposes changes to draft ITU-T Recommendation H.248.80 to address several Editor’s notes.

2.	Discussion

Draft Recommendation ITU-T H.248.80 contains several open Editor's notes.

## 2.1	Editor's note 1:

## 3.2	Terms defined in this Recommendation

{Editor's note: Further analysis is invited if directionality associated with configurations should be discussed by the definitions.}

It is not clear to the Contributor’s that directionality needs to be further described. It is proposed to discuss the need for this at the meeting and potentially remove this Editor’s note.

## 2.2	Editor's note 2:

11.4	Further example with respect to session configurations

{Editor's note: Clauses 11.4 and 11.5 are incomplete. If these are incomplete at the time that the rest of the document is ready for Consent then they will be removed.}

As contributions were accepted at the March Q.3/16 meeting completing these sections it is proposed to remove this editor's note.

## 2.3	Editor's note 3:

Table 10 – Example command encoding – MGC request

{Editor's note 1: Section 10.1.2 defines the smcn/lconf property as of type "Sub-list of String" and specifies that "Each instance of String is defined as type “lcfg” or "mcap" according to the following ABNF ". The then following ABNF grammar excerpt additionally lists the "tcap" attribute. Here we assume that also the "acap" attribute may be used within smcn/lconf property values.

The Contributors agree that the "acap" attribute should be included within the smcn/lconf property values. Therefore we propose to add this to the package and to delete the editor's note.

## 2.4	Editor's note 4:

Table 10 – Example command encoding – MGC request

Editor's note 2: This document's Table 7 contains an MGC request example also containing the smcn/lconf property. There, however, the list of strings is surrounded by curly braces. Here in Table 10 we use square brackets (as well as in case of the smcm/sconf property) because the ABNF rule of "alternativeValue" in H.248.1 Annex B.2 requests to use square brackets in the "sublist" case.}

The Contributors agree that square brackets should be used. Therefore we propose to correct this in the examples and to delete the editor's note.

## 2.4	Editor's note 5:

Table 11 – Example command encoding– MG reply

{Editor's note 1: Aadded the TerminationState descriptor to the MG's reply and  added the smcm/sconf property because assumingly the MG only supports two of the MGC requested four session configurations. Would this be inline with the intended MG behaviour? We assume that in this case the smcn/lconf property would also be returned by the MG, because this specific latent configuration "5" is part of the returned session configuration "4". }

The Contributors agree that it is possible for the MG to return a subset of session configurations. Therefore we propose to capture part of the editor's note in the table and to delete the note.

3.	Proposal

It is proposed to accept the following changes. Changes are marked up against TD 343R1/WP2 from the July, Geneva SG16 meeting.

### 10.1.2 Latent Configurations

{Editor's Note: The IETF media capabilities draft allows for an autonomous use of latent configurations without the need of SDP signalling. This needs to be considered by this work item.}

|  | Property Name: | Latent Configurations |
| --- | --- | --- |
|  | Property ID: | lconf (0x0003) |
|  | Description: | This property allows the MGC to determine if additional latent configurations are supported on the Termination. A latent configuration represents an additional media stream to the one already specified on the Termination. According to clause 3.3.5/[IETF MEDIACAPNEG] the latent configuration must contain the media type and transport protocol, and in most cases must include the media configuration. As such this property also allows the specification of the transport capabilities and media configurations. |
|  | Type: | Sub-list of String |
|  | Possible values: | Each instance of String is defined as type “lcfg”,"mcap", "tcap" or "acap" according to the following ABNF: lcfg = "lcfg:" config-number 1*WSP lcfg-config-list ; For further definitions see clause 3.3.5/ ; [IETF MEDIACAPNEG] tcap = "tcap:" trpr-cap-num 1*WSP proto-list ; For further definition see [IETF RFC 5939] mcap = "mcap:" media-cap-num-list 1*WSP encoding-name "/" clock-rate ["/" encoding-parms] ; For further definitions see clause 3.3.1/ ; [IETF MEDIACAPNEG] acap = "acap:" att-cap-num 1*WSP att-par ; For further definitions see clause 3.4.1/ ; [IETF RFC 5939] |
|  | Default: | None. |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |

## 10.2	Events

None.

## 10.3	Signals

None.

## 10.4	Statistics

None.

## 10.5	Error Codes

None.

## 10.6	Procedures

### 10.6.1	General

The support of this package implies that the media capabilities negotiation defined in [IETF MEDIACAPNEG] is supported by the MG. Therefore the MG shall include the “med-v0” option tag in a response to either an audit or CHOOSE wildcard of the eroas/sdpe property.

As a result the MGC may use the "a=mcap:", "a=mfcap:" and "a=mscap:" media level attributes, the associated “m=” and “pt=” parameters and the enhanced potential configuration attribute (see clause 3.3/[IETF MEDIACAPNEG] in SDP in H.248 Local and Remote Descriptors.

As the "Session Configurations" and "Latent Configurations" properties are set in Termination state they are unaffected by the setting of Reserve Group/Value.

### 10.6.2	Session Configurations

In order to use the Session Configuration functionality rather than supplying the “a=sescap:” attribute the MGC shall supply the “Session Configuration” (smcn/scap) property to the MG. If session configuration functionality is required the MGC shall include this functionality when specifying multiple configurations (especially when multiple stream are used) so the MG can choose the appropriate related configurations.

If the MGC requires the MG to provide the session capabilities in addition to wildcarding the preferred configurations (as described in 9.6.2) the MGC shall wildcard CHOOSE the smcn/sconf property.

### 10.6.3	Latent Configurations

In order to use the Latent Configuration functionality rather than supplying the “a=lcfg:” attribute the MGC shall use the “Latent Configurations” (smcn/lconf) property. It shall also include any related acap, tcap and  mcap attributes. If the MGC requires the MG to supply the additional configurations that it can support without assigning resources the MGC shall wildcard CHOOSE the smcn/lconf  property.

...

Table 7 – Example command encoding – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [123.123.123.4]:55555 Transaction = 60000 { Context = $ { Add = $ {Media { TerminationState { smcn/lconf = [ "tcap:1 RTP/AVP mcap:10 H263-1998/90000", "mcap:11 H264/90000", "lcfg:10 mt=video t=1 m=10¦11" ] } Stream = 1 { LocalControl { ReservedValue = ON }, Local { v=0 c=IN IP4 $ a=creq:med-v0 m=audio $ RTP/AVP 0 a=rtpmap:0 PCMU/8000 a=mcap:1 PCMU/8000 a=mcap:2 g729/8000 a=mcap:3 telephone-event/8000 a=mfcap:3 0-11 a=pcfg:1 m=1,3¦2,3 pt=1:0,2:18,3:100 }, Remote { v=0 o=- 25678 753849 IN IP4 192.0.2.1 s= c=IN IP4 192.0.2.1 t=0 0 a=creq:med-v0 m=audio 23456 RTP/AVP 0 a=rtpmap:0 PCMU/8000 a=mcap:1 PCMU/8000 a=mcap:2 g729/8000 a=mcap:3 telephone-event/8000 a=mfcap:3 0-11 a=pcfg:1 m=1,3¦2,3 pt=1:0,2:18,3:100 } } } } } | The latent configuration is indicated on TerminationState level, using the lconf property from the smcn package. The Offerer indicates the options of two possible latent video configurations: H.263 or H.264 video formats. The preferred configurations are mapped into the LD/RD of the Stream Descriptor. |

As per clause 4.3 / [IETF MEDIACAPNEG] the MG is able to support all the potential configurations and H.263 video but not H.264. It replies with (Table 8):

Table 8 – Example command encoding– MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [125.125.125.111]:55555 Reply = 50000 { Context = 6000 { Add = A5556 {Media { TerminationState { smcn/lconf = [ "mcap:10 H263-1998/90000", "lcfg:1 mt=video t=1 m=10" ] } Stream = 1 { LocalControl { ReservedValue = ON }, Local { v=0 o=- 24351 621814 IN IP4 192.0.2.2 s= c=IN IP4 192.0.2.2 t=0 0 a=csup:med-v0 m=audio 54322 RTP/AVP 0 100 a=rtpmap:0 PCMU/8000 a=rtpmap:100 telephone-event/8000 a=fmtp:100 0-11 a=acfg:1 m=1,3 pt=1:0,3:100 a=pcfg:1 m=2,3 pt=2:18,3:100 }, Remote { v=0 o=- 25678 753849 IN IP4 192.0.2.1 s= c=IN IP4 192.0.2.1 t=0 0 a=csup:med-v0 m=audio 23456 RTP/AVP 0 a=rtpmap:0 PCMU/8000 a=rtpmap:100 telephone-event/8000 a=fmtp:100 0-11 a=acfg:1 m=1,3 pt=1:0,3:100 a=pcfg:1 m=2,3 pt=2:18,3:100 } } } } } | The MG supports the H.263 but not the H.264 video format. The MG therefore confirms the requested H.263 video configuration as a latent configuration. |

## 11.4	Further example with respect to session configurations

...

Table 10 – Example command encoding – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [123.123.123.4]:55555 Transaction = 60000 { Context = $ { Add = $ {Media { TerminationState { smcn/sconf = [ "1 1", "2 2", "3 3,5", "4 4,5" ], smcn/lconf = [ "tcap:2 udptl", "mcap:5 t38", "acap:11 T38FaxVersion:4", "acap:12 T38FaxRateManagement:transferredTCF", "acap:13 T38FaxUdpEC:t38UDPRedundancy", "lcfg:5 mt=image t=2 m=5 a=11,12,13" ] }, Stream = 1 { LocalControl { ReservedValue = ON }, Local { v=0 c=IN IP4 $ a=creq:med-v0 m=audio $ RTP/AVP 18 a=rtpmap:18 G729/8000 a=tcap:1 RTP/AVP a=mcap:1 G729/8000 a=mcap:2,3 PCMU/8000 a=mscap:3 gpmd vbd=yes a=pcfg:1 t=1 a=-ms m=1,3 pt=1:18,3:99 a=pcfg:2 t=1 a=-ms m=2,3 pt=2:0,3:99 a=pcfg:3 t=1 a=-ms m=1 pt=1:18 a=pcfg:4 t=1 a=-ms m=2 pt=2:0 } } } } } | In Table 9 above, as the image/t38 based media description is not offered as a potential configuration but only communicated as latent configuration, the MGC does neither add an image/t38 based session description as separate group to he Stream 1 LD nor does it add a second image/t38 based H.248 stream. Potential future usage of T.38 would required a subsequent SDP offer/answer negotiation procedure, which then would typically trigger corresponding H.248 procedures to modify the created stream endpoint to an image/t38 based session and media description. |

It is assumed that the MG does not support G.729 and that it therefore only supports session configurations 2 (Audio (PCMU), VBD (V.152 PCMU)) and 4 (Audio (PCMU) and Fax Relay (T.38 FoUDPTL/UDP)). An example MG reply is shown in Table 11:

Table 11 – Example command encoding– MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [125.125.125.111]:55555 Reply = 60000 { Context = 6000 { Add = A5678 { Media { TerminationState { smcn/sconf = [ "2 2", "4 4,5" ], smcn/lconf = [ "tcap:2 udptl", "mcap:5 t38", "acap:11 T38FaxVersion:4", "acap:12 T38FaxRateManagement:transferredTCF", "acap:13 T38FaxUdpEC:t38UDPRedundancy", …, // Further acap attributes "lcfg:5 mt=image t=2 m=5 a=11,12,13,…" ] }, Stream = 1 { Local { v=0 o=- 24351 621814 IN IP4 192.0.2.2 s= c=IN IP4 192.0.2.2 t=0 0 a=creq:med-v0 m=audio 53456 RTP/AVP 18 a=rtpmap:18 G729/8000 a=tcap:1 RTP/AVP a=mcap:2,3 PCMU/8000 a=mscap:3 gpmd vbd=yes a=pcfg:2 t=1 a=-ms m=2,3 pt=2:0,3:99 a=pcfg:4 t=1 a=-ms m=2 pt=2:0 } } } } } } | The TerminationState descriptor and the smcm/sconf property are added to the MG's reply because athe MG only supports two of the MGC requested four session configurations. |

___________________