| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4075 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | Q5 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Polycom |  |  |  |
| Title: | Video Resolution in Telepresence Systems |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This document identifies the relationships between units of measure for video displays and cameras and the units used to measure human visual acuity.  It then suggests a range for the CPD (cycles per degree) that should be supported by telepresence systems.

It is proposed that this information should be incorporated into H.TPS-AV.

# Introduction

Telepresence systems in the market today use a variety of display sizes and aspect ratios, and often use different numbers of displays.  The effective resolution (amount of detail) seen by the users of these systems also depends on the room design – in particular, the viewing distance.

In addition, measurements of human visual acuity, display resolution, and camera resolution all use different units.

Polycom believes it is useful for Question 5 to use a consistent measure of resolution, which takes the viewing distance into account.  Ideally this measure would be easy to relate to human visual acuity, and would also be orthogonal to measures of the system size and field of view.

# Discussion

Human visual acuity is the ability to distinguish fine detail.  It is generally measured in cycles per degree (CPD), and therefore measures an angular resolution.  People with excellent visual acuity can resolve 50 CPD – 1.2 arc minute per line pair (or 0.35 mm line pair at 1 meter). [see John C. Russ, The Image Processing Handbook, CRC press, 2006].

Camera resolution is usually measured in lines across the entire horizontal or vertical field of view.  Black and white lines are alternated.  A cycle is equivalent to 2 lines of camera resolution – that is,  a black+white line pair comprises a cycle.  The field of view of a camera is measured in degrees.  Therefore the cycles per degree for a camera can be computed as lines/(2*field_of_view).

Similarly display resolution is measured in pixels (width by height).  For modern flat panel displays, a line is equivalent to a row or column of pixels.  The cycles per degree for a display depends on the subtended angle of the display, which in turn depends on the size of the display and the distance of the display surface from the viewer.  Therefore, the cycles per degree for a display can be computed as pixels/(2*subtended_angle).

## Example 1

A typical 3-screen telepresence system uses 3 cameras with horizontal field of view of 25 degrees.  It uses 65 inch 16x9 displays, and is calibrated to show images at actual size.

Each display has a width of 56.65 inches (1439 mm), and the viewers are seated 3.245 meters from the display.  At this distance, the horizontal subtended angle is also 25 degrees, matching the camera field of view.

The native resolution of each display is 1920 x1080 pixels (1080p); which corresponds to 1920/(2*25) cycles per degree – or about 38.4 CPD.

Similarly, 720p corresponds to about 24 CPD. A 65” 4K display at this viewing distance provides about 71 CPD, which exceeds the resolving power of the human eye.

# Conclusion

Polycom proposes that H.TPS-AV define effective resolution in units of CPD, and offer the text above as baseline text describing the relationships between TV lines, pixels, and CPD.

The exemplary system above illustrates that typical telepresence systems available today offer resolutions between about 25 CPD and 40 CPD.  Such systems have been well received in the marketplace, which suggests that resolutions within that range are quite acceptable.  There is no need to design systems that exceed 50 CPD, since such systems exceed the limits of human acuity.

Therefore, Polycom further proposes that H.TPS-AV include normative text indicating that telepresence systems should offer effective resolutions between 25 CPD and 50 CPD.

__________________