| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4081 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 25 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | Proposed modification to the draft Rec. ITU-T F.USN-SM |  |  |  |
| Purpose: | Proposal |  |  |  |

This document contains proposed modification of Draft Recommendation ITU-T F.USN-SM.

Draft Recommendation F.USN-SM

Capabilities of ubiquitous sensor network (USN) for supporting requirements of smart metering systems

Justification of this Recommendation:

(Editor’s note)  This is  not a part of the main body of F.USN-SM and was initially prepared for information. It is because it could keep used for information until its final state.

Gap analysis on smart metering of ITU and other SDOs

There are two related activities for smart metering issues in the ITU and other SDOs.

- ITU-T

In ITU-T, there is an activity to deal with a topic about smart metering. The title is “Smart metering using connected objects in ubiquitous home networking environments,” and deals with the concept, characteristics, scenario and functional architecture for smart metering in telecommunication perspectives.

- ETSI

ETSI TC M2M (Machine-to-Machine) published Smart Metering Use Cases. This introduces M2M application for smart metering and examples, and it describes the use cases for the smart metering M2M applications.

However, these two activities do not deal with USN capabilities for supporting smart metering services.

(Editor’s Note) In this clause, it will be introduced what issues of smart metering exist and how smart metering is exploited in the other SDOs, and then what issues of them can be standardized in ITU-T will be analyzed.

(Contributor’s Note) Above editor’s note should be deleted.

meter reading data

Draft Recommendation F.USN-SM

Capabilities of ubiquitous sensor network (USN) for supporting requirements of smart metering services

AAP Summary

To be provided before Consent.

Summary

This Recommendation identifies capabilities of ubiquitous sensor network for supporting requirements of smart metering services.

Keywords

Smart grid, smart metering, USN

Contents

Pages

1	Scope	5

2	References	5

3	Definitions	5

3.1	Terms defined elsewhere	5

3.2	Terms defined in this Recommendation	6

4	Abbreviations and acronyms	6

5	Conventions	6

6	Overview of smart metering	6

7	Smart metering scenarios	7

7.1	Scenario I: regularly scheduled remote meter reading	7

7.2	Scenario II: on-demand remote meter reading	8

7.3	Scenario III: demand response	9

7.4	Scenario IV: tariff configuration	10

8	Requirements of smart metering services	11

8.1	Reliable information delivery	11

8.2	Real-time delivery of meter reading data	11

8.3	Bi-directional communication between meters and operators	11

8.4	Security support	11

9	USN-based smart metering services	12

10	USN capabilities for smart metering services	13

10.1	Reliable data delivery	13

10.2	Scalability	13

10.3	Mobility support	13

10.4	Delivery latency	13

10.5	Fault detection and recovery	13

10.6	Security	13

10.7	Connectivity	13

10.8	Unicasting & Multicasting	14

Bibliography	15

Scope

The main purpose of this Recommendation is to identify capabilities of ubiquitous sensor network which supports requirements of smart metering services. The scope of this Recommendation covers the following:

- Overview of smart metering;

- Smart metering scenarios;

- Requirements of  smart metering services;

- USN capabilities for supporting requirements of  smart metering services;

References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T Y.2221]		ITU-T Recommendation Y.2221 (2010), Requirements for support of ubiquitous sensor network (USN) applications and services in the NGN environment

(Editor’s Note) The related references will be added.

(Contributor’s Note) Above editor’s note should be deleted.

Definitions

Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	sensor [ITU-T Y.2221]: An electronic device that senses a physical condition or chemical compound and delivers an electronic signal proportional to the observed characteristic.

3.1.2	sensor network [ITU-T Y.2221]: A network comprised of inter-connected sensor nodes exchanging sensed data by wired or wireless communication.

3.1.3	sensor node [ITU-T Y.2221]: A device consisting of sensor(s) and optional actuator(s) with  capabilities of sensed data processing and networking.

3.1.4	smart grid [ITU-T FG-Smart]: An electricity network that can intelligently integrate the actions of all users connected to it - generators, consumers and those that do both - in order to efficiently deliver sustainable, economic and secure electricity supplies.

3.1.5	ubiquitous sensor network (USN) [ITU-T Y.2221]: A conceptual network built over existing physical networks which makes use of sensed data and provides knowledge services to anyone, anywhere and at anytime, and where the information is generated by using context awareness.

Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	smart meter:  A premise device to monitor and control electrical power, gas and water usage of home appliances based on demand response information from home appliances.

3.2.3	smart metering: An operation to provide information to customers about energy consumption. The information includes how much energy the customers are consuming and how much it costs, etc.

3.2.4	demand response: A smart metering feature that allows consumers to reduce or change their use patterns of electricity, gas and water during peak demand usually in exchange for a financial incentive.

Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

ETSI; European Telecommunications Standards Institute. M2M; Machine-to-Machine.

(Editor’s Note) The abbreviations and acronyms will be added.

(Contributor’s Note) Above editor’s note should be deleted.

Conventions

None.

Overview of smart metering

Not only governments and utility companies, such as electricity, gas, and water suppliers, but also researchers have been interested in automatic meter reading based on communication system, known as smart metering because smart metering system can bring various benefits to customers and governments as well as utility companies.

Smart metering systems perform metering and exchanging metering information between smart meters and utility companies. Sensor network technologies can be used for metering and communication networks can be used for exchanging information. Figure 1 depicts an overall diagram of smart metering systems. Metering information obtained from appliances by sensor nodes with communication module in smart home are collected and delivered to operators of utility companies.

Figure 1: Overview of smart metering

(Editor’s Note) Basic concept of smart metering will be introduced. In addition, several types of smart meters and their applications/characteristics will be identified.

(Editor’s Note) Instead of “communication module” in the smart home side, appropriate words for better understanding is required. At the meeting, there was a proposal to use “communication interface”. For this, further contribution is requested.

(Editor’s Note) Figure 1 and Figure 2 need to be aligned at the next meeting.

(Contributor’s Note) Above editor’s notes are applied so need to be deleted.

Smart metering scenarios

Scenario I: regularly scheduled remote meter reading

This scenario describes procedures that meter reading data, obtained every scheduled time by smart meter, are delivered to smart metering operators.

- Smart meters may be scheduled to obtain meter reading data and transfer them to a smart metering operator.

- According to the schedule, smart meters measure the amount of energy consumption and obtain meter reading data from electric appliances.

- The obtained meter reading data are delivered from the smart meters to a smart metering operator.

Figure 2: Regularly scheduled remote metering

Scenario II: on-demand remote meter reading

This scenario describes procedures that meter reading data obtained and delivered to smart metering operator on demand.

- When a smart metering operator needs to collect information of electricity consumption, , the operator send the message to smart meters for collecting meter reading data obtain from electric appliances.

- The smart meters verify the received the message and then measure energy consumption according to the task of the message, if the message is valid.

- Meter reading data obtained from the appliances are delivered to the smart metering operator.

(Contributor’s Note) On-site meter reading scenario is deleted because on-site meter reading cannot be included into smart metering services.

Figure 3:  On-demand remote meter reading

Scenario III: demand response

This scenario describes how customers respond metering operator’s demands.

- Smart meters may be scheduled to obtain meter reading data and transfer them to a smart metering operator.

- When consumers use electric appliances simultaneously, the total amount of electricity consumption steeply increases.

- Meter reading data about the increasing amount of electricity consumption are reported to the smart metering operator at every scheduled time, and

- the cost of the electricity may also increase according to tariff policies depending on supply and demand.

- (a) The smart metering operator sends a message which may propose to reduce electricity consumption or may inform customers of the increased electricity cost.

(b) The smart metering operator sends a message to shut off electricity by force due to a reason of operational policy.

- (a) The smart meters display the message to the consumers, and the consumers may decide to reduce electricity consumption, or the electric appliances may be switched off by a consumer.

(b) When receiving the shut-off message, the smart meter display the message and shut off electricity. The connected appliance is automatically switched off.

Figure 4: Demand response

Scenario IV: tariff configuration

This scenario describes how cost of energy consumption is decided and configured into the meters.

- Smart meters may be scheduled to obtain meter reading data and transfer them to a smart metering operator.

- The smart meters regularly deliver meter reading data to the metering operator according to predefined schedule.

- The cost of the electricity may be dynamically changed according to tariff policies depending on supply and demand.

- When the cost is changed, a message about change of cost is delivered to all of the meters.

- The changed cost of electricity may be shown on a display, and the changed cost is applied.

Figure 5: Tariff configuration

Requirements of smart metering services

Reliable information delivery

In the smart metering services, meter reading data can be frequently transferred from distributed meters, and also operators can send many control messages to smart meters. When delivery errors occur, the operators cannot collect some of meter reading data from smart meters. As a result, such missing meter reading data would lead to unwanted results. Therefore, the smart metering services are required to provide reliable information delivery, such as meter reading data and messages described in the scenario.

Real-time delivery of meter reading data

Demands of smart metering operators require real-time responses of smart meters. Therefore, meter reading data obtained by smart meters are required to arrive at operators within a given amount of time. In the scenario I-3 and II-3, if the obtained meter reading data are not delivered to operators within a required time, the operators cannot obtain real-time results of smart metering. Therefore, smart metering services are required to guarantee real-time delivery.

Bi-directional communication between meters and operators

Meter reading data are delivered from meters to operators as shown in the scenario I-3, II-3, and III-2, and messages generated by operators are also sent from operators to meters vice versa (scenario III-3, and IV-3). For doing this, the smart metering services are required to support bi-directional communication between meters and operators.

Security support

Operators need to control meters in order that the meters take actions according to the indication of operators. In this case, the operators should be authorized by smart meters for taking secure actions (scenario II-2).

Because metering information may include privacy information, it should not be revealed to unauthenticated third party. Therefore, the metering information is required to be secured to unauthenticated third party.

USN-based smart metering services

The smart metering services shown in Figure 6 require technological facilities to support metering and exchanging metering information between smart meters and metering operators in utility control center. Basic characteristic of USN applications and services is gathering data by sensor networks and transmitting data to a remote server through communication networks. Also, control information is transmitted to sensor networks from the remote server. This feature of USN can be applied to the smart metering services. Sensor nodes in sensor networks have capabilities of metering and delivering metering information to a server of utility control center.

A smart metering system can be implemented by sensor nodes of USN as follows.

- Smart metering sensor nodes: each sensor node has capabilities of smart metering, so sensor nodes process smart metering information for the smart metering procedures, such as meter reading collection and tariff setting. A gateway just delivers information between the sensor nodes and metering operators.

(Contributor’s Note) Smart metering services based on USN in this document are focused on smart metering sensor nodes. Therefore, the model of smart metering gateway is removed.

Figure 6: Smart metering service based on USN

(Editor’s Note) Although two different cases in Figure 2 are explained in this sub-clause, “gateway” and “gateway (smart metering) in figures are unclear. Further discussion on how to modify this figure is required.

(Editor’s Note) Descriptions of relevant communication interfaces (e.g., ZigBee) in the smart home side are also needed.

(Contributor’s Note) Above editor’s note should be deleted.

USN capabilities for smart metering services

Reliable data delivery

USN is recommended to guarantee reliable data delivery for reliable meter reading transmission.

Scalability

New meters (sensor nodes) can be added to or existing meters (sensor nodes) can be removed from existing smart meters (sensor networks). Such a change should not degrade the reliability of the smart metering services.  Therefore, sensor networks in USN are required to support scalability.

Mobility support

In some cases, a customer can change the positions of appliances. This change of position may involve the changes meters’ position in intra-sensor network or inter-sensor networks. Therefore, sensor networks in USN are recommended to support mobility of meters (sensor nodes).

Delivery latency

Meter reading data should be delivered within a given time. For this requirement, USN is recommended to guarantee data delivery within a given time.

Fault detection and recovery

When sensor nodes in USN use wireless communications, there is a possibility of link failure between nodes due to the characteristics of wireless communication protocols. Such a failure can give negative effects on reliability and delivery latency of the smart metering services. USN is recommended to detect faults and recover from the faults.

Security

Meter reading data and messages may be transferred to/from operators and meters by hop-by-hop transmission among sensor nodes. Therefore, hop-by-hop security among sensor nodes is recommended to be implemented.

Connectivity

Meter reading data, obtained from each sensor node, are collected by a gateway, and then the collected meter reading data are delivered to metering operators through transport networks.  In addition, control messages of the operators are also delivered to each sensor node. For doing this, connectivity between sensor nodes, connectivity between sensor nodes and the gateway, and connectivity between the gateway and outside other networks are recommended to be guaranteed.

(Contributor’s Note) The context of this clause, “QoS support” is overlapped with clause 10.1 Reliable data delivery and clause 10.4 Delivery latency, so this clause, “QoS support” need to be removed.

(Editor’s Note) unique address assignment is too strong expression. In the meeting there were some proposals to focus on more generally identification issue rather than address assignment. So, further contribution for modification of this sub-section is required for better understanding.

(Contributor’s Note) Unique address assignment can be one of capabilities to support bidirectional communication, but clause 10.7 connectivity has also duplicated context with this clause. Thus, this clause and editor’s note need to be removed

Unicasting & Multicasting

A unicasting service is required to be supported among sensor nodes in order that obtained meter reading data are delivered to metering operators. Furthermore, control messages of metering operators should be delivered to all of sensor nodes. Therefore, multicasting is required to be supported.

Bibliography

[b-ITU-T Smart-I-6] 	ITU-T FG-Smart Grid, Smart-I-6, “Smart metering using connected objects in ubiquitous home networking environments,” June 2010.

[b-ESTI TR 102.691]	ESTI TR 102.691 v1.1.1, “Machine-to-Machine communications (M2M); Smart Metering Use Cases,” May 2010.

[b-Khalifa] 	T. Khalifa et. al., “A Survey of Communication Protocols for Automatic Meter Reading Applications,” IEEE Communications surveys and tutorials, Vol. PP, Issue 99, January 2010.

[TBD]

__________________