| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4087 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | Proposed texts for requirements of Telepresence services |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes some texts for requirements of telepresence in the aspect of multiple stream and security concerns.

# Introduction

It was agreed to initiate a new draft recommendations on requirements of telepresence in the last meeting. [F.TPS-Reqs]

# Discussion & Proposal

In telepresence, several requirements should be met in various aspects such as network, multimedia handling, security, collaboration and environmental matters.

In this contribution, we have listed up the requirements of telepresence for each aspect. It is proposed to discuss the attached texts and include them into appropriate chapter of F.TPS-Reqs.

Attachment
Proposed text

[Begin proposed text]

X.x Network requirements

The telepresence system requires high bandwidth network for transmitting high-definition audio-visual stream data. In case of 1080p telepresence session, it is required to have about 15Mbit/s of bandwidth for one-to-one communications. This traffic will increase proportionally to the number of participants. Hence, it is important to guarantee pre-defined network bandwidth during telepresence session. In case of QoS-guaranteed network such as NGN, it is possible to reserve bandwidth.

Net001: The telepresence system is recommended to reserve network resource for assuring QoS for the telepresence session.

On the other hand, best-effort network such as Internet cannot provide strictly guaranteed network QoS. For the latter case, the telepresence system can be deployed into dedicated network or manipulate its video quality based on network status.

Net002: The telepresence system is required to be able to probe its network status including transmission delay, average data transfer rate and so on.

Net003: The telepresence system is recommended to adapt the quality of media based on the network status.

In case that telepresence system is attached into private network such as behind of NAT and firewall, it is hard to guarantee bi-directional multimedia session. There are several solutions, such as SBC, TURN, STUN, UPnP and ICE, for traversing NAT. In order to provide convenient operation of telepresence service, it should be easy to configure and use.

Net004: The telepresence system is required to provide a mean to traverse NAT or firewall automatically to assure connectivity of telepresence services with minimal user’s operation.

X.x Multimedia requirements

In contrast to traditional videoconference, since telepresence uses multiple display devices, it should be able to coordinate what stream should be delivered on specific displays. The use of multiple media I/O devices needs more considerations on describing and negotiating for telepresence session.

Mul001: The telepresence system is required to support a means to describe the attribute of session including aspect ratio, spatial relationship, and audio-visual characteristics.

Mul002: The telepresence system is required to support of communication regardless of different aspect ratio.

Besides of 2D video, telepresence may use 3-Dimensional displays and cameras for better immersive experiences. The 3-Dimensional video will have different characteristics and need another session for description method.

Mul003: The telepresence system optionally support 3D multimedia stream.

Mul004: The telepresence system optionally supports a means for describing 3-Dimensional multimedia stream.

X.x Security requirements

Since telepresence is used for confidential talks such as executive strategy meetings, it should be able to guarantee security of sessions. On securing telepresence session, signalling for session setup should be confidential as well as encryption of media session. In addition, data exchange such as file transferring and text should be confidential as well.

Sec001: The telepresence system is required to support of security mechanism for securing sessions.

Sec002: The telepresence system is required to support of negotiation mechanism for making a secured session.

Sec003: The telepresence system is required to support both signal security and media security.

Sec004: The telepresence system is required to support TLS for the signalling.

X.x Collaboration requirements

Telepresence service provides not only HD audio-visual communications but also collaboration functions to users. It should be possible to exchange electronic materials among participating users and give a presentation remotely. In order to collaborate remotely, online documentation is also helpful.

Col001: The telepresence system is required to provide a means of collaboration such as remote presentation and file transferring.

Col002: The telepresence system is recommended to provide a means of online collaboration such as online documentation and whiteboard.

X.x Environmental requirements

In order to provide immersive and realistic experiences to users, some environmental factors should be aligned. These environmental factors include interior design such as shape of desk, wall colour, illumination, position of light and so on.

Env001: The telepresence system is recommended to provide harmonized environment for assuring immersive presences of remote users.

[End of proposed text]

___________________