| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, 24, and 25/16 |  |  |  | TD-47 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editors |  |  |  |
| Title: | H.VSMarch “Architectural requirements for mobile visual surveillance” |  |  |  |
| Purpose: | Information |  |  |  |

This TD contains the clean version of the latest draft new ITU-T Rec. H.VSMarch "Architectural requirements for mobile visual surveillance" as output of the ITU-T Q.21/16 Rapporteur meeting which was held in Andover, 18 - 22 July 2011.

# 1.	Introduction

This new work item of Recommendation H.VSMarch is based on the architecture defined in the Recommendation ITU-T H.VSarch.

The Recommendation H.VSMarch will focus on the architecture of a visual surveillance system with mobile units, such as mobile customer unit or premises unit, as well as on the services flow related to mobile units, such as a mobile customer unit accessing real-time video stream from a visual surveillance system.

# CONTENTS

1	Scope	3

2	References	3

3	Definitions	4

3.1	Terms defined elsewhere	4

3.2	Terms defined in this Recommendation	5

4	Abbreviations	5

5	Conventions	6

6	Functional Requirements	6

6.1	Video/Audio Acquisition (VAA)	6

6.2	Instant Image Snapshot (IIS)	6

6.3	Recording and Playback (RnP)	6

6.4	PTZ Control	6

6.5	Alarming and Linkage Action (ALA)	6

6.6	Service Management	6

7	Functional Architecture	6

7.1	Functional Model	6

7.2	Functional Architecture Framework	6

7.3	Functional Entities	7

7.3.1	MSP	7

7.3.2	VAU	7

7.3.3	M_CU	7

7.4	Reference Points	7

7.4.1	MSP-CMU	8

7.4.2	MSP-SCU	8

7.4.3	MSP-M_CU	8

7.4.4	VAU– M_CU	8

7.4.5	VAU-SCU	8

7.4.6	VAU-MDU	8

7.4.7	VAU-PU	8

7.5	Service Control Flow and Signalling	8

7.5.1	Real-time Video/Audio Acquisition	8

7.5.2	Recording and Playback	8

7.5.3	PTZ Control	8

# Draft new ITU-T Rec. H.VSMarch

Architectural requirements for mobile visual surveillance

AAP Summary

[To be provided before Consent]

Summary

The purpose of this Recommendation is to define a visual surveillance system with mobile units. The mobile visual surveillance system provides the display and storage of the multimedia (such as video, audio, image) captured by multiple remote cameras over an IP network for mobile users. It also provides other functionalities such as remote control, alarming and linkage actions, recording and playback.

This recommendation defines the functional requirements, model, architecture, entities, reference points and service control flow of the mobile visual surveillance.

Keywords

Mobile Visual Surveillance, Camera, Alarming, PTZ Control, Recording and playback, Architecture

# Scope

The Visual Surveillance service is a telecommunication service focusing on video (audio included also) application technology, which is used to capture remotely multimedia (such as audio, video, image and various alarm signal) and present them to the end users with friendly manner, based on managed broadband network with ensured quality, security and reliability ensured.

The Recommendation H.VSMarch will focus on the architecture of a visual surveillance system with mobile units, such as mobile customer unit or premises unit, as well as on the services flow related to mobile units, such as a mobile customer unit accessing real-time video stream from a visual surveillance system. This Recommendation defines the functional requirements, model, architecture, entities, and reference points of the mobile visual surveillance system, based on the architecture defined in the Recommendation ITU-T H.VSarch.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T F.743]	Recommendation ITU-T F.743 (2009), Requirements and service description for visual surveillance

[ITU-T H.VSARCH]	Recommendation ITU-T H.VSARCH (2011), Architectural requirements for visual surveillance

# Definitions

## Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	Application [ITU-T Y.101]: A structured set of capabilities, which provide value-added functionality supported by one or more services.

3.1.2	Customer [ITU-T M.60]: An entity which receives services offered by a service provider based on a contractual relationship. It may include the role of a network user.

NOTE – In this document, Customer has the same meaning with End User.

3.1.3	Functional Architecture [ITU-T Y.2012]: A set of functional entities and the reference points between them used to describe the structure of an NGN. These functional entities are separated by reference points, and thus, they define the distribution of functions.

NOTE 1 – The functional entities can be used to describe a set of reference configurations.  These reference configurations identify which reference points are visible at the boundaries of equipment implementations and between administrative domains.

NOTE 2 – The definition is not only applicable to NGN, but also to other IP packet switch based network.

3.1.4	Functional Entity [ITU-T Y.2012]: An entity that comprises an indivisible set of specific functions. Functional entities are logical concepts, while groupings of functional entities are used to describe practical, physical implementations.

3.1.5	Interface [ITU-T Y.101]: A shared boundary between two functional units.

NOTE – An interface is defined by various characteristics pertaining to the functions, physical interconnections, signal exchanges and other characteristics as appropriate.

3.1.6	Media [ITU-T Y.101]: Plural of medium.

3.1.7	Medium [ITU-T Y.101]: a) Specific physical support for transmission or storage of information. b) Type of presentation of information (i.e., video, audio, text, etc.).

3.1.8	QoS [ITU-T Y.101]: The collective effect of service performances which determine the degree of satisfaction of a user of the service.

3.1.9	Reference Point [ITU-T Y.2012]: A conceptual point at the conjunction of two non-overlapping functional entities that can be used to identify the type of information passing between these functional entities.

NOTE – A reference point may correspond to one or more physical interfaces between pieces of equipment.

3.1.10	Role [ITU-T Y.110]: The role is a business activity which fits in a value chain. The role is constrained by the smallest scale of business activity which could exist independently in the industry and so a marketplace will exist for every relationship between roles.

3.1.11	Service [ITU-T Y.101]: A structure set of capabilities intended to support applications.

3.1.12 Customer Unit[ITU-T H.626]: A device located at the customer part of a Visual Surveillance system and used to present multimedia information (such as audio, video, image, alarm signal, etc) to the end user.

3.1.13 Premises Unit[ITU-T H.626]: A device located at the remote part of a Visual Surveillance system and used to capture multimedia information (such as audio, video, image, alarm signal, etc) from a surveilled object.

3.1.14 Service Platform[ITU-T H.626]: A series of devices and subsystems located at the centred part of a Visual Surveillance system. It is used to integrate all of the capabilities and provide visual surveillance services to customers. The main functions include service control function, media switching, distribution, storage, control and management functions.

3.1.15 Surveillant[ITU-T H.626]: Individual performing surveillance.

3.1.16 Surveilled Object[ITU-T H.626]: The target (such as site, human, and related environment) on which surveillance is performed.

3.1.17 Visual Surveillance[ITU-T H.626]: A telecommunication service focusing on video (but including audio) application technology, which is used to remotely capture multimedia (such as audio, video, image, alarm signal, etc) and present them to the end user in a user friendly manner, based on managed broadband network with ensured quality, security and reliability.

## Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1Mobile Customer Unit

3.2.2Mobile Service Portal

3.2.3Video Access Unit

# Abbreviations

This Recommendation uses the following abbreviations and acronyms:

| AAA | Authentication, Authorization and Accounting |
| --- | --- |
| CMU | Centre Management Unit |
| CU | Customer Unit |
| HTTP | Hypertext Transfer Protocol |
| MCU | Multimedia Control Unit |
| MDU | Media Distribution Unit |
| MSP | Mobile Service Portal |
| MSU | Media Storage Unit |
| M_CU | Mobile Customer Unit |
| PC | Personal Computer |
| PTZ | Pan/Tilt/Zoom |
| PU | Premises Unit |
| RTSP | Real-time streaming protocol |
| SCU | Service Control Unit |
| VAU | Video Access Unit |
| VS | Visual Surveillance |

# Conventions

# Functional Requirements

To provide the function for end users to use mobile terminals to access visual surveillance system, there are some new functional requirements for this system.

## Video/Audio Acquisition (VAA)

## Instant Image Snapshot (IIS)

## Recording and Playback (RnP)

## PTZ Control

## Alarming and Linkage Action (ALA)

## Service Management

# Functional Architecture

Editor’s note: This clause will describe the composed components structure of mobile visual surveillance system, which has been discussed above.

## Functional Model

## Functional Architecture Framework

The high-level functional components are described in Figure 1.

Figure 1: Components for mobile visual surveillance

- M_CU: Mobile Customer Unit, such as software clients running on mobile phones used by the visual surveillance end users.

- MSP: Mobile Service Portal, MSP provides the whole access portal for mobile surveillance users, including user authentication portal, PU list portal, etc. MSP provides service management functions for the end users through communication with SCU, such as user authentication, information querying, programme for surveillance content, synchronization of information with SCU, etc.

- VAU: Video Access Unit, VAU is the key unit to implement the communication between M_CU and the units defined in ITU-T Rec. H.626 (i.e. H.VSarch). The main functions of VAU include obtaining management information from SCU, creating connection with MDU, distributing and transcoding multimedia data, transferring commands of PTZ control, etc.

## Functional Entities

Editor’s note: This clause will describe all of the new entities which are added for mobile visual surveillance system, includes the composition and functions of the entities.

### MSP

### VAU

### M_CU

## Reference Points

Editor’s note: This clause will describe all of the reference points which are added for mobile visual surveillance system, includes the relationship of entities and the functions of reference points.

### MSP-CMU

### MSP-SCU

### MSP-M_CU

### VAU– M_CU

### VAU-SCU

### VAU-MDU

### VAU-PU

## Service Control Flow and Signalling

Editor’s note: This clause will describe the basic service procedure flow and the corresponding signalling and protocols different from those flows in ITU-T H.VSarch, the subsection category can be modified based on contributions in future meeting.

### Real-time Video/Audio Acquisition

### Recording and Playback

### PTZ Control

Bibliography

[b-ITU-T H.Sup.1]	ITU-T H-series Recommendations – Supplement 1 (1999), Application profile – Sign language and lip-reading real-time conversation using low bit rate video communication.

__________________