| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | TD-18a |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 25 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editors |  |  |  |
| Title: | F.OpenUSN “Requirements and reference architecture for open USN service framework” (New): Output draft (Geneva, 20 Feb. – 24 Feb. 2012) |  |  |  |
| Purpose: | Information |  |  |  |

This document contains revised text of draft Recommendation F.OpenUSN based on the discussion during WP2/16 interim meeting which was held on 20 – 24 February 2012.

The following table shows discussion results for contributions.

| Contribution No. | Contribution title and proposals | Agreements |
| --- | --- | --- |
| AVD-4193 | Proposed modification on F.OpenUSN |  |
|  | - Proposed new texts for clause 7 and 8.2 | - Agreed with 3 Editor’ NOTEs. |

CONTENTS

Page

1	Scope	3

2	References	4

3	Definitions	4

3.1	Terms defined elsewhere	4

3.2	Terms defined in this Recommendation	4

4	Abbreviations and acronyms	5

5	Conventions	5

6	Open USN service description and characteristics	5

7	Requirements of open USN service framework	7

7.1	Requirements to communicate with USN middleware	7

7.2	Requirements of Open USN Service Framework	8

7.3	Requirement for accessing LOD	8

7.4	Requirements for Application	8

7.5	Requirements for semantic USN data	8

8	Reference architecture of open USN service framework	8

8.1	Reference architecture	8

8.2	Functional entities	10

Appendix I  Use-cases for open USN services	12

I.1		Use case for already installed USN resources: fish farm monitoring service	12

I.2		Use case for dynamically installed USN resources: pet management service	13

Bibliography	14

Draft new Recommendation ITU-T F.OpenUSN

Requirements and reference architecture for open USN service framework

AAP Summary

To be provided before Consent.

Summary

This Recommendation describes requirements and reference architecture for open USN service framework.

Keywords

USN, semantic-based, open USN

Introduction

The ubiquitous sensor network (USN) is a well-known keyword in ICT area and many SDOs are developing standards for USN and other similar technologies.  However, USN services are not widely spread yet because current USN services require user or application developer to have knowledge of sensors and sensor networks for using USN services or developing USN applications.  For example, if a user wants to know the current temperature of Geneva, user should have the information about to which sensors or sensor networks he has to request the sensed data.  Also, the user has to know how to interpret sensed data he gets. It is the reason that each USN service use proprietary data format for sensor data, as well as proprietary application programming interfaces (APIs) for developing USN applications.

Even with USN middleware which can provide commonly required functionalities by many types of USN applications and services, user has to search, collect, analyse and process the sensed data by himself.  Also, many sensed data cannot be shared or reused by users or systems.  These are the limits of current USN services and by giving ability of catching the meaning of sensed data and ability of analysing the sensed data these limits can be overcame.

The support of an open USN service framework aims to provide flexible and efficient capabilities base on the use of standard interface to USN applications and services, thereby enabling USN applications to take full advantage of the USN capabilities such as users or application developers do not need any knowledge on sensors or sensor networks to use USN services or develop USN applications. The main purpose of open USN service framework is to provide:

- easy access to the global USN resource and sensed data

- easy installation and connection of low-cost sensors

- easy development and distribution of various applications

- easy use of the USN resource and sensed data

# 1	Scope

The objective of this Recommendation is to define an open USN service framework, and provide requirements, and reference architecture of open USN service framework. The use of standard interfaces of open USN service framework will ensure USN service reusability, portability across several USN services, as well as accessibility and interoperability by USN application providers and/or developers.

This Recommendation describes requirements and reference architecture for open USN service framework. The scope of this Recommendation includes:

- Concept of open USN service framework

- Requirements of open USN service framework

- Reference architecture of open USN service framework

- Functional entities of open USN service framework

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T Y.2201]	Recommendation ITU-T Y.2201 (2009), Requirements and capabilities for ITU-T NGN

[ITU-T Y.2221]	Recommendation ITU-T Y.2221 (2009), Requirements for support of Ubiquitous Sensor Network (USN) applications and services in the NGN environment

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	context awareness [ITU-T Y.2201]: A capability to determine or influence a next action in telecommunication or process by referring to the status of relevant entities, which form a coherent environment as a context.

3.1.2	sensor [ITU-T Y.2221]: Electronic device that senses a physical condition or chemical compound and delivers an electronic signal proportional to the observed characteristic.

3.1.3	sensor network [ITU-T Y.2221]: A network comprised of inter-connected sensor nodes exchanging sensed data by wired or wireless communication.

3.1.4	sensor node [ITU-T Y.2221]: A device consisting of sensor(s) and optional actuator(s) with capabilities of sensed data processing and networking.

3.1.5	ubiquitous sensor network [ITU-T Y.2221]: A conceptual network built over existing physical networks which makes use of sensed data and provides knowledge services to anyone, anywhere and at anytime, and where the information is generated by using context awareness.

3.1.6	USN middleware [ITU-T Y.2221]: A set of logical functions to support USN applications and services.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	open USN service: USN service to enable enhanced and flexible service interoperability and provisioning based on the use of standards interfaces.

Editor’s NOTE: The term “open” is not clear and need more clarification. Also, changing title of this Recommendation should also be considered.

3.2.2	USN resource: An entity which provides USN service including sensor, actuator, sensor node, sensor network and gateway.

Editor’s NOTE: This definition should be enhanced. Contributions are invited.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| API | Application Programming Interface |
| --- | --- |
| FE | Functional Entity |
| HTTP | Hypertext Transfer Protocol |
| LOD | Linked Open Data |
| RDF | Resource Description Framework |
| SPARQL | SPARQL Protocol and Query Language |
| URI | Universal Resource Identifier |
| USN | Ubiquitous Sensor Network |

# 5	Conventions

None

# 6	Open USN service description and characteristics

Current USN services require users or application developers to have knowledge of sensors and sensor networks for using USN services or developing USN applications. For example, multiple USN middleware are not easy to access by applications since each USN middleware have proprietary APIs and therefore it is also not easy to access various USN resources which directly attached different USN middleware.

Even the applications can have access to multiple USN middleware, applications have to search, collect, analyse and process the sensed data by themselves.

These are the limits of current USN services and these limits can be overcome by providing unified access method for USN resources via multiple USN middleware and by giving ability of analysing the sensed data based on the semantic of sensed data and USN resources.

Figure 1 shows current USN service framework and open USN service framework.

Figure 1 – Current USN service framework and open USN service framework

Editor’s NOTE 1– The detailed description on Figure 1 is required.

Figure 2 shows multiple USN middleware access provided by open USN service framework.

Figure 2 – Multiple USN middleware access by open USN service framework

Editor’s NOTE 2 – The detailed description on Figure 2 is required.

In open USN service framework, users or application developers do not need any knowledge on sensors or sensor networks to use USN services or develop USN applications.

The ultimate goal of the open USN service framework is to provide the users or application developers with the following services;

- easy access to the open USN service framework

- easy installation and connection of low-cost sensors

- easy development and distribution of various applications

- easy use of the USN resource and sensed data

# 7	Requirements of open USN service framework

The following are requirements of open USN service framework.

Editor’s NOTE – The following requirements are just initial list for discussion and further investigation is needed.

## 7.1	Requirements to communicate with USN middleware

- It is required to provide open interface for USN middleware to access USN resource and to provide sensed data.

- USN resource and sensed data are required to be identified by URI.

- Identifier for USN resource is required to be dynamically assigned when the USN resource is registered to the open USN service framework.

- The characteristics and status of USN resource is required to be managed.

- It is required to provide standardized interface for accessing various USN middleware.

## 7.2	Requirements of Open USN Service Framework

- USN resource management is required according to proper management policies on authentication, authorization and access rights.

- It is required to provide functionality of inheritance and binding of USN middleware management policy

- It is required to provide an inference function to derive the context data by users’ rules.

- It is required to provide a function to update the inferred data in the repository.

## 7.3	Requirement for accessing LOD

- It is required to access the data in Web service form based on HTTP by assigning unique URI to each of RDF data for linking the LOD.

## 7.4	Requirements for Application

- It is required to provide an open protocol for security of outgoing data by applications’ service request.

- It is required to manage logical group of USN resource according to applications’ service requests.

## 7.5	Requirements for semantic USN data

- USN resource and sensed data are required to be represented in RDF.

- It is required to provide functions to store, query, modify, and delete the RDF data in the repository.

- It is required to provide functions to search USN resource and sensed data by analysing intention of applications’ service requests.

- It is required to support standard query language such as SPARQL Protocol and Query Language (SPARQL).

# 8	Reference architecture of open USN service framework

## 8.1	Reference architecture

Figure 3 shows reference architecture of open USN service framework.

Figure 3 – Reference architecture of open USN service framework

TBD

Editor’s NOTE – The following description for FEs are just initial list for discussion and further investigation is needed.

## 8.2	Functional entities

### 8.2.1	Interface FE

Interface FE provides the functions which enables USN application to obtain open USN services and/or the sensed data from the open USN service framework.

Also, it supports the functions which allow establishment or maintenance of connections or disconnection according to the type of a request for data, and access control to handle access rights for user authentication and the use of services.

### 8.2.2	LOD linking FE

LOD linking FE provides the functions which enable users to access the semantic USN data in the world by opening the semantic USN data generated from the open USN service framework to the LOD, and to link between external LOD data and the semantic USN data from the open USN service framework.

Also, it supports the interface for query about the semantic USN data in the LOD, and the functions which allow the application and management of policies that include criteria about selection and publication of data to open to the LOD.

### 8.2.3	Semantic inference FE

Semantic inference FE provides the inference function based on the information descried in the ontology schema and users’ rules by using lots of RDF data stored in the Semantic USN repository.

Through the inference function, the original data are processed into more semantic data such as context data. The inferred data are updated into the Semantic USN repository and utilized in other services. Also, it provides the function to compose variously patterns and levels for inference.

### 8.2.4	Resource group management FE

Resource group management FE provides the function to configure and manage a logical group on USN resource for satisfying applications’ service requests.

On configuring a specific resource group, it can request information from the Semantic USN repository whether there is already the specific resource group, there is USN resource for creating the specific resource group, or not.

It performs the functions to create a resource group according to applications’ service request and to manage list of USN resource which belongs to the resource group. Also, it supports the functions to create, maintain and manage information such as the purpose of the resource group, makers, control with right and so on. It provides the function to manage the lifecycle of each resource group because there is a lifecycle of the resource group according to the duration of service.

### 8.2.5	RDF translation FE

RDF translation FE performs the function to translate automatically metadata of USN resource and raw sensed data collected from USN middleware into semantic USN data format in a form of RDF in real-time according to formal rules. There are translation rules to collect the data from USN middleware and to map the collected data to the ontology schema defined in the open USN service framework. The translated data are automatically stored in the Semantic USN repository.

### 8.2.6	Semantic USN repository FE

Semantic USN repository FE stores huge amount of semantic USN data for a certain period of time as a physical repository. Also, it provides API function to query for inserting new data, searching, querying, and deleting stored data.

### 8.2.7	Semantic query FE

Semantic query FE performs the function to handle middleware query and SPARQL query for providing responses to applications’ information requests. It consists of query analyser, middleware query processor and SPARQL query processor.

The query analyser provides the function to create queries by analysing intention of applications’ requests, to translate the results of query process according to applications’ message specifications, and to deliver the translated data to applications. It classifies queries that applications request into sensed data query, semantic USN data query, and query related to a resource group. The sensed data query, which requests the sensed data from USN resource through USN middleware, is created according to APIs that USN middleware provides. The semantic USN data query, which requests the semantic USN data from the Semantic USN repository, is created in SPARQL query form by translating queries that applications request into SPARQL. Also, this query analyser provides function to handle queries that request creation of a resource group, if the sensed data from a specific resource group are needed.

The middleware query processor performs the function to send sensed data queries to USN middleware, and to collect the resulted data from USN middleware. It provides the functions to manage query stats about lots of sensed data queries created from the query analyser, and to deliver the data received temporarily or periodically from USN middleware to RDF translation FE. However, in some cases such as real-time sensed data request, the sensed can be directly sent to the query analyser.

The SPARQL query processor performs the functions to handle simultaneously lots of SPARQL queries created from the query analyser, and to produce the outcome of query from the Semantic USN repository, and to deliver them the query analyser.

### 8.2.8	Resource ID management FE

Resource ID management FE provides functions to issue and manage the ID and URI of USN resource, and to manage mapping relations with the address of USN resource. Also, it supports the functions which enable USN resource to be automatically registered in the open USN service framework when USN resource is connected in the network such as Internet, and applications to obtain and utilize information about USN resource.

### 8.2.9	Resource catalog FE

Resource catalog FE provides the functions which enable USN resource to actively register its own status and connection information, and by using this information USN middleware or open USN service framework to support network connection and mobility of USN resource.

Resource ID management FE and Resource catalog FE support plug and play function which enables the open USN service framework to use dynamically USN resource due to USN resource which connects automatically to the open USN service framework and registers its own status and property information.

### 8.2.10	Adaptation FE

Adaptation FE provides the functions which handle the protocol and message for setting connection with USN middleware and delivering queries and commands as a kind of interface for processing several types of data generated from various USN middleware in the open USN service framework.

Also, it supports the function to translate the generated data from various USN middleware according to proper message specifications to deal with in the open USN service framework.

Appendix I

Use-cases for open USN services

(This appendix does not form an integral part of this Recommendation)

## I.1	Use case for already installed USN resources: fish farm monitoring service

Figure I.1 shows a use case for a fish farm monitoring service when it is assumed that USN resources for this service are previously installed and connected to open USN service framework.

The purpose of this service is to provide fish wholesalers with a kind of solution based on open USN service framework to figure out the followings: how to find and monitor tuna farming information in real-time and how to buy tuna raised in the cleanest environment.

Figure I.1 – Use case for already installed USN resources

The steps to accomplish this use case are:

- Query about a tuna farm monitoring service through the Interface FE by the Application

- Interpret query meaning by the Semantic query FE

- Discover requested USN resources from the Semantic USN repository FE (assume that B and C USN resources are discovered)

- Request sensed data to the USN middleware via the Adaptation FE by the Semantic query FE

- Collect sensed data from the USN middleware (in this case, from B and C USN resources) via the Adaptation FE

- Store sensed data, which are translated into RDF format by the RDF translation FE, on the Semantic USN repository FE

- Create inference data by the Semantic inference FE and store these data on the Semantic USN repository FE

- Provide contents including sensed and inference data by the Semantic query FE from the Semantic USN repository FE

## I.2	Use case for dynamically installed USN resources: pet management service

Figure I.2 shows a use case for a pet management service when it is assumed that users start to install USN resources for this service.

Figure I.2 – Use case for dynamically installed USN resources

The purpose of this service is to provide pet owners with a kind of solution based on open USN service framework to solve how to take care of their pet while they are away on a trip for a while.

The steps to accomplish this use case are:

- Install (or register) USN resources such as sensors (e.g., illuminant, humidity, gas, temperature sensor, camera, etc.) and actuators (e.g., electronic curtain/window, water/feeding machine, air conditioner, etc.)  to the open USN service framework by the USN middleware via the Adaptation FE as follows

  - Assign resource IDs of USN resources by the Resource ID management FE

  - Store metadata of USN resources collected through USN middleware, which are translated into RDF format by the RDF translation FE, on the Semantic USN repository FE

- Query about a pet management  service through the Interface FE by the Application

- Interpret query meaning by the Semantic query FE

- Discover requested USN resources from the Semantic USN repository FE (assume that these USN resources cannot be found)

  - Create logical groups of USN resources by the Resource group management FE

  - Store these data, which are translated into RDF format by the RDF translation FE, on the Semantic USN repository FE

- Remaining steps are same with from (4) to (8) step in the fish farm monitoring service.

Editor’s NOTE – The steps for remote control of actuators are required.

Bibliography

[b-ITU-T X.yyy]	ITU-T Recommendation X.yyy (date), Title

__________________