| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4263 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.RTCPPROF: Text input for clause 6 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to clause 6.

Discussion

Initial text is proposed, splitted in a brief summary for clause 6 and a new Appendix concerning more background information.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4234, the input draft to this Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 6	Relation to other ITU-T H.248.x-series Recommendations

Purpose of this clause is to identify possible relations (or not) to other Recommendations and past work on H.248 profiles. An overview and more background information is provided by Appendix IV. Below are the three major H.248 Recommendations with support of RTCP extension reports.

## 6.1	ITU-T Rec. H.248.30: RTCP extended performance metrics packages

This Recommendation is tightly coupled to [IETF RFC 3611].  The RTCP performance metrics are directly mapped on H.248 statistics. Near- and far-end measurements are supported.

## 6.2	Draft ITU-T proposal H.248.xnq: Extended Network Quality Metrics Packages for Next Generation Networks

This draft (see Appendix II) was developed in order to address the limitations of the [ITU-T H.248.30] defined measurement framework, as well as the IETF work on additional RTCP performance metrics. The work on the H.248 packages was completed, but the draft was not published by ITU-T in favour of a principle design change, which should be future proven and decoupled from IETF standardization development.

## 6.3	ITU-T Rec. H.248.48: RTCP XR Block Reporting Package

This Recommendation addresses the concerns of above mentioned limitations. The H.248 package and comprehensive measurement concept should obsolete above two Recommendations.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Appendix IV

Relation between “IETF RTCP XR” documents 
and ITU-T H.248.x work products

(This appendix does not form an integral part of this Recommendation)

## IV.1	Background

There were three major steps in ITU-T concerning the support of “RTCP XR”-based measurements and reporting. Figure IV.1 recalls again the timeline and evolution of H.248.x capabilities for support of RTCP XR.

Figure IV.1 – Timeline and evolution of H.248.x capabilities for support of RTCP XR

Figure IV.1 indicates the basic relation between ITU-T and IETF documents, however, the overall situation needs the consideration of other documents as well, see Figure IV.2.

Figure IV.2 – Link between “IETF RTCP XR” documents and ITU-T H.248.x work products

Figure IV.2 indicates three major areas:

- Performance parameters (solely defined by IETF; metric definition based RTCP XR template);

- Performance measurement & reporting variants (for decomposed gateways in the RTP/RTCP bearer plane; defined by [ITU-T H.248.30], [b-ITU-T H.248.xnq] and [ITU-T H.248.48])

- Complementary framework & capabilities (primarily for H.248.48-based performance monitoring models; such as [ITU-T H.248.71], [ITU-T H.248.79], [ITU-T H.248.LOOPB] and [ITU-T H.248.RTPTOPO])

Figure IV.2 indicates the difficulties for service providers, network operators or manufactures in identifying the right technology for a particular deployment scenario (“which is finally given by an H.248 profile specification”). For instance, it is not evident that there are self-contained H.248.48 solutions, or whether always a combination together with the H.248.71 rtcpsdes package would be required.

The next sub-clauses discuss typical capabilities in scope of H.248-based performance monitoring in RTP networks.

IV.2	Capabilities versus H.248.x packages support

This section tries to compare the different approaches of [ITU-T H.248.30], [b-ITU-T H.248.xnq] and [ITU-T H.248.48], their purposes, characteristics, but also principal limitations. The different H.248.x solutions satisfy different set of requirements (or support different capability sets). The capabilities may be structured in multiple areas. Table IV.1 indicates the main categories:

Table IV.1 – “RTCP XR”-based Performance Monitoring: 
Differences H.248.30 vs H.248.xnq vs H.248.48 – Part 1

| Capability |  | (1) H.248.30 | (2) H.248.xnq | (3) H.248.48 | Comments |
| --- | --- | --- | --- | --- | --- |
| A) Supported RTCP XR metric types |  | BT = 1 - 7 (limited) | BT = 8 (limited) | BT = ... (open) | A block type (BT) combines multiple performance metrics. |
| B) Supported RTP Topologies? |  |  |  |  | see also [ITU-T H.248.RTPTOPO] |
| B.1 | RTP Endsystem (RTPE) | Yes | Yes | Yes |  |
| B.2 | Back-to-Back RTP Endsystem (B2BRE) | Yes | Yes | Yes |  |
| B.3 | RTP Translators (RTPTT, RTPMT) | No | No | Yes | „RTP translator“ topology required for e.g. network peering |
| B.4 | RTP Mixer | Yes | Yes | Yes |  |
| C) H.248 MG as Reporting Point (RP) |  |  |  |  | via H.248 statistics … |
| C.1 | Only Near-End? | Yes | Yes | Yes |  |
| C.2 | Centralized RP for both directions? | No | No | Yes | ‚No‘ means that two MGs must be used, one for each direction |
| D) H.248 MG as Filtering Point (FP) |  | No | No | Yes | ‚No‘ because „not applicable“ |
| E) H.248 MG types (Connection Model) |  |  |  |  |  |
| E.1 | Physical-to-RTP (e.g. H.248 RMG, AMG, TMG) | Yes | Yes | Yes |  |
| E.2 | IP-to-IP (incl. RTP-to-RTP) | No | No | Yes |  |

The content of Table IV.1 is somehow reflecting the network evolution path of the past decade, from an initially dominated network architecture with an H.248 MG at the border between an IP network and circuit-oriented networks (such as PSTN, ISDN, 2G-PLMN) towards the direction of “all-IP networks” with it’s H.248 MGs as IP-IP gateway/router entities.

Table IV.2 provides some categories concerning technology status and profile availability.

Table IV.2 – “RTCP XR”-based Performance Monitoring: 
Differences H.248.30 vs H.248.xnq vs H.248.48 – Part 2

| Capability |  | (1) H.248.30 | (2) H.248.xnq | (3) H.248.48 | Comments |
| --- | --- | --- | --- | --- | --- |
| F) Part of H.248 Profile? |  |  |  |  |  |
| F.1 | Standardized Profile? | No | No | Not yet | [ITU-T H.248.48] is on the agenda for Border GWs .. |
| F.2 | Other Profiles? | - | - | - | Out of scope of this Recommendation |
| G) Non-H.248 Reporting? |  |  |  |  | of RTCP XR block types .. |
| G.1 | SIP | Yes | No | planned |  |
| G.2 | H.323 | Yes | No | open |  |
| G.3 | MGCP | Yes | No | open |  |
| G.4 | Diameter | No | No | open |  |

[End Proposed Correction]

_____________________