| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4304a |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | NEC |  |  |  |
| Title: | F.TPS-Reqs: Encoding profiles for interoperability |  |  |  |
| Purpose: | Discussion |  |  |  |

Summary

Interoperability requirements are described in Section 9 of F.TPS-Reqs.  This contribution describes architectural considerations on encoding profiles to enable a wider range of interoperability.

The best way on profiles under multi-vender and multi-point environment is to specify a single encoding profile (both video and audio) for telepresence systems and to enable none of capability exchanges on the profiles.

When telepresence systems interwork with legacy video conference systems in a conference, a baseline profile may be selected.  The baseline profile is needed to interwork all of video systems in the conference, but it is not efficient between telepresence systems in the same conference. 
This contribution here suggests employing dual profile operation.

# Introduction

A gateways or an MCU with full protocol stack conversion is often used as a solution for interoperability issues, but no conversion is better technically (less delay) and also economically.

There are several ways to enhance interoperability of both multi-vendor environment and multi-product-class (high-end and low-end products of a vendor).　The ways are (1) fewer options in standards, (2) less needs of capability exchange, (3) less protocol conversions, and (4) backward compatibility.

This contribution focuses on interoperability of application layer (media stream level).  It does not address interoperability issues of lower layers (physical, data-link, IP-network) and a transport layer (TCP/UDP/ RTP/ RTCP).  It is assumed that the interoperability of low and transport layers would be confirmed by connecting to an actual MCU to be used for a planned video conference.

Interoperability issues on SIP/SDP protocols are also barriers to provide the guaranteed interoperability.  It is expected to be resolved by IETF or other SG of ITU-T, not by Q5/16.

This contribution basically proposes less option in specifications for encoding profiles to minimize capability exchange and to ensure multi-vendors’ and multi-points’ interoperability.

# Discussion

## Fewer options in standards (to guarantee interoperability)

To use protocol convertors is a basic solution to guarantee interoperability, but it often opens new issues, such as increasing end-to–end transport time.  In order to minimize protocol conversion for interoperability, optional specifications should be minimized in standards.

### Video coding profile

The following table shows major H264 profiles including three new profiles approved in Jan/2012.

There are 21 profiles (17+1+3) in the H264 recommendation of ITU-T.  Some of them are used for storage-medias (CD, DVD) and some of them are used for broadcasting video sources.

As for video conferencing, 7 profiles above are feasible, but none of them has been explicitly specified for video conferencing, although BP (Baseline Profile) had been concerned for realtime applications.   However, 7 profiles are still too many to implement guaranteed-interoperability for multi-vendor, multi-point, and mixed (latest and legacy products) environment.

One of ways to enhance interoperability among video conference systems is to specify a single profile for video conference systems and to make no capability exchange among endpoints to verify a common video profile.

An issue here is which encoding profile should be selected as a single profile for video conferencing.  A conventional common profile for realtime applications is BP, but higher compression is needed for telepresence systems to transport multiple HD video streams.

A common profile adopted for multipoint conferences including both telepresence systems and legacy systems is usually the simplest, which all of endpoints (connected at the point of time) can decode.  This way, however, is not efficient especially when majority of endpoints uses telepresence systems and one endpoint is a legacy system.

Dual profile operation is a topic to be discussed, in order not to degrade a video profile among high-end products such as telepresence systems. Details are described in Section 4 (Supplement) of this contribution.

### Audio coding profile

Dual coding for audio protocols also needs to be considered. One is high quality protocol (e.g. G722 or MPEG4-AAC) among high-end products and the other is G711 which all of H.323 terminals are (basically shall be) capable of encoding and decoding.

Audio mixing has been used for multipoint conferences, but no mixing is needed when telepresence systems reconstruct spatial arrangement of audio sources of remote end points.

## Less needs of capability exchange

Capability exchange for a symmetric peer-to-peer conference is the easiest case, but other cases, especially an asymmetric and multipoint conference including both telepresence systems and legacy systems is the most complicated situation to harmonize capabilities among such heterogeneous systems.   The fundamental solution is to minimize needs of the capability exchange.  As for encoding profiles, the best way is to specify a single profile for telepresence systems without option/exception.

## Less protocol conversion

Full protocol conversions are a complete solution for interworking among systems but it is not a fundamental solution for multi-vendor environment.

## Backward compatibility

MPEG4 CODECs are basically updated by adding new profiles/specifications to existing profiles/ specifications.  Recently some of products have software CODECs which are updatable to include latest MPEG4 profiles.  However, all of existing products are not updatable on MPEG4 profiles.  Basically interoperability is to ensure interworking among system employing same specifications, but different generation of such specifications are often technically not possible to interwork without gateways for protocol conversion.

General aspect on interworking of H264-based products is shown below.

# Conclusion

Profile issues on interworking are reviewed.  It is suggested to minimize capability exchange by less option (or no option) on specifications for telepresence systems.  One way to implement guaranteed-interoperability is to specify a single encoding profile for telepresence systems.

As for interworking with legacy conference product, it is suggested dual profile operation for telepresence systems so as not to degrade quality among telepresence systems.

This contribution requests further contributions to other sector members of Q5/16 on this profile issue to guarantee interoperability to end-users under multi-vendor environment, including legacy conference systems.

# Supplement

The following is an example of a dual profile operation.

Thick arrow indicates H264-SCHP (in 2012) is used among H323/H264-based high-end systems.  When the high-end systems interwork with H264 (in 2010), H264 (in 2009), and H264 (in 2003), H264 BP is used.   When the high-end systems interwork with legacy H320 systems, gateways are employed for H323-H320 conversion and H264 BP is used between the high-end systems and gateways.  There is no transcoder in a MCU.

__________________