| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | TD-18 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.TPS-AV |  |  |  |
| Title: | H.TPS-AV "Audio/Video Parameters for Telepresence systems" (New): Input Draft |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document contains the Editor's  input draft (H.TPS-AV Ed. 0.8) for H.TPS-AV "Audio/Video Parameters for Telepresence systems" (New).

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.TPS-AV Ed. 0.1 | Initial baseline text. Output from Boston meeting July 2011. Based on AVD-4092, 4075, 4106. |
| H.TPS-AV Ed. 0.2 | Input text to the November/December ITU-T SG16 meeting. It is a copy of the output of the Boston meeting with changes marks accepted. |
| H.TPS-AV Ed. 0.3 | Output text from the November/December ITU-T SG16 meeting. It incorporates: C.694, C.747, C.748, C.749, C.709 and C.762. |
| H.TPS-AV Ed. 0.4 | Input text to the Geneva, Switzerland meeting February 2012. It is a copy of TD 0645R1/WP2 from the Geneva, Switzerland 21 November – 02 December 2011 meeting with the change marks accepted. |
| H.TPS-AV Ed. 0.5 | Output text from the Geneva meeting February 2012. It incorporates: AVD-4215, AVD-4216, AVD-4217, AVD-4218, and AVD-4219. |
| H.TPS-AV Ed. 0.6 | Input text to the SG 16 meeting April 2012. It is a copy of TD-15 from the Geneva, February 2012 meeting with the change marks accepted and editorial cleanup. TD 768/WP2. |
| H.TPS-AV Ed. 0.7 | Output Text from the Geneva Meeting April 2012. It incorporates: C.899, C.931, C.940 and agreed Requirements Terminology. |
| H.TPS-AV Ed. 0.8 | Input text to the Brisbane, Australia meeting September 2012. It is a copy of TD 0768R1/WP2 from the Geneva, Switzerland 30 April – 11 May 2012 meeting with the change marks accepted. |

Living List Items:

- All parameters that correspond to IETF CLUE information elements need to be periodically checked for accuracy and updated as needed.

- Text for acoustic parameters AudioLevel, ReverberationTime, BackgroundNoise, and SoundInsulation was sent to SG12 for action.  This item is kept as a reminder to consider their reply in future revisions.

- Addition of configuration parameters (number of displays, etc) might be useful for descriptive or conformance purposes.  More contributions are needed.

- Some parameters such as MaxMBPS may be already signalled (H.239, H.241). Parameter definitions need to be updated to indicate this.

CONTENTS

Page

1	Scope	4

2	References	4

3	Definitions	5

3.1	Terms defined elsewhere	5

3.2	Terms defined in this Recommendation	5

4	Abbreviations and acronyms	6

5	Conventions	6

6	H.323 Telepresence Parameter Usage	6

7	Telepresence endpoint parameters	7

7.1	Capture Related Parameters	8

7.2	Render related Parameters	20

7.3	Telepresence system environment parameters:	21

Annex A  <Annex Title - Placeholder>	24

Bibliography	24

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 1: Capture area axis (top view) 12 Figure 2: Capture area axis (3D view) 13

Draft new ITU-T H.TPS-AV

Audio/Video Parameters for Telepresence systems

AAP Summary

[To be provided before Consent]

Summary

This contribution defines the audio/video/environment parameters for telepresence system and gives the specific format definition for these parameters as well.

# 1	Scope

In a telepresence system, multiple media streams need to be captured, encoded, transported, decoded and rendered. Telepresence systems made by different vendors may have diverse solutions or implementations for similar solutions. These solutions or implementations may use disparate techniques to negotiate, describe and control media streams and cause interoperability problems. Some problems may be solved through translation or transcoding, but this requires additional devices and would introduce unnecessary delay and deteriorated performance.

In order to prevent the above problems the capabilities of the telepresence session can:

- Be negotiated between the telepresence end points (or other intermediaries as required) before media transmission in order to agree on a common set of capabilities.

- Assume default capabilities based on support of a common profile.

- A combination of the above.

For example, a receiver could send its audio/video capabilities parameters to the sender through the capability negotiation. The sender would coordinate and send media streams to the receiver according to the receiver's capabilities.

This Recommendation provides details of telepresence endpoint characteristics and how these relate to telepresence in [ITU-T H.323] systems.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[F.TPS-Reqs]	ITU-T Recommendation X.yyy (date), Definitions, requirements, and use cases for Telepresence Systems.

[F/H.TPS-Arch]	ITU-T Recommendation X.yyy (date), Telepresence System Architecture.

[ITU-R BT.709]	ITU-R Recommendation BT.709-5 (04/2002), Parameter values for the HDTV standards for production and international programme exchange.

[ITU-T H.245]	ITU-T Recommendation H.245 (05/2011), Control protocol for multimedia communication.

[ITU-T H.264]	ITU-T Recommendation H.264 (01/2012), Advanced video coding for generic audiovisual services.

[ITU-T H.323]	ITU-T Recommendation H.323 (12/2009), Packet-based multimedia communications systems.

[ITU-T P.56]	ITU-T Recommendation P.56 (12/2011), Objective measurement of active speech level.

[ITU-T P.300]	ITU-T Recommendation P.300 (11/2001), Transmission performance of group audio terminals (GATs).

[ITU-T Q.931]	ITU-T Recommendation Q.931 (05/1998), ISDN user-network interface layer 3 specification for basic call control.

[IETF CLUE FW]		IETF draft-ietf-clue-framework-04, Framework for Telepresence Multi-Streams.

[IETF RFC 4796]		IETF RFC 4796 (2007), The Session Description Protocol (SDP) Content Attribute.

[ISO 11664-2]		ISO 11664-2 (2007) / CIE S 014-2/E (2006), Colorimetry - Part 2: CIE standard illuminants.

[ISO 140-4]		ISO 140-4 (1998), Acoustics - Measurement of sound insulation in buildings and of building elements - Part 4: Field measurements of airborne sound insulation between rooms.

[ISO 717-1]		ISO 717-1 (1996), Acoustics – Rating of sound insulation in buildings and of building elements – Part 1: Airborne Sound Insulation.

[P.supp 16]		ITU-T Recommendation Series P Supplement 16 (11/1988), Guidelines for placement of microphones and loudspeakers in telephone conference rooms and for group audio terminals (GATs).

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	 Camera-Left and Right [IETF CLUE FW]: For media captures, camera-left and camera- right are from the point of view of a person observing the rendered media.  They are the opposite of stage-left and stage-right.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Capability: A terminal has a particular capability if it is able to encode and transmit or receive and decode that particular signal. A capability may also relate to how a terminal captures or renders signal.

3.2.2	Cycles Per Degree: The number of alternating white and black lines that can be captured or rendered per degree of visual field.

{Editor's note: In defining parameters related to capabilities it is important to understand in the general sense what a capability may be. It is expected that this definition will evolve. Further definitions may be required.}

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| 3D | Three dimensional |
| --- | --- |
| CCT | Correlated colour temperature |
| CPD | Cycles Per Degree |
| CRI | Colour Rendering Index |
| RAS | Registration, Admission and Status |
| RTP | Real Time Protocol |
| RTCP | RTP Control Protocol |
| R'w | Weighted apparent sound Reduction index |
| Rw | Weighted sound Reduction index |
| STC | Sound Transmission Class |
| VUI | Video Usability Information |

# 5	Conventions

<Describe any particular notation, style, presentation, etc. used within the Recommendation, if any>

## 5.1	Requirement terminology

In this Recommendation the following conventions are used:

–	"Shall" indicates a mandatory requirement.

–	"Should" indicates a suggested but optional course of action.

–	"May" indicates an optional course of action rather than a recommendation that something take place.

{Editor's note: The text is not yet aligned with this terminology. A contribution from the editor is expected at the next meeting to address this.}

# 6	H.323 Telepresence Parameter Usage

As shown in clause 9 / [F/H.TPS-Arch], a H.323 based telepresence endpoint may communicate with another endpoint using a "peer to peer" or "multi-point communication" mode. In either of these modes the establishment of a telepresence session follows three main phases:

- Call setup phase: including RAS and [ITU-T Q.931] call signalling.

- Media setup phase: including mast-slave determination, capability negotiation and "logical channel" opening through [ITU-T H.245] control signalling.

- Media control phase: including media stream transmission and control using RTP/RTCP and H.245 control signalling.

This Recommendation details parameters that are used to describe telepresence endpoint characteristics which are used in the media setup phase during capability negotiation. The characteristics may be explicitly signalled or a default (not requiring signalling) may be assumed based on the support of this Recommendation.

A telepresence endpoint may have different capabilities related to audio, visual and environmental characteristics. For example these characteristics may include:

- The number of cameras, spatial relationships of cameras and capture capabilities, such as resolution, frame rate and aspect ratio, etc.

- Viewpoint and field of view for cameras include overlap regions, missing regions or whether images of cameras can be composed to a seamless image in whole depth ranges.

- The number, type, size of displays,  spatial relationships of displays and display capabilities, such as resolution, frame rate, aspect ratio, and display border width, etc.

- The number and type of presentation displays, spatial relationships of presentation displays and display capabilities.

- Spatial relationships of microphones and parameters of microphones.

- Spatial relationships of speakers and parameters of speakers.

- System environment parameters, such as colour temperature, etc.

As shown in Figure 10 / [F/H.TPS-Arch] there are a number of interfaces (A1, G1 and G2) where negotiation related to media associated with a telepresence end point may take place. Thus parameters such as those shown in the examples above may need to be transported across these interfaces.

In order that each telepresence endpoint (or intermediate device) can understand what a parameter means it need to be described in detail. Section 7 below provides a list of parameters related to telepresence endpoints.

# 7	Telepresence endpoint parameters

{Editor's note: The following structure has been proposed in order to present the parameters in a technology / solution agnostic manner to focus on the information needed rather than any particular coding or signalling (i.e. ITU-T H.239). As the list of parameters matures it is expected that addition sections will be added (or clause 7 will be modified) to discuss coding / signalling of these parameters.}

This section indicates describes the parameters related to a telepresence endpoint in more detail. The parameters are described according to the format below:

| Identity: | A unique alphanumeric identifier in this Recommendation. |
| --- | --- |
| Description: | A description of the function of the parameter. |
| Format: | The type of parameter, i.e. Boolean, alpha numeric string, integer, sequence of octets, character or enumeration. |
| Possible Values: | The parameter must specify which values can be transported. |
| Default: | Indicate if there is a default that may be assumed. |
| Reference: | Where a parameter has previously been defined a reference to it may be included. |
| Interface: | If the parameter is only applicable to a sub-set of interfaces (A1, B1, G1, G2) defined in section 9.1 / [F/H.TPS-Arch] it should be indicated here. |
| Supplementary Information: | Further information to the use or definition of the parameter. |

NOTE - The text in italics should be replaced according to the description.

{Editor's note: The following parameters are placed here as a placeholder only to facilitate further contribution. These parameters may be described or signalled via already existing Recommendations/standards. Further contribution is needed on them:}

## 7.1	Capture Related Parameters

This clause describes parameters related to the capture of audio and media.

### 7.1.1	General Parameters

#### 7.1.1.1	Media capture content

| Identity: | MedCapContent |
| --- | --- |
| Description: | This parameter indicates what the media capture is used for. |
| Format: | Enumeration. |
| Possible Values: | The values as per the mediacnt values for the content attribute in [IETF RFC 4796]. |
| Default: | main |
| Reference: | - |
| Interface: | - |
| Supplementary Information: | This parameter aligns with the "Content" attribute in [IETF CLUE FW]. |

#### 7.1.1.2	General Spatial Information

The parameters in this clause relate to all types of media captures. It is assumed that all the spatial information parameters share a common origin.

##### 7.1.1.2.1	Scene Area

| Identity: | SceneArea |
| --- | --- |
| Description: | This parameter indicates the area of the overall scene related to the endpoint. It may be larger than what is described by the individual media "Capture Area" parameters. |
| Format: | List of String |
| Possible Values: | 4 list elements each with (X, Y, Z) Where X, Y and Z are numbers. |
| Default: | None. |
| Reference: | See "Area of Scene" in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | This parameter allows a renderer to determine from what area in an overall scene a capture comes from in order to render a media stream correctly. |

##### 7.1.1.2.2	Area Scale

| Identity: | AreaScale |
| --- | --- |
| Description: | This parameter indicates the type of scale that is being applied to the spatial information parameters |
| Format: | Enumeration |
| Possible Values: | "millimeters" The metric system with the millimetre unit is being applied. "unknown" An unknown scale is being used that is the same for all captures in the capture scene entry. This may be used to indicate relative distances. "No scale" This indicates that the scale may be different for each capture in a capture scene entry. <Other?> |
| Default: | <TBD> |
| Reference: | See "Area Scale" in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

#### 7.1.1.3	General Encoding Information

##### 7.1.1.3.1	Overall maximum bandwidth

| Identity: | maxGroupBandwidth |
| --- | --- |
| Description: | This parameter indicates the maximum number of bits per second relating to all encoded media of a particular type (audio or video) sent from the endpoint. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See "maxGroupBandwidth" in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

##### 7.1.1.3.2	Overall maximum macroblocks per second

| Identity: | maxGroupMpbs |
| --- | --- |
| Description: | This parameter indicates the maximum number of H.264 macroblocks per second for all encoded video sent from the endpoint. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See "maxGroupH264Mbps" in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

#### 7.1.1.4	Composed

| Identity: | Composed |
| --- | --- |
| Description: | This parameter indicates that the media capture has been composed from several inputs. |
| Format: | Boolean |
| Possible Values: | "No" The capture is not composed. "Yes" The capture is composed. |
| Default: | No |
| Reference: | See the "Composed" attribute in [IETF CLUE FW]. |
| Interface: | - |
| Supplementary Information: | - |

#### 7.1.1.5	Auto switched

| Identity: | Auto switched |
| --- | --- |
| Description: | This parameter indicates that the content of the media capture depends on a particular rule. For example the video is from the "loudest speaker" determined by some algorithm. |
| Format: | Boolean |
| Possible Values: | "No" The content is not based on a rule. "Yes" Rule based content. |
| Default: | No |
| Reference: | This parameter aligns with the "Auto-switched" attribute in [IETF CLUE FW]. |
| Interface: | - |
| Supplementary Information: | - |

### 7.1.2	Visual parameters

This clause describes the telepresence parameters related to the visual capabilities.

#### 7.1.2.1	Video Colour Gamut

| Identity: | ColourGamut |
| --- | --- |
| Description: | This parameter indicates the Colour Gamut used in a Telepresence Video Stream. |
| Format: | TBD |
| Possible Values: | BT.709, BT.601, BT.1361 |
| Default: | BT.709. |
| Reference: | None. |
| Interface: | None. |
| Supplementary Information: | In the absence of visual usability information (VUI) signalling (or if the VUI signalling indicates "unspecified"), telepresence systems shall process all received video signals per [ITU-R BT.709]. For example: Annex E.2 / [ITU-T H.264] regarding VUI has the "colour primaries" element that allows the signalling of the chromaticity and white point. If the transmitted video is not conformant with [ITU-R BT.709], senders should always signal the black level and range of the luma and chroma signals, transfer characteristics, the colour primaries, and the matrix coefficients in their video bit streams. Receivers may use this information to optimally render the received pictures. See clause 7.3.1.4 for more information on the use of colour and lighting parameters. |

#### 7.1.2.2	Effective Resolution

Human visual acuity is the ability to distinguish fine detail.  It is generally measured in cycles per degree (CPD), and therefore measures an angular resolution.  People with excellent visual acuity can resolve 50 CPD – 1.2 arc minute per line pair (or 0.35 mm line pair at 1 meter).  [b-ImagePro]).Camera resolution is usually measured in lines across the entire horizontal or vertical field of view.  Black and white lines are alternated.  A cycle is equivalent to 2 lines of camera resolution – that is, a black+white line pair comprises a cycle.  The field of view of a camera is measured in degrees.  Therefore the cycles per degree for a camera can be computed as lines/(2*field_of_view).

Similarly display resolution is measured in pixels (width by height).  For modern flat panel displays, a line is equivalent to a row or column of pixels.  The cycles per degree for a display depends on the subtended angle of the display, which in turn depends on the size of the display and the distance of the display surface from the viewer.  Therefore, the cycles per degree for a display can be computed as pixels/(2*subtended_angle).

##### 7.1.2.2.1	Example

A typical 3-screen telepresence system uses 3 cameras with horizontal field of view of 25 degrees.  It uses 65 inch 16x9 displays, and is calibrated to show images at actual size.

Each display has a width of 56.65 inches (1439 mm), and the viewers are seated 3.245 meters from the display.  At this distance, the horizontal subtended angle is also 25 degrees, matching the camera field of view.

The native resolution of each display is 1920 x1080 pixels (1080p); which corresponds to 1920/(2*25) cycles per degree – or about 38.4 CPD.

Similarly, 720p corresponds to about 24 CPD. A 65" 4K display at this viewing distance provides about 71 CPD, which exceeds the resolving power of the human eye.

| Identity: | EffectiveResolution |
| --- | --- |
| Description: | This parameter indicates effective resolution of a rendered video stream as perceived by the viewer. |
| Format: | Floating Point number |
| Possible Values: | Positive numbers in Cycles per Degree |
| Default: | None. |
| Reference: | Not Application |
| Interface: | None. |

Telepresence systems should provide effective resolutions between 25 CPD and 50 CPD.

#### 7.1.2.3	Number of video captures

| Identity: | NumVideoCapture. |
| --- | --- |
| Description: | This parameter indicates the number of video captures. |
| Format: | Not applicable. |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | - |
| Interface: | - |
| Supplementary Information: | This parameter may be implicitly signalled as a result of describing multiple captures. For example each video capture in [IETF CLUE FW] represents a video stream. Therefore the total number of video captures in a capture scene entry indicates the number of video streams. |

#### 7.1.2.4	Video capture spatial information

This clause describes parameters related to spatial information of a single video capture.

{Editor's note: The details of the parameters below are subject to change due to discussions in the CLUE WG on spatial information.}

##### 7.1.2.4.1	Capture area

| Identity: | CapArea |
| --- | --- |
| Description: | This parameter indicates which area the video capture represents from the overall capture scene in terms of a set of four X,Y and Z Cartesian coordinates. |
| Format: | List of String |
| Possible Values: | 4 list elements each with (X, Y, Z) minX:maxX, minY:maxY , minZ:maxZ Where X, Y and Z are numbers. |
| Default: | None. |
| Reference: | See "Area of Capture" in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | This parameter is used to indicate the area of the video capture, and is also used to determine the spatial relationship between video captures. It is also possible to determine from the capture area information where gaps are present due to display borders etc. which should be taken into consideration by a renderer when rendering video streams. |

###### 7.1.2.4.1.1	Example usage of the capture area parameter

Capturers send the Capture Area parameters renderers, in terms of a set of four X, Y and Z coordinates, respectively. For example,the coplanar capture area is represented by for point bottom left (X1, Y1, Z1), bottom right (X2. Y2, Z2), top left (X3, Y3, Z3) and top right (X4 ,Y4, Z4). Following the conventions of "Area of Capture" [IETF CLUE FW], as numbers move from lower to higher, the location goes from camera left to right (in the case of the 'x' dimension), audience front to back (in the case of the 'y' dimension ) or low to high (in the case of the 'z' dimension ), which is shown in figure 1(top view) and figure 2 (3D view).

Figure 1: Capture area axis (top view)

Figure 2: Capture area axis (3D view)

NOTE - The "Capture point", "Scene Area" or origin parameters are not considered in the above diagrams.

On reception of the Capture Area information renderers may use this information in several ways to render the received video streams illustrated below. The examples below only discusses the x-axis for simplicity, the calculations may equally be applied to the Y and Z axis. A scale of millimetres is assumed.First, the capture area can be used to indicate the spatial information of the video capture. For example, a capture of Region A with an area of capture:

bottom left    	bottom right  		top left         		top right

Region A 	(0,2000,0) 		(980,2200,0) 		(0,2000,600) 	(980,2200,600)

The capture area above indicates that in terms of the X-axis the width of 980mm. Considering the Y coordinates this leads to a planer width of 1000mm. Thus the renderer can use this information to render the pictures in correct spatial place.

Second, the capture area can be used to describe the spatial relationship between video captures. For example if we consider the above capture of Region A and add a capture for Region B below:

bottom left    	bottom right  	top left         	top right

Region B 	(990,2200,0) 	(1990,2200,0) 	(990,2200,600) 	(1990,2200,600)

By analysing the capture area of Region A and Region B, respectively, the spatial relationship of the two video captures, that Region A  is to the left of Region B and Region B is to the right of Region A. Therefore the renderer can use this information to render the pictures in correct spatial arrangement.

Third, the capture area can be used to determine the gap between video captures. For example, by analysing the respective X coordinates (i.e. 980 and 990) it can be seen that there is a gap of 10mm between Regions A and B. The renderer can use this information to render the video streams in a continuous manner. That is, if the width of the gap is larger than the width of the borders between the displays on the receiver side, a virtual display border (For example, a black zone) can be added to the captured images to achieve visual continuity. If the width of the gap is smaller than the width of the borders between the displays on the receiver side, the captured images can be cropped and scaled to achieve a continuous vision.

##### 7.1.2.4.2	Capture point

| Identity: | CapPoint |
| --- | --- |
| Description: | This parameter indicates where in the capture scene the video was captured from. |
| Format: | String |
| Possible Values: | X, Y, Z Where X, Y and Z are numbers. |
| Default: | None. |
| Reference: | See "Point of Capture" in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

#### 7.1.2.5	Video capture encoding information

This clause describes parameters related to encoding information of a single video capture.

##### 7.1.2.5.1	Maximum Video Bandwidth

| Identity: | maxVideoBandwidth |
| --- | --- |
| Description: | This parameter indicates the maximum number of bits per second relating to a single video encoding. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See "maxBandwidth" in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

##### 7.1.2.5.2	Maximum H.264 macroblocks per second

| Identity: | maxH264Mpbs |
| --- | --- |
| Description: | This parameter indicates the maximum number of H.264 macroblocks per second relating to a single video encoding. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See "maxH264Mbps" in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

##### 7.1.2.5.3	Maximum video resolution width

| Identity: | maxWidth |
| --- | --- |
| Description: | This parameter indicates the maximum video resolution width in pixels. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See "maxWidth" in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

##### 7.1.2.5.4	Maximum video resolution height

| Identity: | maxHeight |
| --- | --- |
| Description: | This parameter indicates the maximum video resolution width in pixels. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See "maxHeight" in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

##### 7.1.2.5.5	Maximum video framerate

| Identity: | maxFrameRate |
| --- | --- |
| Description: | This parameter indicates the maximum video framerate. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See "maxFrameRate" in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

### 7.1.3	Audio parameters

This clause describes the telepresence parameters related to the audio capabilities.

#### 7.1.3.1	Number of audio captures

| Identity: | NumAudioCapture. |
| --- | --- |
| Description: | This parameter indicates the number of audio captures. |
| Format: | Not applicable. |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | - |
| Interface: | - |
| Supplementary Information: | This parameter may be implicitly signalled as a result of describing multiple captures. For example each audio capture in [IETF CLUE FW] represents a audio stream. Therefore the total number of audio captures in a capture scene entry indicates the number of audio streams. |

#### 7.1.3.2	Audio capture spatial information

This clause describes parameters related to spatial information of a single audio capture.

As per "Capture area" and "Capture point" parameters in clause 7.2.2.4 except the spatial parameters relate to audio instead of video.

#### 7.1.3.3	Audio capture encoding information

This clause describes parameters related to encoding information of a single audio capture.

##### 7.1.3.3.1	Audio Channel Format

| Identity: | AudioChannelFormat |
| --- | --- |
| Description: | This parameter indicates the meaning of the channels is determined. E.g. the number of channels may be determined. |
| Format: | Enumeration |
| Possible Values: | Mono The audio capture a single mono channel. Stereo The audio capture has two channels (left and right) in a stereo format. |
| Default: | None. |
| Reference: | See "Audio Channel Format" in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

##### 7.1.3.3.2	Maximum Audio Bandwidth

| Identity: | maxAudioBandwidth |
| --- | --- |
| Description: | This parameter indicates the maximum number of bits per second relating to a single audio encoding. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See "maxBandwidth" in clause 6.3.2/[IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

#### 7.1.3.4	Audio Level

| Identity: | AudioLevel |
| --- | --- |
| Description: | This parameter indicates the audio level sent in the Telepresence audio stream. |
| Format: | dBov |
| Possible Values: | -31 dBov |
| Default: | -31 dBov |
| Reference: | Not Application |
| Interface: | None. |

Telepresence systems shall send audio to other telepresence systems with an average audio level of AudioLevel dBov as measured by [ITU-T P.56].

When a telepresence system connects to non-telepresence devices, it should use theAudioLevel expected by that device.  If this is unknown, it should use the Telepresence AudioLevel.

{Editor's note :  -31 dBov needs more discussion, especially in the light of the codec overload point assumed.}

#### 7.1.3.5	Send Loudness Rating

| Identity: | SendLoudnessRating |
| --- | --- |
| Description: | This parameter indicates the acoustic to electric transfer function of the Telepresence audio stream. |
| Format: | dB |
| Possible Values: | TBD |
| Default: | TBD |
| Reference: | Not Application |
| Interface: | None. |

Telepresence systems shall send audio to other telepresence systems with a SendLoudnessRating of X dB, measured in accordance with [ITU-T P.300].

#### 7.1.3.6	Receive Loudness Rating

| Identity: | ReceiveLoudnessRating |
| --- | --- |
| Description: | This parameter indicates the electric to acoustic transfer function of the Telepresence audio stream. |
| Format: | dB |
| Possible Values: | TBD |
| Default: | TBD |
| Reference: | Not Application |
| Interface: | None. |

Telepresence systems shall receive audio from other telepresence systems with a nominal ReceiveLoudnessRating of Y dB, measured in accordance with [ITU-T P.300].

{Editor's note :  ReceiveLoudnessRating would appear to be a Render related parameter, in which case it is misplaced. It also to be confirmed whether this parameter is needed.}

#### 7.1.3.7	weightedMicrophoneLoudspeakerCouplingLoss

| Identity: | weightedMicrophoneLoudspeakerCouplingLoss. |
| --- | --- |
| Description: | This parameter indicates the weightedMicrophoneLoudspeakerCouplingLoss between microphones and loudspeakers. |
| Format: | Numeric, dB. |
| Possible Values: | ≥ X dB. |
| Default: | None. |
| Reference: | - |
| Interface: | - |
| Supplementary Information: | Maximizing the weightedMicrophoneLoudspeakerCouplingLoss of the system facilitates the work of the Acoustic Echo Canceller, hence guaranteeing full-duplex behaviour. Note: This can only be measured for static microphones. {Editor’s note: We need text describing how to measure it (it needs to be measurable without the codec).} |

## 7.2	Render related Parameters

Further contributions are likely to document parameters like the following:

- The number and arrangement of displays

- The number and spatial arrangement of loud speakers.

## 7.3	Telepresence system environment parameters:

### 7.3.1	Colour and lighting parameters

#### 7.3.1.1	Illuminant

| Identity: | Type. |
| --- | --- |
| Description: | This parameter describes the profile of the visible light at a telepresence endpoint. It is characterised by a CIE standard illuminant value. |
| Format: | Enumeration. |
| Possible Values: | "Dx" Illuminant representing Daylight (x is in integer, i.e. D65 Default, D50, 55, 75). "E" Illuminant representing an equal energy radiator. "FLx" Illuminant representing fluorescent lamp (x is an integer, i.e. FL1, FL2, FL12 etc). "FL3.x" Illuminant representing a three band fluorescent lamps (x is an integer, i.e.FL3.1, FL3.14). "HPx" Illuminant representing a high-pressure lamp spectra (x is an integer, i.e. HP1 Standard high pressure lamp HP2 Colour enhanced high pressure sodium lamp HP3to HP5 High Pressure metal halide lamps) |
| Default: | None. |
| Reference: | [b-CIE 15.3:2004] and [ISO 11664-2] |
| Interface: | - |
| Supplementary Information: | If the CRI value and colour temperature are be omitted they are implied by the type. For example, type "A" indicates that the light source is tungsten-filament lighting with CRI of more than 95 and colour temperature of 2856K. |

#### 7.3.1.2	Colour Rendering Index

| Identity: | CRI. |
| --- | --- |
| Description: | This parameter describes the colour rendering index (CRI) of the visible (ambient) light at the telepresence endpoint. The method of calculation of the CRI is according to [b-CIE 13.3-1995]. |
| Format: | Numeric. |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | [b-CIE 13.3-1995] |
| Interface: | - |
| Supplementary Information: | If the light source is a standard CIE illuminant, the CRI value can be omitted. |

#### 7.3.1.3	Colour temperature

| Identity: | ColTemp. |
| --- | --- |
| Description: | This parameter describes the correlated colour temperature (CCT) [b-CIE 17.4-1987] of the visible (ambient) light at the telepresence endpoint. |
| Format: | Numeric, degrees Kelvin. |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | - |
| Interface: | - |
| Supplementary Information: | If the light source is a standard CIE illuminant, the CCTvalue can be omitted. |

#### 7.3.1.4 Colour and lighting parameter usage

One of the goals of telepresence is to provide a "being there" experience. One of the aspects of the human vision system is to perform chromatic adaptation based on the ambient lighting conditions so that objects appear to have the same colour under various lighting conditions. Therefore people at the same location would generally perceive scenes in the same way. However in telepresence systems people are at disparate locations with different lighting conditions. Therefore a local scene would be perceived to be a different colour by people at the remote end to the local people.

Capture devices such as cameras can perform "white balancing" on the captured images so that the colour more closely correlates to what a person sees. In transforming the image colour the device must estimate the ambient light (illuminant) and its white point in order to scale the image colours correctly. The "type of lighting", "colour rendering index" and "colour temperature" are local environmental parameters that may be utilised for white balancing colour enhancement.

When an image is transmitted to the remote end and displayed it will in many cases be under a different ambient light. As discussed above, if the colours of the image and luminance of the image are perceived by the remote participants as being different to the local conditions then the "being there" experience is diminished. In order to maximize the "being there" experience the display equipment may transform the colours of the image to take into account the local ambient light (illuminant) and white point and chromaticity of the received image. The white point and chromaticity of the received image may be based on pre-agreed profile (i.e. the support of [ITU-R BT.709 which includes a D65 illuminant) or through in-band signalling such as the VUI parameters in [ITU-T H.264]. The "Video colour gamut" parameter (clause 7.1.2.1) provides a means for the communication of this information.

{Editor's note : Reverbation time, Background Noise, and Sound Insulation should probably be grouped into their own section (similar to lighting)}

### 7.3.2	Reverberation time

Reverberation time should be typically between 0.3 and 0.4 seconds, but at a minimum of 0.2 seconds, when measured in the 125 Hz-4 kHz octave bands. Such values would apply to medium size meeting rooms with a volume of approximately 100 m3 or less. For larger meeting rooms, a slightly higher reverberation time would be acceptable.

This represents somewhat lower reverberation times than it is usual in meeting rooms today, but these values are absolutely attainable in practice.

The audio quality will worsen gradually with increasing reverberation times (T) above 0.4 s.

| Identity: | ReverberationTime |
| --- | --- |
| Description: | This parameter indicates the reverberation time in a telepresence meeting room. |
| Format: | T [s] |
| Possible Values: | between 0.3 s and 0.4 s |
| Default: | none |
| Reference: | See clause 3 / [P.supp 16] for further information regarding reverberation considerations. |
| Interface: | none |

### 7.3.3	Ambient noise

Care should be taken to minimize the ambient noise level, including both noise from technical installations (air-conditioning, in particular) and any other external noise transmitted into the room. The ambient noise is measured as an Equivalent A-weighted Sound Pressure Level (LAeq).For a high quality experience, the noise level should be less or equal to 30 dBA.

This can be a difficult goal to meet in some buildings, and most telepresence systems will work well with higher ambient noise levels like 35 dBA. However, the quality of experience will in general gradually decrease with increasing noise level.

| Identity: | AmbientNoise |
| --- | --- |
| Description: | This parameter indicates the Equivalent A-Weighted Sound Pressure Level in a telepresence meeting room environment measured at all relevant seating positions. |
| Format: | LAeq [dBA] |
| Possible Values: | <= 30 dBA |
| Default: | none |
| Reference: | See clause 3 / [P.supp 16] for further information regarding ambient noise. |
| Interface: | none |

### 7.3.4	Sound insulation

Sound insulation between rooms is rated with a “Weighted apparent sound Reduction index” (R’w) which can only be measured in the field (finished buildings). Specific building elements are rated with a “Weighted sound Reduction index” (Rw) and can only be measured in laboratories. See [ISO 140-4], [ISO 717-1] for more information.

Recommendations for sound insulation will vary according to the local building plan and external noise sources. However, one universal recommendation for privacy can be stated. To ensure that confidential information cannot be overheard from outside the room, the weighted apparent sound reduction index () (R’w) of the room boundaries is recommended to be R’w ≥ 48 dB.  See [ISO 140-4] for more information. This approximately corresponds to a sound transmission class STC ≥ 50.

Note: STC is another measure of sound reduction. See [b-ASTM E90] and [b-ASTM E413] for more information.

Doors in the meeting room should have Rw ≥ 35 dB (STC ≥ 35). For walls that include doors, it will be difficult to achieve R’w ≥ 48 dB since the door will be a weak link reducing the R’w of the total construction.

| Identity: | SoundInsulation |
| --- | --- |
| Description: | This parameter indicates the Weighted apparent sound Reduction index (R’w) used between a Telepresence meeting room and adjacent rooms. |
| Format: | R’w [dB] |
| Possible Values: | ≥ 48 dB |
| Default: | none |
| Reference: | None. |
| Interface: | none |

Annex A

<Annex Title - Placeholder>

(This annex forms an integral part of this Recommendation)

<Body of annex A>

Bibliography

[b-ImagePro]	John C.Russ, The Image Processing Handbook, CRC Press, 2006.

[b-ASTM E90]	ASTM E90 - 09 Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements

[b-ASTM E413]	ASTM E413-10, Classification for Rating Sound Insulation.

[b-CIE 15.3:2004]	CIE 15.3:2004, Colorimetry, Third Edition, 2004.

[b-CIE 13.3-1995]	CIE 13.3-1995, Method of Measuring and Specifying Colour Rendering Properties of Light Sources, 1995.

[b-CIE 17.4-1987]	CIE 17.4-1987, International Lighting Vocabulary, 1987.

____