| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4400 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: Clause 8 – Update of TLS session control package |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes package updates according the agreements of the 2013-01 meeting.

1	Introduction

Contributions C-4 and C-14 from last meeting (2013-01) resulted in an agreement in drafting individual H.248 packages for TLS bearer control.

Copy of COM 16 – C 14/Figure 1 – H.248.TLS – Package redesign proposal – Overview

This contribution continues the update process as initiated by C-20 of the TLS basic session control package.

2	Discussion

The basic package design may and is aligned with the TCP basic connection control package (see above Figure). This contribution follows therefore primarily the TCP proposals of AVD-4393.

Some items in more detail:

- Main changes according H.248.TCP due to alignment purposes of both base packages

- state model (new Annex)

- signalling examples (new Appendix)

- the TLS profile related text is moved to the tlscn package section

- Property sepplink in new tlsesc package (AVD-4417)

- TLS vs DTLS: see proposal at MEGACO list → proposal of a separate, new H.248.DTLS work item (see AVD-4418)

- it has to be noted that the text is not yet fully consistent (e.g., (D)TLS should be reverted back again to TLS, dependent on work item on DTLS)

- TLS vs TCP: main differences 
→ Release: TLS always bidirectional; we’ll propose to hide TLS ClosureAlert details to MGC

The reader of this contribution is recommended to start with the signalling examples at the end of the document.

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 20 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

References & abbreviations needs to be updated according agreed changes.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 8	TLS basic session control package

|  | Package Name: | TLS basic session control package |
| --- | --- | --- |
|  | Package ID: | tlsbsc (0x####) |
|  | Description: | This package provides the functionality to be able to establish and /release a TLS (or DTLS) based security session. Furthermore, the capability to block the start of TLS session establishment procedures at the MG bearer interface. NOTE 1 – The package design follows similar connection-oriented bearer (or media) protocols, such as the H.248 tcpbcc package [ITU-T H.248.TCP]. Terminology: “(D)TLS security session” correlates to “TCP bearer connection” and the “Q.1950 BNC” concept. NOTE 2 – The differences between DTLS and TLS are subject of [ITU-T H.248.DTLS]. |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

## 8.1.1	Security session establishment blocking

|  | Property Name: | Security session establishment blocking |
| --- | --- | --- |
|  | Property ID: | bceb (0x0001) NOTE 1: Identifier aligned with H.248 tcpbcc package. |
|  | Description: | This property defines the MG behaviour before and during TLS session establishment. NOTE 2: See Annex A concerning a state model based illustration. |
|  | Type: | Enumeration |
|  | Possible values: | See Table X Table X – Security session establishment preparedness (of TLS endpoints) Value; MG behaviour:. “blocked”, (0x0001); - Bearer resources are prepared (i.e., resources for TLS endpoint are reserved and allocated to H.248 termination/SEP); - TLS role assignment: still open, not yet decided; - TLS session still closed; - H.248 counterpart state = “Blocked” (Figure A.1); - any incoming TLS packet shall be silently discarded (e.g., TLS ClientHello requests shall be not acknowledged).. “unblocked”, (0x0002); - MG shall be ready for incoming or/and outgoing TLS security session control procedures; - TLS role assignment: relates to an (initial) TLS server role effectively; - TLS session state: transitioning from closed to …; - H.248 counterpart state = “Idle” (Figure A.1); TLS security session control options: - Incoming TLS ClientHello: TLS server role confirmed; - Signal tlsbsc/EstBNC: start of TLS session establishment by sending of TLS ClientHello; TLS client role assigned. NOTE 1: ##. |
|  | Default: | None. Subject matter of H.248 profile specifications. |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 8.2	Events

### 8.2.1	TLS session state change (“BNC change”)

|  | Event Name: | TLS session state change |
| --- | --- | --- |
|  | Event ID: | BNCChange (0x0001) |
|  | Description: | This event occurs whenever a change to a (TLS) bearer network connection occurs. For example a bearer (i.e., a TLS security session) has been established or a bearer has been released. The event is related to a simplified state transitioning model, which is described in Annex A. |

#### 8.2.1.1	EventsDescriptor parameters

##### 8.2.1.1.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type, (0x0001) |
|  | Description: | The type of state transitioning, given by the state after the transition. This is used to request the MG to notify it of a particular bearer event. |
|  | Type: | Sublist of Enumeration {Contributor’s note: in order to serve scenario Events=1{tlsc/BNCChange{Type=[Est,Rel]}} |
|  | Optional: | No |
|  | Possible values: | Value; Name; Final state (Annex A.2); Semantic. Est, [0x01]; Bearer Established; Established; notify MGC successful TLS security session establishment.. Rel, [0x05]; Bearer Released; ### Idle; notify MGC successful TLS security session release.. |
|  | Default: | Type=[Est,Rel] The MGC would be then automatically notified about all state transitions when subscribing to event tlsbsc/BNCChange. |

#### 8.2.1.2	ObservedEventsDescriptor parameters

##### 8.2.1.2.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type, (0x0001) |
|  | Description: | This is used to indicate what change has occurred to the TLS security session. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | Est, [0x01] Bearer Established Rel, [0x05] Bearer Release |
|  | Default: | None |

##### 8.2.1.2.2	Session Termination Initiator

{Contributor’s note (2013-06): such detailed level of information seems to be not required for MGC, hm?}

|  | Parameter Name: | Session Termination Initiator |
| --- | --- | --- |
|  | Parameter ID: | sti (0x0001) |
|  | Description: | Indicates which side initiated the termination of the TLS session. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | “local” (0x0001) Session termination was initiated by the MG “remote” (0x0002) Session termination was initiated by the remote TLS-endpoint |
|  | Default: | NA |

##### 8.2.1.2.3	Session Termination Alert Description

{Contributor’s note (2013-06): such detailed level of information seems to be not required for MGC, hm?}

|  | Parameter Name: | Session Termination Alert Description |
| --- | --- | --- |
|  | Parameter ID: | stad (0x0002) |
|  | Description: | Indicates the alert description for the termination of the TLS session. The values are as assigned by IANA for the TLS Alert Registry. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | NA? {Editor’s note (2012-02): seems to be contradicting with above “IANA value assignment” indication. Hm?. Contributions are solicited.} |
|  | Default: | NA |

{Discussion (related to clause 11) – Package-less semantic (“default MG behaviour”):

Package-less mode NOT possible. Any notification mechanism implies the explicit usage (and definition) of an H.248 event.

Conclusion: event as such should be kept …).}

## 8.3	Signals

### 8.3.1	Establish (BNC)

|  | Signal Name: | Establish BNC |
| --- | --- | --- |
|  | Signal ID: | EstBNC (0x0003) |
|  | Description: | This signal triggers the bearer control function to send bearer establishment (i.e., this signal is used to establish the TLS security session: the MG takes the TLS client role and initiates the start of a TLS session establishment by sending a TLS ClientHello request). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 8.3.1.1	Additional parameters

#### None.

### 8.3.2	Release (BNC)

|  | Signal Name: | Release BNC |
| --- | --- | --- |
|  | Signal ID: | RelBNC (0x0004) |
|  | Description: | This signal triggers the bearer control function to send bearer release (i.e., this signal is used to close the TLS security session: the MG sends a TLS alert close_notify request). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 8.3.2.1	Additional parameters

None.

{Discussion (related to clause 11) – Package-less semantic (“default MG behaviour”) concerning close/release:

Option 1:

The trigger for bearer release would be tight to the H.248 Command request SUB.req for stream or termination.

Other options?

None!?

Conclusion: a packageless semantic could be defined.}

### 8.3.2	Closure Alert

{Editor’s note: signal closure alert needs yet to be merged with above Release BNC signal.}

|  | Signal Name: | Closure Alert |
| --- | --- | --- |
|  | Signal ID: | cal (0x0004) |
|  | Description: | This signal is used to indicate an abnormal condition for the TLS-session. The severity of the condition determines whether the session will be closed or whether it may continue. If the al parameter indicates “fatal”, or if the ad parameter is set to “close_notify(0)” the session will be terminated. |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

8.3.1.1	Additional parameters

8.3.1.1.1	Alert Level

{Contributor’s note (2013-06): such detailed level of information seems to be not required for MGC, hm?}

|  | Parameter Name: | Alert Level |
| --- | --- | --- |
|  | Parameter ID: | al (0x0001) |
|  | Description: | Indicates the TLS alert level as defined in [IETF RFC 5246]. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for “AlertLevel”, refer to [IETF RFC 5246]. |
|  | Default: | NA |

8.3.1.1.2	Alert Description

{Contributor’s note (2013-06): such detailed level of information seems to be not required for MGC, hm?}

|  | Parameter Name: | Alert Description |
| --- | --- | --- |
|  | Parameter ID: | ad (0x0002) |
|  | Description: | Indicates the TLS alert description level as defined in [IETF RFC 5246]. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for “AlertDescription”, refer to [IETF RFC 5246]. |
|  | Default: | NA |

## 8.4	Statistics

None.

## 8.5	Error Codes

None.

## 8.6	Procedures

### 8.6.1	TLS endpoint creation

The MGC is responsible for creating a TLS session enabled SEP in the MG. This may be indicated on the H.248 interface through the use of the SDP “m=” element <proto>..

The MG shall reserve “TLS bearer resources” (such as a “TLS encryption(decryption function”) and allocates them to the SEP. The TLS role (client/server) is not (yet) determined by the SDP “m=” line information.

NOTE 1 – This functionality is synonym to the “bearer type indication” semantic as defined by the BNC Characteristics property (BNCChar) of the bearer characteristics package (BCP), see clause A.3.1.1/[ITU-T Q.1950].

### 8.6.2	TLS endpoint role assignment

The TLS role, - client or server -, is primarily relevant during the establishment phase of communication (of perspective of H.248 gateways). Table X summarizes MGC actions and their results in TLS role assignments:

Table X – TLS endpoint role assignment (at establishment phase)

|  | TCP role | H.248 indication | MG behaviour: |
| --- | --- | --- | --- |
| 1 | not yet assigned | H.248 state “Blocked” of SEP, either due to explicit signalling of bceb property or correspondent default value | Any incoming TLS PDU shall be silently discarded. TBD |
| 2 | TLS server | H.248 state “Idle” by “unblocking” the SEP | Expect incoming (TLS) ClientHello, subsequent completion of the establishment related handshake procedure. |
| 2 | TLS client | H.248 state “Idle” of SEP plus signal EstBNC enabled. | Start of TLS session establishment related handshake procedure. |

### 8.6.3	TLS security session establishment

{Editor’s note (2013-06 meeting): align with § 8.6.3/H.248.TCP as soon as procedure stable …}

The MG will shall follow TLS security session establishment according to the indicated TLS.

### 8.6.4	TLS application data transfer

#### 8.6.4.1	MG externally

The TLS session is ready for application data transfer when remote and local TLS endpoints are both completed the TLS handshake procedures for session establishment.

The StreamMode property of the LocalControl Descriptor affects the application data flow rather than the TLS control information used to establish or close the TLS session. The requested MG behaviour is defined by Table Z:

{Editor’s note (2013-06 meeting): Consensus required on this behaviour. See H.248.TCP … }

Table Z – Impact of StreamMode on TLS bearer traffic at external MG interface

| Traffic direction | StreamMode settings | MG behaviour: |
| --- | --- | --- |
| Outgoing | Sending enabled (i.e., values SendOnly, SendRecv, LoopBack) | all TLS PDUs sent |
|  | Sending disbled (i.e., values RecvOnly, Inactive) | only TLS PDUs related to - TLS handshake protocol - TLS cipher change protocol and - TLS alert protocol but not native application data over TLS record protocol. |
| Incoming | Receiving enabled (i.e., values RecvOnly, SendRecv, LoopBack) | correspondent to “outoging” side |
|  | Receiving disbled (i.e., values SendOnly, Inactive) | correspondent to “outoging” side |

#### 8.6.4.2	MG internally

This clause is only relevant for H.248 contexts with TLS SEPP(s).

Thus application data may be received at a SEP in time where the other SEP of the context is not yet able to process or send the application data. The behaviour of the MG in such a situation is implementation and/or application dependent. If application independent or unambiguous behaviour would be required, then the SEPP interlinkage property of the tlsesc package (see clause X.1.A.B) should be used.

### 8.6.5	TLS session release

The trigger for a TLS session termination might come from different sources, see Fig. X:

Figure X – Principle stimuli for triggering TLS session release

- Bearer control protocol – Security session: The TLS session is terminated by the remote TLS endpoint by sending a (TLS) close_notify alert or a fatal alert;

- Bearer control protocol – Transport connection: The underlying L4 transport connection is terminated;

- TLS protocol: A non-recoverable error condition has been detected in the TLS function, e.g. the authentication of a received TLS segment has failed;

- The MGC indicates the closure of the TLS session by the signal tlsbsc/RelBNC;

- The release of the security session of the partner SEP triggers the release autonomously in the MG (Note: not supported by base package tlsbsc, related to inter-linkage property tlsesc/sepplink, see clause 9.x.y.z);

- In case the MG acts in an application aware mode, the application part in the MG may indicate the release of the TLS session;

8.6.5.1	Bearer control protocol – Security session: TLS alert protocol

{Editor’s note (2013-06): the procedure overlapps with the TLS session maintenance package procedures. The text is not yet aligned with clause 10.6.x. Need to be fixed.}

In case a TLS alert is received that leads to the termination of the TLS session, then the behaviour of the TLS protocol is as specified in [IETF RFC 5246], i.e. in case of a received TLS close_notify the TLS endpoint will reply a TLS close_notify on its own and the TLS session is closed.

Impact on underlying IP transport connection:

The MG shall not autonomously initiate the release of the (if connection-oriented) IP transport connection as well. 
NOTE: extension package tlsesc allows to tightly couple bearer control procedures for L4 and TLS, see property tlsesc/plslink in clause 9.1.2.

If requested the MGC shall be notified by the event tlsbsc/BNCChange{Type=Rel}. Handling of the TLS-specific AlertDescription as received in the TLS alert message is for further studies.

Impact on H.248 SEP and possibly SEPP:

The SEP as such still exists after a successful TLS session release (state Idle). Removal of the SEP follows usual H.248 SUBtract.req or MODify.req commands.

Property tlsesc/sepplink (clause 9.1.1) allows to trigger a TLS session release at the partner SEPP.

8.6.5.2	Bearer control protocol – Transport connection: L4 bearer release

In case the underlying IP transport connection is released, e.g. due to a received TCP-FIN or TCP-RST in case of TCP transport, then

- default mode (base package:

- enhanced mode (extension package): the MG may autonomously and immediately initiate the closure of the TLS session (dependent on property setting of tlsesc/plslink). The release procedures of the IP transport connection shall be delayed as long as the TLS session is successfully released (see also Appendix V?.4.2.2.1).

NOTE: extension package tlsesc allows to couple bearer control procedures for L4 and TLS, see clause 9.1.2.

If requested the MGC will be notified by the event tlsbsc/BNCChange{Type=Rel}.

8.6.5.3		TLS protocol: non recoverable error condition detected

{Editor’s note (2013-06 meeting): below information might overlap with the TLS session maintenance package … }

In case an error condition is detected locally in the TLS protocol layer which results in the termination of the TLS session, the procedures as defined in [IETF RFC 5246] apply. A TLS alert will be sent to the remote TLS endpoint with the AlertLevel set to “fatal” and the AlertDescription is provided accordingly.

If requested the MGC will be notified by the event tlsbsc/BNCChange{Type=Rel}.

8.6.5.4	H.248: 	MGC driven TLS session release

In case the H.248 signal tlsbsc/RelBNC is received the TLS session closure shall be initiated by the MG by sending a TLS alert close_notify towards remote TLS entity.

The TLS alert signal does not generate the notification TLS-Session Termination Alert (sta) in return.

8.6.5.5		MG autonomous release initiated on partner SEP

This capability is not supported by the base package, subject of extenstion package tlsesc (property sepplink), see clause 9.

The TLS enabled SEP might receive the indication that the security session of the related SEP is released. If such an indication is detected the termination of the TLS session is initiated by sending a TLS alert (close_notify) to the remote TLS endpoint. The closure of the underlying TCP connection is initiated by the MG autonomously.

The event TLS-Session Termination Alert (sta) is not reported to the MGC in this case, as the MGC has the option to be notified by the related SEP about the release of the security session.

8.6.5.6		Release indication received from TLS application

In case the MG operates in a TLS-application aware mode, an error condition in the application aware entity might be detected that requires the release of the TLS session. In this situation the TLS session will be closed autonomously by sending a TLS alert (close_notify).

The underlying IP transport connection shall be not affected (base package behaviour).

{Editor’s Note:

Observation: There are a lot of commonalities to H.248.TCP:

- Bearer connection state “established” and “closed” are reported by event notifications

- MGC may trigger the release of the bearer connection by a signal (“close”)

- The property MG Autonomous Bearer Release Indicator (abri) allows a “MGC-strictly controlled” behaviour as well as a”MG-autonomous” behaviour. The property  is present in both packages, indeed this indicator is bearer type independent

- Release procedure differs only in minor protocol dependent details (TCP: half closure; TLS: “duplex” closure)

Thus defining a bearer protocol independent package to control the bearer connection setup and release might be an option. Such a packet could be the base for other bearer connection protocols as well like SCTP, DTLS, etc. Such a package could simplify interworking functions between different bearer connection protocols.}

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Annex A

State modelling for 
TLS bearer connection endpoints

(This annex forms an integral part of this Recommendation.)

## A.1	Introduction and purpose

The TLS basic session control package (clause 8) is tightly coupled to a state model, because the event (tlsbsc/BNCChange) and signal (tlsbsc/EstBNC) are related to state transitioning behaviour.

It has to be noted that the IETF TLS related RFCs do not define any explicit state model with regards to TLS security session endpoints. However, a simplified view is sufficient from perspective of H.248 gateway control procedures.

The underlying state model of package tlsbsc is described by this Annex.

## A.2	Original state model for TLS session endpoints

There is not any standardized state model for TLS (in comparison to TCP). The TLS transport model [b-IETF RFC 6353] provides some modelling related information.

## A.2	Simplified state model for H.248-based TLS basic session control

The TLS basic session control package makes a couple of assumptions, similar as outlined in Annex A/[H.248.TCP] for TCP.

The correspondent simplified model for TLS is depicted by Figure A.1:

Figure A.1 – Simplified state model for H.248-based TLS basic session control

[…]

### ex 8.6.5	Completion of the initial TLS handshake

Transitioning from Idle to Established:

The bearer establishment related TLS handshake procedures will be performed autonomously by the MG with the properties as defined by the TLS endpoint profile.

The initial TLS handshake is completed when the following both conditions are true:

- The TLS-Finish message has been sent, using the negotiated set of security parameters.

- The TLS-Finish message has been received and has been successfully verified.

At this point in time TLS application data may be sent and received using the negotiated security parameters. The event tlsbsc/BNCChange{type=Est} can be used to notify the MGC accordingly.

[End Proposed Correction]

Change proposal #4:

[Begin Proposed Correction]

Appendix ???

Illustration of protocol semantics 
of the TLS basic session control package

(This appendix does not form an integral part of this Recommendation.)

## ???.1	Overview

The tlsbsc package (clause 8) defines the basic TLS security session control with scope on support for establishment and release. The illustrated use cases are abstracted examples.

## ???.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (TLS) is highlighted besides the H.248 interface.

Furthermore all Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local TLS session endpoint.

## ???.3	Establishment of TLS security sessions

### ???.3.1	Successful establishment – Terminating side

See Figure ???.1, termed as use case (E.1):

Figure ???.1 – Successful establishment – Terminating side

More details for this example:

- The MGC blocks the start of TLS session establishment (1).

- Early incoming TLS messages are discarded by the MG (2).

- The MGC unblocks the TLS SEP (3).

- TLS session establishment (5 to 7); MG acting as TLS server.

- Optional notification of MGC concerning available TLS session for application data transfer (8), if subscription to event by MGC (4).

### ???.3.2	Successful establishment – Originating side

See Figure ???.2, use case (E.2)

Figure ???.2 – Successful establishment – Originating side

More details for this example:

- Steps 1 to 3 as in clause ???.3.1

- TLS session establishment (5 to 7), triggered by MGC (4); MG acting as TLS client.

- Optional notification of MGC concerning available TLS session for application data transfer (9), if subscription to event by MGC (8).

### ???.3.3	Unsuccessful establishment

FIXTHIS: usage of g/cause still under discussion!

## ???.4	Release of TLS security sessions

### ???.4.1	Successful release – Terminating side

See Figure ???.4, termed as use case (R.1):

Figure ???.4 – Successful release – Terminating side

FIXTHIS: usage of g/cause still under discussion!

More details for this example:

- MG receives an incoming TLS close_notify alert (2), the indication for TLS session release.

- The MG acknowledges the release request (3).

- Optional notification of MGC concerning successfully released TLS session (4), if subscription to event by MGC (1).

### ???.4.2	Successful release – Originating side

See Figure ???.5, use cases (R.2) …

Figure ???.5 – Successful release – Originating side

More details for this example:

- MGC triggers the TLS session release (1).

- MG sends an outgoing TLS close_notify alert (2), the indication for TLS session release to wards remote TLS endpoint X.

- Successful release (3).

- Optional notification of MGC concerning successfully released TLS session (5), if subscription to event by MGC (4).

### ???.4.3	Unsuccessful release

FIXTHIS: usage of g/cause still under discussion!

[End Proposed Correction]