| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-08a |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Rapporteur |  |  |  |
| Title: | Report for Q2/16 “Packet-based conversational multimedia systems and functions” |  |  |  |
| Purpose: | Information |  |  |  |

Summary

Question 2/16 met for three sessions during the week of 17-21 June 2013, reviewing a number of contributions on a variety of topics.  A proposed enhancement was introduced for the next revision of H.225.0 to better assist H.323 systems in performing proper language selection.  Experts considered issues related to H.323 security, specifically errors found in H.235.0 and H.235.6.  Experts also made progress on the Advanced Multimedia System and enhancements to H.323 systems that would allow for similar functionality, agreeing to begin work on a new Annex to H.323 and a new H.46x-series of Recommendations to support the new H.323 Associative Simple Endpoint Type (ASSET).  Lastly, during a joint session with Questions 1, 2 and 5/16 it was agreed to initiate new work on enhancements to H.32x-series Recommendations to provide better support for web applications, initially developing a new H-series Supplement.

# Question 2/16 – Packet-based conversational multimedia systems and functions

The objectives for this meeting were:

- Coordinate with other SDOs, Questions, or Study Groups

- Review items proposed for the H.323-Series Implementors' Guide

- Progress work on H.323-related work:

  - H.323-Series Implementors' Guide

  - H.460.SessionID

  - H.245

- Progress work on AMS-related work:

  - H.325

  - Signalling (e.g., H.AMS.cis)

  - Syntax

  - Message flows

- Discuss miscellaneous and new work items

# Documentation

The list of meeting contributions was reviewed (TD-02). The following documents were relevant to Q2/16:

AVD: 4357, 4358, 4359, 4360, 4361, 4423, 4424, 4444, 4454a

TD: 04, 08, 11, 27

## Review of the meeting agenda

The meeting agenda appeared in TD-04 and was approved by the meeting.

## Incoming Liaison Statements

No incoming liaisons for Q2/16.

# Discussion

AVD-4357 – Proposed revision of H.235.6 to support of cryptographic key lengths greater than 4096 bits [Spranto]

It was noted that section 8.5 needs to be modified to align with other proposed changes.  We need to check H.245 to see if the OIDs in Table 6 need to be recorded in H.245.  The new OIDs introduced in Table 6 should be assigned the next available number.  It was agreed to revise H.235.6 as per this contribution and per the comments provide against section 8.5.  The contributor kindly agreed to serve as editor and bring text to the next SG16 meeting.

AVD-4358 – Proposed revision of H.235.0 to support of cryptographic key lengths larger than 2048 bits [Spranto]

After discussing this contribution, it was agreed to create a new revised H.235.0.  The contributor kindly offered to serve as editor and bring text to the next SG16 meeting.

AVD-4359 – H.323 Associative SET (ASSET) Framework description and proposed call flows [Spranto]

This document was discussed at length during the meeting. It specifies a way to introduce much of the H.325-like functionality into H.323 systems.

A point raised was that if a Gatekeeper wants to terminate a call, it would not know that the main terminal device and a number of ASSET devices are participating in the same call, since each signalling connection looks like a distinct call to the Gatekeeper.  ASSET devices periodically poll the main terminal device to see that the call is still active.

There are potential security implications with this model that need to be considered.  Those security considerations need to be documented as a part of the specification.

It should be possible to discover, associate, and invoke applications in ASSETs even after the primary terminal initiates a call.

AVD-4360 – H.323 Associative Simple Endpoint Type (ASSET) Definition [Spranto]

This document contains a rough outline of the text that would comprise the new Annex to H.323 defining the Associative SET device type (ASSET).  It was agreed to create a new work item to define this new Annex to H.323.

AVD-4361 – H.323 Associative SET (ASSET) Feature [Spranto]

This contribution proposes the creation of a new series of H.46x documents that describe ASSETs, with one document serving as the base document for requirements placed on ASSETs.  This document defines the basic call messages that are exchanged between the main terminal and ASSETs.

It was agreed to initiate a new work item (H.46x.0) to define the base specification for the series of ASSET specifications that will follow, with this document serving as the initial text.  Other ASSET specifications in the series will define vertical applications, such as file transfer, electronic whiteboard, etc.

AVD-4423 – H.325/AMS Message Primitives [Cisco]

This document was reviewed during the meeting.  Experts encouraged the contributor to specify the signalling using JSON syntax as discussed during the meeting.  A contribution is expected at the next SG16 meeting.

AVD-4424 – H.325/AMS Signalling using JSON [Cisco]

It was agreed to modify the working assumptions to indicate that H.323/AMS will use JSON rather than XML for signalling.

We discussed defining a URN namespace called “itu” for use in H.325.  Since there may be others within the ITU interested in using URNs for other applications, it was agreed to create a proposed syntax for URNs and then solicit input from other interested parties in the ITU.  The Rapporteur agreed to work the URN syntax and discuss this with the TSB.  This may result in a liaison statement sent from the next SG16 meeting.

AVD-4444 – Integration of Web Services in H.32x series Terminals and MCUs [Polycom]

This was presenting during a joint meeting of Q1, 2, and 5/16.  It was generally agreed that studying how we might enhance H.32x systems to support web applications, including interoperating, WebRTC media support, MCU control, conference transferring, etc. is worthwhile.  Further contributions related to WebRTC and web applications in general are encouraged and initially directed to Question 2 to define the use cases and scope.  We will determine which Questions will handle specific protocol enhancements or additions subsequently.  We decided to create a new work item to capture use cases in an H-series supplement.

AVD-4454a – H.225.0 Amd. Proposed changes for language cap exchange [Spranto]

The text describing the RCF “language” field should indicate that this is to allow the Gatekeeper to provision the endpoint with language information.  It was agreed to accept this text toward H.225.0v8.

TD-11 – AMS Working Assumptions [Editor]

This was reviewed during the meeting.  Changes agreed during the meeting were made to the text and the output appears in TD-27.

# Intellectual property statements

Members were reminded to make disclosures of known patents to the ITU Secretariat as defined in the IPR policy in a timely manner.  Nobody indicated having knowledge of patents or pending patent applications, the use of which may be required to implement Recommendations being considered during the meeting.

# Presentation of reports

The draft report for Q2/16 was reviewed and the experts accepted its content. The report can be found in TD-08a.

# Outgoing liaison statements

None.

# Work Programme

| Acronym | Title | Editor | Consent / Approval | Reference |
| --- | --- | --- | --- | --- |
| AMS Catalogue | AMS Catalogue | TBD | N/A | TD 211/WP2 (2009-10) |
| AMS Project Description | AMS Project Description | Tyler Johnson (Tyler_Johnson@unc.edu) | N/A | TD 562/WP2 (2008-04) |
| AMS Working Assumptions | AMS Working Assumptions | Paul Jones (paulej@packetizer.com | N/A | TD-27 (2013-06) |
| H.245 | Revised Rec. H.245 "Control protocol for multimedia communication" (Rev.) | Paul E. Jones (paulej@packetizer.com) | 2014 | H.245 + TD-23 (2011/07) + TD 644/WP2 (2011/11) |
| H.323 IG | Implementors' Guide for Recommendations of the H.323 System | S. Horne (s.horne@spranto.com) | 2013-11 | TD 74R1/Plen (2013-01) |
| H.325 (H.ams) | AMS: Advanced multimedia systems for next generation and other packet-switched networks | Paul E. Jones (paulej@packetizer.com) | 2015 | TD 671R1/WP2 (2011-11) |
| H.460.SessionID | End-to-End Session Identifier for H.323 Systems (New) | Paul Jones (paulej@packetizer.com | 2014 | COM16-C.552 (2011/03) |
| H.AMS.CIS | Common Interface Syntax | Paul Jones (paulej@packetizer.com) | 2015 | AMS-0001 (2009-12) |
| H.EMQ | A cooperative architecture for enhanced multimedia QoS/QoE (New) | Ki-Jong Koo (kjkoo@etri.re.kr) Seong-Ho Jeong (shjeong@hufs.ac.kr) | 2014 | TD 816/WP2 |
| H.supp-web-apps | Supplement describe use cases to enhancements to H.32x-series recommendations to support web applications | TBD | 2015 | AVD-4444 (2013-06) |
| H.235.0 | H.323 security: Framework for security in H-series (H.323 and other H.245-based) multimedia systems | Simon Horne (s.horne@spranto.com) | 2013 | AVD-4358 (2013-06) |
| H.235.6 | H.323 security: Encryption profile with native H.235/H.245 key management | Simon Horne (s.horne@spranto.com) | 2013 | AVD-4357 (2013-06) |
| H.225.0v8 | Call signalling protocols and media stream packetization for packet-based multimedia communication systems | Paul E. Jones (paulej@cisco.com) | TBD | H.225.0v7 + AVD-4454a (2013-06) |
| H.323 Annex for ASSET | H.323 Associative Simple Endpoint Type (ASSET) | Simon Horne (s.horne@spranto.com) | 2015 | AVD-4360 (2013-06) |
| H.asset | H.323 Associative Set (ASSET) Feature | Simon Horne (s.horne@spranto.com) | 2015 | AVD-4361 (2013-06) |

__________________