| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 144 R3 (PLEN/16) |
|  |  | English only Original: English |
| Question(s): | 21/16 | Geneva, 28 October – 8 November 2013 |
| TD |  |  |
| Source: | Editor F.UDExch |  |
| Title: | Consent: F.746.2 (ex F.UDExch) “User data exchange between functional components in network entities or terminals” (New) |  |

This document contains the draft new Recommendation F.746.2 (ex F.UDExch) “User data exchange between functional components in network entities or terminals” for Consent at this SG 16 meeting.

AAP Summary

Under many circumstances there is a need to exchange user data from information terminals or network entities to another depository. The data exchange process might be initiated by users manually or by the network applications automatically, and multiple categories of data might be involved in one exchange process.

This Recommendation provides the scenario descriptions of user data exchange and describes a basic approach of exchanging user data between terminals or between terminals and network entities by using data packages encapsulating formatted user data. The functionality of the functional components which implement the data exchange and general guidance of generating and parsing such data packages are given as well. This Recommendation also describes the preferred XML representations used when encapsulating corresponding user data including but not limited to contacts, messages and other user data categories.

CONTENTS

Page

1 Scope 4 2 References 4 3 Definitions 5 3.1 Terms defined in this Recommendation 5 4 Abbreviations 5 5 General 6 6 Scenarios of user data exchange 6 6.1 User data grouping 6 6.1.1 User private data 6 6.1.2 Device configuration data 6 6.1.3 Personalized application data 7 6.2 Scenario description 7 7 User data exchange method 7 7.1 General procedure 7 7.2 Data package 8 7.2.1 An example 8 7.2.2 < DataExchangeInfo> element 9 7.2.3 <DataSection> element 10 8 Functionality of data exchange functional components 10 8.1 Basic functionality 10 8.2 Security consideration 10 9 Preferred User data reference XML expression 10 9.1 General 10 9.2 Contacts data expression 11 9.3 Calendar data expression 11 9.4 Message data expression 11 9.4.1 SMS expression 11 9.4.2 MMS expression 12 9.5 Other 13

List of Figures

Page

Figure 1 – Data exchange procedure 8

Draft new Recommendation ITU-T F.746.2 (ex F.UDExch)

User data exchange between functional components in 
network entities or terminals

Summary

This Recommendation provides the scenario descriptions of user data exchange and describes a basic approach of exchanging user data between terminals or between terminals and network entities by using data packages encapsulating formatted user data. The functionality of the functional components which implement the data exchange and general guidance of generating and parsing such data package are given as well. This Recommendation also describes the preferred XML representations used when encapsulating corresponding user data including but not limited to contacts, messages and other user data categories.

Keywords

User data exchange, Data exchanging agent (DXA), XML

1	Scope

The general purpose of this document is to provide scenarios descriptions and a simple approach to exchange user data between terminals or entities.

This document is applicable to terminals or network entities which choose to implement user data exchanging functions as described in this document.

2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T X.200]	ITU-T Recommendation X.200 (1994), Information technology - Open Systems Interconnection - Basic Reference Model: The basic model.

[ISO 8601]	ISO 8601 (2004) Data elements and interchange formats--information interchange-- representation of dates and times

[ARIB STD-T64-C.S0015-0]	ARIB STD-T64-C.S0015-0 (2000), Short Message Service For Wideband Spread Spectrum Cellular Systems.

[ETSI TS 123 140]	ETSI TS 123 140 V6.16.0 (2009), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); Multimedia Messaging Service (MMS); Functional description; Stage 2.

[ETSI TS 124 011]	ETSI TS 124 011 V10.0.0 (2011), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface.

[ETSI TS 126 140]	ETSI TS 126 140 V7.1.0 (2007), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); Multimedia Messaging Service (MMS); Media formats and codes.

[ETSI TS 127 005]	ETSI TS 127 005 V3.2.0 (2002), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS);Use of Data Terminal Equipment - Data Circuit terminating; Equipment (DTE - DCE) interface for Short Message Service (SMS) and Cell Broadcast Service (CBS).

[IETF RFC 5545]	IETF RFC 5545 (2009), Internet Calendaring and Scheduling Core Object Specification (iCalendar).

[IETF RFC 6321]	IETF RFC 6321 (2011), xCal: The XML Format for iCalendar.

[IETF RFC 6350]	IETF RFC 6350 (2011), vCard Format Specification.

[IETF RFC 6351]	IETF RFC 6351 (2011), xCard: vCard XML Representation.

3	Definitions

3.1	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.1.1	User data: “User data” is the data that is created or stored in terminals or entities by the users intentionally, and related to the private information, historical operation, personal tendency or favourites of the users. This definition of “User data” is different with the one defined in X.200. It is independent of whether this data is transported by the user and has no correlation with the counterpart of “Control data” as well.

3.1.2	Data exchanging agent: A software or program deployed in terminals or entities where the main function is to put data into or extract data from the data package described in this document. Furthermore it monitors the data exchange process and takes actions when specific user data is encountered after the data exchange process. A typical form of the functional component is an XML parser combined with application-dependent functionalities.

4	Abbreviations

| DXA | Data eXchanging Agent |
| --- | --- |
| FTP | File Transfer Protocol |
| HTTP | Hypertext Transfer Protocol |
| IM | Instant Message |
| MMS | Multimedia Message Service |
| SD | Secure Digital |
| SIM | Subscriber Identity Module |
| SMS | Short Message Service |
| USB | Universal Serial Bus |
| USIM | User Services Identity Module |
| W3C | World Wide Web Consortium |
| XML | Extensible Markup Language |

5	General

Under many circumstances there is a need to exchange user data from information terminals or network entities to another depository, for example the user wants a terminal to be substituted by another new terminal without losing personal data stored in the original one such as contacts and already received messages. The data exchange process might be initiated by users manually or by the network applications automatically, and multiple categories of data might be involved in one exchange process. However the terminals or entities acting as source and destination in the exchange process may be of different operating systems or use different internal data expressions for the same kind of data. Without a widely applicable approach, exchanging user data freely is difficult to realize.

To facilitate the exchange process and make it as simple as possible for the users, this document describes an exchange approach by using a data package as a container to encapsulate user data, and specifies the recommended data expression for parsing or generating the data package.

6	Scenarios of user data exchange

6.1	User data grouping

The user data includes user personal data, device configuration parameters and personalized application data, and can be stored in terminals or entities no matter what the physical appearance is.

The user data may take various forms such as binary, text or multi-media files according to the corresponding data category. It could be stored in different locations such as internal memory (RAM, ROM etc.) and external memory (SIM/USIM, SD card, hard disk etc.).

6.1.1	User private data

The user private data is highly related to the user of a terminal. Usually it is created intentionally by the user for special purpose or generated by the users operations and contains important information of the user. Examples of user private data include:

- Contact information (including phonebook)

- Messages (including SMS, MMS, IM and Emails)

- Calendar information

- Favourite URL lists and bookmarks

- Calls history and browsing history

6.1.2	Device configuration data

Device configuration data is the data that a terminal allows users to access and modify, so as to personalize the way the terminal looks like or behaves. Examples of device configuration data include:

- Terminal theme, fonts, wallpaper, screensaver

- Language settings

- Date/Time and location settings

- Energy saving settings

- Accessibility settings

6.1.3	Personalized application data

Personalized application data is the data that applications allow users to access and modify, so as to personalize the way an application operates or behaves, including:

- Service dependent setting or parameter

- Application setting or application files

6.2	Scenario description

The basic user data exchange scenarios is that a terminal or an entity wants to receive discrete sets of user data from another terminal or to send data to another terminal. For example, a mobile phone wants to retrieve all the contacts and calendar data stored in another terminal, another example would be that an entity in the network wants to receive a status report, or wants to retrieve the browsing history of the streaming media player on the terminal for later statistics purpose.

In those cases, typically these user data are of small sizes and may be stored with different formats in the two terminals because of different operation platform or different data management applications. To exchange data, normally the data providing party will generate a data package which contains multiple categories of data, and the other party receives the data package and performs data analysing, followed by certain operations according to the original purpose of this data acquainting process.

As in the description above, these scenario of data exchanging covers the following operations:

- Export: Export selected user data from party A as an accessible data package.

- Transfer: Party A transfer this package to party B by any approach. This step doesn’t need to be real time. Any extra processing during the transfer step could be conducted on the data package, or even move the data package to another depository and reach it sometime later when necessary.

- Analyse: Party B open the package and performs carefully analysing, retrieve all the data it needs or supports according to the pre-defined data format or expression, followed by certain operations according to the retrieved data.

7	User data exchange method

7.1	General procedure

For the typical user data exchange scenario, the data exchange process can be summarized into several steps: Export, Transfer, and Analyse.

The data package which contains all the user data forms the basis of data exchanging. To address the compatibility issue, the data package should be defined in a well-known format, so that it could be supported and applied in various applications cross different operation systems and terminals. Furthermore, the data package should be able to encapsulate multiple categories of data, and also extendable to new data set definitions. Since XML is designed to describe structured data, naturally extendable, and widely supported by many applications and operating systems, in this document XML representation is chosen to encapsulate user data in the data package.

The Transfer step is only a simple data package transferring. Transferring data package as files is already defined clearly in protocols for various accessing techniques, such as FTP, HTTP protocols via internet access, Bluetooth and infrared via short ranged wireless communication and USB storage via wired connections, data package transferring could also be incorporated in other protocols and applications. Meanwhile, this step doesn’t need to be real time. Any extra processing during the transfer step could be conducted on the data package, or even move the data package to another depository and access it sometime later when necessary.

The other steps Export and Analyse could be implemented by functional components. These components are deployed in terminals or entities acting as data exchange agents to manage the data exchange. The data exchange process is illustrated in Figure 1.

Terminal or entities

DXA

Data package

(Export)

DXA

Terminal or entities

(Analyse)

Required data

Retrieved data

Transferring, restoring or processing.

Data package

Figure 1 – Data exchange procedure

7.2	Data package

7.2.1	An example

The data package is encoded in XML structure, a simply example of a data package containing the contacts information and calendar records plus MSN instant message history, which is used for exchanging private user data between two terminals, is illustrated as follows:

<?xml version="1.0" encoding="UTF-8"?>

<DataExchangeInfo>

<UdxVersion>1.0</UdxVersion>

<Encryption>0</Encryption>

<RecordInfo>

<CreateTime>20100810T1430</CreateTime>

<VendorInfo>MobilePhone_Vendor_X</VendorInfo>

<DeviceInfo>TypeNumber_2321C</DeviceInfo>

<UserAgent>Data_export_Manager</UserAgent>

</RecordInfo>

<DataSection>

<vcards xmlns="urn:ietf:params:xml:ns:vcard-4.0">

……//XML structured data for Contacts is stored here

</vcards>

<icalendar xmlns="urn:ietf:params:xml:ns:icalendar-2.0">

……//XML structured data for calendar is stored here

</icalendar>

<MSN:Records MSN:xmlns="http://MSN.com/MSN-xml/conversation-records">

……//XML structured data for MSN message history is stored here

</MSN:Records>

</DataSection>

</DataExchangeInfo>

7.2.2	< DataExchangeInfo> element

The root element in this XML structure is <DataExchangeInfo>, and it is also the beginning of the XML structure. When packing data into <DataExchangeInfo> element, the format should follow the XML Schema 1.1 description of the <DataExchangeInfo> element as listed below:

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema">

<xs:element name="DataExchangeInfo" type="DataExchangeInfoType"/>

<xs:complexType name="DataExchangeInfoType">

<xs:sequence>

<xs:element name="UdxVersion" type="xs:string" default="1.0"/>

<xs:element name="Encryption" type="xs:boolean" default ="0"/>

<xs:element name="RecordInfo" type="RecordInfoType"/>

<xs:element name="DataSection" type="DataSectionType"/>

</xs:sequence>

<xs:complexType name="RecordInfoType">

<xs:sequence>

<xs:element name="CreateTime" type="xs:datetime" minOccurs="0"/>

<xs:element name="VendorInfo" type="xs:string" minOccurs="0"/>

<xs:element name="DeviceInfo" type="xs:string" minOccurs="0"/>

<xs:element name="UserAgent" type="xs:string" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="DataSectionType">

<xs:all>

<xs:any minOccurs="0" maxOccurs="unbounded"/>

</xs:all>

</xs:complexType>

</xs:complexType>

</xs:schema>

In the <DataExchangeInfo> element, there are several child elements:

- <UdxVersion>: this element indicates the version of this XML file.

- <RecordInfo>: this element contains descriptive information about the data package. Such as:

  - <CreateTime>: A string indicating the creating time of this package according to the format specified in [ISO 8601] could be put in this field.

  - <VendorInfo>: A string indicating information of the terminal vendor could be put in this field.

  - <DeviceInfo>: A string indicating information of the terminal itself could be put in this field.

  - <UserAgent>: A string indicating information of the application generating the data package could be put in this field.

- <DataSection>: this element is the main body contains the data to be exchanged.

7.2.3	<DataSection> element

The <DataSection> is the main element contains the data to be exchanged. Any XML formulated user data that follows the corresponding XML representations defined in existing specifications or defined by applications itself could be stored in this element.

8	Functionality of data exchange functional components

8.1	Basic functionality

The DXA should be able to generate and parse XML data package. When parsing the XML data package, the DXA should be able to ignore unsupported data in the data package without interrupting the data exchange process or leading to unexpected exception.

The DXA should be capable of supporting the updating of data set definition. This functionality may be realized by updating the corresponding XML definition of the supported data.

The DXA should be able to avoid data collision when storing the retrieved data from the exchange process to internal storage of the data receiving side, possibly by watching the user data records already existing. When data collision might occur, actions such as prompts to user or user data merging according to predefined rules should be taken.

8.2	Security consideration

Whether or not to take action to ensure the security of user data exchanging depends on the application implementation or the data sensitivity. For some real-time implementations of user data exchange the security issue could be addressed in the transportation layer by secure file transfer. For other off-line implementations, for example generated data package would be temporarily stored in public websites, simple password protecting or XML encryption according to W3C XML Encryption Syntax and Processing specification may be applied on the data package to prevent unauthorized access of the user data.

When XML encryption is used to create the data package, the DXA should be capable of encrypting the user data and decrypting the XML encrypted data according to the W3C XML Encryption Syntax and Processing specification.

9	Preferred User data reference XML expression

9.1	General

Whether the data retrieved from the data package could be successfully understood depends on the XML expression of  user data. If the XML expression of some kind of user data in the data package matches perfectly with the internal storing format of that kind of user data in the terminal or entities which receives the data package, then the user data exchange could be done without any compatibility problem or mismatch, otherwise this exchange process would still probably suffer data loss or data mismatch.

Due to the fact that every kind of user data has its own distinctive format, which could be different over terminals or different operating systems, moreover some kinds of user data even still do not have popular or globally-accepted formats at present. It is hard to specify preferred XML expression format for all kinds of user data mentioned in this document. To address the data exchange problem to such an extent that this document could be useful to exchange the most important user data, in this clause the XML expression will be specified for those kinds of user data which are most frequently required to be exchanged and also have well accepted data format, such as the user private data.

9.2	Contacts data expression

For representing and exchanging contacts and relevant information about individuals and other entities, IETF have already specified vCard data format in [IETF RFC 6350], which is a text-based format.

The XML representation for vCard has already been defined as well. The underlying data structure is exactly the same with the original vCard text format, which supplies the 1-to-1 mapping between the original vCard format and the XML representation. The vCard XML representation is preferred in this document to represents the contacts in user data exchanging.

The contacts XML representation should follow the vCard XML formats, which is specified in [IETF RFC 6351]“vCard XML representation”.

9.3	Calendar data expression

For the calendaring and scheduling data, IETF have already specified iCalendar data format in [IETF RFC 5545]. The iCalendar data format is a widely deployed interchange format. While many applications and services on the internet support and generate calendar data, Similar to vCard format, a iCalendar XML representation has been defined, Which also supplies the 1-to-1 mapping between the original iCalendar format and the XML representation. The iCalendar XML representation is preferred in this document to represents the calendaring data in user data exchanging.

The calendaring XML representation should follow the iCalendar XML formats, which is specified in [IETF RFC 6321]“xCal: The XML format for iCalendar”.

9.4	Message data expression

9.4.1	SMS expression

For exchanging the SMS data stored in mobile terminals or other storage, the schema of recommended XML representation following XML Schema 1.1 is shown as below:

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema">

<xs:element name = "RecordOfSMS" type="RecordOfSMSType”"/>

<xs:complexType name="RecordOfSMSType">

<xs:sequence>

<xs:element name="SMSField" type="SMSFieldType" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="SMSFieldType">

<xs:sequence>

<xs:element name="Orignal_Address" type="xs:string" minOccurs="0"/>

<xs:element name="Destination_Address" type="xs:string" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="Data" type="xs:string" minOccurs="0"/>

<xs:element name="TimeStamp" type="xs:string" minOccurs="0"/>

<xs:element name="MessageInfo" type="MessageInfoType" minOccurs="0"/>

<xs:element name="EXTENSION" type="xs:string" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MessageInfoType">

<xs:choice>

<xs:element name="MT_SMS" type="xs:boolean" default="1"/>

<xs:element name="MO_SMS" type="xs:boolean" default="0"/>

<xs:element name="Draft_SMS" type="xs:boolean" default="0"/>

<xs:element name="Delivery_Report" type="xs:boolean" default="0"/>

</xs:choice>

</xs:complexType>

</xs:schema>

When converting SMS data to XML representation, the SMS information data fields should be mapped to XML elements correctly, the specific data format and meaning for each SMS information field stored as character string in XML elements shall follow the definitions correspondingly given in [ETSI TS 127 005] and [ARIB STD-T64-C.S0015-0].

9.4.2	MMS expression

For exchanging the MMS data stored in mobile terminals or other storage, the schema of recommended XML representation following XML Schema 1.1 is shown as below:

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema">

<xs:element name="RecordOfMMS" type="RecordOfMMSType"/>

<xs:complexType name="RecordOfMMSType">

<xs:sequence>

<xs:element name="MMSField" type="MMSFieldType" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MMSFieldType">

<xs:sequence>

<xs:element name="MMSContent" type="MMSContentType" minOccurs="0"/>

<xs:element name="ContentInMMS" type="ContentInMMSType" minOccurs="0"/>

<xs:element name="Message_Type" type="xs:string" minOccurs="0"/>

<xs:element name="MMS_Version" type="xs:string" minOccurs="0"/>

<xs:element name="Message_ID" type="xs:string" minOccurs="0"/>

<xs:element name="Sender_address" type="xs:string" minOccurs="0"/>

<xs:element name="Recipient_address" type="xs:string" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="Message_class" type="xs:string" minOccurs="0"/>

<xs:element name="Date_and_time" type="xs:string" minOccurs="0"/>

<xs:element name="Priority" type="xs:string" minOccurs="0"/>

<xs:element name="Subject" type="xs:string" minOccurs="0"/>

<xs:element name="MM_State" type="xs:string" minOccurs="0"/>

<xs:element name="EXTENSION" type="xs:string" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="MMSContentType">

<xs:sequence>

<xs:element name="charset" type="xs:string" minOccurs="0"/>

<xs:element name="Content-Transfer-Encoding" type="xs:string" minOccurs="0"/>

<xs:element name="Content-Location" type="xs:string" minOccurs="0"/>

<xs:element name="Data" type="xs:string" minOccurs="0"/>

<xs:element name="Content-type" type="xs:string" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ContentInMMSType">

<xs:sequence>

<xs:element name="ContentInfo" type="xs:string" minOccurs="0"/>

<xs:element name="Type" type="xs:string" minOccurs="0"/>

<xs:element name="Start" type="xs:string" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

When converting MMS data to XML representation, the MMS information data fields should be mapped to XML elements correctly, the specific data format and meaning for each MMS information field stored as character string in XML elements shall follow the definitions correspondingly given in [ETSI TS 126 140]、[ETSI TS 123 140] and [ETSI TS 124 011].

9.5	Other

More preferred XML representations could be specified if exchange of those data is of interest to the users. Meanwhile, only when the formats are relatively stable and mature enough, it is possible to choose or make a XML expression for those kinds of user data.

_________________