| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD XX(WP 1/16) |
|  |  | English only Original: English |
| Question(s): | 21/16 | Geneva, 28 October-8 November 2013 |
| TD |  |  |
| Source: | Editors H.VSMprot |  |
| Title: | H.VSMprot “Signalling and Protocols for Mobile Visual Surveillance” (New): Output draft |  |

Introduction

This document contains the editors’ output draft for H.VSMprot “Signalling and Protocols for Mobile Visual Surveillance” (New) from the SG16, Geneva, 28 October – 8 November 2013 meeting.

Draft new Recommendation ITU-T H.VSMprot

Signalling and Protocols for Mobile Visual Surveillance

# CONTENTS

1	Scope	3

2	References	3

3	Definitions	4

3.1	Terms defined elsewhere	4

3.2	Terms defined in this Recommendation	4

4	Abbreviations	4

5	Protocol Reference Points	4

5.1.	Reference point Cms: CMU-MSP	5

5.2.	Reference point Msm: MSP-M_CU	12

6	Signalling and Protocols	15

6.1	Real-time Video/Audio Acquisition	15

6.2	Recording and playback	15

6.3	PTZ Control	16

Bibliography	16

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 5-1 – Functional Architecture Framework of mobile Visual Surveillance 5

Draft new Recommendation H.VSMprot

Signalling and Protocols for Mobile Visual Surveillance

AAP Summary

[To be provided before Consent]

Summary

TBD.

Keywords

Mobile Visual Surveillance, Architecture, Signalling and Protocols

1	Scope

The Visual Surveillance service is a telecommunication service focusing on video (audio included also) application technology, which is used to capture remotely multimedia (such as audio, video, image and various alarm signal) and present them to the end users with friendly manner, based on managed broadband network with ensured quality, security and reliability ensured.

The Recommendation H.VSMprot focuses on the Signalling and Protocols of a visual surveillance system with mobile units, such as mobile customer unit or premises unit, as well as on the services flow related to mobile units, such as a mobile customer unit accessing real-time video stream from a visual surveillance system.

This Recommendation defines the reference points, signalling and protocols of the mobile visual surveillance system, based on the architecture defined in the Recommendation ITU-T H.VSMarch.

Furthermore, this recommendation will utilize and extend some existing protocols, such as SIP/RTP/RTSP/HTTP, etc. and it should give the message format as well as message text.

2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T F.743]	Recommendation ITU-T F.743 (2009), Requirements and service description for visual surveillance.

[ITU-T H.626]	Recommendation ITU-T H.626 (2011), Architectural requirements for visual surveillance.

[ITU-T H. 627]	Recommendation ITU-T H.627 (2012), Signalling and Protocols for visual surveillance.

[ITU-T H.VSMarch]	Recommendation ITU-T H.VSMarch (2011), Architectural requirements for mobile visual surveillance.

[IETF RFC 2326]	IETF RFC 2326 (1998), Real Time Streaming Protocol (RTSP).

[IETF RFC 2616]	IETF RFC 2616 (1999), Hypertext Transfer Protocol -- HTTP/1.1.

[IETF RFC 3261]	IETF RFC 3261 (2002), SIP: Session Initiation Protocol.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

3	Definitions

3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>

4	Abbreviations

TBD.

5	Protocol Reference Points

Recommendation [ITU-T H.VSMarch] describes overall architecture framework of mobile Visual Surveillance as the following figure, and it also introduces reference points of architecture. This document gives more detailed description of those reference points as well as some newly defined reference points.

Figure 5-1 – Functional Architecture Framework of mobile Visual Surveillance

TBD.

The interface protocols which involved in mobile video surveillance include following protocols:

- Reference point Cms: CMU-MSP, uses the Web Service interface protocol (SOAP/HTTP);

- Reference point Msm: MSP-M_CU, uses the HTTP protocol;

- Reference point Mv: M_CU-VAU, uses the UDP communication protocol;

- Reference point Vm: VAU-M_CU, uses the HTTP/RTSP, RTP/RTCP protocol;

- Reference point Psm: PSS-M_CU, uses the HTTP/RTSP, RTP/RTCP protocol;

- Reference point Vs: VAU-SCU, uses the HTTP protocol;

- Reference point Mdv: MDU-VAU, uses the RTP/RTCP protocol;

- Reference point Pv: PU-VAU, uses the RTP/RTCP protocol;

- Reference point Vp: VAU-PSS, uses the RTSP protocol;

  - Reference point Cms: CMU-MSP

5.1.1 Interface Function

The interface is used for querying the attributions of the surveillance point when the users login, and the information of all surveillance points available for mobile surveillance service.

- User authentication and surveillance point query: When user logs in MSP by M_CU, MSP could get the UserID and password of the current user (If user choose the universal sign-in way, MSP needs to translate the MDN number into the corresponding UserID and password locally), then MSP authenticates the user to the CMU of video surveillance platform via the User ID and password. If authentication failures, the failed message would return. If success, CMU would inquiry the surveillance point information according to current UserID, CMU returns the corresponding surveillance point and parameter list under the UserID in the MSP.

- Inquiry the enterprise surveillance points attribution: When enterprise users login Enterprise Self-Sevice System through the MSP, MSP could get the inquired  information of the current surveillance point list that enterprise has activated on CMU video surveillance platform through the enterprise account (such as CustID).

- RTSP media stream entrance inquiry: When user inquiry some kind of media streaming of one surveillance point, CMU would returns RTSP access address that user get through internal inquiry scheduling on the platform, which need the access token.

When universal sign-in user requests resource, according to the original interface specification, the method called "reqUserChannelInfo".

5.1.2 Interface Protocol

The interface is hosted on the IP protocol, using the Web Service interface protocol (SOAP/HTTP).

5.1.3 Message Format

- The enterprise attribute information query

MSP query the enterprise attribute information (used when handling business opening worksheet by Telecom administrator) according to the corporate identify (customer ID)

Table 5-1-1: Interface: MSP->CMU

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CustomerId | Enterprise ID | string(64) | Telecom administrator manually input when the MSP service opened (18 bit) |

Explanation: the first six bits of customerID refer to the domain ID (Platform domain)

Table 5-1-2: Interface: CMU->MSP

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CustomerName | Customer names | String(60) |  |
| CustomerType | Customer type | int(4) | Only : Enterprise:1 |
| Address | Customer address | String(100) |  |
| PhoneNumber | Customer phone number | String(30) |  |
| Fax | Customer fax number | String(30) |  |
| ZipCode | Customer zip code | String(6) |  |
| Email | Customer E-mail | String(60) |  |
| TransactPerson | The operator | String(30) |  |
| AdminAccount | video surveillance admin account | String(64) |  |
| ResultCode | Return code | Int(1) | 0 for enterprise ID not exist 1 for accession success |

Explanation: User use "adminAccount prefix" to login MSP Enterprise Self Service page, the user ID and user account are the same in format.

- Search enterprise information list

MSP query the enterprise attribute information (when handling business opening worksheet by Telecom administrator) according to the corporate identify (customer ID) or customer name.

Table 5-1-3: Interface: MSP->CMU

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| searchCID | Enterprise ID | string(64) | Telecom administrator manually input when the MSP service opened (18) |
| searchCN | Enterprise name | string(255) | Company name |

Explanation: the first six bits of customerID refer to the domain ID (Platform domain).

Table 5-1-4: Interface: CMU->MSP

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CustomerName | Customer names | String(60) |  |
| CustomerType | Customer type | int(4) | Only : Enterprise:1 |
| Address | Customer address | String(100) |  |
| PhoneNumber | Customer phone number | String(30) |  |
| Fax | Customer fax number | String(30) |  |
| ZipCode | Customer zip code | String(6) |  |
| Email | Customer E-mail | String(60) |  |
| TransactPerson | The operator | String(30) |  |
| AdminAccount | video surveillance admin account | String(64) |  |

Explanation: User use "adminAccount prefix" to login MSP Enterprise Self Service page, the user ID and user account are the same in format.

- Accept MSP user authentication

Accept MSP user authentication, including user’s login through the mobile phone and the enterprise administrator.

Table 5-1-5: Interface: MSP->CMU

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” Note: when the corresponding video surveillance platform without the customer domain, only need fill the user name. |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| AccountType | User account type | Int(1) | 0 for user 1 for enterprise administrator |

Explanation: The account is MSP user account or the enterprise administrator account which has prefix processed.

Table 5-1-6: Interface: CMU->MSP

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Authorization | Certification results | Int(1) | 0 for failed, 1 for success |
| The following list of fields |  |  |  |
| CategoryID | Catalogue number | String(128) |  |
| CategoryName | Catalogue name | string(128) |  |
| PID | The parent ID | string(128) | The parent directory CategoryID When the root directory PID element value defined for null values |

- User account query under the customer

The enterprise administrator could query for the list of all user accounts opened by enterprise.

Table 5-1-7: Interface: MSP->CMU

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CustomerId | Enterprise ID | string(64) |  |

Explanation:the first six bits of  customerID refer to the domain ID.

Table 5-1-8: Interface: CMU->MSP

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| ResultCode | Return code | Int(1) | 0 for enterprise ID not exist 1 for accession success |
| Account | User account | String(64) | “username@userdomain” Note: when the corresponding video surveillance platform without the customer domain, only need fill the user name. |

- Surveillance point query where user attribution

Query the authorized surveillance list under the specific user account.

Table 5-1-9: Interface: MSP->CMU

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” Note: when the corresponding video surveillance platform without the customer domain, only need fill the user name. |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| CategoryID | Catalogue number | string(64) | Catalogue numbers is a null value representing the query root directory |

Explanation: The account is MSP user account which has prefix processed.

Table 5-1-10: Interface: CMU->MSP

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| PuId-ChannelNo | PU Channel Number | string(128) |  |
| PuName | PUname | string(128) |  |
| PuProperty | PUProperty | structure type | Include ptzFlag, playbackFlag, online parameters at present, |
| PtzFlag | Control authority flag | int(1) | 0 for no authorization; 1 for authorization |
| PlaybackFlag | Playback authority flag | int(1) | 0 for no authorization; 1 for authorization |
| Online | Online flag | Int(1) | 0 for offline; 1 for online |
| ResultCode | Return code | Int(1) | 0 for no user account; 1 for success |

- Query the VAU entrance

Query the RTSP address of VAU entrance、PTZ entrance.

Table 5-1-11: reqVauUrlReques Interface

| MSP-CMU interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” Note: when the corresponding video surveillance platform without the customer domain, only need fill the user name. |
| M_cuIp (Optional) | M_CU’s IP address | String(32) | Because VAU can span multiple network deployment. This parameter is provided to CMUand used to determine the return VAU which segment address. By default M_CU fill in 0.0.0.0, CMU returns VAU foreign service public address. |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| NetType | Net Type | Int(2) | 0:Wap 1:Net |
| PuId-ChannelNo | Surveillance point unique identifier | String(128) |  |
| StreamingType | Streaming media type | Int(4) | 0:Normal 1:CDMA1x 2:EVDO 3:RESERVED |
| PlayMethod | Play type | Int(1) | 0: real-time video |

Response  message (video surveillance system ->MSP)

Table 5-1-12: reqVauAddResponse Interface

| CMU-MSP interface parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CMUPtzUrl | CMU PTZ URL address | String(128) | Return CMU PTZ URL address |
| VideoPlayUrl | Direct access entrance | String(128) | Return Direct access entrance URL address (PSS or VAU address) |

Among these, in addition to the VideoPlayUrl VAU/PSS address, it should contain the following fields:

Table 5-1-13:  contain fields

| Parameter | Parameter name | Data type | Notes |
| --- | --- | --- | --- |
| Account | User account | string(64) | “username@userdomain” Note: when the corresponding video surveillance platform without the customer domain, only need fill the user name. |
| NetType | Net Type | Int(2) | 0:Wap 1:Net |
| PuId-ChannelNo | Surveillance point unique identifier | String(128) |  |
| StreamingType | Streaming media type | Int(4) | 0:Normal 1:CDMA1x 2:EVDO 3:RESERVED |
| PlayMethod | Play type | Int(1) | 0: real-time video (fixed for 0 at present) |
| ExpireTime | Requesting the link failure time | String(16) | 20120406155530 Year-month-day-hour-min-sec |
| HashToken | MD5 abstract | String(64) | Sixteen hexadecimal calculated by the MD5 (Account=xx& Password =MD5 (password) &PuId-ChanelId=xx&StreamingType=xx& ExpireTime=xx) |

HashToken requirements:

- If the HashToken parameter values in VideoPlayUrl was artificially modified, VAU should denied accessment;

- If VideoPlayUrl video link requests time exceeds the ExpireTime failure time, the link is invalid;

Video streaming could play sustainably in the established RTSP media channels, not subject to ExpireTime time limit;

  - Reference point Msm: MSP-M_CU

5.2.1 Interface Function

M_CU and MSP interface use Web Service interface protocol (SOAP/HTTP), mainly to complete the following functions:

- Send the service request; realize video surveillance business docking and authentication through MSP;

- Get control point play entrance;

- Check version;

5.2.2 Interface Protocol

The interface is hosted on the IP protocol, using the Web Service interface protocol (SOAP/HTTP).

5.2.3 Message Format

Table 5-2-1: M_CU Authentication certification and Packet access

| M_CU→MSP |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| LoginType | Login type | Int(1) | 0: Account-password login 1: Local login |
| Version | M_CU version | String(16) | For example: V1.05.004, larger version needs to be consistent with MSP. If the version of MSP>M_CU that M_CU L / class; if M_CU>MSP version, then prompts the current version of M_CU is too high, please use the low version. |

Table 5-2-2: MSP-MCU

| MSP→M_CU |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter nam | Data type(unit: bit) | Notes |
| result | Certification results | Int(2) | 0:Success 1: user name not exist 2:wrong password 3: M_CU version too new 4:M_CU version too old |
| The following list |  |  |  |
| CategoryID | CategoryID | String(128) |  |
| CategoryName | CategoryName | string(128) |  |
| PID | The parent ID | string(128) | The parent directory CategoryID When the root directory PID element value defined for null values |

Table 5-2-3: Surveillance points list query

| M_CU→MSP |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| M_cuIp | M_CU’s IP address | String(32) | Because VAU can span multiple network deployment. This parameter is provided to CMU and used to determine the return VAU which segment address. By default M_CU fill in 0.0.0.0, CMU returns VAU foreign service public address. |
| Account | User account | string(64) | “username@userdomain” |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| CategoryID | Catalogue number | string(64) | Catalogue numbers is a null value representing the query root directory |

Table 5-2-4: returns list of surveillance point

| MSP→M_CU |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| PuId-ChannelNo | PU number and Channel Number | String(64) |  |
| PuProperty | PU ability of PTZ control | Int(4) | If no control ability, default is 0 |
| PuName | Surveillance point name | String(64) | For prompting the user to PU the specific significance in M_CU |
| Online | Online flag | Int(1) | 0 for offline; 1 for online |

PuId-ChannelNo: Corresponding to the PU number and channel number；

PuProperty: PU ability of PTZ control

Table 5-2-5: Video surveillance point play entrance query

| M_CU→MSP |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| NetType | Net Type | Int(2) | 0:Wap 1:Net |
| PuId-ChannelNo | PU number and Channel Number | String(64) |  |
| StreamingType | Streaming media type | Int(2) | 0:Normal 1:CDMA1x 2:EVDO 3:RESERVED |

Table 5-2-6: Returns the link address

| MSP→M_CU |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| CMUPtzUrl | CMU PTZ URL address | String(128) | Return CMU PTZ URL address |
| VideoPlayUrl | Direct access entrance | String(128) | Return Direct access entrance URL address (PSS or VAU address) |

Table 5-2-7: The statistics of mobile phone surveillance video data flows, duration

| M_CU→MSP |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type(unit: bit) | Notes |
| Account | User account | string(64) | “username@userdomain” |
| Password | User password | string(64) | Mandatory, 32 uppercase characters for MD5 coding of Password, such as: "96E79218965EB72C92A549DD5A330112" |
| NetType | Net Type | Int(2) | 0:Wap 1:Net |
| BRun | Start / end flag | Int(2) | 0: Start 1: Process (heartbeat) 2:Quit |
| NStreamBit | Video data flow | Int(2) | The unit is KB |
| NStreamTime | Video data time | Int(2) | The unit is Second |

Note: the statistical MCU access to the MSP, and the total length of the video data flow from MCU to MSP, report, in theory every normal exit report once. But the need to consider abnormal exit MCU, general users in more than 10 seconds, you need every 10 seconds to be reported to MSP a, MSP judge, statistics according to the account, to "exit" identification report shall prevail, or report the last to process shall prevail.

6	Signalling and Protocols

[NOTE – This clause will provide the identification of signalling or protocols.]

The following is some illustrative items:

- The SIP message format should comply with the definition in RFC3261;

- The RTSP message format should comply with the definition in RFC2326;

- The HTTP message format should comply with HTTP1.1 defined in RFC2616;

- RTP/RTCP signaling and message refers to RFC3550;

- TBD.

6.1	Real-time Video/Audio Acquisition

The Real-time Video/Audio Acquisition is used to view the real-time surveillance video/audio data by mobile terminals. After MSP performs authentication and returns the VAU information, the M_CU choose and request to the VAU for real-time video/audio data.

6.2	Recording and playback

The playback function is used to play audio and video which has been previously recorded and stored.

6.2.1	Playback in platform

6.2.2	Playback in PU

6.3	PTZ Control

The PTZ Control function is used to pan, tilt and zoom cameras, switch auxiliary peripherals (such as flashlight, fan and rain brush), and to preset the position of related physical devices.

Bibliography

[b-ITU-T H.Sup.1]	ITU-T H-series Recommendations – Supplement 1 (1999), Application profile – Sign language and lip-reading real-time conversation using low bit rate video communication.

________________