| Question(s): | 5 |  | Meeting, date: |  | Geneva, 28th October – 8th November 2013 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 1 | Intended type of document (R-C-TD): |  | C |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |  |  |
| Title: | F/H.TPS-SIG: Proposal for a new work item “Signalling for telepresence enabled conferencing” |  |  |  |  |  |
| Contact: | Weiwei Yang Huawei Technologies China |  |  |  | Tel: +86 29 87674367 Fax: +86 29 87674151 Email: tommy@huawei.com |  |
| Contact: | Jing Xiao Huawei Technologies China |  |  |  | Tel: +86 755 36830146 Fax: +86 755 36830131 Email: lennard.xiao@huawei.com |  |
| Contact: | Christian Groves NTEC Australia Australia |  |  |  | Tel: +61 3 9391 3457 Fax: +61 3 9391 3457 Email: Christian.Groves@nteczone.com |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

# Introduction

This Contribution proposes to start a new work item on “Signalling for telepresence enabled conferencing”.

# Discussion

The Oslo Q.5/16 meeting discussed starting work on Telepresence system signalling. A number of potential areas for contribution were highlighted including:

- A signalling framework,

- RTP usage,

- protocol usage,

- potential signalling channels, and

- analysis of approaches to integration the CLUE advertisement/configure methodology with H.323 capability exchange and negotiation.

It was agreed that the October/November Q.5/16 meeting would be a suitable time to launch new work in this area.

2.1 IETF Progress

The IETF CLUE working group have been working on drafts < draft-kyzivat-clue-signaling-05>, <draft-presta-clue-protocol-01> and < draft-ietf-clue-data-model-schema-00> which together provide more detail with respect to a CLUE protocol solution. The work is still far from maturity hopefully there are a number of aspects that have been agreed which may form the basis of work in Q.5. These aspects are noted below:

- CLUE Signalling channel: It has been agreed that CLUE will be its own protocol carried over a SCTP/DTLS/UDP transport. CLUE messages themselves will not be carried as part of session establishment call signalling however the CLUE channel will be established via SIP/SDP signalling.

- CLUE Messages: The basic messaging of Advertisements and Configures has been agreed.

- CLUE Versioning: It has been agreed that versioning will take place inside the CLUE protocol.

- The CLUE messages will be described utilising XML.

- The CLUE protocol is point to point. That is the messages are terminated and not transparently forwarded to other endpoints.

- In order to link CLUE captures and encoding to media a SDP construct is required to correlate them together. Currently the SDP grouping mechanism is proposed.

The work of the IETF CLUE WG is solely focussed on the support of CLUE in SIP based systems. There is no consideration of H.323. Thus in order to support CLUE in H.32x systems a signalling solution needs to be defined by ITU-T SG16.

2.2 Relation to H.323

ITU-T H.460.1 clause 5 provides a summary of the different methods that may be used to add functionality to H.323, i.e.

If the feature is applicable to a large number of applications, then it may be appropriate to add it to the H.225.0 ASN.1 base specification directly. Note that, as ITU-T Rec. H.323 is now fairly mature, it is not expected that such features will be encountered, and a strong case will need to be made for taking this course of action. The most likely situation in which this will occur is in the case of extending already existing parameters such as AliasAddress and TransportAddress.

•	Or if the feature only requires a few parameters, then define it as a GEF module.

•	Or if the feature requires data to be carried in H.225.0 RAS or H.225.0 Annex G, then define it as a GEF module.

•	Or if the feature contains numerous parameters, then define it as either ASN.1 (or similar method such as ABNF or XML) to be encoded into a raw element of a GEF content construct, or define an H.450 supplementary service.

ITU-T H.450.1 defines a generic functional protocol for the support of supplementary services in ITU-T H.323 systems. This is largely implemented through the carriage of application data protocol messages in H.225 messages.

ITU-T H.245 clause B.2.2.11 defines a way to introduce new capabilities into H.245 without the need to specify new syntax. Appendix VII provides procedures and template for defining new capabilities with ITU-T H.245 generic capabilities. It also allows the specification of new messages through the use of generic message as described in clause B.15/ITU-T H.245.

Recommendation ITU-T H.225.0 describes call signalling protocols and media stream packetization for packet-based multimedia communication systems and terminal to gatekeeper signalling RAS.

Recommendation ITU-T H.245 specifies syntax and semantics of terminal information messages as well as procedures to use them for in-band negotiation at the start of or during communication. The messages cover receiving and transmitting capabilities as well as mode preference from the receiving end, logical channel signalling, and Control & Indication.

Looking how CLUE is used in conjunction with SIP/SDP it can be seen that there are parallels with the functionality that H.245 provides. CLUE can be used during establishment and during communication. It is used to indicate transmitting capabilities and allows a preference to be sent from the receiving end. The support of CLUE is negotiated via SIP/SDP session signalling which is equivalent of H.225.0 call signalling.

Based on these observations it can be seen from the H.323 protocol addition guidelines and from an architectural and protocol perspective that making additions to H.245 via generic message and capabilities to support CLUE information would be an appropriate way forward.

# Proposal

It is proposed to accept the following as a baseline skeleton for a new work item H.TPS-SIG “Signalling for telepresence enabled conferencing”.

Annex Q5.1
Proposed new ITU-T Recommendation H.TPS.SIG

| Question: | 5/16 | Proposed new ITU-T Recommendation | Geneva, 28th October – 8th November 2013 |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference and title: | Recommendation ITU-T H.TPS-SIG "Signalling for telepresence enabled conferencing" |  |  |  |  |
| Base text: | This contribution |  | Timing: | 2014/2015 |  |
| Editor(s): | Christian/Lennard/Tommy? |  |  |  |  |
| Scope: |  |  |  |  |  |
| The scope of this Recommendation is the description of signalling mechanisms related to the support of telepresence in conferencing systems. It covers signalling related to peer-to-peer and multi-point call processes including telepresence support covering the follow phases: - Registration process - Call session setup process - Media stream transport and control process - Call session release process - Deregistration process In order to signal information regarding the characteristics of the telepresence session this Recommendation utilises the CLUE protocol [IETF CLUE PROT] as a means of describing, co-ordinating and controlling multiple media streams. |  |  |  |  |  |
| Summary |  |  |  |  |  |
| Support of telepresence in H.323 networks |  |  |  |  |  |
| Relations to ITU-T Recommendations or to other standards (approved or under development): |  |  |  |  |  |
| F.TPS-Req, F/H.TPS-Arch, H.TPS-AVP |  |  |  |  |  |
| Liaisons with other study groups or with other standards bodies: |  |  |  |  |  |
| Possibly with IETF (WG CLUE), 3GPP. |  |  |  |  |  |
| Supporting members that have committed to contribute in an active manner to this work item: |  |  |  |  |  |
| Huawei |  |  |  |  |  |

Draft new Recommendation H.TPS-SIG

Signalling for telepresence enabled conferencing

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

The scope of this Recommendation is the description of signalling mechanisms related to the support of telepresence in conferencing systems. It covers signalling related to peer-to-peer and multi-point call processes including telepresence support covering the follow phases:

- Registration process

- Call session setup process

- Media stream transport and control process

- Call session release process

- Deregistration process

In order to signal information regarding the characteristics of the telepresence session this Recommendation utilises the CLUE protocol [IETF CLUE PROT] as a means of describing, co-ordinating and controlling multiple media streams.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[F.TPS-Reqs]	Recommendation ITU-T F.yyy (date), Definitions, requirements, and use cases for Telepresence Systems.

[H.TPS-AV]	Recommendation ITU-T H.yyy (date), Audio/Video Parameters for Telepresence systems.

[ITU-T H.245]	Recommendation ITU-T H.245 (05/2011), Control protocol for multimedia communication.

[ITU-T H.323]	Recommendation ITU-T H.323 (12/2009), Packet-based multimedia communications systems.

[IETF CLUE FW]	IETF draft-ietf-clue-framework-10, Framework for Telepresence Multi-Streams.

[IETF CLUE PROT]	IETF draft-kyzivat-clue-signaling-05, CLUE Signalling.

# 3	Definitions

<Check in the ITU-T Terms and definitions database on the public website whether the term is already defined in another Recommendation. It may be more consistent to refer to such a definition rather than redefine it>

## 3.1	Terms defined elsewhere

<Normally terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

<abbr>	<definition>

<Include all abbreviations and acronyms used in this Recommendation>

# 5	Conventions

Information elements related to ITU-T H.series Recommendations are indicated in bold.

Information elements related to IETF Recommendations are indicated using itallics.

# 6	Framework

{Contributor’s note: The intention of this section is to provide a linkage to architecture document and to indicate what signalling interfaces are being covered by this draft Recommendation. It will also provide a high level introduction of how telepresence support is introduced into H.series Recommendations.}

# 7	Signalling support for CLUE

{Contributor’s note: This section shall contain information regarding the generic capabilities and messages as described in section 2.2 in the contribution description. It is excluded here for the sake of brevity.}

# 8	Signalling Procedures

{Contributor’s note: Many of the procedures will mirror the CLUE signalling draft. This draft is still in a state of flux with several major issues yet to be clarified, e.g. interaction between the CLUE channel and established media, version handling, media correlation etc.}

## 8.1	Registration

{Contributor’s note: The intention of this section is to describe how the support of telepresence may be indicated through RAS}

## 8.2	Call session setup

{Contributor’s note: The intention of this section is to describe how a telepresence enabled conference is set up. E.g. how the CLUE Signalling messages relate in an overall call flow.}

## 8.3	Media stream transport and control

{Contributor’s note: The intention of this section is to detail the interaction between CLUE signalling messages and logical channels whilst the session has been established. This will cover issues such as protocol state co-ordination etc.}

## 8.4	Call session release

{Contributor’s note: The intention of this section is to describe the call release process where CLUE signalling messages are used. i.e. the need of the CLUE state machine to be closed gracefully}

## 8.5	Deregistration

{Contributor’s note: The intention of this section is to describe if there are any consideration with regards to the registration process particular to telepresence sessions.

# 9	Multipoint control

{Contributor’s note: The intention of this section is to describe any behaviour specific to MCUs with regards to telepresence support. CLUE itself being a point to point protocol simplifies protocol handing. The support of multipoint is still an open area of discussion in CLUE, i.e. the method of support of switched and composed captures is still to be agreed.}

# 10	Security

{Contributor’s note: The intention of this section is to describe security considerations around the support of telepresence (in particular CLUE signalling messages). The IETF CLUE work assumes the use of a DTLS/SCTP/IP transport. Therefore for CLUE signalling in H.series based conferences to be equivalent in functionality a secure method of signalling H.245 messages is needed.

# 11	Interworking

{Contributor’s note: The intention of this section is to describe considerations / procedures related to interworking H.series and SIP based telepresence sessions. Also interworking with non-telepresence enabled endpoints.}

Annex A

<Annex Title>

(This annex forms an integral part of this Recommendation.)

<Body of annex A>

Appendix I

<Appendix Title>

(This appendix does not form an integral part of this Recommendation.)

<Body of appendix I>

Bibliography

[b-ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title