| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4487 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TCP: completion of TCP extended connection control package (clause 9 & Appendix IV) (by reference to H.248.SEPLINK) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides a revision of clause 9 and Appendix IV due to new reference to H.248.SEPLINK.

Discussion

Main editor's note to be resolved:

{Editor's note (2013-10 meeting): clause 9.1.1 (sepplink property) is dependent on the discussion the proposal for a generic "interlinkage" property, see  C-258 & C-264 ("H.248.SEPLINK"). If H.248.SEPLINK would be agreed, then following updates would be required for H.248.TCP:
a) clause 9.1.1: deleted
b) clause 9.6: updated by refererring to H.248.SEPLINK property
c) Appendix IV: updated.
The tcpecc package as such remains.).

Resolution:

We are suggesting to refer from clause 9.1.1 to clause 7.1.1/H.248.SEPLINK, rather then deleting clause 9.1.1. Because H.248.SEPLINK defines a generic, application-independent interlinkage capability, whereas the tcpecc package provides the usage and procedural information when applied on concrete bearer type "TCP".

The ori property is TCP-specific and may not simply replaced by the H.248.SEPLINK interlinkage capability (see discussion in AVD-4483).

Furthermore, Appendix IV is updated as well.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4473).

Change proposal #1:

[Begin Proposed Correction]

# 2	References

…

[ITU-T H.248.SEPLINK]	Recommendation ITU-T H.248.SEPLINK (2014), Gateway control protocol: Stream endpoint interlinkage package.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 9	TCP extended connection control package

|  | Package Name: | TCP extended connection control package |
| --- | --- | --- |
|  | Package ID: | tcpecc (0x####) |
|  | Description: | This extension package augments the base package behaviour by additional support of - TCP bearer connection control behaviour across a Stream Endpoint Pair (SEPP), rather instead of SEP-only scope; and - unidirectional TCP bearer connection release. |
|  | Version: | 1 |
|  | Extends: | tcpbcc |

## 9.1	Properties

### 9.1.1	Stream endpoint interlinkage capability

### Void clause. Property interlinkage topology (linktopo) from [ITU-T H.248.SEPLINK], clause 7.1.1 may be used for extended TCP connection control procedures at establishment or release phase.

### 9.1.2	Oneway Release Indicator

|  | Property Name: | Oneway Release Indicator |
| --- | --- | --- |
|  | Property ID: | ori (0x0003) |
|  | Description: | This property defines the behaviour in the MG in case a TCP-FIN is received for an established TCP connection. It instructs the MG whether or not to send a TCP-FIN. |
|  | Type: | Boolean |
|  | Possible values: | Table 23 defines the protocol semantic per value. Table 23 – Semantic of property ori Value; MG semantic for TCP bearer connection. False; Indicate the duplex closure of the TCP bearer connection by sending a TCP-FIN autonomously.. True; Keep the TCP bearer connection in the half-closed state, i.e. the MG shall not generate the TCP-FIN autonomously. The MGC would explicitly release the TCP bearer connection via signal tcpbcc/RelBNC.. |
|  | Default: | False (because this relates to the MG behaviour according the base package (tcpbcc), see clause 8). |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 9.2	Events

None.

## 9.3	Signals

None.

## 9.3	Statistics

None.

## 9.5	Error codes

None.

## 9.6	Procedures

### 9.6.1	Introduction

The procedures are related to the extension package tcpecc and are on top of the base package tcpbcc procedures according clause 8. The interlinkage property seplink/linktopo (according to [H.248.SEPLINK]) could be applied for bearer type "TCP" and the establishment and release procedures for TCP.

### 9.6.2	TCP bearer connection establishment

Only relevant for incoming TCP bearer connection establishment requests from MG external bearer interface. Outgoing procedures are not affected.

#### 9.6.2.1	Stream endpoint pair interlinkage disabled

If property seplink/linktopo is not used (or disabled per default), then the incoming TCP bearer connection establishment procedure shall be identical to base package tcpbcc procedures.

NOTE – Clause IV.3.1 provides signalling examples.

#### 9.6.2.2	Stream endpoint pair interlinkage enabled

The interlinkage property seplink/linktopo may be enabled for TCP establishment (see example H.248 syntax in Table x.1), then the incoming TCP bearer connection establishment request shall be propagated to the partner SEP, triggering there an outgoing TCP-SYN.

Table x.1 – TCP bearer connection establishment – Interlinkage enabled

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:54321 Transaction = 1 { Context = 1 { Add = ip/1/$/$ { ; Termination T1 Media { Stream = 1 { ; SEP S1 LocalControl { seplink/linktopo = ["<interlinkedSEP>:TCP:TCP:est"] }}…} | Interlinkage (for stream S1) is enabled from termination T1 to termination T2 (given by property parameter value "interlinkedSEP"). |

NOTE – Clause IV.3.2 provides signalling examples.

### 9.6.3	TCP bearer connection release

Only relevant for incoming TCP bearer connection release requests from MG external bearer interface. Outgoing procedures are not affected.

#### 9.6.3.1	Stream endpoint pair interlinkage disabled and Oneway release indicator disabled

If property seplink/linktopo is not used (or disabled per default), then the incoming TCP bearer connection release procedure shall be not propagated to the partner SEP.

If the property ori is set to False, then the incoming TCP bearer connection release procedure shall lead to a bidirectional TCP bearer connection release, hence identical to base package tcpbcc procedures.

#### 9.6.3.2	Stream endpoint pair interlinkage enabled and Oneway release indicator disabled

The interlinkage property seplink/linktopo may be enabled for TCP release (see Table x.2), then incoming TCP bearer connection release request shall be propagated to the partner SEP, triggering there an outgoing TCP-FIN.

If the property ori is set to False, then see clause 9.6.2.1.

NOTE – Appendix IV.5.3 provides signalling examples.

#### 9.6.3.3	Stream endpoint pair interlinkage disabled and Oneway release indicator enabled

If property seplink/linktopo is not used (or disabled per default), then see clause 9.6.2.1.

If the property ori is set to True, then the incoming TCP bearer connection release procedure shall lead to a one TCP bearer connection release (so called TCP half closure).

#### 9.6.3.4	Stream endpoint pair interlinkage enabled and Oneway release indicator enabled

The interlinkage property seplink/linktopo may be enabled for TCP release (see Table x.2), then see clause 9.6.2.2.

If the property ori is set to True, then see clause 9.6.2.3.

Table x.2 – TCP bearer connection release – Interlinkage and 
oneway release indicator enabled

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:54321 Transaction = 1 { Context = 1 { Add = ip/1/$/$ { ; Termination T1 Media { Stream = 1 { ; SEP S1 LocalControl { seplink/linktopo = ["<interlinkedSEP>:TCP:TCP:rel"] tcpecc/ori = TRUE }}…} | Both tcpecc package capabilities are enabled. |

NOTE – Appendix IV.5.3 provides signalling examples.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Appendix IV

Illustration of the TCP extended connection control package protocol semantics

(This appendix does not form an integral part of this Recommendation.)

## IV.1	Overview

The tcpecc package (clause 9) defines an augmented TCP bearer connection control to the base package (tcpbcc, clause 8). The effect of each individual package capability is illustrated in clauses IV.3 ([ITU-T H.248.SEPLINK] property seplink/linktopo applied on TCP) and IV.4 (property tcpecc/ori). Clause IV.5 describes examples of combined usage of both capabilities.

The illustrated use cases are some examples. An exhaustive consideration of all possible combinations is out of scope.

## IV.2	Conventions

An H.248 context with a single SEPP, given by SEPs T1(S1) and T2(S1), is considered. The two tcpecc package capabilities are related to individual SEPs (as opposed to Context-level properties); however, the scope is on SEP T1(S1) property settings only.

Further, both capabilities provide only an extended behaviour (in contrast to the base package) for incoming TCP bearer connection control requests (i.e. the terminating side). The originating side is therefore out of scope of this Appendix.

Furthermore all Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) are not discussed, but could be for instance related to state changes of the local TCP bearer connection endpoint(s) (as defined by the base package).

## IV.3	Usage of SEPP Interlinkage property

Property seplink/linktopo is defined in clause 7.1.1/[ITU-T H.248.SEPLINK] and TCP-specific interlinkage procedures are defined in clause 9.1.1. Interlinkage may bes used for TCP bearer connection establishment and release procedures.

### IV.3.1	Property "seplink/linktopo" not used for interlinkage

There will be decoupled SEPs in case of unused interlinkage (abstracted by semantic FALSE in Figure IV.1), i.e., reflect the semantic of the base package. There are two separate TCP bearer connection segments, - the two segments related to TCP connections "X  MG (SEPT1(S1))" and "Y  MG (SEPT2(S1))" -, from the perspective of the MG.

Figure IV.1 – Property "seplink/linktopo" not used for interlinkage

Here: an incoming TCP bearer connection establishment request (via TCP-SYN) or release request (via TCP-FIN) at SEP T1(S1) does not affect SEP T2(S1).

Figure IV.2 illustrates correspondent example TCP bearer establishment signalling scenarios:

Figure IV.2 – Signalling example for TCP bearer connection establishment, property "seplink/linktopo " not used for interlinkage

It may be emphasized that the seplink/linktoposetting at SEP T2(S1) does not have any impact in these scenarios.

If an incoming TCP bearer connection establishment request from remote X should also lead to establishment procedures towards remote TCP endpoint Y, then the MGC needs to be involved (see E.1 example).

### IV.3.2	Property "seplink/linktopo= enabled for establishment and release"

See Figure IV.3. Semantic TRUE (i.e., a property value setting of seplink/linktopo equal to "<interlinkedSEP>:TCP:TCP:est,rel") leads to a forwarding of the TCP bearer connection control request within a SEPP.

Figure IV.3 – Property "seplink/linktopo" enabled for establishment and release at SEP T1(S1)

The TCP open and closure procedures, when initiated by remote TCP endpoints (X or Y), could now have an end-to-end scope. E.g., the three-way handshake for TCP bearer connection establishment or a TCP half-closure for oneway TCP bearer connection release would be lead by remote TCP endpoints (X, Y) and forwarded via the SEPP, under the condition that both seplink/linktopoproperties (of SEPT1(S1) and SEPT2(S1)) would be enabled for interlinkage.

Figure IV.4 illustrates correspondent signalling scenarios at the example of TCP bearer establishment:

Figure IV.4 – Signalling example for TCP bearer connection establishment, 
property "seplink/linktopo" enabled

It may be emphasized that the seplink/linktoposetting at SEP T2(S1) does not have any impact in these scenarios (because the property only consider incoming procedures at SEP T1(S1)).

## IV.4	Usage of Oneway Release Indicator property

Property tcpecc/ori is defined in clause 9.1.2. The property affects only TCP bearer connection release procedures.

### IV.4.1	Property "ori = FALSE"

See Figure IV.5. Semantic FALSE implies base package behaviour, i.e., a complete TCP bearer connection release.

Figure IV.5 – Property "ori = FALSE"

Here: an incoming TCP bearer connection release request (via TCP-FIN) at SEPT1(S1) triggers an immediate TCP half-closure of the reverse direction towards X, but does not affect SEPT2(S1).

### IV.4.2	Property "ori = TRUE"

Semantic TRUE results in a one-way TCP bearer connection release, see Figure IV.6.

Figure IV.6 – Property "ori = TRUE"

## IV.5	Combined consideration of both properties

Only TCP bearer connection release (due to ori property) needs to be considered.

### IV.5.1	Property settings: "ori = FALSE" and "seplink/linktopo= TRUE"

See Figure IV.7. The ori property codepoint leads to a complete TCP bearer connection release between MG and remote TCP endpoint X, and the seplink/linktopoproperty codepoint results in an initiation of a TCP half-closure procedure from SEPT2(S1) towards remote TCP endpoint Y. Whether the entire TCP bearer connection with Y will be completely released depends on remote TCP endpoint Y.

Figure IV.7 – Property settings: "ori = FALSE" and "seplink/linktopo enabled for release"

### IV.5.2	Property settings: "ori = TRUE" and "seplink/linktopo= enabled for release"

See Figure IV.8. The ori property codepoint leads to an oneway TCP bearer connection release only between MG and remote TCP endpoint X, and the seplink/linktopoproperty codepoint results again in an initiation of a TCP half-closure procedure from SEPT2(S1) towards remote TCP endpoint Y. Whether the entire TCP bearer connection with Y will be completely released depends on remote TCP endpoint Y.

Figure IV.8 – Property settings: "ori = TRUE" and "seplink/linktopoenabled for release"

### IV.5.3	Signalling examples

Figure IV.9 summarizes the impact of both properties (Note: only enabled seplink/linktopodepicted; unused interlinkage does not provide any new information).

Figure IV.9 – Signalling example for TCP bearer connection release and example value combinations for both properties

Use case (C.1): the incoming TCP bearer connection release request from remote TCP endpoint X is propagated on the other side to remote TCP endpoint Y, and the base package behaviour of a bidirectional TCP connection release towards X.

Use case (C.2): Property ori enforces a TCP half-closure in the direction to X, and seplink/linktopothe start of TCP bearer connection release towards Y.

[End Proposed Correction]

____________________