| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4504 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.39 (Rev.): Wildcard "~" – Continuation of discussion |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution tries to progress the additonal support of new wildcard "~".

1	Background

First discussion cycled at 2013-06 meetings (see AVDs 4370, 4371, 4372 and 4463). The topic wasn't discussed at 2013-10 meeting due to different priorities.

2	Discussion

We'd like to recall the concerns raised against wildcard "~":

Q.3 report (2013-06 meeting):

| AVD-4370. | H.248.39 (Rev): New SDP wildcard proposal. | Alcatel-Lucent |
| --- | --- | --- |

This contribution was discussed. It was agreed to add the text that is not related to the “~” wildcard. It was felt that they were several issues related to this new wildcard:

- The ability to have “short form” syntax may be equally applicable to both the text and binary H.248 encodings.

- It was thought that using “~” was not appropriate for the first command on a termination.

- Further analysis should be made as to whether this should be a generic mechanism or should be related to specific SDP elements.

| AVD-4371. | H.248.39 (Rev): new clause 6.15.10 - Wildcarding SDP “a=fingerprint:”. | Alcatel-Lucent |
| --- | --- | --- |

This contribution was discussed. It was agreed to add the text that is not related to the “~” wildcard.

| AVD-4372. | H.248.39 (Rev): clause 6.15.9 - Update proposals. | Alcatel-Lucent |
| --- | --- | --- |

This contribution was discussed. It was agreed to add the text that is not related to the “~” wildcard. Several issues were identified during the discussions. It was agreed that these be documented in editor’s notes:

- It was identified that it was not possible to indicate recursiveness via H.248 wildcarding. This should be investigated.

- The introduced examples shows the case were wildcarding is used across multiple SDP lines. This may be cause ambiguous behaviour. This needs to be investigated further.

| AVD-4463. | H.248.39 (Rev): living list items - further SDP element candidates for "~" wildcarding. | Alcatel-Lucent |
| --- | --- | --- |

This contribution was discussed. Rather than creating a living list it was agreed to document the attributes in an editor’s note.

Discussion of comments:

- Generic or SDP attribute specific wildcard

Question: limited to specific SDP elements or not?

Reply: we can't identify any specific SDP element for exclusion, based on a first evaluation. The principle as such should be generic (H.248.39). The SDP section in H.248 profiles lists anyway the set of supported SDP elements, which might be then the right place to indicate wildcard usage or not. Thus, the same proceeding as with wildcard "-".

Propopsal: extended text (in comparison to original AVD-4370)

- Usage related to H.248 command order

Question: only in subsequent commands, or use cases for first command, too?

Reply: the general case is related to subsequent commands. However, the general principle in H.248 in support of provisioning of resource related values (via management actions, at MGC and/or MG level) could lead to scenarios where the wildcard "~" could be applied as well in the first command. We can't exclude such scenarios, despite the fact that they look rather theoretical at this stage.

Propopsal: extended text (in comparison to original AVD-4370)

- Wildcarding across multiple SDP line

Question: how could we guarantee unambiguity for such wildcarding?

Reply: this subject is similar to implicit vs explicit signalling and potential interaction issues. Whenever unambiguity couldn't be guaranteed, then explicit signalling must be used.

Propopsal: extended text (in comparison to original AVD-4370)

- Recursive wildcarding

Question: unambiguity in case of recursive wildcarding?

Reply: this subject is unclear, might disappeared since the restructering of clause 6.15.9 at the last meeting (hm)?

Proposal: -

- H.248 encoding mode

Question: only in text-encoding mode, or also in binary?

Reply: wildcarding in binary mode is in general rather "tricky" (see Annex A/H.248.1) in comparison to the explicit characters "$", "*" and "-", which are universally applicable for all kind of wildcarding in text mode (such as TerminationIDs, ContextIDs,… besides SDP elements). It could be also observed that wildcard "-" is also not (yet) supported in binary mode.

On the other side, SDP is presently used primarily in IP networks and all correspondent, standardized H.248 profiles support all text-encoding mode (with one or two exeptions). Thus, support in text-encoding mode is mandatory and binary support is optional.

Thus, support of wildcard "~" in binary should be coupled with support of wildcard "-" (which is much more important and used).

Propopsal: limitation to text mode (firstly).

3	Conclusions

We've tried to address the major concerns. The usage of wildcard "~" is limited in principle, as the limited usage of wildcard "-". A revised change request is proposed for the main body of Rec. H.248.39. A new Appendix I shall illustrate the benefit of wildcard "~".

4	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4515).

Change proposal #1:

[Begin Proposed Correction]

# 1	Scope

This Recommendation describes the principles to be used to identify a single SDP sub-field and how to apply wildcarding to that sub-field. The sub-field may be fully specified, wildcarded (CHOOSE "$" or ALL "*") or not significant "-" (see clause 5.2.2.2). Being able to identify a single sub-field is of use when requesting the MG to choose a parameter, specifying a range or for the purposes of auditing.

Furthermore a new wildcard "~" is introduced (see clause 5.2.2.3), which could be considered as a short notation syntax (versus fully specified SDP information) and allows performance optimization as well as the reduction of syntax errors (due to avoidance of the repetition of redundant information).

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 5	H.248 wildcarding guiding principles

## 5.1	General wildcarding principles by H.248

See clause 6.3/[ITU-T H.248.1]: The overriding guidance provided by [ITU-T H.248.1] on the use of CHOOSE in SDP is: "the use of CHOOSE is allowed in place of a single parameter value".

Subsequent principles are:

1)	In any SDP scheme each mandatory element SHALL therefore be represented by a single CHOOSE "$".

2)	Any part that contain text tokens (e.g. those in " ") that are mandatory SHALL also be included in the wildcard command.

3)	Optional parts of SDP scheme should not be included unless the wildcard or value applies to them.

4)	When an item specifies a wildcard for a single sub-field, this applies to the nested ABNF in that parameter.

## 5.2	Specific wildcarding principles for H.248 embedded SDP

## 5.2.1	Background: IETF SDP

The core SDP [IETF RFC 2327], as well as its successor [IETF RFC 4566], does not specify any wildcards. SDP lines are therefore always fully specified.

[ITU-T H.248.49] provides, inter alia, complementary information concerning relation and usage of H.248 and the various IETF RFCs on SDP.

### 5.2.2	H.248-introduced SDP wildcards

### 5.2.2.1	Purpose

Wildcards are used (by the MGC) for resource management or/and the indication of particular modes of operation at MG level (aware versus agnostic). The scope of SDP wildcards in H.248 messages is therefore limited to SDP media descriptions (see clause 7.1.8.1.1.2/[ITU-T H.248.1]), within the LD and RD.

#### 5.2.2.2	Wildcard "-"

Wildcard "-", with the semantic of "not significant" (or "agnostic", "unaware"), was introduced (in 2005) by the so-called "H.248 Ia profile version 1" (see clause 5.15/[ETSI ES 283 018]), motivated by the fact for the control of dedicated "IP-to-IP interworking modes" in border gateways (see clause G.2.2/[ETSI TR 183 068]).

Wildcard "-" is only supported in H.248 text-encoding mode.

#### 5.2.2.3	Wildcard "~"

Purpose of this wildcard is the preservation of MG-level resources or the configuration data or resources.

Semantic of wildcard "~":

maintain unchanged already 
a) signaled attribute values, or
b) previously wildcarded attribute values, or
c) provisioned attribute values.

Use cases for (a, b):

The wildcard is basically used in H.248 command replies (when SDP information is replied).

The wildcard is typically used in subsequent H.248 command requests, i.e., this wildcard is not used in the first H.248 command request, when initially creating a stream endpoint or termination.

Use case (c): The wildcard could be actually already used in the first H.248 command request, under the condition that correspondent resources already prepared via management actions (at MGC and MG level).

Purpose/motivation:

Not using this wildcard implies the complete replication of all SDP information (i.e., fully specified SDP lines) by the MGC, in order not to release previously requested resources at the MG. The MG must process entirely such SDP data and compare it with past received SDP. That’s a burdensome and unnecessary task, at MGC and MG side, which might be abbreviated by this wildcard. Appendix I illustrates an example.

Wildcard "~" is only supported in H.248 text-encoding mode. Wildcard "~" shall not be used in case of ambiguous SDP media descriptions (e.g., when multiple SDP lines would be linked via an identifier, which is wildcarded).

# 6	Identification of single parameters

The subclauses below define which parts of SDP are classified as "single sub-fields (values)" for the purposes of H.248.1 wildcarding. Each of these sub-fields may be fully specified, designated as CHOOSE "$", designated as ALL "*" or designated as not significant "-". The sub-fields should not be partially wildcarded.

Each of the examples indicates what type of wildcarding is not valid. It also highlights some valid types of wildcarding and what should be returned.

NOTE 1 – For the purposes of the examples below, "?" is used to denote the potential wildcards:

•	"$" 		for Choose

•	"*" 		for All,

•	"–" 		"Not Significant" (NOTE 2), and

•	"~" 		"Maintain unchanged" (or "Preserve") (see clause 5.2.2.3).

NOTE 2 – Not to be confused with Annex B.2/H.248.1 encoding for NULL Context (see also clause 5.2.2.2).

ABNF grammar in subsequent clauses is copied from Appendix A (SDP Grammar) in [IETF RFC 2327] (which is replaced by clause 9/[IETF RFC 4566]) and other IETF RFCs concerning SDP extensions.

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Appendix I
Example signalling scenarios for 
wildcard "~"

(This appendix does not form an integral part of this Recommendation)

## I.1	Purpose

The benefit of wildcard "~" is illustrated with an example without (clause I.2) and with (clause I.3) wildcarding. The value of this wildcard is primarily motivated by the "size of SDP information". E.g., in context of the discussion of large SDP objects (also known as "SDP blob"), such as used in the signalling SDP media descriptions for

- multi-media types/formats/parameters/etc. configurations,

- key material as part of cryptographical information,

- fingerprints as part of cryptographical information,

- authentication related information, and

- others.

## I.2	Signalling without wildcard "~"

The usage of wildcard "~" is illustrated at the example of SDP attribute "a=fingerprint". This SDP element is an example representation of SDP elements which may get fairly large value size. A ADD.request command example without wildcard usage is used as starting point, see Table I.1.

Table I.1 – Example command encoding – MGC Add request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { Add = ip/$/$/$ { Media { Stream = 1 { LocalControl { ... }, Local { v=0 c=IN IP4 $ m=audio $ UDP/TLS/RTP/SAVP - b=AS:7 a=setup:passive a=fingerprint: sha-256 0E:CB:C3:2A:EF:E8:3F:6A:FD:19:07:20:10:80:69:B4:17:E2:3E:3B:2C:89:41:FB:06:85:E6:71:26:A2:19:51 }, Remote { v=0 c=IN IP4 135.117.113.26 m=audio 56376 UDP/TLS/RTP/SAVP - a=fingerprint:sha-256 8F:AA:57:46:7C:C1:D3:50:74:8D:22:C9:10:12:BE:AD:F0:13:9C:4F:D1:4F:B7:F3:96:5B:C3:70:FC:C5:AA:AE } … } | The SDP attribute "a=fingerprint" is contained in both the LD and RD. The value is fully specified in the request command in MGC-to-MG direction. |

A possible example MG reply is indicated in Table I.2:

Table I.2 – Example command encoding– MG Add reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [125.125.125.111]:55555 Reply = 1 { Context = <C1> { Add = <IP_A> { Media { Stream = 1 { Local { v=0 c=IN IP4 172.40.7.1 b=AS:7 m=audio 23914 UDP/TLS/RTP/SAVP - a=setup:passive a=fingerprint:sha-256 0E:CB:C3:2A:EF:E8:3F:6A:FD:19:07:20:10:80:69:B4:17:E2:3E:3B:2C:89:41:FB:06:85:E6:71:26:A2:19:51 … }, Remote { v=0 c=IN IP4 135.117.113.26 m=audio 56376 UDP/TLS/RTP/SAVP - a=fingerprint:sha-256 8F:AA:57:46:7C:C1:D3:50:74:8D:22:C9:10:12:BE:AD:F0:13:9C:4F:D1:4F:B7:F3:96:5B:C3:70:FC:C5:AA:AE } ... } | The entire SDP value is replied by the MG. Now, the MGC should validate that information by comparing the sent and received data. |

The stream endpoint is modified in a subsequent command cycle, e.g., by following MODify.request command (Table I.3):

Table I.3 – Example command encoding – MGC Modify request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = <C1> { Modify = <IP_A> { Media { Stream = 1 { LocalControl { ... }, Local { v=0 c=IN IP4 172.40.7.1 b=AS:7 m=audio 23914 UDP/TLS/RTP/SAVP - a=setup:passive a=fingerprint: sha-256 0E:CB:C3:2A:EF:E8:3F:6A:FD:19:07:20:10:80:69:B4:17:E2:3E:3B:2C:89:41:FB:06:85:E6:71:26:A2:19:51 a=rtcp-mux }, Remote { v=0 c=IN IP4 135.117.113.26 m=audio 56376 UDP/TLS/RTP/SAVP - a=fingerprint:sha-256 8F:AA:57:46:7C:C1:D3:50:74:8D:22:C9:10:12:BE:AD:F0:13:9C:4F:D1:4F:B7:F3:96:5B:C3:70:FC:C5:AA:AE a=rtcp-mux } … } | The MGC has to provide the full specification of the SDP attribute again. The MG has to validate that information by comparing (and possibly updating) the information received in the previous command request (Table I.1). This is a cumbersome and unnecessary tasks when the value itself was not changed (as here). |

A possible example MG reply is indicated in Table I.4:

Table I.4 – Example command encoding– MG Modify reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [125.125.125.111]:55555 Reply = 2 { Context = <C1> { Modify = <IP_A> { Media { Stream = 1 { Local { v=0 c=IN IP4 172.40.7.1 b=AS:7 m=audio 23914 UDP/TLS/RTP/SAVP - a=setup:passive a=fingerprint:sha-256 0E:CB:C3:2A:EF:E8:3F:6A:FD:19:07:20:10:80:69:B4:17:E2:3E:3B:2C:89:41:FB:06:85:E6:71:26:A2:19:51 a=rtcp-mux … }, Remote { v=0 c=IN IP4 135.117.113.26 m=audio 56376 UDP/TLS/RTP/SAVP - a=fingerprint:sha-256 8F:AA:57:46:7C:C1:D3:50:74:8D:22:C9:10:12:BE:AD:F0:13:9C:4F:D1:4F:B7:F3:96:5B:C3:70:FC:C5:AA:AE a=rtcp-mux } … } | Same comment as in Table I.2. Now, the MGC might be loaded with an unnecessary task. |

## I.3	Signalling using wildcard "~"

The same signalling example, with it's two command request/reply cycles, is now used with wildcard "~". The wildcard as such is applicable due to unchanged parameter values. Table I.5 provides again the starting point:

Table I.5 – Example command encoding – MGC Add request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { Add = ip/$/$/$ { Media { Stream = 1 { LocalControl { ... }, Local { v=0 c=IN IP4 $ m=audio $ UDP/TLS/RTP/SAVP - b=AS:7 a=setup:passive a=fingerprint: sha-256 0E:CB:C3:2A:EF:E8:3F:6A:FD:19:07:20:10:80:69:B4:17:E2:3E:3B:2C:89:41:FB:06:85:E6:71:26:A2:19:51 }, Remote { v=0 c=IN IP4 135.117.113.26 m=audio 56376 UDP/TLS/RTP/SAVP - a=fingerprint:sha-256 8F:AA:57:46:7C:C1:D3:50:74:8D:22:C9:10:12:BE:AD:F0:13:9C:4F:D1:4F:B7:F3:96:5B:C3:70:FC:C5:AA:AE } … } | This is identical to Table I.1, i.e., a fully specified SDP value (because such cryptographical information is normally out of scope of any provisioning). |

A possible example MG reply is indicated in Table I.6:

Table I.6 – Example command encoding– MG Add reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [125.125.125.111]:55555 Reply = 1 { Context = <C1> { Add = <IP_A> { Media { Stream = 1 { Local { v=0 c=IN IP4 172.40.7.1 b=AS:7 m=audio 23914 UDP/TLS/RTP/SAVP - a=setup:passive a=fingerprint:~ ~ … }, Remote { v=0 c=IN IP4 135.117.113.26 m=audio 56376 UDP/TLS/RTP/SAVP - a=fingerprint:~ ~ } … } | The two SDP attribute parameter values could be both wildcarded "~". Thus, the MGC is simply acknowledged about the accepted values by the MG. |

Table I.7 illustrates the MODify.request command:

Table I.7 – Example command encoding – MGC Modify request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = <C1> { Modify = <IP_A> { Media { Stream = 1 { LocalControl { ... }, Local { v=0 c=IN IP4 172.40.7.1 b=AS:7 m=audio 23914 UDP/TLS/RTP/SAVP - a=setup:passive a=fingerprint: ~ ~ a=rtcp-mux }, Remote { v=0 c=IN IP4 135.117.113.26 m=audio 56376 UDP/TLS/RTP/SAVP - a=fingerprint:~ ~ a=rtcp-mux } … } | The MGC could use again "~" wildcarding. This simplifies processing at MGC and MG side! |

A possible example MG reply is indicated in Table I.8:

Table I.8 – Example command encoding– MG Modify reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [125.125.125.111]:55555 Reply = 2 { Context = <C1> { Modify = <IP_A> { Media { Stream = 1 { Local { v=0 c=IN IP4 172.40.7.1 b=AS:7 m=audio 23914 UDP/TLS/RTP/SAVP - a=setup:passive a=fingerprint:~ ~ a=rtcp-mux … }, Remote { v=0 c=IN IP4 135.117.113.26 m=audio 56376 UDP/TLS/RTP/SAVP - a=fingerprint:~ ~ a=rtcp-mux } … } | And the MG could also use again "~" wildcarding. This simplifies processing at MG and MGC side! |

## I.4	Evaluation of example

Wildcard "~" could be applied in 3-out-of-4 commands in this example (see clause I.3 versus I.2). It could be also demonstrated that wildcard "~" is used in command replies (see Tables I.6 and I.8) besides the usage in the MODify command request (see Table I.5).

Furthermore, it should be underlined that the processing savings are not only related to the sending H.248 entity, but rather both H.248 entities (MGC and MG) benefit from wildcarding.

[End Proposed Correction]

____________________

Blob: binary or basic large object, relates to a collection of data stored as a single entity.