| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4530 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co. Ltd. |  |  |  |
| Title: | H.TPS-AV: Default values of loudness ratings and weighted microphone-loudspeaker coupling loss |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution addresses clarifications related to default values of loudness ratings and weighted microphone-loudspeaker coupling loss in sections 7.1.3.5, 7.1.3.6 and 7.2.2.1 of draft Rec. H.TPS-AV and proposes new text.

# Introduction

Progress is being made on draft Rec. H.TPS-AV, but a number of areas remain to be completed. This contribution addresses some clarifications for sections 7.1.3.5, 7.1.3.6 and 7.2.2.1 in the latest version of draft Rec. H.TPS-AV [1], dealing with default values for the parameters of sending and receiving loudness ratings, and weighted microphone-loudspeaker coupling loss.

# Discussion

In previous meetings it has been agreed that audio levels in Telepresence systems should be specified in terms of sending and receiving loudness ratings, generally following Rec. P.300 [2]. The current text in sections 7.1.3.5 and 7.2.2.1 of H.TPS-AV indicates “TBD” as the default value for these parameters, while possible values are indicated by a formula depending on correction factors derived from the actual microphone and loudspeaker placement. This is incorrect, in that the default values should be determined by the formulae, and the possible values should give an indication of the overall range of values likely to be encountered, taking into account transducer tolerances and the correction for the actual microphone and loudspeaker placement. This range will be wider for receiving compared to sending, to take into account the user volume control.

In addition, in the supplementary information for sending loudness rating, there is mention of a term “microphone-speaker operating distance (ds)”. To avoid any confusion with the word “loudspeaker” used elsewhere in the document, it is suggested that this term should be “microphone-talker operating distance (ds)”.

For weighted microphone-loudspeaker coupling loss in section 7.1.3.6, possible values are indicated by “≥ X dB”. In fact, guidance on a suitable value is given in [3], where a value of ≥ 46 db is specified for nominal volume, and ≥40 db for maximum volume.

In addition, it is suggested the title of this parameter should be written correctly, rather than concatenated as for the identity.

# Proposal

With the above discussion in mind it is proposed to accept the following updates to H.TPS-AV. Changes are marked up against TD 89R1 (WP1/16) from the November 2013 SG16 meeting in Geneva.

Start proposed change 1

7.1.3.5	Sending Loudness Rating

| Identity: | SendLoudnessRating |
| --- | --- |
| Description: | This parameter indicates the acoustic to electric transfer function of the Telepresence audio stream. |
| Format: | Numeric, in dB |
| Possible Values: | +4 to +16 |
| Default: | (+ 13–Fs) |
| Reference: | - |
| Interface: | - |
| Supplementary Information: | The purposes of specifying Sending Loudness Rating for the sound capture are: - Set the microphone preamplifiers gain value in order to use the whole dynamic of the A to D converter without saturating it. - Set the microphone preamplifiers gain value of all the microphones in all the rooms in communication in such way that the same sound pressure levels at any conferee position create the same electrical value before the A to D conversion. - Set the nominal electrical sending level from the Telepresence system to be compatible with that from conventional telephones. |

Telepresence systems shall send audio to other telepresence systems with a SendLoudnessRating of (+ 13–Fs) dB, measured in accordance with [ITU-T P.300].

In order to take into account the difference between the reference test positioning and the actual microphone-talker operating distance (ds), for which the terminal is adjusted, the following correction factor Fs is defined:

(ds in meters)

End proposed change 1

Start proposed change 2

7.2.2.1	Receiving Loudness Rating

| Identity: | ReceiveLoudnessRating |
| --- | --- |
| Description: | This parameter indicates the electric to acoustic transfer function of the Telepresence audio stream. |
| Format: | Numeric, in dB |
| Possible Values: | -26 to +18 |
| Default: | (+ 5–Fr) |
| Reference: | - |
| Interface: | - |

End proposed change 2

Start proposed change 3

7.1.3.5	Weighted Microphone-Loudspeaker Coupling Loss

| Identity: | weightedMicrophoneLoudspeakerCouplingLoss. |
| --- | --- |
| Description: | This parameter indicates the minimum weighted coupling loss between each microphone and loudspeaker pair. In practice, the worst case (ie the one where the coupling loss is minimum) will be the case where the microphone and loudspeaker are physically closest together. |
| Format: | Numeric, in dB. |
| Possible Values: | ≥ 46 dB nominal volume, ≥ 40 dB maximum volume |
| Default: | None. |
| Reference: | - |
| Interface: | - |

End proposed change 3

# References

[1] Draft Rec. H.TPS-AV "Audio/video parameters for telepresence systems", TD 89R1 (WP 1/16), November 2013

[2] ITU-T Recommendation P.300 (11/2001), “Transmission performance of group audio terminals (GATs)”.

[3] ETSI ES 202 740 V1.3.2 (2010-09), “Transmission requirements for wideband VoIP loudspeaking and handsfree terminals from a QoS perspective as perceived by the user.”

__________________