| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4544 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Cisco Systems, Inc. |  |  |  |
| Title: | End-to-End Session Identifier for IP-based Multimedia Communication Systems |  |  |  |
| Purpose: | Proposal |  |  |  |

Abstract

This document describes an end-to-end Session Identifier for use in IP-based Multimedia Communication systems that enables endpoints, intermediate devices, and management systems to identify a session end-to-end, associate multiple endpoints with a given multipoint conference, and track communication sessions when they are redirected.

# Introduction

Identification of signalling sessions, including H.323 and SIP sessions, is important for a variety of reasons, including network management, signalling/media correlation, billing, and troubleshooting.  With the right construction, it should also be possible to use the Session Identifier (“Session ID”) to identify participants in a multipoint conference.

H.323 and SIP both introduce a number of different identifiers to help identify communication sessions, but unfortunately each has certain limitations that make them less than ideal for end-to-end session identification.

Note that this proposal was originally submitted as C.552 to the March 2011 meeting of ITU-T SG16.  Since a desire was expressed during that meeting to produce a solution that is interoperable with SIP-based systems, it was agreed that the work on the end-to-end session identifier would progress in the IETF and, as that work concluded, introduce the same capability in H.300-series systems in such a way as to be interoperable with SIP systems.  The work has not yet concluded, but the requirements document is nearly complete and the solution draft is currently being actively progressed in the INSIPID working group.

# Review of H.323 Identifiers

H.323 has a number of identifiers defined already that relate to the topic of this contribution.  They are the Conference Identifier, Call Identifier, Global Call Identifier, and Thread Identifier.

The Conference Identifier is a globally unique identifier that, in theory, is shared by all participants in a conference.  It is not designed to identify an end-to-end session or call.  Rather, it merely aims to logically group endpoints that are part of the same conference.

The Call Identifier is a globally unique identifier that is intended to identify a call end-to-end.  In practice, though, it fails to serve that function for a variety of reasons.  One reason is that interworking functions sometimes terminate calls and re-originate calls.  In so doing, they establish a unique Call Identifier for each leg of a call.  Certain intermediary devices also “join” call legs using Third Party Pause and Re-Routing procedures.  Each of the endpoints that might have been joined together has a different view of what the Call Identifier is.  (As an aside, the same issues exist with SIP’s Call-ID header.)

The Global Call Identifier was introduced because the Call Identifier suffered from the aforementioned issues.  This identifier does, in fact, provide for end-to-end identification of a call.  However, as a part of service interaction (e.g., call transfer or joining call legs), the Global Call Identifier may change multiple times during what the user perceives as a single call.  Further, there are race conditions that can occur.  Per 10.3.3.1.1/H.323, a Gatekeeper may perform a transfer on behalf of an endpoint and would assign a new Global Call Identifier to the call.  What happens if two Gatekeepers perform a transfer in parallel and each transmits a new Global Call Identifier assignment?  The two endpoints in the resulting call may incorrectly assume different values.

Finally, there is the Thread Identifier.  This identifier is intended to help track a call as it is transferred or otherwise modified.  Consider, for example, a call placed from party A to B.  Suppose that A then transfers the call using H.450.2 to C.  To avoid the issue where B may be billed for initiating a new SETUP, B would insert a Thread Identifier for the call assigned by A.  This identifier is not intended to identify a call end-to-end, but merely to help track a call as it is transferred.

So, while there are already a number of call-related identifiers defined in H.323, none provide for a means of identifying a call end-to-end in all cases.

# Session Identifier Usage

The Session Identifier is intended to uniquely identify a communication session end-to-end.  This document does not specify how the Session Identifier is to be used, but merely defines the identifier in such a way as to enable it to be used for situations encountered in real-world deployments of IP-based multimedia communication systems, including:

- End-to-end identification of a communication session

- Association of session signalling and media flows, made possible by including the session identifier in media-related messages (e.g., RSVP or RTCP)

- Identification of devices taking part in the same multipoint conference

- Tracking sessions transferred from one endpoint to another

- Identification of recorded sessions

- Logging for the purposes of accounting, billing, debugging, etc.

Note that there we are not proposing nor recommending to specify a particular use of the Session ID.  Rather, we merely propose to define it and the associated signalling procedures.

# Constructing the Session Identifier

The Session Identifier comprises of two UUIDs that are assigned by the two endpoints engaged in an end-to-end communication session.  The first endpoint in the session creates a UUID and transmits that to the second endpoint.  Likewise, the second endpoint creates a UUID and transmits that to the first endpoint, along with the UUID received from the first endpoint.  The set comprising these two UUIDs is referred to as the Session ID.

Note that it does not matter which endpoint might be considered the originating or terminating endpoint when one looks as the values of the Session ID.  For the purposes of session identification, it is only important that each endpoint create a UUID and transmit that value to the remote endpoint.  Intermediaries such as session border controllers do not change any Session Identifier component received from an endpoint in a session, though an intermediary may create a UUID on behalf of an endpoint that does not support the Session ID signalling procedures.

Since the Session ID is defined as a set of UUIDs, the order in which the UUIDs appear is not important. Consider the following example of what constitutes a Session ID.

aeffa652b22911dfa81f12313a006823

be11afc8b22911df86c412313a006823

Endpoint A’s UUID

Endpoint B’s UUID

{aeffa652b22911dfa81f12313a006823, be11afc8b22911df86c412313a006823}

Resulting Session Identifier

Figure 1 - Session ID Components

In the above example, the UUIDs are presented for readability as a string of hexadecimal characters that correspond to the binary values comprising the UUID (refer to Table 21/H.225.0).  Within H.323, we propose to use the GloballyUniqueID type to contain each UUID value, perhaps using a SEQUENCE such that the first value in the SEQUENCE refers to the endpoint that transmitted created the UUID.

# Transmitting the Session Identifier in H.323

Each session initiated must have a unique, locally generated UUID associated with the session.  This value shall remain unchanged throughout the duration of the session and shall persist even when the session is transferred or diverted.  As examples, if an endpoint initiates a call that is diverted using H.450.2, H.450.3, or a “Facility Call Forwarded” (prior to CONNECT or after CONNECT), the endpoint shall use the same UUID it created for the diverted call.  However, if the user hangs up a call and initiates a new call, even to the same remote destination, a new and unique UUID would be created for that new call.

There are several possible places to insert the UUID.  We would like to discuss three options.

## Use of the Call Linkage field

Since the CallLinkage field is already defined in H.323, a logical place to put the Session Identifier components is in that data structure.  It might be introduced as per the following syntax:

CallLinkage ::= SEQUENCE

{

globalCallId	GloballyUniqueID OPTIONAL,

threadId	GloballyUniqueID OPTIONAL,

...,

sessionID	SEQUENCE (SIZE(1..2)) OF GloballyUniqueID OPTIONAL

}

The issue with the above syntax is that it requires that all endpoints and intermediary devices be upgraded to the most recent version of H.225.0.  Otherwise, the new field might be silently discarded.

## Conveying the identifier in structures alongside CallIdentifier

A very logical place to encode and transport the Session ID is alongside the CallIdentifier.  As an example, the identifier might be encoded as shown below:

SetupAcknowledge-UUIE ::= SEQUENCE

{

protocolIdentifier  ProtocolIdentifier,

callIdentifier      CallIdentifier,

tokens              SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens        SEQUENCE OF CryptoH323Token OPTIONAL,

...,

sessionID           SEQUENCE (SIZE(1..2)) OF GloballyUniqueID OPTIONAL

}

Again, the issue with this approach is that this will require endpoints to upgrade H.225.0 in order to provide support for the Session ID.

## Conveying the identifier using generic data

To ensure that the Session ID can be conveyed by all modern H.323 devices, a better approach might be to use the genericData field found in H323-UU-PDU.  This is important, since intermediary devices could insert this value even for older devices as calls are carried across an administrative domain.  This can be formally defined as an H.460.x extension that supports H.323v4 or newer devices.

The new capability identifier would be:

| Feature name: | Session Identifier |
| --- | --- |
| Feature Description: | This feature allows two communicating endpoints to exchange a globally unique end-to-end session identifier |
| Feature identifier type: | Standard |
| Feature identifier value: | tbd |

Table 1 – Session Identifier Feature

This identifier would be used as the id in the GenericData.id field.

There would be one or more generic parameters to transport the UUID values:

| Parameter name: | Session-ID-UUID |
| --- | --- |
| Parameter description: | This parameter contains a Session-ID-UUID value |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 1 |
| Parameter type: | id.nonStandard or raw (TBD) |
| Parameter cardinality: | Once or twice in H.225.0 call signalling or RAS messages |

Table 2 – Session Identifier UUID Parameter

Endpoints shall encode their own UUID first and the UUID received by the remote endpoint after its own UUID.

# Signalling Procedures

An H.323 endpoint shall convey its Session ID UUID in all transmitted messages.  For the initial ARQ and SETUP message, only a single UUID would be present since the calling device would not have the UUID of the remote endpoint.  However, the called endpoint shall transmit both its own UUID value and the value received from the calling endpoint.  The DRQ or other RAS messages transmitted after the endpoint has received the UUID of the remote endpoint shall contain both UUID values.

Intermediaries that wish to utilize the Session Identifier would take note of the UUIDs transmitted in each direction between endpoints.  Intermediaries would not alter the UUIDs, though an intermediary may insert a UUID value in the event that the calling or called endpoint does not support the Session ID signalling procedures.  If performing interworking between H.323 and another protocol, the intermediary would convert the Session ID fields into a format suitable for the other session protocol.  Note: the IETF has a preliminary mechanism defined for how an intermediary shall construct a UUID in the case that it creates a UUID.  We propose to adopt the same mechanism or modify it as necessary to ensure interoperability.

A session may be transferred at any point and without any explicit signalling.  This is not uncommon for SBCs or PBXes that provide various call control functions.  When the session is transferred, joined, or merged, an H.225.0 call signalling message might be received bearing a new Session ID UUID value from the remote endpoint.  When a new UUID is received, the receiving endpoint shall immediately utilize this new UUID and construct a new Session Identifier value for transmission in subsequent messages.  The receiving endpoint would not generate a new UUID of its own in response, however.  Once an endpoint generates a Session ID UUID, it remains constant from the first SETUP or response message to a SETUP until the session is fully terminated, living through call transfers, diversions, etc.

# Identifying Endpoints in a Conference

While the use of the Session ID between two communicating endpoints is fairly straight-forward and require little explanation, using the identifier to identify participants in a conference may be less obvious and, therefore, expanded upon with this explanation.

Identifying users in a multipoint conference is beneficial to network elements that process media flows related to that multipoint conference.  While H.323 has a Conference Identifier that can be used to relate participants in a conference, the practice of many MCUs is to associate participants in a conference based on an alias address (e.g., a phone number), rather than the Conference Identifier.  Use of the Session ID as proposed in this document could be used by network elements to identify conference participants.

The way this might work is that an MCU might assign the same UUID to each participant in the conference.  The Session IDs for all participants would still be unique, as the Session ID is the concatenation of UUIDs assigned be both A and B in a call.  However, all participants in the same multipoint conference would share one UUID value.

Though this document does not propose how session signalling and media flows would be correlated, one could include the Session ID in media-related protocols, including RTP, RTCP, and RSVP.  We do not propose a method here and leave it for further study.

# Conclusion

Previously, it was agreed to initiate a new work item for this topic.  During this meeting, we would like to get agreement on the way forward in specifying the Session ID syntax and procedures.

______________________