| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4582 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.SCTP: clause 5 "conventions" - input proposal |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes input material for clause "conventions", similar as for bearer types "TCP" and "TLS" (see H.248.TCP, H.248.TLS).

Discussion

We'd like to highlight following aspects:

- SCTP terminology: (RFC 4960) provides a fairly precise and detailed terminology (in contrast to TCP and TLS), which facilitates the conventions and concepts (e.g., our baseline related to "endpoint", "packet", etc).
 proposal to add the base SCTP terms in clause 3.1

- SCTP multiplexing: the SCTP concept of "SCTP stream to SCTP association" mapping …
 proposal to add an Editor's note ("relates inter alia to "SCTP bearer" concept, clause 5.3)

- SCTP mode of operation: related to "TCP mode", we could expect (at least) a "SCTP relay mode" and a "SCTP proxy" mode … however, this topic couldn't yet analyzed due to time reasons
 proposal to add an Editor's note (clause 5.4)

- SCTP (H.248) signalling examples: not yet submitted, but planned, but conventions already inserted in this contribution …
 proposal to add an Editor's note (clause 5)

The update of conventions (clause 5) impacts as well clauses 1, 2 and 3.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against the output draft from last meeting (see TD 113 (WP 1/16)).

Change proposal #1:

[Begin Proposed Correction]

# 1	Scope

FIXTHIS, at least following items should be addressed by this Recommendation:

- Use case(s)

- Interworking models (MG)

- Control of SCTP endpoints in MG

- Modelling of H.248 terminations / stream endpoints

- H.248 signalling elements

- Relation to existing H.248.x Recommendations

## 1.1	Applicability statements

Table 1 summarizes all possible SCTP-based interfaces of H.248 entities, - under the assumption of an underlying IP network -, and their relevance for this Recommendation.

Table 1 – Principal SCTP-based interfaces of H.248 entities 
and their relevance for this Recommendation

| SCTP-based transport | H.248 entity | Relevance |
| --- | --- | --- |
| Call control interface (e.g., SIP-over-SCTP) | MGC | Out of scope of this Recommendation. |
| Gateway control interface (H.248) | MGC, MG | Out of scope of this Recommendation. Possible H.248 transport modes are indicated by [ITU-T H.248.67]. Usage of a SCTP-based H.248 transport mode would be typically specified by an H.248 profile (as part of clause 6.12 in the profile definition template (see Appendix III in [ITU-T H.248.1])). |
| Bearer interface | MG | Within the scope of this Recommendation. |

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway control protocol: Version 3.

[ITU-T H.248.67]	Recommendation ITU-T H.248.67 (12/2009), Gateway control protocol: Transport mode indication package.

[IETF RFC 4960]	IETF RFC 4960 (2007), Stream Control Transmission Protocol.

[IETF RFC 5061]	IETF RFC 5061 (2007), Stream Control Transmission Protocol (SCTP) Dynamic Address Reconfiguration.

[IETF RFC 6525]	IETF RFC 6525 (2012), Stream Control Transmission Protocol (SCTP) Stream Reconfiguration.

{Editor's note (2013-10): following references might be needed

- draft-ietf-mmusic-sctp-sdp

- draft-ietf-tsvwg-sctp-dtls-encaps

- draft-marcon-msrp-over-webrtc-data-channels

- draft-ietf-rtcweb-data-protocol

- H.248.DTLS

- others

}

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

# 3	Definitions

## 3.1	Terms defined elsewhere

<Normally terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

3.1.1	SCTP association [IETF RFC 4960]:  "A protocol relationship between SCTP endpoints, composed of the two SCTP endpoints and protocol state information including Verification Tags and the currently active set of Transmission Sequence Numbers (TSNs), etc.  An association can be uniquely identified by the transport addresses used by the endpoints in the association. Two SCTP endpoints MUST NOT have more than one SCTP association between them at any given time."

3.1.2	SCTP chunk [IETF RFC 4960]: "A unit of information within an SCTP packet, consisting of a chunk header and chunk-specific content."

3.1.3	SCTP endpoint [IETF RFC 4960]:  "The logical sender/receiver of SCTP packets. On a multi-homed host, an SCTP endpoint is represented to its peers as a combination of a set of eligible destination transport addresses to which SCTP packets can be sent and a set of eligible source transport addresses from which SCTP packets can be received.  All transport addresses used by an SCTP endpoint must use the same port number, but can use multiple IP addresses. A transport address used by an SCTP endpoint must not be used by another SCTP endpoint. In other words, a transport address is unique to an SCTP endpoint."

3.1.4	SCTP packet [IETF RFC 4960]: "The unit of data delivery across the interface between SCTP and the connectionless packet network (e.g., IP).  An SCTP packet includes the common SCTP header, possible SCTP control chunks, and user data encapsulated within SCTP DATA chunks."

3.1.5	SCTP path [IETF RFC 4960]: "The route taken by the SCTP packets sent by one SCTP endpoint to a specific destination transport address of its peer SCTP endpoint.  Sending to different destination transport addresses does not necessarily guarantee getting separate paths."

3.1.6	SCTP stream [IETF RFC 4960]: "A unidirectional logical channel established from one to another associated SCTP endpoint, within which all user messages are delivered in sequence except for those submitted to the unordered delivery service.	
Note: The relationship between stream numbers in opposite directions is strictly a matter of how the applications use them. It is the responsibility of the SCTP user to create and manage these correlations if they are so desired."

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>

[End Proposed Correction]

Change proposal #4:

[Begin Proposed Correction]

# 5	Conventions

{Editor's note (2014-03): clauses 5.1 & 5.2  provide up-front information because it is planned to submit example signalling flows in a future meeting (which would use that conventions). }

## 5.1	Naming of stream endpoints and terminations

The notations "Tx" and "Tx(Sy)" are used for naming of terminations and stream endpoints (SEP) respectively, with

- "Tx" as TerminationID value;

- "Sy" as StreamID value and

- numerical variables x and y.

## 5.2	Conventions used in signalling flows

The following conventions are used in the example signalling flows:

| SCTP Est.req SCTP Est.ack | Abstracted (SCTP message/procedure independent) representationfor establishment requests/acknowledgements of new SCTP associations. |
| --- | --- |
| SCTP Rel.req SCTP Rel.ack | Abstracted (protocol-independent) representation for release requests/acknowledgements of existing SCTP associations. |

## 5.3	SCTP bearer

The notation of SCTP bearer connection relates to a SCTP association in the network user plane. There might be also a control plane SCTP association (between MGC and MG) in case of a SCTP-based H.248 transport mode [ITU-T H.248.67].

{Editor's note (2014-03): it is yet open whether we need to consider
a) "unmultiplexed SCTP bearers" only (i.e., a 1:1 relation of SCTP Stream to SCTP association from H.248 perspective), or/and
b) "multiplexed SCTP bearers" (i.e., there might be following model: an H.248 "multi-stream-to-termination structure" with "H.248 termination = SCTP association" and "H.248 SEP = SCTP Stream" and multiple SCTP Streams).
Such aspects may impact above convention on "SCTP bearer". }

## 5.3	SCTP bearer control

There are connection control information flows for SCTP bearer connections in the network user plane (e.g., IP media/data/bearer plane) and the control plane (here H.248). Figure  1 outlines the used conventions for the differentiation of both control flows:

Figure 1 – Conventions for SCTP bearer control

## 5.4	SCTP endpoint notations

The notion of endpoint represents different concepts in each network plane (H.248 as control protocol belongs to the network control plane and the MG bearer interface to the network user plane), see Figure 2:

Figure 2 – Conventions for SCTP endpoint types

{Editor's note (2014-03): the concept of "SCTP mode" in above Figure is still not analyzed in detail … }

Usage in

- control plane (here H.248): H.248 terminations/stream endpoint with SCTP processing are denoted as SCTP-enabled termination or stream endpoint (SEP) respectively;

- user plane (SCTP): a SCTP bearer connection endpoint is defined by the SCTP itself, see clause 3.1.3 "SCTP endpoint".

Whether the (H.248) SCTP-enabled SEP provides a "SCTP endpoint" is dependent on the H.248 connection model and enabled MG-internal interworking functions (see clauses 6 and 7).

[End Proposed Correction]

____________________