| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-12 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, CH, 10 - 14 March, 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.78 |  |  |  |
| Title: | H.248.78 "Gateway control protocol: Bearer-level application level gateway” (Rev.): Output draft Ed. 0.5 |  |  |  |
| Purpose: | Meeting output (Draft Recommendation) |  |  |  |

Summary

This document contains the editor's output draft for draft revised H.248.78 "Gateway control protocol: Bearer-level application level gateway” (Ed. 0.5).

__________________

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.78 Rev. Ed. 0.0 | Proposal (AVD-4382) to WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/AVD-4382.zip |
| H.248.78 Rev. Ed. 0.1 | Output (TD-12) from WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/TD-12.zip |
| H.248.78 Rev. Ed. 0.2 | Input (TD 78/WP1) to ITU-T SG16 Geneva meeting, Oct/Nov 2013 Location: http://itu.int/md/T13-SG16-131028-TD-WP1-0078 |
| H.248.78 Rev. Ed. 0.3 | Output (TD 78R1/WP1) from ITU-T SG16 Geneva meeting, Oct/Nov 2013 Location: |
| H.248.78 Rev. Ed. 0.4 | Input (AVD-4472) to WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/AVD-4472.zip |
| H.248.78 Rev. Ed. 0.5 | Output (TD-12) from WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/TD-12.zip |

|  |  | International Telecommunication Union |  |  |
| --- | --- | --- | --- | --- |
| ITU-T | H.248.78 |  |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (03/2013) |
|  | SERIES H: AUDIOVISUAL AND MULTIMEDIA SYSTEMS Infrastructure of audiovisual services – Communication procedures |  |  |  |
|  | Gateway control protocol: Bearer-level application level gateway |  |  |  |
|  | Recommendation ITU-T H.248.78 |  |  |  |

ITU-T H-SERIES RECOMMENDATIONS

AUDIOVISUAL AND MULTIMEDIA SYSTEMS

| CHARACTERISTICS OF VISUAL TELEPHONE SYSTEMS | H.100–H.199 |
| --- | --- |
| INFRASTRUCTURE OF AUDIOVISUAL SERVICES |  |
| General | H.200–H.219 |
| Transmission multiplexing and synchronization | H.220–H.229 |
| Systems aspects | H.230–H.239 |
| Communication procedures | H.240–H.259 |
| Coding of moving video | H.260–H.279 |
| Related systems aspects | H.280–H.299 |
| Systems and terminal equipment for audiovisual services | H.300–H.349 |
| Directory services architecture for audiovisual and multimedia services | H.350–H.359 |
| Quality of service architecture for audiovisual and multimedia services | H.360–H.369 |
| Supplementary services for multimedia | H.450–H.499 |
| MOBILITY AND COLLABORATION PROCEDURES |  |
| Overview of Mobility and Collaboration, definitions, protocols and procedures | H.500–H.509 |
| Mobility for H-Series multimedia systems and services | H.510–H.519 |
| Mobile multimedia collaboration applications and services | H.520–H.529 |
| Security for mobile multimedia systems and services | H.530–H.539 |
| Security for mobile multimedia collaboration applications and services | H.540–H.549 |
| Mobility interworking procedures | H.550–H.559 |
| Mobile multimedia collaboration inter-working procedures | H.560–H.569 |
| BROADBAND, TRIPLE-PLAY AND ADVANCED MULTIMEDIA SERVICES |  |
| Broadband multimedia services over VDSL | H.610–H.619 |
| Advanced multimedia services and applications | H.620–H.629 |
| Ubiquitous sensor network applications and Internet of Things | H.640–H.649 |
| IPTV MULTIMEDIA SERVICES AND APPLICATIONS FOR IPTV |  |
| General aspects | H.700–H.719 |
| IPTV terminal devices | H.720–H.729 |
| IPTV middleware | H.730–H.739 |
| IPTV application event handling | H.740–H.749 |
| IPTV metadata | H.750–H.759 |
| IPTV multimedia application frameworks | H.760–H.769 |
| IPTV service discovery up to consumption | H.770–H.779 |
| Digital Signage | H.780–H.789 |

For further details, please refer to the list of ITU-T Recommendations.

Draft revised Recommendation ITU-T H.248.78 Gateway control protocol: Bearer-level application level gateway

AAP Summary

[To be added]

Summary Recommendation ITU-T H.248.78 defines a package that allows bearer-level application protocol (such as real time streaming protocol (RTSP) or message session relay protocol (MSRP)) messages received by a media gateway (MG) to be communicated to a media gateway controller (MGC). This allows the MGC to perform application-level gateway functionality such as network address translation on the messages and then forward the messages to the MG for sending to the destination. The (03/2013) revision incorporates an update to correctly refer to the "Message Session Relay Protocol".

This revision of Recommendation ITU-T H.248.78 adds a further H.248 package for support of MG located application-level gateway functionality-

History Edition; Recommendation; Approval; Study Group. 1.0; ITU-T H.248.78; 2010-09-13; 16. 2.0; ITU-T H.248.78; 2013-03-16; 16.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications, information and communication technologies (ICTs). The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure, e.g., interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met. The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementers are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database at http://www.itu.int/ITU-T/ipr/.

ITU 2013

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

Table of Contents

Page

1	Scope	1

2	References	1

3	Definitions	1

3.1	Terms defined elsewhere	1

3.2	Terms defined in this Recommendation	2

4	Abbreviations and acronyms	2

5	Conventions	2

6	Application-level gateway functionality	3

6.1	MGC strictly controlled MG embedded bearer-level ALG	4

6.2	MG autonomous embedded bearer-level ALG	6

7	MGC Controlled Bearer Level ALG package	6

7.1	Properties	6

7.2	Events	6

7.2.1	Detect bearer level message	6

7.3	Signals	9

7.3.1	Send bearer level message	9

7.4	Statistics	9

7.5	Error codes	9

7.6	Procedures	9

7.6.1	Application control protocol messages detection initiation	9

7.6.2	Detection of application control protocol messages	9

7.6.3	Modification and sending of application control protocol messages	10

7.6.4	Examples	10

8	MG located Bearer Level ALG	12

8.1	Properties	12

8.1.1	Protocol type bearer level ALG	12

8.1.2	Upper layer protocol filter	12

8.1.3	Indication of "source of replaced source address information"	13

8.2	Events	14

8.3	Signals	14

8.4	Statistics	14

8.5	Error codes	14

8.6	Procedures	14

8.6.1	General characteristics of the MG located bearer level ALG service	14

8.6.2	Enabling the MG located bearer level ALG service	15

8.6.3	Indication of affected application protocol(s)	15

8.6.4	Indication of relevant address information and source of replaced address information	15

8.6.4	L4+ protocol individual B-ALG service configuration	15

8.6.6	B-ALG: L4+ protocol specific translation details	15

8.6.7	Examples	16

Appendix I  MG located bearer level ALG service –  Illustration and signalling examples	16

I.1	Illustration of concept	16

I.2	Illustration of example use cases	18

I.2.1	Use case "Unidirectional B-ALG with destination address information translation"	18

I.2.2	Use case "Bidirectional B-ALG with source & destination address information translation"	19

Bibliography	21

To TSB: to be inserted – Table of Tables & Table of Figures

Draft revised Recommendation ITU-T H.248.78

Gateway control protocol: Bearer-level application level gateway

# 1	Scope

In a split media gateway controller (MGC) and media gateway (MG) environment, call-level application level gateways (ALG) are common in order to translate address information (i.e., network address and port translation (NAPT)) between two signalling domains. Typically, the application signalling traverses the MGC. ITU-T H.248 is therefore not involved in these translations other than to set the necessary media transport parameters on applicable terminations.

Increasingly bearer-level application control signalling is being used which instead traverses the MG. This bearer-level application signalling typically is used within a session that has been established by the MGC; thus, some coordination between the two levels is needed.

This Recommendation provides functionality that allows the MGC and MG to coordinate bearer-level application level gateway functionality.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway control protocol: Version 3.

[ITU-T H.248.69]	Recommendation ITU-T H.248.69 (2009), Gateway control protocol: Packages for interworking between MSRP and H.248.

[IETF RFC 2326]	IETF RFC 2326 (1998), Real Time Streaming Protocol (RTSP).

[ETSI TS 123 228]	ETSI TS 123 228 (2010), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; IP Multimedia Subsystem (IMS); Stage 2 (3GPP TS 23.228 version 9.4.0 Release 9).

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following term defined elsewhere:

3.1.1	application level gateway (ALG) [ETSI TS 123 228]: Is an application specific functional entity that allows communication between disparate address realm or IP versions, e.g., an IPv6 node to communicate with an IPv4 node and vice versa, when certain applications carry network addresses in the payloads like SIP/SDP. NA(P)T-PT or NA(P)T is application unaware whereas ALGs are application specific translation entities that allow a host running an application to communicate transparently with another host running the same application but in a different IP version or IP address realm.

NOTE – This definition originates from the first ALG description in clause 2.9 of [b-IETF RFC 2663].

## 3.2	Terms defined in this Recommendation

None.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

ALG	Application-Level Gateway

B2B	Back to Back

B-ALG	Bearer Level Application-Level Gateway

DNS	Domain Name System

HTML	Hypertext Mark-up Language

HTTP	Hypertext Transfer Protocol

IP	Internet Protocol

IPv4	Internet Protocol Version 4

IPv6	Internet Protocol Version 6

L3	Layer three

L4	Layer four

L4+	Above layer four

MG	Media Gateway

MGC	Media Gateway Controller

MSRP	Message Session Relay Protocol

NAPT	Network Address and Port Translation

OSI	Open Systems Interconnection

RTSP	Real-Time Streaming Protocol

SCTP	Stream Control Transport Protocol

SDP	Session Description Protocol

SIP	Session Initiation Protocol

TCP	Transmission Control Protocol

UDP	User Datagram Protocol

# 5	Conventions

Elements of the ITU-T H.248 protocol model, e.g., Context, Termination, Stream, Event are represented using the first letter capitalized. Property, Event, Signal and Parameter identities are given in italics.

The suffix ".req" added to an ITU-T H.248 command name stands for a command request, while the suffix ".rep" stands for a command reply. For example "Notify.req" represents a Notify Request.

# 6	Application-level gateway functionality

Address information carried at layer four (L4) or above (L4+) according to the ISO 7-layer model is primarily related to call/session control signalling (like the session initiation protocol (SIP)). Such call-level ALGs are consequently out of scope of ITU-T H.248 Recommendations. However, there are IP applications with bearer-level transport of "L4+ address information". These services may need a bearer-level ALG located in the ITU-T H.248 MG.

Examples use cases (see also Figure 1) which may require a bearer-level ALG are:

–	Message session relay protocol (MSRP)-over-TCP/IP (instant messaging in session mode; ITU-T H.248 MG as in-path node between MSRP client and MSRP server, or between MSRP relay; not necessarily using the procedures of [ITU-T H.248.69]);

–	Real time streaming protocol (RTSP)-over-{TCP|UDP}/IP (streaming service; ITU-T H.248 MG as in-path node, not necessarily where the MGC is involved in RTSP signalling);

–	Hypertext transfer protocol (HTTP)-over-TCP/IP (web service; ITU-T H.248 MG as in-path node between HTTP client/server);

–	SIP-over-{SCTP|TCP|UDP}/IP (e.g., in case of bearer-path coupled SIP signalling and SIP traffic forwarded by ITU-T H.248 Context in ipr mode).

Figure 1 – Examples for potential ALG

There are fundamentally two options for controlling ITU-T H.248 MG embedded bearer-level ALG functions:

1)	MGC controlled mode:

The MGC explicitly provides the mapping information for the "L4+ NAPT" function. Any potential "L3/L4 NAPT" function would not be tightly coupled with the "L4+ NAPT" function from MG perspective; or

2)	MG autonomous mode:

The MG would monitor the IP bearer packet flow and try to detect messages which need "L4+ NAPT" handling, and the mapping information would be derived from local "L3/L4 NAPT" information or other (e.g., DNS) information.

## 6.1	MGC strictly controlled MG embedded bearer-level ALG

This clause describes a network scenario where a MGC controls a MG that handles both a bearer-level application protocol (e.g., RTSP) and its associated media streams. See Figure 2 for an illustration.

Figure 2 – RTSP and media traversing a MG

When analysing the signalling flow for the network scenario, it is assumed that an appropriate call/session control protocol (e.g., SIP) is used to establish a termination/stream for the bearer-level application protocol (e.g., RTSP). The use of the application-level protocol may result in additional media bearer(s) needing to be established. The basic assumption is that the MGC is still responsible for establishing these media flows. Therefore, some coordination is needed between the bearer/MG level and the MGC. This is illustrated by the signalling flow in Figure 3.

Figure 3 – RTSP and media traversal signalling flow

For the purposes of the steps below, it is assumed that terminations T1 and T2 have already been established and are set to enable reception and sending bearer-level application protocol signalling.

In step 1: The MG receives a RTSP request message including media information via Termination T1.

For example:

SETUP RTSP://example.com/foo/bar/baz.rm RTSP/2.0

CSeq: 302

Transport: RTP/AVP;unicast;dest_addr=":4588"/":4589",

RTP/AVP/TCP;unicast;interleaved=0-1

In step 2: In order to inform the MGC of this new media request, the MG sends a Notify.req message to MGC with the information from the received RTSP message.

In step 3: The MGC issues a Notify.rep message.

In step 4: Based on the information from the RTSP SETUP message, the MGC would request two additional new Terminations (i.e., T3 and T4, not shown in the signalling flow) for the media flow (back to back mode. See [b-ITU-T H.248.64] for more information regarding this mode). The MG will provide a NAPT function for media flow.

In step 5: The MGC sends a Modify.req message to the MG. Updated media information is included in this modify message.

In step 6: MG sends the updated RTSP request to the next node via Termination T2.

SETUP RTSP://example.com/foo/bar/baz.rm RTSP/2.0

CSeq: 302

Transport: RTP/AVP;unicast;dest_addr=":8000"/":8001",

RTP/AVP/TCP;unicast;interleaved=0-1

Step 7: The MG sends a Modify.rep message.

Clause 7 defines the "MGC Controlled Bearer Level ALG" package, which provides the necessary elements to support such signalling flows.

## 6.2	MG autonomous embedded bearer-level ALG

Where a bearer-level application protocol traverses a MG, one theoretical mode of operation is to use an MG embedded bearer-level ALG. In this configuration the ALG would inspect bearer-level application protocol packets and then perform the necessary network address translations. As the embedded function would be independent of the MGC, the mappings would need to be determined via a non-ITU-T H.248 method, such as DNS, provisioning, etc.

However, as shown in the network scenario as presented by Figure 2, bearer-level application protocols such as RTSP may also establish, manipulate or remove media streams. If the bearer-level ALG is embedded in the MG and is independent of the MGC, the MG would be required to autonomously manipulate the characteristics of the media stream. The ITU-T H.248 connection model does not allow such behaviour.

The implementation of such an embedded MG would mean that whilst the network address information may be correctly mapped as the bearer-level application protocol traverses the MG, any subsequent attempts to establish a bearer by the application protocol would fail (as the necessary streams would not be open on the MG).

Therefore, this Recommendation does not provide generic procedures for the support of an autonomous embedded bearer-level ALG.

# 7	MGC Controlled Bearer Level ALG package

|  | Package name: | MGC Controlled Bearer Level ALG |
| --- | --- | --- |
|  | Package ID: | mcbalg (0x0108) |
|  | Description: | This package allows a MGC to receive bearer-level application messages in order to perform ALG functionality (such as NAPT) on them. It defines an event allowing the MG to detect bearer-level messages and send them to the MGC and a signal containing an updated bearer-level message which allows the MGC to request the MG to send bearer level messages. The package assumes that the MGC has a decode/encode logic for the application protocol in question that allows it to receive the native messages, perform ALG functionality and recode them with the updated information. |
|  | Version: | 1 |
|  | Extends: | None |

## 7.1	Properties

None.

## 7.2	Events

### 7.2.1	Detect bearer level message

|  | Event name: | Detect bearer level message |
| --- | --- | --- |
|  | Event ID: | det (0x0001) |
|  | Description: | This event is used by the MGC to indicate to the MG which bearer-level messages should be detected. Once detected by the MG, these messages are sent to the MGC. |

#### 7.2.1.1	EventsDescriptor parameters

##### 7.2.1.1.1	Protocol Filter

|  | Parameter name: | Protocol Filter |
| --- | --- | --- |
|  | Parameter ID: | pf (0x0001) |
|  | Description: | This parameter indicates the application protocol that shall be detected by the MG and reported to the MGC. |
|  | Type: | Unsigned Integer |
|  | Optional: | Yes |
|  | Possible values: | A value representing an application protocol, using the well-known or registered port number assigned to that protocol by IANA (http://www.iana.org/assignments/port-numbers). For example: 80 World Wide Web HTTP 554 Real Time Streaming Protocol (RTSP) 2855 MSRP |
|  | Default: | 0, indicating that the protocol is determined via the local and remote descriptors. |

##### 7.2.1.1.2	Message Filter

|  | Parameter name: | Message Filter |
| --- | --- | --- |
|  | Parameter ID: | mf (0x0002) |
|  | Description: | This parameter indicates the messages that shall be detected by the MG and reported to the MGC. |
|  | Type: | Sub-list of String |
|  | Optional: | Yes |
|  | Possible values: | The possible values are determined according to the "message name" field (or an analogous field such as "method") associated with the application control protocol being detected. For example, in the case of RTSP this would be the "Method" as described in clause 6.1 of [IETF RFC 2326]. Where the application control protocol is a binary protocol, the value will be encoded as hexadecimal octet according to clause B.3 of [ITU-T H.248.1]. Where the application control protocol is a text protocol, case sensitivity with regard to the "message name" is as per the application protocol. An additional value "*" (wildcard ALL) is defined to allow the MGC to indicate that all messages with valid message names for the particular application protocol are detected and reported. |
|  | Default: | "*" |

##### 7.2.1.1.3	Forwarding Flag

|  | Parameter name: | Forwarding Flag |
| --- | --- | --- |
|  | Parameter ID: | ff (0x0003) |
|  | Description: | This parameter indicates whether the MG forwards the detected message to other Terminations in the Context. This allows the MGC to be either fully in control of the application control protocol messages or to only be aware of the information contained in the application protocol messages. |
|  | Type: | Boolean |
|  | Optional: | Yes |
|  | Possible values: | True – On detection, the message is forwarded to the other applicable Terminations in the Context as well as being notified in an ObservedEvent. False – On detection, the message is NOT sent inwards to the Context, it is only notified in an ObservedEvent. |
|  | Default: | False |

#### 7.2.1.2	ObservedEventsDescriptor parameters

##### 7.2.1.2.1	Message Content

|  | Parameter name: | Message Content |
| --- | --- | --- |
|  | Parameter ID: | mc (0x0001) |
|  | Description: | This parameter contains the content of the detected application control protocol message. It contains only the L4+ information. |
|  | Type: | Binary encoding "Octet String", Text encoding "String" |
|  | Optional: | No |
|  | Possible values: | Binary encoding: Structure of Message Content 8; 7; 6; 5; 4; 3; 2; 1. MSB LSB; Octet 1. ...; .... MSB LSB; Octet n. NOTE – Octet 1 contains the most significant octet of data. Text encoding: If the application protocol is a text format protocol (i.e., the protocol messages always consist of octets which have a standard textual representation under UTF-8), the message shall be used "as is" as the parameter value. Message characters that are not valid for inclusion in an ITU-T H.248 quotedString (see clause B.2 of [ITU-T H.248.1]) shall be encoded using the triplet "%xx" where "xx" are the hexadecimal digits representing the character's octet. The "%" character shall also be encoded using such a triplet (i.e., as "%25"). If the application protocol is a binary format protocol, the binary data shall be encoded according to the procedures of clause B.4 of [ITU-T H.248.1]. |
|  | Default: | None |

## 7.3	Signals

### 7.3.1	Send bearer level message

|  | Signal name: | Send bearer level message |
| --- | --- | --- |
|  | Signal ID: | sblm (0x0001) |
|  | Description: | This signal allows the MGC to send an application protocol message to a MG, to be sent from one of the MG's Terminations/Streams. |
|  | Signal type: | Brief |
|  | Duration: | Not Applicable |

#### 7.3.1.1	Additional parameters

##### 7.3.1.1.1	Message content

As per clause 7.2.1.2.1.

## 7.4	Statistics

None.

## 7.5	Error codes

None.

## 7.6	Procedures

### 7.6.1	Application control protocol messages detection initiation

In order to request the MG to provide the MGC with certain application control protocol messages, the MGC shall set the "Detect bearer level message" (mcbalg/det) event on the termination receiving bearer-level application protocol messages. If multiple application control protocols are to be detected there shall be one instance of the mcbalg/det event for each.

The MGC may indicate the applicable application protocol to be detected via the use of the "Protocol Filter" (pf) parameter. If not provided, the MG shall determine the protocol via information in the Local and Remote Descriptors on the applicable Stream. If the Event is set on a Termination with multiple streams without an indication of the StreamID or "Protocol Filter", of if the MG is unable to determine the protocol, it shall return error code "472 Required Information Missing".

The MGC shall indicate which messages shall be detected through the use of the "Message Filter" (mf) parameter. This parameter allows a list of messages to be provided. Instead of a list of values, the MGC may also use the value "ALL", which represents all the messages that can be represented within a "message name" field.

The MGC may also indicate through the use of the "Forwarding Flag" (ff) parameter what action the MG should take upon detection of an applicable message. In addition to notifying the MGC, the MG may either forward the message into the Context or not, depending on the setting of the parameter.

### 7.6.2	Detection of application control protocol messages

Once the mcbalg/det event is set, the MG shall detect messages according to the parameters of that event. If an applicable message is detected, the MG shall send an ObservedEvent containing the "Message Content" (mc) parameter. By default, the message is not sent to other Terminations in the Context, unless the forwarding flag parameter was set to "True".

### 7.6.3	Modification and sending of application control protocol messages

On reception of the ObservedEvent containing the L4+ message content the MGC may analyse and update the contents of the message as appropriate. For example, it may modify address information to perform a NAPT function.

NOTE – It is assumed that the MG has already performed all necessary layer 1-3 functions and firewall functions on received messages in order to detect the L4+ message.

The MGC may then send the updated message content through the Termination(s) that would have normally sent the outgoing application protocol message (i.e., in the case of a two Termination B2B Context, the Termination that did not generate the mcbalg/det event) using the "Send Bearer Level Message" (mcbalg/sblm) signal. To do so, the MG shall place the updated protocol message in the signal's "Message Content" (mc) parameter. If the Termination has multiple streams, the MGC should also provide the applicable StreamID to unambiguously indicate on which Stream it should send the application protocol message.

On reception of the mcbalg/sblm signal the MG shall then send the application protocol message to the indicated destination. If the MGC requires further processing by the Context associated with the Termination that received the message, the MGC shall send the mcbalg/sblm signal from the Termination that received the original message with a signal direction equal to "internal". If the MG is unable to decode and/or encode the message for sending from the received message content parameter it shall respond with error code 449 "Unsupported or Unknown Parameter or Property Value".

### 7.6.4	Examples

The examples in this clause illustrate the usage of the MGC Controlled Bearer Level ALG package.

The examples assume that the MGC has already added two Terminations (T1 and T2) as a result of session/call control signalling. The applicable bearer level application protocol is RTSP and the SETUP and DESCRIBE methods are to be detected. Stream 2 is associated with this application protocol stream.

#### 7.6.4.1	Application control protocol message detection initiation

The MGC requests the MG to detect RTSP SETUP and DESCRIBE messages and to be notified when such messages are detected. The messages are not to be forwarded to other Terminations in the Context. Parameter "ff" is not shown as this is the default behaviour.

Option 1 – Protocol filter used

MGC to MG1:

MEGACO/3 [123.123.123.4]:55555

Transaction = 10001 {

Context = 1234 {

Modify = T1 {

Events = 2223 {

mcbalg/det{pf=554,mf=[SETUP,DESCRIBE]}

},

}

}

}

Option 2 – StreamId used to indicate protocol

MGC to MG1:

MEGACO/3 [123.123.123.4]:55555

Transaction = 10001 {

Context = 1234 {

Modify = T1 {

Events = 2223 {

mcbalg/det{stream=2,pf=554,mf=[SETUP,DESCRIBE]}}

}

}

}

#### 7.6.4.2	Message detection and notification

The MG receives the following RTSP message:

SETUP RTSP://example.com/foo/bar/baz.rm RTSP/2.0

CSeq:302

Transport:RTP/AVP;unicast;dest_addr=":4588"/":4589",

RTP/AVP/TCP;unicast;interleaved=0-1

As the method is "SETUP", this is notified to the MGC using an ObservedEvent:

MEGACO/3 [125.125.125.111]:55555

Transaction = 10002 {

Context = 1234 {

Notify = T1 {ObservedEvents =2223 {

mcbalg/det{ stream=2, mc=

"SETUP RTSP://example.com/foo/bar/baz.rm RTSP/2.0

CSeq:302

Transport:RTP/AVP;unicast;dest_addr=%22:4588%22; /%22:4589%22,

RTP/AVP/TCP;unicast;interleaved=0-1"}}

}

}

}

#### 7.6.4.3	Message modification and sending

On reception of the ObservedEvent, the MGC determines that port translation is necessary and changes the ports in the SETUP message:

SETUP RTSP://example.com/foo/bar/baz.rm RTSP/2.0

CSeq:302

Transport:RTP/AVP;unicast;dest_addr=":8000"/":8001",

RTP/AVP/TCP;unicast;interleaved=0-1

It then sends the modified message through the MG using the mcbalg/sblm signal:

MEGACO/3 [123.123.123.4]:55555

Transaction = 10003 {

Context = 1234 {

Modify = T2 {

Signals {

mcbalg/sblm{stream=2,mc=

"SETUP RTSP://example.com/foo/bar/baz.rm RTSP/2.0

CSeq:302

Transport:RTP/AVP;unicast;dest_addr=%22;:8000%22; /%22;:8001%22,

RTP/AVP/TCP;unicast;interleaved=0-1"}

},

}

}

}

# 8	MG located Bearer Level ALG

|  | Package name: | MG located Bearer Level ALG |
| --- | --- | --- |
|  | Package ID: | mgbalg (0x0###) |
|  | Description: | The actual ALG function with respect to the replacement of address information at application layer is executed by the MGC when using package mcbalg (clause 7). This package moves that function down to the MG itself. The MGC is only involved in enabling that function and (if required) providing address information to be inserted. |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

### 8.1.1	Protocol type bearer level ALG

|  | Property name: | Protocol type bearer level ALG |
| --- | --- | --- |
|  | Property ID: | ptbalg (0x0001) |
|  | Description: | This property enables/disables the MG located Bearer Level ALG based on the protocol type. |
|  | Type: | Boolean |
|  | Possible values: | ON to enable MG located bearer level ALG OFF to disable MG located bearer level ALG |
|  | Default: | OFF |
|  | Defined in: | LocalControl on an individual stream or TerminationState on the root termination. The variants of non-root and root termination level shall be mutually exclusive. An H.248 profile should indicate the supported variant. |
|  | Characteristics: | Read/write |

### 8.1.2	Upper layer protocol filter

|  | Property name: | Upper layer protocol filter |
| --- | --- | --- |
|  | Property ID: | ulpf (0x0002) |
|  | Description: | This property specifies the stream's upper layer protocols type that the filtering is applied to. The term "upper layer" denotes the protocol layer above the layer 4 transport protocol, i.e., an "upper layer" protocol is a "L4+" protocol, e.g., HTTP, MSRP, or RTSP. |
|  | Type: | Sub-list of Integer |
|  | Possible values: | A single value or a sub-list. A value representing an application protocol, using the well known or registered port number assigned to that protocol by IANA (http://www.iana.org/assignments/port-numbers). For example: 80 World Wide Web HTTP 554 Real Time Streaming Protocol (RTSP) 2855 MSRP |
|  | Default: | 0, indicating that the protocol is determined via the local and remote descriptors. |
|  | Defined in: | LocalControl on an individual stream or TerminationState on the root termination. The variants of non-root and root termination level shall be mutually exclusive. An H.248 profile should indicate the supported variant. |
|  | Characteristics: | Read/write |

### 8.1.3	Indication of "source of replaced source address information"

|  | Property name: | Source of replaced source address information part |
| --- | --- | --- |
|  | Property ID: | sosaip (0x0003) |
|  | Description: | Communication addresses in Lx-PDUs (protocol data unit at layer Lx; here primarily layers concerned above L3) contain address information of the traffic source and/or traffic sink. The ALG function may be required for the replacement of source information, for example as a result of a particular translation behaviour of remote NAT devices in the IP bearer path. The translation of source address information only is a very common ALG scenario. This property allows the MGC to indicate the source(s) used for replaced source address information. |
|  | Type: | Sublist of Enumeration |
|  | Possible values: | A single value or a sublist, according to the value of ulpf: NR [0x00] No replacement SD [0x01] Contained in H.248 Stream Descriptor PR [0x02] Provisioned in MG IP [0x03] IP (transport) address information (NOTE – This codepoint is for further studies and should be used with caution.) If the value is a sublist, the order of elements in the list shall correspond to the list of affected protocols as indicated by property ulpf. |
|  | Default: | SD |
|  | Defined in: | LocalControl on an individual stream or TerminationState on the root termination. The variants of non-root and root termination level shall be mutually exclusive. An H.248 profile should indicate the supported variant. |
|  | Characteristics: | Read/write |

### 8.1.4	Indication of "source of replaced destination address information"

|  | Property name: | Source of replaced destination address information part |
| --- | --- | --- |
|  | Property ID: | sodaip (0x0004) |
|  | Description: | Communication addresses in Lx-PDUs (protocol data unit at layer Lx; here primarily layers concerned above L3) contain address information of the traffic source and/or traffic sink. The ALG function may be required for the replacement of destination information, for example as a result of particular translation behaviour of remote NAT devices in the IP bearer path. This property allows the MGC to indicate the source used for replaced destination address information. |
|  | Type: | Sublist of Enumeration |
|  | Possible values: | A single value or a sublist, according to the value of ulpf NR [0x00] No replacement SD [0x01] Contained in H.248 Stream Descriptor PR [0x02] Provisioned in MG IP [0x03] Source IP (transport) address information If the value is a sublist, the order of elements in the list shall correspond to the list of affected protocols as indicated by property ulpf. |
|  | Default: | SD |
|  | Defined in: | LocalControl on an individual stream or TerminationState on the root termination. The variants of non-root and root termination level shall be mutually exclusive. An H.248 profile should indicate the supported variant. |
|  | Characteristics: | Read/write |

## 8.2	Events

None.

## 8.3	Signals

None.

## 8.4	Statistics

None.

## 8.5	Error codes

None.

## 8.6	Procedures

### 8.6.1	General characteristics of the MG located bearer level ALG service

#### 8.6.1.1	Directionality

The MG located bearer level ALG service affects only one traffic direction, thus belongs to the category of unidirectional address translation functions (as opposed to bidirectional ones). Appendix I.1 illustrates the concept.

#### 8.6.1.2	Context aspects

The MG located bearer level ALG is enabled at the termination in the outgoing direction (see also Appendix I.1).

### 8.6.2	Enabling the MG located bearer level ALG service

When a MG located bearer level ALG service is required for incoming packet traffic, the MGC shall set the property protocol type bearer level ALG (mgbalg/ptbalg) to "ON".

### 8.6.3	Indication of affected application protocol(s)

The bearer level ALG function is normally application specific, i.e., enabled for selected application(s). The MGC should use property upper layer protocol filter (mgbalg/ulpf) to indicate the application(s).

### 8.6.4	Indication of relevant address information and source of replaced address information

The MG located bearer level ALG service needs to know whether source or/and destination address information should be replaced. And if yes, the MG needs to know which address information has to be used for the ALG service. The MGC should provide a correspondent indication via properties mgbalg/sosaip and mgbalg/sodaip, else, a default property value shall be used.

The MG needs to know which address information has to be used for the ALG service. There are basically three options with respect to the source of replaced address information:

Table 1 – Principle options for the indication of replaced address information

| Variant | Source of translated address information | Indicated by MGC via: |
| --- | --- | --- |
| 1 | Contained in H.248 Stream Descriptor (NOTE 1). E.g., as part of the SDP media description, other than "c=" and/or "m=" line address information. Such as related to (general or application specific) SDP attributes. | Explicitly by MGC. Example: The SDP "a=path:" attribute (clause 8.2/[IETF RFC 4975]), used by application protocol ‘MSRP’. |
| 2 | Provisioned in MG. | Not indicated by the MGC. The MG is configured for that variant via management plane actions. |
| 3 | IP L3 (and L4) source (transport) address information of incoming IP packet (NOTE 2). | Default option, i.e., when neither variant 1 nor 2 is used. |
| NOTE 1 –This address information is located in the Local descriptor (if mgbalg/sosaip=SD), or in the Remote descriptor (if mgbalg/sodaip=SD) or in both. NOTE 2 – I.e., numerical IP address information. |  |  |

### 8.6.4	L4+ protocol individual B-ALG service configuration

The B-ALG service could be enabled on multiple upper layer protocol types in parallel: property value of mgbalg/ulpf would be a list format. Then, properties mgbalg/sosaip and mgbalg/sodaip shall provide the same list format, which allows the L4+ protocol individual configuration of the B-ALG service.

### 8.6.6	B-ALG: L4+ protocol specific translation details

The translation of address information at L4+ level may be not defined in a generic manner because it is dependent on the particular L4+ protocol. The protocol dependent application details are therefore out of scope of this Recommendation (as in case of the mcbalg package, see e.g. clause 7.6.4). Such information could be e.g. part of an H.248 profile specification.

### 8.6.7	Examples

### 8.6.7.1	Examples for MSRP-based instant messaging service

Use case:

MSRP is used in session-mode, leading to the transport of MSRP messages via TCP/IP packets in the bearer plane (as opposed to the page-mode, which uses the SIP MESSAGE method in the signalling plane).

Two example B-ALG configurations for MSRP are described (inclusive example H.248 signalling) in Appendix I.2.1 and I.2.2.

Appendix I

MG located bearer level ALG service – 
Illustration and signalling examples

(This appendix does not form an integral part of this Recommendation)

Clause 8 defines the B-ALG variant of a MG located bearer level ALG service. This Appendix illustrates the concept and provides example H.248 signalling.

## I.1	Illustration of concept

The configuration of a particular MG located bearer level ALG service is subject of four properties (as defined by the mgbalg package). Figure I.1 illustrates the resulting action when the package properties would be associated to a specific context (i.e., part of LocalControl descriptor).

Figure I.1 – Concept of MG located bearer level ALG service

The B-ALG configuration is part of the Stream descriptor of termination TB in Figure 1. The resulting B-ALG service is thus effective in traffic direction from X to Y.

Background information:

The B-ALG represents a NAT function at protocol layer L4+ ("L4+ NAT"). The behaviour of a L4+ NAT function should be consistent with the usual L3/L4 NAT functions, from a H.248 perspective. Such H.248 context models for L3/L4 NAT are e.g. illustrated in Annex H in [b-ETSI TR 183 068].

Essentially, the effect of a L3/L4 NAT function becomes visible at the MG external bearer interface in outgoing direction. Thus, any local, unidirectional NAT in X-to-Y direction is controlled at termination TB (and not at TA), because the replaced address information is subject of the Stream descriptor of TB in case of L3/L4 NAT.

The same principle applies for local L4+ NAT, i.e., the B-ALG in this Recommendation.

If translation is required, the relevant address information part is controlled by the two properties mgbalg/sosaip and mgbalg/sodaip (for source and destination address information). Figure I.1 depicts the three options concerning the source of replaced address information by their associated property codepoints.

## I.2	Illustration of example use cases

## Some B-ALG services are illustrated using the example of MSRP traffic (see introduction in clause 6 and Figure 1 concerning L4+ address information elements).

### I.2.1	Use case "Unidirectional B-ALG with destination address information translation"

This example considers a unidirectional B-ALG for MSRP, taking the assumption that the originating MSRP client (here X) could already insert the correct L4+ source address information (i.e., the MSRP header "From-Path" parameter value is already valid from end-to-end perspective). However, the L4+ destination address information of incoming MSRP messages is not correct, thus a correspondent B-ALG service needs to be enforced in the MG (see Figure I.2).

Figure I.2 – Use case "Unidirectional B-ALG with destination address 
information translation" – MSRP bearer traffic

The TCP/IP header is used as the source of replaced address information (see example H.248 syntax in Table I.1). The translated MSRP "To-Path" element contains a numerical IP address value (note: a named IP address format would imply a reverse DNS lookup).

Table I.1 – Example command encoding– B-ALG configuration by MGC

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:54321 Transaction = … { Context = … { Add = ip/$ { ; Termination TB, SEP S1 .Media { Stream = 1 { LocalControl { mgbalg/ptbalg = ON ; B-ALG enabled mgbalg/ulpf = 2855 ; for MSRP traffic mgbalg/sosaip = NR ; no source address replacem. mgbalg/sodaip = IP ; from TCP/IP header info. } ... Local { v=0 c=IN IP6 <IP_addr> m=message <port> TCP/MSRP – ; NOTE 1 }, Remote { v=0 c=IN IP6 <IP_addr> m=message <port> TCP/MSRP – }…} | NOTE 1 – The "m=" line <proto> provides an application-aware indication due to the enforced B-ALG function (for application 'MSRP'). |

### I.2.2	Use case "Bidirectional B-ALG with source & destination address information translation"

This example considers a bidirectional B-ALG for MSRP with the need to adapt L4+ source and destination address information. Hence, both MSRP header elements "To-Path" and "From-Path" needs to be modified by the MG. The replaced address information is provided by the MGC to the MG via the (MSRP-related) SDP attribute "a=path:…", which are subject of the LD and RD.

Figure I.3 depicts the bidirectional B-ALG service, as composed by two unidirectional B-ALG functions associated to the two terminations TA and TB.

Figure I.3 – Use case "Bidirectional B-ALG with source & destination 
address information translation" – MSRP bearer traffic

Example H.248 syntax is abstracted in Table I.2.

Table I.2 – Example command encoding– B-ALG configuration by MGC

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:54321 Transaction = … { Context = … { Add = ip/$ { ; Termination TB, SEP S1 .Media { Stream = 1 { LocalControl { mgbalg/ptbalg = ON ; B-ALG enabled mgbalg/ulpf = 2855 ; for MSRP traffic mgbalg/sosaip = SD ; for both address info. mgbalg/sodaip = SD ; from Stream Descriptor } ... Local { v=0 c=IN IP6 <IP_addr> ; NOTE 1 m=message <port> TCP/MSRP – ; NOTE 1 a=path:… ; NOTE 2 }, Remote { v=0 c=IN IP6 <IP_addr> ; NOTE 3 m=message <port> TCP/MSRP – ; NOTE 3 a=path:… ; NOTE 4 }…} Add = ip/$ { ; Termination TA, SEP S1 .Media { Stream = 1 { LocalControl { mgbalg/ptbalg = ON ; B-ALG enabled mgbalg/ulpf = 2855 ; for MSRP traffic mgbalg/sosaip = SD ; for both address info. mgbalg/sodaip = SD ; from Stream Descriptor } ... Local { v=0 c=IN IP6 <IP_addr> ; NOTE 5 m=message <port> TCP/MSRP – ; NOTE 5 a=path:… ; NOTE 6 }, Remote { v=0 c=IN IP6 <IP_addr> ; NOTE 7 m=message <port> TCP/MSRP – ; NOTE 7 a=path:… ; NOTE 8 }…} | Configuration of unidirectional B-ALG in X-to-Y direction (i.e., Termination TB: - The local L3/L4 source address information is subject of the correspondent LD SDP "c="-/"m="-line contents (NOTE 1). - The local L4+ source address information is subject of LD SDP attribute "a=path:…" (NOTE 2). - The local L3/L4 destination address information is subject of the correspondent RD SDP "c="-/"m="-line contents (NOTE 3). - The local L4+ destination address information is subject of RD SDP attribute "a=path:…" (NOTE 4). Configuration of unidirectional B-ALG in Y-to-X direction (i.e., Termination TA: - The local L3/L4 source address information is subject of the correspondent LD SDP "c="-/"m="-line contents (NOTE 5). - The local L4+ source address information is subject of LD SDP attribute "a=path:…" (NOTE 6). - The local L3/L4 destination address information is subject of the correspondent RD SDP "c="-/"m="-line contents (NOTE 7). - The local L4+ destination address information is subject of RD SDP attribute "a=path:…" (NOTE 8). |

Bibliography

[b-ETSI TR 183 068]	ETSI 183 068 V3.1.1 (2009-08), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

[b-ITU-T H.248.64]		Recommendation ITU-T H.248.64 (03/2013), Gateway control protocol: IP router packages.

[b-IETF RFC 2663]		IETF RFC 2663 (1999), IP Network Address Translator (NAT) Terminology and Considerations.

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | Telecommunication management, including TMN and network maintenance |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Terminals and subjective and objective assessment methods |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks, open system communications and security |
| Series Y | Global information infrastructure, Internet protocol aspects and next-generation networks |
| Series Z | Languages and general software aspects for telecommunication systems |

________________________

____________________