| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-18 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, CH, 10 - 14 March, 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.WEBRTC |  |  |  |
| Title: | H.248.WEBRTC "Guidelines on the use of H.248 capabilities for web-based real-time communication services in H.248 profiles" (New): Output draft Ed. 0.10 |  |  |  |
| Purpose: | Meeting output (Draft Recommendation) |  |  |  |

Summary

This document contains the editor's output draft for draft new H.248.WEBRTC "Guidelines on the use of H.248 capabilities for web-based real-time communication services in H.248 profiles" (Ed. 0.10).

__________________

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.WEBRTC Ed. 0.1 | Input (AVD-4275) to WP 2/16 Brisbane meeting (2012-09) Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/AVD-4275.zip |
| H.248.WEBRTC Ed. 0.2 | Output (TD-26) from WP 2/16 Brisbane meeting (2012-09) Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/TD-26.zip |
| H.248.WEBRTC Ed. 0.3 | Input (TD 22/WP1) from SG16 Geneva meeting, January 2013 Location: http://itu.int/md/T13-SG16-130114-TD-WP1-0022 |
| H.248.WEBRTC Ed. 0.4 | Output (TD 22R1/WP1) from SG16 Geneva meet., Jan 2013 Location: http://itu.int/md/T13-SG16-130114-TD-WP1-0022 |
| H.248.WEBRTC Ed. 0.5 | Input (AVD-4369) to WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/AVD-4369.zip |
| H.248.WEBRTC Ed. 0.6 | Output (TD-19) from WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/TD-19.zip |
| H.248.WEBRTC Ed. 0.7 | Input (TD 86/WP1) to SG16 Geneva meeting, Oct/Nov 2013 Location: https://www.itu.int/md/dologin_md.asp?lang=en&id=T13-SG16-131028-TD-WP1-0086!MSW-E |
| H.248.WEBRTC Ed. 0.8 | Output (TD 86R1/WP1) from SG16 Geneva meeting, Oct/Nov 2013 Location: https://www.itu.int/md/dologin_md.asp?lang=en&id=T13-SG16-131028-TD-WP1-0086!R1!MSW-E |
| H.248.WEBRTC Ed. 0.9 | Input (AVD-4478) to WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/AVD-4478.zip |
| H.248.WEBRTC Ed. 0.10 | Output (TD-18) from WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/TD-18.zip |
| H.248.WEBRTC Ed. 0.11 |  |

CONTENTS

Page

1 Scope 6 2 References 6 3 Definitions 8 3.1 Terms defined elsewhere 8 3.2 Terms defined in this Recommendation 8 4 Abbreviations and acronyms 9 5 Conventions 10 5.1 Acronym usage 10 5.2 Prescriptive language 10 6 WEBRTC core technology overview 10 7 H.248 WEBRTC gateways: example use cases 11 7.1 Point-to-point communication 11 7.1.1 Use case #1: WebRTC-to-WebRTC interworking 11 7.1.2 Use case #2: WebRTC-to-NGN/IMS interworking 12 7.1.3 Use case #3: WebRTC-to-PSTN/ISDN interworking 12 7.2 Multipoint communication 13 8 Functional requirements for H.248 WEBRTC gateways 13 8.1 Requirements related to NAT traversal support 13 8.1.1 Media latching based NAT traversal support 13 8.1.2 ICE based NAT traversal support 13 8.2 Requirements related to communication topologies 13 8.2.1 H.248 WEBRTC gateway for point-to-point interworking 13 8.2.2 H.248 WEBRTC gateway as WEBRTC conferencing point 13 8.3 Requirements related to bearer traffic multiplexing 13 8.4 Requirements related to RTP-to-RTP type interworking 14 8.4.1 RTCP control flow component 14 8.4.2 RTP profiles 14 8.4.3 RTP multiplexing 14 8.4.4 RTP topologies 14 8.4.5 RTCP based services, report types and packet formats 14 8.4.6 Resource allocation rules for RTP/RTCP 15 8.5 Requirements related to media-aware type of interworking 15 8.5.1 Requirements related to WEBRTC audio 15 8.5.2 Requirements related to WEBRTC video 15 8.5.3 Requirements related to WEBRTC data 15 8.5.4 Requirements related to WEBRTC text 16 8.6 Requirements related to bearer security 16 8.6.1 Requirements related to transport security 16 8.6.2 Requirements related to media security 16 8.7 Requirements related to WEBRTC emergency services 17 8.8 Requirements related to QoS support 17 8.8.1 IP traffic control: QoS marking 17 8.8.2 IP traffic control: traffic policing – IP byterate policing 17 8.8.3 Improved transport robustness for RTP 17 8.9 Requirements related to bearer-path coupled congestion controls 17 8.10 Requirements related to performance monitoring 17 8.10.1 Requirements related to measurement point (MP) 17 8.10.2 Requirements related to collection point (CP) 17 8.10.3 Requirements related to reporting point (RP) 18 8.10.4 Requirements related to filtering point (FP) 18 8.10.5 Requirements related to loopback point (LP) 18 8.11 Requirements related to SDP-based description, declaration and negotiation of WEBRTC media configurations 18 8.11.1 Requirements related to supported SDP elements 18 8.11.2 Requirements related to SDP Offer/Answer 18 9 H.248 profile specification guidelines 18 Appendix L1 Living List – WebRTC media component "data" – Discussion of interworking scenarios and H.248 connection models 18 L1.1 Scope 18 L1.2 Possible interworking models by H.248 WebRTC gateways 19 L1.3 Relevant documentation 20 Appendix L2 Living List – H.248 WebRTC gateway capabilities for 3GPP Rel-12 22 L2.1 3GPP references 22 L2.2 H.248 WebRTC media gateway: media plane interworking scenarios 22 Bibliography - Placeholder 23

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 1 – Use case #1: WebRTC-to-WebRTC interworking 11 Figure 2 – Use case #2: WebRTC-to-NGN/IMS interworking 12 Figure 3 – Use case #3: WebRTC-to-PSTN/ISDN interworking 12 Figure L1.1 – H.248 WebRTC gateway – Interworking and scope on Data Channel (multiple IM sessions) 19 Figure L1.2 – H.248 WebRTC gateway – Example model for H.248 SEPs 20 Figure L1.3 – Landscape of related IETF Documents ("WebRTC data channel") 21 Figure L2.1 – 3GPP Rel-12 – H.248 WebRTC media gateway: media plane interworking scenarios 23

Draft new Recommendation ITU-T H.248.xx (ex H.248.WEBRTC)

Guidelines on the use of H.248 capabilities for 
Web-based Real-Time Communication Services in H.248 Profiles

AAP Summary

<Mandatory material to be provided before Consent>

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

Web-based real-time communication is a standardized service by the IETF and W3C (called WEBRTC or RTCWEB service) and defines a particular protocol suite for IP-based communication in web browser environments. This application is related to multimedia conversational services which cover typical service components such as telephony, conferencing, instant messaging, etc.

This Recommendation

- describes example use cases with H.248 WEBRTC gateway involvement;

- identifies H.248 capabilities for such gateways; and

- provides guidelines for the specification of H.248 profiles for dedicated H.248 WEBRTC gateway types.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.37]	Recommendation ITU-T H.248.37 (06/2008), Gateway control protocol: IP NAPT traversal package.

[ITU-T H.248.39]	Recommendation ITU-T H.248.39 (05/2006), Gateway control protocol: H.248 SDP parameter identification and wildcarding.

[ITU-T H.248.48]	Recommendation ITU-T H.248.48 (02/2012), Gateway control protocol: RTCP XR block reporting package.

[ITU-T H.248.50]	Recommendation ITU-T H.248.50 (09/2010), Gateway control protocol: NAT traversal toolkit packages.

[ITU-T H.248.52]	Recommendation ITU-T H.248.52 (06/2008), Gateway control protocol: QoS support packages.

[ITU-T H.248.53]	Recommendation ITU-T H.248.53 (03/2009), Gateway control protocol: Traffic management packages.

[ITU-T H.248.57]	Recommendation ITU-T H.248.57 (03/2013), Gateway control protocol: RTP control protocol package.

[ITU-T H.248.71]	Recommendation ITU-T H.248.71 (02/2010), Gateway control protocol: RTCP support packages.

[ITU-T H.248.77]	Recommendation ITU-T H.248.77 (09/2010), Gateway control protocol: Secure real-time transport protocol (SRTP) package and procedures.

[ITU-T H.248.78]	Recommendation ITU-T H.248.78 (03/2013), Gateway control protocol: Bearer-level application level gateway.

[ITU-T H.248.87]	Recommendation ITU-T H.248.87 (01/2014), Gateway control protocol: Guidelines on the use of ITU-T H.248 capabilities for performance monitoring in RTP networks in ITU-T H.248 profiles.

[ITU-T H.248.88]	Recommendation ITU-T H.248.88 (01/2014), Gateway control protocol: RTP topology dependent RTCP handling by ITU-T H.248 media gateways with IP terminations.

[ITU-T H.248.DTLS]	Draft Recommendation ITU-T H.248.DTLS (XX/2014), Gateway control protocol: H.248 support for control of transport security using DTLS.

[ITU-T H.248.RTPMUX]	Draft Recommendation ITU-T H.248.RTPMUX (XX/2014), Gateway control protocol: H.248 support for RTP multiplexing.

[ITU-T H.248.SCTP]	Draft Recommendation ITU-T H.248.SCTP (XX/2014), Gateway control protocol: H.248 support for control of SCTP bearer connections.

[ITU-T H.248.SEPLINK]	Draft Recommendation ITU-T H.248.SEPLINK (XX/2014), Gateway control protocol: Stream endpoint interlinkage package.

[ITU-T H.320]	Recommendation ITU-T H.320 (03/2004), Narrow-band visual telephone systems and terminal equipment.

[ITU-T H.324]	Recommendation ITU-T H.324 (04/2009), Terminal for low bit-rate multimedia communication.

[ITU-T T.140]	Recommendation ITU-T T.140 (02/1998), Protocol for multimedia application text conversation.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 4585]	IETF RFC 4585 (2006), Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF).

[IETF RFC 4961]	IETF RFC 4961 (2007), Symmetric RTP / RTP Control Protocol (RTCP).

[IETF RFC 5109]	IETF RFC 5109 (2007), RTP Payload Format for Generic Forward Error Correction.

[IETF RFC 5124]	IETF RFC 5124 (2008), Extended Secure RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/SAVPF).

[IETF RFC 5506]	IETF RFC 5506 (2009), Support for Reduced-Size Real-Time Transport Control Protocol (RTCP): Opportunities and Consequences.

[IETF RFC 5576]	IETF RFC 5576 (2009), Source-Specific Media Attributes in the Session Description Protocol (SDP).

[IETF RFC 7007]	IETF RFC 7007 (2013), Update to Remove DVI4 from the Recommended Codecs for the RTP Profile for Audio and Video Conferences with Minimal Control (RTP/AVP).

[IETF rtcweb-overview]	IETF draft-ietf-rtcweb-overview (2014), Overview: Real Time Protocols for Brower-based Applications.

[IETF rtp-usage]	IETF draft-ietf-rtcweb-rtp-usage (2014), Web Real-Time Communication (WebRTC): Media Transport and Use of RTP.

[IETF rtcweb-secur]	IETF draft-ietf-rtcweb-security (2014), Security Considerations for WebRTC.

[IETF rtcweb- security-arch]	IETF draft-ietf-rtcweb-security-arch (2014), WebRTC Security Architecture.

[IETF rtcweb-transports]	IETF draft-ietf-rtcweb-transports (2014), Transports for RTCWEB.

[IETF xrblock-webrtc]	IETF raft-singh-xrblock-webrtc-additional-stats (201#), Additional RTP Control Protocol (RTCP) Extended Report (XR) Metrics for WebRTC Statistics API.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	media path [IETF rtcweb-overview]: The path that media data follows from one browser to another.

NOTE – Relates to the bearer plane interface (at MG) in case of WEBRTC H.248 gateway.

3.1.2	web browser [ITU-T H.351]: A software application capable of rendering HTML and XHTML documents.

{Editor’s note (2012-09): there might be a better definition (in the context of WEBRTC). Contributions are solicited.}

3.1.3	signalling path [IETF rtcweb-overview]: The communication channels used between entities participating in signaling to transfer signaling. There may be more entities in the signaling path than in the media path..

NOTE – Relates to the signalling plane call control interface (at MGC) in case of a WEBRTC H.248 gateway.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	WEBRTC H.248 gateway: a decomposed gateway according H.248 (i.e., a MGC-MG tandem), using an H.248 profile with support of web-based real-time communication services.

NOTE – The particular gateway type may be further qualified, - based on the application specific profile -, such as WEBRTC PSTN gateway, WEBRTC border gateway, WEBRTC IMS access gateway, etc.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| BFCP | Binary Floor Control Protocol |
| --- | --- |
| DTLS | Datagram Transport Layer Security |
| HTTP | Hyper-text transfer protocol |
| ICE | Interactive Connectivity Establishment |
| IMS | IP multimedia subsystem |
| IP | Internet protocol |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |
| LD | Local descriptor |
| MG | Media gateway |
| MGC | Media gateway controller |
| MSRP | Message Session Relay Protocol |
| MTI | Mandatory To Implement |
| NACK | (RTP/RTCP) Negative Acknowledgements |
| NAT | Network address translation |
| NAT-T | NAT Traversal |
| NGN | Next generation network |
| PSTN | Public Switched Telephone Network |
| RD | Remote descriptor |
| RTCP | RTP Control Protocol |
| RTCWEB | Real-Time Communication in WEB-browsers (as work item in IETF) |
| RTP | Real-time Transport Protocol |
| SCTP | Stream Control Transmission Protocol |
| SDP | Session description protocol |
| SEP | (H.248) Stream endpoint |
| SEPP | (H.248) Stream endpoint pair |
| SIP | Session initiation protocol |
| SSRC | Synchronization Source |
| SRTP | Secure RTP |
| STUN | Session Traversal Utilities for NAT |
| TCP | Transmission control protocol |
| TLS | Transport Layer Security |
| TURN | Traversal Using Relays around NAT |
| UDP | User datagram protocol |
| W3C | World Wide Web Consortium (http://www.w3.org/) |
| WEB | "WWW” (World Wide Web) |
| WEBRTC | Web-based Real-Time Communication = Real-Time Communication in WEB-browsers (as work item in W3C) |

# 5	Conventions

## 5.1	Acronym usage

The two acronyms RTCWEB and WEBRTC denote the work split on the same service in the two different SDOs IETF and W3C. Both are synonym from perspective of H.248 entities. This Recommendation uses the acronym WEBRTC only for the underlying communication service.

## 5.2	Prescriptive language

This document provides a list of items, labelled as R-x/y, where x refers to the clause number and y a number within that clause. Such items use the following keywords with meanings as prescribed below:

The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

The keywords "is prohibited from" indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required.  Thus this requirement need not be present to claim conformance.

The keywords "can optionally" indicate an optional requirement which is permissible, without implying any sense of being recommended. This term is not intended to imply that the vendor’s implementation must provide the option and the feature can be optionally enabled by the network operator/service provider.  Rather, it means the vendor may optionally provide the feature and still claim conformance with the specification.

# 6	WEBRTC core technology overview

The H.248 WEBRTC gateway is peering to WEBRTC endpoints, given by IP host entities with "WEBRTC capable web browsers” as application instance. WEBRTC endpoints are characterized by following mandatory (and optional) capabilities (according to the "RTCWEB protocol specification" [IETF rtcweb-overview]):

- WEBRTC as multimedia conversational communication service with application components audio, video and data (which covers again a bunch of "data services);

- Application data transport (see [IETF rtcweb-transports]):

  - audio, video: RTP/RTCP in specific capability sets (see [IETF rtp-usage])

  - data:

- Application data framing and securing (see [IETF rtcweb-security-arch], [IETF rtcweb-secur]):

  - audio, video: SRTP

  - data: DTLS

- Application data formats (see [IETF ###]):

  - audio codec types:

  - video codec types:

  - data:

- NAT traversal techniques for end-to-end IP media path connectivity

- Quality of service

- Performance monitoring

# 7	H.248 WEBRTC gateways: example use cases

Some exemple use cases are described in this clause.

## 7.1	Point-to-point communication

All examples here are characterized by a two-party call (between A and B) and that party A is always using WebRTC browser environment.

### 7.1.1	Use case #1: WebRTC-to-WebRTC interworking

See Figure 1.

Figure 1 – Use case #1: WebRTC-to-WebRTC interworking

Use case #1 relates to a browser-to-browser scenario: the WEBRTC H.248 gateway may be needed e.g. due to different NAT-T requirements in the two networks where the WebRTC endpoints reside, due to codec mismatch between both WebRTC endpoints, due to different QoS architecture of the networks where the WebRTC endpoints reside, etc.

### 7.1.2	Use case #2: WebRTC-to-NGN/IMS interworking

Figure 2 depicts an interworking scenario with NGN/IMS networks.

Figure 2 – Use case #2: WebRTC-to-NGN/IMS interworking

Use case #2 is fundamentally motivated by the fact that NGN/IMS IP user equipment is supposed to be not fully identical to WebRTC endpoints (in terms of capabilities as listed in clause 8).

### 7.1.3	Use case #3: WebRTC-to-PSTN/ISDN interworking

Figure 3 outlines the general request in supporting of still existing PSTN/ISDN infrastructures.

Figure 3 – Use case #3: WebRTC-to-PSTN/ISDN interworking

Use case #3 provides again many variants, such as

- the down-negotiation of an initial multimedia call request to an audio-only call,

- the transcoding between different audio codecs,

- the support of PSTN/ISDN multimedia terminals (e.g. [ITU-T H.324], [ITU-T H.320]).

## 7.2	Multipoint communication

Multiparty calls are in scope of WEBRTC and may demand for "RTP mixer”, as e.g. provided by media servers. Such a network scenario may therefore relate to H.248 profile types with regards to media servers (aka media resource processors).

# 8	Functional requirements for H.248 WEBRTC gateways

There are general and use case specific functional requirements for H.248 WEBRTC gateways. WEBRTC gateway requirements could be classified in subsequent functional areas.

## 8.1	Requirements related to NAT traversal support

### 8.1.1	Media latching based NAT traversal support

R-8.1.1/1: The H.248 WEBRTC gateway is required to support media latching according to the H.248 IP NAPT traversal package [ITU-T H.248.37].

R-8.1.1/2: The H.248 WEBRTC gateway is recommended to support the media latching associated address reporting capability according to the H.248 address reporting package [ITU-T H.248.37].

{Editor’s note (2014-03 meeting): depends on further anlaysis related to ICE-based address reporting …}

### 8.1.2	ICE based NAT traversal support

R-8.1.2/1: The H.248 WEBRTC gateway is required to support ICE for UDP connections, according to [ITU-T H.248.50].

R-8.1.2/2: …

{Editor’s note (2014-03 meeting): the ICE related requirements should be more detailed … as well as required associated H.248.50 building blocks…}

## 8.2	Requirements related to communication topologies

### 8.2.1	H.248 WEBRTC gateway for point-to-point interworking

R-8.2.1/1: The H.248 WEBRTC gateway is required to support an H.248 (IP, IP) or (IP, non-IP connection models (use cases #1,2 and #3).

### 8.2.2	H.248 WEBRTC gateway as WEBRTC conferencing point

R-8.2.2/1: The H.248 WEBRTC gateway is required to support the conferencing extensions for RTP media as indicated in clause 5.1/[IETF rtp-usage].

## 8.3	Requirements related to bearer traffic multiplexing

R-8.3.1/1: The H.248 WEBRTC gateway is required to support at least two UDP transport connections for a single, multiplexed WEBRTC multimedia call.

NOTE – This is the bearer configuration with a maximum degree of multiplexing. All audio and video traffic share a single UDP transport connection by usage of RTP transport and media multiplexing. And all data traffic is sharing a single UDP transport connection with a single DTLS session with a single SCTP association.

## 8.4	Requirements related to RTP-to-RTP type interworking

### 8.4.1	RTCP control flow component

R-8.4.1/1: The H.248 WEBRTC gateway is required to support always RTCP control flow besides the RTP media flow (see clause 4.1/[IETF rtp-usage]).

RTCP based services …

### 8.4.2	RTP profiles

R-8.4.2/1: The H.248 WEBRTC gateway is required to support RTP profile "RTP/SAVPF" according to [IETF RFC 5124, IETF RFC 7007] (see clause 4.2/[IETF rtp-usage]).

### 8.4.3	RTP multiplexing

R-8.4.3/1: The H.248 WEBRTC gateway is required to support RTP media multiplexing ("SSRC multiplexing") (see clause 4.4/[IETF rtp-usage]).

NOTE 1 – The multiplexing mode will only be used when all participants agree. If not, unmultiplexed mode will be used. Thus, any MGC involved in the call control level end-to-end capability negotiations could actually downgrade to unmultiplexed mode.

R-8.4.3/2: The H.248 WEBRTC gateway is required to support RTP transport multiplexing ("RTP / RTCP transport multiplexing") (see clause 4.5/[IETF rtp-usage]).

NOTE 2 – Same comment as in NOTE 1: backward compatibility requires to support unmultiplexed mode as well.

### 8.4.4	RTP topologies

### NOTE – All required RTP topologies are already outlined by [ITU-T H.248.88].

R-8.4.4/1: Use case independent: the H.248 WEBRTC gateway is required to comply with the "RTP topology" behaviour according to [ITU-T H.248.88].

R-8.4.4/2: Use case #1: the H.248 WEBRTC gateway is required to support topology

- "RTP transparent forwarding",

- "RTP transport translator" or

- "RTP media translator",

dependent on the call-level end-to-end negotiation of the used "RTP configuration" for WebRTC.

R-8.4.4/3: Use case #2: The H.248 WEBRTC gateway is required to support, in addition to RTP topologies of use case #1, the toplogy

- "Back-to-back RTP end system (B2BRE)"

due to the "legacy usage of RTP" in the "NGN/IMS IP domain".

NOTE – All existing H.248 profiles for border gateways are not explicit on RTP topologies in detail. Thus, the B2BRE topology represents a default topology (due to the back-to-back IP host configuration of an RTP-RTP context).

R-8.4.4/4: Use case #3: The H.248 WEBRTC gateway is required to support the "RTP endsystem" (RTPE) topology (see clause 7.1/[ITU-T H.248.88]).

### 8.4.5	RTCP based services, report types and packet formats

R-8.4.5/1: The H.248 WEBRTC gateway is required to support reduced size RTCP according to [IETF RFC 5506] (see clause 4.6/[IETF rtp-usage]).

### 8.4.6	Resource allocation rules for RTP/RTCP

#### 8.4.6.1	Port allocation rules for RTP and RTCP

R-8.4.6.1/1: The H.248 WEBRTC gateway is required to support symmetric RTP/RTCP according to [IETF RFC 4961] (see clause 4.7/[IETF rtp-usage]), which relates to the H.248-controlled port allocation as defined by [ITU-T H.248.57].

### 8.4.6.2	SSRC allocation rules for RTP

R-8.4.6.2/1: The H.248 WEBRTC gateway is required to support signalled RTP synchronisation source (SSRC) identifiers according to [IETF RFC 5576] (see clause 4.8/[IETF rtp-usage]).

#### 8.4.6.3	CNAME allocation rules for RTP/RTCP

R-8.4.6.2/1: The H.248 WEBRTC gateway is required to support unique RTCP CNAME value(s) (see clause 4.9/[IETF rtp-usage]).

NOTE – A single CNAME might be sufficient for a single H.248 MG entity.

R-8.4.6.2/2: The H.248 WEBRTC gateway can optionally be requested to report received RTCP CNAME value(s) from remote WebRTC clients (according to the RTCP Source Description package as defined by [ITU-T H.248.71].

## 8.5	Requirements related to media-aware type of interworking

### 8.5.1	Requirements related to WEBRTC audio

#### 8.5.1.1	Requirements related to media format (audio)

R-8.5.1.1/1: The H.248 WEBRTC gateway is required to support audio transcoding between the WEBRTC MTI audio codec(s) and other audio codecs as typically used in NGNs.

#### 8.5.1.2	Requirements related to media adaptation and rate control (audio)

FIXTHIS … see clause 7/[IETF rtp-usage]

### 8.5.2	Requirements related to WEBRTC video

#### 8.5.2.1	Requirements related to media format (video)

R-8.5.2.1/1: The H.248 WEBRTC gateway as IP-IP media gateway is not required to support video transcoding (because, if required at all, provided by centralized media servers).

R-8.5.2.1/2: The H.248 WEBRTC gateway as media server can optionally support video transcoding between the WEBRTC MTI video codec(s) and other video codecs as typically used in NGNs.

NOTE – The optional tagging of this requirement is due to the fact that it is expected that potential scenarios for video transcoding might be fairly low and exceptional. It is rather expected that WebRTC endpoints will negotiate a common video codec, or fallback to a default codec, or even not use video (TBC).

#### 8.5.2.2	Requirements related to media adaptation and rate control (video)

FIXTHIS … see clause 7/[IETF rtp-usage]

### 8.5.3	Requirements related to WEBRTC data

### 8.5.3.1	Requirements for MSRP-based instant messaging

R-8.5.3.1/1: The H.248 WEBRTC gateway is required to support transparent forwarding of MSRP protocol data units ("MSRP messages"), independent of the underlying (WebRTC specific or legacy) transport (protocol stack). This includes interworking between WebRTC clients or a WebRTC and non-WebRTC client.

R-8.5.3.1/2: The H.248 WEBRTC gateway is required to support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to "TCP" for MSRP traffic.

R-8.5.3.1/3: The H.248 WEBRTC gateway can optionally support a bearer level application gateway (B-ALG) at MSRP layer according to [ITU-T H.248.78]. Such a capability relates to a specific MSRP B2BUA function with the purpose of a L4+ level NAT traversal support.

#### 8.5.3.2	Requirements for BFCP-based conference control

R-8.5.3.2/1: The H.248 WEBRTC gateway is required to support transparent forwarding of BFCP protocol data units ("BFCP messages"), independent of the underlying (WebRTC specific or legacy) transport (protocol stack). This includes interworking between WebRTC clients or a WebRTC and non-WebRTC client.

R-8.5.3.2/2: The H.248 WEBRTC gateway is required to support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to "TCP" for BFCP traffic.

### 8.5.4	Requirements related to WEBRTC text

### WebRTC text telephony (text conversation) is considered as "data traffic" (despite the fact of the usual RTP-based transport in non-WebRTC environments).

R-8.5.4/1: The H.248 WEBRTC gateway is required to support transparent forwarding of T.140 protocol data units (according to [ITU-T T.140]), independent of the underlying (WebRTC specific or legacy) transport (protocol stack). This includes interworking between WebRTC clients or a WebRTC and non-WebRTC client.

R-8.5.4/2: The H.248 WEBRTC gateway is required to support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to "RTP/UDP" for T.140 traffic.

## 8.6	Requirements related to bearer security

### 8.6.1	Requirements related to transport security

R-8.6.1/1: The H.248 WEBRTC gateway is required to support transport security for UDP using DTLS, according to [ITU-T H.248.DTLS].

{Editor’s note (2014-03 meeting): the specific H.248.DTLS use cases should be detailed (such as related to e2ae, etc)…}

R-8.6.1/2: The H.248 WEBRTC gateway is required to use the DTLS session for SCTP-encapsulated WebRTC data traffic, according to [ITU-T H.248.DTLS, H.248.SCTP].

R-8.6.1/3: The H.248 WEBRTC gateway is required to use the DTLS session for SRTP related key exchange procedures (for WebRTC audio and video traffic), according to [ITU-T H.248.DTLS, H.248.77].

### 8.6.2	Requirements related to media security

R-8.6.2/1: The H.248 WEBRTC gateway is required to support media security for RTP using SRTP, according to [ITU-T H.248.77].

{Editor’s note (2014-03 meeting): the specific H.248.77 use cases should be detailed …}

R-8.6.2/2: The H.248 WEBRTC gateway is required to support the media-path coupled SRTP key exchange according to requirement [R-8.6.1/2].

R-8.6.2/3: The H.248 WEBRTC gateway can optionally support the signalling-path coupled SRTP key exchange, i.e., the "SDES" (SDP security descriptions) related method according to [ITU-T H.248.77].

## 8.7	Requirements related to WEBRTC emergency services

FIXTHIS {related to draft-aboba-rtcweb-ecrit}

## 8.8	Requirements related to QoS support

### 8.8.1	IP traffic control: QoS marking

R-8.8.1/1: The H.248 WEBRTC gateway is required to support DiffServ based QoS marking according to [ITU-T H.248.52] (see clause x.x/[IETF rtp-usage]).

NOTE – The H.248 WEBRTC MG would then provide the so-called "DS pre-marker" role (see Appendix IV/[ITU-T H.248.52]).

### 8.8.2	IP traffic control: traffic policing – IP byterate policing

R-8.8.2/1: The H.248 WEBRTC gateway is required to support IP byterate policing according to [ITU-T H.248.53].

### 8.8.3	Improved transport robustness for RTP

### See clause 6/[IETF rtp-usage].

R-8.8.3/1: The H.248 WEBRTC gateway is recommended to support negative acknowledgements (NACKs) for RTP data packets [IETF RFC 4585] (see clause 6.1/[IETF rtp-usage]).

FIXTHIS {detailed NACK profile from perspective of H.248 WEBRTC gateways.}

R-8.8.3/2: The H.248 WEBRTC gateway can optionally support Forward Error Correction (FEC) for RTP [IETF RFC 5109] (see clause 6.2/[IETF rtp-usage]).

FIXTHIS {detailed NACK profile from perspective of H.248 WEBRTC gateways.}

## 8.9	Requirements related to bearer-path coupled congestion controls

FIXTHIS {related to H.248.82, clause 7.3/[IETF rtp-usage],  draft-alvestrand-rtcweb-congestion, …}

## 8.10	Requirements related to performance monitoring

The requirements framework in this clause follows the [ITU-T H.248.87] "guidelines on the use of ITU-T H.248 capabilities for performance monitoring in RTP networks".

### 8.10.1	Requirements related to measurement point (MP)

R-8.10.1/1: The H.248 WEBRTC gateway is required to execute measurements for the performance metrics as defined by ######## FIXTHIS ####.

{Editor’s note (2014-03 meeting): H.248 nt / rtp package statistics should be deprecated and replaced by their revised (RTCP XR) metrics …}

### 8.10.2	Requirements related to collection point (CP)

### WebRTC clients (as peered by H.248 WEBRTC gateway) shall/should/can (still open) support the measurement of following performance metrics:

### draft-alvestrand-rtcweb-stats-registry

### the nine metrics as defined by the "XR Block Metrics for WebRTC Statistics API" ([IETF xrblock-webrtc])

### When WebRTC clients support the measurement of RTCP XR based metrics, then it is expected that they report their measurements via RTCP along the WebRTC media path.

R-8.10.2/1: The H.248 WEBRTC gateway is required to collect remote measurement data via incoming RTCP reports.

### 8.10.3	Requirements related to reporting point (RP)

R-8.10.3/1: The H.248 WEBRTC gateway is required to support the reporting of RTCP XR related H.248 statistics according to the RTCP XR block reporting package [ITU-T H.248.48].

### 8.10.4	Requirements related to filtering point (FP)

FIXTHIS

### 8.10.5	Requirements related to loopback point (LP)

### None.

## 8.11	Requirements related to SDP-based description, declaration and negotiation of WEBRTC media configurations

### 8.11.1	Requirements related to supported SDP elements

FIXTHIS

### 8.11.2	Requirements related to SDP Offer/Answer

FIXTHIS {related to H.248.80)}

# 9	H.248 profile specification guidelines

{Editor’s note (2013-01 meeting): this clause would/should follow the structure of clause 8 of H.248.81.}

Appendix L1
Living List – WebRTC media component "data" – Discussion 
of interworking scenarios and H.248 connection models

(This appendix does only contain working material.)

{Editor's note (2013-01 meeting):

Purpose: this Appendix parks potential revision material for H.248.77 as long as evaluation still ongoing.

Ref: T13-SG16-C-0050

Initial proposal plus possible solution (in yellow highlighted), see below:

## L1.1	Scope

WebRTC media component data and discussion of interworking options by H.248 WebRTC gateways.

## L1.2	Possible interworking models by H.248 WebRTC gateways

Figure L1.1 illustrates an example of WebRTC endpoints (EP) with four media components: audio, video and two instant messaging (IM) sessions. IM is MSRP based in this example, which represents a WebRTC data application. The two indicated H.248 contexts (1 and 2) are located in different H.248 MG entities. A H.248 WebRTC gateway application is indicated by the connection model in H.248 context 1:

- audio and video: possibly multiplexed RTP is unmultiplexed (but out of scope of this contribution);

- data: SCTP/DTLS/UDP transport is interworked to TCP transport.

Figure L1.1 indicates the various EP types.

Figure L1.1 – H.248 WebRTC gateway – Interworking and scope on Data Channel 
(multiple IM sessions)

There are a couple of questions related to the outlined interworking model in Figure L1.1. The most important one is related to the SCTP Association endpoint ("STCP Assoc EP a1") at Termination Ta1. The SCTP association contains two SCTP Streams ("StrP a1.3" and "StrP a1.4"), which are terminated at Ta and interworked to the two TCP endpoints in Termination Tc1. The basic question is related to applicable H.248 SEP modeling?

Figure L1.2 indicates one possible solution.

Figure L1.2 – H.248 WebRTC gateway – Example model for H.248 SEPs

Comments:

- Solution A "hierarchical SEPs" (= Figure L1.2): here SEP 255 represents the "tunnel endpoint" (SCTP Association/DTLS/UDP/IP) and SEP 3 and SEP 4 the two bearer connections for WebRTC data (MSRP over SCTP Streams plus data channel control);

- Solution B "one-SEP-per-Data channel": would lead to a bundling based model;

- It should be noted that the outlined example provides a worst case scenario because we got a) multiple WebRTC data channels, which are multiplexed in a single b) L4-over-L4 tunnel.

- However, the question/problem of H.248 SEP modelling remains if we would just consider single WebRTC data channels …

- … keeping in mind the different lifetime and bearer connection control procedures at the various protocol layers:

  - UDP transport connection

  - DTLS session

  - SCTP association

  - SCTP stream and

  - WebRTC data control channel (tbc)

Thus, a distribution of hierarchical (associated) SEPs seems to be beneficial from H.248 control (and MGC) perspective.

- Solution A seems to be similar than H.248 Mux descriptor? (tbc)

## L1.3	Relevant documentation

Figure L1.3 attempts an overview (status: 2013-10):

Figure L1.3 – Landscape of related IETF Documents ("WebRTC data channel")

Appendix L2
Living List – H.248 WebRTC gateway capabilities for 3GPP Rel-12

(This appendix does only contain working material.)

{Editor's note (2014-02 meeting): purpose: this Appendix provides the link to correspondent 3GPP documents (without repeating that information).}

## L2.1	3GPP references

See (status 2014-02):

- 23.701 Annex A

- 23.228 Annex ?

## L2.2	H.248 WebRTC media gateway: media plane interworking scenarios

WebRTC supports three main media components (Figure L2.1): 1) audio including DTMF and 2) video , which are both using SRTP/UDP transport, and 3) data using WebRTC DataChannels (based on SCTP/DTLS/UDP transport). Hence any specific media other than audio and video uses WebRTC DataChannels for transport.

Figure L2.1 – 3GPP Rel-12 – H.248 WebRTC media gateway: media plane interworking scenarios

Bibliography - Placeholder

[b-ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title.

______________________