| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-25 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor |  |  |  |
| Title: | H.248.SCTP "Gateway control protocol: H.248 support for control of SCTP bearer connections" (New): Output draft Ed. 0.4 |  |  |  |
| Purpose: | Output draft |  |  |  |

Summary

This document contains the editor's output draft for new H.248.SCTP "Gateway control protocol: H.248 support for control of SCTP bearer connections" (Ed.0.4)

__________________

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.SCTP Ed. 0.1 | Initial draft (C-260) to Geneva meeting (2013-10) Location: http://www.itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-C-0260 |
| H.248.SCTP Ed. 0.2 | Output (TD-113) from Geneva meeting (2013-10) Location: http://www.itu.int/md/T13-SG16-131028-TD-WP1-0113/en |
| H.248.SCTP Ed. 0.3 | Input (AVD-4568) to Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/AVD-4568.zip |
| H.248.SCTP Ed. 0.4 | Output (TD-25) from Geneva meeting (2014-03) |
| H.248.SCTP Ed. 0.5 |  |
| H.248.SCTP Ed. 0.6 |  |
| H.248.SCTP Ed. 0.7 |  |
| H.248.SCTP Ed. 0.8 |  |
| H.248.SCTP Ed. 0.9 |  |
| H.248.SCTP Ed. 0.10 |  |

CONTENTS

Page

1 Scope 5 1.1 Applicability statements 5 2 References 6 3 Definitions 6 3.1 Terms defined elsewhere 6 3.2 Terms defined in this Recommendation 7 4 Abbreviations and acronyms 7 5 Conventions 8 5.1 Naming of stream endpoints and terminations 8 5.2 Conventions used in signalling flows 8 5.3 SCTP bearer 8 5.4 SCTP bearer control 8 5.5 SCTP endpoint notations 9 6 Use case descriptions 10 6.1 Use cases related to SCTP transport modes 10 6.2 Bearer connection network use cases with H.248 IP-IP gateways 10 6.2.1 High-level use case categories 10 6.2.2 Use case variations 12 6.3 Bearer connection network use cases with H.248 "SCTP to non-SCTP" gateways 12 7 Models 12 7.1 Network model from H.248 entity point of view 12 8 SCTP basic connection control control package(s) 14 8.6 Procedures 14 8.6.x SCTP application data transfer 14 9 Package-less SCTP control 15 I.1 Purpose 15 I.2 SDP codepoints related to SDP "m=" line <proto> element 15 I.3 SDP codepoints related to SDP attributes 16

List of Tables

Page

Table 1 – Principal SCTP-based interfaces of H.248 entities and their relevance for this Recommendation 5 Table 2 – Impact of StreamMode on SCTP bearer traffic at external MG interface 14 Table I.1 – SDP codepoints related to SDP "m=" line <proto> element 15

List of Figures

Page

Figure 1 – Conventions for SCTP bearer control 9 Figure 2 – Conventions for SCTP endpoint types 9 Figure 3 – SCTP transport modes 10 Figure 4 – Bearer connection network use cases with H.248 IP-IP gateways 11 Figure 5 – Network model from H.248 entity point of view ("half call/bearer connection model") 12 Figure 6 – Two-termination context with scope on a single SCTP termination only 13 Figure 7 – Two-termination context with context-level SCTP scope 13

Draft new ITU-T Recommendation H.248.SCTP

Gateway control protocol: 
H.248 support for control of SCTP bearer connections

AAP Summary

<To be provided before Consent>

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

FIXTHIS, at least following items should be addressed by this Recommendation:

- Use case(s)

- Interworking models (MG)

- Control of SCTP endpoints in MG

- Modelling of H.248 terminations / stream endpoints

- H.248 signalling elements

- Relation to existing H.248.x Recommendations

## 1.1	Applicability statements

Table 1 summarizes all possible SCTP-based interfaces of H.248 entities, - under the assumption of an underlying IP network -, and their relevance for this Recommendation.

Table 1 – Principal SCTP-based interfaces of H.248 entities 
and their relevance for this Recommendation

| SCTP-based transport | H.248 entity | Relevance |
| --- | --- | --- |
| Call control interface (e.g., SIP-over-SCTP) | MGC | Out of scope of this Recommendation. |
| Gateway control interface (H.248) | MGC, MG | Out of scope of this Recommendation. Possible H.248 transport modes are indicated by [ITU-T H.248.67]. Usage of a SCTP-based H.248 transport mode would be typically specified by an H.248 profile (as part of clause 6.12 in the profile definition template (see Appendix III in [ITU-T H.248.1])). |
| Bearer interface | MG | Within the scope of this Recommendation. |

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway control protocol: Version 3.

[ITU-T H.248.67]	Recommendation ITU-T H.248.67 (12/2009), Gateway control protocol: Transport mode indication package.

[ITU-T H.248.DTLS]	Draft Recommendation ITU-T H.248.DTLS (XX/2014), Gateway control protocol: H.248 support for control of transport security using DTLS.

[IETF RFC 4960]	IETF RFC 4960 (2007), Stream Control Transmission Protocol.

[IETF RFC 5061]	IETF RFC 5061 (2007), Stream Control Transmission Protocol (SCTP) Dynamic Address Reconfiguration.

[IETF RFC 6525]	IETF RFC 6525 (2012), Stream Control Transmission Protocol (SCTP) Stream Reconfiguration.

{Editor's note (2013-10): following references might be needed

- draft-ietf-mmusic-sctp-sdp

- draft-ietf-tsvwg-sctp-dtls-encaps

- draft-marcon-msrp-over-webrtc-data-channels

- draft-ietf-rtcweb-data-protocol

- H.248.DTLS

- others

}

# 3	Definitions

## 3.1	Terms defined elsewhere

<Normally terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

3.1.1	SCTP association [IETF RFC 4960]:  "A protocol relationship between SCTP endpoints, composed of the two SCTP endpoints and protocol state information including Verification Tags and the currently active set of Transmission Sequence Numbers (TSNs), etc.  An association can be uniquely identified by the transport addresses used by the endpoints in the association. Two SCTP endpoints MUST NOT have more than one SCTP association between them at any given time."

3.1.2	SCTP chunk [IETF RFC 4960]: "A unit of information within an SCTP packet, consisting of a chunk header and chunk-specific content."

3.1.3	SCTP endpoint [IETF RFC 4960]:  "The logical sender/receiver of SCTP packets. On a multi-homed host, an SCTP endpoint is represented to its peers as a combination of a set of eligible destination transport addresses to which SCTP packets can be sent and a set of eligible source transport addresses from which SCTP packets can be received.  All transport addresses used by an SCTP endpoint must use the same port number, but can use multiple IP addresses. A transport address used by an SCTP endpoint must not be used by another SCTP endpoint. In other words, a transport address is unique to an SCTP endpoint."

3.1.4	SCTP packet [IETF RFC 4960]: "The unit of data delivery across the interface between SCTP and the connectionless packet network (e.g., IP).  An SCTP packet includes the common SCTP header, possible SCTP control chunks, and user data encapsulated within SCTP DATA chunks."

3.1.5	SCTP path [IETF RFC 4960]: "The route taken by the SCTP packets sent by one SCTP endpoint to a specific destination transport address of its peer SCTP endpoint.  Sending to different destination transport addresses does not necessarily guarantee getting separate paths."

3.1.6	SCTP stream [IETF RFC 4960]: "A unidirectional logical channel established from one to another associated SCTP endpoint, within which all user messages are delivered in sequence except for those submitted to the unordered delivery service.	
Note: The relationship between stream numbers in opposite directions is strictly a matter of how the applications use them. It is the responsibility of the SCTP user to create and manage these correlations if they are so desired."

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| DTLS | Datagram Transport Layer Security |
| --- | --- |
| IP | Internet Protocol |
| IPv4 | Internet Protocol Version 4 |
| IPv6 | Internet Protocol Version 6 |
| L3 | Layer three |
| L4 | Layer four |
| L4+ | Above layer four |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| SCTP | Stream Control Transmission Protocol |
| SDP | Session Description Protocol |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SIP | Session Initiation Protocol |
| UDP |  |

# 5	Conventions

<Describe any particular notation, style, presentation, etc. used within the Recommendation, if any>

{Editor's note (2014-03): clauses 5.1 & 5.2  provide up-front information because it is planned to submit example signalling flows in a future meeting (which would use that conventions). }

## 5.1	Naming of stream endpoints and terminations

The notations "Tx" and "Tx(Sy)" are used for naming of terminations and stream endpoints (SEP) respectively, with

- "Tx" as TerminationID value;

- "Sy" as StreamID value and

- numerical variables x and y.

## 5.2	Conventions used in signalling flows

The following conventions are used in the example signalling flows:

| SCTP Est.req SCTP Est.ack | Abstracted (SCTP message/procedure independent) representationfor establishment requests/acknowledgements of new SCTP associations. |
| --- | --- |
| SCTP Rel.req SCTP Rel.ack | Abstracted (protocol-independent) representation for release requests/acknowledgements of existing SCTP associations. |

## 5.3	SCTP bearer

The notation of SCTP bearer connection relates to a SCTP association in the network user plane. There might be also a control plane SCTP association (between MGC and MG) in case of a SCTP-based H.248 transport mode [ITU-T H.248.67].

{Editor's note (2014-03): it is yet open whether we need to consider
a) "unmultiplexed SCTP bearers" only (i.e., a 1:1 relation of SCTP Stream to SCTP association from H.248 perspective), or/and
b) "multiplexed SCTP bearers" (i.e., there might be following model: an H.248 "multi-stream-to-termination structure" with "H.248 termination = SCTP association" and "H.248 SEP = SCTP Stream" and multiple SCTP Streams).
Such aspects may impact above convention on "SCTP bearer". }

## 5.4	SCTP bearer control

There are connection control information flows for SCTP bearer connections in the network user plane (e.g., IP media/data/bearer plane) and the control plane (here H.248). Figure  1 outlines the used conventions for the differentiation of both control flows:

Figure 1 – Conventions for SCTP bearer control

## 5.5	SCTP endpoint notations

The notion of endpoint represents different concepts in each network plane (H.248 as control protocol belongs to the network control plane and the MG bearer interface to the network user plane), see Figure 2:

Figure 2 – Conventions for SCTP endpoint types

{Editor's note (2014-03): the concept of "SCTP mode" in above Figure is still not analyzed in detail … }

Usage in

- control plane (here H.248): H.248 terminations/stream endpoint with SCTP processing are denoted as SCTP-enabled termination or stream endpoint (SEP) respectively;

- user plane (SCTP): a SCTP bearer connection endpoint is defined by the SCTP itself, see clause 3.1.3 "SCTP endpoint".

Whether the (H.248) SCTP-enabled SEP provides a "SCTP endpoint" is dependent on the H.248 connection model and enabled MG-internal interworking functions (see clauses 6 and 7).

Editor’s note: the terminology related to endpoints and user vs. control plane may need further analysis and checked for consistency against H.248.1.

# 6	Use case descriptions

Basic use cases may be identified from the perspective of

- H.248 MG type: IP-to-IP gateway (see clause 6.2) or IP-to-non-IP gateway (see clause 6.3) in case of usual two-party communication services; and

- Multiparty service types (see clause 6.4).

## It may be noted that the various network use cases may be abstracted by a single bearer connection model, see clause 7.2.

## 6.1	Use cases related to SCTP transport modes

The SCTP protocol could be used in various transport protocol stack variants. Multiple SCTP transport modes are possible (Figure 3).

Figure 3 – SCTP transport modes

This Recommendation is not dependent on any SCTP transport mode, but focuses on native SCTP/IP as example.

## 6.2	Bearer connection network use cases with H.248 IP-IP gateways

### 6.2.1	High-level use case categories

The notion "IP-to-IP" (briefly "IP-IP") indicates an (IP, IP) connection model (as part of clause 6.4 in the profile definition template (see Appendix III in [ITU-T H.248.1])).

Each H.248 stream endpoint (SEP) may be associated with a non-SCTP or different SCTP protocol stack variants, leading to various bearer network connection use cases (see Figure 4):

Editor’s note: the first version of this Recommendation will not need to provide protocol solutions for all these network interworking scenarios, only for those that are justified. Contributions are solicited.

Editor’s note: in addition to these interworking scenarios, use cases showing concrete examples of application layers that require SCTP transport are needed.

- Use case #1.1: an H.248 IP-IP MG located in the middle of an end-to-end SCTP bearer connection, there are two sub-variants

  - Use case #1.1.1: the end-to-end SCTP bearer connection is divided in two separate SCTP associations ("SCTP proxy mode");

  - Use case #1.1.2: the end-to-end SCTP bearer connection relates to a single SCTP association ("SCTP relay mode");

- Use case #1.2: FIXTHIS;

- Use case #1.3: FIXTHIS;

- Use case #1.4: FIXTHIS;

- Use case #2.1: FIXTHIS;

- Use case #2.2: FIXTHIS;

- Use case #2.3: FIXTHIS;

- Use case #3.1: FIXTHIS;

- Use case #3.2: FIXTHIS;

Figure 4 – Bearer connection network use cases with H.248 IP-IP gateways

All these use cases are basically of type "application-agnostic".

### 6.2.2	Use case variations

#### 6.2.2.1	"SCTP-to-SCTP transparent forwarding"

This scenario is a variation of use case #1.1.2 and requires the MG behaviour of SCTP transparent forwarding.

#### 6.2.2.2	"SCTP-to-SCTP application aware processing"

This scenario is a variation of use case #1.1.1, #2.1 or #3.1. The MG may be required to be application aware in case of   FIXTHIS  .

## 6.3	Bearer connection network use cases with H.248 "SCTP to non-SCTP" gateways

Relates to use cases #1.4, #2.3 and #3.2.

Example: WebRTC data channel interworking as illustrated in Appendix I.3/[ITU-T H.248.DTLS]; all that examples relate to use case #3.2.

# 7	Models

## 7.1	Network model from H.248 entity point of view

The following network model (Figure 5) illustrates the relevant areas covered by this Recommendation. The H.248 MG peers with a SCTP-capable IP host remote endpoint.

Figure 5 – Network model from H.248 entity point of view 
("half call/bearer connection model")

This Recommendation addresses primarily

- signalling capabilities and procedures at the H.248 interface;

- aspects and control of SCTP modes of operation in the H.248 MG; and

- configuration and procedures of the native "SCTP/IP" or tunneled "SCTP/tunnel/IP" protocol stack for H.248-controlled bearers.

## 7.2	Bearer connection model

### 7.2.1	Model for single SCTP termination scope only

Figure 6 details the generic connection-model where a SCTP-enabled termination is connected to a single other termination.

Figure 6 – Two-termination context with scope on a single SCTP termination only

### 7.2.2	Model for context-level SCTP termination scope

Figure 7 represents the general model for use cases with H.248 (SCTP, SCTP) connections models, i.e., two-termination-contexts with both enabled for SCTP. The type of interworking function (IWF) determines the particular MG behaviour concerning SCTP handling.

Figure 7 – Two-termination context with context-level SCTP scope

# 8	SCTP basic connection control control package(s)

|  | Package Name: | SCTP basic connection control package |
| --- | --- | --- |
|  | Package ID: | sctpbcc (0x####) |
|  | Description: | This package provides the functionality to establish and release a SCTP bearer connection. |
|  | Version: | 1 |
|  | Extends: | None |

[…]

## 8.6	Procedures

[…]

### 8.6.x	SCTP application data transfer

#### 8.6.x.1	MG external

The SCTP bearer connection is ready for application data transfer when remote and local SCTP endpoints are both transitioned to SCTP association state "ESTABLISHED" (see Figure 3/[IETF RFC 4960]).

The StreamMode property of the LocalControl Descriptor affects the application data flow rather than the SCTP control information used to establish or close the SCTP association. The requested MG behaviour is defined by Table 2:

Table 2 – Impact of StreamMode on SCTP bearer traffic at external MG interface

| Traffic direction | StreamMode settings | MG behaviour: |
| --- | --- | --- |
| Outgoing | Sending enabled (i.e., values SendOnly, SendRecv, LoopBack) | All SCTP packets sent |
|  | Sending disabled (i.e., values RecvOnly, Inactive) | Only SCTP packets with SCTP control chunks are sent. SCTP packets with SCTP data chunks only are blocked. |
| Incoming | Receiving enabled (i.e., values RecvOnly, SendRecv, LoopBack) | All SCTP packets accepted |
|  | Receiving disabled (i.e., values SendOnly, Inactive) | Only SCTP packets with SCTP control chunks are accepted (optional piggybacked SCTP data chunks would be removed???). SCTP packets with SCTP data chunks only are blocked. |

{Editor's note (2014-03): above table provides initial input for discussion. The situation is slightly different as in case of TCP and DTLS:

- SCTP: multiple SCTP chunks are multiplexed on a single SCTP packet

  - control information: in SCTP header

  - control information: explicit SCTP control chunks (ca. 10 types)

- TCP: control information solely part of TCP header

- TLS: dedicatel sub-protocol for control information; i.e., data relates to TLS messages with application data, and control to TLS negotiation/cipher/alert protocol

We need to consider (at least) following two aspects:

- SCTP control chunks: any types which could be similar handled as "data"?

- draft-ietf-tsvwg-sctp-ndata (http://tools.ietf.org/wg/tsvwg/draft-ietf-tsvwg-sctp-ndata/ )

). }

# 9	Package-less SCTP control

FIXTHIS

Annex A

<Annex Title>

(This annex forms an integral part of this Recommendation.)

<Body of annex A>

Appendix I

SDP for gateway control - IANA considerations

(This appendix does not form an integral part of this Recommendation.)

## I.1	 Purpose

The H.248 gateway control protocol reuses normally the SDP codepoints as defined by the IETF or other organizations. However, gateway control may demand for additional SDP codepoints. The ones idendified by this Recommendation are collected in this clause and are subject of registration with IANA.

## I.2	SDP codepoints related to SDP "m=" line <proto> element

This Recommendation identifies following bearer type indication requirements (Table x1):

Table I.1 – SDP codepoints related to SDP "m=" line <proto> element

| Type |  | SDP name (value) |  | Use case |  | IANA status |
| --- | --- | --- | --- | --- | --- | --- |
| proto | "SCTP" | application agnostic bearer type indication | not yet registered, but already subject of work item "draft-ietf-mmusic-sctp-sdp" |
| proto | "SCTP/DTLS" | application agnostic, transport security protocol specific bearer type indication |  |
| proto | "DTLS/SCTP" | application agnostic, SCTP as tunnel over DTLS |  |
| proto |  | "SCTP/<application>" |  | application aware bearer type indication |  | not yet registered, but subject of application specific IETF RFCs |

There are not any additional SDP codepoints, which need to be registered with IANA.

## I.3	SDP codepoints related to SDP attributes

None.

Bibliography

[b-IETF RFC 4347]	IETF RFC 4347 (2006), Datagram Transport Layer Security.

______________________