| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-31 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.Sup.Term |  |  |  |
| Title: | H.Sup.Term: "ITU-T H.248.x-series – Supplement on common H.248 terminology" (New): Output draft Ed. 0.1 |  |  |  |
| Purpose: | Meeting output (Draft) |  |  |  |

Summary

This document contains the editor's output draft for draft new H.Sup.Term: "ITU-T H.248.x-series – Supplement on common H.248 terminology" (Ed. 0.1).

__________________

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.Sup.Term Ed. 0.1 | Initial draft (AVD-4507) to WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/AVD-4507.zip |
| H.Sup.Term Ed. 0.2 | Output (TD-31) from WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/TD-31.zip |
| H.Sup.Term Ed. 0.3 |  |
| H.Sup.Term Ed. 0.4 |  |
| H.Sup.Term Ed. 0.5 |  |
| H.Sup.Term Ed. 0.6 |  |
| H.Sup.Term Ed. 0.7 |  |
| H.Sup.Term Ed. 0.8 |  |
| H.Sup.Term Ed. 0.9 |  |
| H.Sup.Term Ed. 0.10 |  |

New Supplement # to the ITU-T H-Series (ex H.Supp.Term)

Gateway Control Protocol:
ITU-T H.248.x-series - Supplement on common H.248 terminology

CONTENTS

Page

1 Scope 6 2 References 6 3 Definitions 6 3.1 Terms defined elsewhere 6 3.2 Terms defined in this Supplement 6 4 Abbreviations and acronyms 7 5 Conventions 7 6 Handling of new terms 7 Appendix L Living List [from H.248.78 Rev] – Discussion of terminology related to B2BUA entities 8 L.1 Models – Decomposed gateway and B2BUA entities 8 L.2 Terminology – Proposals for definition 9

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 1 – Positioning of H.SuppTerm versus H.248.x-series of Recommendations 5 Figure L.1 – B2BUA entities and decomposed (H.248) gateway model – H.248 IP-IP gateway embedded in SIP network 8 Figure L.2 – Media plane B2BUA functions as defined by ITU-T (examples) 9

New Supplement # to the ITU-T H-Series (ex H.Supp.Term)

Gateway Control Protocol:
ITU-T H.248.x-series - Supplement on common H.248 terminology

Summary

This Supplement provides a central place for H.248 related terms, that concern multiple H.248.x Recommendations, Supplements or the entire H.248.x-series. This Supplement is a living document.

Keywords

GCP, H.248

Introduction

All ITU-T Recommendations with respect to the ITU-T defined Gateway Control Protocol (GCP) are published within the H.248.x-series of Recommendations. The core protocol itself is defined by [ITU-T H.248.1]. Protocol extensions (in form of so called H.248 packages) and other material (such as profile guidelines) are subject of self contained Recommendations within [ITU-T H.248.x].

Basically, all of these GCP-related Recommendations (plus some Supplements) share the same terminology. The set of GCP terms were historically divided in two classes: general GCP terms and package specific GCP terms. The general terms with global GCP scope are subject of the core protocol specification [ITU-T H.248.1], whereas package specific terms are typically located in the concerned H.248.x-Recommendation (Figure 1).

Figure 1 – Positioning of H.SuppTerm versus H.248.x-series of Recommendations

It has to be noted that a self-contained Recommendation on GCP terminology was unfortunately not produced. In the meanwhile were more and more Recommendations discussed terminology across multiple separate H.248.x-Recommendations. There are two basic issues:

- such terms affect a few H.248 documents, but not the entire series, leading to the question of the right place for such terminology; and

- the addition of new, global H.248 terms would imply a revision of H.248.1, which is a cumbersome activity.

A solution to both problems would be a single document, acting as a central place for such GCP related terminology.

# 1	Scope

In scope of this Supplement are

- GCP related terms only and

- which concern multiple H.248 documents.

Out of scope of this Supplement are

- terms not related to H.248 technology,

- terms which just affect a single H.248.x Recommendation.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.x]	The ITU-T H.248.x-series of Recommendations (Gateway Control Protocol).

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

Basic reference are the GCP terms according to clause 3.2/[ITU-T H.248.1].

## 3.2	Terms defined in this Supplement

NOTE - The terms in this clause are not in the usual alpabetical order, rather chronologically listed.

This Supplement defines the following terms:

3.2.1	stream endpoint tuple (SEPT): the generalization of SEPP towards multiple associated Stream Endpoints (SEP) within the same Context. All SEPs share thus the same StreamID value. The stream topology is given by the topology descriptor settings.

{Editor’s note (2014-03 meeting): further candidates are e.g.

3.2.1	Basic H.248 master-slave control mode: An H.248 decomposed gateway following a hierarchical control model in the network control plane (primarily related to call service control and media/bearer control functions). The control hierarchy constitutes a master-slave relationship. With MGC as master and MG as slave entity due to considered distribution models on the decomposed gateway entities

NOTE 1 - [b-ITU-T Q.Sup31] Annexes C.2 or C.3 provides examples of functional-to-physical mapping models according basic H.248 master-slave control mode.

3.2.2	MG autonomous mode: For a particular function, the MGC delegates some (but not all) service specific control decisions down to the MG level. The MG provides a local decision function. The MG autonomous control is limited to decisions with MG available information only.

3.2.3	MGC-strictly controlled mode: Synonym to the basic H.248 master-slave control mode. All control decisions are exclusively under MGC responsibility, the MG degenerates to a pure execution unit.

.}

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

GCP	Gateway Control Protocol

# 5	Conventions

None

# 6	Handling of new terms

A new H.248.x-Recommendation under development identifies new terminology. Such terms are normally subject of clause 3.2 according to the Recommendation template. It has to be decided whether a new term is only limited to this Recommendation, or whether multiple terms could benefit from this new term. The term should be added to this Supplement in the second case. The new H.248.x-Recommendation may then refer these terms within clause 3.1 "Terms defined elsewhere".

Appendix L

Living List [from H.248.78 Rev] – 
Discussion of terminology related to B2BUA entities

{Editor’s note (2013-10 meeting): discussion triggered by C-247 (http://www.itu.int/md/T13-SG16-C-0247/en ). This living list contains the discussion and proposals for B2BUA related terms.
Any terminology work should be aligned with IETF (such as WG STRAW).
The terminology could be finally moved to to e.g. H.248.79, H.248.1 or a Supplement.}

## L.1	Models – Decomposed gateway and B2BUA entities

Figure L.1 illustrates an H.248 gateway embedded in an IP network with SIP as application control signalling protocol:

Figure L.1 – B2BUA entities and decomposed (H.248) gateway model 
– H.248 IP-IP gateway embedded in SIP network

Two back-to-back user agent (B2BUA) entities could be identified, tightly coupled to the SIP-based IP signalling path and the IP bearer path for media traffic:

- SIP B2BUA and

- Media plane B2BUA.

The SIP B2BUA is, strictly speaken, on top (and thus out of scope) of the H.248 MGC function (see Fig. L.1).

NOTE 1 – However, in many network (reference) models are both functions coupled ("melted"), due to various "session border controller/gateway) concepts, integrated gateway models, etc.

The Media plane B2BUA exists in many H.248.x applications, see some examples in Figure L.2:

Figure L.2 – Media plane B2BUA functions as defined by ITU-T (examples)

Issue: there is not yet any explicit terminology available for B2BUA entities.

## L.2	Terminology – Proposals for definition

Proceeding:

- It is expected that the IETF will finally provide a definition for "SIP B2BUA", as part of [b-IETF draft-ietf-straw-b2bua-taxonomy]

- The terms for both B2BUA entities needs to be aligned.

- Thus, a hierachical terminology is required.

Proposals:

…

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	back-to-back IP host (B2BIH) mode (clause 3.2.1/[ITU-T H.248.64]): A context of two or more IP-based terminations, where each termination appears to the rest of the network as an IP host (in contrast to an IP router).

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.x1	B2BUA (back-to-back user agent):: provides a service interworking function (according clause 3.2/[ITU-T Y.1251]) with following limitations,

- network planes: only IP control and IP user plane;

- protocol stack: same protocol at each side of the service interworking function.

NOTE –  hence, a B2BUA is not a proxy.

3.2.x2	Proxy (derived from [ITU-T Y.2902]): a system authorized to work on behalf of another system including responding to protocol requests.

NOTE 1 – hence, a proxy does not provide any interworking function (neither service interworking nor network interworking).

NOTE 2 – proxy as network element is an intermediary node in the sense of identical protocol stacks at incoming and outgoing interfaces of the proxy.

3.2.x3	SIP B2BUA: provides a B2BUA function for the SIP, i.e., a service interworking function (according clause 3.2/[ITU-T Y.1251]) in the IP control plane.

NOTE – the service interworking function may include as well the interworking of the protocol stack for SIP transport (e.g., SIP-over-UDP/IP4 to SIP-over-SCTP/IPv6 interworking).

3.2.x3	Media plane B2BUA: provides a B2BUA function for media traffic (NOTE), i.e., a service interworking function (according clause 3.2/[ITU-T Y.1251]) in the IP user plane.

NOTE – Term "media" is used as a placeholder for IP application data traffic, which covers the IP application protocol itself plus eventually subjacent protocol layers. The IP media path (plane) is also known as IP bearer path (plane) or IP data path (plane).

3.2.x3	B2BIH (back-to-back IP host): provides a media plane B2BUA function with primary scope on IP protocol layer(s) L3 (and possibly L4) only, despite the fact that an IP host as such is terminating the entire protocol stack.

NOTE – Example: the B2BIH mode (according [ITU-T H.248.64]) defines a specific media plane B2BUA function with scope on mode of operations as relevant for IP router/gateways.

____________________