| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-32 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.Sup.IANA |  |  |  |
| Title: | H.Sup.IANA: "SDP codepoints for gateway control – Release 1" (New): Output draft Ed. 0.1 |  |  |  |
| Purpose: | Meeting output (Draft) |  |  |  |

Summary

This document contains the editor's output draft for draft new H.Sup.IANA: "SDP codepoints for gateway control – Release 1" (Ed. 0.1).

__________________

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.SUPP.IANA Ed. 0.1 | Initial draft (AVD-4570) to WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/AVD-4570.zip |
| H.SUPP.IANA Ed. 0.2 | Output (TD-32) from WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/TD-32.zip |
| H.SUPP.IANA Ed. 0.3 |  |
| H.SUPP.IANA Ed. 0.4 |  |

New Supplement # to the ITU-T H-Series (ex H.Supp.IANA)

Gateway Control Protocol:
ITU-T H.248.x-series - SDP codepoints for gateway control 
– Release 1

Summary

Gateway control protocols may require additional SDP codepoints, - beyond e.g. SDP information used by call control protocols -, for specific interworking functions supported at bearer plane level. Supplement X to ITU-T H-series Recommendations summarizes SDP codepoints that have been identified in the time-frame from June 2000 to July 2014. It identifies SDP codepoints that meet ITU-T H.248.x sub-series requirements for gateway control protocols and are for general use by the wider standards community. This Supplement – Release 1 covers: • ###. This Supplement is used as base input for the IETF driven registration process of SDP codepoints with IANA.

History Edition; Recommendation; Approval; Study Group. 1.0; ITU-T H Suppl. X; 2014-X-X; 16.

CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 5 4 Abbreviations and acronyms 5 5 Conventions 5 6 Registration process 5 7 SDP codepoints for IANA registration 7 7.1 SDP codepoints related to wildcarding 7 7.2 SDP codepoints related to SDP "m=" line <proto> element 7 7.3 SDP codepoints related to SDP "a=" lines 7

List of Tables

Page

Table 1 – SDP codepoints related to wildcarding 7 Table 2 – SDP codepoints related to SDP "m=" line <proto> element 7

List of Figures

Page

Figure 1 – IANA SDP registry – Process for H.248-specific SDP registrations 6

New Supplement # to the ITU-T H-Series (ex H.Supp.IANA)

Gateway Control Protocol:
ITU-T H.248.x-series - SDP codepoints for gateway control 
– Release 1

# 1	Scope

Gateway control protocols (such as ITU-T H.248 [ITU-T H.248.1]) may require additional SDP codepoints, - beyond e.g. SDP information used by call control protocols -, for specific interworking functions supported at bearer plane level.

Supplement X to ITU-T H-series Recommendations [ITU-T H.248.x] summarizes SDP codepoints that have been identified in the time-frame from June 2000 to July 2014. It identifies SDP codepoints that meet ITU-T H.248.x sub-series requirements for gateway control protocols and are for general use by the wider standards community.

This Supplement – Release 1 covers:

•	###.

This Supplement is used as base input for the IETF driven registration process of SDP codepoints with IANA.

# 2	References

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.x]	The ITU-T H.248.x-series of Recommendations (Gateway Control Protocol).

[IETF IANA SDP]	IETF draft-mmusic-sdp-for-gw (03/2014), SDP codepoints for gateway control.

See clauses below for individual references.

# 3	Definitions

None.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

SDP	Session Description Protocol

RFC	(IETF) Request for Comments

# 5	Conventions

None

# 6	Registration process

IANA registrations should be fundamentally based on IETF Standards-Track RFCs, primarily in case of technologies owned by the IETF, such as SDP. The ITU-T could, but should not bypass the IETF in case of SDP related registrations with IANA. The ultimate goal of unambiguous protocol semantics implies a single instance, which reviews and coordinates registration requests with IANA. Figure 1 summarizes the principle process followed by ITU-T Study Group 16 with respect to IANA registration requests originated by ITU-T H.248 protocol:

Figure 1 – IANA SDP registry – Process for H.248-specific SDP registrations

The process in more detail:

- ITU-T: H.248 protocol specifications reuse existing, IANA-registered SDP codepoints, but can identify new SDP codepoints in addition, as e.g. subject of dedicated use cases.
Such extensions to the H.248 gateway control protocol are normally related to new H.248 package definitions, which are published as part of the H.248.x-series of ITU-T Recommendations. However, new SDP codepoints could be also derived as the result of (package-less) H.248 procedures or specific SDP usage according to an H.248 profile.

- ITU-T: This Supplement is used as a central place for 
a) the documentation of already registered H.248-specific SDP codepoints with IANA, and
b) the collection of newly identified codepoints.

- IETF: working group MMUSIC is presently responsible for the SDP. The content of the ITU-T Supplement is mapped on an IETF draft [IETF IANA SDP] (which later becomes an RFC), which becomes the baseline for the IANA registration request.

- IANA: registration on request by IETF.

The process is not a singular activity, rather open for recurrent updates because

- ITU-T Supplements support "Release" versions, and

- IETF RFCs could be replaced (obsoleted) by RFC revisions.

{Editor's note (2014-03): the outlined process is actually fairly similar with H.Supp2, which documents the registration of H.248 packages with IANA. New releases of this Supplement are continuously produced as long as new H.248 packages are developed. Of course, there's a major difference: 1st The ITU-T is technology owner of H.248 and the assigned central instance for the coordinated development of H.248 packages, and 2nd H.248 package identifiers are NOT used by IETF work products, thus, the IETF is allowed to bypass the IETF concerning the registration of such IANA codepoints. However in case of SDP, the situation is quite different … }

# 7	SDP codepoints for IANA registration

# {Editor’s note: Further code points need to be documented e.g. [ITU-T H.248.15].}

## 7.1	SDP codepoints related to wildcarding

The H.248-specific SDP wildcard "-" (see clause 5.2.2.2/[ITU-T H.248.39]) could be used in various SDP lines and line fields.

Table 1 – SDP codepoints related to wildcarding

| Type | SDP name (value) | Reference |
| --- | --- | --- |
| various SDP field elements | "-" | [ITU-T H.248.39] |

Such a wildcard represents a codepoint space, but is out of scope of any registry with IANA.

NOTE – The wildcard character is compliant with the SDP grammar according to [IETF RFC 4566]. If, then a future SDP RFC could explicitly provide non-normative information about the usage of value "-".

## 7.2	SDP codepoints related to SDP "m=" line <proto> element

See Table 2:

Table 2 – SDP codepoints related to SDP "m=" line <proto> element

| Type | SDP name (value) | Reference |
| --- | --- | --- |
| proto | "TLS" | [ITU-T H.248.TLS] |
| proto | "TCP/TLS" "SCTP/TLS" | [ITU-T H.248.TLS] |
| proto | "DTLS" | [ITU-T H.248.DTLS] |
| proto | "UDP/DTLS" "DCCP/DTLS" | [ITU-T H.248.DTLS] |
| proto | "SCTP" | [ITU-T H.248.SCTP] NOTE 1 |
| proto | "SCTP/DTLS" | [ITU-T H.248.SCTP] NOTE 1 |
| proto | "DTLS/SCTP" | [ITU-T H.248.SCTP] NOTE 1 |
| NOTE 1 – Table entry may be deleted again when IETF draft "draft-ietf-mmusic-sctp-sdp" becomes an RFC. |  |  |

## 7.3	SDP codepoints related to SDP "a=" lines

None.

{Editor’s note: An Appendix will be added in the future documenting the IETF drafts registering SDP codepoints, i.e. [IETF IANA SDP].}

____________________