| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 213 (PLEN/16) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Sapporo, Japan, 30 June – 11 July 2014 |
| TD |  |  |
| Source: | Editor H.248.39 |  |
| Title: | Consent: H.248.39 "H.248 SDP parameter identification and wildcarding” (Rev.) |  |

This TD contains draft revised H.248.39 "H.248 SDP parameter identification and wildcarding”, which is proposed for Consent at this SG16 meeting.

AAP Summary

Revised Recommendation ITU-T H.248.39 contains a new package allowing advanced wildcarding of the Session Description Protocol (SDP) when used in H.248. The new wildcards allow: multiple value response, multiple line response, wildcard inter-linkage and the maintenance of previously set values. The new wildcards reduce the size signalling messages between a media gateway controller and a media gateway. Guidelines for wildcarding additional SDP attributes are also included.

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.39 Rev. Ed. 0.1 | Initial draft due to C.49 from the Geneva January 2013 meeting. |
| H.248.39 Rev. Ed. 0.2 | Input to the June 2013 meeting based on TD 48(WP1/16). |
| H.248.39 Rev. Ed. 0.3 | Output from the June 2013 meeting. Changes as a result of: AVD-4370, AVD-4371, AVD-4372 and AVD-4463. |
| H.248.39 Rev. Ed. 0.4 | Input to the October 2013 meeting based on TD-21 from the June 2013 meeting. |
| H.248.39 Rev. Ed. 0.5 | Output from the October 2013 meeting based. Changes due to: C.245. |
| H.248.39 Rev. Ed. 0.6 | Input text to the Geneva, 10 – 14 March 2014 meeting. |
| H.248.39 Rev. Ed. 0.7 | Output text from the Geneva, 10 – 14 March 2014 meeting. Changes due to: AVD-4504 and AVD-4538. |
| H.248.39 Rev. Ed. 0.8 | Input text to the Sapporo, 30 June-11 July 2014 meeting. Location: http://www.itu.int/md/T13-SG16-140630-TD-WP1-0158en |
| H.248.39 Rev. Ed. 0.9 | Output text from the Sapporo, 30 June – 14 July 2014 meeting. Changes due to: C.521r1. C.456r1 and C.457r1. |

CONTENTS

Page

1 Scope 5 2 References 5 3 Terms and Definitions 6 4 Abbreviations 6 5 H.248 wildcarding guiding principles 7 5.1 General wildcarding principles by H.248 7 5.2 Specific wildcarding principles for H.248 embedded SDP 7 5.2.1 Background: IETF SDP 7 5.2.2 H.248-introduced SDP wildcards 7 6 Identification of single parameters 7 6.1 Protocol version 8 6.2 Owner/Creator 8 6.3 Session name 9 6.4 URI of description 9 6.5 Email address 9 6.6 Phone number 10 6.7 Encryption key 10 6.8 Time description 11 6.9 Repeat 11 6.10 Time zone adjustments 12 6.11 Media description 13 6.12 Media title and session information 13 6.13 Connection information 14 6.14 Bandwidth information 14 6.15 Attributes 15 6.15.1 RTP map 16 6.15.2 Packetization time 16 6.15.3 Parameter format 17 6.15.4 Path 17 6.15.5 SDP H.248 package attribute 18 6.15.6 RTCP attribute 18 6.15.7 Silence suppression attribute 19 6.15.8 Media description with multiple payload types and multiple attribute lines 19 6.15.9 Source-specific media attributes 20 6.15.10 Fingerprint attribute 24 7 Use for Wildcarding (CHOOSE or ALL) 25 8 Use in auditing 25 8.1 Audit value 25 8.2 Audit capability 26 9 Advanced SDP Wildcarding Package 26 9.1 Properties 27 9.1.1 Advanced Wildcarding Support 27 9.2 Events 27 9.3 Signals 27 9.4 Statistics 27 9.5 Error Codes 27 9.6 Procedures 27 9.6.1 General 27 9.6.2 Wildcarded Value Response 28 9.6.3 Wildcarded Line Response 28 9.6.4 Wildcarding inter-linkage 29 9.6.5 Maintain Value 29 9.6.6 Interactions 30 Appendix I Example signalling scenarios for the maintain value wildcard "~" 31 I.1 Purpose 31 I.2 Signalling without the maintain value wildcard "~" 31 I.3 Signalling using the maintain value wildcard "~" 34 I.4 Evaluation of example 36 Bibliography 37

List of Tables

Page

Table 6-1/H.248.39 – Protocol version 9 Table 6-2/H.248.39 – Owner/Creator 9 Table 6-3/H.248.39 – Session name 10 Table 6-4/H.248.39 – URI of Description 10 Table 6-5/H.248.39 – Email address 10 Table 6-6/H.248.39 – Phone number 11 Table 6-7/H.248.39 – Encryption key 11 Table 6-8/H.248.39 – Time description 12 Table 6-9/H.248.39 – Repeat 12 Table 6-10/H.248.39 – Time zone adjustments 13 Table 6-11/H.248.39 – Media description 14 Table 6-12/H.248.39 – Media title and session information 14 Table 6-13/H.248.39 – Connection information 15 Table 6-14/H.248.39 – Bandwidth information 15 Table 6-15/H.248.39 – Attributes 16 Table 6-15.1/H.248.39 – RTP map 17 Table 6-15.2/H.248.39 – Packetization time 17 Table 6-15.3/H.248.39 – Parameter format 18 Table 6-15.4/H.248.39 – Path 18 Table 6-15.5/H.248.39 – SDP H.248 package attribute 19 Table 6-15.6/H.248.39 – RTCP attribute 19 Table 6-15.7/H.248.39 – Silence suppression attribute 20 Table 6-15.8/H.248.39 – Examples 21 Table 6-15.9.1.1/H.248.39 –Media attribute “ssrc” 21 Table 6-15.9.1.2/H.248.39 – Source-specific attribute “cname” 22 Table 6-15.9.1.3/H.248.39 – Source-specific attribute “previous-ssrc” 22 Table 6-15.9.1.4/H.248.39 – Source-specific attribute “fmtp” 23 Table 6-15.9.2/H.248.39 –Media attribute “ssrc-group” 23 Table 6-15.10/H.248.39 – Attribute “fingerprint” 24 Table 6-15.10.1/H.248.39 – Examples for attribute “fingerprint” 25 Table 7/H.248.39 – Examples 25 Table 8-1/H.248.39 – Examples 26 Table I.1 – Example command encoding – MGC Add request 32 Table I.2 – Example command encoding– MG Add reply 32 Table I.3 – Example command encoding – MGC Modify request 33 Table I.4 – Example command encoding– MG Modify reply 33 Table I.5 – Example command encoding – MGC Add request 34 Table I.6 – Example command encoding– MG Add reply 34 Table I.7 – Example command encoding – MGC Modify request 35 Table I.8 – Example command encoding– MG Modify reply 35

Draft revised ITU-T Recommendation H.248.39

Gateway control protocol: 
H.248 SDP parameter identification and wildcarding

Summary

This Recommendation describes the principles used to identify a single SDP sub-field and how to apply wildcarding to that sub-field. The sub-field may be fully specified, wildcarded (CHOOSE "$" or ALL "*") or not significant "-". Being able to identify a single sub-field is of use when requesting the MG to choose a parameter, specifying a range or for the purposes of auditing. Further specification of how SDP can be wildcarded will assist in interoperation between a Media Gateway Controller and a Media Gateway.

This revision incorporates guidelines for the wildcarding of source-specific media attributes. It also includes a package to indicate MG support for advanced wildcarding procedures that allow a MGC to request multiple SDP parameter values and/or multiple SDP lines to be returned in response to a CHOOSE request. It also allows the linking of multiple CHOOSE wildcards so that they return the same value. Furthermore a new maintain value wildcard is introduced to allow the retention of a previously set value to avoid repetition of redundant information.

# 1	Scope

This Recommendation describes the principles to be used to identify a single SDP sub-field and how to apply wildcarding to that sub-field. The sub-field may be fully specified, wildcarded (CHOOSE "$" or ALL "*") or not significant "-" (see clause 5.2.2.2). Being able to identify a single sub-field is of use when requesting the MG to choose a parameter, specifying a range or for the purposes of auditing.

This Recommendation contains a package that allows a MGC to request multiple SDP parameter values and/or multiple SDP lines to be returned in response to a CHOOSE request. It also allows the linking of multiple CHOOSE wildcards so that they return the same value. A new “maintain value” wildcard "~" is introduced. It is a short notation syntax (versus fully specified SDP information) that allows performance optimization as well as the reduction of syntax errors (due to avoidance of the repetition of redundant information).

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway control protocol: Version 3.

[ITU-T H.248.15]	Recommendation ITU-T H.248.15 (03/2013), Gateway control protocol: SDP H.248 package attribute.

# 3	Terms and Definitions

None.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

| ABNF | Augmented Backus–Naur Form |
| --- | --- |
| ATM | Asynchronous Transfer Mode |
| DTLS | Datagram Transport Layer Security |
| FFS | For further study |
| IP | Internet Protocol |
| LD | Local Descriptor |
| MGC | Media Gateway Controller |
| MG | Media Gateway |
| MIME | Multipurpose Internet Mail Extensions |
| RD | Remote Descriptor |
| RTP | Real-time Transport Protocol |
| SDP | Session Description Protocol |
| SSRC | Sending Source |
| TLS | Transport Layer Security |
| UDP | User Datagram Protocol |
| VoATM | Voice over ATM |
| VoIP | Voice over IP |

# 5	H.248 wildcarding guiding principles

## 5.1	General H.248 wildcarding principles

See clause 6.3/[ITU-T H.248.1]: The overriding guidance provided by [ITU-T H.248.1] on the use of CHOOSE in SDP is: "the use of CHOOSE is allowed in place of a single parameter value".

Subsequent principles are:

1)	In any SDP scheme each mandatory element SHALL therefore be represented by a single CHOOSE "$".

2)	Any part that contain text tokens (e.g. those in " ") that are mandatory SHALL also be included in the wildcard command.

3)	Optional parts of SDP scheme should not be included unless the wildcard or value applies to them.

4)	When an item specifies a wildcard for a single sub-field, this applies to the nested ABNF in that parameter.

## 5.2	Specific wildcarding principles for H.248 embedded SDP

## 5.2.1	IETF SDP wildcards

The core SDP [b-IETF RFC 2327], as well as its successor [b-IETF RFC 4566], does not specify any wildcards. SDP lines are therefore always fully specified.

[b-ITU-T H.248.49] provides, inter alia, complementary information concerning relation and usage of H.248 and the various IETF RFCs on SDP.

### 5.2.2	SDP wildcards introduced by specifications other than H.248.1

Wildcards are used (by the MGC) for resource management or/and the indication of particular modes of operation at MG bearer level (aware versus agnostic). The scope of SDP wildcards in H.248 messages is therefore limited to SDP media descriptions (see clause 7.1.8.1.1.2/[ITU-T H.248.1]), within the LD and RD.

#### 5.2.2.1	Wildcard “-”

Wildcard “-”, with the semantic of “not significant” (or “agnostic”, “unaware”), was introduced (in 2005) by the so-called “H.248 Ia profile version 1” (see clause 5.15/[b-ETSI ES 283 018]), motivated by the fact for the control of dedicated “IP-to-IP interworking modes” in border gateways (see clause G.2.2/[b-ETSI TR 183 068]).

#### 5.2.2.2	Wildcards “r”, “l” and “[0-F]”

This Recommendation introduces the wildcarded value response “r”, wildcarded line response “l”, wildcarding inter-linkage “[0-F]” and the maintain value “~” wildcards via the “Advanced SDP Wildcarding Package” (clause 9).

# 6	Identification of single parameters

The subclauses below define which parts of SDP are classified as "single sub-fields (values)" for the purposes of H.248.1 wildcarding. Each of these sub-fields may be fully specified, designated as CHOOSE "$", designated as ALL "*" or designated as not significant "-". The sub-fields should not be partially wildcarded.

Each of the examples indicates what type of wildcarding is not valid. It also highlights some valid types of wildcarding and what should be returned.

NOTE 1 – For the purposes of the examples below, "?" is used to denote the potential wildcards:

•	"$" 		for Choose

•	"*" 		for All, and

•	"–" 		"Not Significant" (NOTE 2)

NOTE 2 – Not to be confused with Annex B.2/H.248.1 encoding for NULL Context (see also clause 5.2.2.2).

ABNF grammar in subsequent clauses is copied from Appendix A (SDP Grammar) in [b-IETF RFC 2327] and other IETF RFCs concerning SDP extensions.

NOTE - Whilst [b-IETF RFC 2327] is replaced by clause 9/[b-IETF RFC 4566]) references to [b-IETF RFC 2327] have been used as [ITU-T H.248.1] specifies the use of this RFC for SDP.

## 6.1	Protocol version

Table 6-1/H.248.39 – Protocol version

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | v= (protocol version) |  |
| ABNF: (Appendix A/[b-IETF RFC 2327]) | proto-version = "v=" 1*DIGIT CRLF |  |
| H.248/SDP encoding example with applied wildcarding: |  | Result: |  |
| v=? |  | Is valid. It would give v= 1*DIGIT The only parameter that may be wildcarded is 1*DIGIT. |  |

## 6.2	Owner/Creator

Table 6-2/H.248.39 – Owner/Creator

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | o= (owner/creator and session identifier) |  |
| ABNF: (Appendix A/[b-IETF RFC 2327]) | origin-field = "o=" username space sess-id space sess-version space nettype space addrtype space addr CRLF |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| o=? o=? ? o=? ? ? o=? ? ? ? o=? ? ? ? ? |  | Are all invalid. There are six mandatory parameters "username", "sess-id", "sess-version", "nettype", "addrtype" and "addr". |  |
| o=? ? ? ? ? ? |  | Would return: o=username sess-id sess-version nettype addrtype addr |  |

## 6.3	Session name

Table 6-3/H.248.39 – Session name

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | s= (session name) |  |
| ABNF: (Appendix A/[b-IETF RFC 2327]) | session-name-field = "s=" text CRLF |  |
| H.248/SDP encoding example with applied wildcarding: |  | Result: |  |
| s=? |  | Is valid. It would give s=text The only parameter that may be wildcarded is "text". |  |

## 6.4	URI of description

Table 6-4/H.248.39 – URI of Description

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | u=* (URI of description) |  |
| ABNF: (Appendix A/‌[b-IETF RFC 2327]) | uri-field = ["u=" uri CRLF] uri= ;defined in RFC 1630 |  |
| H.248/SDP encoding example with applied wildcarding: |  | Result: |  |
| FFS. |  | – |  |

## 6.5	Email address

Table 6-5/H.248.39 – Email address

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | e=* (email address) |  |
| ABNF: (Appendix A/‌[b-IETF RFC 2327]) | email-fields = *("e=" email-address CRLF) email-address = email ¦ email "(" email-safe ")" ¦ email-safe "<" email ">" email = ;defined in RFC 822 |  |
| H.248/SDP encoding example with applied wildcarding: |  | Result: |  |
| FFS. |  | – |  |

## 6.6	Phone number

Table 6-6/H.248.39 – Phone number

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | p=* (phone number) |  |
| ABNF: (Appendix A/‌[b-IETF RFC 2327]) | phone-fields = *("p=" phone-number CRLF) phone-number = phone ¦ phone "(" email-safe ")" ¦ email-safe "<" phone ">" phone = "+" POS-DIGIT 1*(space ¦ "-" ¦ DIGIT) |  |
| H.248/SDP encoding example with applied wildcarding: |  | Result: |  |
| FFS. |  | – |  |

## 6.7	Encryption key

Table 6-7/H.248.39 – Encryption key

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | k=* (encryption key) |  |
| ABNF: (Appendix A/‌[b-IETF RFC 2327]) | key-field = ["k=" key-type CRLF] key-type = "prompt" ¦ "clear:" key-data ¦ "base64:" key-data ¦ "uri:" uri key-data = email-safe ¦ "~" ¦ " There is one mandatory field "key-type". |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| k=? |  | Would give k=key-type |  |
| k=?:? |  | Would give k=key-type:key-data where key-type is "clear" or "base64", k=key-type:uri where key-type is "uri". |  |

## 6.8	Time description

Table 6-8/H.248.39 – Time description

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/‌[b-IETF RFC 2327]) | t= (time the session is active) |  |
| ABNF: (Appendix A/‌[b-IETF RFC 2327]) | time-fields = 1*( "t=" start-time space stop-time *(CRLF repeat-fields) CRLF) [zone-adjustments CRLF] |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| t=? |  | Is invalid. There are two mandatory parameters "start-time" and "stop-time". |  |
| t=? ? |  | Would give t=start-time stop-time |  |
| NOTE – For repeat-fields and zone-adjustments see below. |  |  |  |

## 6.9	Repeat

Table 6-9/H.248.39 – Repeat

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/‌[b-IETF RFC 2327]) | r=* (zero or more repeat times) |  |
| ABNF: (Appendix A/‌[b-IETF RFC 2327]) | repeat-fields = "r=" repeat-interval space typed-time 1*(space typed-time) |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| r=? |  | Is invalid. There are two mandatory parameters "repeat-interval" and "typed-time". |  |
| r=?:? |  | Would give r=repeat-interval typed-time |  |
| r=? ? ? |  | Would give r=repeat-interval typed-time typed-time |  |

## 6.10	Time zone adjustments

Table 6-10/H.248.39 – Time zone adjustments

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | z=* (time zone adjustments) |  |
| ABNF: (Appendix A/[‌b-IETF RFC 2327]) | zone-adjustments = time space ["-"] typed-time *(space time space ["-"] typed-time) |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| z=? z=? - z=-? z=? ? ? |  | Are invalid. There are two mandatory parameters "time" and "typed-time". These two parameters can be grouped multiple times thus an odd number of wildcards are invalid. |  |
| z=? ? |  | Would give z=time typed-time |  |
| z=? -? |  | Would give z=time –typed-time |  |
| z=? ? ? ? |  | Would give z=time typed-time time typed-time |  |

## 6.11	Media description

Table 6-11/H.248.39 – Media description

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | m= (media name and transport address) |  |
| ABNF: (Appendix A/[‌b-IETF RFC 2327]) | media-field = "m=" media space port ["/" integer] space proto 1*(space fmt) CRLF |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| m=? m=? ? m=? ?/? m=? /? m=? /? ? |  | Are invalid. There are four mandatory parameters "media", "port", "proto" and "fmt" and one optional parameter "integer" that may be wildcarded. |  |
| m=? ? ? ? |  | Would give m=media port proto fmt |  |
| m=? ?/? ? ? |  | Would give m=media port/integer proto fmt |  |
| m=? ? ? ? ? ? |  | Would give m=media port proto fmt fmt This example is considering a "m=" line with multiple, here two, payload formats. See also clause 6.15.7 |  |
| m=? ?/? ? ? ? ? |  | Would give m=media port/integer proto fmt fmt This example is considering a "m=" line with multiple, here two, payload formats. See also clause 6.15.7 |  |

## 6.12	Media title and session information

Table 6-12/H.248.39 – Media title and session information

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | i=* (media title) |  |
| ABNF: (Appendix A/[‌b-IETF RFC 2327]) | information-field = ["i=" text CRLF] |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| i=? |  | Is valid. The only parameter that may be wildcarded is "text". |  |
| i=? |  | Would give z i=text |  |

## 6.13	Connection information

Table 6-13/H.248.39 – Connection information

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | c=* (connection information - optional if included at session-level) |  |
| ABNF: (Appendix A/[‌b-IETF RFC 2327]) | connection-field = ["c=" nettype space addrtype space connection-address CRLF] |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| c=? c=? ? |  | Are invalid. There are three mandatory parameters "nettype", "addrtype" and "connection-address" which may be wildcarded. |  |
| c=? ? ? |  | Would give c=nettype addrtype connection-address |  |

## 6.14	Bandwidth information

Table 6-14/H.248.39 – Bandwidth information

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | b=* (bandwidth information) |  |
| ABNF: (Appendix A/[‌b-IETF RFC 2327]) | bandwidth-fields = *("b=" bwtype ":" bandwidth CRLF) |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| b=? |  | Is invalid as ? is only for one parameter value. There are two mandatory values "bwtype" and "bandwidth" which may be wildcarded. |  |
| b=?:? |  | Would give b=bwtype:bandwidth |  |

## 6.15	Attributes

Table 6-15/H.248.39 – Attributes

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | a=* (zero or more media attribute lines) |  |
| ABNF: (Appendix A/[b-IETF RFC 2327]) | attribute-fields = *("a=" attribute CRLF) attribute = (att-field ":" att-value) ¦ att-field |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=?: a=? ? |  | Is invalid. There is one mandatory parameter "att-field" and one optional parameter "att-value". |  |
| a=? |  | Would give a=att-field This will only wildcard a single "a=value", e.g. "a=recvonly" or "a=sendrecv" or "a=sendonly". |  |
| a=?:? |  | Would give a=att-field:att-value Depending on the value of the first parameter it may not be valid to wildcard the second value. The "att-value" field shall return all the mandatory parameters and may return the optional parameters associated with attribute type. |  |
| a=rtpmap:? |  | Should at least give: a=rtpmap:payload encodingname/clockrate and may give: a=rtpmap:payload encodingname/clockrate encodingparameters |  |

Where multiple attributes of the same type are sent by the MGC, it should fully specify at least one of the parameters to uniquely identify the instance of the attribute. For example: the payload type in the rtpmap attribute.

For a description of wildcarding various "att-value" parameters, see the clauses below.

### 6.15.1	RTP map

Table 6-15.1/H.248.39 – RTP map

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | – |  |
| ABNF: (Appendix A/[b-IETF RFC 2327]) | a=rtpmap:<payload type> <encoding name>/<clock rate>[/<encodingparameters>] |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=rtpmap:? ? a=rtpmap:? ?/ a=rtpmap:? ? ? a=rtpmap:? ?//? |  | Are invalid. There are three mandatory parameters "payload type", "encoding name", "clock rate" and one optional parameter "encodingparameters". |  |
| a=rtpmap:? ?/? |  | Would give a=rtpmap:payloadtype encodingname/clockrate |  |
| a=rtpmap:? ?/?/? |  | Would give a=rtpmap:payloadtype encodingname/clockrate/encoding parameters |  |

### 6.15.2	Packetization time

Table 6-15.2/H.248.39 – Packetization time

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | – |  |
| ABNF: (Appendix A/[b-IETF RFC 2327]) | a=ptime:<packet time> There is one mandatory parameter "packet time". |  |
| H.248/SDP encoding example with applied wildcarding: |  | Result: |  |
| a=ptime:? |  | Would give a=ptime:packettime |  |

### 6.15.3	Parameter format

Table 6-15.3/H.248.39 – Parameter format

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | – |  |
| ABNF: (Appendix A/[b-IETF RFC 2327]) | a=fmtp:<format> <format specific parameters> |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=fmtp:? |  | Is invalid as there are two mandatory parameters "format" and "format specific parameters". |  |
| a=fmtp:? ? |  | Would give a=fmtp:format formatspecificparameters |  |

### 6.15.4	Path

Table 6-15.4/H.248.39 – Path

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6/[b-IETF RFC 2327]) | – |  |
| ABNF: (Appendix A/[b-IETF RFC 2327]) | a=path:" MSRP-URL *(SP MSRP-URL) msrp-scheme"://"[userinfo "@"] hostport ["/" session-id] ";" transport |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=path:msrp://? |  | Is invalid as "?" is only for one parameter value. There are two mandatory values "hostport" and "transport". |  |
| a=path:msrp://?;? |  | Would give a=path:msrp://hostport;transport |  |
| a=path:msrp://?@?;? |  | Would give a=path:msrp://userinfo@hostport; transport |  |
| a=path:msrp://?/?;? |  | Would give a=path:msrp://hostport/session-id; transport |  |
| a=path:msrp://?@?/?;? |  | Would give a=path:msrp://userinfo@hostport/ session-id;transport |  |

### 6.15.5	SDP H.248 package attribute

Table 6-15.5/H.248.39 – SDP H.248 package attribute

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: ([ITU-T H.248.15])) | – |  |
| ABNF: ([ITU-T H.248.15]) [ | a=h248item:<package name>/<property name> = <value> |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=h248item:? a=h248item:? ? a=h248item:? ? ? a=h248item:?/ =? a=h248item:? ?=? |  | Are invalid. There are three mandatory parameters "package name", "property name" and "value". |  |
| a=h248item:?/?=? |  | Would give a=h248item:packagename/ propertyname=value |  |

### 6.15.6	RTCP attribute

Table 6-15.6/H.248.39 – RTCP attribute

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: ([b-IETF RFC 3605]) | – |  |
| ABNF: (Clause 2.1/[b-IETF RFC 3605]) | rtcp-attribute = "a=rtcp:" port [nettype space addrtype space connection-address] CRLF |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=rtcp:? ? a=rtcp:? ? ? |  | Are invalid. There is one mandatory parameter "port". There are three optional parameters "nettype", "addrtype" and "connection-address" but all three must be included. |  |
| a=rtcp:? |  | Would give a=rtcp:port |  |
| a=rtcp:? ? ? ? |  | Would give a=rtcp:port nettype addrtype connection-address |  |

### 6.15.7		Silence suppression attribute

The "silenceSupp" attribute is defined in clause 5.6.3.2/[b-IETF RFC 3108]. This attribute may be used for enabling and disabling of silence suppression mode for voice codec types without embedded silence suppression support (e.g., [b-ITU-T G.711] or [b-ITU-T G.726]). This attribute may be also used for non-ATM bearer types like RTP/UDP/IP.

The "silenceSupp" attribute provides five optional sub-fields for parameterization of the silence suppression function. Their usage may depend on the ephemeral H.248 Termination type, e.g., VoIP may only use the <silenceSuppEnable> sub-field, whereas VoATM may use additional sub-fields (see explicit statement in clause 5.6/[b-IETF RFC 3108]).

Table 6-15.7/H.248.39 – Silence suppression attribute

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 5.6.3.2/[b-IETF RFC 3108]) | – |  |
| ABNF: (Clause 5.6.3.2/[b-IETF RFC 3108]) | a=silenceSupp: <silenceSuppEnable> <silenceTimer> <suppPref> <sidUse> <fxnslevel> |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=silenceSupp:? a=silenceSupp:? ? a=silenceSupp:? ? ? a=silenceSupp:? ? ? ? |  | Are invalid. There are five mandatory parameters "silenceSuppEnable", "silenceTimer", "suppPref", "sidUse" and "fxnslevel". |  |
| a=silenceSupp:? ? ? ? ? |  | Would give a=silenceSupp:silenceSuppEnable silenceTimer suppPref sidUse fxnslevel That means that the H.248 MG would reply with the default value for "silenceSuppEnable". That default value may, for instance, be defined within an H.248 Profile specification. |  |

### 6.15.8	Media description with multiple payload types and multiple attribute lines

Clause 6.11 shows a wildcarding example with multiple list entries in <fmt list> in the "m=" line.

The fourth "m=" line parameter <fmt list> is a list of one or more payload formats. This subclause is considering another wildcarding example with multiple list entries in <fmt list>.

The unambiguous resource selection by the MG may require correspondent "a=" lines in case of wildcarding in the list of formats in the "m=" line. This is reflected in 7.1.8/[ITU-T H.248.1].

NOTE 1 – To avoid ambiguity when requesting the MG to reserve and commit resources, the MGC should supply as much information as needed when using under-specification (i.e., CHOOSE) so that the MG can make an unambiguous selection. For example, when using CHOOSE without specifying the required application type (e.g., "media name" in case of SDP encoding), further information may be needed (e.g., attribute lines in case of SDP encoding).

Such a wildcarding configuration must therefore consider the combination of a single "m=" and multiple "a=" lines.

NOTE 2 – Clause 6.11 has examples with scope on "m=" line only.

Table 6-15.8/H.248.39 – Examples

| H.248/SDP encoding example with applied wildcarding: | Result: |
| --- | --- |
| H.248 request: Local{ v=0 c=IN IP4 $ m=audio$ RTP/AVP $ $ a=rtpmap:$ G729D/8000 a=rtpmap:$ G726-16/8000 a=ptime:10 } | H.248 reply: Local{ v=0 c=IN IP4 11.9.19.65 m=audio 5000 RTP/AVP 98 99 a=rtpmap:98 G729D/8000 a=rtpmap:99 G726-16/8000 a=ptime:10 } |

In this example the MGC specifies the encoding name for the two RTP payload types. Thus the MG cannot choose a different encoding name. If the MGC would not send the "a=" line, the MG could choose the encoding name as well. This latter option is not preferred.

### 6.15.9	Source-specific media attributes

Source-specific media attributes are defined in [b-IETF RFC 5576]. These attributes may be used to describe RTP media sources in SDP, to associate attributes with these sources, and to express relationships among such traffic sources.  It also defines several source-level attributes that can be used to describe properties of media sources.

Structure of [b-IETF RFC 5576] SDP attributes:

- Media Attributes:

- the "ssrc" media attribute (see clause 6.15.9.1)

- the "ssrc-group" media attribute (see clause 6.15.9.2)

- Source Attributes

- the "cname" source attribute (see clause 6.15.9.1.2)

- the "previous-ssrc" source attribute (see clause 6.15.9.1.3)

- the "fmtp" source attribute (see clause 6.15.9.1.4)

#### 6.15.9.1	The "ssrc" media attribute

##### 6.15.9.1.1	ABNF structure

See Table 6-15.9.1.1.

Table 6-15.9.1.1/H.248.39 –Media attribute “ssrc”

| SDP Specification: |  |  |
| --- | --- | --- |
| Definition: (Clause 4.1/[b-IETF RFC 5576]) | – |
| ABNF: (Clause 10/[b-IETF RFC 5576]) | a=ssrc:<ssrc-id> <attribute> a=ssrc:<ssrc-id> <attribute>:<value> Formal: ssrc-attr = "ssrc:" ssrc-id SP attribute ; The base definition of "attribute" is in RFC 4566. ; (It is the content of "a=" lines.) ssrc-id = integer ; 0 .. 2**32 - 1 attribute =/ ssrc-attr |

The media attribute "a=ssrc:" must contain an additional source attribute (see next sub-clauses). The SDP wildcarding is also demonstrated in these sub-clauses.

##### 6.15.9.1.2	Wildcarding with source attribute "cname"

See Table 6-15.9.1.2.

Table 6-15.9.1.2/H.248.39 – Source-specific attribute “cname”

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6.1/[b-IETF RFC 5576]) | – |  |
| ABNF: (Clause 10/[b-IETF RFC 5576]) | a=ssrc:<ssrc-id> cname:<cname> Formal: cname-attr = "cname:" cname cname = byte-string ; Following the syntax conventions for CNAME as defined in [b-IETF RFC 3550]. ; The definition of "byte-string" is in [b-IETF RFC 4566]. attribute =/ cname-attr |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=ssrc:<ssrc-id> cname:? |  | Would give: a=ssrc:ssrc-id cname:cname i.e., MG used CNAME value (typically at the level of root), which is either provisioned or locally generated, based on [b-IETF RFC 6222]. |  |

##### 6.15.9.1.3	Wildcarding with source attribute "previous-ssrc"

See Table 6-15.9.1.3.

Table 6-15.9.1.3/H.248.39 – Source-specific attribute “previous-ssrc”

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6.2/[b-IETF RFC 5576]) | – |  |
| ABNF: (Clause 10/[b-IETF RFC 5576]) | a=ssrc:<ssrc-id> previous-ssrc:<ssrc-id> ... Formal: previous-ssrc-attr = "previous-ssrc:" ssrc-id *(SP ssrc-id) attribute =/ previous-ssrc-attr |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=ssrc:? |  | Is invalid. |  |
| a=ssrc:? previous-ssrc:? |  | Would give: a= ssrc: ssrc-id1 previous-ssrc: ssrc-id2, ssrc-id3 … ssrc-idn The use of a multiple value response wildcard is required to return a list of previous ssrcs. |  |

NOTE - The example above illustrates wildcarding a previous ssrc value. The use of the previous-ssrc attribute is dependent on SSRC identifier collision leading to an allocation of a new SSRC identifier. The conditions leading to such an occurrence are not in scope of this Recommendation however the information below details consideration when using such wildcarding:

Such a use case would involve further procedures. For example:

- The MG notifies MGC that SSRC collision was detected, e.g. using a new H.248 event.

- The MGC requests MG to allocate a new local SSRC and includes a wildcarded a=ssrc attribute:
a=ssrc:$ previous-ssrc:$

- The MG replies with a new SSRC and with an old SSRC:
a=ssrc:<new-ssrc-id> previous-ssrc:<old-ssrc-id>

- The MGC uses MG’s reply to generate new a SDP offer containing the MG replied attribute line a=ssrc:<new-ssrc-id> previous-ssrc:<old-ssrc-id>

Variant of step (2):

(2)	The MGC requests the MG to allocate a new local SSRC and includes a partially wildcarded a=ssrc attribute:
a=ssrc:$ previous-ssrc:<old-ssrc-id>
(If the MGC stored  <old-ssrc-id>)

##### 6.15.9.1.4	Wildcarding with source attribute “fmtp"

See Table 6-15.9.1.4.

Table 6-15.9.1.4/H.248.39 – Source-specific attribute “fmtp”

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6.3/[b-IETF RFC 5576]) | – |  |
| ABNF: (Clause 10/[b-IETF RFC 5576]) | a=ssrc:<ssrc> fmtp:<format> <format specific parameters> Formal: Not part of [b-IETF RFC 5576] |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=ssrc:? fmtp:? ? |  | The syntax is valid however the results would be uncertain. The usage in the following row is preferred. |  |
| a=ssrc:? fmtp:<format> <format specific parameters> |  | Would give a=ssrc:ssrc-id fmtp:<format> <format specific parameters> |  |

#### 6.15.9.2	The "ssrc-group" media attribute

See Table 6-15.9.2.

Table 6-15.9.2/H.248.39 –Media attribute “ssrc-group”

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 4.2/[ b-IETF RFC 5576]) | – |  |
| ABNF: (Clause 10/[b-IETF RFC 5576]) | a=ssrc-group:<semantics> <ssrc-id> ... Formal: ssrc-group-attr = "ssrc-group:" semantics *(SP ssrc-id) semantics = "FEC" / "FID" / token ; Matches [b-IETF RFC 5888] definition and ; IANA registration rules in this doc. token = <as defined in [b-IETF RFC 4566] attribute =/ ssrc-group-attr |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=ssrc-group:? ? a=ssrc-group:? ?R |  | Are invalid. Because there are semantic limitations concerning the group semantics according to clause 4.2/[b-IETF RFC 5576]. E.g., meaningful tokens are "FEC" and "FID", but other tokens such "LS", "SFR", "ANAT" or "CS" are normally not applicable in this service context. |  |
| a=ssrc-group:token ?R (NOTE) |  | Would give a=ssrc-group:token list of ssrc-ids |  |
| NOTE – This SDP attribute is interlinked with the SDP "ssrc" media attribute (see clause 6.15.9.1). Thus, unambiguous SDP wildcarding requires the usage of the advanced wildcarding capabilities according to clause 9. In particular: the application of wildcarding interlinkage (see clause 9.6.4). |  |  |  |

The wildcarding of “a=ssrc-group” must also consider the relation to any wildcarding performed on “a=ssrc”. A “ssrc” chosen as a result of a CHOOSE wildcard on one attribute may need to be used on another attribute.  The wildcarding inter-linkage wildcard is used to link the CHOOSE instances. See clause 9.6.4 for more information.

Example:

Wildcarding a=ssrc-group using wildcarding inter-linkage in conjunction with [b-IETF RFC 4588] specified MIME subtype rtx is as follows:

a=ssrc-group:FID $1 $2

a=ssrc:$1 cname:$

a=ssrc:$2 cname:$

Where the MG then allocates one “original stream SSRC” and one “retransmission stream SSRC” and where is returns the “original stream SSRC” in the first a=ssrc attribute lines and the “retransmission stream SSRC” in the second a=ssrc attribute line, for example:

Replied Local Descriptor:

...

a=ssrc-group:FID <original-stream-SSRC> <retransmission-stream-SSRC>

a=ssrc:<original-stream-SSRC> cname:<MG-CNAME>

a=ssrc:<retransmission-stream-SSRC> cname:<MG-CNAME>

See also the example in clause 9.6.4.

### 6.15.10	Fingerprint attribute

The fingerprint attribute is defined by [b-IETF RFC 4572], and is used for transport security protocols such as TLS and DTLS.

Table 6-15.10/H.248.39 – Attribute “fingerprint”

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 5/[b-IETF RFC 4572]) | – |  |
| ABNF: (Clause 5/[b-IETF RFC 4572]) | a=fingerprint:<hash-func> <fingerprint> |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=fingerprint:? |  | Is invalid. Because the mandatory 2-tuple of “name of hash function” and “hash value itself” needs to be both individually wildcarded. |  |
| a=fingerprint:? ? |  | Would give a=fingerprint:hash-func fingerprint |  |

#### 6.15.10.1	Examples

See Table 6-15.10.1.

Table 6-15.10.1/H.248.39 – Examples for attribute “fingerprint”

| H.248/SDP encoding examples with applied wildcarding: | Results: |
| --- | --- |
| Example 1: MGC: a=fingerprint:$ $ | MG would choose a hash function and hash value (‘fingerprint’); and reply to the MGC, e.g.: a=fingerprint: sha-384 fingerprint |
| Example 2: MGC: a=fingerprint:md5 $ | MG would choose a hash value for md5 algorithm; and reply to the MGC, e.g.: a=fingerprint:md5 fingerprint |

# 7	Use for Wildcarding (CHOOSE or ALL)

The MGC should fully specify as many of the SDP parameters as possible when specifying an SDP line in H.248. As indicated above, the MGC shall provide either a fully specified value, wildcard (CHOOSE "$" or ALL "*") or not significant value "-" for each parameter.

For example (see Table 7): The MGC requests that the port be selected and issues the transaction below.

NOTE – Full SDP omitted for brevity.

Table 7/H.248.39 – Examples

| H.248/SDP encoding example with applied wildcarding: | Result: |
| --- | --- |
| H.248 request: Transaction = 10 { Context = 20 { Modify = 30 { Media { Stream = 1 { Local { m=audio $ RTP/AVP 4 }}}}}} | H.248 reply (the port is returned): Reply = 10 { Context = 20 { Modify = 30 { Media { Stream = 1 { Local { m=audio 1111 RTP/AVP 4 }}}}}} |

# 8	Use in auditing

## 8.1	Audit value

Individual auditing of Local and Remote SDP was introduced in [ITU-T H.248.1] Version 3.

For use in Auditing, the MGC should fully specify as many of the parameters in the SDP line as possible, to minimize the number of combinations that are returned. The actual parameters that the MGC requests to be returned should be designated with the ALL "*" wildcard. The parameters of which the MGC has no knowledge and does not require their values to be returned shall be designated with "-" (not significant). In the reply from the MG, these parameters will be marked as "-". Examples follow in Table 8.1.

Table 8-1/H.248.39 – Examples

| H.248/SDP encoding examples with applied wildcarding: | Results: |
| --- | --- |
| The MGC requests that all the attributes be returned with transaction: Transaction = 10 { Context = 20 { AuditValue = 30{ Audit{ Media{ Local{ a=*:* }}}}}} | This could potentially return a long list of all the attributes on the MG. |
| Whereas transaction: Transaction = 10 { Context = 20 { AuditValue = 30{ Audit{ Media{ Local{ a=ptime:* }}}}}} | This would only return the value of the ptime attribute. |
| If the MGC audits a line with multiple parameters (for example: "c=") and it only wants to know the type of network, it would issue the following transaction: Transaction = 10 { Context = 20 { AuditValue = 30{ Audit{ Media{ Local{ c=- * - }}}}}} | It would return the connection line: c=- IP4 – |

## 8.2	Audit capability

Wildcarding of SDP elements for Audit Capabilities is for further study.

# 9 Advanced SDP Wildcarding Package

|  | Package name: | Advanced SDP Wildcarding Package |
| --- | --- | --- |
|  | Package ID: | aswp (0x011c) |
|  | Description: | This package allows the MG to indicate whether or not it supports advanced SDP wildcarding procedures: multiple value response, multiple line response, wildcarding inter-linkage and maintain value. A MGC may use this information to determine what procedures and syntax to use. |
|  | Version: | 1 |
|  | Extends: | None. |

## 9.1 Properties

### 9.1.1 Advanced Wildcarding Support

|  | Property name: | Advanced Wildcarding Support |
| --- | --- | --- |
|  | Property ID: | aws (0x0001) |
|  | Description: | This property indicates whether advanced SDP wildcarding procedures: multiple value response, multiple line response, wildcarding interlinkage and maintain value (as described by clause 9.6) are supported. |
|  | Type: | Sub-list of Enumeration |
|  | Possible values: | MVR Multiple value response MLR Multiple line response WI Wildcard interlinkage MV Maintain value |
|  | Default: | None (unless provisioned otherwise) |
|  | Defined in: | TerminationState (Root Termination only) |
|  | Characteristics: | Read/Write |

## 9.2 Events

None.

## 9.3 Signals

None.

## 9.4 Statistics

None.

## 9.5	Error Codes

None.

## 9.6	Procedures

### 9.6.1	General

The support of the Advanced Wildcarding Support Package (awsp) indicates that a MG supports procedures that allow the following wildcards:

- Multiple value response – In response to a multiple value wildcard request the MG will return (if applicable) multiple values for the wildcarded parameter. The response uses a single SDP line for the response.

- Multiple line response - In response to a multiple line wildcard request the MG will return (if applicable) multiple values for the wildcarded parameter. The response uses multiple SDP lines for the response, one value per SDP line.

- Wildcarding inter-linkage – This allows the MGC to indicate to the MG that a response to one wildcard is used for subsequent wildcards in the particular command.

- Maintain value – This allows the MGC to indicate to the MG that a particular value shall remain unchanged from a previously signalled, wildcarded or provisioned value.

A MGC should perform a packages audit (see clause 7.2.5/[ITU-T H.248.1]) on the Root Termination to determine if a MG supports the “Advanced Wildcarding Support” package. If the MGC requires the use of advanced wildcards it shall set the “Advanced Wildcarding Support” property (awsp/aws) property to the wildcards types it requires unless it has already been set via provisioning.

The Advanced Wildcarding Support Package is only for the H.248.1 encoding of SDP, either when SDP is used natively in H.248 text encoding mode or with the use of SDP equivalents H.248 properties (clause C.11/[ITU-T H.248.1]) in binary encoding mode. It does not relate to the use of CHOOSE wildcards for TerminationIDs or StreamIDs.

If the MGC attempts to use advanced wildcarding procedures toward a MG that does not support this package it is likely error code 403 “Syntax error in transaction request” will be returned.

### 9.6.2	Wildcarded Value Response

A MGC may indicate that it requires a wildcarded multiple value response by appending the character “R” or “r” to a CHOOSE wildcard, i.e. “$R” or “$r”. It may be used where the SDP ABNF definition allows a multiplicity of values, e.g. The ABNF “* (SP value)”. The MGC may use this when it does not know how many values will be returned.

In response to a wildcard multiple value request a MG will return multiple values (if applicable) of the SDP parameter where the wildcard was issued using a single SDP line.

For example: Utilising the “ssrc-group” as defined in 6.15.9.2 a H.248 command containing:

a=ssrc-group:FID $R

would return:

a=srrc-group:FID <ssrc1> <sssrc2> <ssrc3> etc.

## 9.6.3	Wildcarded Line Response

A MGC may indicate that it requires a wildcarded multiple line response by appending the character “L” or “l” to a CHOOSE wildcard, i.e. “$L” or “$l”. This is used where the SDP response may require additional SDP lines but the MGC has no idea how many lines will be returned.

In response to a wildcard multiple value request a MG will return multiple lines (if applicable) of the SDP parameter where the wildcard was issued using a single SDP line.

Only one wildcarded line response request is allowed in an SDP description. If there are other CHOOSE wildcards in the SDP line then these shall be executed independently per line response

For example: Utilising the “ssrc” as defined in clause 6.15.9.2 a H.248 command containing:

a=ssrc:$L cname:$

would return:

a=ssrc:<ssrc2> cname:<cnameA>

a=ssrc:<ssrc1> cname:<cnameB>

a=ssrc:<ssrc3> cname:<cnameC>

etc.

### 9.6.4	Wildcarding inter-linkage

An MGC may indicate that multiple CHOOSE wildcard instances are related by appending a HEX digit (which represent a wildcard label) to a CHOOSE wildcard, i.e. “$[0-F] or $[0-f]”. This allows an MGC to indicate that the resultant value of one CHOOSE should be used for the result of another CHOOSE. This is only used when the identical SDP parameter (semantically and syntactically) is used across SDP lines. Only one wildcard label may be used per wildcard however several wildcard labels may be used per SDP line and SDP description. CHOOSE wildcards with the same wildcard label may be used across SDP lines. The MG will use the value in response to the first CHOOSE wildcard for the results of CHOOSEs with the same label on subsequent lines.

For example: Utilising the “ssrc” and “ssrc-group” attributes as defined in clause 6.15.9.2 a H.248 command containing:

a=ssrc:$1 cname:$

a=ssrc:$2 cname:$

a=ssrc-group:FID $1 $2

would return:

a=ssrc:<ssrca> cname:<cnameA>

a=ssrc:<ssrcb> cname:<cnameB>

a=ssrc-group:FID <ssrca> <ssrcb>

### 9.6.5	Maintain value

The purpose of this wildcard is to allow the MGC to request the preservation of previously set SDP values. The maintain value wildcard "~" maintains unchanged:

- signalled attribute values, or

- previously wildcarded attribute values, or

- provisioned attribute values.

Not using this wildcard implies the complete replication of all SDP information (i.e., fully specified SDP lines) by the MGC, in order not to release previously requested resources at the MG. The MG must process entirely such SDP data and compare it with past received SDP. This process may be avoided through the use of this wildcard.

Use cases for (a, b):

- The wildcard is used in H.248 command replies (when SDP information is replied).

- The wildcard is typically used in subsequent H.248 command requests, i.e., this wildcard is not used in the first H.248 command request, when initially creating a stream endpoint or termination.

Use case (c): The wildcard could be used in the first H.248 command request, under the condition that correspondent values are already prepared via management actions (at MGC and MG level).

Wildcard "~" shall not be used in case of ambiguous SDP media descriptions (e.g., when multiple SDP lines would be linked via an identifier, which is wildcarded).

### 9.6.6 Interactions

### 9.6.6.1 Interactions between wildcarding qualifiers "Value Response", "Line Response" and "Interlinkage"

The scope of the wildcarding inter-linkage label is limited to the Command, Termination/Stream and Group where the wildcard is issued. For example:

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Add = T1 { Media { Stream = 1 { Local { v=0 c=IN IP4 $ m=audio $1 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 $ m=audio $1 RTP/AVP 8 } } } } } } | Even though Stream 1 and 2 both use an interlinked wildcard label $1 it does not mean that the MG should use the same value for Stream 2. |

A MGC shall not use both the Wildcard Value Response “R” and Wildcard Line Response “L” in the same wildcard. E.g. “$LR” is not valid.

A MGC may utilise a wildcard label with a multiple value response (e.g. $1R or $R1) or with a multiple line response request (e.g. $1L or $L1) in combination with wildcarding inter-linkage. A MGC may use the same wildcard label for both multiple value or multiple line response requests. This indicates that multiple values from one SDP line should be used to populate a value on multiple SDP lines and vice versa.

The use of a multiple values response request and a multiple line response request on one SDP line is valid.

The following example illustrates the use of a multiple value and multiple line response request in an SDP description:

a=ssrc-group:FID $1R

a=ssrc:$1L cname:$

The wildcard inter-linkage element “1” indicates that the “R” and the “L” wildcards are related. Therefore the MG uses the results of the “R” CHOOSE wildcard to populate multiple instance of a= lines for the response, e.g.

a=ssrc-group:FID <ssrc2> <ssrc1> <ssrc3>

a=ssrc:<ssrc2> cname:<cnameA>

a=ssrc:<ssrc1> cname:<cnameB>

a=ssrc:<ssrc3> cname:<cnameC>

#### 9.6.6.2 Additional interactions with wildcarding qualifier "Maintain Value"

The combined usage of multiple wildcarding qualifiers in combination with the "Maintain Value" is principally feasible, but conditional, dependent on particular SDP elements and/or associated SDP lines.

It is not recommended to use the wildcarding qualifier "Maintain Value" with the other wildcarding qualifiers per SDP line.

Appendix I
Example signalling scenarios for
the maintain value wildcard "~"

(This appendix does not form an integral part of this Recommendation)

## I.1	Purpose

The benefit of the maintain value wildcard "~" is illustrated through two examples: one without (clause I.2) and one with (clause I.3) wildcarding. The value of this wildcard is primarily motivated by the "size of SDP information". E.g., in context of the discussion of large SDP objects (also known as "SDP blob"), such as used in the signalling SDP media descriptions for:

- multi-media types/formats/parameters/etc. configurations

- key material as part of cryptographical information

- fingerprints as part of cryptographical information

## I.2	Signalling without the maintain value wildcard "~"

The usage of the maintain value wildcard "~" is illustrated using an example of the SDP attribute "a=fingerprint". This SDP element is an example representation of SDP elements which may have a large value size. An ADD.request command example without wildcard usage is used as starting point, see Table I.1.

Table I.1 – Example command encoding – MGC Add request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { Add = ip/$/$/$ { Media { Stream = 1 { LocalControl { ... }, Local { v=0 c=IN IP4 $ m=audio $ UDP/TLS/RTP/SAVP - b=AS:7 a=setup:passive a=fingerprint: sha-256 0E:CB:C3:2A:EF:E8:3F:6A:FD:19:07:20:10:80:69:B4:17:E2:3E:3B:2C:89:41:FB:06:85:E6:71:26:A2:19:51 }, Remote { v=0 c=IN IP4 135.117.113.26 m=audio 56376 UDP/TLS/RTP/SAVP - a=fingerprint:sha-256 8F:AA:57:46:7C:C1:D3:50:74:8D:22:C9:10:12:BE:AD:F0:13:9C:4F:D1:4F:B7:F3:96:5B:C3:70:FC:C5:AA:AE } … } | The SDP attribute "a=fingerprint" is contained in both the LD and RD. The value is fully specified in the request command in MGC-to-MG direction. |

A possible example MG reply is indicated in Table I.2:

Table I.2 – Example command encoding– MG Add reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [125.125.125.111]:55555 Reply = 1 { Context = <C1> { Add = <IP_A> { Media { Stream = 1 { Local { v=0 c=IN IP4 172.40.7.1 b=AS:7 m=audio 23914 UDP/TLS/RTP/SAVP - a=setup:passive a=fingerprint:sha-256 0E:CB:C3:2A:EF:E8:3F:6A:FD:19:07:20:10:80:69:B4:17:E2:3E:3B:2C:89:41:FB:06:85:E6:71:26:A2:19:51 … }, Remote { v=0 c=IN IP4 135.117.113.26 m=audio 56376 UDP/TLS/RTP/SAVP - a=fingerprint:sha-256 8F:AA:57:46:7C:C1:D3:50:74:8D:22:C9:10:12:BE:AD:F0:13:9C:4F:D1:4F:B7:F3:96:5B:C3:70:FC:C5:AA:AE } ... } | The entire SDP value is replied by the MG. Now, the MGC should validate that information by comparing the sent and received data. |

The stream endpoint is modified in a subsequent command cycle, e.g., by the following MODify.‌request command (Table I.3):

Table I.3 – Example command encoding – MGC Modify request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = <C1> { Modify = <IP_A> { Media { Stream = 1 { LocalControl { ... }, Local { v=0 c=IN IP4 172.40.7.1 b=AS:7 m=audio 23914 UDP/TLS/RTP/SAVP - a=setup:passive a=fingerprint: sha-256 0E:CB:C3:2A:EF:E8:3F:6A:FD:19:07:20:10:80:69:B4:17:E2:3E:3B:2C:89:41:FB:06:85:E6:71:26:A2:19:51 a=rtcp-mux }, Remote { v=0 c=IN IP4 135.117.113.26 m=audio 56376 UDP/TLS/RTP/SAVP - a=fingerprint:sha-256 8F:AA:57:46:7C:C1:D3:50:74:8D:22:C9:10:12:BE:AD:F0:13:9C:4F:D1:4F:B7:F3:96:5B:C3:70:FC:C5:AA:AE a=rtcp-mux } … } | The MGC has to provide the full specification of the SDP attribute again. The MG has to validate that information by comparing (and possibly updating) the information received in the previous command request (Table I.1). This is a cumbersome and unnecessary task when the value itself was not changed (as here). |

A possible example MG reply is indicated in Table I.4:

Table I.4 – Example command encoding– MG Modify reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [125.125.125.111]:55555 Reply = 2 { Context = <C1> { Modify = <IP_A> { Media { Stream = 1 { Local { v=0 c=IN IP4 172.40.7.1 b=AS:7 m=audio 23914 UDP/TLS/RTP/SAVP - a=setup:passive a=fingerprint:sha-256 0E:CB:C3:2A:EF:E8:3F:6A:FD:19:07:20:10:80:69:B4:17:E2:3E:3B:2C:89:41:FB:06:85:E6:71:26:A2:19:51 a=rtcp-mux … }, Remote { v=0 c=IN IP4 135.117.113.26 m=audio 56376 UDP/TLS/RTP/SAVP - a=fingerprint:sha-256 8F:AA:57:46:7C:C1:D3:50:74:8D:22:C9:10:12:BE:AD:F0:13:9C:4F:D1:4F:B7:F3:96:5B:C3:70:FC:C5:AA:AE a=rtcp-mux } … } | Same comment as in Table I.2. Now, the MGC might be loaded with an unnecessary task again. |

## I.3	Signalling using the maintain value wildcard "~"

The same signalling example, with two command request/reply cycles, is now used with wildcard "~". The wildcard as such is applicable due to unchanged parameter values. Table I.5 provides the starting point:

Table I.5 – Example command encoding – MGC Add request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { Add = ip/$/$/$ { Media { Stream = 1 { LocalControl { ... }, Local { v=0 c=IN IP4 $ m=audio $ UDP/TLS/RTP/SAVP - b=AS:7 a=setup:passive a=fingerprint: sha-256 0E:CB:C3:2A:EF:E8:3F:6A:FD:19:07:20:10:80:69:B4:17:E2:3E:3B:2C:89:41:FB:06:85:E6:71:26:A2:19:51 }, Remote { v=0 c=IN IP4 135.117.113.26 m=audio 56376 UDP/TLS/RTP/SAVP - a=fingerprint:sha-256 8F:AA:57:46:7C:C1:D3:50:74:8D:22:C9:10:12:BE:AD:F0:13:9C:4F:D1:4F:B7:F3:96:5B:C3:70:FC:C5:AA:AE } … } | This is identical to Table I.1, i.e., a fully specified SDP value (because such cryptographic information is normally out of scope of any provisioning). |

A possible example MG reply is indicated in Table I.6:

Table I.6 – Example command encoding– MG Add reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [125.125.125.111]:55555 Reply = 1 { Context = <C1> { Add = <IP_A> { Media { Stream = 1 { Local { v=0 c=IN IP4 172.40.7.1 b=AS:7 m=audio 23914 UDP/TLS/RTP/SAVP - a=setup:passive a=fingerprint:~ ~ … }, Remote { v=0 c=IN IP4 135.117.113.26 m=audio 56376 UDP/TLS/RTP/SAVP - a=fingerprint:~ ~ } … } | The two SDP attribute parameter values could be both wildcarded "~". Thus, the MG acknowledges that it has accepted values sent by the MGC. |

Table I.7 illustrates the MODify.request command:

Table I.7 – Example command encoding – MGC Modify request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = <C1> { Modify = <IP_A> { Media { Stream = 1 { LocalControl { ... }, Local { v=0 c=IN IP4 172.40.7.1 b=AS:7 m=audio 23914 UDP/TLS/RTP/SAVP - a=setup:passive a=fingerprint: ~ ~ a=rtcp-mux }, Remote { v=0 c=IN IP4 135.117.113.26 m=audio 56376 UDP/TLS/RTP/SAVP - a=fingerprint:~ ~ a=rtcp-mux } … } | The MGC could use "~" wildcarding again. This simplifies processing at the MGC and MG side. |

A possible example MG reply is indicated in Table I.8.

Table I.8 – Example command encoding– MG Modify reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [125.125.125.111]:55555 Reply = 2 { Context = <C1> { Modify = <IP_A> { Media { Stream = 1 { Local { v=0 c=IN IP4 172.40.7.1 b=AS:7 m=audio 23914 UDP/TLS/RTP/SAVP - a=setup:passive a=fingerprint:~ ~ a=rtcp-mux … }, Remote { v=0 c=IN IP4 135.117.113.26 m=audio 56376 UDP/TLS/RTP/SAVP - a=fingerprint:~ ~ a=rtcp-mux } … } | And the MG could also use "~" wildcarding again. This simplifies processing at the MG and MGC side. |

## I.4	Evaluation of example

The maintain value wildcard "~" could be applied in 3-out-of-4 commands in this example (see clause I.3 versus I.2). It could be also demonstrated that wildcard "~" is used in command replies (see Tables I.6 and I.8) besides the usage in the MODify command request (see Table I.5).

Furthermore, it should be emphasized that the processing savings are not only related to the sending H.248 entity, but rather both H.248 entities (MGC and MG) benefit from wildcarding.

Bibliography

[b-ITU-T G.711]		Recommendation ITU-T G.711 (11/1988), Pulse code modulation (PCM) of voice frequencies.

[b-ITU-T G.726]		Recommendation ITU-T G.726 (12/1990), 40, 32, 24, 16 kbit/s adaptive differential pulse code modulation (ADPCM).

[b-ITU-T H.248.49]		Recommendation ITU-T H.248.49 (08/2007), Gateway control protocol: Session description protocol RFC and capabilities packages.

[b-ETSI ES 283 018]	ETSI ES 283 018 (2007), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Resource and Admission Control: H.248 Profile for controlling Border Gateway Functions (BGF) in the Resource and Admission Control Subsystem (RACS); Protocol specification.
NOTE - This is also called “H.248 Ia profile version 1”.

[b-ETSI TR 183 068]	ETSI TR 183 068 (2009), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Resource and Admission Control: Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

[b-IETF RFC 2327]	IETF RFC 2327 (1998), SDP: Session Description Protocol.

[b-IETF RFC 3108]	IETF RFC 3108 (2001), Conventions for the use of the Session Description Protocol (SDP) for ATM Bearer Connections.

[b-IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications

[b-IETF RFC 3605]	IETF RFC 3605 (2003), Real Time Control Protocol (RTCP) attribute in Session Description Protocol (SDP).

[b-IETF RFC 4572]	IETF RFC 4572 (2006), Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP).

[b-IETF RFC 4566]	IETF RFC 4566 (2006), SDP: Session Description Protocol.

[b-IETF RFC 4588]	IETF RFC 4588 (4588), RTP Retransmission Payload Format.

[b-IETF RFC 5576]	IETF RFC 5576 (2009), Source-Specific Media Attributes in the Session Description Protocol.

[b-IETF RFC 5888]	IETF RFC 5888 (2010) The Session Description Protocol (SDP) Grouping Framework.

[b-IETF RFC 6222]	IETF RFC 6222 (2011), Guidelines for Choosing RTP Control Protocol (RTCP) Canonical Names (CNAMEs).

________________

____________________

Blob: binary or basic large object, relates to a collection of data stored as a single entity.