| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4651 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | H.TPS-SIG: Interaction of CLUE protocol and H.323 protocol stack |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution tries to consider the interaction of CLUE protocol and H.323 protocol stack in the H.323 based telepresence system. It’s proposed to provide some recommendations on dealing with intermediary stages in non-atomic operations, and provide constraints on when CLUE-configured media can be sent.

Introduction

A telepresence system defined by ITU-T H.420 (ex H.TPS-ARCH) will involve both H.323 protocol stack and CLUE protocol. It’s necessary to consider how to interact with each other. H.245 capability and logic channel exchange are used to negotiate the specific media capabilities that can be delivered to specific addresses on a device. Meanwhile, a CLUE protocol transported via a CLUE data channel or existed H.245 channel, is used to negotiate the capture sources available, their attributes and any constraints in their use, along which captures the far end provides a device wishes to receive.

Discussion

For H.323 based telepresence system, it encounters the following questions:

(1) For implementation, CLUE protocol will be a new protocol stack separately with the existed H.323 protocol stack, how do these two protocol stacks with their own state machines to interact with each other?

(2) Information about media streams controlled by CLUE is split between two message types: H.245 and CLUE. Some operations require changes to both H.245 and CLUE messaging, such as advertising support for a new transmissible capture with associated stream. How to cooperate with each other for capability negotiation?

(3) The non-atomic nature of H.245/CLUE protocol interaction may mean that there are temporary periods. We need to give some recommendations for operating with non-atomic operations.

(4) Though there are two optional methods for CLUE messages carriage, via SCTP channel or existing H.245 channel, issues for the interaction between two protocol stacks are nearly the same.

At the same time, there is an editor’s note in clause 8.4 of H.TPS-SIG:

“The intention of this section is to detail the interaction between CLUE signalling messages and logical channels whilst the session has been established. This will cover issues such as protocol state co-ordination etc.”

It’s also indicated that interaction between CLUE signalling messages and logic channels should be considered. Besides of that, the contributors thought that there are also the same issues between CLUE signalling and other H.245 signalling.

On the other hand, the draft of CLUE signalling in IETF CLUE [draft-ietf-clue-signaling-03] has also discussed on interaction of CLUE protocol and SDP negotiations. Many interaction issues between CLUE protocol and SDP negotiations are similar with which between CLUE protocol and H.245 capability/logic channel negotiations.

So, it’s proposed to start discussion on interaction between CLUE protocol and existed H.323 protocol stack, and try to give some principles, constraints, recommendations as below as a start.

Conclusion

It’s proposed to start discussion on interaction between CLUE protocol and existed H.323 protocol stack and accept the following content as the initial text in recommendation of H.TPS-SIG.

Draft new ITU-T H.TPS-SIG

Signalling for telepresence-enabled conferencing

……

- 7.3   Interaction of CLUE protocol and H.323 protocol stack

Information about media streams controlled by CLUE is split between two message types: H.245 capability message, which defines basic media types and limits, and the CLUE message, which defines properties of capture devices available, scene information and additional constraints. As a result certain operations, such as advertising support for a new transmissible capture with associated stream, require changes to both H.245 and CLUE messaging.

A telepresence endpoint should have the ability to get or update protocol states from both the H.323 (H.245 included) and CLUE protocol stack, and to determine how to implement the coming operations according to both protocol states above and the signalling procedures.

This section defines some principles for interaction between H.323 (H.245 included) and CLUE protocols, provides some recommendations on dealing with intermediary stages in non-atomic operations, and mandates additional constraints on when CLUE-configured media can be sent.

- 7.3.1   Independence of H.245 and CLUE negotiation

To avoid the need to implement interlocking state machines with the potential to reach invalid states if messages were to be lost, or be rewritten en-route by middle boxes, the state machines in H.323 (H.245 included) and CLUE operate independently. The state of the CLUE does not restrict when an implementation may send a new H.245 message, and likewise the implementation’s ability to send a new CLUE ADVERTISEMENT or CONFIGURE message is not restricted by the results of or the state of the most recent H.245 capability/logic channel negotiation.

A device in a CLUE-enabled call MUST NOT delay an ongoing H.245 capability/logic channel messages transaction; an implementation MUST NOT delay an H.245 capability/logic channel messages exchange while waiting for CLUE negotiation to complete or for a CONFIGURE message to arrive.

A device in a CLUE-enabled call MUST NOT delay any mandatory state transitions in the CLUE participant or media provider/consumer state machines due to the presence or absence of an ongoing H.245 capability/logic channel messages exchange.

- 7.3.2   Constraints on sending media

While H.245 capability and CLUE message states do not impose constraints on each other, both impose constraints on the sending of media – CLUE controlled media MUST NOT be sent unless it has been negotiated in both CLUE and H.245 capability exchange: an implementation MUST NOT send a specific CLUE capture encoding unless its most recent H.245 capability/logic channel exchange contains an active media channel for that encoding AND the far end has sent a CLUE CONFIGURE message specifying a valid capture for that encoding.

- 7.3.3   Recommendations for operating with non-atomic operations

CLUE-capable devices MUST be able to handle states in which CLUE messages make reference to EncodingIDs that do not match the most recently received H.245 capability/logic channel messages, irrespective of the order in which H.245 capability/logic channel and CLUE messages are received. While these mis-matches will usually be transitory a device MUST be able to cope with such mismatches remaining indefinitely. However, this document makes some recommendations on message ordering for these non-atomic transitions.

CLUE-capable devices SHOULD ensure that any inconsistencies between H.245 capability/logic channel signalling and CLUE signalling are temporary by sending updated H.245 message (such as H.245 TCS/ACK, OLC/ACK) or CLUE messages as soon as the relevant state machines and other constraints permit.

__________________